PART 129--OPERATIONS: FOREIGN AIR CARRIERS AND FOREIGN OPERATORS OF U.S.-

REGISTERED AIRCRAFT ENGAGED IN COMMON CARRIAGE

Special Federal Aviation Regulation No. 38-2 [Note]

Sec.

129.1  Applicability.

129.11  Operations specifications.

129.13  Airworthiness and registration certificates.

129.14  Maintenance program and minimum equiupment list requirements for

U.S.-registered aircraft.

129.15  Flight crewmember certificates.

129.17  Radio equipment.

129.18  Traffic Alert and Collision Avoidance System.

129.19  Air traffic rules and procedures.

129.21  Control of traffic.

129.23  Transport category cargo service airplanes: Increased zero fuel and

landing weights.

129.25  Airplane security.

129.26  Use of X-ray system.

129.27  Prohibition against carriage of weapons.

129.29  Prohibition against smoking.

Appendix A to Part 129--Application for Operations Specifications by Foreign

Air Carriers

Authority: 49 U.S.C. App. 1346, 1354(a), 1356, 1357, 1421, 1502, 1511, and

1522; 49 U.S.C. 106(g) (revised Pub. L. 97-449, January 12, 1983).

Source: Docket No. 1994, 29 FR 1720, Feb. 5, 1964, unless otherwise noted.

Special Federal Aviation Regulation No. 38-2

EDITORIAL NOTE: For the text of SFAR No. 38-2, see Part 121 of this

chapter.

Sec. 129.1  Applicability.

(a) Except as provided in paragraph (b) of this section, this part

prescribes rules governing the operation within the United States of each

foreign air carrier holding a permit issued by the Civil Aeronautics Board or

the Department of Transportation under section 402 of the Federal Aviation

Act of 1958 (49 U.S.C. 1372) or other appropriate economic or exemption

authority issued by the Civil Aeronautics Board or the Department of

Transportation.

(b) Section 129.14 also applies to U.S.-registered aircraft operated in

common carriage by a foreign person or foreign air carrier solely outside the

United States. For the purpose of this part, a foreign person is any person,

not a citizen for the United States, who operates a U.S.-registered aircraft

in common carriage solely outside the United States.

[Doc. No. 24856, Amdt. 129-14, 52 FR 20029, May 28, 1987]

Sec. 129.11   Operations specifications.

(a) Each foreign air carrier shall conduct its operations within the United

States in accordance with operations specifications issued by the

Administrator under this part and in accordance with the Standards and

Recommended Practices contained in Part I (International Commercial Air

Transport) of Annex 6 (Operation of Aircraft) to the Convention on

International Civil Aviation Organization. Operations specifications shall

include:

(1) Airports to be used;

(2) Routes or airways to be flown, and

(3) Such operations rules and practices as are necessary to prevent

collisions between foreign aircraft and other aircraft.

(4) Registration marketings of each U.S.-registered aircraft.

(b) An application for the issue or amendment of operations specifications

must be submitted in duplicate, at least 30 days before beginning operations

in the United States, to the Flight Standards District Office in the area

where the applicant's principal business office is located or to the Regional

Flight Standards Division Manager having jurisdiction over the area to be

served by the operations. If a military airport of the United States is to be

used as a regular, alternate, refueling, or provisional airport, the

applicant must obtain written permission to do so from the Washington

Headquarters of the military organization concerned and submit two copies of

that written permission with his application. Detailed requirements governing

applications for the issue or amendment of operations specifications are

contained in Appendix A.

(c) No person operating under this part may operate or list on its

operations specifications any airplane listed on operations specifications

issued under Part 125.

[Doc. No. 1994, 29 FR 1720, Feb. 5, 1964, as amended by Amdt. 129-14, 52 FR

20029, May 28, 1987; Amdt. 129-19, 54 FR 39294, Sept. 25, 1989; 54 FR 51972,

Dec. 19, 1989]

Sec. 129.13   Airworthiness and registration certificates.

(a) No foreign air carrier may operate any aircraft within the United

States unless that aircraft carries current registration and airworthiness

certificates issued or validated by the country of registry and displays the

nationality and registration markings of that country.

(b) No foreign air carrier may operate a foreign aircraft within the United

States except in accordance with the limitations on maximum certificated

weights prescribed for that aircraft and that operation by the country of

manufacture of the aircraft.

Sec. 129.14  Maintenance program and minimum equipment list requirements for

U.S.-registered aircraft.

(a) Each foreign air carrier and each foreign person operating a U.S.-

registered aircraft within or outside the United States in common carriage

shall ensure that each aircraft is maintained in accordance with a program

approved by the Administrator.

(b) No foreign air carrier or foreign person may operate a U.S.-registered

aircraft with inoperable instruments or equipment unless the following

conditions are met:

(1) A master minimum equipment list exists for the aircraft type.

(2) The foreign operator submits for review and approval its aircraft

minimum equipment list based on the master minimum equipment list, to the FAA

Flight Standards District Office having geographic responsibility for the

operator. The foreign operator must show, before minimum equipment list

approval can be obtained, that the maintenance procedures used under its

maintenance program are adequate to support the use of its minimum equipment

list.

(3) For leased aircraft maintained and operated under a U.S. operator's

continuous airworthiness maintenance program and FAA-approved minimum

equipment list, the foreign operator submits the U.S. operator's approved

continuous airworthiness maintenance program and approved aircraft minimum

equipment list to the FAA office prescribed in paragraph (b)(2) of this

section for review and evaluation. The foreign operator must show that it is

capable of operating under the lessor's approved maintenance program and that

it is also capable of meeting the maintenance and operational requirements

specified in the lessor's approved minimum equipment list.

(4) The FAA letter of authorization permitting the operator to use an

approved minimum equipment list is carried aboard the aircraft. The minimum

equipment list and the letter of authorization constitute a supplemental type

certificate for the aircraft.

(5) The approved minimum equipment list provides for the operation of the

aircraft with certain instruments and equipment in an inoperable condition.

(6) The aircraft records available to the pilot must include an entry

describing the inoperable instruments and equipment.

(7) The aircraft is operated under all applicable conditions and

limitations contained in the minimum equipment list and the letter

authorizing the use of the list.

[Docket No. 24856, Amdt. No. 129-14, 52 FR 20029, May 28, 1987]

Sec. 129.15   Flight crewmember certificates.

No person may act as a flight crewmember unless he holds a current

certificate or license issued or validated by the country in which that

aircraft is registered, showing his ability to perform his duties connected

with operating that aircraft.

[Amdt. 129-3, 30 FR 16074, Dec. 24, 1965]

Sec. 129.17   Radio equipment.

(a) Subject to the applicable laws and regulations governing ownership and

operation of radio equipment, each foreign air carrier shall equip its

aircraft with such radio equipment as is necessary to properly use the air

navigation facilities, and to maintain communications with ground stations,

along or adjacent to their routes in the United States.

(b) Whenever VOR navigational equipment is required by paragraph (a) of

this section, at least one distance measuring equipment unit (DME), capable

of receiving and indicating distance information from the VORTAC facilities

to be used, must be installed on each airplane when operated at or above

24,000 feet MSL within the 50 states, and the District of Columbia.

[Doc. No. 1994, 29 FR 1720, Feb. 5, 1964, as amended by Amdt. 129-2, 30 FR

10288, Aug. 19, 1965, Amdt. 129-7, 41 FR 47230, Oct. 30, 1976]

Sec. 129.18  Traffic Alert and Collision Avoidance System.

(a) After December 30, 1993, no foreign air carrier may operate in the

United States a turbine powered airplane that has a maximum passenger seating

configuration, excluding any pilot seat, of more than 30 seats unless it is

equipped with--

(1) A TCAS II traffic alert and collision avoidance system capable of

coordinating with TCAS units that meet the specifications of TSO C-119, and

(2) The appropriate class of Mode S transponder.

(b) After February 9, 1995, no foreign air carrier may operate in the

United States a turbine powered airplane that has a passenger seating

configuration, excluding any pilot seat, of 10 to 30 seats unless it is

equipped with a traffic alert and collision avoidance system. If a TCAS II

system is installed, it must be capable of coordinating with TCAS units that

meet the specifications of TSO C-119.

[Doc. No. 25355, 54 FR 951, Jan. 10, 1989, as amended at Amdt. 129-21, 55 FR

13247, Apr. 9, 1990]

*****************************************************************************

55 FR 13242, No. 68, Apr. 9, 1990

SUMMARY: This rule revises the schedule for installing Traffic Alert and

Collision Avoidance Systems (TCAS II) on airplanes with more than 30

passenger seats. The TCAS II system will provide a collision avoidance

capability that operates independently of the ground-based Air Traffic

Control (ATC) system and in areas where there is no ATC coverage. Congress

recently passed legislation permitting an extension of the schedule. This

action implements the legislation, reduces the prospect that carriers will

divert critical maintenance and modification resources away from other safety

programs to meet the TCAS II schedule, and allows the FAA to evaluate the

operation of TCAS II in the total ATC environment.

EFFECTIVE DATE: May 9, 1990.

*****************************************************************************

Sec. 129.19   Air traffic rules and procedures.

(a) Each pilot must be familiar with the applicable rules, the navigational

and communications facilities, and the air traffic control and other

procedures, of the areas to be traversed by him within the United States.

(b) Each foreign air carrier shall establish procedures to assure that each

of its pilots has the knowledge required by paragraph (a) of this section and

shall check the ability of each of its pilots to operate safely according to

applicable rules and procedures.

(c) Each foreign air carrier shall conform to the practices, procedures,

and other requirements prescribed by the Administrator for U.S. air carriers

for the areas to be operated in.

Sec. 129.21   Control of traffic.

(a) Subject to applicable immigration laws and regulations, each foreign

air carrier shall furnish the ground personnel necessary to provide for two-

way voice communication between its aircraft and ground stations, at places

where the Administrator finds that voice communication is necessary and that

communications cannot be maintained in a language with which ground station

operators are familiar.

(b) Each person furnished by a foreign air carrier under paragraph (a) of

this section must be able to speak both English and the language necessary to

maintain communications with the aircraft concerned, and shall assist ground

personnel in directing traffic.

Sec. 129.23   Transport category cargo service airplanes: Increased zero fuel

and landing weights.

(a) Notwithstanding the applicable structural provisions of the transport

category airworthiness regulations, but subject to paragraphs (b) through (g)

of this section, a foreign air carrier may operate (for cargo service only)

any of the following transport category airplanes (certificated under Part 4b

of the Civil Air Regulations effective before March 13, 1956) at increased

zero fuel and landing weights--

(1) DC-6A, DC-6B, DC-7B, and DC-7C; and

(2) L-1049 B, C, D, E, F, G, and H, and the L-1649A when modified in

accordance with supplemental type certificate SA 4-1402.

(b) The zero fuel weight (maximum weight of the airplane with no disposable

fuel and oil) and the structural landing weight may be increased beyond the

maximum approved in full compliance with applicable rules only if the

Administrator finds that--

(1) The increase is not likely to reduce seriously the structural strength;

(2) The probability of sudden fatigue failure is not noticeably increased;

(3) The flutter, deformation, and vibration characteristics do not fall

below those required by applicable regulations; and

(4) All other applicable weight limitations will be met.

(c) No zero fuel weight may be increased by more than five percent, and the

increase in the structural landing weight may not exceed the amount, in

pounds, of the increase in zero fuel weight.

(d) Each airplane must be inspected in accordance with the approved special

inspection procedures, for operations at increased weights, established and

issued by the manufacturer of the type of airplane.

(e) A foreign air carrier may not operate an airplane under this section

unless the country of registry requires the airplane to be operated in

accordance with the passenger-carrying transport category performance

operating limitations in Part 121 or the equivalent.

(f) The Airplane Flight Manual for each airplane operated under this

section must be appropriately revised to include the operating limitations

and information needed for operation at the increased weights.

(g) Each airplane operated at an increased weight under this section must,

before it is used in passenger service, be inspected under the special

inspection procedures for return to passenger service established and issued by

the manufacturer and approved by the Administrator.

[Amdt. 129-1, 29 FR 19098, Dec. 30, 1964]

Sec. 129.25  Airplane security.

(a) The following are definitions of terms used in this section:

(1) "Approved security program" means a security program required by Part

108 of this title approved by the Administrator.

(2) "Certificate holder" means a person holding an FAA air carrier

operating certificate or operating certificate when that person engages in

scheduled passenger or public charter operations, or both.

(3) "Passenger seating configuration" means the total number of seats for

which the aircraft is type certificated that can be made available for

passenger use aboard a flight and includes that seat in certain airplanes

which may be used by a representative of the Administrator to conduct flight

checks but is available for revenue purposes on other occasions.

(4) "Private charter" means any charter for which the charterer engages the

total capacity of an airplane for the carriage only of:

(i) Passengers in civil or military air movements conducted under contract

with the Government of the United States or the Government of a foreign

country; or

(ii) Passengers invited by the charterer, the cost of which is borne

entirely by the charterer and not directly or indirectly by the individual

passengers.

(5) "Public charter" means any charter that is not a "private charter."

(6) "Scheduled passenger operations" means holding out to the public of air

transportation service for passengers from identified air terminals at a set

time announced by timetable or schedule published in a newspaper, magazine,

or other advertising medium.

(7) "Sterile area" means an area to which access is controlled by the

inspection of persons and property in accordance with an approved security

program or a security program used in accordance with Sec. 129.25.

(b) Each foreign air carrier landing or taking off in the United States

shall adopt and use a security program, for each scheduled and public charter

passenger operation, that meets the requirements of--

(1) Paragraph (c) of this section for each operation with an airplane

having a passenger seating configuration of more than 60 seats;

(2) Paragraph (c) of this section for each operation that will provide

deplaned passengers access, that is not controlled by a certificate holder

using an approved security program or a foreign air carrier using a security

program required by this section, to a sterile area;

(3) Paragraph (c) of this section for each operation with an airplane

having a passenger seating configuration of more than 30 seats but less than

61 seats for which the FAA has notified the foreign air carrier that a threat

exists; and

(4) Paragraph (d) of this section for each operation with an airplane

having a passenger seating configuration of more than 30 seats but less than

61 seats, when the the Director of Civil Aviation Security or a designate of

the Director has not notified the foreign air carrier in writing that a

threat exists with respect to that operation.

(c) Each security program required by paragraph (b) (1), (2), or (3) of

this section shall be designed to--

(1) Prevent or deter the carriage aboard airplanes of any explosive,

incendiary device or a deadly or dangerous weapon on or about each

individual's person or accessible property, except as provided in Sec. 129.27

of this part, through screening by weapon-detecting procedures or facilities;

(2) Prohibit unauthorized access to airplanes;

(3) Ensure that baggage is accepted by a responsible agent of the foreign

air carrier; and

(4) Prevent cargo and checked baggage from being loaded aboard its

airplanes unless handled in accordance with the foreign air carrier's

security procedures.

(d) Each security program required by paragraph (b)(4) of this section

shall include the procedures used to comply with the applicable requirements

of paragraphs (h)(2) and (i) of this section regarding law enforcement

officers.

(e) Each foreign air carrier required to adopt and use a security program

pursuant to paragraph (b) of this section shall have a security program

acceptable to the Administrator. A foreign air carrier's security program is

acceptable only if the Administrator finds that the security program provides

passengers a level of protection similar to the level of protection provided

by U.S. air carriers serving the same airports. Foreign air carriers shall

employ procedures equivalent to those required of U.S. air carriers serving

the same airport if the Administrator determines that such procedures are

necessary to provide passengers a similar level of protection. The following

procedures apply for acceptance of a security program by the Administrator:

(1) Unless otherwise authorized by the Administrator, each foreign air

carrier required to have a security program by paragraph (b) of this section

shall submit its program to the Administrator at least 90 days before the

intended date of passenger operations. The proposed security program must be

in English unless the Administrator requests that the proposed program be

submitted in the official language of the foreign air carrier's country. The

Administrator will notify the foreign air carrier of the security program's

acceptability, or the need to modify the proposed security program for it to

be acceptable under this part, within 30 days after receiving the proposed

security program. The foreign air carrier may petition the Administrator to

reconsider the notice to modify the security program within 30 days after

receiving a notice to modify.

(2) In the case of a security program previously found to be acceptable

pursuant to this section, the Administrator may subsequently amend the

security program in the interest of safety in air transportation or in air

commerce and in the public interest within a specified period of time. In

making such an amendment, the following procedures apply:

(i) The Administrator notifies the foreign air carrier, in writing, of a

proposed amendment, fixing a period of not less than 45 days within which the

foreign air carrier may submit written information, views, and arguments on

the proposed amendment.

(ii) At the end of the comment period, after considering all relevant

material, the Administrator notifies the foreign air carrier of any amendment

to be adopted and the effective date, or rescinds the notice of proposed

amendment. The foreign air carrier may petition the Administrator to

reconsider the amendment, in which case the effective date of the amendment

is stayed until the Administrator reconsiders the matter.

(3) If the Administrator finds that there is an emergency requiring

immediate action with respect to safety in air transportation or in air

commerce that makes the procedures in paragraph (e)(2) of this section

impractical or contrary to the public interest, the Administrator may issue

an amendment to the foreign air carrier security program, effective without

stay on the date the foreign air carrier receives notice of it. In such a

case, the Administrator incorporates in the notice of amendment the finding

and a brief statement of the reasons for the amendment.

(4) A foreign air carrier may submit a request to the Administrator to

amend its security program. The requested amendment must be filed with the

Administrator at least 45 days before the date the foreign carrier proposes

that the amendment would become effective, unless a shorter period is allowed

by the Administrator. Within 30 days after receiving the requested amendment,

the Administrator will notify the foreign air carrier whether the amendment

is acceptable. The foreign air carrier may petition the Administrator to

reconsider a notice of unacceptability of the requested amendment within 45

days after receiving notice of unacceptability.

(5) Each foreign air carrier required to use a security program by

paragraph (b) of this section shall, upon request of the Administrator and in

accordance with the applicable law, provide information regarding the

implementation and operation of its security program.

(f) No foreign air carrier may land or take off an airplane in the United

States, in passenger operations, after receiving a bomb or air piracy threat

against that airplane, unless the following actions are taken:

(1) If the airplane is on the ground when a bomb threat is received and the

next scheduled flight of the threatened airplane is to or from a place in the

United States, the foreign air carrier ensures that the pilot in command is

advised to submit the airplane immediately for a security inspection and an

inspection of the airplane is conducted before the next flight.

(2) If the airplane is in flight to a place in the United States when a

bomb threat is received, the foreign air carrier ensures that the pilot in

command is advised immediately to take the emergency action necessary under

the circumstances and a security inspection of the airplane is conducted

immediately after the next landing.

(3) If information is received of a bomb or air piracy threat against an

airplane engaged in an operation specified in paragraph (f)(1) or (f)(2) of

this section, the foreign air carrier ensures that notification of the threat

is given to the appropriate authorities of the State in whose territory the

airplane is located or, if in flight, the appropriate authorities of the

State in whose territory the airplane is to land.

(g) Each foreign air carrier conducting an operation for which a security

program is required by paragraph (b) (1), (2), or (3) of this section shall

refuse to transport--

(1) Any person who does not consent to a search of his or her person in

accordance with the security program; and

(2) Any property of any person who does not consent to a search or

inspection of that property in accordance with the security program.

(h) At airports within the United States not governed by Part 107 of this

chapter, each foreign air carrier engaging in public charter passenger

operations shall--

(1) When using a screening system required by paragraph (b) of this

section, provide for law enforcement officers meeting the qualifications and

standards, and in the number and manner, specified in Part 107; and

(2) When using an airplane having a passenger seating configuration of more

than 30 but less than 61 seats for which a screening system is not required

by paragraph (b) of this section, arrange for law enforcement officers

meeting the qualifications and standards specified in Part 107 to be

available to respond to an incident and provide to appropriate employees,

including crewmembers, current information with respect to procedures for

obtaining law enforcement assistance at that airport.

(i) At airports governed by Part 107 of this chapter, each foreign air

carrier engaging in scheduled passenger operations or public charter

passenger operations when using an airplane with a passenger seating

configuration of more than 30 but less than 61 seats for which a screening

system is not required by paragraph (b) of this section shall arrange for law

enforcement officers meeting the qualifications and standards specified in

Part 107 to be available to respond to an incident and provide to appropriate

employees, including crewmembers, current information with respect to

procedures for obtaining law enforcement assistance at that airport.

(j) Unless otherwise authorized by the Administrator, each foreign air

carrier required to conduct screening under this part shall use procedures,

facilities, and equipment for detecting explosives, incendiaries, and deadly

or dangerous weapons to inspect each person entering a sterile area at each

preboarding screening checkpoint in the United States for which it is

responsible, and to inspect all accessible property under that person's

control.

[Amdt. 129-11, 46 FR 3790, Jan. 15, 1981; 46 FR 7936, Jan. 26, 1981, as

amended by Amdt. 129-16, 52 FR 48509, Dec. 22, 1987; Amdt. 129-18, 54 FR

11121, Mar. 16, 1989; Amdt. 129-22, 56 FR 30126, July 1, 1991]

*****************************************************************************

56 FR 30122, No. 126, July 1, 1991

SUMMARY: The FAA is amending the Federal Aviation Regulations to require

foreign air carriers that land or take off in the United States to provide

passengers a level of protection similar to the level of protection provided

by U.S. air carriers at the same airport. To ensure that foreign air carrier

security programs contain procedures which provide a similar level of

protection, the Administrator could amend those programs according to the

procedures in this rule. This action is needed to ensure that appropriate

security measures are implemented by foreign air carriers operating into and

out of the United States. This action also implements Congressional

legislation enacted on November 16, 1990. The intended effect of this rule is

to increase the safety and security of passengers aboard foreign air carriers

on flights to and from the United States by reducing the risk of fatalities

and property damage attributable to criminal acts against civil aviation.

EFFECTIVE DATE: July 31, 1991.

*****************************************************************************

Sec. 129.26   Use of X-ray system.

(a) No foreign air carrier may use an X-ray system in the United States to

inspect carry-on and checked articles unless:

(1) For a system manufactured prior to April 25, 1974, it meets either the

guidelines issued by the Food and Drug Administration (FDA), Department of

Health, Education, and Welfare and published in the Federal Register (38 FR

21442, August 8, 1973); or the performance standards for cabinet X-ray

systems designed primarily for the inspection of carry-on baggage issued by

the FDA and published in 21 CFR 1020.40 (39 FR 12985, April 10, 1974);

(2) For a system manufactured after April 24, 1974, it meets the standards

for cabinet X-ray systems designed primarily for the inspection of carry-on

baggage issued by the FDA and published in 21 CFR 1020.40 (39 FR 12985, April

10, 1974);

(3) A program for initial and recurrent training of operators of the system

has been established, which includes training in radiation safety, the

efficient use of X-ray systems, and the identification of weapons and other

dangerous articles;

(4) Procedures have been established to ensure that such operator of the

system will be provided with an individual personnel dosimeter (such as a

film badge or thermoluminescent dosimeter). Each dosimeter used will be

evaluated at the end of each calendar month, and records of operator duty

time and the results of dosimeter evaluations will be maintained by the

foreign air carrier; and

(5) The system meets the imaging requirements set forth in an accepted

Foreign Air Carrier Security Program using the step wedge specified in

American Society for Testing and Materials Standard F792-82.

(b) No foreign air carrier may use an X-ray system as specified in

paragraph (a) of this section--

(1) Unless within the preceding 12 calendar months a radiation survey has

been conducted which shows that the system meets the applicable performance

standards in 21 CFR 1020.40 or guidelines published by the Food and Drug

Administration in the Federal Register of August 8, 1973 (38 FR 21442);

(2) After the system is initially installed or after it has been moved from

one location to another, unless a radiation survey is conducted which shows

that the system meets the applicable performance standards in 21 CFR 1020.40

or guidelines published by the Food and Drug Administration in the Federal

Register on August 8, 1973 (38 FR 21442); except that a radiation survey is

not required for an X-ray system that is moved to another location, if the

foreign air carrier shows that the system is so designed that it can be moved

without altering its performance:

(3) That is not in full compliance with any defect notice or modification

order issued for that system by the Food and Drug Administration, Department

of Health, Education, and Welfare, unless that Administration has advised the

FAA that the defect or failure to comply is not such as to create a

significant risk or injury, including genetic injury, to any person; and

(4) Unless a sign is posted in a conspicuous place at the screening station

and on the X-ray system which notifies passengers that carry-on and checked

articles are being inspected by an X-ray system and advises them to remove

all X-ray, scientific, and high-speed film from their carry-on and checked

articles before inspection. This sign shall also advise passengers that they

may request an inspection to be made of their photographic equipment and film

packages without exposure to an X-ray system. If the X-ray system exposes any

carry-on or checked articles to more than 1 milliroentgen during the

inspection, the foreign air carrier shall post a sign which advises

passengers to remove film of all kinds from their articles before inspection.

If requested by passengers, their photographic equipment and film packages

shall be inspected without exposure to an X-ray system.

(c) Each foreign air carrier shall maintain at least one copy of the

results of the most recent radiation survey conducted under paragraph (b)(1)

or (b)(2) of this section at the place where the X-ray system is in operation

and shall make it available for inspection upon request by the Administrator.

(d) The American Society for Testing and Materials Standard F792-82,

"Design and Use of Ionizing Radiation Equipment for the Detection of Items

Prohibited in Controlled Access Areas," described in this section is

incorporated by reference herein and made a part hereof pursuant to 5 U.S.C.

552(a)(1). All persons affected by these amendments may obtain copies of the

standard from the American Society for Testing and Materials, 1916 Race

Street, Philadelphia, PA 19103. In addition, a copy of the standard may be

examined at the FAA Rules Docket, Docket No. 24115, 800 Independence Avenue

SW., Washington, DC, weekdays, except Federal holidays, between 8:30 a.m. and

5 p.m.

[Doc. No. 15286, Amdt. 129-6, 41 FR 30106, July 22, 1976, as amended by Amdt.

129-8, 43 FR 11978, Mar. 23, 1978; Amdt. 129-10, 44 FR 54467, Sept. 20, 1979;

Amdt. 129-13, 50 FR 25657, June 20, 1985; Doc. No. 26268, Amdt. No. 129-23,

56 FR 48374, Sept. 24, 1991]

*****************************************************************************

56 FR 48370, No. 185, Sept. 24, 1991

SUMMARY: The FAA is amending the airplane operator security regulations by

removing the exception to meeting the current X-ray imaging standard for X-

ray screening systems in use prior to July 22, 1985. Each United States air

carrier conducting screening under a mandatory security program will be

required to use only X-ray systems that meet the current X-ray imaging

standard required under its approved security program to screen carry-on and

checked articles. Likewise, each foreign air carrier that lands or takes off

in the United States will be required to use only X-ray screening systems

that meet the current X-ray imaging standard under its accepted security

program to screen carry-on and checked articles in the United States. This

action is needed due to the increased sophistication of terrorist acts. The

intended effect is to increase the safety of passengers and crewmembers

aboard aircraft by providing an upgraded aid at airport screening points to

prevent the carriage of explosives, incendiaries, or deadly or dangerous

weapons.

EFFECTIVE DATE: October 24, 1991.

*****************************************************************************

Sec. 129.27   Prohibition against carriage of weapons.

(a) No person may, while on board an aircraft being operated by a foreign

air carrier in the United States, carry on or about his person a deadly or

dangerous weapon, either concealed or unconcealed. This paragraph does not

apply to--

(1) Officials or employees of the state of registry of the aircraft who are

authorized by that state to carry arms; and

(2) Crewmembers and other persons authorized by the foreign air carrier to

carry arms.

(b) No foreign air carrier may knowingly permit any passenger to carry, nor

may any passenger carry, while aboard an aircraft being operated in the

United States by that carrier, in checked baggage, a deadly or dangerous

weapon, unless:

(1) The passenger has notified the foreign air carrier before checking the

baggage that the weapon is in the baggage; and

(2) The baggage is carried in an area inaccessible to passengers.

[Doc. No. 15286, Amdt. 129-6, 41 FR 30107, July 22, 1976]

Sec. 129.29  Prohibition against smoking.

No person may smoked and no operator shall permit smoking in the passenger

cabin or lavatory during any scheduled airline flight segment in air

transportation or intrastate air transportation which is:

(a) Between any two points within Puerto Rico, the United States Virgin

Islands, the District of Columbia, or any State of the United States (other

than Alaska or Hawaii) or between any two points in any one of the above-

mentioned jurisdictions (other than Alaska or Hawaii);

(b) Within the State of Alaska or within the State of Hawaii; or

(c) Scheduled in the current Worldwide or North American Edition of the

Official Airline Guide for 6 hours or less in duration and between any point

listed in paragraph (a) of this section and any point in Alaska or Hawaii, or

between any point in Alaska and any point in Hawaii.

[Doc. No. 25590, Amdt. 129-20, 55 FR 8367, Mar. 7, 1990]

*****************************************************************************

55 FR 8364, No. 45, Mar. 7, 1990

SUMMARY: This rule implements the prohibition against smoking contained in

Public Law 101-164 enacted on November 21, 1989. This legislation makes it

unlawful for any person to smoke in the passenger cabin or lavatory of an

airplane during most scheduled flight segments in the United States. The

statutory prohibition applies to U.S. and foreign air carriers.

In addition to implementing the smoking prohibition, this rule amends the

Federal Aviation Regulations to conform with an amendment to the Federal

Aviation Act of 1958 that prohibits persons from tampering with smoke

detectors installed in airplane lavatories.

EFFECTIVE DATE: February 25, 1990.

*****************************************************************************

Appendix A to Part 129--Application for Operations Specifications by Foreign

Air Carriers

(a) General. Each application must be executed by an authorized officer or

employee of the applicant having knowledge of the matter set forth therein,

and must have attached thereto two copies of the appropriate written

authority issued to that officer or employee by the applicant. Negotiations

for permission to use airports under U.S. military jurisdiction is effected

through the respective embassy of the foreign government and the United

States Department of State.

(b) Format of application. The following outline must be followed in

completing the information to be submitted in the application.

Application for Foreign Air Carrier Operations Specifications

(OUTLINE)

In accordance with the Federal Aviation Act of 1958 (49 U.S.C. 1372) and

Part 129 of the Federal Air Regulations, application is hereby made for the

issuance of Foreign Operations Specifications.

Give exact name and full post office address of applicant.

Give the name, title, and post office address (within the United States if

possible) of the official or employee to whom correspondence in regard to the

application is to be addressed.

Unless otherwise specified, the applicant must submit the following

information only with respect to those parts of his proposed operations that

will be conducted within the United States.

Section I. Operations. State whether the operation proposed is day or

night, visual flight rules, instrument flight rules, or a particular

combination thereof.

Sec. II. Operational plans. State the route by which entry will be made

into the United States, and the route to be flown therein.

Sec. III. A. Route. Submit a map suitable for aerial navigation upon which

is indicated the exact geographical track of the proposed route from the last

point of foreign departure to the United States terminal, showing the regular

terminal, and alternate airports, and radio navigational facilities. This

material will be indicated in a manner that will facilitate identification.

The applicant may use any method that will clearly distinguish the

information, such as different colors, different types of lines, etc. For

example, if different colors are used, the identification will be

accomplished as follows:

1. Regular route: Black.

2. Regular terminal airport: Green circle.

3. Alternate airports: Orange circle.

4. The location of radio navigational facilities which will be used in

connection with the proposed operation, indicating the type of facility to be

used, such as radio range ADF, VOR, etc.

B. Airports. Submit the following information with regard to each regular

terminal and alternate to be used in the conduct of the proposed operation:

1. Name of airport or landing area.

2. Location (direction distance to and name of nearest city or town).

Sec. IV. Radio facilities: Communications. List all ground radio

communication facilities to be used by the applicant in the conduct of the

proposed operations within the United States and over that portion of the

route between the last point of foreign departure and the United States.

Sec. V. Aircraft. Submit the following information in regard to each type

and model aircraft to be used.

A. Aircraft.

1. Manufacturer and model number.

2. State of origin.

3. Single-engine or multiengine. If multiengine, indicate number of

engines.

4. What is the maximum takeoff and landing weight to be used for each type

of aircraft?

5. Registration markings of each U.S.-registered aircraft.

B. Aircraft Radio. List aircraft radio equipment necessary for instrument

operation within the United States.

C. Licensing. State name of country by whom aircraft are certificated.

Sec. VI. Airmen. List the following information with respect to airmen to

be employed in the proposed operation within the United States.

A. State the type and class of certificate held by each flight crewmember.

B. State whether or not pilot personnel have received training in the use

of navigational facilities necessary for en route operation and instrument

letdowns along or adjacent to the route to be flown within the United States.

C. State whether or not personnel are familiar with those parts of the

Federal Air Regulations pertaining to the conduct of foreign air carrier

operations within the United States.

D. State whether pilot personnel are able to speak and understand the

English language to a degree necessary to enable them to properly communicate

with Airport Traffic Control Towers and Airway Radio Communication Stations

using radiotelephone communications.

Sec. VII. Dispatchers.

A. Describe briefly the dispatch organization which you propose to set up

for air carrier operations within the United States.

B. State whether or not the dispatching personnel are familiar with the

rules and regulations prescribed by the Federal Air Regulations governing air

carrier operations.

C. Are dispatching personnel able to read and write the English language to

a degree necessary to properly dispatch flights within the United States?

D. Are dispatching personnel certificated by the country of origin?

Sec. VIII. Additional Data.

A. Furnish such additional information and substantiating data as may serve

to expedite the issuance of the operations specifications.

B. Each application shall be concluded with a statement as follows:

I certify that the above statements are true.

Signed this ------ day of ------ 19----

------------------------ (Name of Applicant)---------------------------------

By---------------------------------------------------------------------------

(Name of person duly authorized to execute this application on behalf of the

applicant.)

[Doc. No. 1994, 29 FR 1720, Feb. 5, 1964, as amended by Amdt. 129-14, 52 FR

20029, May 28, 1987; Amdt. 129-19, 54 FR 39294, Sept. 25, 1989; 54 FR 51972,

Dec. 19, 1989]