Title 14--Aeronautics and Space

CHAPTER I--FEDERAL AVIATION ADMINISTRATION, DEPARTMENT OF TRANSPORTATION

SUBCHAPTER C--AIRCRAFT

PART 23--AIRWORTHINESS STANDARDS: NORMAL, UTILITY, ACROBATIC, AND

COMMUTER CATEGORY AIRPLANES

Special Federal Aviation Regulations

SFAR No. 23

SFAR No. 41

Subpart A--General

Sec. 23.1 Applicability.

Sec. 23.2 Special retroactive requirements.

Sec. 23.3 Airplane categories.

Subpart B--Flight

General

Sec. 23.21 Proof of compliance.

Sec. 23.23 Load distribution limits.

Sec. 23.25 Weight limits.

Sec. 23.29 Empty weight and corresponding center of gravity.

Sec. 23.31 Removable ballast.

Sec. 23.33 Propeller speed and pitch limits.

Performance

Sec. 23.45 General.

Sec. 23.49 Stalling speed.

Sec. 23.51 Takeoff.

Sec. 23.53 Takeoff speeds.

Sec. 23.55 Accelerate-stop distance.

Sec. 23.57 Takeoff path.

Sec. 23.59 Takeoff distance and takeoff run.

Sec. 23.61 Takeoff flight path.

Sec. 23.65 Climb: All engines operating.

Sec. 23.67 Climb: one engine inoperative.

Sec. 23.75 Landing.

Sec. 23.77 Balked landing.

Flight Characteristics

Sec. 23.141 General.

Controllability and Maneuverability

Sec. 23.143 General.

Sec. 23.145 Longitudinal control.

Sec. 23.147 Directional and lateral control.

Sec. 23.149 Minimum control speed.

Sec. 23.151 Acrobatic maneuvers.

Sec. 23.153 Control during landings.

Sec. 23.155 Elevator control force in maneuvers.

Sec. 23.157 Rate of roll.

Trim

Sec. 23.161 Trim.

Stability

Sec. 23.171 General.

Sec. 23.173 Static longitudinal stability.

Sec. 23.175 Demonstration of static longitudinal stability.

Sec. 23.177 Static directional and lateral stability.

Sec. 23.179 [Removed. Amdt. No. 23-45, 58 FR 42158, Aug. 6,

1993]

Sec. 23.181 Dynamic stability.

Stalls

Sec. 23.201 Wings level stall.

Sec. 23.203 Turning flight and accelerated stalls.

Sec. 23.205 Critical engine inoperative stalls.

Sec. 23.207 Stall warning.

Spinning

Sec. 23.221 Spinning.

Ground and Water Handling Characteristics

Sec. 23.231 Longitudinal stability and control.

Sec. 23.233 Directional stability and control.

Sec. 23.235 Taxiing, takeoff, and landing condition.

Sec. 23.239 Spray characteristics.

Miscellaneous Flight Requirements

Sec. 23.251 Vibration and buffeting.

Sec. 23.253 High speed characteristics.

Subpart C--Structure

General

Sec. 23.301 Loads.

Sec. 23.302 Canard or tandem wing configurations.

Sec. 23.303 Factor of safety.

Sec. 23.305 Strength and deformation.

Sec. 23.307 Proof of structure.

Flight Loads

Sec. 23.321 General.

Sec. 23.331 Symmetrical flight conditions.

Sec. 23.333 Flight envelope.

Sec. 23.335 Design airspeeds.

Sec. 23.337 Limit maneuvering load factors.

Sec. 23.341 Gust loads factors.

Sec. 23.345 High lift devices.

Sec. 23.347 Unsymmetrical flight conditions.

Sec. 23.349 Rolling conditions.

Sec. 23.351 Yawing conditions.

Sec. 23.361 Engine torque.

Sec. 23.363 Side load on engine mount.

Sec. 23.365 Pressurized cabin loads.

Sec. 23.367 Unsymmetrical loads due to engine failure.

Sec. 23.369 Rear lift truss.

Sec. 23.371 Gyroscopic and areodynamic loads.

Sec. 23.373 Speed control devices.

Control Surface and System Loads

Sec. 23.391 Control surface loads.

Sec. 23.395 Control system loads.

Sec. 23.397 Limit control forces and torques.

Sec. 23.399 Dual control system.

Sec. 23.405 Secondary control system.

Sec. 23.407 Trim tab effects.

Sec. 23.409 Tabs.

Sec. 23.415 Ground gust conditions.

Horizontal Stabilizing and Balancing Surfaces

Sec. 23.421 Balancing loads.

Sec. 23.423 Maneuvering loads.

Sec. 23.425 Gust loads.

Sec. 23.427 Unsymmetrical loads.

Vertical Surfaces

Sec. 23.441 Maneuvering loads.

Sec. 23.443 Gust loads.

Sec. 23.445 Outboard fins or winglets.

Ailerons, Wing Flaps, and Special Devices

Sec. 23.455 Ailerons.

Sec. 23.457 Wing flaps.

Sec. 23.459 Special devices.

Ground Loads

Sec. 23.471 General.

Sec. 23.473 Ground load conditions and assumptions.

Sec. 23.477 Landing gear arrangement.

Sec. 23.479 Level landing conditions.

Sec. 23.481 Tail down landing conditions.

Sec. 23.483 One-wheel landing conditions.

Sec. 23.485 Side load conditions.

Sec. 23.493 Braked roll conditions.

Sec. 23.497 Supplementary conditions for tail wheels.

Sec. 23.499 Supplementary conditions for nose wheels.

Sec. 23.505 Supplementary conditions for skiplanes.

Sec. 23.507 Jacking loads.

Sec. 23.509 Towing loads.

Sec. 23.511 Ground load; unsymmetrical loads on multiple-wheel

units.

Water Loads

Sec. 23.521 Water load conditions.

Sec. 23.523 Design weights and center of gravity positions.

Sec. 23.525 Application of loads.

Sec. 23.527 Hull and main float load factors.

Sec. 23.529 Hull and main float landing conditions.

Sec. 23.531 Hull and main float takeoff condition.

Sec. 23.533 Hull and main float bottom pressures.

Sec. 23.535 Auxiliary float loads.

Sec. 23.537 Seawing loads.

Emergency Landing Conditions

Sec. 23.561 General.

Sec. 23.562 Emergency landing dynamic conditions.

Fatigue Evaluation

Sec. 23.571 Pressurized cabin.

Sec. 23.572 Wing, empennage, and associated structures.

Sec. 23.573 Damage tolerance and fatigue evaluation of

structure.

Subpart D--Design and Construction

Sec. 23.601 General.

Sec. 23.603 Materials and workmanship.

Sec. 23.605 Fabrication methods.

Sec. 23.607 Self-locking nuts.

Sec. 23.609 Protection of structure.

Sec. 23.611 Accessibility.

Sec. 23.613 Material strength properties and design values.

Sec. 23.615 [Removed. Amdt. No. 23-45, 58 FR 42164, Aug. 6,

1993]

Sec. 23.619 Special factors.

Sec. 23.621 Casting factors.

Sec. 23.623 Bearing factors.

Sec. 23.625 Fitting factors.

Sec. 23.627 Fatigue strength.

Sec. 23.629 Flutter.

Wings

Sec. 23.641 Proof of strength.

Control Surfaces

Sec. 23.651 Proof of strength.

Sec. 23.655 Installation.

Sec. 23.657 Hinges.

Sec. 23.659 Mass balance.

Control Systems

Sec. 23.671 General.

Sec. 23.672 Stability augmentation and automatic and power-

operated systems.

Sec. 23.673 Primary flight controls.

Sec. 23.675 Stops.

Sec. 23.677 Trim systems.

Sec. 23.679 Control system locks.

Sec. 23.681 Limit load static tests.

Sec. 23.683 Operation tests.

Sec. 23.685 Control system details.

Sec. 23.687 Spring devices.

Sec. 23.689 Cable systems.

Sec. 23.693 Joints.

Sec. 23.697 Wing flap controls.

Sec. 23.699 Wing flap position indicator.

Sec. 23.701 Flap interconnection.

Landing Gear

Sec. 23.721 General.

Sec. 23.723 Shock absorption tests.

Sec. 23.725 Limit drop tests.

Sec. 23.726 Ground load dynamic tests.

Sec. 23.727 Reserve energy absorption drop test.

Sec. 23.729 Landing gear extension and retraction system.

Sec. 23.731 Wheels.

Sec. 23.733 Tires.

Sec. 23.735 Brakes.

Sec. 23.737 Skis.

Floats and Hulls

Sec. 23.751 Main float buoyancy.

Sec. 23.753 Main float design.

Sec. 23.755 Hulls.

Sec. 23.757 Auxiliary floats.

Personnel and Cargo Accommodations

Sec. 23.771 Pilot compartment.

Sec. 23.773 Pilot compartment view.

Sec. 23.775 Windshields and windows.

Sec. 23.777 Cockpit controls.

Sec. 23.779 Motion and effect of cockpit controls.

Sec. 23.781 Cockpit control knob shape.

Sec. 23.783 Doors.

Sec. 23.785 Seats, berths, litters, safety belts, and shoulder

harnesses.

Sec. 23.787 Baggage and cargo compartments.

Sec. 23.803 Emergency evacuation.

Sec. 23.807 Emergency exits.

Sec. 23.811 Emergency exit marking.

Sec. 23.813 Emergency exit access.

Sec. 23.815 Width of aisle.

Sec. 23.831 Ventilation.

Pressurization

Sec. 23.841 Pressurized cabins.

Sec. 23.843 Pressurization tests.

Fire Protection

Sec. 23.851 Fire extinguishers.

Sec. 23.853 Compartment interiors.

Sec. 23.859 Combustion heater fire protection.

Sec. 23.863 Flammable fluid fire protection.

Sec. 23.865 Fire protection of flight controls, engine mounts,

and other flight structure.

Lightning Evaluation

Sec. 23.867 Lightning protection of structure.

Miscellaneous

Sec. 23.871 Leveling means.

Subpart E--Powerplant

General

Sec. 23.901 Installation.

Sec. 23.903 Engines.

Sec. 23.904 Automatic power reserve system.

Sec. 23.905 Propellers.

Sec. 23.907 Propeller vibration.

Sec. 23.909 Turbocharger systems.

Sec. 23.925 Propeller clearance.

Sec. 23.929 Engine installation ice protection.

Sec. 23.933 Reversing systems.

Sec. 23.934 Turbojet and turbofan engine thrust reverser

systems tests.

Sec. 23.937 Turbopropeller-drag limiting systems.

Sec. 23.939 Powerplant operating characteristics.

Sec. 23.943 Negative acceleration.

Fuel System

Sec. 23.951 General.

Sec. 23.953 Fuel system independence.

Sec. 23.954 Fuel system lightning protection.

Sec. 23.955 Fuel flow.

Sec. 23.957 Flow between interconnected tanks.

Sec. 23.959 Unusable fuel supply.

Sec. 23.961 Fuel system hot weather operation.

Sec. 23.963 Fuel tanks: general.

Sec. 23.965 Fuel tank tests.

Sec. 23.967 Fuel tank installation.

Sec. 23.969 Fuel tank expansion space

Sec. 23.971 Fuel tank sump.

Sec. 23.973 Fuel tank filler connection.

Sec. 23.975 Fuel tank vents and carburetor vapor vents.

Sec. 23.977 Fuel tank outlet.

Sec. 23.979 Pressure fueling systems.

Fuel System Components

Sec. 23.991 Fuel pumps.

Sec. 23.993 Fuel system lines and fittings.

Sec. 23.994 Fuel system components.

Sec. 23.995 Fuel valves and controls.

Sec. 23.997 Fuel strainer or filter.

Sec. 23.999 Fuel system drains.

Sec. 23.1001 Fuel jettisoning system.

Oil System

Sec. 23.1011 General.

Sec. 23.1013 Oil tanks.

Sec. 23.1015 Oil tank tests.

Sec. 23.1017 Oil lines and fittings.

Sec. 23.1019 Oil strainer or filter.

Sec. 23.1021 Oil system drains.

Sec. 23.1023 Oil radiators.

Sec. 23.1027 Propeller feathering system.

Cooling

Sec. 23.1041 General.

Sec. 23.1043 Cooling tests.

Sec. 23.1045 Cooling test procedures for turbine engine powered

airplanes.

Sec. 23.1047 Cooling test procedures for reciprocating engine-

powered airplanes.

Liquid Cooling

Sec. 23.1061 Installation.

Sec. 23.1063 Coolant tank tests.

Induction System

Sec. 23.1091 Air induction system.

Sec. 23.1093 Induction system icing protection.

Sec. 23.1095 Carburetor deicing fluid flow rate.

Sec. 23.1097 Carburetor deicing fluid system capacity.

Sec. 23.1099 Carburetor deicing fluid system detail design.

Sec. 23.1101 Induction air preheater design.

Sec. 23.1103 Induction system ducts.

Sec. 23.1105 Induction system screens.

Sec. 23.1107 Induction system filters.

Sec. 23.1109 Turbocharger bleed air system.

Sec. 23.1111 Turbine engine bleed air system.

Exhaust System

Sec. 23.1121 General.

Sec. 23.1123 Exhaust system.

Sec. 23.1125 Exhaust heat exchangers.

Powerplant Controls and Accessories

Sec. 23.1141 Powerplant controls: general.

Sec. 23.1142 Auxiliary power unit controls.

Sec. 23.1143 Engine controls.

Sec. 23.1145 Ignition switches.

Sec. 23.1147 Mixture controls.

Sec. 23.1149 Propeller speed and pitch controls.

Sec. 23.1153 Propeller feathering controls.

Sec. 23.1155 Turbine engine reverse thrust and propeller pitch

settings below the flight regime.

Sec. 23.1157 Carburetor air temperature controls.

Sec. 23.1163 Powerplant accessories.

Sec. 23.1165 Engine ignition systems.

Sec. 23.1181 Designated fire zones; regions included.

Powerplant Fire Protection

Sec. 23.1182 Nacelle areas behind firewalls.

Sec. 23.1183 Lines, fittings, and components.

Sec. 23.1189 Shutoff means.

Sec. 23.1191 Firewalls.

Sec. 23.1192 Engine accessory compartment diaphragm.

Sec. 23.1193 Cowling and nacelle.

Sec. 23.1195 Fire extinguishing systems.

Sec. 23.1197 Fire extinguishing agents.

Sec. 23.1199 Extinguishing agent containers.

Sec. 23.1201 Fire extinguishing system materials.

Sec. 23.1203 Fire detector system.

Subpart F--Equipment

General

Sec. 23.1301 Function and installation.

Sec. 23.1303 Flight and navigation instruments.

Sec. 23.1305 Powerplant instruments.

Sec. 23.1307 Miscellaneous equipment.

Sec. 23.1309 Equipment, systems, and installations.

Instruments: Installation

Sec. 23.1311 Electronic display instrument systems.

Sec. 23.1321 Arrangement and visibility.

Sec. 23.1322 Warning, caution, and advisory lights.

Sec. 23.1323 Airspeed indicating system.

Sec. 23.1325 Static pressure system.

Sec. 23.1327 Magnetic direction indicator.

Sec. 23.1329 Automatic pilot system.

Sec. 23.1331 Instruments using a power source.

Sec. 23.1335 Flight director systems.

Sec. 23.1337 Powerplant instruments.

Electrical Systems and Equipment

Sec. 23.1351 General.

Sec. 23.1353 Storage battery design and installation.

Sec. 23.1357 Circuit protective devices.

Sec. 23.1361 Master switch arrangement.

Sec. 23.1365 Electric cables and equipment.

Sec. 23.1367 Switches.

Lights

Sec. 23.1381 Instrument lights.

Sec. 23.1383 Landing lights.

Sec. 23.1385 Position light system installation.

Sec. 23.1387 Position light system dihedral angles.

Sec. 23.1389 Position light distribution and intensities.

Sec. 23.1391 Minimum intensities in the horizontal plane of

position lights.

Sec. 23.1393 Minimum intensities in any vertical plane of

position lights.

Sec. 23.1395 Maximum intensities in overlapping beams of

position lights.

Sec. 23.1397 Color specifications.

Sec. 23.1399 Riding light.

Sec. 23.1401 Anticollision light system.

Safety Equipment

Sec. 23.1411 General.

Sec. 23.1413 Safety belts and harnesses.

Sec. 23.1415 Ditching equipment.

Sec. 23.1416 Pneumatic de-icer boot system.

Sec. 23.1419 Ice protection.

Miscellaneous Equipment

Sec. 23.1431 Electronic equipment.

Sec. 23.1435 Hydraulic systems.

Sec. 23.1437 Accessories for multiengine airplanes.

Sec. 23.1438 Pressurization and pneumatic systems.

Sec. 23.1441 Oxygen equipment and supply.

Sec. 23.1443 Minimum mass flow of supplemental oxygen.

Sec. 23.1445 Oxygen distribution system.

Sec. 23.1447 Equipment standards for oxygen dispensing units.

Sec. 23.1449 Means for determining use of oxygen.

Sec. 23.1450 Chemical oxygen generators.

Sec. 23.1457 Cockpit voice recorders.

Sec. 23.1459 Flight recorders.

Sec. 23.1461 Equipment containing high energy rotors.

Subpart G--Operating Limitations and Information

Sec. 23.1501 General.

Sec. 23.1505 Airspeed limitations.

Sec. 23.1507 Operating maneuvering speed.

Sec. 23.1511 Flap extended speed.

Sec. 23.1513 Minimum control speed.

Sec. 23.1519 Weight and center of gravity.

Sec. 23.1521 Powerplant limitations.

Sec. 23.1522 Auxiliary power unit limitations.

Sec. 23.1523 Minimum flight crew.

Sec. 23.1524 Maximum passenger seating configuration.

Sec. 23.1525 Kinds of operation.

Sec. 23.1527 Maximum operating altitude.

Sec. 23.1529 Instructions for Continued Airworthiness.

Markings And Placards

Sec. 23.1541 General.

Sec. 23.1543 Instrument markings: general.

Sec. 23.1545 Airspeed indicator.

Sec. 23.1547 Magnetic direction indicator.

Sec. 23.1549 Powerplant and auxiliary power unit instruments.

Sec. 23.1551 Oil quantity indicator.

Sec. 23.1553 Fuel quantity indicator.

Sec. 23.1555 Control markings.

Sec. 23.1557 Miscellaneous markings and placards.

Sec. 23.1559 Operating limitations placard.

Sec. 23.1561 Safety equipment.

Sec. 23.1563 Airspeed placards.

Sec. 23.1567 Flight maneuver placard.

Airplane Flight Manual and Approved Manual Material

Sec. 23.1581 General.

Sec. 23.1583 Operating limitations.

Sec. 23.1585 Operating procedures.

Sec. 23.1587 Performance information.

Sec. 23.1589 Loading information.

Appendix A to Part 23--Simplified Design Load Criteria for

Conventional, Single-Engine Airplanes of 6,000 Pounds or Less

Maximum Weight

Appendix B to Part 23--[Reserved]

Appendix C to Part 23--Basic Landing Conditions

Appendix D to Part 23--Wheel Spin-Up and Spring-Back Loads

Appendix E to Part 23--Limited Weight Credit for Airplanes Equipped

With Standby Power

Appendix F to Part 23--Test Procedure

Appendix G to Part 23--Instructions for Continued Airworthiness

Appendix H to Part 23--Installation of An Automatic Power Reserve

(APR) System

Appendix I to Part 23--Seaplane Loads

SFAR No. 23

1. Applicability. An applicant is entitled to a type certificate in the

normal category for a reciprocating or turbopropeller multiengine powered

small airplane that is to be certificated to carry more than 10 occupants and

that is intended for use in operations under Part 135 of the Federal Aviation

Regulations if he shows compliance with the applicable requirements of Part

23 of the Federal Aviation Regulations, as supplemented or modified by the

additional airworthiness requirements of this regulation.

2. References. Unless otherwise provided, all references in this regulation

to specific sections of Part 23 of the Federal Aviation Regulations are those

sections of Part 23 in effect on March 30, 1967.

Flight Requirements

3. General. Compliance must be shown with the applicable requirements of

Subpart B of Part 23 of the Federal Aviation Regulations in effect on March

30, 1967, as supplemented or modified in sections 4 through 10 of this

regulation.

Performance

4. General. (a) Unless otherwise prescribed in this regulation, compliance

with each applicable performance requirement in sections 4 through 7 of this

regulation must be shown for ambient atmospheric conditions and still air.

(b) The performance must correspond to the propulsive thrust available

under the particular ambient atmospheric conditions and the particular flight

condition. The available propulsive thrust must correspond to engine power or

thrust, not exceeding the approved power or thrust less--

(1) Installation losses; and

(2) The power or equivalent thrust absorbed by the accessories and services

appropriate to the particular ambient atmospheric conditions and the

particular flight condition.

(c) Unless otherwise prescribed in this regulation, the applicant must

select the take-off, en route, and landing configurations for the airplane.

(d) The airplane configuration may vary with weight, altitude, and

temperature, to the extent they are compatible with the operating procedures

required by paragraph (e) of this section.

(e) Unless otherwise prescribed in this regulation, in determining the

critical engine inoperative takeoff performance, the accelerate-stop

distance, takeoff distance, changes in the airplane's configuration, speed,

power, and thrust, must be made in accordance with procedures established by

the applicant for operation in service.

(f) Procedures for the execution of balked landings must be established by

the applicant and included in the Airplane Flight Manual.

(g) The procedures established under paragraphs (e) and (f) of this section

must--

(1) Be able to be consistently executed in service by a crew of average

skill;

(2) Use methods or devices that are safe and reliable; and

(3) Include allowance for any time delays, in the execution of the

procedures, that may reasonably be expected in service.

5. Takeoff--(a) General. The takeoff speeds described in paragraph (b), the

accelerate-stop distance described in paragraph (c), and the takeoff distance

described in paragraph (d), must be determined for--

(1) Each weight, altitude, and ambient temperature within the operational

limits selected by the applicant;

(2) The selected configuration for takeoff;

(3) The center of gravity in the most unfavorable position;

(4) The operating engine within approved operating limitation; and

(5) Takeoff data based on smooth, dry, hard-surface runway.

(b) Takeoff speeds. (1) The decision speed V1is the calibrated airspeed on

the ground at which, as a result of engine failure or other reasons, the

pilot is assumed to have made a decision to continue or discontinue the

takeoff. The speed V1 must be selected by the applicant but may not be less

than--

(i) 1.10 Vs1;

(ii) 1.10 VMC;

(iii) A speed that permits acceleration to V1 and stop in accordance with

paragraph (c) allowing credit for an overrun distance equal to that required

to stop the airplane from a ground speed of 35 knots utilizing maximum

braking; or

(iv) A speed at which the airplane can be rotated for takeoff and shown to

be adequate to safely continue the takeoff, using normal piloting skill, when

the critical engine is suddenly made inoperative.

(2) Other essential takeoff speeds necessary for safe operation of the

airplane must be determined and shown in the Airplane Flight Manual.

(c) Accelerate-stop distance. (1) The accelerate-stop distance is the sum

of the distances necessary to--

(i) Accelerate the airplane from a standing start to V1; and

(ii) Decelerate the airplane from V1 to a speed not greater than 35 knots,

assuming that in the case of engine failure, failure of the critical engine

is recognized by the pilot at the speed V1. The landing gear must remain in

the extended position and maximum braking may be utilized during

deceleration.

(2) Means other than wheel brakes may be used to determine the accelerate-

stop distance if that means is available with the critical engine inoperative

and--

(i) Is safe and reliable;

(ii) Is used so that consistent results can be expected under normal

operating conditions; and

(iii) Is such that exceptional skill is not required to control the

airplane.

(d) All engines operating takeoff distance.  The all engine operating

takeoff distance is the horizontal distance required to takeoff and climb to

a height of 50 feet above the takeoff surface according to procedures in FAR

23.51(a).

(e) One-engine-inoperative takeoff. The maximum weight must be determined

for each altitude and temperature within the operational limits established

for the airplane, at which the airplane has takeoff capability after failure

of the critical engine at or above V1 determined in accordance with paragraph

(b) of this section. This capability may be established--

(1) By demonstrating a measurably positive rate of climb with the airplane

in the takeoff configuration, landing gear extended; or

(2) By demonstrating the capability of maintaining flight after engine

failure utilizing procedures prescribed by the applicant.

6. Climb--(a) Landing climb: All-engines-operating.  The maximum weight

must be determined with the airplane in the landing configuration, for each

altitude, and ambient temperature within the operational limits established

for the airplane and with the most unfavorable center of gravity and out-of-

ground effect in free air, at which the steady gradient of climb will not be

less than 3.3 percent, with:

(1) The engines at the power that is available 8 seconds after initiation

of movement of the power or thrust controls from the mimimum flight idle to

the takeoff position.

(2) A climb speed not greater than the approach speed established under

section 7 of this regulation and not less than the greater of 1.05MC or

1.10VS1.

(b) En route climb, one-engine-inoperative.  (1) the maximum weight must be

determined with the airplane in the en route configuration, the critical

engine inoperative, the remaining engine at not more than maximum continuous

power or thrust, and the most unfavorable center of gravity, at which the

gradient at climb will be not less than--

(i) 1.2 percent (or a gradient equivalent to 0.20 Vso 2 , if greater) at

5,000 feet and an ambient temperature of 41 deg. F. or

(ii) 0.6 percent (or a gradient equivalent to 0.01 Vso 2 , if greater) at

5,000 feet and ambient temperature of 81 deg. F.

(2) The minimum climb gradient specified in subdivisions (i) and (ii) of

subparagraph (1) of this paragraph must vary linearly between 41 deg. F. and

81 deg. F. and must change at the same rate up to the maximum operational

temperature approved for the airplane.

7. Landing. The landing distance must be determined for standard atmosphere

at each weight and altitude in accordance with FAR 23.75(a), except that

instead of the gliding approach specified in FAR 23.75(a)(1), the landing may

be preceded by a steady approach down to the 50-foot height at a gradient of

descent not greater than 5.2 percent (3 deg.) at a calibrated airspeed not

less than 1.3s1.

Trim

8. Trim--(a) Lateral and directional trim.  The airplane must maintain

lateral and directional trim in level flight at a speed of Vh or VMO/MMO,

whichever is lower, with landing gear and wing flaps retracted.

(b) Longitudinal trim. The airplane must maintain longitudinal trim during

the following conditions, except that it need not maintain trim at a speed

greater than VMO/MMO:

(1) In the approach conditions specified in FAR 23.161(c) (3) through (5),

except that instead of the speeds specified therein, trim must be maintained

with a stick force of not more than 10 pounds down to a speed used in showing

compliance with section 7 of this regulation or 1.4 Vs1 whichever is lower.

(2) In level flight at any speed from VH or VMO/MMO, whichever is lower, to

either Vx or 1.4 Vs1, with the landing gear and wing flaps retracted.

Stability

9. Static longitudinal stability. (a) In showing compliance with the

provisions of FAR 23.175(b) and with paragraph (b) of this section, the

airspeed must return to within +/-7 1/2  percent of the trim speed.

(b) Cruise stability. The stick force curve must have a stable slope for a

speed range of +/-50 knots from the trim speed except that the speeds need

not exceed VFC/MFC or be less than 1.4 Vs1. This speed range will be

considered to begin at the outer extremes of the friction band and the stick

force may not exceed 50 pounds with--

(i) Landing gear retracted;

(ii) Wing flaps retracted;

(iii) The maximum cruising power as selected by the applicant as an

operating limitation for turbine engines or 75 percent of maximum continuous

power for reciprocating engines except that the power need not exceed that

required at VMO/MMO:

(iv) Maximum takeoff weight; and

(v) The airplane trimmed for level flight with the power specified in

subparagraph (iii) of this paragraph.

VFC/MFC may not be less than a speed midway between VMO/MMO and VDF/MDF,

except that, for altitudes where Mach number is the limiting factor, MFC need

not exceed the Mach number at which effective speed warning occurs.

(c) Climb stability. For turbopropeller powered airplanes only. In showing

compliance with FAR 23.175(a), an applicant must in lieu of the power

specified in FAR 23.175(a)(4), use the maximum power or thrust selected by

the applicant as an operating limitation for use during climb at the best

rate of climb speed except that the speed need not be less than 1.4 Vs1.

Stalls

10. Stall warning. If artificial stall warning is required to comply with

the requirements of FAR 23.207, the warning device must give clearly

distinguishable indications under expected conditions of flight. The use of a

visual warning device that requires the attention of the crew within the

cockpit is not acceptable by itself.

Control Systems

11. Electric trim tabs. The airplane must meet the requirements of FAR

23.677 and in addition it must be shown that the airplane is safely

controllable and that a pilot can perform all the maneuvers and operations

necessary to effect a safe landing following any probable electric trim tab

runaway which might be reasonably expected in service allowing for

appropriate time delay after pilot recognition of the runaway. This

demonstration must be conducted at the critical airplane weights and center

of gravity positions.

Instruments: Installation

12. Arrangement and visibility. Each instrument must meet the requirements

of FAR 23.1321 and in addition--

(a) Each flight, navigation, and powerplant instrument for use by any pilot

must be plainly visible to him from his station with the minimum practicable

deviation from his normal position and line of vision when he is looking

forward along the flight path.

(b) The flight instruments required by FAR 23.1303 and by the applicable

operating rules must be grouped on the instrument panel and centered as

nearly as practicable about the vertical plane of each pilot's forward

vision. In addition--

(1) The instrument that most effectively indicates the attitude must be on

the panel in the top center position;

(2) The instrument that most effectively indicates airspeed must be

adjacent to and directly to the left of the instrument in the top center

position;

(3) The instrument that most effectively indicates altitude must be

adjacent to and directly to the right of the instrument in the top center

position; and

(4) The instrument that most effectively indicates direction of flight must

be adjacent to and directly below the instrument in the top center position.

13. Airspeed indicating system. Each airspeed indicating system must meet

the requirements of FAR 23.1323 and in addition--

(a) Airspeed indicating instruments must be of an approved type and must be

calibrated to indicate true airspeed at sea level in the standard atmosphere

with a mimimum practicable instrument calibration error when the

corresponding pilot and static pressures are supplied to the instruments.

(b) The airspeed indicating system must be calibrated to determine the

system error, i.e., the relation between IAS and CAS, in flight and during

the accelerate takeoff ground run. The ground run calibration must be

obtained between 0.8 of the mimimum value of V1 and 1.2 times the maximum

value of V1, considering the approved ranges of altitude and weight. The

ground run calibration will be determined assuming an engine failure at the

mimimum value of V1.

(c) The airspeed error of the installation excluding the instrument

calibration error, must not exceed 3 percent or 5 knots whichever is greater,

throughout the speed range from VMO to 1.3S1 with flaps retracted and from

1.3 VSO to VFE with flaps in the landing position.

(d) Information showing the relationship between IAS and CAS must be shown

in the Airplane Flight Manual.

14. Static air vent system. The static air vent system must meet the

requirements of FAR 23.1325. The altimeter system calibration must be

determined and shown in the Airplane Flight Manual.

Operating Limitations and Information

15. Maximum operating limit speed VMO/MMO. Instead of establishing

operating limitations based on VME and VNO, the applicant must establish a

maximum operating limit speed VMO/MMO in accordance with the following:

(a) The maximum operating limit speed must not exceed the design cruising

speed Vc and must be sufficiently below VD/MD or VDF/MDF to make it highly

improbable that the latter speeds will be inadvertently exceeded in flight.

(b) The speed Vmo must not exceed 0.8 VD/MD or 0.8 VDF/MDF unless flight

demonstrations involving upsets as specified by the Administrator indicates a

lower speed margin will not result in speeds exceeding VD/MD or VDF.

Atmospheric variations, horizontal gusts, and equipment errors, and airframe

production variations will be taken into account.

16. Minimum flight crew. In addition to meeting the requirements of FAR

23.1523, the applicant must establish the minimum number and type of

qualified flight crew personnel sufficient for safe operation of the airplane

considering--

(a) Each kind of operation for which the applicant desires approval;

(b) The workload on each crewmember considering the following:

(1) Flight path control.

(2) Collision avoidance.

(3) Navigation.

(4) Communications.

(5) Operation and monitoring of all essential aircraft systems.

(6) Command decisions; and

(c) The accessibility and ease of operation of necessary controls by the

appropriate crewmember during all normal and emergency operations when at his

flight station.

17. Airspeed indicator. The airspeed indicator must meet the requirements

of FAR 23.1545 except that, the airspeed notations and markings in terms of

VNO and VNE must be replaced by the VMO/MMO notations. The airspeed indicator

markings must be easily read and understood by the pilot. A placard adjacent

to the airspeed indicator is an acceptable means of showing compliance with

the requirements of FAR 23.1545(c).

Airplane Flight Manual

18. General. The Airplane Flight Manual must be prepared in accordance with

the requirements of FARs 23.1583 and 23.1587, and in addition the operating

limitations and performance information set forth in sections 19 and 20 must

be included.

19. Operating limitations. The Airplane Flight Manual must include the

following limitations--

(a) Airspeed limitations. (1) The maximum operating limit speed VMO/MMO and

a statement that this speed limit may not be deliberately exceeded in any

regime of flight (climb, cruise, or descent) unless a higher speed is

authorized for flight test or pilot training;

(2) If an airspeed limitation is based upon compressibility effects, a

statement to this effect and information as to any symptoms, the probable

behavior of the airplane, and the recommended recovery procedures; and

(3) The airspeed limits, shown in terms of VMO/MMO instead of VNO and VNE.

(b) Takeoff weight limitations. The maximum takeoff weight for each airport

elevation, ambient temperature, and available takeoff runway length within

the range selected by the applicant. This weight may not exceed the weight at

which:

(1) The all-engine operating takeoff distance determined in accordance with

section 5(d) or the accelerate-stop distance determined in accordance with

section 5(c), which ever is greater, is equal to the available runway length;

(2) The airplane complies with the one-engine-inoperative takeoff

requirements specified in section 5(e); and

(3) The airplane complies with the one-engine-inoperative en route climb

requirements specified in section 6(b), assuming that a standard temperature

lapse rate exists from the airport elevation to the altitude of 5,000 feet,

except that the weight may not exceed that corresponding to a temperature of

41 deg. F at 5,000 feet.

20. Performance information. The Airplane Flight Manual must contain the

performance information determined in accordance with the provisions of the

performance requirements of this regulation. The information must include the

following:

(a) Sufficient information so that the take-off weight limits specified in

section 19(b) can be determined for all temperatures and altitudes within the

operation limitations selected by the applicant.

(b) The conditions under which the performance information was obtained,

including the airspeed at the 50-foot height used to determine landing

distances.

(c) The performance information (determined by extrapolation and computed

for the range of weights between the maximum landing and takeoff weights)

for--

(1) Climb in the landing configuration; and

(2) Landing distance.

(d) Procedure established under section 4 of this regulation related to the

limitations and information required by this section in the form of guidance

material including any relevant limitations or information.

(e) An explanation of significant or unusual flight or ground handling

characteristics of the airplane.

(f) Airspeeds, as indicated airspeeds, corresponding to those determined

for takeoff in accordance with section 5(b).

21. Maximum operating altitudes. The maximum operating altitude to which

operation is permitted, as limited by flight, structural, powerplant,

functional, or equipment characteristics, must be specified in the Airplane

Flight Manual.

22. Stowage provision for Airplane Flight Manual. Provision must be made

for stowing the Airplane Flight Manual in a suitable fixed container which is

readily accessible to the pilot.

23. Operating procedures. Procedures for restarting turbine engines in

flight (including the effects of altitude) must be set forth in the Airplane

Flight Manual.

Airframe Requirements

FLIGHT LOADS

24. Engine torque. (a) Each turbopropeller engine mount and its supporting

structure must be designed for the torque effects of--

(1) The conditions set forth in FAR 23.361(a).

(2) The limit engine torque corresponding to takeoff power and propeller

speed, multiplied by a factor accounting for propeller control system

malfunction, including quick feathering action, simultaneously with 1 g level

flight loads. In the absence of a rational analysis, a factor of 1.6 must be

used.

(b) The limit torque is obtained by multiplying the mean torque by a factor

of 1.25.

25. Turbine engine gyroscopic loads. Each turbopropeller engine mount and

its supporting structure must be designed for the gyroscopic loads that

result, with the engines at maximum continuous r.p.m., under either--

(a) The conditions prescribed in FARs 23.351 and 23.423; or

(b) All possible combinations of the following:

(1) A yaw velocity of 2.5 radius per second.

(2) A pitch velocity of 1.0 radians per second.

(3) A normal load factor of 2.5.

(4) Maximum continuous thrust.

26. Unsymmetrical loads due to engine failure. (a) Turbopropeller powered

airplanes must be designed for the unsymmetrical loads resulting from the

failure of the critical engine including the following conditions in

combination with a single malfunction of the propeller drag limiting system,

considering the probable pilot corrective action on the flight controls.

(1) At speeds between VMC and VD, the loads resulting from power failure

because of fuel flow interruption are considered to be limit loads.

(2) At speeds between VMC and VC, the loads resulting from the

disconnection of the engine compressor from the turbine or from loss of the

turbine blades are considered to be ultimate loads.

(3) The time history of the thrust decay and drag buildup occurring as a

result of the prescribed engine failures must be substantiated by test or

other data applicable to the particular engine-propeller combination.

(4) The timing and magnitude of the probable pilot corrective action must

be conservatively estimated, considering the characteristics of the

particular engine-propeller-airplane combination.

(b) Pilot corrective action may be assumed to be initiated at the time

maximum yawing velocity is reached, but not earlier than two seconds after

the engine failure. The magnitude of the corrective action may be based on

the control forces specified in FAR 23.397 except that lower forces may be

assumed where it is shown by analysis or test that these forces can control

the yaw and roll resulting from the prescribed engine failure conditions.

Ground Loads

27. Dual wheel landing gear units. Each dual wheel landing gear unit and

its supporting structure must be shown to comply with the following:

(a) Pivoting. The airplane must be assumed to pivot about one side of the

main gear with the brakes on that side locked. The limit vertical load factor

must be 1.0 and the coefficient of friction 0.8. This condition need apply

only to the main gear and its supporting structure.

(b) Unequal tire inflation. A 60-40 percent distribution of the loads

established in accordance with FAR 23.471 through FAR 23.483 must be applied

to the dual wheels.

(c) Flat tire. (1) Sixty percent of the loads specified in FAR 23.471

through FAR 23.483 must be applied to either wheel in a unit.

(2) Sixty percent of the limit drag and side loads and 100 percent of the

limit vertical load established in accordance with FARs 23.493 and 23.485

must be applied to either wheel in a unit except that the vertical load need

not exceed the maximum vertical load in paragraph (c)(1) of this section.

Fatigue Evaluation

28. Fatigue evaluation of wing and associated structure. Unless it is shown

that the structure, operating stress levels, materials, and expected use are

comparable from a fatigue standpoint to a similar design which has had

substantial satisfactory service experience, the strength, detail design, and

the fabrication of those parts of the wing, wing carrythrough, and attaching

structure whose failure would be catastrophic must be evaluated under

either--

(a) A fatigue strength investigation in which the structu@ is @own by

analysis, tests, or both to be able to withstand the repeated loads of

variable magnitude expected in service; or

(b) A fail-safe strength investigation in which it is shown by analysis,

tests, or both that catastrophic failure of the structure is not probable

after fatigue, or obvious partial failure, of a principal structural element,

and that the remaining structure is able to withstand a static ultimate load

factor of 75 percent of the critical limit load factor at Vc. These loads

must be multiplied by a factor of 1.15 unless the dynamic effects of failure

under static load are otherwise considered.

Design and Construction

29. Flutter. For Multiengine turbopropeller powered airplanes, a dynamic

evaluation must be made and must include--

(a) The significant elastic, inertia, and aerodynamic forces associated

with the rotations and displacements of the plane of the propeller; and

(b) Engine-propeller-nacelle stiffness and damping variations appropriate

to the particular configuration.

Landing Gear

30. Flap operated landing gear warning device. Airplanes having retractable

landing gear and wing flaps must be equipped with a warning device that

functions continuously when the wing flaps are extended to a flap position

that activates the warning device to give adequate warning before landing,

using normal landing procedures, if the landing gear is not fully extended

and locked. There may not be a manual shut off for this warning device. The

flap position sensing unit may be installed at any suitable location. The

system for this device may use any part of the system (including the aural

warning device) provided for other landing gear warning devices.

Personnel and Cargo Accommodations

31. Cargo and baggage compartments.  Cargo and baggage compartments must be

designed to meet the requirements of FAR 23.787 (a) and (b), and in addition

means must be provided to protect passengers from injury by the contents of

any cargo or baggage compartment when the ultimate forward inertia force is

9g.

32. Doors and exits. The airplane must meet the requirements of FAR 23.783

and FAR 23.807 (a)(3), (b), and (c), and in addition:

(a) There must be a means to lock and safeguard each external door and exit

against opening in flight either inadvertently by persons, or as a result of

mechanical failure. Each external door must be operable from both the inside

and the outside.

(b) There must be means for direct visual inspection of the locking

mechanism by crewmembers to determine whether external doors and exits, for

which the initial opening movement is outward, are fully locked. In addition,

there must be a visual means to signal to crewmembers when normally used

external doors are closed and fully locked.

(c) The passenger entrance door must qualify as a floor level emergency

exit. Each additional required emergency exit except floor level exits must

be located over the wing or must be provided with acceptable means to assist

the occupants in descending to the ground. In addition to the passenger

entrance door:

(1) For a total seating capacity of 15 or less, an emergency exit as

defined in FAR 23.807(b) is required on each side of the cabin.

(2) For a total seating capacity of 16 through 23, three emergency exits as

defined in 23.807(b) are required with one on the same side as the door and

two on the side opposite the door.

(d) An evacuation demonstration must be conducted utilizing the maximum

number of occupants for which certification is desired. It must be conducted

under simulated night conditions utilizing only the emergency exits on the

most critical side of the aircraft. The participants must be representative

of average airline passengers with no prior practice or rehearsal for the

demonstration. Evacuation must be completed within 90 seconds.

(e) Each emergency exit must be marked with the word "Exit" by a sign which

has white letters 1 inch high on a red background 2 inches high, be self-

illuminated or independently internally electrically illuminated, and have a

minimum luminescence (brightness) of at least 160 microlamberts. The colors

may be reversed if the passenger compartment illumination is essentially the

same.

(f) Access to window type emergency exits must not be obstructed by seats

or seat backs.

(g) The width of the main passenger aisle at any point between seats must

equal or exceed the values in the following table.

Minimum main

passenger aisle

width

Less

than 25

inches   25 inches

Total seating    from     and more

capacity      floor    from floor

10 through 23  9 inches  15 inches.

Miscellaneous

33. Lightning strike protection. Parts that are electrically insulated from

the basic airframe must be connected to it through lightning arrestors unless

a lightning strike on the insulated part--

(a) Is improbable because of shielding by other parts; or

(b) Is not hazardous.

34. Ice protection. If certification with ice protection provisions is

desired, compliance with the following requirements must be shown:

(a) The recommended procedures for the use of the ice protection equipment

must be set forth in the Airplane Flight Manual.

(b) An analysis must be performed to establish, on the basis of the

airplane's operational needs, the adequacy of the ice protection system for

the various components of the airplane. In addition, tests of the ice

protection system must be conducted to demonstrate that the airplane is

capable of operating safely in continuous maximum and intermittent maximum

icing conditions as described in FAR 25, Appendix C.

(c) Compliance with all or portions of this section may be accomplished by

reference, where applicable because of similarity of the designs, to analysis

and tests performed by the applicant for a type certificated model.

35. Maintenance information. The applicant must make available to the owner

at the time of delivery of the airplane the information he considers

essential for the proper maintenance of the airplane. That information must

include the following:

(a) Description of systems, including electrical, hydraulic, and fuel

controls.

(b) Lubrication instructions setting forth the frequency and the lubricants

and fluids which are to be used in the various systems.

(c) Pressures and electrical loads applicable to the various systems.

(d) Tolerances and adjustments necessary for proper functioning.

(e) Methods of leveling, raising, and towing.

(f) Methods of balancing control surfaces.

(g) Identification of primary and secondary structures.

(h) Frequency and extent of inspections necessary to the proper operation

of the airplane.

(i) Special repair methods applicable to the airplane.

(j) Special inspection techniques, including those that require X-ray,

ultrasonic, and magnetic particle inspection.

(k) List of special tools.

Propulsion

GENERAL

36. Vibration characteristics. For turbopropeller powered airplanes, the

engine installation must not result in vibration characteristics of the

engine exceeding those established during the type certification of the

engine.

37. In-flight restarting of engine. If the engine on turbopropeller powered

airplanes cannot be restarted at the maximum cruise altitude, a determination

must be made of the altitude below which restarts can be consistently

accomplished. Restart information must be provided in the Airplane Flight

Manual.

38. Engines--(a) For turbopropeller powered airplanes. The engine

installation must comply with the following requirements:

(1) Engine isolation. The powerplants must be arranged and isolated from

each other to allow operation, in at least one configuration, so that the

failure or malfunction of any engine, or of any system that can affect the

engine, will not--

(i) Prevent the continued safe operation of the remaining engines; or

(ii) Require immediate action by any crewmember for continued safe

operation.

(2) Control of engine rotation. There must be a means to individually stop

and restart the rotation of any engine in flight except that engine rotation

need not be stopped if continued rotation could not jeopardize the safety of

the airplane. Each component of the stopping and restarting system on the

engine side of the firewall, and that might be exposed to fire, must be at

least fire resistant. If hydraulic propeller feathering systems are used for

this purpose, the feathering lines must be at least fire resistant under the

operating conditions that may be expected to exist during feathering.

(3) Engine speed and gas temperature control devices. The powerplant

systems associated with engine control devices, systems, and instrumentation

must provide reasonable assurance that those engine operating limitations

that adversely affect turbine rotor structural integrity will not be exceeded

in service.

(b) For reciprocating-engine powered airplanes.  To provide engine

isolation, the powerplants must be arranged and isolated from each other to

allow operation, in at least one configuration, so that the failure or

malfunction of any engine, or of any system that can affect that engine, will

not--

(1) Prevent the continued safe operation of the remaining engines; or

(2) Require immediate action by any crewmember for continued safe

operation.

39. Turbopropeller reversing systems. (a) Turbopropeller reversing systems

intended for ground operation must be designed so that no single failure or

malfunction of the system will result in unwanted reverse thrust under any

expected operating condition. Failure of structural elements need not be

considered if the probability of this kind of failure is extremely remote.

(b) Turbopropeller reversing systems intended for in-flight use must be

designed so that no unsafe condition will result during normal operation of

the system, or from any failure (or reasonably likely combination of

failures) of the reversing system, under any anticipated condition of

operation of the airplane. Failure of structural elements need not be

considered if the probability of this kind of failure is extremely remote.

(c) Compliance with this section may be shown by failure analysis, testing,

or both for propeller systems that allow propeller blades to move from the

flight low-pitch position to a position that is substantially less than that

at the normal flight low-pitch stop position. The analysis may include or be

supported by the analysis made to show compliance with the type certification

of the propeller and associated installation components. Credit will be given

for pertinent analysis and testing completed by the engine and propeller

manufacturers.

40. Turbopropeller drag-limiting systems.  Turbopropeller drag-limiting

systems must be designed so that no single failure or malfunction of any of

the systems during normal or emergency operation results in propeller drag in

excess of that for which the airplane was designed. Failure of structural

elements of the drag-limiting systems need not be considered if the

probability of this kind of failure is extremely remote.

41. Turbine engine powerplant operating characteristics. For turbopropeller

powered airplanes, the turbine engine powerplant operating characteristics

must be investigated in flight to determine that no adverse characteristics

(such as stall, surge, or flameout) are present to a hazardous degree, during

normal and emergency operation within the range of operating limitations of

the airplane and of the engine.

42. Fuel flow. (a) For turbopropeller powered airplanes--

(1) The fuel system must provide for continuous supply of fuel to the

engines for normal operation without interruption due to depletion of fuel in

any tank other than the main tank; and

(2) The fuel flow rate for turbopropeller engine fuel pump systems must not

be less than 125 percent of the fuel flow required to develop the standard

sea level atmospheric conditions takeoff power selected and included as an

operating limitation in the Airplane Flight Manual.

(b) For reciprocating engine powered airplanes, it is acceptable for the

fuel flow rate for each pump system (main and reserve supply) to be 125

percent of the takeoff fuel consumption of the engine.

Fuel System Components

43. Fuel pumps. For turbopropeller powered airplanes, a reliable and

independent power source must be provided for each pump used with turbine

engines which do not have provisions for mechanically driving the main pumps.

It must be demonstrated that the pump installations provide a reliability and

durability equivalent to that provided by FAR 23.991(a).

44. Fuel strainer or filter. For turbopropeller powered airplanes, the

following apply:

(a) There must be a fuel strainer or filter between the tank outlet and the

fuel metering device of the engine. In addition, the fuel strainer or filter

must be--

(1) Between the tank outlet and the engine-driven positive displacement

pump inlet, if there is an engine-driven positive displacement pump;

(2) Accessible for drainage and cleaning and, for the strainer screen,

easily removable; and

(3) Mounted so that its weight is not supported by the connecting lines or

by the inlet or outlet connections of the strainer or filter itself.

(b) Unless there are means in the fuel system to prevent the accumulation

of ice on the filter, there must be means to automatically maintain the fuel

flow if ice-clogging of the filter occurs; and

(c) The fuel strainer or filter must be of adequate capacity (with respect

to operating limitations established to insure proper service) and of

appropriate mesh to insure proper engine operation, with the fuel

contaminated to a degree (with respect to particle size and density) that can

be reasonably expected in service. The degree of fuel filtering may not be

less than that established for the engine type certification.

45. Lightning strike protection. Protection must be provided against the

ignition of flammable vapors in the fuel vent system due to lightning

strikes.

Cooling

46. Cooling test procedures for turbopropeller powered airplanes. (a)

Turbopropeller powered airplanes must be shown to comply with the

requirements of FAR 23.1041 during takeoff, climb en route, and landing

stages of flight that correspond to the applicable performance requirements.

The cooling test must be conducted with the airplane in the configuration and

operating under the conditions that are critical relative to cooling during

each stage of flight. For the cooling tests a temperature is "stabilized"

when its rate of change is less than 2 deg. F. per minute.

(b) Temperatures must be stabilized under the conditions from which entry

is made into each stage of flight being investigated unless the entry

condition is not one during which component and engine fluid temperatures

would stabilize, in which case, operation through the full entry condition

must be conducted before entry into the stage of flight being investigated in

order to allow temperatures to reach their natural levels at the time of

entry. The takeoff cooling test must be preceded by a period during which the

powerplant component and engine fluid temperatures are stabilized with the

engines at ground idle.

(c) Cooling tests for each stage of flight must be continued until--

(1) The component and engine fluid temperatures stabilize;

(2) The stage of flight is completed; or

(3) An operating limitation is reached.

Induction System

47. Air induction. For turbopropeller powered airplanes--

(a) There must be means to prevent hazardous quantities of fuel leakage or

overflow from drains, vents, or other components of flammable fluid systems

from entering the engine intake system; and

(b) The air inlet ducts must be located or protected so as to minimize the

ingestion of foreign matter during takeoff, landing, and taxiing.

48. Induction system icing protection. For turbopropeller powered

airplanes, each turbine engine must be able to operate throughout its flight

power range without adverse effect on engine operation or serious loss of

power or thrust, under the icing conditions specified in Appendix C of FAR

25. In addition, there must be means to indicate to appropriate flight

crewmembers the functioning of the powerplant ice protection system.

49. Turbine engine bleed air systems. Turbine engine bleed air systems of

turbopropeller powered airplanes must be investigated to determine--

(a) That no hazard to the airplane will result if a duct rupture occurs.

This condition must consider that a failure of the duct can occur anywhere

between the engine port and the airplane bleed service; and

(b) That if the bleed air system is used for direct cabin pressurization,

it is not possible for hazardous contamination of the cabin air system to

occur in event of lubrication system failure.

Exhaust System

50. Exhaust system drains. Turbopropeller engine exhaust systems having low

spots or pockets must incorporate drains at such locations. These drains must

discharge clear of the airplane in normal and ground attitudes to prevent the

accumulation of fuel after the failure of an attempted engine start.

Powerplant Controls and Accessories

51. Engine controls. If throttles or power levers for turbopropeller

powered airplanes are such that any position of these controls will reduce

the fuel flow to the engine(s) below that necessary for satisfactory and safe

idle operation of the engine while the airplane is in flight, a means must be

provided to prevent inadvertent movement of the control into this position.

The means provided must incorporate a positive lock or stop at this idle

position and must require a separate and distinct operation by the crew to

displace the control from the normal engine operating range.

52. Reverse thrust controls. For turbopropeller powered airplanes, the

propeller reverse thrust controls must have a means to prevent their

inadvertent operation. The means must have a positive lock or stop at the

idle position and must require a separate and distinct operation by the crew

to displace the control from the flight regime.

53. Engine ignition systems. Each turbopropeller airplane ignition system

must be considered an essential electrical load.

54. Powerplant accessories. The powerplant accessories must meet the

requirements of FAR 23.1163, and if the continued rotation of any accessory

remotely driven by the engine is hazardous when malfunctioning occurs, there

must be means to prevent rotation without interfering with the continued

operation of the engine.

Powerplant Fire Protection

55. Fire detector system. For turbopropeller powered airplanes, the

following apply:

(a) There must be a means that ensures prompt detection of fire in the

engine compartment. An overtemperature switch in each engine cooling air exit

is an acceptable method of meeting this requirement.

(b) Each fire detector must be constructed and installed to withstand the

vibration, inertia, and other loads to which it may be subjected in

operation.

(c) No fire detector may be affected by any oil, water, other fluids, or

fumes that might be present.

(d) There must be means to allow the flight crew to check, in flight, the

functioning of each fire detector electric circuit.

(e) Wiring and other components of each fire detector system in a fire zone

must be at least fire resistant.

56. Fire protection, cowling and nacelle skin. For reciprocating engine

powered airplanes, the engine cowling must be designed and constructed so

that no fire originating in the engine compartment can enter, either through

openings or by burn through, any other region where it would create

additional hazards.

57. Flammable fluid fire protection. If flammable fluids or vapors might be

liberated by the leakage of fluid systems in areas other than engine

compartments, there must be means to--

(a) Prevent the ignition of those fluids or vapors by any other equipment;

or

(b) Control any fire resulting from that ignition.

Equipment

58. Powerplant instruments. (a) The following are required for

turbopropeller airplanes:

(1) The instruments required by FAR 23.1305 (a)(1) through (4), (b)(2) and

(4).

(2) A gas temperature indicator for each engine.

(3) Free air temperature indicator.

(4) A fuel flowmeter indicator for each engine.

(5) Oil pressure warning means for each engine.

(6) A torque indicator or adequate means for indicating power output for

each engine.

(7) Fire warning indicator for each engine.

(8) A means to indicate when the propeller blade angle is below the low-

pitch position corresponding to idle operation in flight.

(9) A means to indicate the functioning of the ice protection system for

each engine.

(b) For turbopropeller powered airplanes, the turbopropeller blade position

indicator must begin indicating when the blade has moved below the flight

low-pitch position.

(c) The following instruments are required for reciprocating-engine powered

airplanes:

(1) The instruments required by FAR 23.1305.

(2) A cylinder head temperature indicator for each engine.

(3) A manifold pressure indicator for each engine.

Systems and Equipments

GENERAL

59. Function and installation. The systems and equipment of the airplane

must meet the requirements of FAR 23.1301, and the following:

(a) Each item of additional installed equipment must--

(1) Be of a kind and design appropriate to its intended function;

(2) Be labeled as to its identification, function, or operating

limitations, or any applicable combination of these factors, unless misuse or

inadvertent actuation cannot create a hazard;

(3) Be installed according to limitations specified for that equipment; and

(4) Function properly when installed.

(b) Systems and installations must be designed to safeguard against hazards

to the aircraft in the event of their malfunction or failure.

(c) Where an installation, the functioning of which is necessary in showing

compliance with the applicable requirements, requires a power supply, such

installation must be considered an essential load on the power supply, and

the power sources and the distribution system must be capable of supplying

the following power loads in probable operation combinations and for probable

durations:

(1) All essential loads after failure of any prime mover, power converter,

or energy storage device.

(2) All essential loads after failure of any one engine on two-engine

airplanes.

(3) In determining the probable operating combinations and durations of

essential loads for the power failure conditions described in subparagraphs

(1) and (2) of this paragraph, it is permissible to assume that the power

loads are reduced in accordance with a monitoring procedure which is

consistent with safety in the types of operations authorized.

60. Ventilation. The ventilation system of the airplane must meet the

requirements of FAR 23.831, and in addition, for pressurized aircraft the

ventilating air in flight crew and passenger compartments must be free of

harmful or hazardous concentrations of gases and vapors in normal operation

and in the event of reasonably probable failures or malfunctioning of the

ventilating, heating, pressurization, or other systems, and equipment. If

accumulation of hazardous quantities of smoke in the cockpit area is

reasonably probable, smoke evacuation must be readily accomplished.

Electrical Systems and Equipment

61. General. The electrical systems and equipment of the airplane must meet

the requirements of FAR 23.1351, and the following:

(a) Electrical system capacity. The required generating capacity, and

number and kinds of power sources must--

(1) Be determined by an electrical load analysis, and

(2) Meet the requirements of FAR 23.1301.

(b) Generating system. The generating system includes electrical power

sources, main power busses, transmission cables, and associated control,

regulation, and protective devices. It must be designed so that--

(1) The system voltage and frequency (as applicable) at the terminals of

all essential load equipment can be maintained within the limits for which

the equipment is designed, during any probable operating conditions;

(2) System transients due to switching, fault clearing, or other causes do

not make essential loads inoperative, and do not cause a smoke or fire

hazard;

(3) There are means, accessible in flight to appropriate crewmembers, for

the individual and collective disconnection of the electrical power sources

from the system; and

(4) There are means to indicate to appropriate crewmembers the generating

system quantities essential for the safe operation of the system, including

the voltage and current supplied by each generator.

62. Electrical equipment and installation.  Electrical equipment controls,

and wiring must be installed so that operation of any one unit or system of

units will not adversely affect the simultaneous operation of to the safe

operation.

63. Distribution system. (a) For the purpose of complying with this

section, the distribution system includes the distribution busses, their

associated feeders and each control and protective device.

(b) Each system must be designed so that essential load circuits can be

supplied in the event of reasonably probable faults or open circuits,

including faults in heavy current carrying cables.

(c) If two independent sources of electrical power for particular equipment

or systems are required by this regulation, their electrical energy supply

must be insured by means such as duplicate electrical equipment, throwover

switching, or multichannel or loop circuits separately routed.

64. Circuit protective devices. The circuit protective devices for the

electrical circuits of the airplane must meet the requirements of FAR

23.1357, and in addition circuits for loads which are essential to safe

operation must have individual and exclusive circuit protection.

[Doc. No. 8070, 34 FR 189, Jan. 7, 1969, as amended by SFAR 23-1, 34 FR

20176, Dec. 24, 1969; 35 FR 1102, Jan. 28, 1970]

SFAR No. 41

Editorial Note: For the text of SFAR No. 41, see Part 21 of this chapter.

Sec. 23.1  Applicability.

(a) This part prescribes airworthiness standards for the issue of type

certificates, and changes to those certificates, for airplanes in the normal,

utility, acrobatic, and commuter categories.

(b) Each person who applies under Part 21 for such a certificate or change

must show compliance with the applicable requirements of this part.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-34, 52 FR

1825, Jan. 15, 1987]

Sec. 23.2  Special retroactive requirements.

(a) Notwithstanding Secs. 21.17 and 21.101 of this chapter and irrespective

of the type certification basis, each normal, utility, and acrobatic category

airplane having a passenger seating configuration, excluding pilot seats, of

nine or less, manufactured after December 12, 1986, or any such foreign

airplane for entry into the United States must provide a safety belt and

shoulder harness for each forward- or aft-facing seat which will protect the

occupant from serious head injury when subjected to the inertia loads

resulting from the ultimate static load factors prescribed in Sec.

23.561(b)(2) of this part, or which will provide the occupant protection

specified in Sec. 23.562 of this part when that section is applicable to the

airplane. For other seat orientations, the seat/restraint system must be

designed to provide a level of occupant protection equivalent to that

provided for forward- or aft-facing seats with a safety belt and shoulder

harness installed.

(b) Each shoulder harness installed at a flight crewmember station, as

required by this section, must allow the crewmember, when seated with the

safety belt and shoulder harness fastened, to perform all functions necessary

for flight operations.

(c) For the purpose of this section, the date of manufacture is:

(1) The date the inspection acceptance records, or equivalent, reflect that

the airplane is complete and meets the FAA approved type design data; or

(2) In the case of a foreign manufactured airplane, the date the foreign

civil airworthiness authority certifies the airplane is complete and issues

an original standard airworthiness certificate, or the equivalent in that

country.

[Amdt. 23-36, 53 FR 30812, Aug. 15, 1988]

Sec. 23.3  Airplane categories.

(a) The normal category is limited to airplanes that have a seating

configuration, excluding pilot seats, of nine or less, a maximum certificated

takeoff weight of 12,500 pounds or less, and intended for nonacrobatic

operation. Nonacrobatic operation includes:

(1) Any maneuver incident to normal flying;

(2) Stalls (except whip stalls); and

(3) Lazy eights, chandelles, and steep turns, in which the angle of bank is

not more than 60 degrees.

(b) The utility category is limited to airplanes that have a seating

configuration, excluding pilot seats, of nine or less, a maximum certificated

takeoff weight of 12,500 pounds or less, and intended for limited acrobatic

operation. Airplanes certificated in the utility category may be used in any

of the operations covered under paragraph (a) of this section and in limited

acrobatic operations. Limited acrobatic operation includes:

(1) Spins (if approved for the particular type of airplane); and

(2) Lazy eights, chandelles, and steep turns, in which the angle of bank is

more than 60 degrees.

(c) The acrobatic category is limited to airplanes that have a seating

configuration, excluding pilot seats, of nine or less, a maximum certificated

takeoff weight of 12,500 pounds or less, and intended for use without

restrictions, other than those shown to be necessary as a result of required

flight tests.

(d) The commuter category is limited to propeller-driven, multiengine

airplanes that have a seating configuration excluding pilot seats, of 19 or

less, and a maximum certificated takeoff weight of 19,000 pounds or less,

intended for nonacrobatic operation as described in paragraph (a) of this

section.

(e) Airplanes may be type certificated in more than one category of this

part if the requirements of each requested category are met.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-4, 32 FR

5934, Apr. 14, 1967; Amdt. 23-34, 52 FR 1825, Jan. 15, 1987; 52 FR 34745,

Sept. 14, 1987]

Subpart B--Flight

--------[You Cited: 14 CFR Subpart A thru Subpart B as of Mar. 10, 1994]-------

Subpart A--General

Sec. 23.1  Applicability.

(a) This part prescribes airworthiness standards for the issue of type

certificates, and changes to those certificates, for airplanes in the normal,

utility, acrobatic, and commuter categories.

(b) Each person who applies under Part 21 for such a certificate or change

must show compliance with the applicable requirements of this part.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-34, 52 FR

1825, Jan. 15, 1987]

Sec. 23.2  Special retroactive requirements.

(a) Notwithstanding Secs. 21.17 and 21.101 of this chapter and irrespective

of the type certification basis, each normal, utility, and acrobatic category

airplane having a passenger seating configuration, excluding pilot seats, of

nine or less, manufactured after December 12, 1986, or any such foreign

airplane for entry into the United States must provide a safety belt and

shoulder harness for each forward- or aft-facing seat which will protect the

occupant from serious head injury when subjected to the inertia loads

resulting from the ultimate static load factors prescribed in Sec.

23.561(b)(2) of this part, or which will provide the occupant protection

specified in Sec. 23.562 of this part when that section is applicable to the

airplane. For other seat orientations, the seat/restraint system must be

designed to provide a level of occupant protection equivalent to that

provided for forward- or aft-facing seats with a safety belt and shoulder

harness installed.

(b) Each shoulder harness installed at a flight crewmember station, as

required by this section, must allow the crewmember, when seated with the

safety belt and shoulder harness fastened, to perform all functions necessary

for flight operations.

(c) For the purpose of this section, the date of manufacture is:

(1) The date the inspection acceptance records, or equivalent, reflect that

the airplane is complete and meets the FAA approved type design data; or

(2) In the case of a foreign manufactured airplane, the date the foreign

civil airworthiness authority certifies the airplane is complete and issues

an original standard airworthiness certificate, or the equivalent in that

country.

[Amdt. 23-36, 53 FR 30812, Aug. 15, 1988]

Sec. 23.3  Airplane categories.

(a) The normal category is limited to airplanes that have a seating

configuration, excluding pilot seats, of nine or less, a maximum certificated

takeoff weight of 12,500 pounds or less, and intended for nonacrobatic

operation. Nonacrobatic operation includes:

(1) Any maneuver incident to normal flying;

(2) Stalls (except whip stalls); and

(3) Lazy eights, chandelles, and steep turns, in which the angle of bank is

not more than 60 degrees.

(b) The utility category is limited to airplanes that have a seating

configuration, excluding pilot seats, of nine or less, a maximum certificated

takeoff weight of 12,500 pounds or less, and intended for limited acrobatic

operation. Airplanes certificated in the utility category may be used in any

of the operations covered under paragraph (a) of this section and in limited

acrobatic operations. Limited acrobatic operation includes:

(1) Spins (if approved for the particular type of airplane); and

(2) Lazy eights, chandelles, and steep turns, in which the angle of bank is

more than 60 degrees.

(c) The acrobatic category is limited to airplanes that have a seating

configuration, excluding pilot seats, of nine or less, a maximum certificated

takeoff weight of 12,500 pounds or less, and intended for use without

restrictions, other than those shown to be necessary as a result of required

flight tests.

(d) The commuter category is limited to propeller-driven, multiengine

airplanes that have a seating configuration excluding pilot seats, of 19 or

less, and a maximum certificated takeoff weight of 19,000 pounds or less,

intended for nonacrobatic operation as described in paragraph (a) of this

section.

(e) Airplanes may be type certificated in more than one category of this

part if the requirements of each requested category are met.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-4, 32 FR

5934, Apr. 14, 1967; Amdt. 23-34, 52 FR 1825, Jan. 15, 1987; 52 FR 34745,

Sept. 14, 1987]

Subpart B--Flight

General

Sec. 23.21  Proof of compliance.

(a) Each requirement of this subpart must be met at each appropriate

combination of weight and center of gravity within the range of loading

conditions for which certification is requested. This must be shown--

(1) By tests upon an airplane of the type for which certification is

requested, or by calculations based on, and equal in accuracy to, the results

of testing; and

(2) By systematic investigation of each probable combination of weight and

center of gravity, if compliance cannot be reasonably inferred from

combinations investigated.

(b) The following general tolerances are allowed during flight testing.

However, greater tolerances may be allowed in particular tests:

Item                      Tolerance

Weight                             +5%, -10%.

Critical items affected by weight  +5%, -1%.

C.G                                +/-7% total travel.

Sec. 23.23   Load distribution limits.

(a) Ranges of weights and centers of gravity within which the airplane may

be safely operated must be established. If a weight and center of gravity

combination is allowable only within certain lateral load distribution limits

that could be inadvertently exceeded, these limits must be established for

the corresponding weight and center of gravity combinations.

(b) The load distribution limits may not exceed any of the following:

(1) The selected limits;

(2) The limits at which the structure is proven; or

(3) The limits at which compliance with each applicable flight requirement

of this subpart is shown.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55463, Dec. 20, 1976; Amdt. 23-45, 58 FR 42156, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.25  Weight limits.

(a) Maximum weight. The maximum weight is the highest weight at which

compliance with each applicable requirement of this Part (other than those

complied with at the design landing weight) is shown. In addition, for

commuter category airplanes, the applicant must establish a maximum zero fuel

weight. The maximum weight must be established so that it is--

(1) Not more than--

(i) The highest weight selected by the applicant;

(ii) The design maximum weight, which is the highest weight at which

compliance with each applicable structural loading condition of this part

(other than those complied with at the design landing weight) is shown; or

(iii) The highest weight at which compliance with each applicable flight

requirement is shown, except for airplanes equipped with standby power rocket

engines, in which case it is the highest weight established in accordance

with Appendix E of this part; or

(2) Not less than the weight with--

(i) Each seat occupied, assuming a weight of 170 pounds for each occupant

for normal and commuter category airplanes, and 190 pounds for utility and

acrobatic category airplanes, except that seats other than pilot seats may be

placarded for a lesser weight; and

(A) Oil at full capacity, and

(B) At least enough fuel for maximum continuous power operation of at least

30 minutes for day-VFR approved airplanes and at least 45 minutes for night-

VFR and IFR approved airplanes; or

(ii) The required minimum crew, and fuel and oil to full tank capacity.

(b) Minimum weight. The minimum weight (the lowest weight at which

compliance with each applicable requirement of this part is shown) must be

established so that it is not more than the sum of--

(1) The empty weight determined under Sec. 23.29;

(2) The weight of the required minimum crew (assuming a weight of 170

pounds for each crewmember); and

(3) The weight of--

(i) For turbojet powered airplanes, 5 percent of the total fuel capacity of

that particular fuel tank arrangement under investigation, and

(ii) For other airplanes, the fuel necessary for one-half hour of operation

at maximum continuous power.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13086, Aug. 13, 1969; Amdt. 23-21, 43 FR 2317, Jan. 16, 1978; Amdt. 23-34, 52

FR 1825, Jan. 15, 1987; Amdt. No. 23-45, 58 FR 42156, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.29  Empty weight and corresponding center of gravity.

(a) The empty weight and corresponding center of gravity must be determined

by weighing the airplane with--

(1) Fixed ballast;

(2) Unusable fuel determined under Sec. 23.959; and

(3) Full operating fluids, including--

(i) Oil;

(ii) Hydraulic fluid; and

(iii) Other fluids required for normal operation of airplane systems,

except potable water, lavatory precharge water, and water intended for

injection in the engines.

(b) The condition of the airplane at the time of determining empty weight

must be one that is well defined and can be easily repeated.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-21, 43 FR 2317, Jan. 16, 1978]

Sec. 23.31  Removable ballast.

Removable ballast may be used in showing compliance with the flight

requirements of this subpart, if--

(a) The place for carrying ballast is properly designed and installed, and

is marked under Sec. 23.1557; and

(b) Instructions are included in the airplane flight manual, approved

manual material, or markings and placards, for the proper placement of the

removable ballast under each loading condition for which removable ballast is

necessary.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-13, 37 FR 20023, Sept. 23, 1972]

Sec. 23.33  Propeller speed and pitch limits.

(a) General. The propeller speed and pitch must be limited to values that

will assure safe operation under normal operating conditions.

(b) Propellers not controllable in flight. For each propeller whose pitch

cannot be controlled in flight--

(1) During takeoff and initial climb at Vy, the propeller must limit the

engine r.p.m., at full throttle or at maximum allowable takeoff manifold

pressure, to a speed not greater than the maximum allowable takeoff r.p.m.;

and

(2) During a closed throttle glide at the placarded "never-exceed speed",

the propeller may not cause an engine speed above 110 percent of maximum

continuous speed.

(c) Controllable pitch propellers without constant speed controls. Each

propeller that can be controlled in flight, but that does not have constant

speed controls, must have a means to limit the pitch range so that--

(1) The lowest possible pitch allows compliance with paragraph (b)(1) of

this section; and

(2) The highest possible pitch allows compliance with paragraph (b)(2) of

this section.

(d) Controllable pitch propellers with constant speed controls. Each

controllable pitch propeller with constant speed controls must have--

(1) With the governor in operation, a means at the governor to limit the

maximum engine speed to the maximum allowable takeoff r.p.m.; and

(2) With the governor inoperative, the propeller blades at the lowest

possible pitch, with takeoff power, the airplane stationary, and no wind,

either--

(i) A means to limit the maximum engine speed to 103 percent of the maximum

allowable takeoff r.p.m., or

(ii) For an engine with an approved overspeed, a means to limit the maximum

engine and propeller speed to not more than the maximum approved overspeed.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965; Amdt.

No. 23-45, 58 FR 42156, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Performance

Sec. 23.45  General.

(a) Unless otherwise prescribed, the performance requirements of this

subpart must be met for still air; and

(1) Standard atmospheric conditions for normal, utility, and acrobatic

category airplanes; or

(2) Ambient atmospheric conditions for commuter category airplanes.

(b) The performance data must correspond to the propulsive power or thrust

available under the particular ambient atmospheric conditions, the particular

flight condition, and the relative humidity specified in paragraph (d) of

this section.

(c) The available propulsive thrust must correspond to engine power or

thrust, not exceeding the approved power or thrust, less--

(1) Installation losses; and

(2) The power or equivalent thrust absorbed by the accessories and services

appropriate to the particular ambient atmospheric conditions and the

particular flight condition.

(d) The performance, as affected by engine power or thrust, must be based

on a relative humidity of--

(1) 80 percent, at and below standard temperature; and

(2) 34 percent, at and above standard temperature, plus 50 deg.F.

(3) Between the two temperatures listed in paragraphs (d)(1) and (d)(2) of

this section, the relative humidity must vary linearly.

(e) For commuter category airplanes, the following also apply:

(1) Unless otherwise prescribed, the applicant must select the takeoff, en

route, approach, and landing configurations for the airplane;

(2) The airplane configuration may vary with weight, altitude, and

temperature, to the extent they are compatible with the operating procedures

required by paragraph (e)(3) of this section;

(3) Unless otherwise prescribed, in determining the critical-engine-

inoperative takeoff performance, takeoff flight path, the accelerate-stop

distance, takeoff distance, and landing distance, changes in the airplane's

configuration, speed, power, and thrust must be made in accordance with

procedures established by the applicant for operation in service;

(4) Procedures for the execution of missed approaches and balked landings

associated with the conditions prescribed in Secs. 23.67(e)(3) and 23.77(c)

must be established; and

(5) The procedures established under paragraphs (e)(3) and (e)(4) of this

section must--

(i) Be able to be consistently executed by a crew of average skill;

(ii) Use methods or devices that are safe and reliable; and

(iii) Include allowance for any reasonably expected time delays in the

execution of the procedures.

[Amdt. 23-21, 43 FR 2317, Jan. 16, 1978, as amended by Amdt. 23-34, 52 FR

1826, Jan. 15, 1987; Amdt. No. 23-45, 58 FR 42156, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.49  Stalling speed.

(a) Vs 0 is the stalling speed, if obtainable, or the minimum steady speed,

in knots (CAS), at which the airplane is controllable, with the--

(1) Applicable power or thrust condition set forth in paragraph (e) of this

section;

(2) Propellers in the takeoff position;

(3) Landing gear extended;

(4) Wing flaps in the landing position;

(5) Cowl flaps closed;

(6) Center of gravity in the most unfavorable position within the allowable

landing range; and

(7) Weight used when VS0 is being used as a factor to determine compliance

with a required performance standard.

(b) Except as provided in Sec. 23.49(c), VS0 at maximum weight may not

exceed 61 knots for--

(1) Single-engine airplanes; and

(2) Multiengine airplanes of 6,000 pounds or less maximum weight that

cannot meet the minimum rate of climb specified in Sec. 23.67(b) with the

critical engine inoperative.

(c) All single-engine airplanes, and those multiengine airplanes of 6,000

pounds or less maximum weight with a VS0 of more than 61 knots that do not

meet the requirements of Sec. 23.67(b)(2)(i), must comply with Sec.

23.562(d).

(d) VS1 is the calibrated stalling speed, if obtainable, or the minimum

steady speed, in knots, at which the airplane is controllable, with the--

(1) Applicable power or thrust condition set forth in paragraph (e) of this

section;

(2) Propellers in the takeoff position;

(3) Airplane in the condition existing in the test in which VS1 is being

used; and

(4) Weight used when VS1 is being used as a factor to determine compliance

with a required performance standard.

(e) VS0 and VS1 must be determined by flight tests, using the procedure

specified in Sec. 23.201.

(f) The following power or thrust conditions must be used to meet the

requirements of this section:

(1) For reciprocating engine-powered airplanes, engines idling, throttles

closed or at not more than the power necessary for zero thrust at a speed not

more than 110 percent of the stalling speed.

(2) For turbine engine-powered airplanes, the propulsive thrust may not be

greater than zero at the stalling speed, or, if the resultant thrust has no

appreciable effect on the stalling speed, with engines idling and throttles

closed.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13086, Aug. 13, 1969; Amdt. 23-21, 43 FR 2317, Jan. 16, 1978; Amdt. 23-44,

58 FR 38639, July 19, 1993]

*****************************************************************************

58 FR 38634, No. 136, July 19, 1993

SUMMARY: This final rule amends the stalling speed requirements applicable to

single-engine airplanes and to certain multiengine small airplanes of less

than 6,000 pounds maximum weight. The rule permits those airplanes to have a

stall speed greater than 61 knots, provided they meet certain additional

occupant protection standards. These changes are needed to permit the design

and type certification of higher performance airplanes with increased cruise

speeds and better specific fuel consumption. The amendments are intended to

achieve the benefits of certificating higher performance airplanes while

affording their occupants the same level of protection in an emergency

landing that is presently provided by airplanes with a 61-knot stall speed.

EFFECTIVE DATE: August 18, 1993.

*****************************************************************************

Sec. 23.51  Takeoff.

(a) For each airplane (except a skiplane for which landplane takeoff data

has been determined under this paragraph and furnished in the Airplane Flight

Manual) the distance required to takeoff and climb over a 50-foot obstacle

must be determined with--

(1) The engines operating within approved operating limitations; and

(2) The cowl flaps in the normal takeoff position.

(b) The starting point for measuring seaplane and amphibian takeoff

distance may be the point at which a speed of not more than three knots is

reached.

(c) Takeoffs made to determine the data required by this section may not

require exceptional piloting skill or exceptionally favorable conditions.

(d) For commuter category airplanes, takeoff performance and data as

required by Secs. 23.53 through 23.59 must be determined and included in the

Airplane Flight Manual--

(1) For each weight, altitude, and ambient temperature within the

operational limits selected by the applicant;

(2) For the selected configuration for takeoff;

(3) For the most unfavorable center of gravity position;

(4) With the operating engine within approved operating limitations;

(5) On a smooth, dry, hard surface runway; and

(6) Corrected for the following operational correction factors:

(i) Not more than 50 percent of nominal wind components along the takeoff

path opposite to the direction of takeoff and not less than 150 percent of

nominal wind components along the takeoff path in the direction of takeoff;

and

(ii) Effective runway gradients.

[Amdt. 23-21, 43 FR 2317, Jan. 16, 1978, as amended by Amdt. 23-34, 52 FR

1826, Jan. 15, 1987]

Sec. 23.53   Takeoff speeds.

(a) For multiengine normal, utility, and acrobatic category airplanes, the

rotation speed, VR, may not be less than VMC determined in accordance with

Sec. 23.149.

(b) Each normal, utility, and acrobatic category airplane, upon reaching a

height of 50 feet above the takeoff surface level, must have reached a speed

of not less than the following:

(1) For multiengine airplanes, the higher of--

(i) 1.1 VMC; or

(ii) Any lesser speed, not less than 1.2 VS1, that is shown to be safe for

continued flight or land-back, if applicable, under all conditions, including

turbulence and complete failure of the critical engine.

(2) For single-engine airplanes, any speed, not less than 1.2 VS1, that is

shown to be safe under all conditions, including turbulence and complete

engine failure.

(c) For commuter category airplanes, the following apply:

(1) The takeoff decision speed, V1, is the calibrated airspeed on the

ground at which, as a result of engine failure or other reasons, the pilot is

assumed to have made a decision to continue or discontinue the takeoff. The

takeoff decision speed, V1, must be selected by the applicant but may not be

less than the greater of the following:

(i) 1.10 VS1;

(ii) 1.10 VMC established in accordance with Sec. 23.149;

(iii) A speed at which the airplane can be rotated for takeoff and shown to

be adequate to safely continue the takeoff, using normal piloting skill, when

the critical engine is suddenly made inoperative; or

(iv) VEF plus the speed gained with the critcial engine inoperative during

the time interval between the instant that the critical engine is failed and

the instant at which the pilot recognizes and reacts to the engine failure as

indicated by the pilot's application of the first retarding means during the

accelerate-stop determination of Sec. 23.55.

(2) The takeoff safety speed, V2, in terms of calibrated airspeed, must be

selected by the applicant so as to allow the gradient of climb required in

Sec. 23.67 but must not be less than V1 or less than 1.2VS1.

(3) The critical engine failure speed, VEF, is the calibrated airspeed at

which the critical engine is assumed to fail. VEF must be selected by the

applicant but not less than VMC determined in accordance with Sec. 23.149.

(4) The rotation speed, VR in terms of calibrated airspeed, must be

selected by the applicant and may not be less than the greater of the

following:

(i) V1; or

(ii) The speed determined in accordance with Sec. 23.57(c) that allows

attaining the initial climb out speed, V2, before reaching a height of 35

feet above the takeoff surface.

(5) For any given set of conditions, such as weight, altitude,

configuration, and temperature, a single value of VR must be used to show

compliance with both the one-engine-inoperative takeoff and all-engines-

operating takeoff requirements:

(i) One-engine-inoperative takeoff determined in accordance with Sec.

23.57; and

(ii) All-engines-operating takeoff determined in accordance with Sec.

23.59.

(6) The one-engine-inoperative takeoff distance, using a normal rotation

rate at a speed of 5 knots less than VR established in accordance with

paragraphs (c)(4) and (5) of this section, must be shown not to exceed the

corresponding one-engine-inoperative takeoff distance determined in

accordance with Secs. 23.57 and 23.59 using the established VR. The take off

distance determined in accordance with Sec. 23.59 and the takeoff must be

safely continued from the point at which the airplane is 35 feet above the

takeoff surface at a speed not less than 5 knots less than the established V2

speed.

(7) The applicant must show, with all engines operating, that marked

increases in the scheduled takeoff distances determined in accordance with

Sec. 23.59 do not result from over-rotation of the airplane and out-of-trim

conditions.

[Amdt. 23-34, 52 FR 1826, Jan. 15, 1987; 52 FR 34745, Sept. 14, 1987, as

amended by Amdt. 23-45, 58 FR 42156, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.55  Accelerate-stop distance.

For each commuter category airplane, the accelerate-stop distance must be

determined as follows:

(a) The accelerate-stop distance is the sum of the distances necessary to--

(1) Accelerate the airplane from a standing start to V1; and

(2) Come to a full stop from the point at which V1 is reached assuming that

in the case of engine failure, the pilot has decided to stop as indicated by

application of the first retarding means at the speed V1.

(b) Means other than wheel brakes may be used to determine the accelerate-

stop distance if that means is available with the critical engine inoperative

and if that means--

(1) Is safe and reliable;

(2) Is used so that consistent results can be expected under normal

operating conditions; and

(3) Is such that exceptional skill is not required to control the airplane.

[Amdt. 23-34, 52 FR 1826, Jan. 15, 1987]

Sec. 23.57  Takeoff path.

For each commuter category airplane, the takeoff path is as follows:

(a) The takeoff path extends from a standing start to a point in the

takeoff at which the airplane is 1,500 feet above the takeoff surface or at

which the transition from the takeoff to the en route configuration is

completed, whichever point is higher; and

(1) The takeoff path must be based on the procedures prescribed in Sec.

23.45;

(2) The airplane must be accelerated on the ground to VEF at which point

the critical engine must be made inoperative and remain inoperative for the

rest of the takeoff; and

(3) After reaching VEF, the airplane must be accelerated to V2.

(b) During the acceleration to speed V2, the nose gear may be raised off

the ground at a speed not less than VR. However, landing gear retraction may

not be initiated until the airplane is airborne.

(c) During the takeoff path determination, in accordance with paragraphs

(a) and (b) of this section--

(1) The slope of the airborne part of the takeoff path must be positive at

each point;

(2) The airplane must reach V2 before it is 35 feet above the takeoff

surface, and must continue at a speed as close as practical to, but not less

than V2, until it is 400 feet above the takeoff surface;

(3) At each point along the takeoff path, starting at the point at which

the airplane reaches 400 feet above the takeoff surface, the available

gradient of climb may not be less than--

(i) 1.2 percent for two-engine airplanes;

(ii) 1.5 percent for three-engine airplanes;

(iii) 1.7 percent for four-engine airplanes; and

(4) Except for gear retraction and automatic propeller feathering, the

airplane configuration may not be changed, and no change in power or thrust

that requires action by the pilot may be made, until the airplane is 400 feet

above the takeoff surface.

(d) The takeoff path must be determined by a continuous demonstrated

takeoff or by synthesis from segments. If the takeoff path is determined by

the segmental method--

(1) The segments must be clearly defined and must be related to the

distinct changes in the configuration, power or thrust, and speed;

(2) The weight of the airplane, the configuration, and the power or thrust

must be constant throughout each segment and must correspond to the most

critical condition prevailing in the segment;

(3) The flight path must be based on the airplane's performance without

ground effect;

(4) The takeoff path data must be checked by continuous demonstrated

takeoffs up to the point at which the airplane is out of ground effect and

its speed is stabilized to ensure that the path is conservative relative to

the continuous path; and

(5) The airplane is considered to be out of the ground effect when it

reaches a height equal to its wing span.

[Amdt. 23-34, 52 FR 1827, Jan. 15, 1987]

Sec. 23.59  Takeoff distance and takeoff run.

For each commuter category airplane--

(a) Takeoff distance is the greater of--

(1) The horizontal distance along the takeoff path from the start of the

takeoff to the point at which the airplane is 35 feet above the takeoff

surface as determined under Sec. 23.57; or

(2) With all engines operating, 115 percent of the horizontal distance

along the takeoff path, with all engines operating, from the start of the

takeoff to the point at which the airplane is 35 feet above the takeoff

surface, as determined by a procedure consistent with Sec. 23.57.

(b) If the takeoff distance includes a clearway, the takeoff run is the

greater of--

(1) The horizontal distance along the takeoff path from the start of the

takeoff to a point equidistant between the point at which VLOF is reached and

the point at which the airplane is 35 feet above the takeoff surface as

determined under Sec. 23.57; or

(2) With all engines operating, 115 percent of the horizontal distance

along the takeoff path, with all engines operating, from the start of the

takeoff to a point equidistant between the point at which VLOF is reached and

the point at which the airplane is 35 feet above the takeoff surface

determined by a procedure consistent with Sec. 23.57.

[Amdt. 23-34, 52 FR 1827, Jan. 15, 1987]

Sec. 23.61  Takeoff flight path.

For each commuter category airplane, the takeoff flight path must be

determined as follows:

(a) The takeoff flight path begins 35 feet above the takeoff surface at the

end of the takeoff distance determined in accordance with Sec. 23.59.

(b) The net takeoff flight path data must be determined so that they

represent the actual takeoff flight paths, as determined in accordance with

Sec. 23.57 and with paragraph (a) of this section, reduced at each point by a

gradient of climb equal to--

(1) 0.8 percent for two-engine airplanes;

(2) 0.9 percent for three-engine airplanes; and

(3) 1.0 percent for four-engine airplanes.

(c) The prescribed reduction in climb gradient may be applied as an

equivalent reduction in acceleration along that part of the takeoff flight

path at which the airplane is accelerated in level flight.

[Amdt. 23-34, 52 FR 1827, Jan. 15, 1987]

Sec. 23.65  Climb: All engines operating.

Sec. 23.65  Climb: All engines operating.

(a) Each airplane must have a steady angle of climb at sea level of at

least 1:12 for landplanes or 1:15 for seaplanes and amphibians with--

(1) A speed not less than 1.2 VS1;

(2) Not more than maximum continuous power on each engine;

(3) The landing gear retracted;

(4) The wing flaps in the takeoff position; and

(5) The cowl flaps or other means for controlling the engine cooling air

supply in the position used in the cooling tests required by Secs. 23.1041

through 23.1047.

(b) Each airplane with engines for which the takeoff and maximum continuous

power ratings are identical and that has fixed-pitch, two-position, or

similar propellers, may use a lower propeller pitch setting than that allowed

by Sec. 23.33 to obtain rated engine r.p.m. at Vx, if--

(1) The airplane shows marginal performance (such as when it can meet the

rate of climb requirements of paragraph (a) of this section but has

difficulty in meeting the angle of climb requirements of paragraph (a) of

this section or of Sec. 23.77); and

(2) Acceptable engine cooling is shown at the lower speed associated with

the best angle of climb.

(c) Each turbine engine-powered airplane must be able to maintain a steady

gradient of climb of at least 4 percent at a pressure altitude of 5,000 feet

and a temperature of 81 degrees F (standard temperature plus 40 degree F)

with the airplane in the configuration prescribed in paragraph (a) of this

section.

(d) In addition for commuter category airplanes, performance data must be

determined for variations in weight, altitude, and temperature at the most

critical center of gravity for which approval is requested.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13086, Aug. 13, 1969; Amdt. 23-21, 43 FR 2317, Jan. 16, 1978; Amdt. 23-34, 52

FR 1827, Jan. 15, 1987; Amdt. 23-24, 52 FR 34745, Sept. 14, 1987; Amdt. No.

23-45, 58 FR 42156, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.67  Climb: one engine inoperative.

(a) For normal, utility, and acrobatic category, reciprocating engine-

powered multiengine airplanes, one-engine-inoperative climb gradients must be

determined with the--

(1) Critical engine inoperative, and its propeller in the minimum drag

position;

(2) Remaining engines at not more than maximum continuous power or thrust;

(3) Landing gear retracted;

(4) Wing flaps in the most favorable position; and

(5) Means for controlling the engine cooling air supply in the position

used in the engine cooling tests required by Secs. 23.1041 through 23.1047.

(b) For normal, utility, and acrobatic category reciprocating engine-

powered multiengine airplanes, the following apply:

(1) Each airplane of more than 6,000 pounds maximum weight must be able to

maintain a steady climb gradient of at least 1.5 percent at a pressure

altitude of 5,000 feet at a speed not less than 1.2 VS1 and at standard

temperature (41 deg.F) with the airplane in the configuration prescribed in

paragraph (a) of this section.

(2) For each airplane of 6,000 pounds or less maximum weight, the following

apply:

(i) Each airplane that meets the requirements of Sec. 23.562(d), or that

has a VS0 of 61 knots or less, must have its steady climb gradient determined

at a pressure altitude of 5,000 feet at a speed of not less than 1.2 VS1, and

at standard temperature (41 deg.F), with the airplane in the configuration

prescribed in paragraph (a) of this section.

(ii) Except for those airplanes that meet the requirements prescribed in

Sec. 23.562(d), each airplane with a VS0 of more than 61 knots must be able

to maintain the steady climb gradient prescribed in paragraph (b)(1) of this

section.

(c) For normal, utility, and acrobatic category turbine engine-powered

multiengine airplanes the following apply:

(1) The steady climb gradient must be determined at each weight, altitude,

and ambient temperature within the operational limits established by the

applicant, with the airplane in the configuration prescribed in paragraph (a)

of this section.

(2) Each airplane must be able to maintain at least the following climb

gradients with the airplane in the configuration prescribed in paragraph (a)

of this section:

(i) 1.5 percent at a pressure altitude of 5,000 feet at a speed not less

than 1.2 VS1, and at standard temperature (41 deg.F); and

(ii) 0.75 percent at a pressure altitude of 5,000 feet at a speed not less

than 1.2 VS1 and 81 deg.F (standard temperature plus 40 deg.F).

(3) The minimum climb gradient specified in paragraphs (c)(2) (i) and (ii)

of this section must vary linearly between 41 deg.F and 81 deg.F and must

change at the same rate up to the maximum operating temperature approved for

the airplane.

(d) For all multiengine airplanes, the speed for best rate of climb with

one engine inoperative must be determined.

(e) For commuter category airplanes, the following apply:

(1) Takeoff climb: The maximum weight at which the airplane meets the

minimum climb performance specified in paragraphs (e)(1) (i) and (ii) of this

section must be determined for each altitude and ambient temperature within

the operating limitations established for the airplane, out of ground effect

in free air, with the airplane in the takeoff configuration, with the most

critical center of gravity, the critical engine inoperative, the remaining

engines at the maximum takeoff power or thrust, and the propeller of the

inoperative engine windmilling with the propeller controls in the normal

position, except that, if an approved automatic propeller feathering system

is installed, the propeller may be in the feathered position:

(i) Takeoff, landing gear extended. The minimum steady gradient of climb

between the lift-off speed, VLOF, and until the landing gear is retracted

must be measurably positive for two-engine airplanes, not less than 0.3

percent for three-engine airplanes, or 0.5 percent for four-engine airplanes

at all points along the flight path; and

(ii) Takeoff, landing gear retracted. The minimum steady gradient of climb

must not be less than 2 percent for two-engine airplanes, 2.3 percent for

three-engine airplanes, and 2.6 percent for four-engine airplanes at the

speed V2, until the airplane is 400 feet above the takeoff surface. For

airplanes with fixed landing gear, this requirement must be met with the

landing gear extended.

(2) En route climb: The maximum weight must be determined for each altitude

and ambient temperature within the operational limits established for the

airplane, at which the steady gradient of climb is not less than 1.2 percent

for two-engine airplanes, 1.5 percent for three-engine airplanes, and 1.7

percent for four-engine airplanes at an height of 1,500 feet above the

takeoff surface, with the airplane in the en route configuration, the

critical engine inoperative, the remaining engine at the maximum continuous

power or thrust, and the most unfavorable center of gravity.

(3) Approach: In the approach configuration corresponding to the normal

all-engines-operating procedure in which VS1 for this configuration does not

exceed 110 percent of the VS1 for the related landing configuration, the

steady gradient of climb may not be less than 2.1 percent for two-engine

airplanes, 2.4 percent for three-engine airplanes, and 2.7 percent for four-

engine airplanes, with--

(i) The critical engine inoperative and the remaining engines at the

available takeoff power or thrust;

(ii) The maximum landing weight; and

(iii) A climb speed established in connection with the normal landing

procedures but not exceeding 1.5 VS1.

[Amdt. 23-21, 43 FR 2317, Jan. 16, 1978, as amended by Amdt. 23-34, 52 FR

1827, Jan. 15, 1987; Amdt. 23-24, 52 FR 34745, Sept. 14, 1987; Amdt. 23-39,

55 FR 18575, May 2, 1990; Amdt. 23-42, 56 FR 351, Jan. 3, 1991; Amdt. 23-44,

58 FR 38639, July 19, 1993]

*****************************************************************************

58 FR 38634, No. 136, July 19, 1993

SUMMARY: This final rule amends the stalling speed requirements applicable to

single-engine airplanes and to certain multiengine small airplanes of less

than 6,000 pounds maximum weight. The rule permits those airplanes to have a

stall speed greater than 61 knots, provided they meet certain additional

occupant protection standards. These changes are needed to permit the design

and type certification of higher performance airplanes with increased cruise

speeds and better specific fuel consumption. The amendments are intended to

achieve the benefits of certificating higher performance airplanes while

affording their occupants the same level of protection in an emergency

landing that is presently provided by airplanes with a 61-knot stall speed.

EFFECTIVE DATE: August 18, 1993.

*****************************************************************************

Sec. 23.75  Landing.

For airplanes (except skiplanes for which landplane landing data have been

determined under this section and furnished in the Airplane Flight Manual),

the horizontal distance necessary to land and come to a complete stop (or to

a speed of approximately 3 knots for water landings of seaplanes and

amphibians) from a point 50 feet above the landing surface must be determined

as follows:

(a) A steady approach with a calibrated airspeed of not less than 1.3 VS1

must be maintained down to the 50-foot height and--

(1) The steady approach must be at a gradient of descent not greater than

5.2 percent (3 degrees) down to the 50-foot height.

(2) In addition, an applicant may demonstrate by tests that a maximum

steady approach gradient steeper than 5.2 percent, down to the 50-foot

height, is safe. The gradient must be established as an operating limitation

and the information necessary to display the gradient must be available to

the pilot by an appropriate instrument.

(b) The landing may not require more than average piloting skill when

landing during the atmospheric conditions expected to be encountered in

service, including crosswinds and turbulence.

(c) The landing must be made without excessive vertical acceleration or

tendency to bounce, nose over, ground loop, porpoise, or water loop.

(d) It must be shown that a safe transition to the balked landing

conditions of Sec. 23.77 can be made from the conditions that exist at the

50-foot height.

(e) The pressures on the wheel braking system may not exceed those

specified by the brake manufacturer.

(f) Means other than wheel brakes may be used if that means--

(1) Is safe and reliable;

(2) Is used so that consistent results can be expected in service; and

(3) Is such that no more than average skill is required to control the

airplane.

(g) If any device is used that depends on the operation of any engine, and

the landing distance would be increased when a landing is made with that

engine inoperative, the landing distance must be determined with that engine

inoperative unless the use of other compensating means will result in a

landing distance not more than that with each engine operating.

(h) In addition, for commuter category airplanes, the following apply:

(1) The landing distance must be determined for standard temperatures at

each weight, altitude, and wind condition within the operational limits

established by the applicant;

(2) A steady gliding approach, or a steady approach at a gradient of

descent not greater than 5.2 percent (3 deg.), at a calibrated airspeed not

less than 1.3VS1 must be maintained down to the 50-foot height; and

(3) The landing distance data must include correction factors for not more

than 50 percent of the nominal wind components along the landing path

opposite to the direction of landing and not less than 150 percent of the

nominal wind components along the landing path in the direction of landing.

[Amdt. 23-21, 43 FR 2318, Jan. 16, 1978, as amended by Amdt. 23-34, 52 FR

1828, Jan. 15, 1987; Amdt. 23-42, 56 FR 351, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.77  Balked landing.

(a) For balked landings, each normal, utility, and acrobatic category

airplane must be able to maintain a steady angle of climb at sea level of at

least 1:30 with--

(1) Takeoff power on each engine;

(2) The landing gear extended; and

(3) The wing flaps in the landing position, except that if the flaps may

safely be retracted in two seconds or less without loss of altitude and

without sudden changes of angle of attack or exceptional piloting skill, they

may be retracted.

(b) Each normal, utility, and acrobatic category turbine engine-powered

airplane must be able to maintain a steady rate of climb of at least zero at

a pressure altitude of 5,000 feet at 81 degrees F (standard temperature plus

40 degrees F), with the airplanes in the configuration prescribed in

paragraph (a) of this section.

(c) For each commuter category airplane, with all engines operating, the

maximum weight must be determined with the airplane in the landing

configuration for each altitude and ambient temperature within the

operational limits established for the airplane, with the most unfavorable

center of gravity and out-of-ground effect in free air, at which the steady

gradient of climb will not be less than 3.3 percent with--

(1) The engines at the power or thrust that is available 8 seconds after

initiation of movement of the power or thrust controls from the minimum

flight-idle position to the takeoff position.

(2) A climb speed not greater than the approach speed established under

Sec. 23.75 and not less than the greater of 1.05 VMC or 1.10VS1.

[Amdt. 23-21, 43 FR 2318, Jan. 16, 1978, as amended by Amdt. 23-34, 52 FR

1828, Jan. 15, 1987; Amdt. 23-24, 52 FR 34745, Sept. 14, 1987]

Flight Characteristics

Sec. 23.141  General.

The airplane must meet the requirements of Secs. 23.143 through 23.253 at

all practical loading conditions and operating altitudes for which

certification has been requested, not exceeding the maximum operating

altitude established under Sec. 23.1527, and without requiring exceptional

piloting skill, alertness, or strength.

[Amdt. 23-45, 58 FR 42156, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Controllability and Maneuverability

Sec. 23.143  General.

(a) The airplane must be safely controllable and maneuverable during--

(1) Takeoff;

(2) Climb;

(3) Level flight;

(4) Descent; and

(5) Landing (power on and power off with the wing flaps extended and

retracted).

(b) It must be possible to make a smooth transition from one flight

condition to another (including turns and slips) without danger of exceeding

the limit load factor, under any probable operating condition (including, for

multiengine airplanes, those conditions normally encountered in the sudden

failure of any engine).

(c) If marginal conditions exist with regard to required pilot strength,

the "strength of pilots" limits must be shown by quantitative tests. In no

case may the limits exceed those prescribed in the following table:

Values in pounds of force as

applied to the stick, control

wheel, or rudder pedals      Pitch  Roll  Yaw

(a) For temporary application:

Stick                             60    30

Wheel (Two hands on rim)          75    60

Wheel (One hand on rim)           50

Rudder Pedal                                150

(b) For prolonged application      10     5   20

[Doc. No, 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31819, Nov. 19, 1973; Amdt. 23-17, 41 FR 55464, Dec. 20, 1976; Amdt. 23-45,

58 FR 42157, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.145  Longitudinal control.

(a) With the airplane as nearly as possible in trim at 1.3 VS1, it must be

possible, at speeds below the trim speed, to pitch the nose downward so that

the rate of increase in airspeed allows prompt acceleration to the trim speed

with--

(1) Maximum continuous power on each engine;

(2) Power off; and

(3) Wing flap and landing gear--

(i) retracted, and

(ii) extended.

(b) No change in trim or exertion of more control force, as specified in

Sec. 23.143(c), than can be readily applied with one hand for a short period

of time may be required for the following maneuvers:

(1) With the landing gear extended, the flaps retracted, and the airplanes

as nearly as possible in trim at 1.4 VS1, extend the flaps as rapidly as

possible and allow the airspeed to transition from 1.4 VS1 to 1.4 VSO:

(i) With power off; and

(ii) With the power necessary to maintain level flight in the initial

condition.

(2) With the landing gear and flaps extended--

(i) With power off and the airplane as nearly as possible in trim at 1.3

VSO, quickly apply takeoff power or thrust and retract the flaps as rapidly

as possible to the recommended go-around setting while attaining and

maintaining, as a minimum, the speed used to show compliance with Sec. 23.77.

Retract the gear when positive rate of climb is established; and

(ii) With power off and in level flight at 1.1VSO, and the airplane as

nearly as possible in trim at 1.2 VSO, it must be possible to maintain

approximately level flight while retracting the flaps as rapidly as possible

with simultaneous application of not more than maximum continuous power. If

gated flap positions are provided, the airplane may be retrimmed between each

stage of retraction, and the airplane may accelerate to a speed that is 1.1

times the minimum steady flight speed obtained for the flap gate position.

(3) With maximum takeoff power, landing gear retracted, flaps in the

takeoff position, and the airplane as nearly as possible in trim at VFE,

appropriate to the takeoff flap position, retract the flaps as rapidly as

possible while maintaining constant speed.

(4) With power off, flaps and landing gear retracted, and the airplane as

nearly as possible trim at 1.4 VS, apply takeoff power rapidly while

maintaining the same airspeed.

(5) With power off, landing gear and flaps extended, and the airplane as

nearly as possible in trim at 1.4 VSO, obtain and maintain airspeeds between

1.1 VSO and either 1.7 VSO or VFE, whichever is lower.

(c) At speeds above VMO/MMO and up to VD/MD, a maneuvering capability of

1.5 g must be demonstrated to provide a margin to recover from upset or

inadvertent speed increase.

(d) It must be possible, with a pilot control force of not more than 10

pounds, to maintain a speed of not more than 1.3 VSO, during a power-off

glide with landing gear and wing flaps extended, for any weight of the

airplane, up to and including the maximum weight.

(e) By using normal flight and power controls, except as otherwise noted in

paragraphs (e)(1) and (e)(2) of this section, it must be possible to

establish a zero rate of descent at an attitude suitable for a controlled

landing without exceeding the operational and structural limitations of the

airplane, as follows:

(1) For single-engine and multiengine airplanes, without the use of the

primary longitudinal control system.

(2) For multiengine airplanes--

(i) Without the use of the primary directional control; and

(ii) If a single failure of any one connecting or transmitting link would

affect both the longitudinal and directional primary control system, without

the primary longitudinal and directional control system.

[Amdt. 23-45, 58 FR 42157, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.147   Directional and lateral control.

For each multiengine airplane, it must be possible, while holding the wings

level within 5 degrees, to make sudden changes in heading safely in both

directions. This must be shown at 1.4 VS1 with heading changes up to 15

degrees (except that the heading change at which the rudder force corresponds

to the limits specified in Sec. 23.143 need not be exceeded), with the--

(a) Critical engine inoperative and its propeller in the minimum drag

position;

(b) Remaining engines at maximum continuous power;

(c) Landing gear--

(1) Retracted; and

(2) Extended; and

(d) Flaps in the most favorable climb position.

[Amdt. 23-45, 58 FR 42157, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.149   Minimum control speed.

(a) VMC is the calibrated airspeed at which, when the critical engine is

suddenly made inoperative, it is possible to maintain control of the airplane

with that engine still inoperative and then maintain straight flight at the

same speed with an angle of bank of not more than 5 degrees. The ability to

maintain straight and level flight at VMC in a static condition with a bank

angle of not more than 5 degrees must also be demonstrated. The method used

to simulate critical engine failure must represent the most critical mode of

powerplant failure, with respect to controllability expected in service.

(b) VMC may not exceed 1.2 VS1, where VS1 is determined at the maximum

takeoff weight, with--

(1) Maximum available takeoff power or thrust on the engines;

(2) The most unfavorable center of gravity;

(3) The airplane trimmed for takeoff;

(4) The maximum sea level takeoff weight, or any lesser weight necessary to

show VMC;

(5) The airplane in the most critical takeoff configuration, with the

propeller controls in the recommended takeoff position and the landing gear

retracted; and

(6) The airplane airborne and the ground effect negligible.

(c) A minimum speed to intentionally render the critical engine inoperative

must be established and designated as the safe, intentional, one-engine-

inoperative speed, VSSE.

(d) At Vmc, the rudder pedal force required to maintain control may not

exceed 150 pounds, and it may not be necessary to reduce power or thrust of

the operative engines. During the maneuver, the airplane may not assume any

dangerous attitude and it must be possible to prevent a heading change of

more than 20 degrees.

[Amdt. 23-21, 43 FR 2318, Jan. 16, 1978, as amended by Amdt. No. 23-45, 58

FR 42157, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.151  Acrobatic maneuvers.

Each acrobatic and utility category airplane must be able to perform safely

the acrobatic maneuvers for which certification is requested. Safe entry

speeds for these maneuvers must be determined.

Sec. 23.153   Control during landings.

It must be possible, while in the landing configuration, to safely complete

a landing without exceeding the one hand control force specified in Sec.

23.143(c) following an approach to land--

(a) At a speed 5 knots less than the speed used in complying with the

requirements of Sec. 23.75 and with the airplane in trim, or as nearly as

possible in trim, and without the trimming control being moved throughout the

maneuver;

(b) At an approach gradient equal to the steepest recommended for

operational use; and

(c) With only those power or thrust changes that would be made when landing

normally from an approach at 1.3 VS1.

[Amdt. 23-45, 58 FR 42157, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.155  Elevator control force in maneuvers.

(a) The elevator control force needed to achieve the positive limit

maneuvering load factor may not be less than:

(1) For wheel controls, W/100 (where W is the maximum weight) or 20 pounds,

whichever is greater, except that it need not be greater than 50 pounds; or

(2) For stick controls, W/140 (where W is the maximum weight) or 15 pounds,

whichever is greater, except that it need not be greater than 35 pounds.

(b) The requirement of paragraph (a) of this section must be met at 75

percent of maximum continuous power for reciprocating engines, or the maximum

power or thrust selected by the applicant as an operating limitation for use

during cruise for reciprocating or turbine engines, and with the wing flaps

and landing gear retracted--

(1) In a turn, with the trim setting used for wings level flight at VA; and

(2) In a turn with the trim setting used for the maximum wings level flight

speed, except that the speed may not exceed VNE or VMO/MMO, whichever is

appropriate.

(c) Compliance with the requirements of this section may be demonstrated by

measuring the normal acceleration that is achieved with the limiting stick

force or by establishing the stick force per g gradient and extrapolating to

the appropriate limit.

[Amdt. 23-14, 38 FR 31819, Nov. 19, 1973; 38 FR 32784, Nov. 28, 1973, as

amended by Amdt. 23-45, 58 FR 42158, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.157  Rate of roll.

(a) Takeoff. It must be possible, using a favorable combination of

controls, to roll the airplane from a steady 30-degree banked turn through an

angle of 60 degrees, so as to reverse the direction of the turn within:

(1) For an airplane of 6,000 pounds or less maximum weight, 5 seconds from

initiation of roll; and

(2) For an airplane of over 6,000 pounds maximum weight,

(W+500)/1,300

seconds, but not more than 10 seconds, where W is the weight in pounds.

(b) The requirement of paragraph (a) of this section must be met when

rolling the airplane in each direction with--

(1) Flaps in the takeoff position;

(2) Landing gear retracted;

(3) For a single-engine airplane, at maximum takeoff power; and for a

multiengine airplane with the critical engine inoperative and the propeller

in the minimum drag position, and the other engines at maximum takeoff power;

and

(4) The airplane trimmed at a speed equal to the greater of 1.2 VS1 or 1.1

VMC, or as nearly as possible in trim for straight flight.

(c) Approach. It must be possible, using a favorable combination of

controls, to roll the airplane from a steady 30-degree banked turn through an

angle of 60 degrees, so as to reverse the direction of the turn within:

(1) For an airplane of 6,000 pounds or less maximum weight, 4 seconds from

initiation of roll; and

(2) For an airplane of over 6,000 pounds maximum weight,

(W+2,800)/2,200

seconds, but not more than 7 seconds, where W is the weight in pounds.

(d) The requirement of paragraph (c) must be met when rolling the airplane

in either direction in the following conditions:

(1) Flaps extended;

(2) Landing gear extended;

(3) All engines operating at idle power or thrust and with all engines

operating at the power or thrust for level flight; and

(4) The airplane trimmed at the speed that is used in determining

compliance with Sec. 23.75.

[Amdt. 23-14, 38 FR 31819, Nov. 19, 1973, as amended by Amdt. No. 23-45, 58

FR 42158, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Trim

Sec. 23.161  Trim.

(a) General. Each airplane must meet the trim requirements of this section

after being trimmed, and without further pressure upon, or movement of, the

primary controls or their corresponding trim controls by the pilot or the

automatic pilot.

(b) Lateral and directional trim. The airplane must maintain lateral and

directional trim in level flight with the landing gear and wing flaps

retracted as follows:

(1) For normal, utility, and acrobatic category airplanes at a speed of 0.9

VH, VC, or VM0, whichever is the lower; and

(2) For commuter category airplanes, at a speed of VH or VMO/MMO, whichever

is lower.

(c) Longitudinal trim. The airplane must maintain longitudinal trim under

each of the following conditions, except that it need not maintain trim at a

speed greater than VMO/MMO:

(1) A climb with maximum continuous power at--

(i) The speed used in determining the climb performance required by Sec.

23.65 of this part with the landing gear retracted, and the flaps in the

takeoff position; and

(ii) The recommended all-engines-operating climb speed specified in Sec.

23.1585(a)(2)(i) of this part.

(2) An approach at a gradient of descent of 5.2 percent (3 degrees) with

the landing gear extended, and with--

(i) Flaps retracted and at a speed of 1.4 VS1; and

(ii) The applicable airspeed and flap position used in showing compliance

with Sec. 23.75.

(3) Level flight at any speed with the landing gear and wing flaps

retracted as follows:

(i) For normal, utility, and acrobatic category airplanes, at any speeds

from the lesser of VH and VN0 or VM0, as applicable, to 1.4 VS1; and

(ii) For commuter category airplanes, at a speed of VH or VMO/MMO,

whichever is lower, to either VX or 1.4VS1.

(4) A descent at 0.9 VNO or 0.9 VMO, whichever is applicable, with power

off and with the landing gear and flaps retracted.

(d) In addition, each multiengine airplane must maintain longitudinal and

directional trim, and the lateral control force must not exceed 5 pounds, at

the speed used in complying with Sec. 23.67 for normal, utility, and

acrobatic categories and at a speed between VY and 1.4 VS1 for commuter

category with--

(1) The critical engine inoperative, and if applicable, its propeller in

the minimum drag position;

(2) The remaining engines at maximum continuous power;

(3) The landing gear retracted;

(4) Wing flaps in the position selected for showing compliance with Sec.

23.67 for normal, utility, and acrobatic category airplanes and wing flaps

retracted for commuter category airplanes.

(5) An angle of bank of not more than five degrees.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-21, 43 FR

2318, Jan. 16, 1978; Amdt. 23-34, 52 FR 1828, Jan. 15, 1987; Amdt. 23-42, 56

FR 351, Jan. 3, 1991; Amdt. 23-42, 56 FR 5455, Feb. 11, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Stability

Sec. 23.171  General.

The airplane must be longitudinally, directionally, and laterally stable

under Secs. 23.173 through 23.181. In addition, the airplane must show

suitable stability and control "feel" (static stability) in any condition

normally encountered in service, if flight tests show it is necessary for

safe operation.

Sec. 23.173  Static longitudinal stability.

Under the conditions specified in Sec. 23.175 and with the airplane trimmed

as indicated, the characteristics of the elevator control forces and the

friction within the control system must be as follows:

(a) A pull must be required to obtain and maintain speeds below the

specified trim speed and a push required to obtain and maintain speeds above

the specified trim speed. This must be shown at any speed that can be

obtained, except that speeds requiring a control force in excess of 40 pounds

or speeds above the maximum allowable speed or below the minimum speed for

steady unstalled flight, need not be considered.

(b) The airspeed must return to within the tolerances specified for

applicable categories of airplanes when the control force is slowly released

at any speed within the speed range specified in paragraph (a) of this

section. The applicable tolerances are--

(1) The airspeed must return to within plus or minus 10 percent of the

original trim airspeed; and

(2) For commuter category airplanes, the airspeed must return to within

plus or minus 7.5 percent of the original trim airspeed for the cruising

condition specified in Sec. 23.175(b).

(c) The stick force must vary with speed so that any substantial speed

change results in a stick force clearly perceptible to the pilot.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31820 Nov. 19, 1973; Amdt. 23-34, 52 FR 1828, Jan. 15, 1987]

Sec. 23.175  Demonstration of static longitudinal stability.

Static longitudinal stability must be shown as follows:

(a) Climb. The stick force curve must have a stable slope, at speeds

between 85 and 115 percent of the trim speed, with--

(1) Flaps in the climb position;

(2) Landing gear retracted;

(3) All reciprocating engines operating at maximum continuous power, or

turbine engines operating at the maximum power selected by the applicant as

an operating limitation for use during climb; and

(4) The airplane trimmed for VY, except that the speed need not be less

than 1.4 VS1.

(b) Cruise--Landing gear retracted (or fixed gear). (1) For the cruise

conditions specified in paragraphs (b) (2) and (3) of this section, the

following apply:

(i) The speed need not be less than 1.3 VS1.

(ii) For airplanes with VNE established under Sec. 23.1505(a), the speed

need not be greater than VNE.

(iii) For airplanes with VMO/MMO established under Sec. 23.1505(c), the

speed need not be greater than a speed midway between VMO/MMO and the lesser

of VD/MD or the speed demonstrated under Sec. 23.251, except that for

altitudes where Mach number in the limiting factor, the speed need not exceed

that corresponding to the Mach number at which effective speed warning

occurs.

(2) High speed cruise. The stick force curve must have a stable slope at

all speeds within a range that is the greater of 15 percent of the trim speed

plus the resulting free return speed range or 40 knots plus the resulting

free return speed range for normal, utility, and acrobatic category

airplanes, above and below the trim speed. For commuter category airplanes,

the stick force curve must have a stable slope for a speed range of 50 knots

from the trim speed, except that the speeds need not exceed VFC/MFC or be

less than 1.4 VS1 and this speed range is considered to begin at the outer

extremes of the friction band with a stick force not to exceed 50 pounds. In

addition, for commuter category airplanes, VFC/MFC may not be less than a

speed midway between VMO/MMO and VDF/MDF, except that, for altitudes where

Mach number is the limiting factor, MFC need not exceed the Mach number at

which effective speed warning occurs. These requirements for all categories

of airplane must be met with--

(i) Flaps retracted.

(ii) Seventy-five percent of maximum continuous power for reciprocating

engines or, for turbine engines, the maximum cruising power or thrust

selected by the applicant as an operating limitation, except that the power

need not exceed that required at VNE for airplanes with VNE established under

Sec. 23.1505(a), or that required at VMO/MMO for airplanes with VMO/MMO

established under Sec. 23.1505(c).

(iii) The airplane trimmed for level flight.

(3) Low speed cruise. The stick force curve must have a stable slope under

all the conditions prescribed in paragraph (b)(2) of this section, except

that the power is that required for level flight at a speed midway between

1.3 VS1 and the trim speed obtained in the high speed cruise condition under

paragraph (b)(2) of this section.

(c) Landing gear extended (airplanes with retractable gear). The stick

force curve must have a stable slope at all speeds within a range from 15

percent of the trim speed plus the resulting free return speed range below

the trim speed, to the trim speed (except that the speed range need not

include speeds less than 1.4 VS1 nor speeds greater than VLE, with--

(1) Landing gear extended;

(2) Flaps retracted;

(3) 75 percent of maximum continuous power for reciprocating engines, or

for turbine engines, the maximum cruising power or thrust selected by the

applicant as an operating limitation, except that the power need not exceed

that required for level flight at VLE; and

(4) The airplane trimmed for level flight.

(d) Approach and landing. The stick force curve must have a stable slope at

speeds between 1.1 VS1 and 1.8 VS1 with--

(1) Wing flaps in the landing position;

(2) Landing gear extended;

(3) The airplane trimmed at a speed in compliance with Sec. 23.161(c)(2).

(4) Both power off and enough power to maintain a 3 deg. angle of descent.

[Amdt. 23-7, 34 FR 13087, Aug. 13, 1969, as amended by Amdt. 23-14, 38 FR

31820, Nov. 19, 1973; Amdt. 23-17, 41 FR 55464, Dec. 20, 1976; Amdt. 23-34,

52 FR 1828, Jan. 15, 1987; Amdt. 23-45, 58 FR 42158, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.177  Static directional and lateral stability.

(a) Three-control airplanes. The stability requirements for three-control

airplanes are as follows:

(1) The static directional stability, as shown by the tendency to recover

from a skid with the rudder free, must be positive for any landing gear and

flap position appropriate to the takeoff, climb, cruise, approach, and

landing configurations. This must be shown with symmetrical power up to

maximum continuous power, and at speeds from 1.2 VS1 up to the maximum

allowable speed for the condition being investigated in the takeoff, climb,

cruise, and approach configurations. For the landing configuration, the power

must be up to that necessary to maintain a three degree angle of descent in

coordinated flight. The angle of sideslip for these tests must be appropriate

to the type of airplane. At larger angles of sideslip, up to that at which

full rudder is used or a control force limit in Sec. 23.143 is reached,

whichever occurs first, and at speeds from 1.2 VS1 to VA, the rudder pedal

force must not reverse.

(2) The static lateral stability, as shown by the tendency to raise the low

wing in a sideslip, must be positive for any landing gear and flap position.

This must be shown with symmetrical power, up to 75 percent of maximum

continuous power, at speeds above 1.2 VS1 in the takeoff configuration and

1.3 VS1 in other configurations, up to the maximum allowable speed for the

configuration being investigated in the takeoff, climb, approach, and cruise

configurations. For the landing configuration, the power must be up to that

necessary to maintain a three degree angle of descent in coordinated flight.

The angle of bank for these tests must be appropriate to the type of airplane

but in no case may the constant heading sideslip angle be less than that

obtainable with 10 deg. bank, or, if less, the maximum bank angle obtainable

with full rudder deflection or 150 pounds rudder force. The static lateral

stability must not be negative at 1.2 VS1.

(3) In straight, steady slips at 1.2 VS1 for any landing gear and flap

positions, and for any symmetrical power conditions up to 50 percent of

maximum continuous power, the aileron and rudder control movements and forces

must increase steadily, but not necessarily in constant proportion, as the

angle of slip is increased up to the maximum appropriate to the type of

airplane. At larger slip angles, up to the angle at which full rudder or

aileron control is used or a control force limit contained in Sec. 23.143 is

obtained, the aileron and rudder control movements and forces must not

reverse as the angle of sideslip is increased. Enough bank must accompany the

sideslip to hold a constant heading. Rapid entry into, and recovery from, a

maximum sideslip considered appropriate for the airplane must not result in

uncontrollable flight characteristics.

(b) Two-control (or simplified control) airplanes. The stability

requirements for two-control airplanes are as follows:

(1) The directional stability of the airplane must be shown by showing

that, in each configuration, it can be rapidly rolled from a 45 degree bank

in one direction to a 45 degree bank in the opposite direction without

showing dangerous skid characteristics.

(2) The lateral stability of the airplane must be shown by showing that it

will not assume a dangerous attitude or speed when the controls are abandoned

for two minutes. This must be done in moderately smooth air with the airplane

trimmed for straight level flight at 0.9 VH or VC, whichever is lower, with

flaps and landing gear retracted, and with a rearward center of gravity.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan, 9, 1965, as

amended by Amdt. 23-21, 43 FR 2318, Jan. 16, 1978; Amdt. No. 23-45, 58 FR

42158, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.179  [Removed. Amdt. No. 23-45, 58 FR 42158, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.181   Dynamic stability.

(a) Any short period oscillation not including combined lateral-directional

oscillations occurring between the stalling speed and the maximum allowable

speed appropriate to the configuration of the airplane must be heavily damped

with the primary controls--

(1) Free; and

(2) In a fixed position.

(b) Any combined lateral-directional oscillations ("Dutch roll") occurring

between the stalling speed and the maximum allowable speed appropriate to the

configuration of the airplane must be damped to 1/10 amplitude in 7 cycles

with the primary controls--

(1) Free; and

(2) In a fixed position.

(c) If it is determined that the function of a stability augmentation

system, reference Sec. 23.672, is needed to meet the flight characteristic

requirements of this part, the primary control requirements of paragraphs

(a)(2) and (b)(2) of this section are not applicable to the tests needed to

verify the acceptability of that system.

(d) During the conditions as specified in Sec. 23.175, when the

longitudinal control force required to maintain speeds differing from the

trim speed by at least plus and minus 15 percent is suddenly released, the

response of the airplane must not exhibit any dangerous characteristics nor

be excessive in relation to the magnitude of the control force released. Any

long-period oscillation of flight path, phugoid oscillation, that results

must not be so unstable as to increase the pilot's workload or otherwise

endanger the airplane.

[Amdt. 23-21, 43 FR 2318, Jan. 16, 1978, as amended by Amdt. No. 23-45, 58

FR 42158, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Stalls

Sec. 23.201  Wings level stall.

(a) For an airplane with independently controlled roll and directional

controls, it must be possible to produce and to correct roll by unreversed

use of the rolling control and to produce and to correct yaw by unreversed

use of the directional control, up to the time the airplane pitches.

(b) For an airplane with interconnected lateral and directional controls (2

controls) and for an airplane with only one of these controls, it must be

possible to produce and correct roll by unreversed use of the rolling control

without producing excessive yaw, up to the time the airplane pitches.

(c) The wings level stall characteristics must be demonstrated in flight as

follows: Starting from a speed above the stall warning speed, the elevator

control must be pulled back so that the rate of speed reduction will not

exceed one knot per second until a stall is produced, as shown by an

uncontrollable downward pitching motion of the airplane, until the control

reaches the stop or until the activation of an artificial stall barrier, for

example, stick pusher. Normal use of the elevator control for recovery is

allowed after the pitching motion has unmistakably developed or after the

control has been held against the stop for not less than two seconds. In

addition, engine power may not be increased for recovery until the speed has

increased to approximately 1.2 VS1.

(d) Except where made inapplicable by the special features of a particular

type of airplane, the following apply to the measurement of loss of altitude

during a stall:

(1) The loss of altitude encountered in the stall (power on or power off)

is the change in altitude (as observed on the sensitive altimeter testing

installation) between the altitude at which the airplane pitches and the

altitude at which horizontal flight is regained.

(2) If power is required during stall recovery, the power used must be that

used under the normal operating procedures selected by the applicant for this

maneuver; however, the power used to regain level flight may not be increased

until the speed has increased to approximately 1.2 VS1.

(e) During the recovery part of the maneuver, it must be possible to

prevent more than 15 degrees of roll or yaw by the normal use of controls.

(f) Compliance with the requirements of this section must be shown under

the following conditions:

(1) Wing flaps: Full up, full down, and intermediate, if appropriate.

(2) Landing gear: Retracted and extended.

(3) Cowl flaps: Appropriate to configuration.

(4) Power: Power off, and 75 percent maximum continuous power. If the

power-to-weight ratio at 75 percent continuous power provides undesirable

stall characteristics at extremely nose-high attitudes, the test may be

accomplished with the power or thrust required for level flight in the

landing configuration at maximum landing weight and a speed of 1.4 VSO, but

the power may not be less than 50 percent of maximum continuous power.

(5) Trim:  The airplane trimmed at a speed as near 1.5 VS1 as practicable.

(6) Propeller: Full increase rpm position for the power off condition.

[Amdt. 23-14, 38 FR 31820, Nov. 19, 1973, as amended by Amdt. 23-45, 58 FR

42158, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.203  Turning flight and accelerated stalls.

Turning flight and accelerated stalls must be demonstrated in flight tests

as follows:

(a) Establish and maintain a coordinated turn in a 30 degree bank. Reduce

speed by steadily and progressively tightening the turn with the elevator

until the airplane is stalled or until the elevator has reached its stop. The

rate of speed reduction must be constant, and:

(1) For a turning flight stall, may not exceed one knot per second; and

(2) For an accelerated stall, be 3 to 5 knots per second with steadily

increasing normal acceleration.

(b) When the stall has fully developed or the elevator has reached its

stop, it must be possible to regain wings level flight by normal use of the

flight controls but without increasing power, and without--

(1) Excessive loss of altitude;

(2) Undue pitchup;

(3) Uncontrollable tendency to spin;

(4) Exceeding a bank angle of 60 degrees in the original direction of the

turn or 30 degrees in the opposite direction in the case of turning flight

stalls, and without exceeding a bank angle of 90 degrees in the original

direction of the turn or 60 degrees in the opposite direction in the case of

accelerated stalls; and

(5) Exceeding the maximum permissible speed or allowable load factor.

(c) Compliance with the requirements of this section must be shown with:

(1) Wing Flaps:  Retracted, fully extended, and in each intermediate

position, as appropriate.

(2) Landing gear: Retracted and extended;

(3) Cowl flaps: Appropriate to configuration;

(4) Power: Power or thrust off, and 75 percent maximum continuous power or

thrust. If the power-to-weight ratio at 75 percent continuous power or thrust

provides undesirable stall characteristics at extremely nose-high attitudes,

the test may be accomplished with the power or thrust required for level

flight in the landing configuration at maximum landing weight and a speed of

1.4 VS0, but the power may not be less than 50 percent of maximum continuous

power.

(5) Trim: The airplane trimmed at a speed as near 1.5 VS1 as practicable.

[Amdt. 23-14, 38 FR 31820, Nov. 19, 1973, as amended by Amdt. No. 23-45, 58

FR 42159, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.205  Critical engine inoperative stalls.

(a) A multiengine airplane may not display any undue spinning tendency and

must be safely recoverable without applying power to the inoperative engine

when stalled. The operating engines may be throttled back during the recovery

from stall.

(b) Compliance with paragraph (a) of the section must be shown with:

(1) Wing flaps: Retracted and set to the position used to show compliance

with Sec. 23.67.

(2) Landing gear: Retracted.

(3) Cowl flaps: Appropriate to level flight critical engine inoperative.

(4) Power: Critical engine inoperative and the remaining engine(s) at 75

percent maximum continuous power or thrust or the power or thrust at which

the use of maximum control travel just holds the wings laterally level in the

approach to stall, whichever is lesser.

(5) Propeller: Normal inoperative position for the inoperative engine.

(6) Trim: Level flight, critical engine inoperative, except that for an

airplane of 6,000 pounds or less maximum weight that has a stalling speed of

61 knots or less and cannot maintain level flight with the critical engine

inoperative, the airplane must be trimmed for straight flight, critical

engine inoperative, at a speed as near 1.5 VS1 as practicable.

[Amdt. 23-14, 38 FR 31820, Nov. 19, 1973, as amended by Amdt. No. 23-45, 58

FR 42159, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.207  Stall warning.

(a) There must be a clear and distinctive stall warning, with the flaps and

landing gear in any normal position, in straight and turning flight.

(b) The stall warning may be furnished either through the inherent

aerodynamic qualities of the airplane or by a device that will give clearly

distinguishable indications under expected conditions of flight. However, a

visual stall warning device that requires the attention of the crew within

the cockpit is not acceptable by itself.

(c) For the stall tests required by Sec. 23.201(c), the stall warning must

begin at a speed exceeding the stalling speed by a margin of not less than 5

knots, but not more than the greater of 10 knots or 15 percent of the

stalling speed, and must continue until the stall occurs.

(d) For all other stall tests, the stall warning must begin at not less

than 5 knots above the stall speed and be sufficiently in advance of the

stall for the stall to be averted by action after the stall warning first

occurs. In addition, when following the procedures of Sec. 23.1585, the stall

warning must not operate during a normal takeoff, a takeoff continued with

one engine inoperative or approach to landing.

[Amdt. 23-7, 34 FR 13087, Aug. 13, 1969, as amended by Amdt. 23-45, 58 FR

42159, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight airworthiness standards

for normal, utility, acrobatic, and commuter category airplanes. The changes

are based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, in St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Spinning

Sec. 23.221  Spinning.

(a) Normal category. Except as provided in paragraph (d) of this section, a

single-engine, normal category airplane must demonstrate compliance with

either the one-turn spin or the spin-resistant requirements of this

paragraph.

(1) One-turn spin. The airplane must recover from a one-turn spin or a

three-second spin, whichever takes longer, in not more than one additional

turn after the controls have been applied for recovery. In addition--

(i) For both the flaps-retracted and flaps-extended conditions, the

applicable airspeed limit and positive limit maneuvering load factor must not

be exceeded;

(ii) There must be no excessive back pressure during the spin or recovery;

(iii) It must be impossible to obtain unrecoverable spins with any use of

the flight or engine power controls either at the entry into or during the

spin; and

(iv) For the flaps-extended condition, the flaps may be retracted during

the recovery, but not before rotation has ceased.

(2) Spin resistant. The airplane must be demonstrated to be spin resistant

by the following:

(i) During the stall maneuvers contained in Sec. 23.201, the pitch control

must be pulled back and held against the stop. Then, using ailerons and

rudders in the proper direction, it must be possible to maintain wings-level

flight within 15 degrees of bank and to roll the airplane from a 30-degree

bank in one direction to a 30-degree bank in the other direction;

(ii) Reduce the airplane speed using pitch control at a rate of

approximately 1 knot per second until the pitch control reaches the stop;

then with the pitch control pulled back and held against the stop, apply full

rudder control in a manner to promote spin entry, for a period of 7 seconds

or through a 360-degree heading change, whichever occurs first. If the 360-

degree heading change is reached first, it must have taken no fewer than 4

seconds. This maneuver must be performed first with the ailerons in the

neutral position, and then with the ailerons deflected opposite the direction

of turn in the most adverse manner. Power or thrust and airplane

configuration must be set in accordance with Sec. 23.201(f) without change

during the maneuver. At the end of 7 seconds or a 360 degree heading change,

the airplane must respond immediately and normally to primary flight controls

applied to regain coordinated, unstalled flight without reversal of control

effect and without exceeding the temporary control forces specified by Sec.

23.143(c); and

(iii) Compliance with Secs. 23.201 and 23.203 must be demonstrated with the

airplane in uncoordinated flight, corresponding to one ball width

displacement on a slip-skid indicator, unless one ball width displacement

cannot be obtained with full rudder, in which case the demonstration must be

with full rudder applied.

(b) Utility category. A utility category airplane must meet the

requirements of paragraph (a) of this section or the requirements of

paragraph (c) of this section if approval for spinning is requested.

(c) Acrobatic category. An acrobatic category airplane must meet the

following requirements:

(1) The airplane must recover from any point in a spin, in not more than

one and one-half additional turns after normal recovery application of the

controls. Prior to normal recovery application of the controls, the spin test

must proceed for six turns or 3 seconds, whichever takes longer, with flaps

retracted, and one turn or 3 seconds, whichever takes longer, with flaps

extended. However, beyond 3 seconds, the spin may be discontinued when spiral

characteristics appear with flaps retracted.

(2) For both the flaps-retracted and flaps-extended conditions, the

applicable airspeed limit and positive limit maneuvering load factor may not

be exceeded. For the flaps-extended condition, the flaps may be retracted

during recovery, if a placard is installed prohibiting intentional spins with

flaps extended.

(3) It must be impossible to obtain unrecoverable spins with any use of the

flight or engine power controls either at the entry into or during the spin.

(d) Airplanes "characteristically incapable of spinning". If it is desired

to designate an airplane as "characteristically incapable of spinning", this

characteristic must be shown with--

(1) A weight five percent more than the highest weight for which approval

is requested;

(2) A center of gravity at least three percent aft of the rearmost position

for which approval is requested;

(3) An available elevator up-travel four degrees in excess of that to which

the elevator travel is to be limited for approval; and

(4) An available rudder travel seven degrees, in both directions, in excess

of that to which the rudder travel is to be limited for approval.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13087, Aug. 13, 1969; Amdt. 23-42, 56 FR 352, Jan. 3, 1991; 56 FR 12584,

Mar. 26, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Ground and Water Handling Characteristics

Sec. 23.231  Longitudinal stability and control.

(a) A landplane may have no uncontrollable tendency to nose over in any

reasonably expected operating condition, including rebound during landing or

takeoff. Wheel brakes must operate smoothly and may not induce any undue

tendency to nose over.

(b) A seaplane or amphibian may not have dangerous or uncontrollable

porpoising characteristics at any normal operating speed on the water.

Sec. 23.233  Directional stability and control.

(a) A 90 degree cross-component of wind velocity, demonstrated to be safe

for taxiing, takeoff and landing must be established and must not be less

than 0.2 VSO.

(b) The airplane must be satisfactorily controllable in power-off landings

at normal landing speed, without using brakes or engine power to maintain a

straight path until the speed has decreased to at least 50 percent of the

speed at touchdown.

(c) The airplane must have adequate directional control during taxiing.

(d) Seaplanes must demonstrate satisfactory directional stability and

control for water operations up to the maximum wind velocity specified in

paragraph (a) of this section.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42159, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.235  Taxiing, takeoff, and landing condition.

(a) The airplane must be demonstrated to have satisfactory characteristics

and the shock-absorbing mechanism must not damage the structure of the

airplane when the airplane is taxied on the roughest ground that may be

reasonably expected in normal operation, and when takeoffs and landings are

performed on unpaved runways having the roughest surface that may reasonably

be expected in normal operation.

(b) A wave height, demonstrated to be safe for operation, and any necessary

water handling procedures for seaplanes and amphibians, must be established.

[Amdt. No. 23-45, 58 FR 42159, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.239  Spray characteristics.

Spray may not dangerously obscure the vision of the pilots or damage the

propellers or other parts of a seaplane or amphibian at any time during

taxiing, takeoff, and landing.

Miscellaneous Flight Requirements

Sec. 23.251  Vibration and buffeting.

There must be no vibration or buffeting severe enough to result in

structural damage, and each part of the airplane must be free from excessive

vibration, under any appropriate speed and power conditions up to VD/MD. In

addition, there must be no buffeting in any normal flight condition severe

enough to interfere with the satisfactory control of the airplane or cause

excessive fatigue to the flight crew. Stall warning buffeting within these

limits is allowable.

[Amdt. No. 23-45, 58 FR 42159, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.253  High speed characteristics.

If a maximum operating speed VMO/MMO is established under Sec. 23.1505(c),

the following speed increase and recovery characteristics must be met:

(a) Operating conditions and characteristics likely to cause inadvertent

speed increases (including upsets in pitch and roll) must be simulated with

the airplane trimmed at any likely speed up to VMO/MMO. These conditions and

characteristics include gust upsets, inadvertent control movements, low stick

force gradients in relation to control friction, passenger movement, leveling

off from climb, and descent from Mach to airspeed limit altitude.

(b) Allowing for pilot reaction time after occurrence of the effective

inherent or artificial speed warning specified in Sec. 23.1303, it must be

shown that the airplane can be recovered to a normal attitude and its speed

reduced to VMO/MMO, without--

(1) Exceptional piloting strength or skill;

(2) Exceeding VD/MD, the maximum speed shown under Sec. 23.251, or the

structural limitations; or

(3) Buffeting that would impair the pilot's ability to read the instruments

or to control the airplane for recovery.

(c) There may be no control reversal about any axis at any speed up to the

maximum speed shown under Sec. 23.251. Any reversal of elevator control force

or tendency of the airplane to pitch, roll, or yaw must be mild and readily

controllable, using normal piloting techniques.

[Amdt. 23-7, 34 FR 13087, Aug. 13, 1969; as amended by Amdt. 23-26, 45 FR

60170, Sept. 11, 1980; Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Subpart C--Structure

General

Sec. 23.301  Loads.

(a) Strength requirements are specified in terms of limit loads (the

maximum loads to be expected in service) and ultimate loads (limit loads

multiplied by prescribed factors of safety). Unless otherwise provided,

prescribed loads are limit loads.

(b) Unless otherwise provided, the air, ground, and water loads must be

placed in equilibrium with inertia forces, considering each item of mass in

the airplane. These loads must be distributed to conservatively approximate

or closely represent actual conditions. Methods used to determine load

intensities and distribution on canard and tandem wing configurations must be

validated by flight test measurement unless the methods used for determining

those loading conditions are shown to be reliable or conservative on the

configuration under consideration.

(c) If deflections under load would significantly change the distribution

of external or internal loads, this redistribution must be taken into

account.

(d) Simplified structural design criteria may be used if they result in

design loads not less than those prescribed in Secs. 23.331 through 23.521.

For conventional, single-engine airplanes with design weights of 6,000 pounds

or less, the design criteria of Appendix A of this part are an approved

equivalent of Secs. 23.321 through 23.459. If Appendix A is used, the entire

Appendix must be substituted for the corresponding sections of this part.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-28, 47 FR 13315, Mar. 29, 1982; Amdt. 23-42, 56 FR 352,

Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.302   Canard or tandem wing configurations.

The forward structure of a canard or tandem wing configuration must:

(a) Meet all requirements of subpart C and subpart D of this part

applicable to a wing; and

(b) Meet all requirements applicable to the function performed by these

surfaces.

[Doc. No. 25811, 56 FR 352, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.303  Factor of safety.

Unless otherwise provided, a factor of safety of 1.5 must be used.

Sec. 23.305  Strength and deformation.

(a) The structure must be able to support limit loads without detrimental,

permanent deformation. At any load up to limit loads, the deformation may not

interfere with safe operation.

(b) The structure must be able to support ultimate loads without failure

for at least three seconds, except local failures or structural instabilities

between limit and ultimate load are acceptable only if the structure can

sustain the required ultimate load for at least three seconds. However when

proof of strength is shown by dynamic tests simulating actual load

conditions, the three second limit does not apply.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.307  Proof of structure.

(a) Compliance with the strength and deformation requirements of Sec.

23.305 must be shown for each critical load condition. Structural analysis

may be used only if the structure conforms to those for which experience has

shown this method to be reliable. In other cases, substantiating load tests

must be made. Dynamic tests, including structural flight tests, are

acceptable if the design load conditions have been simulated.

(b) Certain parts of the structure must be tested as specified in Subpart D

of this part.

Flight Loads

Sec. 23.321  General.

(a) Flight load factors represent the ratio of the aerodynamic force

component (acting normal to the assumed longitudinal axis of the airplane) to

the weight of the airplane. A positive flight load factor is one in which the

aerodynamic force acts upward, with respect to the airplane.

(b) Compliance with the flight load requirements of this subpart must be

shown--

(1) At each critical altitude within the range in which the airplane may be

expected to operate;

(2) At each weight from the design minimum weight to the design maximum

weight; and

(3) For each required altitude and weight, for any practicable distribution

of disposable load within the operating limitations specified in Secs.

23.1583 through 23.1589.

(c) When significant, the effects of compressibility must be taken into

account.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.331  Symmetrical flight conditions.

(a) The appropriate balancing horizontal tail load must be accounted for in

a rational or conservative manner when determining the wing loads and linear

inertia loads corresponding to any of the symmetrical flight conditions

specified in Secs. 23.333 through 23.341.

(b) The incremental horizontal tail loads due to maneuvering and gusts must

be reacted by the angular inertia of the airplane in a rational or

conservative manner.

(c) Mutual influence of the aerodynamic surfaces must be taken into account

when determining flight loads.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-42, 56

FR 352, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.333  Flight envelope.

(a) General. Compliance with the strength requirements of this subpart must

be shown at any combination of airspeed and load factor on and within the

boundaries of a flight envelope (similar to the one in paragraph (d) of this

section) that represents the envelope of the flight loading conditions

specified by the maneuvering and gust criteria of paragraphs (b) and (c) of

this section respectively.

(b) Maneuvering envelope. Except where limited by maximum (static) lift

coefficients, the airplane is assumed to be subjected to symmetrical

maneuvers resulting in the following limit load factors:

(1) The positive maneuvering load factor specified in Sec. 23.337 at speeds

up to VD;

(2) The negative maneuvering load factor specified in Sec. 23.337 at VC;

and

(3) Factors varying linearly with speed from the specified value at VC to

0.0 at VD for the normal and commuter category, and --1.0 at VD for the

acrobatic and utility categories.

(c) Gust envelope. (1) The airplane is assumed to be subjected to

symmetrical vertical gusts in level flight. The resulting limit load factors

must correspond to the conditions determined as follows:

(i) Positive (up) and negative (down) gusts of 50 f.p.s. at VC must be

considered at altitudes between sea level and 20,000 feet. The gust velocity

may be reduced linearly from 50 f.p.s. at 20,000 feet to 25 f.p.s. at 50,000

feet.

(ii) Positive and negative gusts of 25 f.p.s. at VD must be considered at

altitudes between sea level and 20,000 feet. The gust velocity may be reduced

linearly from 25 f.p.s. at 20,000 feet to 12.5 f.p.s. at 50,000 feet.

(iii) In addition, for commuter category airplanes, positive (up) and

negative (down) rough air gusts of 66 f.p.s. at VB must be considered at

altitudes between sea level and 20,000 feet. The gust velocity may be reduced

linearly from 66 f.p.s. at 20,000 feet to 38 f.p.s. at 50,000 feet.

(2) The following assumptions must be made:

(i) The shape of the gust is--

Ude              2(Pi)s

U =  ----   (1 - cos  ----     )

2                25C

Where--

s =Distance penetrated into gust (ft.);

C =Mean geometric chord of wing (ft.); and

Ude =Derived gust velocity referred to in subparagraph (1) of this section.

(ii) Gust load factors vary linearly with speed between VC and VD .

(d) Flight envelope.

[ ...Illustration appears here... ]

Flight Envelope

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13087, Aug. 13, 1969; Amdt. 23-34, 52 FR 1829, Jan. 15, 1987]

Sec. 23.335  Design airspeeds.

Except as provided in paragraph (a) (4) of this section, the selected

design airspeeds are equivalent airspeeds (EAS).

(a) Design cruising speed, VC. For VC the following apply:

(1) VC (in knots) may not be less than--

(i) 33 W/S (for normal, utility, and commuter category airplanes); and

(ii) 36<radical>W/S (for acrobatic category airplanes).

(2) For values of W/S more than 20, the multiplying factors may be

decreased linearly with W/S to a value of 28.6 where W/S =100.

(3) VC need not be more than 0.9 VH at sea level.

(4) At altitudes where an MD is established, a cruising speed MC limited by

compressibility may be selected.

(b) Design dive speed VD. For VD, the following apply:

(1) VD/MD may not be less than 1.25 VC/MC; and

(2) With VC min, the required minimum design cruising speed, VD (in knots)

may not be less than--

(i) 1.40 Vc min (for normal and commuter category airplanes);

(ii) 1.50 VC min (for utility category airplanes); and

(iii) 1.55 VC min (for acrobatic category airplanes).

(3) For values of W/S more than 20, the multiplying factors in paragraph

(b)(2) of this section may be decreased linearly with W/S to a value of 1.35

where W/S=100.

(4) Compliance with paragraphs (b) (1) and (2) of this section need not be

shown if VD/MD is selected so that the minimum speed margin between VC/MC and

VD/MD is the greater of the following:

(i) The speed increase resulting when, from the initial condition of

stabilized flight at VC/MC,  the airplane is assumed to be upset, flown for

20 seconds along a flight path 7.5 deg. below the initial path, and then

pulled up with a load factor of 1.5 (0.5 g. acceleration increment). At least

75 percent maximum continuous power for reciprocating engines, and maximum

cruising power for turbines, or, if less, the power required for VC/MC for

both kinds of engines, must be assumed until the pullup is initiated, at

which point power reduction and pilot-controlled drag devices may be used.

(ii) Mach 0.05 (at altitudes where an MD is established).

(c) Design maneuvering speed VA. For VA,  the following applies:

(1) VA may not be less than VS<radical>n where--

(i) VS is a computed stalling speed with flaps retracted at the design

weight, normally based on the maximum airplane normal force coefficients,

CNA; and

(ii) n is the limit maneuvering load factor used in design

(2) The value of VA need not exceed the value of VC used in design.

(d) Design speed for maximum gust intensity, VB. For VB, the following

apply:

(1) VB may not be less than the speed determined by the intersection of the

line representing the maximum positive lift Cn max and the line representing

the rough air gust velocity on the gust V-n diagram, or <radical>(ng) VS1,

whichever is less, where:

(i) ng the positive airplane gust load factor due to gust, at speed VC (in

accordance with Sec. 23.341), and at the particular weight under

consideration; and

(ii) VS1 is the stalling speed with the flaps retracted at the particular

weight under consideration.

(2) VB need not be greater than VC.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13088, Aug. 13, 1969; Amdt. 23-16, 40 FR 2577, Jan. 14, 1975; Amdt. 23-34, 52

FR 1829, Jan. 15, 1987; Amdt. 23-24, 52 FR 34745, Sept. 14, 1987]

Sec. 23.337  Limit maneuvering load factors.

(a) The positive limit maneuvering load factor n may not be less than--

(1)

2.1+[24,000/(W+10,000)]

for normal and commuter category airplanes, except that n need not be more

than 3.8

(2) 4.4 for utility category airplanes; or

(3) 6.0 for acrobatic category airplanes.

(b) The negative limit maneuvering load factor may not be less than--

(1) 0.4 times the positive load factor for the normal utility and commuter

categories; or

(2) 0.5 times the positive load factor for the acrobatic category.

(c) Maneuvering load factors lower than those specified in this section may

be used if the airplane has design features that make it impossible to exceed

these values in flight.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13088, Aug. 13, 1969; Amdt. 23-34, 52 FR 1829, Jan. 15, 1987]

Sec. 23.341  Gust loads factors.

(a) The gust load for a canard or tandem wing configuration must be

computed using a rational analysis, considering the criteria of Sec.

23.333(c), to develop the gust loading on each lifting surface or may be

computed in accordance with paragraph (b) of this section provided that the

resulting net loads are shown to be conservative with respect to the gust

criteria of Sec. 23.333(c).

(b) In the absence of a more rational analysis for conventional

configurations, the gust load factors must be computed as follows:

KgUdeVa

n = 1 +  ------------

498(W/S)

Where--

Kg=0.88micro-g/5.3+micro-g=gust alleviation factor;

micro-g=2(W/S)/<rho>Cag=airplane mass ratio;

Ude=Derived gust velocities referred to in Sec. 23.333(c) (f.p.s.);

<rho>=Density of air (slugs/cu.ft.);

W/S =Wing loading (p.s.f.);

C =Mean geometric chord (ft.);

g =Acceleration due to gravity (ft./sec.**2)

V =Airplane equivalent speed (knots); and

a =Slope of the airplane normal force coefficient curve CNA per radian if the

gust loads are applied to the wings and horizontal tail surfaces

simultaneously by a rational method. The wing lift curve slope CL per

radian may be used when the gust load is applied to the wings only and

the horizontal tail gust loads are treated as a separate condition.

[Amdt. 23-7, 34 FR 13088, Aug. 13, 1969, as amended by Amdt. 23-42, 56 FR

352, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.345  High lift devices.

(a) If flaps or similar high lift devices to be used for takeoff, approach,

or landing are installed, the airplane, with the flaps fully deflected at VF,

is assumed to be subjected to symmetrical maneuvers and gusts resulting in

limit load factors within the range determined by--

(1) Maneuvering, to a positive limit load factor of 2.0; and

(2) Positive and negative gust of 25 feet per second acting normal to the

flight path in level flight.

(b) VF must be assumed to be not less than 1.4 VS or 1.8 VSF, whichever is

greater, where--

VS is the computed stalling speed with flaps retracted at the design weight;

and

VSF is the computed stalling speed with flaps fully extended at the design

weight.

However, if an automatic flap load limiting device is used, the airplane may

be designed for the critical combinations of airspeed and flap position

allowed by that device.

(c) In designing the flaps and supporting structures, the following must be

accounted for:

(1) A head-on gust having a velocity of 25 feet per second (EAS).

(2) The slipstream effects specified in Sec. 23.457(b).

(d) In determining external loads on the airplane as a whole, thrust,

slipstream, and pitching acceleration may be assumed to be zero.

(e) The requirements of Sec. 23.457, and this section may be complied with

separately or in combination.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13088, Aug. 13, 1969; Amdt. 23-23, 43 FR 50592, Oct. 30, 1978]

Sec. 23.347  Unsymmetrical flight conditions.

The airplane is assumed to be subjected to the unsymmetrical flight

conditions of Secs. 23.349 and 23.351. Unbalanced aerodynamic moments about

the center of gravity must be reacted in a rational or conservative manner,

considering the principal masses furnishing the reacting inertia forces.

Sec. 23.349  Rolling conditions.

The wing and wing bracing must be designed for the following loading

conditions:

(a) Unsymmetrical wing loads appropriate to the category. Unless the

following values result in unrealistic loads, the rolling accelerations may

be obtained by modifying the symmetrical flight conditions in Sec. 23.333(d)

as follows:

(1) For the acrobatic category, in conditions A and F, assume that 100

percent of the semispan wing airload acts on one side of the plane of

symmetry and 60 percent of this load acts on the other side.

(2) For normal, utility, and commuter categories, in Condition A, assume

that 100 percent of the semispan wing airload acts on one side of the

airplane, and 70 percent of this load acts on the other side. For airplanes

of more than 1,000 pounds design weight, the latter percentage may be

increased linearly with weight up through 75 percent at 12,500 pounds to the

maximum gross weight of the airplane.

(b) The loads resulting from the aileron deflections and speeds specified

in Sec. 23.455, in combination with an airplane load factor of at least two

thirds of the positive maneuvering load factor used for design. Unless the

following values result in unrealistic loads, the effect of aileron

displacement on wing torsion may be accounted for by adding the following

increment to the basic airfoil moment coefficient over the aileron portion of

the span in the critical condition determined in Sec. 23.333(d):

<Delta>cm=--0.01<delta>

where--

<Delta>cm is the moment coefficient increment; and

<delta> is the down aileron deflection in degrees in the critical condition.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13088, Aug. 13, 1969; Amdt. 23-34, 52 FR 1829, Jan. 15, 1987]

Sec. 23.351  Yawing conditions.

The airplane must be designed for yawing loads on the vertical surfaces

resulting from the loads specified in Secs. 23.441 through 23.445.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-42, 56

FR 352, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.361  Engine torque.

(a) Each engine mount and its supporting structure must be designed for the

effects of--

(1) A limit engine torque corresponding to takeoff power and propeller

speed acting simultaneously with 75 percent of the limit loads from flight

condition A of Sec. 23.333(d);

(2) A limit engine torque corresponding to maximum continuous power and

propeller spped acting simultaneously with the limit loads

from flight condition A of Sec. 23.333(d); and

(3) For turbopropeller installations, in addition to the conditions

specified in paragraphs (a)(1) and (a)(2) of this section, a limit engine

torque corresponding to takeoff power and propeller speed, multiplied by a

factor accounting for propeller control system malfunction, including quick

feathering, acting simultaneously with lg level flight loads. In the absence

of a rational analysis, a factor of 1.6 must be used.

(b) For turbine engine installations, the engine mounts and supporting

structure must be designed to withstand each of the following:

(1) A limit engine torque load imposed by sudden engine stoppage due to

malfunction or structural failure (such as compressor jamming).

(2) A limit engine torque load imposed by the maximum acceleration of the

engine.

(c) The limit engine torque to be considered under paragraph (a) of this

section must be obtained by multiplying the mean torque by a factor of--

(1) 1.25 for turbopropeller installations;

(2) 1.33 for engines with five or more cylinders; and

(3) Two, three, or four, for engines with four, three, or two cylinders,

respectively.

[Amdt. 23-26, 45 FR 60171, Sept. 11, 1980, as amended by Amdt. No. 23-45, 58

FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.363  Side load on engine mount.

(a) Each engine mount and its supporting structure must be designed for a

limit load factor in a lateral direction, for the side load on the engine

mount, of not less than--

(1) 1.33, or

(2) One-third of the limit load factor for flight condition A.

(b) The side load prescribed in paragraph (a) of this section may be

assumed to be independent of other flight conditions.

Sec. 23.365  Pressurized cabin loads.

For each pressurized compartment, the following apply:

(a) The airplane structure must be strong enough to withstand the flight

loads combined with pressure differential loads from zero up to the maximum

relief valve setting.

(b) The external pressure distribution in flight, and any stress

concentrations, must be accounted for.

(c) If landings may be made with the cabin pressurized, landing loads must

be combined with pressure differential loads from zero up to the maximum

allowed during landing.

(d) The airplane structure must be strong enough to withstand the pressure

differential loads corresponding to the maximum relief valve setting

multiplied by a factor of 1.33, omitting other loads.

(e) If a pressurized cabin has two or more compartments separated by

bulkheads or a floor, the primary structure must be designed for the effects

of sudden release of pressure in any compartment with external doors or

windows. This condition must be investigated for the effects of failure of

the largest opening in the compartment. The effects of intercompartmental

venting may be considered.

Sec. 23.367  Unsymmetrical loads due to engine failure.

(a) Turbopropeller airplanes must be designed for the unsymmetrical loads

resulting from the failure of the critical engine including the following

conditions in combination with a single malfunction of the propeller drag

limiting system, considering the probable pilot corrective action on the

flight controls:

(1) At speeds between VMC and VD, the loads resulting from power failure

because of fuel flow interruption are considered to be limit loads.

(2) At speeds between VMC and VC, the loads resulting from the

disconnection of the engine compressor from the turbine or from loss of the

turbine blades are considered to be ultimate loads.

(3) The time history of the thrust decay and drag buildup occurring as a

result of the prescribed engine failures must be substantiated by test or

other data applicable to the particular engine-propeller combination.

(4) The timing and magnitude of the probable pilot corrective action must

be conservatively estimated, considering the characteristics of the

particular engine-propeller-airplane combination.

(b) Pilot corrective action may be assumed to be initiated at the time

maximum yawing velocity is reached, but not earlier than 2 seconds after the

engine failure. The magnitude of the corrective action may be based on the

limit pilot forces specified in Sec. 23.397 except that lower forces may be

assumed where it is shown by analysis or test that these forces can control

the yaw and roll resulting from the prescribed engine failure conditions.

[Amdt. 23-7, 34 FR 13089, Aug. 13, 1969]

Sec. 23.369  Rear lift truss.

(a) If a rear lift truss is used, it must be designed for conditions of

reversed airflow at a design speed of--

V=8.7 <radical>W/S + 8.7 (knots)

(b) Either aerodynamic data for the particular wing section used, or a

value of CL equalling -0.8 with a chordwise distribution that is triangular

between a peak at the trailing edge and zero at the leading edge, must be

used.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13089, Aug. 13, 1969; 34 FR 17509, Oct. 30, 1969; Amdt. No. 23-45, 58 FR

42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.371  Gyroscopic and areodynamic loads.

For turbine-powered airplanes, each engine mount and its supporting

structure must be designed for the combined gyroscopic and aerodynamic

loads that result, with the

engines at maximum continuous r.p.m., under either of the following

conditions:

(a) The conditions prescribed in Secs. 23.351 and 23.423.

(b) All possible combinations of the following:

(1) A yaw velocity of 2.5 radians per second.

(2) A pitch velocity of 1 radian per second.

(3) A normal load factor of 2.5.

(4) Maximum continuous thrust.

[Amdt. 23-7, 34 FR 13089, Aug. 13, 1969, as amended by Amdt. 23-26, 45 FR

60171, Sept. 11, 1980; Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.373  Speed control devices.

If speed control devices (such as spoilers and drag flaps) are incorporated

for use in enroute conditions--

(a) The airplane must be designed for the symmetrical maneuvers and gusts

prescribed in Secs. 23.333, 23.337, and 23.341, and the yawing maneuvers and

lateral gusts in Secs. 23.441 and 23.443, with the device extended at speeds

up to the placard device extended speed; and

(b) If the device has automatic operating or load limiting features, the

airplane must be designed for the maneuver and gust conditions prescribed in

paragraph (a) of this section at the speeds and corresponding device

positions that the mechanism allows.

[Amdt. 23-7, 34 FR 13089, Aug. 13, 1969]

Control Surface and System Loads

Sec. 23.391  Control surface loads.

(a) The control surface loads specified in Secs. 23.397 through 23.459 are

assumed to occur in the conditions described in Secs. 23.331 through 23.351.

(b) If allowed by the following sections, the values of control surface

loading in Appendix B of this part may be used, instead of particular control

surface data, to determine the detailed rational requirements of Secs. 23.397

through 23.459, unless these values result in unrealistic loads.

Sec. 23.395  Control system loads.

(a) Each flight control system and its supporting structure must be

designed for loads corresponding to at least 125 percent of the computed

hinge moments of the movable control surface in the conditions prescribed in

Secs. 23.391 through 23.459. In addition, the following apply:

(1) The system limit loads need not exceed the higher of the loads that can

be produced by the pilot and automatic devices operating the controls.

However, autopilot forces need not be added to pilot forces. The system must

be designed for the maximum effort of the pilot or autopilot, whichever is

higher. In addition, if the pilot and the autopilot act in opposition, the

part of the system between them may be designed for the maximum effort of the

one that imposes the lesser load. Pilot forces used for design need not

exceed the maximum forces prescribed in Sec. 23.397(b).

(2) The design must, in any case, provide a rugged system for service use,

considering jamming, ground gusts, taxiing downwind, control inertia, and

friction. Compliance with this subparagraph may be shown by designing for

loads resulting from application of the minimum forces prescribed in Sec.

23.397(b).

(b) A 125 percent factor on computed hinge moments must be used to design

elevator, aileron, and rudder systems. However, a factor as low as 1.0 may be

used if hinge moments are based on accurate flight test data, the exact

reduction depending upon the accuracy and reliability of the data.

(c) Pilot forces used for design are assumed to act at the appropriate

control grips or pads as they would in flight, and to react at the

attachments of the control system to the control surface horns.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13089, Aug. 13, 1969]

Sec. 23.397  Limit control forces and torques.

(a) In the control surface flight loading condition, the airloads on

movable surfaces and the corresponding deflections need not exceed those that

would result in flight from the application of any pilot force within the

ranges specified in paragraph (b) of this section. In applying this

criterion, the effects of control system boost and servo-mechanisms, and the

effects of tabs must be considered. The automatic pilot effort must be used

for design if it alone can produce higher control surface loads than the

human pilot.

(b) The limit pilot forces and torques are as follows:

Maximum forces

or torques for

design weight,

weight equal to

or less than     Minimum forces

Control            5,000 pounds /1/   or torques /2/

Aileron:

Stick                      67 lbs            40 lbs.

Wheel /3/                  50 D in.-lbs /4/  40 D in.-lbs./4/

Elevator:

Stick                      167 lbs           100 lbs.

Wheel (symmetrical)        200 lbs           100 lbs.

Wheel (unsymmetrical) /5/                    100 lbs.

Rudder                      200 lbs           150 lbs.

/1/ For design weight (W) more than 5,000 pounds, the

specified maximum values must be increased linearly with

weight to 1.18 times the specified values at a design weight

of 12,500 pounds and for commuter category airplanes, the

specified values must be increased linearly with weight to

1.35 times the specified values at a design weight of 19,000

pounds.

/2/ If the design of any individual set of control systems or

surfaces makes these specified minimum forces or torques

inapplicable, values corresponding to the present hinge

moments obtained under Sec. 23.415, but not less than 0.6 of

the specified minimum forces or torques, may be used.

/3/ The critical parts of the aileron control system must also

be designed for a single tangential force with a limit value

of 1.25 times the couple force determined from the above

criteria.

/4/ D=wheel diameter (inches).

/5/ The unsymmetrical force must be applied at one of the

normal handgrip points on the control wheel.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13089, Aug. 13, 1969; Amdt. 23-17, 41 FR 55464, Dec. 20, 1976; Amdt. 23-34,

52 FR 1829, Jan. 15, 1987; Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.399  Dual control system.

Each dual control system must be designed for the pilots operating in

opposition, using individual pilot forces not less than--

(a) 0.75 times those obtained under Sec. 23.395; or

(b) The minimum forces specified in Sec. 23.397(b).

Sec. 23.405  Secondary control system.

Secondary controls, such as wheel brakes, spoilers, and tab controls, must

be designed for the maximum forces that a pilot is likely to apply to those

controls.

Sec. 23.407  Trim tab effects.

The effects of trim tabs on the control surface design conditions must be

accounted for only where the surface loads are limited by maximum pilot

effort. In these cases, the tabs are considered to be deflected in the

direction that would assist the pilot. These deflections must correspond to

the maximum degree of "out of trim" expected at the speed for the condition

under consideration.

Sec. 23.409  Tabs.

Control surface tabs must be designed for the most severe combination of

airspeed and tab deflection likely to be obtained within the flight envelope

for any usable loading condition.

Sec. 23.415  Ground gust conditions.

(a) The control system must be investigated as follows for control surface

loads due to ground gusts and taxiing downwind:

(1) If an investigation of the control system for ground gust loads is not

required by paragraph (a)(2) of this section, but the applicant elects to

design a part of the control system of these loads, these loads need only be

carried from control surface horns through the nearest stops or gust locks

and their supporting structures.

(2) If pilot forces less than the minimums specified in Sec. 23.397(b) are

used for design, the effects of surface loads due to ground gusts and taxiing

downwind must be investigated for the entire control system according to the

formula:

H=KcSq

where--

H =limit hinge moment (ft.-lbs.);

c =means chord of the control surface aft of the hinge line (ft.);

S =area of control surface aft of the hinge line (sq. ft.);

q =Dynamic pressure (p.s.f.) based on a design speed not less than

14.6<radical>W/S+14.6 (f.p.s.) except that the design speed need not

exceed 88 (f.p.s.); and

K =limit hinge moment factor for ground gusts derived in paragraph (b) of

this section. (For ailerons and elevators, a positive value of K

indicates a moment tending to depress the surface and a negative value of

K indicates a moment tending to raise the surface).

(b) The limit hinge moment factor K  for ground gusts must be derived as

follows:

Surface        K                      Position of controls

(a) Aileron      0.75  Control column locked lashed in mid-position.

(b) Aileron   +/-0.50  Ailerons at full throw; + moment on one aileron, -

moment on the other.

(c) Elevator  +/-0.75  (c) Elevator full up (-).

(d) Elevator           (d) Elevator full down (+).

(e) Rudder    +/-0.75  (e) Rudder in neutral.

(f) Rudder             (f) Rudder at full throw.

(c) The tie-down attachment fittings and the surrounding structure must be

designed for limit load conditions resulting from wind speeds up to 65 knots

horizontally from any direction for the weight determined to be critical for

tie-down.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13089, Aug. 13, 1969; Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Horizontal Stabilizing and Balancing Surfaces

Sec. 23.421  Balancing loads.

(a) A horizontal surface balancing load is a load necessary to maintain

equilibrium in any specified flight condition with no pitching acceleration.

(b) Horizontal balancing surfaces must be designed for the balancing loads

occurring at any point on the limit maneuvering envelope and in the flap

conditions specified in Sec. 23.345.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13089, Aug. 13, 1969; Amdt. 23-42, 56 FR 352, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.423  Maneuvering loads.

Each horizontal surface and its supporting structure, and the main wing of

a canard or tandem wing configuration, if that surface has pitch control,

must be designed for the maneuvering loads imposed by the following

conditions:

(a) A sudden movement of the pitching control, at the speed VA, to the

maximum aft movement, and the maximum forward movement, as limited by the

control stops, or pilot effort, whichever is critical.

(b) A sudden aft movement of the pitching control at speeds above VA,

followed by a forward movement of the pitching control resulting in the

following combinations of normal and angular acceleration:

Normal         Angular

acceleration    acceleration

Condition           (n)        (radian/sec**2)

Nose-up pitching    1.0           +39nm.Vx(nm-1.5)

Nose-down ptiching  nm            -39nm.Vx(nm-1.5)

where--

(1) nm=positive limit maneuvering load factor used in the design of the

airplane; and

(2) V=initial speed in knots.

The conditions in this paragraph involve loads corresponding to the loads

that may occur in a "checked maneuver" (a maneuver in which the pitching

control is suddenly displaced in one direction and then suddenly moved in the

opposite direction). The deflections and timing of the "checked maneuver"

must avoid exceeding the limit maneuvering load factor. The total horizontal

surface load for both nose-up and nose-down pitching conditions is the sum of

the balancing loads at V and the specified value of the normal load factor n,

plus the maneuvering load increment due to the specified value of the angular

acceleration.

[Doc. No. 25811, 56 FR 353, Jan. 3, 1991; Amdt. 23-42, 56 FR 5455, Feb. 11,

1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.425  Gust loads.

(a) Each horizontal surface, other than a main wing, must be designed for

loads resulting from--

(1) Gust velocities specified in Sec. 23.333(c) with flaps retracted; and

(2) Positive and negative gusts of 25 f.p.s. nominal intensity at VF

corresponding to the flight conditions specified in Sec. 23.345(a)(2).

(b) [Reserved]

(c) When determining the total load on the horizontal surfaces for the

conditions specified in paragraph (a) of this section, the initial balancing

loads for steady unaccelerated flight at the pertinent design speeds VF, VC,

and VD must first be determined. The incremental load resulting from the

gusts must be added to the initial balancing load to obtain the total load.

(d) In the absence of a more rational analysis, the incremental load due to

the gust must be computed as follows only on airplane configurations with

aft-mounted, horizontal surfaces, unless its use elsewhere is shown to be

conservative:

Kg Ude V<alpha>ht Sht         de

<Delta>Lht =  ---------------------    (1-  -- )

498                    da

where--

<Delta>Lht=Incremental horizontal tailload (lbs.);

Kg=Gust alleviation factor defined in Sec. 23.341;

Ude=Derived gust velocity (f.p.s.);

V=Airplane equivalent speed (knots);

<alpha>ht=Slope of aft horizontal lift curve (per radian);

Sht=Area of aft horizontal lift surface (ft**2); and

de

(1-  --  ) = Downwash factor

da

[Doc. No. 4080, 20 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13089 Aug. 13, 1969; Amdt. 23-42, 56 FR 353, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.427  Unsymmetrical loads.

(a) Horizontal surfaces other than main wing and their supporting

structure must be designed for unsymmetrical loads arising from yawing and

slipstream effects, in combination with the loads prescribed for the flight

conditions set forth in Secs. 23.421 through 23.425.

(b) In the absence of more rational data for airplanes that are

conventional in regard to location of engines, wings, horizontal surfaces

other than main wing, and fuselage shape:

(1) 100 percent of the maximum loading from the symmetrical flight

conditions may be assumed on the surface on one side of the plane of

symmetry; and

(2) The following percentage of that loading must be applied to the

opposite side:

Percent=100-10 (n-1), where n is the specified positive maneuvering load

factor, but this value may not be more than 80 percent.

(c) For airplanes that are not conventional (such as airplanes with

horizontal surfaces other than main wing having appreciable dihedral or

supported by the vertical tail surfaces) the surfaces and supporting

structures must be designed for combined vertical and horizontal surface

loads resulting from each prescribed flight condition taken separately.

[Amdt. 23-14, 38 FR 31820, Nov. 19, 1973, as amended by Amdt. 23-42, 56 FR

353, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Vertical Surfaces

Sec. 23.441  Maneuvering loads.

(a) At speeds up to VA, the vertical surfaces must be designed to

withstand the following conditions. In computing the loads, the yawing

velocity may be assumed to be zero:

(1) With the airplane in unaccelerated flight at zero yaw, it is assumed

that the rudder control is suddenly displaced to the maximum deflection, as

limited by the control stops or by limit pilot forces.

(2) With the rudder deflected as specified in paragraph (a)(1) of this

section, it is assumed that the airplane yaws to the resulting sideslip

angle. In lieu of a rational analysis, an overswing angle equal to 1.3 times

the static sideslip angle of paragraph (a)(3) of this section may be assumed.

(3) A yaw angle of 15 degrees with the rudder control maintained in the

neutral position (except as limited by pilot strength).

(b) [Reserved.]

(c) The yaw angles specified in paragraph (a)(3) of this section may be

reduced if the yaw angle chosen for a particular speed cannot be exceeded

in--

(1) Steady slip conditions;

(2) Uncoordinated rolls from steep banks; or

(3) Sudden failure of the critical engine with delayed corrective action.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13090, Aug. 13, 1969; Amdt. 23-14, 38 FR 31821, Nov. 19, 1973; Amdt. 23-28,

47 FR 13315, Mar. 29, 1982; Amdt. 23-42, 56 FR 353, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.443  Gust loads.

(a) Vertical surfaces must be designed to withstand, in unaccelerated

flight at speed VC, lateral gusts of the values prescribed for VC in Sec.

23.333(c).

(b) In addition, for commuter category airplanes, the airplane is assumed

to encounter derived gusts normal to the plane of symmetry while in

unaccelerated flight at VB, VC, VD, and VF. The derived gusts and airplane

speeds corresponding to these conditions, as determined by Secs. 23.341 and

23.345, must be investigated. The shape of the gust must be as specified in

Sec. 23.333(c)(2)(i).

(c) In the absence of a more rational analysis, the gust load must be

computed as follows:

Lvt=KgtUdeV<alpha>vtSvt

-----------------------

498

where--

Lvt=Vertical surface load (lbs.);

Kgt=0.88<micro>gt/5.3+<micro>gt=gust alleviation factor;

<micro>gt=2W/PCtg<alpha>vtSvt(K/1t)**2=lateral mass ratio;

Ude=Derived gust velocity (f.p.s.);

P=Air density (slugs/cu.ft.);

W =Airplane weight (lbs.);

Svt=Area of vertical surface (ft.**2);

Ct=Mean geometric chord of vertical surface (ft.);

<alpha>vt=Lift curve slope of vertical surface (per radian);

K =Radius of gyration in yaw (ft.);

1t=Distance from airplane c.g. to lift center of vertical surface (ft.);

g =Acceleration due to gravity (ft./sec.**2); and

V =Airplane equivalent speed (knots).

[Amdt. 23-7, 34 FR 13090, Aug. 13, 1969, as amended by Amdt. 23-34, 52 FR

1830, Jan. 15, 1987; 52 FR 7262, Mar. 9, 1987; Amdt. 23-24, 52 FR 34745,

Sept. 14, 1987; Amdt. 23-42, 56 FR 353, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.445  Outboard fins or winglets.

(a) If outboard fins or winglets are included on the horizontal surfaces or

wings, the horizontal surfaces or wings must be designed for their maximum

load in combination with loads induced by the fins or winglets and moments or

forces exerted on the horizontal surfaces or wings by the fins or winglets.

(b) If outboard fins or winglets extend above and below the horizontal

surface, the critical vertical surface loading (the load per unit area as

determined under Secs. 23.441 and 23.443) must be applied to--

(1) The part of the vertical surfaces above the horizontal surface with 80

percent of that loading applied to the part below the horizontal surface; and

(2) The part of the vertical surfaces below the horizontal surface with 80

percent of that loading applied to the part above the horizontal surface.

(c) The end plate effects of outboard fins or winglets must be taken

into account in applying the yawing conditions of Secs. 23.441 and 23.443 to

the vertical surfaces in paragraph (b) of this section.

(d) When rational methods are used for computing loads, the maneuvering

loads of Sec. 23.441 on the vertical surfaces and the one-g horizontal

surface load, including induced loads on the horizontal surface and moments

or forces exerted on the horizontal surfaces by the vertical surfaces, must

be applied simultaneously for the structural loading condition.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31821, Nov. 19, 1973; Amdt. 23-42, 56 FR 353, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Ailerons, Wing Flaps, and Special Devices

Sec. 23.455  Ailerons.

(a) The ailerons must be designed for the loads to which they are

subjected--

(1) In the neutral position during symmetrical flight conditions; and

(2) By the following deflections (except as limited by pilot effort),

during unsymmetrical flight conditions:

(i) Sudden maximum displacement of the aileron control at VA. Suitable

allowance may be made for control system deflections.

(ii) Sufficient deflection at VC, where VC is more than VA, to produce a

rate of roll not less than obtained in paragraph (a)(2)(i) of this section.

(iii) Sufficient deflection at VD to produce a rate of roll not less than

one-third of that obtained in paragraph (a)(2)(i) of this section.

(b) [Reserved]

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13090, Aug. 13, 1969; Amdt. 23-42, 56 FR 353, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.457  Wing flaps.

(a) The wing flaps, their operating mechanisms, and their supporting

structures must be designed for critical loads occurring in the flaps-

extended flight conditions with the flaps in any position. However, if an

automatic flap load limiting device is used, these components may be designed

for the critical combinations of airspeed and flap position allowed by that

device.

(b) The effects of propeller slipstream, corresponding to takeoff power,

must be taken into account at not less than 1.4 VS, where VS is the computed

stalling speed with flaps fully retracted at the design weight. For the

investigation of slipstream effects, the load factor may be assumed to be

1.0.

Sec. 23.459  Special devices.

The loading for special devices using aerodynamic surfaces (such as slots

and spoilers) must be determined from test data.

Ground Loads

Sec. 23.471  General.

The limit ground loads specified in this subpart are considered to be

external loads and inertia forces that act upon an airplane structure. In

each specified ground load condition, the external reactions must be placed

in equilibrium with the linear and angular inertia forces in a rational or

conservative manner.

Sec. 23.473  Ground load conditions and assumptions.

(a) The ground load requirements of this subpart must be complied with at

the design maximum weight except that Secs. 23.479, 23.481, and 23.483 may be

complied with at a design landing weight (the highest weight for landing

conditions at the maximum descent velocity) allowed under paragraphs (b) and

(c) of this section.

(b) The design landing weight may be as low as--

(1) 95 percent of the maximum weight if the minimum fuel capacity is enough

for at least one-half hour of operation at maximum continuous power plus a

capacity equal to a fuel weight which is the difference between the design

maximum weight and the design landing weight; or

(2) The design maximum weight less the weight of 25 percent of the total

fuel capacity.

(c) The design landing weight of a multiengine airplane may be less than

that allowed under paragraph (b) of this section if--

(1) The airplane meets the one-engine-inoperative climb requirements of

Sec. 23.67 (a) or (b)(1); and

(2) Compliance is shown with the fuel jettisoning system requirements of

Sec. 23.1001.

(d) The selected limit vertical inertia load factor at the center of

gravity of the airplane for the ground load conditions prescribed in this

subpart may not be less than that which would be obtained when landing with a

descent velocity (V), in feet per second, equal to 4.4 (W/S) 1/4, except that

this velocity need not be more than 10 feet per second and may not be less

than seven feet per second.

(e) Wing lift not exceeding two-thirds of the weight of the airplane may be

assumed to exist throughout the landing impact and to act through the center

of gravity. The ground reaction load factor may be equal to the inertia load

factor minus the ratio of the above assumed wing lift to the airplane weight.

(f) Energy absorption tests (to determine the limit load factor

corresponding to the required limit descent velocities) must be made under

Sec. 23.723(a) unless specifically exempted by that section.

(g) No inertia load factor used for design purposes may be less than 2.67,

nor may the limit ground reaction load factor be less than 2.0 at design

maximum weight, unless these lower values will not be exceeded in taxiing at

speeds up to takeoff speed over terrain as rough as that expected in service.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13090, Aug. 13, 1969; Amdt. 23-28, 47 FR 13315, Mar. 29, 1982; Amdt. No.

23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.477  Landing gear arrangement.

Sections 23.479 through 23.483, or the conditions in Appendix C, apply to

airplanes with conventional arrangements of main and nose gear, or main and

tail gear.

Sec. 23.479  Level landing conditions.

(a) For a level landing, the airplane is assumed to be in the following

attitudes:

(1) For airplanes with tail wheels, a normal level flight attitude.

(2) For airplanes with nose wheels, attitudes in which--

(i) The nose and main wheels contact the ground simultaneously; and

(ii) The main wheels contact the ground and the nose wheel is just clear of

the ground.

The attitude used in paragraph (a)(2)(i) of this section may be used in the

analysis required under paragraph (a)(2)(ii) of this section.

(b) When investigating landing conditions, the drag components simulating

the forces required to accelerate the tires and wheels up to the landing

speed (spin-up) must be properly combined with the corresponding

instantaneous vertical ground reactions, and the forward-acting horizontal

loads resulting from rapid reduction of the spin-up drag loads (spring-back)

must be combined with vertical ground reactions at the instant of the peak

forward load, assuming wing lift and a tire-sliding coefficient of friction

of 0.8. However, the drag loads may not be less than 25 percent of the

maximum vertical ground reactions (neglecting wing lift).

(c) In the absence of specific tests or a more rational analysis for

determining the wheel spin-up and spring-back loads for landing conditions,

the method set forth in appendix D of this part must be used. If appendix D

of this part is used, the drag components used for design must not be less

than those given by appendix C of this part.

(d) For airplanes with tip tanks or large overhung masses (such as turbo-

propeller or jet engines) supported by the wing, the tip tanks and the

structure supporting the tanks or overhung masses must be designed for the

effects of dynamic responses under the level landing conditions of either

paragraph (a)(1) or (a)(2)(ii) of this section. In evaluating the effects of

dynamic response, an airplane lift equal to the weight of the airplane may be

assumed.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55464, Dec. 20, 1976; Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.481  Tail down landing conditions.

(a) For a tail down landing, the airplane is assumed to be in the following

attitudes:

(1) For airplanes with tail wheels, an attitude in which the main and tail

wheels contact the ground simultaneously.

(2) For airplanes with nose wheels, a stalling attitude, or the maximum

angle allowing ground clearance by each part of the airplane, whichever is

less.

(b) For airplanes with either tail or nose wheels, ground reactions are

assumed to be vertical, with the wheels up to speed before the maximum

vertical load is attained.

Sec. 23.483  One-wheel landing conditions.

For the one-wheel landing condition, the airplane is assumed to be in the

level attitude and to contact the ground on one side of the main landing

gear. In this attitude, the ground reactions must be the same as those

obtained on that side under Sec. 23.479.

Sec. 23.485  Side load conditions.

(a) For the side load condition, the airplane is assumed to be in a level

attitude with only the main wheels contacting the ground and with the shock

absorbers and tires in their static positions.

(b) The limit vertical load factor must be 1.33, with the vertical ground

reaction divided equally between the main wheels.

(c) The limit side inertia factor must be 0.83, with the side ground

reaction divided between the main wheels so that--

(1) 0.5 (W) is acting inboard on one side; and

(2) 0.33 (W) is acting outboard on the other side.

(d) The side loads prescribed in paragraph (c) of this section are assumed

to be applied at the ground contact point and the drag loads may be assumed

to be zero.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.493  Braked roll conditions.

Under braked roll conditions, with the shock absorbers and tires in their

static positions, the following apply:

(a) The limit vertical load factor must be 1.33.

(b) The attitudes and ground contacts must be those described in Sec.

23.479 for level landings.

(c) A drag reaction equal to the vertical reaction at the wheel multiplied

by a coefficient of friction of 0.8 must be applied at the ground contact

point of each wheel with brakes, except that the drag reaction need not

exceed the maximum value based on limiting brake torque.

Sec. 23.497  Supplementary conditions for tail wheels.

In determining the ground loads on the tail wheel and affected supporting

structures, the following apply:

(a) For the obstruction load, the limit ground reaction obtained in the

tail down landing condition is assumed to act up and aft through the axle at

45 degrees. The shock absorber and tire may be assumed to be in their static

positions.

(b) For the side load, a limit vertical ground reaction equal to the static

load on the tail wheel, in combination with a side component of equal

magnitude, is assumed. In addition--

(1) If a swivel is used, the tail wheel is assumed to be swiveled 90

degrees to the airplane longitudinal axis with the resultant ground load

passing through the axle;

(2) If a lock, steering device, or shimmy damper is used, the tail wheel is

also assumed to be in the trailing position with the side load acting at the

ground contact point; and

(3) The shock absorber and tire are assumed to be in their static

positions.

Sec. 23.499  Supplementary conditions for nose wheels.

In determining the ground loads on nose wheels and affected supporting

structures, and assuming that the shock absorbers and tires are in their

static positions, the following conditions must be met:

(a) For aft loads, the limit force components at the axle must be--

(1) A vertical component of 2.25 times the static load on the wheel; and

(2) A drag component of 0.8 times the vertical load.

(b) For forward loads, the limit force components at the axle must be--

(1) A vertical component of 2.25 times the static load on the wheel; and

(2) A forward component of 0.4 times the vertical load.

(c) For side loads, the limit force components at ground contact must be--

(1) A vertical component of 2.25 times the static load on the wheel; and

(2) A side component of 0.7 times the vertical load.

Sec. 23.505  Supplementary conditions for skiplanes.

In determining ground loads for skiplanes, and assuming that the airplane

is resting on the ground with one main ski frozen at rest and the other skis

free to slide, a limit side force equal to 0.036 times the design maximum

weight must be applied near the tail assembly, with a factor of safety of 1.

[Amdt. 23-7, 34 FR 13090, Aug. 13, 1969]

Sec. 23.507  Jacking loads.

(a) The airplane must be designed for the loads developed when the aircraft

is supported on jacks at the design maximum weight assuming the following

load factors for landing gear jacking points at a three-point attitude and

for primary flight structure jacking points in the level attitude:

(1) Vertical-load factor of 1.35 times the static reactions.

(2) Fore, aft, and lateral load factors of 0.4 times the vertical static

reactions.

(b) The horizontal loads at the jack points must be reacted by inertia

forces so as to result in no change in the direction of the resultant loads

at the jack points.

(c) The horizontal loads must be considered in all combinations with the

vertical load.

[Amdt. 23-14, 38 FR 31821, Nov. 19, 1973]

Sec. 23.509  Towing loads.

The towing loads of this section must be applied to the design of tow

fittings and their immediate attaching structure.

(a) The towing loads specified in paragraph (d) of this section must be

considered separately. These loads must be applied at the towing fittings and

must act parallel to the ground. In addition:

(1) A vertical load factor equal to 1.0 must be considered acting at the

center of gravity; and

(2) The shock struts and tires must be in there static positions.

(b) For towing points not on the landing gear but near the plane of

symmetry of the airplane, the drag and side tow load components specified for

the auxiliary gear apply. For towing points located outboard of the main

gear, the drag and side tow load components specified for the main gear

apply. Where the specified angle of swivel cannot be reached, the maximum

obtainable angle must be used.

(c) The towing loads specified in paragraph (d) of this section must be

reacted as follows:

(1) The side component of the towing load at the main gear must be reacted

by a side force at the static ground line of the wheel to which the load is

applied.

(2) The towing loads at the auxiliary gear and the drag components of the

towing loads at the main gear must be reacted as follows:

(i) A reaction with a maximum value equal to the vertical reaction must be

applied at the axle of the wheel to which the load is applied. Enough

airplane inertia to achieve equilibrium must be applied.

(ii) The loads must be reacted by airplane inertia.

(d) The prescribed towing loads are as follows, where W is the design

maximum weight:

Load

Tow point        Position      Magnitude        No.           Direction

Main gear                           0.225W  1                Forward,

2                parallel to

3                drag axis.

4                Forward, at 30

deg. to drag

axis.

Aft, parallel

to drag axis.

Aft, at 30

deg. to drag

axis.

Auxiliary gear  Swiveled              0.3W  5                Forward.

forward                     6                Aft.

Swiveled aft          0.3W  7                Forward.

8                Aft.

Swiveled 45          0.15W  9                Forward, in

deg. from                   10               plane of

forward                                      wheel.

Aft, in plane

of wheel.

Swiveled 45          0.15W  11               Forward, in

deg. from aft               12               plane of

wheel.

Aft, in plane

of wheel.

[Amdt. 23-14, 38 FR 31821, Nov. 19, 1973]

Sec. 23.511  Ground load; unsymmetrical loads on multiple-wheel units.

(a) Pivoting loads. The airplane is assumed to pivot about on side of the

main gear with--

(1) The brakes on the pivoting unit locked; and

(2) Loads corresponding to a limit vertical load factor of 1, and

coefficient of friction of 0.8 applied to the main gear and its supporting

structure.

(b) Unequal tire loads. The loads established under Secs. 23.471 through

23.483 must be applied in turn, in a 60/40 percent distribution, to the dual

wheels and tires in each dual wheel landing gear unit.

(c) Deflated tire loads. For the deflated tire condition--

(1) 60 percent of the loads established under Secs. 23.471 through 23.483

must be applied in turn to each wheel in a landing gear unit; and

(2) 60 percent of the limit drag and side loads, and 100 percent of the

limit vertical load established under Secs. 23.485 and 23.493 or lesser

vertical load obtained under paragraph (c)(1) of this section, must be

applied in turn to each wheel in the dual wheel landing gear unit.

[Amdt. 23-7, 34 FR 13090, Aug. 13, 1969]

Water Loads

Sec. 23.521  Water load conditions.

(a) The structure of seaplanes and amphibians must be designed for water

loads developed during takeoff and landing with the seaplane in any attitude

likely to occur in normal operation at appropriate forward and sinking

velocities under the most severe sea conditions likely to be encountered.

(b) Unless the applicant makes a rational analysis of the water loads,

Secs. 23.523 through 23.537 apply.

(c) Floats previously approved by the FAA may be installed on airplanes

that are certificated under this part, provided that the floats meet the

criteria of paragraph (a) of this section.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight airworthiness standards

for normal, utility, acrobatic, and commuter category airplanes. The changes

are based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, in St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.523  Design weights and center of gravity positions.

(a) Design weights. The water load requirements must be met at each

operating weight up to the design landing weight except that, for the takeoff

condition prescribed in Sec. 23.531, the design water takeoff weight (the

maximum weight for water taxi and takeoff run) must be used.

(b) Center of gravity positions. The critical centers of gravity within the

limits for which certification is requested must be considered to reach

maximum design loads for each part of the seaplane structure.

[Amdt. No. 23-45, 58 FR 42160, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.525   Application of loads.

(a) Unless otherwise prescribed, the seaplane as a whole is assumed to be

subjected to the loads corresponding to the load factors specified in Sec.

23.527.

(b) In applying the loads resulting from the load factors prescribed in

Sec. 23.527, the loads may be distributed over the hull or main float bottom

(in order to avoid excessive local shear loads and bending moments at the

location of water load application) using pressures not less than those

prescribed in Sec. 23.533(c).

(c) For twin float seaplanes, each float must be treated as an equivalent

hull on a fictitious seaplane with a weight equal to one-half the weight of

the twin float seaplane.

(d) Except in the takeoff condition of Sec. 23.531, the aerodynamic lift on

the seaplane during the impact is assumed to be  2/3  of the weight of the

seaplane.

[Amdt. No. 23-45, 58 FR 42161, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.527   Hull and main float load factors.

(a) Water reaction load factors nw must be computed in the following

manner:

(1) For the step landing case

C1(VSO)**2

nw = -------------------------

(Tan**(2/3)b)W**(1/3)

(2) For the bow and stern landing cases

C1(VSO)**2                    K1

nw = ------------------------- x ------------------

(Tan**(2/3)b)W**(1/3)     (1+(rx)**2)**(2/3)

(b) The following values are used:

(1) nw=water reaction load factor (that is, the water reaction divided by

seaplane weight).

(2) C1=empirical seaplane operations factor equal to 0.012 (except that

this factor may not be less than that necessary to obtain the minimum value

of step load factor of 2.33).

(3) VSO=seaplane stalling speed in knots with flaps extended in the

appropriate landing position and with no slipstream effect.

(4) b=Angle of dead rise at the longitudinal station at which the load

factor is being determined in accordance with figure 1 of appendix I of this

part.

(5) W=seaplane landing weight in pounds.

(6) K1=empirical hull station weighing factor, in accordance with figure 2

of appendix I of this part.

(7) rx=ratio of distance, measured parallel to hull reference axis, from

the center of gravity of the seaplane to the hull longitudinal station at

which the load factor is being computed to the radius of gyration in pitch of

the seaplane, the hull reference axis being a straight line, in the plane of

symmetry, tangential to the keel at the main step.

(c) For a twin float seaplane, because of the effect of flexibility of the

attachment of the floats to the seaplane, the factor K1 may be reduced at the

bow and stern to 0.8 of the value shown in figure 2 of appendix I of this

part. This reduction applies only to the design of the carrythrough and

seaplane structure.

[Amdt. No. 23-45, 58 FR 42161, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.529   Hull and main float landing conditions.

(a) Symmetrical step, bow, and stern landing. For symmetrical step, bow,

and stern landings, the limit water reaction load factors are those computed

under Sec. 23.527. In addition--

(1) For symmetrical step landings, the resultant water load must be applied

at the keel, through the center of gravity, and must be directed

perpendicularly to the keel line;

(2) For symmetrical bow landings, the resultant water load must be applied

at the keel, one-fifth of the longitudinal distance from the bow to the step,

and must be directed perpendicularly to the keel line; and

(3) For symmetrical stern landings, the resultant water load must be

applied at the keel, at a point 85 percent of the longitudinal distance from

the step to the stern post, and must be directed perpendicularly to the keel

line.

(b) Unsymmetrical landing for hull and single float seaplanes.

Unsymmetrical step, bow, and stern landing conditions must be investigated.

In addition--

(1) The loading for each condition consists of an upward component and a

side component equal, respectively, to 0.75 and 0.25 tan b times the

resultant load in the corresponding symmetrical landing condition; and

(2) The point of application and direction of the upward component of the

load is the same as that in the symmetrical condition, and the point of

application of the side component is at the same longitudinal station as the

upward component but is directed inward perpendicularly to the plane of

symmetry at a point midway between the keel and chine lines.

(c) Unsymmetrical landing; twin float seaplanes. The unsymmetrical loading

consists of an upward load at the step of each float of 0.75 and a side load

of 0.25 tan b at one float times the step landing load reached under Sec.

23.527. The side load is directed inboard, perpendicularly to the plane of

symmetry midway between the keel and chine lines of the float, at the same

longitudinal station as the upward load.

[Amdt. No. 23-45, 58 FR 42161, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.531   Hull and main float takeoff condition.

For the wing and its attachment to the hull or main float--

(a) The aerodynamic wing lift is assumed to be zero; and

(b) A downward inertia load, corresponding to a load factor computed from

the following formula, must be applied:

CTO(VSI)**2

n = ---------------------

(Tan**(2/3)b)W**(1/3)

Where--

n=inertia load factor;

CTO=empirical seaplane operations factor equal to 0.004;

VS1=seaplane stalling speed (knots) at the design takeoff weight with the

flaps extended in the appropriate takeoff position;

b=angle of dead rise at the main step (degrees); and

W=design water takeoff weight in pounds.

[Amdt. No. 23-45, 58 FR 42161, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.533  Hull and main float bottom pressures.

(a) General. The hull and main float structure, including frames and

bulkheads, stringers, and bottom plating, must be designed under this

section.

(b) Local pressures. For the design of the bottom plating and stringers and

their attachments to the supporting structure, the following pressure

distributions must be applied:

(1) For an unflared bottom, the pressure at the chine is 0.75 times the

pressure at the keel, and the pressures between the keel and chine vary

linearly, in accordance with figure 3 of appendix I of this part. The

pressure at the keel (p.s.i.) is computed as follows:

C2K2(VSI)**2

PK = ----------------

Tan(bk)

where--

Pk=pressure (p.s.i.) at the keel;

C2=0.00213;

K2=hull station weighing factor, in accordance with figure 2 of appendix I of

this part;

VS1=seaplane stalling speed (knots) at the design water takeoff weight with

flaps extended in the appropriate takeoff position; and

bK=angle of dead rise at keel, in accordance with figure 1 of appendix I of

this part.

(2) For a flared bottom, the pressure at the beginning of the flare is the

same as that for an unflared bottom, and the pressure between the chine and

the beginning of the flare varies linearly, in accordance with figure 3 of

appendix I of this part. The pressure distribution is the same as that

prescribed in paragraph (b)(1) of this section for an unflared bottom except

that the pressure at the chine is computed as follows:

C3K2(VSI)**2

Pch = ----------------

Tan(b)

where--

Pch=pressure (p.s.i.) at the chine;

C3=0.0016;

K2=hull station weighing factor, in accordance with figure 2 of appendix I of

this part;

VS1=seaplane stalling speed (knots) at the design water takeoff weight with

flaps extended in the appropriate takeoff position; and

b=angle of dead rise at appropriate station.

The area over which these pressures are applied must simulate pressures

occurring during high localized impacts on the hull or float, but need not

extend over an area that would induce critical stresses in the frames or in

the overall structure.

(c) Distributed pressures. For the design of the frames, keel, and chine

structure, the following pressure distributions apply:

(1) Symmetrical pressures are computed as follows:

C4K2(VSO)**2

P = ----------------

Tan(b)

where--

P=pressure (p.s.i.);

C4=0.078 C1 (with C1 computed under Sec. 23.527);

K2=hull station weighing factor, determined in accordance with figure 2 of

appendix I of this part;

VS0=seaplane stalling speed (knots) with landing flaps extended in the

appropriate position and with no slipstream effect; and

b=angle of dead rise at appropriate station.

(2) The unsymmetrical pressure distribution consists of the pressures

prescribed in paragraph (c)(1) of this section on one side of the hull or

main float centerline and one-half of that pressure on the other side of the

hull or main float centerline, in accordance with figure 3 of appendix I of

this part.

(3) These pressures are uniform and must be applied simultaneously over the

entire hull or main float bottom. The loads obtained must be carried into the

sidewall structure of the hull proper, but need not be transmitted in a fore

and aft direction as shear and bending loads.

[Amdt. No. 23-45, 58 FR 42161, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.535  Auxiliary float loads.

(a) General. Auxiliary floats and their attachments and supporting

structures must be designed for the conditions prescribed in this section. In

the cases specified in paragraphs (b) through (e) of this section, the

prescribed water loads may be distributed over the float bottom to avoid

excessive local loads, using bottom pressures not less than those prescribed

in paragraph (g) of this section.

(b) Step loading. The resultant water load must be applied in the plane of

symmetry of the float at a point three-fourths of the distance from the bow

to the step and must be perpendicular to the keel. The resultant limit load

is computed as follows, except that the value of L need not exceed three

times the weight of the displaced water when the float is completely

submerged:

C5(VSO)**2(W**(2/3))

L = ------------------------------

Tan**(2/3)bs(1+(ry)**2)**(2/3)

where--

L=limit load (lbs.);

C5=0.0053;

VS0=seaplane stalling speed (knots) with landing flaps extended in the

appropriate position and with no slipstream effect;

W=seaplane design landing weight in pounds;

bs=angle of dead rise at a station  3/4  of the distance from the bow to the

step, but need not be less than 15 degrees; and

ry=ratio of the lateral distance between the center of gravity and the plane

of symmetry of the float to the radius of gyration in roll.

(c) Bow loading. The resultant limit load must be applied in the plane of

symmetry of the float at a point one-fourth of the distance from the bow to

the step and must be perpendicular to the tangent to the keel line at that

point. The magnitude of the resultant load is that specified in paragraph (b)

of this section.

(d) Unsymmetrical step loading. The resultant water load consists of a

component equal to 0.75 times the load specified in paragraph (a) of this

section and a side component equal to 0.025 tan b times the load specified in

paragraph (b) of this section. The side load must be applied perpendicularly

to the plane of symmetry of the float at a point midway between the keel and

the chine.

(e) Unsymmetrical bow loading. The resultant water load consists of a

component equal to 0.75 times the load specified in paragraph (b) of this

section and a side component equal to 0.25 tan b times the load specified in

paragraph (c) of this section. The side load must be applied perpendicularly

to the plane of symmetry at a point midway between the keel and the chine.

(f) Immersed float condition. The resultant load must be applied at the

centroid of the cross section of the float at a point one-third of the

distance from the bow to the step. The limit load components are as follows:

vertical = PgV

CXP(V**(2/3))(K VSO)**2

aft = -----------------------

2

CYP(V**(2/3))(K VSO)**2

side = -----------------------

2

where--

P=mass density of water (slugs/ft. 3 )

V=volume of float (ft. 3 );

CX=coefficient of drag force, equal to 0.133;

CY=coefficient of side force, equal to 0.106;

K=0.8, except that lower values may be used if it is shown that the floats

are incapable of submerging at a speed of 0.8 Vso in normal operations;

Vso=seaplane stalling speed (knots) with landing flaps extended in the

appropriate position and with no slipstream effect; and

g=acceleration due to gravity (ft/sec**2).

(g) Float bottom pressures. The float bottom pressures must be established

under Sec. 23.533, except that the value of K2 in the formulae may be taken

as 1.0. The angle of dead rise to be used in determining the float bottom

pressures is set forth in paragraph (b) of this section.

[Amdt. No. 23-45, 58 FR 42162, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.537  Seawing loads.

Seawing design loads must be based on applicable test data.

[Amdt. No. 23-45, 58 FR 42163, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Emergency Landing Conditions

Sec. 23.561  General.

(a) The airplane, although it may be damaged in emergency landing

conditions, must be designed as prescribed in this section to protect each

occupant under those conditions.

(b) The structure must be designed to protect each occupant during

emergency landing conditions when--

(1) Proper use is made of the seats, safety belts, and shoulder harnesses

provided for in the design;

(2) The occupant experiences the static inertia loads corresponding to the

following ultimate load factors--

(i) Upward, 3.0g for normal, utility, and commuter category airplanes, or

4.5g for acrobatic category airplanes;

(ii) Forward, 9.0g;

(iii) Sideward, 1.5g; and

(3) The items of mass within the cabin, that could injure an occupant,

experience the static inertia loads corresponding to the following ultimate

load factors--

(i) Upward, 3.0g;

(ii) Forward, 18.0g; and

(iii) Sideward, 4.5g.

(c) Each airplane with retractable landing gear must be designed to protect

each occupant in a landing--

(1) With the wheels retracted;

(2) With moderate descent velocity; and

(3) Assuming, in the absence of a more rational analysis--

(i) A downward ultimate inertia force of 3 g; and

(ii) A coefficient of friction of 0.5 at the ground.

(d) If it is not established that a turnover is unlikely during an

emergency landing, the structure must be designed to protect the occupants in

a complete turnover as follows:

(1) The likelihood of a turnover may be shown by an analysis assuming the

following conditions--

(i) Maximum weight;

(ii) Most forward center of gravity position;

(iii) Longitudinal load factor of 9.0g;

(iv) Vertical load factor of 1.0g; and

(v) For airplanes with tricycle landing gear, the nose wheel strut failed

with the nose contacting the ground.

(2) For determining the loads to be applied to the inverted airplane after

a turnover, an upward ultimate inertia load factor of 3.0g and a coefficient

of friction with the ground of 0.5 must be used.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13090, Aug. 13, 1969; Amdt. 23-24, 52 FR 34745, Sept. 14, 1987; Amdt. 23-36,

53 FR 30812, Aug. 15, 1988]

Sec. 23.562  Emergency landing dynamic conditions.

(a) Each seat/restraint system for use in a normal, utility, or acrobatic

category airplane must be designed to protect each occupant during an

emergency landing when--

(1) Proper use is made of seats, safety belts, and shoulder harnesses

provided for in the design; and

(2) The occupant is exposed to the loads resulting from the conditions

prescribed in this section.

(b) Except for those seat/restraint systems that are required to meet

paragraph (d) of this section, each seat/restraint system for crew or

passenger occupancy in a normal, utility, or acrobatic category airplane,

must successfully complete dynamic tests or be demonstrated by rational

analysis supported by dynamic tests, in accordance with each of the following

conditions. These tests must be conducted with an occupant simulated by an

anthropomorphic test dummy (ATD) defined by 49 CFR Part 572, Subpart B, or an

FAA-approved equivalent, with a nominal weight of 170 pounds and seated in

the normal upright position.

(1) For the first test, the change in velocity may not be less than 31 feet

per second. The seat/restraint system must be oriented in its nominal

position with respect to the airplane and with the horizontal plane of the

airplane pitched up 60 degrees, with no yaw, relative to the impact vector.

For seat/restraint systems to be installed in the first row of the airplane,

peak deceleration must occur in not more than 0.05 seconds after impact and

must reach a minimum of 19g. For all other seat/restraint systems, peak

deceleration must occur in not more than 0.06 seconds after impact and must

reach a minimum of 15g.

(2) For the second test, the change in velocity may not be less than 42

feet per second. The seat/restraint system must be oriented in its nominal

position with respect to the airplane and with the vertical plane of the

airplane yawed 10 degrees, with no pitch, relative to the impact vector in a

direction that results in the greatest load on the shoulder harness. For

seat/restraint systems to be installed in the first row of the airplane, peak

deceleration must occur in not more than 0.05 seconds after impact and must

reach a minimum of 26g. For all other seat/restraint systems, peak

deceleration must occur in not more than 0.06 seconds after impact and must

reach a minimum of 21g.

(3) To account for floor warpage, the floor rails or attachment devices

used to attach the seat/restraint system to the airframe structure must be

preloaded to misalign with respect to each other by at least 10 degrees

vertically (i.e., pitch out of parallel) and one of the rails or attachment

devices must be preloaded to misalign by 10 degrees in roll prior to

conducting the test defined by paragraph (b)(2) of this section.

(c) Compliance with the following requirements must be shown during the

dynamic tests conducted in accordance with paragraph (b) of this section:

(1) The seat/restraint system must restrain the ATD although seat/restraint

system components may experience deformation, elongation, displacement, or

crushing intended as part of the design.

(2) The attachment between the seat/restraint system and the test fixture

must remain intact, although the seat structure may have deformed.

(3) Each shoulder harness strap must remain on the ATD's shoulder during

the impact.

(4) The safety belt must remain on the ATD's pelvis during the impact.

(5) The results of the dynamic tests must show that the occupant is

protected from serious head injury.

(i) When contact with adjacent seats, structure, or other items in the

cabin can occur, protection must be provided so that the head impact does not

exceed a head injury criteria (HIC) of 1,000.

(ii) The value of HIC is defined as--

1          t2          2.5

HIC= { (t2-t1) [----------     S   a(t)dt ]      }

(t2-t1)       t1                  Max

Where: t1 is the initial integration time, expressed in seconds, t2 is the

final integration time, expressed in seconds, (t2-t1) is the time

duration of the major head impact, expressed in seconds, and a(t) is the

resultant deceleration at the center of gravity of the head form

expressed as a multiple of g (units of gravity).

(iii) Compliance with the HIC limit must be demonstrated by measuring the

head impact during dynamic testing as prescribed in paragraphs (b)(1) and

(b)(2) of this section or by a separate showing of compliance with the head

injury criteria using test or analysis procedures.

(6) Loads in individual shoulder harness straps may not exceed 1,750

pounds. If dual straps are used for retaining the upper torso, the total

strap loads may not exceed 2,000 pounds.

(7) The compression load measured between the pelvis and the lumbar spine

of the ATD may not exceed 1,500 pounds.

(d) For all single-engine airplanes with a VS0 of more than 61 knots at

maximum weight, and those multiengine airplanes of 6,000 pounds or less

maximum weight with a VS0 of more than 61 knots at maximum weight that do not

comply with Sec. 23.67(b)(2)(i):

(1) The ultimate load factors of Sec. 23.561(b) must be increased by

multiplying the load factors by the square of the ratio of the increased

stall speed to 61 knots. The increased ultimate load factors need not exceed

the values reached at a VS0 of 79 knots. The upward ultimate load factor for

acrobatic category airplanes need not exceed 5.0g.

(2) The seat/restraint system test required by paragraph (b)(1) of this

section must be conducted in accordance with the following criteria:

(i) The change in velocity may not be less than 31 feet per second.

(ii)(A) The peak deceleration (gp) of 19g and 15g must be increased and

multiplied by the square of the ratio of the increased stall speed to 61

knots:

gp=19.0 (VS0/61)**2 or gp=15.0 (VS0/61)**2

(B) The peak deceleration need not exceed the value reached at a VS0 of 79

knots.

(iii) The peak deceleration must occur in not more than time (tr), which

must be computed as follows:

31      .96

tr = -------- = ---

32.2(gp)   gp

where--

gp=The peak deceleration calculated in accordance with paragraph (d)(2)(ii)

of this section

tr=The rise time (in seconds) to the peak deceleration.

(e) An alternate approach that achieves an equivalent, or greater, level of

occupant protection to that required by this section may be used if

substantiated on a rational basis.

[Amdt. 23-36, 53 FR 30812, Aug. 15, 1988, as amended by Amdt. 23-44, 58 FR

38639, July 19, 1993]

*****************************************************************************

58 FR 38634, No. 136, July 19, 1993

SUMMARY: This final rule amends the stalling speed requirements applicable to

single-engine airplanes and to certain multiengine small airplanes of less

than 6,000 pounds maximum weight. The rule permits those airplanes to have a

stall speed greater than 61 knots, provided they meet certain additional

occupant protection standards. These changes are needed to permit the design

and type certification of higher performance airplanes with increased cruise

speeds and better specific fuel consumption. The amendments are intended to

achieve the benefits of certificating higher performance airplanes while

affording their occupants the same level of protection in an emergency

landing that is presently provided by airplanes with a 61-knot stall speed.

EFFECTIVE DATE: August 18, 1993.

*****************************************************************************

Fatigue Evaluation

Sec. 23.571  Pressurized cabin.

The strength, detail design, and fabrication of the pressure cabin

structure must be evaluated under one of the following:

(a) A fatigue strength investigation, in which the structure is shown by

analysis, tests, or both to be able to withstand the repeated loads of

variable magnitude expected in service. Analysis alone is considered

acceptable only when it is conservative and applied to simple structures.

(b) A fail safe strength investigation, in which it is shown by analysis,

tests, or both that catastrophic failure of the structure is not probable

after fatigue failure, or obvious partial failure, of a principal structural

element, and that the remaining structures are able to withstand a static

ultimate load factor of 75 percent of the limit load factor at VC,

considering the combined effects of normal operating pressures, expected

external aerodynamic pressures, and flight loads. These loads must be

multiplied by a factor of 1.15 unless the dynamic effects of failure under

static load are otherwise considered.

(c) The damage tolerance evaluation of Sec. 23.573(b).

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31821, Nov. 19, 1973; Amdt. No. 23-45, 58 FR 42163, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.572   Wing, empennage, and associated structures.

(a) The strength, detail design, and fabrication of those parts of the

wings (including canards, tandem wings, and winglets/tip fins), empennage,

their carry-through and attaching structures, whose failure would be

catastrophic, must be evaluated under one of the following unless it is

shown that the structure, operating stress level, materials, and expected

uses are comparable, from a fatigue standpoint, to a similar design that has

had extensive satisfactory service experience:

(1) A fatigue strength investigation in which the structure is shown by

analysis, tests, or both to be able to withstand the repeated loads of

variable magnitude expected in service. Analysis alone is acceptable only

when it is conservative and applied to simple structures; or

(2) A fail-safe strength investigation in which it is shown by analysis,

tests, or both, that catastrophic failure of the structure is not probably

after fatigue failure, or obvious partial failure, of a principal structural

element, and that the remaining structure is able to withstand a static

ultimate load factor of 75 percent of the critical limit load factor at Vc.

These loads must be multiplied by a factor of 1.15 unless the dynamic effects

of failure under static load are otherwise considered.

(3) The damage tolerance evaluation of Sec. 23.573(b).

(b) Each evaluation required by this section must--

(1) Include typical loading spectra (e.g. taxi, ground-air-ground cycles,

maneuver, gust);

(2) Account for any significant effects due to the mutual influence of

aerodynamic surfaces; and

(3) Consider any significant effects from propeller slipstream loading, and

buffet from vortex impingements.

[Amdt. 23-7, 34 FR 13090, Aug. 13, 1969, as amended by Amdt. 23-14, 38 FR

31821, Nov. 19, 1973; Amdt. 23-34, 52 FR 1830, Jan. 15, 1987; Amdt. 23-38, 54

FR 39511, Sept. 26, 1989; Amdt. No. 23-45, 58 FR 42163, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.573   Damage tolerance and fatigue evaluation of structure.

(a) Composite airframe structure. Composite airframe structure must be

evaluated under this paragraph instead of Secs. 23.571 and 23.572. The

applicant must evaluate the composite airframe structure, the failure of

which would result in catastrophic loss of the airplane, in each wing

(including canards, tandem wings, and winglets), empennage, their

carrythrough and attaching structure, moveable control surfaces and their

attaching structure fuselage, and pressure cabin using the damage-tolerance

criteria prescribed in paragraphs (a)(1) through (a)(4) of this section

unless shown to be impractical. If the applicant establishes that damage-

tolerance criteria is impractical for a particular structure, the structure

must be evaluated in accordance with paragraphs (a)(1) and (a)(6) of this

section. Where bonded joints are used, the structure must also be evaluated

in accordance with paragraph (a)(5) of this section. The effects of material

variability and environmental conditions on the strength and durability

properties of the composite materials must be accounted for in the

evaluations required by this section.

(1) It must be demonstrated by tests, or by analysis supported by tests,

that the structure is capable of carrying ultimate load with damage up to the

threshold of detectability considering the inspection procedures employed.

(2) The growth rate or no-growth of damage that may occur from fatigue,

corrosion, manufacturing flaws or impact damage, under repeated loads

expected in service, must be established by tests or analysis supported by

tests.

(3) The structure must be shown by residual strength tests, or analysis

supported by residual strength tests, to be able to withstand critical limit

flight loads, considered as ultimate loads, with the extent of detectable

damage consistent with the results of the damage tolerance evaluations. For

pressurized cabins, the following loads must be withstood:

(i) Critical limit flight loads with the combined effects of normal

operating pressure and expected external aerodynamic pressures.

(ii) The expected external aerodynamic pressures in 1g flight combined with

a cabin differential pressure equal to 1.1 times the normal operating

differential pressure without any other load.

(4) The damage growth, between initial detectability and the value selected

for residual strength demonstrations, factored to obtain inspection

intervals, must allow development of an inspection program suitable for

application by operation and maintenance personnel.

(5) The limit load capacity of each bonded joint must be substantiated by

one of the following methods:

(i) The maximum disbonds of each bonded joint consistent with the

capability to withstand the loads in paragraph (a)(3) of this section must be

determined by analysis, tests, or both. Disbonds of each bonded joint greater

than this must be prevented by design features; or

(ii) Proof testing must be conducted on each production article that will

apply the critical limit design load to each critical bonded joint; or

(iii) Repeatable and reliable non-destructive inspection techniques must be

established that ensure the strength of each joint.

(6) Structural components for which the damage tolerance method is shown to

be impractical must be shown by component fatigue tests, or analysis

supported by tests, to be able to withstand the repeated loads of variable

magnitude expected in service. Sufficient component, subcomponent, element,

or coupon tests must be done to establish the fatigue scatter factor and the

environmental effects. Damage up to the threshold of detectability and

ultimate load residual strength capability must be considered in the

demonstration.

(b) Metallic airframe structure. If the applicant elects to use Sec.

23.571(c) or Sec. 23.572(a)(3), then the damage tolerance evaluation must

include a determination of the probable locations and modes of damage due to

fatigue, corrosion, or accidental damage. The determination must be by

analysis supported by test evidence and, if available, service experience.

Damage at multiple sites due to fatigue must be included where the design is

such that this type of damage can be expected to occur. The evaluation must

incorporate repeated load and static analyses supported by test evidence. The

extent of damage for residual strength evaluation at any time within the

operational life of the airplane must be consistent with the initial

detectability and subsequent growth under repeated loads. The residual

strength evaluation must show that the remaining structure is able to

withstand critical limit flight loads, considered as ultimate, with the

extent of detectable damage consistent with the results of the damage

tolerance evaluations. For pressurized cabins, the following load must be

withstood:

(1) The normal operating differential pressure combined with the expected

external aerodynamic pressures applied simultaneously with the flight loading

conditions specified in this part, and

(2) The expected external aerodynamic pressures in 1g flight combined with

a cabin differential pressure equal to 1.1 times the normal operating

differential pressure without any other load.

(c) Inspection. Based on evaluations required by this section, inspections

or other procedures must be established as necessary to prevent catastrophic

failure and must be included in the Airworthiness Limitations section of the

Instructions for Continued Airworthiness required by Sec. 23.1529.

[Amdt. No. 23-45, 58 FR 42163, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Subpart D--Design and Construction

Sec. 23.601  General.

The suitability of each questionable design detail and part having an

important bearing on safety in operations, must be established by tests.

Sec. 23.603  Materials and workmanship.

(a) The suitability and durability of materials used for parts, the failure

of which could adversely affect safety, must--

(1) Be established by experience or tests;

(2) Meet approved specifications that ensure their having the strength and

other properties assumed in the design data; and

(3) Take into account the effects of environmental conditions, such as

temperature and humidity, expected in service.

(b) Workmanship must be of a high standard.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55464, Dec. 20, 1976; Amdt. 23-23, 43 FR 50592, Oct. 10, 1978]

Sec. 23.605  Fabrication methods.

(a) The methods of fabrication used must produce consistently sound

structures. If a fabrication process (such as gluing, spot welding, or heat-

treating) requires close control to reach this objective, the process must be

performed under an approved process specification.

(b) Each new aircraft fabrication method must be substantiated by a test

program.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-23, 43 FR 50592, Oct. 10, 1978]

Sec. 23.607  Self-locking nuts.

No self-locking nut may be used on any bolt subject to rotation in

operation unless a nonfriction locking device is used in addition to the

self-locking device.

[Amdt. 23-17, 41 FR 55464, Dec. 20, 1976]

Sec. 23.609  Protection of structure.

Each part of the structure must--

(a) Be suitably protected against deterioration or loss of strength in

service due to any cause, including--

(1) Weathering;

(2) Corrosion; and

(3) Abrasion; and

(b) Have adequate provisions for ventilation and drainage.

Sec. 23.611  Accessibility.

Means must be provided to allow inspection (including inspection of

principal structural elements and control systems), close examination,

repair, and replacement of each part requiring maintenance, adjustments for

proper alignment and function, lubrication or servicing.

[Amdt. 23-7, 34 FR 13090, Aug. 13, 1969]

Sec. 23.613  Material strength properties and design values.

(a) Material strength properties must be based on enough tests of material

meeting specifications to establish design values on a statistical basis.

(b) Design values must be chosen to minimize the probability of structural

failure due to material variability. Except as provided in paragraph (e) of

this section, compliance with this paragraph must be shown by selecting

design values that ensure material strength with the following probability:

(1) Where applied loads are eventually distributed through a single member

within an assembly, the failure of which would result in loss of structural

integrity of the component; 99 percent probability with 95 percent

confidence.

(2) For redundant structure, in which the failure of individual elements

would result in applied loads being safely distributed to other load carrying

members; 90 percent probability with 95 percent confidence.

(c) The effects of temperature on allowable stresses used for design in an

essential component or structure must be considered where thermal effects are

significant under normal operating conditions.

(d) The design of the structure must minimize the probability of

catastrophic fatigue failure, particularly at points of stress concentration.

(e) Design values greater than the guaranteed minimums required by this

section may be used where only guaranteed minimum values are normally allowed

if a "premium selection" of the material is made in which a specimen of each

individual item is tested before use to determine that the actual strength

properties of that particular item will equal or exceed those used in design.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-23, 43 FR 50592, Oct. 30, 1978; Amdt. No. 23-45, 58 FR

42163, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.615  [Removed. Amdt. No. 23-45, 58 FR 42164, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.619  Special factors.

The factor of safety prescribed in Sec. 23.303 must be multiplied by the

highest pertinent special factors of safety prescribed in Secs. 23.621

through 23.625 for each part of the structure whose strength is--

(a) Uncertain;

(b) Likely to deteriorate in service before normal replacement; or

(c) Subject to appreciable variability because of uncertainties in

manufacturing processes or inspection methods.

[Amdt. 23-7, 34 FR 13091, Aug. 13, 1969]

Sec. 23.621  Casting factors.

(a) General. The factors, tests, and inspections specified in paragraphs

(b) through (d) of this section must be applied in addition to those

necessary to establish foundry quality control. The inspections must meet

approved specifications. Paragraphs (c) and (d) of this section apply to any

structural castings except castings that are pressure tested as parts of

hydraulic or other fluid systems and do not support structural loads.

(b) Bearing stresses and surfaces.  The casting factors specified in

paragraphs (c) and (d) of this section--

(1) Need not exceed 1.25 with respect to bearing stresses regardless of the

method of inspection used; and

(2) Need not be used with respect to the bearing surfaces of a part whose

bearing factor is larger than the applicable casting factor.

(c) Critical castings. For each casting whose failure would preclude

continued safe flight and landing of the airplane or result in serious injury

to occupants, the following apply:

(1) Each critical casting must either--

(i) Have a casting factor of not less than 1.25 and receive 100 percent

inspection by visual, radiographic, and either magnetic particle, penetrant

or other approved equivalent non-destructive inspection method; or

(ii) Have a casting factor of not less than 2.0 and receive 100 percent

visual inspection and 100 percent approved non-destructive inspection. When

an approved quality control procedure is established and an acceptable

statistical analysis supports reduction, non-destructive inspection may be

reduced from 100 percent, and applied on a sampling basis.

(2) For each critical casting with a casting factor less than 1.50, three

sample castings must be static tested and shown to meet--

(i) The strength requirements of Sec. 23.305 at an ultimate load

corresponding to a casting factor of 1.25; and

(ii) The deformation requirements of Sec. 23.305 at a load of 1.15 times

the limit load.

(3) Examples of these castings are structural attachment fittings, parts of

flight control systems, control surface hinges and balance weight

attachments, seat, berth, safety belt, and fuel and oil tank supports and

attachments, and cabin pressure valves.

(d) Non-critical castings. For each casting other than those specified in

paragraph (c) or (e) of this section, the following apply:

(1) Except as provided in paragraphs (d) (2) and (3) of this section, the

casting factors and corresponding inspections must meet the following table:

Casting factor                            Inspection

2.0 or more                      100 percent visual.

Less than 2.0 but more than 1.5  100 percent visual, and magnetic particle or

penetrant or equivalent nondestructive

inspection methods.

1.25 through 1.50                100 percent visual, magnetic particle or

penetrant, and radiographic, or approved

equivalent nondestructive inspection

methods.

(2) The percentage of castings inspected by nonvisual methods may be

reduced below that specified in subparagraph (d)(1) of this section when an

approved quality control procedure is established.

(3) For castings procured to a specification that guarantees the mechanical

properties of the material in the casting and provides for demonstration of

these properties by test of coupons cut from the castings on a sampling

basis--

(i) A casting factor of 1.0 may be used; and

(ii) The castings must be inspected as provided in paragraph (d)(1) of this

section for casting factors of "1.25 through 1.50" and tested under paragraph

(c)(2) of this section.

(e) Non-structural castings. Castings used for non-structural purposes do

not require evaluation, testing or close inspection.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965; Amdt.

No. 23-45, 58 FR 42164, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.623  Bearing factors.

(a) Each part that has clearance (free fit), and that is subject to

pounding or vibration, must have a bearing factor large enough to provide for

the effects of normal relative motion.

(b) For control surface hinges and control system joints, compliance with

the factors prescribed in Secs. 23.657 and 23.693, respectively, meets

paragraph (a) of this section.

[Amdt. 23-7, 34 FR 13091, Aug. 13, 1969]

Sec. 23.625  Fitting factors.

For each fitting (a part or terminal used to join one structural member to

another), the following apply:

(a) For each fitting whose strength is not proven by limit and ultimate

load tests in which actual stress conditions are simulated in the fitting and

surrounding structures, a fitting factor of at least 1.15 must be applied to

each part of--

(1) The fitting;

(2) The means of attachment; and

(3) The bearing on the joined members.

(b) No fitting factor need be used for joint designs based on comprehensive

test data (such as continuous joints in metal plating, welded joints, and

scarf joints in wood).

(c) For each integral fitting, the part must be treated as a fitting up to

the point at which the section properties become typical of the member.

(d) For each seat, berth, safety belt, and harness, its attachment to the

structure must be shown, by analysis, tests, or both, to be able to withstand

the inertia forces prescribed in Sec. 23.561 multiplied by a fitting factor

of 1.33.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13091, Aug. 13, 1969]

Sec. 23.627  Fatigue strength.

The structure must be designed, as far as practicable, to avoid points of

stress concentration where variable stresses above the fatigue limit are

likely to occur in normal service.

Sec. 23.629  Flutter.

(a) It must be shown by one of the methods specified in paragraph (b), (c),

or (d) of this section, or a combination of these methods, that the airplane

is free from flutter, control reversal, and divergence for any condition of

operation within the limit V-n envelope, and at all speeds up to the speed

specified for the selected method. In addition--

(1) Adequate tolerances must be established for quantities which affect

flutter, including speed, damping, mass balance, and control system

stiffness; and

(2) The natural frequencies of main structural components must be

determined by vibration tests or other approved methods.

(b) A rational analysis may be used to show that the airplane is free from

flutter, control reversal, and divergence if the analysis shows freedom from

flutter for all speeds up to 1.2VD.

(c) Flight flutter tests may be used to show that the airplane is free from

flutter, control reversal, and divergence if it is shown by these tests

that--

(1) Proper and adequate attempts to induce flutter have been made within

the speed range up to VD;

(2) The vibratory response of the structure during the test indicates

freedom from flutter;

(3) A proper margin of damping exists at VD; and

(4) There is no large and rapid reduction in damping as VD is approached.

(d) Compliance with the rigidity and mass balance criteria (pages 4-12), in

Airframe and Equipment Engineering Report No. 45 (as corrected) "Simplified

Flutter Prevention Criteria" (published by the Federal Aviation

Administration) may be accomplished to show that the airplane is free from

flutter, control reversal, or divergence if--

(1) VD/MD for the airplane is less than 260 knots (EAS) and less than Mach

0.5,

(2) The wing and aileron flutter prevention criteria, as represented by the

wing torsional stiffness and aileron balance criteria, are limited in use to

airplanes without large mass concentrations (such as engines, floats, or fuel

tanks in outer wing panels) along the wing span, and

(3) The airplane--

(i) Does not have a T-tail or boom tail,

(ii) Does not have unusual mass distributions or other unconventional

design features that affect the applicability of the criteria, and

(iii) Has fixed-fin and fixed-stabilizer surfaces.

(e) For turbopropeller-powered airplanes, the dynamic evaluation must

include--

(1) Whirl mode degree of freedom which takes into account the stability of

the plane of rotation of the propeller and significant elastic, inertial, and

aerodynamic forces, and

(2) Propeller, engine, engine mount, and airplane structure stiffness and

damping variations appropriate to the particular configuration.

(f) Freedom from flutter, control reversal, and divergence up to VD/MD must

be shown as follows:

(1) For airplanes that meet the criteria of paragraphs (d)(1) through

(d)(3) of this section, after the failure, malfunction, or disconnection of

any single element in any tab control system.

(2) For airplanes other than those described in paragraph (f)(1) of this

section, after the failure, malfunction, or disconnection of any single

element in the primary flight control system, any tab control system, or any

flutter damper.

(g) For airplanes showing compliance with the fail-safe criteria of Secs.

23.571 and 23.572, the airplane must be shown by analysis or test to be free

from flutter to VD/MD after fatigue failure, or obvious partial failure of a

principle structural element.

(h) For airplanes showing compliance with the damage-tolerance criteria of

Sec. 23.573, the airplane must be shown by analysis or test to be free from

flutter to VD/MD with the extent of damage for which residual strength is

demonstrated.

[Amdt. 23-23, 43 FR 50592, Oct. 30, 1978, as amended by Amdt. 23-31, 49 FR

46867, Nov. 28, 1984; Amdt. No. 23-45, 58 FR 42164, Aug. 6, 1993; 58 FR

51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Wings

Sec. 23.641  Proof of strength.

The strength of stressed-skin wings must be proven by load tests or by

combined structural analysis and load tests.

Control Surfaces

Sec. 23.651  Proof of strength.

(a) Limit load tests of control surfaces are required. These tests must

include the horn or fitting to which the control system is attached.

(b) In structural analyses, rigging loads due to wire bracing must be

accounted for in a rational or conservative manner.

Sec. 23.655  Installation.

(a) Movable surfaces must be installed so that there is no interference

between any surfaces, their bracing, or adjacent fixed structure, when one

surface is held in its most critical clearance positions and the others are

operated through their full movement.

(b) If an adjustable stabilizer is used, it must have stops that will limit

its range of travel to that allowing safe flight and landing.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42164, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.657  Hinges.

(a) Control surface hinges, except ball and roller bearing hinges, must

have a factor of safety of not less than 6.67 with respect to the ultimate

bearing strength of the softest material used as a bearing.

(b) For ball or roller bearing hinges, the approved rating of the bearing

may not be exceeded.

(c) Hinges must have enough strength and rigidity for loads parallel to the

hinge line.

Sec. 23.659  Mass balance.

The supporting structure and the attachment of concentrated mass balance

weights used on control surfaces must be designed for--

(a) 24 g normal to the plane of the control surface;

(b) 12 g fore and aft; and

(c) 12 g parallel to the hinge line.

Control Systems

Sec. 23.671  General.

(a) Each control must operate easily, smoothly, and positively enough to

allow proper performance of its functions.

(b) Controls must be arranged and identified to provide for convenience in

operation and to prevent the possibility of confusion and subsequent

inadvertent operation.

Sec. 23.672  Stability augmentation and automatic and power-operated systems.

If the functioning of stability augmentation or other automatic or power-

operated systems is necessary to show compliance with the flight

characteristics requirements of this part, such systems must comply with Sec.

23.671 and the following:

(a) A warning, which is clearly distinguishable to the pilot under expected

flight conditions without requiring the pilot's attention, must be provided

for any failure in the stability augmentation system or in any other

automatic or power-operated system that could result in an unsafe condition

if the pilot was not aware of the failure. Warning systems must not activate

the control system.

(b) The design of the stability augmentation system or of any other

automatic or power-operated system must permit initial counteraction of

failures without requiring exceptional pilot skill or strength, by either the

deactivation of the system or a failed portion thereof, or by overriding the

failure by movement of the flight controls in the normal sense.

(c) It must be shown that, after any single failure of the stability

augmentation system or any other automatic or power-operated system--

(1) The airplane is safely controllable when the failure or malfunction

occurs at any speed or altitude within the approved operating limitations

that is critical for the type of failure being considered;

(2) The controllability and maneuverability requirements of this part are

met within a practical operational flight envelope (for example, speed,

altitude, normal acceleration, and airplane configuration) that is described

in the Airplane Flight Manual (AFM); and

(3) The trim, stability, and stall characteristics are not impaired below a

level needed to permit continued safe flight and landing.

[Amdt. No. 23-45, 58 FR 42164, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.673  Primary flight controls.

(a) Primary flight controls are those used by the pilot for the immediate

control of pitch, roll, and yaw.

(b) The design of two-control airplanes must minimize the likelihood of

complete loss of lateral or directional control in the event of failure of

any connecting or transmitting element in the control system.

Sec. 23.675  Stops.

(a) Each control system must have stops that positively limit the range of

motion of each movable aerodynamic surface controlled by the system.

(b) Each stop must be located so that wear, slackness, or takeup

adjustments will not adversely affect the control characteristics of the

airplane because of a change in the range of surface travel.

(c) Each stop must be able to withstand any loads corresponding to the

design conditions for the control system.

[Amdt. 23-17, 41 FR 55464, Dec. 20, 1976]

Sec. 23.677  Trim systems.

(a) Proper precautions must be taken to prevent inadvertent, improper, or

abrupt trim tab operation. There must be means near the trim control to

indicate to the pilot the direction of trim control movement relative to

airplane motion. In addition, there must be means to indicate to the pilot

the position of the trim device with respect to the range of adjustment. This

means must be visible to the pilot and must be located and designed to

prevent confusion.

(b) Trimming devices must be designed so that, when any one connecting or

transmitting element in the primary flight control system fails, adequate

control for safe flight and landing is available with--

(1) For single-engine airplanes, the longitudinal trimming devices; or

(2) For multiengine airplanes, the longitudinal and directional trimming

devices.

(c) Tab controls must be irreversible unless the tab is properly balanced

and has no unsafe flutter characteristics. Irreversible tab systems must have

adequate rigidity and reliability in the portion of the system from the tab

to the attachment of the irreversible unit to the airplane structure.

(d) It must be demonstrated that the airplane is safely controllable and

that the pilot can perform all maneuvers and operations necessary to effect a

safe landing following any probable powered trim system runaway that

reasonably might be expected in service, allowing for appropriate time delay

after pilot recognition of the trim system runaway. The demonstration must be

conducted at critical airplane weights and center of gravity positions.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13091, Aug. 13, 1969; Amdt. 23-34, 52 FR 1830, Jan. 15, 1987; Amdt. 23-42,

56 FR 353, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.679  Control system locks.

If there is a device to lock the control system on the ground or water:

(a) There must be a means to--

(1) Give unmistakable warning to the pilot when lock is engaged; or

(2) Automatically disengage the device when the pilot operates the primary

flight controls in a normal manner.

(b) The device must be installed to limit the operation of the airplane so

that, when the device is engaged, the pilot receives unmistakable warning at

the start of the takeoff.

(c) The device must have a means to preclude the possibility of it becoming

inadvertently engaged in flight.

[Amdt. No. 23-45, 58 FR 42164, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.681  Limit load static tests.

(a) Compliance with the limit load requirements of this part must be shown

by tests in which--

(1) The direction of the test loads produces the most severe loading in the

control system; and

(2) Each fitting, pulley, and bracket used in attaching the system to the

main structure is included.

(b) Compliance must be shown (by analyses or individual load tests) with

the special factor requirements for control system joints subject to angular

motion.

Sec. 23.683  Operation tests.

(a) It must be shown by operation tests that, when the controls are

operated from the pilot compartment with the system loaded as prescribed in

paragraph (b) of this section, the system is free from--

(1) Jamming;

(2) Excessive friction; and

(3) Excessive deflection.

(b) The prescribed test loads are--

(1) For the entire system, loads corresponding to the limit airloads on the

appropriate surface, or the limit pilot forces in Sec. 23.397(b), whichever

are less; and

(2) For secondary controls, loads not less than those corresponding to the

maximum pilot effort established under Sec. 23.405.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13091, Aug. 13, 1969]

Sec. 23.685  Control system details.

(a) Each detail of each control system must be designed and installed to

prevent jamming, chafing, and interference from cargo, passengers, loose

objects, or the freezing of moisture.

(b) There must be means in the cockpit to prevent the entry of foreign

objects into places where they would jam the system.

(c) There must be means to prevent the slapping of cables or tubes against

other parts.

(d) Each element of the flight control system must have design features, or

must be distinctively and permanently marked, to minimize the possibility of

incorrect assembly that could result in malfunctioning of the control system.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55464, Dec. 20, 1976]

Sec. 23.687  Spring devices.

The reliability of any spring device used in the control system must be

established by tests simulating service conditions unless failure of the

spring will not cause flutter or unsafe flight characteristics.

Sec. 23.689  Cable systems.

(a) Each cable, cable fitting, turnbuckle, splice, and pulley used must

meet approved specifications. In addition--

(1) No cable smaller than 1/8  inch diameter may be used in primary control

systems;

(2) Each cable system must be designed so that there will be no hazardous

change in cable tension throughout the range of travel under operating

conditions and temperature variations; and

(3) There must be means for visual inspection at each fairlead, pulley,

terminal, and turnbuckle.

(b) Each kind and size of pulley must correspond to the cable with which it

is used. Each pulley must have closely fitted guards to prevent the cables

from being misplaced or fouled, even when slack. Each pulley must lie in the

plane passing through the cable so that the cable does not rub against the

pulley flange.

(c) Fairleads must be installed so that they do not cause a change in cable

direction of more than three degrees.

(d) Clevis pins subject to load or motion and retained only by cotter pins

may not be used in the control system.

(e) Turnbuckles must be attached to parts having angular motion in a manner

that will positively prevent binding throughout the range of travel.

(f) Tab control cables are not part of the primary control system and may

be less than 1/8  inch diameter in airplanes that are safely controllable

with the tabs in the most adverse positions.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13091, Aug. 13, 1969]

Sec. 23.693  Joints.

Control system joints (in push-pull systems) that are subject to angular

motion, except those in ball and roller bearing systems, must have a special

factor of safety of not less than 3.33 with respect to the ultimate bearing

strength of the softest material used as a bearing. This factor may be

reduced to 2.0 for joints in cable control systems. For ball or roller

bearings, the approved ratings may not be exceeded.

Sec. 23.697  Wing flap controls.

(a) Each wing flap control must be designed so that, when the flap has been

placed in any position upon which compliance with the performance

requirements of this part is based, the flap will not move from that position

unless the control is adjusted or is moved by the automatic operation of a

flap load limiting device.

(b) The rate of movement of the flaps in response to the operation of the

pilot's control or automatic device must give satisfactory flight and

performance characteristics under steady or changing conditions of airspeed,

engine power, and attitude.

Sec. 23.699  Wing flap position indicator.

There must be a wing flap position indicator for--

(a) Flap installations with only the retracted and fully extended position,

unless--

(1) A direct operating mechanism provides a sense of "feel" and position

(such as when a mechanical linkage is employed); or

(2) The flap position is readily determined without seriously detracting

from other piloting duties under any flight condition, day or night; and

(b) Flap installation with intermediate flap positions if--

(1) Any flap position other than retracted or fully extended is used to

show compliance with the performance requirements of this part; and

(2) The flap installation does not meet the requirements of paragraph

(a)(1) of this section.

Sec. 23.701  Flap interconnection.

(a) The main wing flaps and related movable surfaces as a system must--

(1) Be synchronized by mechanical connection; or

(2) Maintain synchronization so that the occurrence of an unsafe condition

has been shown to be extremely improbable; or

(b) The airplane must be shown to have safe flight characteristics with any

combination of extreme positions of individual movable surfaces (mechanically

interconnected surfaces are to be considered as a single surface).

(c) If an interconnection is used in multiengine airplanes, it must be

designed to account for the unsummetrical loads resulting from flight with

the engines on one side of the plane of symmetry inoperative and the

remaining engines at takeoff power. For single-engine airplanes, and

multiengine airplanes with no slipstream effects on the flaps, it may be

assumed that 100 percent of the critical air load acts on one side and 70

percent on the other.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31821, Nov. 19, 1973; Amdt. 23-42, 56 FR 353, Jan. 3, 1991; Amdt. 23-42, 56

FR 5455, Feb. 11, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Landing Gear

Sec. 23.721   General.

For commuter category airplanes that have a passenger seating

configuration, excluding pilot seats, of 10 or more, the following general

requirements for the landing gear apply:

(a) The main landing-gear system must be designed so that if it fails due

to overloads during takeoff and landing (assuming the overloads to act in the

upward and aft directions), the failure mode is not likely to cause the

spillage of enough fuel from any part of the fuel system to consitute a fire

hazard.

(b) Each airplane must be designed so that, with the airplane under

control, it can be landed on a paved runway with any one or more landing-gear

legs not extended without sustaining a structural component failure that is

likely to cause the spillage of enough fuel to consitute a fire hazard.

(c) Compliance with the provisions of this section may be shown by analysis

or tests, or both.

[Amdt. 23-34, 52 FR 1830, Jan. 15, 1987]

Sec. 23.723  Shock absorption tests.

(a) It must be shown that the limit load factors selected for design in

accordance with Sec. 23.473 for takeoff and landing weights, respectively,

will not be exceeded. This must be shown by energy absorption tests except

that analysis based on tests conducted on a landing gear system with

identical energy absorption characteristics may be used for increases in

previously approved takeoff and landing weights.

(b) The landing gear may not fail, but may yield, in a test showing its

reserved energy absorption capacity, simulating a descent velocity of 1.2

times the limit descent velocity, assuming wing lift equal to the weight of

the airplane.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-23, 43 FR 50593, Oct. 30, 1978]

Sec. 23.725  Limit drop tests.

(a) If compliance with Sec. 23.723(a) is shown by free drop tests, these

tests must be made on the complete airplane, or on units consisting of wheel,

tire, and shock absorber, in their proper relation, from free drop heights

not less than those determined by the following formula:

h (inches) = 3.6 (W/S)  1/2

However, the free drop height may not be less than 9.2 inches and need not be

more than 18.7 inches.

(b) If the effect of wing lift is provided for in free drop tests, the

landing gear must be dropped with an effective weight equal to

h+(1-L)d

We = W x  ------------

h+d

where--

We =the effective weight to be used in the drop test (lbs.);

h = specified free drop height (inches);

d = deflection under impact of the tire (at the approved inflation pressure)

plus the vertical component of the axle travel relative to the drop mass

(inches);

W=WM for main gear units (lbs), equal to the static weight on that unit with

the airplane in the level attitude (with the nose wheel clear in the case

of nose wheel type airplanes);

W=WT for tail gear units (lbs.), equal to the static weight on the tail unit

with the airplane in the tail-down attitude;

W=WN for nose wheel units lbs.), equal to the vertical component of the

static reaction that would exist at the nose wheel, assuming that the

mass of the airplane acts at the center of gravity and exerts a force of

1.0 g downward and 0.33 g forward; and

L= the ratio of the assumed wing lift to the airplane weight, but not more

than 0.667.

(c) The limit inertia load factor must be determined in a rational or

conservative manner, during the drop test, using a landing gear unit

attitude, and applied drag loads, that represent the landing conditions.

(d) The value of d used in the computation of We in paragraph (b) of this

section may not exceed the value actually obtained in the drop test.

(e) The limit inertia load factor must be determined from the drop test in

paragraph (b) of this section according to the following formula:

We

n = nj  ----  + L

W

where--

nj=the load factor developed in the drop test (that is, the acceleration (dv/

dt) in g's recorded in the drop test) plus 1.0; and

We, W, and L are the same as in the drop test computation.

(f) The value of n determined in accordance with paragraph (e) may not be

more than the limit inertia load factor used in the landing conditions in

Sec. 23.473.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13091, Aug. 13, 1969]

Sec. 23.726  Ground load dynamic tests.

(a) If compliance with the ground load requirements of Secs. 23.479 through

23.483 is shown dynamically by drop test, one drop test must be conducted

that meets Sec. 23.725 except that the drop height must be--

(1) 2.25 times the drop height prescribed in Sec. 23.725(a); or

(2) Sufficient to develop 1.5 times the limit load factor.

(b) The critical landing condition for each of the design conditions

specified in Secs. 23.479 through 23.483 must be used for proof of strength.

[Amdt. 23-7, 34 FR 13091, Aug. 13, 1969]

Sec. 23.727  Reserve energy absorption drop test.

(a) If compliance with the reserve energy absorption requirement in Sec.

23.723(b) is shown by free drop tests, the drop height may not be less than

1.44 times that specified in Sec. 23.725.

(b) If the effect of wing lift is provided for, the units must be dropped

with an effective mass equal to We=Wh/(h+d), when the symbols and other

details are the same as in Sec. 23.725.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13091, Aug. 13, 1969]

Sec. 23.729  Landing gear extension and retraction system.

(a) General. For airplanes with retractable landing gear, the following

apply:

(1) Each landing gear retracting mechanism and its supporting structure

must be designed for maximum flight load factors with the gear retracted and

must be designed for the combination of friction, inertia, brake torque, and

air loads, occurring during retraction at any airspeed up to 1.6 VS1 with

flaps retracted, and for any load factor up to those specified in Sec. 23.345

for the flaps-extended condition.

(2) The landing gear and retracting mechanism, including the wheel well

doors, must withstand flight loads, including loads resulting from all yawing

conditions specified in Sec. 23.351, with the landing gear extended at any

speed up to at least 1.6 VS1 with the flaps retracted.

(b) Landing gear lock. There must be positive means (other than the use of

hydraulic pressure) to keep the landing gear extended.

(c) Emergency operation. For a landplane having retractable landing gear

that cannot be extended manually, there must be means to extend the landing

gear in the event of either--

(1) Any reasonably probable failure in the normal landing gear operation

system; or

(2) Any reasonably probable failure in a power source that would prevent

the operation of the normal landing gear operation system.

(d) Operation test. The proper functioning of the retracting mechanism must

be shown by operation tests.

(e) Position indicator. If a retractable landing gear is used, there must

be a landing gear position indicator (as well as necessary switches to

actuate the indicator) or other means to inform the pilot that the gear is

secured in the extended (or retracted) position. If switches are used, they

must be located and coupled to the landing gear mechanical system in a manner

that prevents an erroneous indication of either "down and locked" if the

landing gear is not in a fully extended position, or of "up and locked" if

the landing gear is not in the fully retracted position. The switches may be

located where they are operated by the actual landing gear locking latch or

device.

(f) Landing gear warning. For landplanes, the following aural or equally

effective landing gear warning devices must be provided:

(1) A device that functions continuously when one or more throttles are

closed beyond the power settings normally used for landing approach if the

landing gear is not fully extended and locked. A throttle stop may not be

used in place of an aural device. If there is a manual shutoff for the

warning device prescribed in this paragraph, the warning system must be

designed so that when the warning has been suspended after one or more

throttles are closed, subsequent retardation of any throttle to, or beyond,

the position for normal landing approach will activate the warning device.

(2) A device that functions continuously when the wing flaps are extended

beyond the maximum approach flap position, using a normal landing procedure,

if the landing gear is not fully extended and locked. There may not be a

manual shutoff for this warning device. The flap position sensing unit may be

installed at any suitable location. The system for this device may use any

part of the system (including the aural warning device) for the device

required in paragraph (f)(1) of this section.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13091, Aug. 13, 1969; Amdt. 23-21, 43 FR 2318, Jan. 1978; Amdt. 23-26, 45 FR

60171, Sept. 11, 1980; Amdt. No. 23-45, 58 FR 42164, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.731  Wheels.

(a) The maximum static load rating of each wheel may not be less than the

corresponding static ground reaction with--

(1) Design maximum weight; and

(2) Critical center of gravity.

(b) The maximum limit load rating of each wheel must equal or exceed the

maximum radial limit load determined under the applicable ground load

requirements of this part.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.733  Tires.

(a) Each landing gear wheel must have a tire whose approved tire ratings

(static and dynamic) are not exceeded--

(1) By a load on each main wheel tire) to be compared to the static rating

approved for such tires) equal to the corresponding static ground reaction

under the design maximum weight and critical center of gravity; and

(2) By a load on nose wheel tires (to be compared with the dynamic rating

approved for such tires) equal to the reaction obtained at the nose wheel,

assuming the mass of the airplane to be concentrated at the most critical

center of gravity and exerting a force of 1.0 W downward and 0.31 W forward

(where W is the design maximum weight), with the reactions distributed to the

nose and main wheels by the principles of statics and with the drag reaction

at the ground applied only at wheels with brakes.

(b) If specially constructed tires are used, the wheels must be plainly and

conspicuously marked to that effect. The markings must include the make,

size, number of plies, and identification marking of the proper tire.

(c) Each tire installed on a retractable landing gear system must, at the

maximum size of the tire type expected in service, have a clearance to

surrounding structure and systems that is adequate to prevent contact between

the tire and any part of the structure of systems.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13092, Aug. 13, 1969; Amdt. 23-17, 41 FR 55464, Dec. 20, 1976; Amdt. No.

23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.735  Brakes.

(a) Brakes must be provided so that the brake kinetic energy capacity

rating of each main wheel brake assembly is not less than the kinetic energy

absorption requirements determined under either of the following methods:

(1) The brake kinetic energy absorption requirements must be based on a

conservative rational analysis of the sequence of events expected during

landing at the design landing weight.

(2) Instead of a rational analysis, the kinetic energy absorption

requirements for each main wheel brake assembly may be derived from the

following formula:

KE=0.0443 WV**2/N

where--

KE=Kinetic energy per wheel (ft.-lb.);

W=Design landing weight (lb.);

V=Airplane speed in knots. V must be not less than Vs<radical>, the poweroff

stalling speed of the airplane at sea level, at the design landing

weight, and in the landing configuration; and

N=Number of main wheels with brakes.

(b) Brakes must be able to prevent the wheels from rolling on a paved

runway with takeoff power on the critical engine, but need not prevent

movement of the airplane with wheels locked.

(c) If antiskid devices are installed, the devices and associated systems

must be designed so that no single probable malfunction or failure will

result in a hazardous loss of braking ability or directional control of the

airplane.

[Amdt. 23-7, 34 FR 13092, Aug. 13, 1969, as amended by Amdt. 23-24, 44 FR

68742, Nov. 29, 1979; Amdt. 23-42, 56 FR 354, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.737  Skis.

The maximum limit load rating of each ski must equal or exceed the maximum

limit load determined under the applicable ground load requirements of this

part.

[Amdt. No. 73-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Floats and Hulls

Sec. 23.751  Main float buoyancy.

(a) Each main float must have--

(1) A buoyancy of 80 percent in excess of the buoyancy required by that

float to support its portion of the maximum weight of the seaplane or

amphibian in fresh water; and

(2) Enough watertight compartments to provide reasonable assurance that the

seaplane or amphibian will stay afloat without capsizing if any two

compartments of any main float are flooded.

(b) Each main float must contain at least four watertight compartments

approximately equal in volume.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.753  Main float design.

Each seaplane main float must meet the requirements of Sec. 23.521.

[Amdt. No. 23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.755  Hulls.

(a) The hull of a hull seaplane or amphibian of 1,500 pounds or more

maximum weight must have watertight compartments designed and arranged so

that the hull auxiliary floats, and tires (if used), will keep the airplane

afloat without capsizing in fresh water when--

(1) For airplanes of 5,000 pounds or more maximum weight, any two adjacent

compartments are flooded; and

(2) For airplanes of 1,500 pounds up to, but not including, 5,000 pounds

maximum weight, any single compartment is flooded.

(b) The hulls of hull seaplanes or amphibians of less than 1,500 pounds

maximum weight need not be compartmented.

(c) Bulkheads with watertight doors may be used for communication between

compartments.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.757  Auxiliary floats.

Auxiliary floats must be arranged so that, when completely submerged in

fresh water, they provide a righting moment of at least 1.5 times the

upsetting moment caused by the seaplane or amphibian being tilted.

Personnel and Cargo Accommodations

Sec. 23.771  Pilot compartment.

For each pilot compartment--

(a) The compartment and its equipment must allow each pilot to perform his

duties without unreasonable concentration or fatigue;

(b) Where the flight crew are separated from the passengers by a partition,

an opening or openable window or door must be provided to facilitate

communication between flight crew and the passengers; and

(c) The aerodynamic controls listed in Sec. 23.779, excluding cables and

control rods, must be located with respect to the propellers so that no part

of the pilot or the controls lies in the region between the plane of rotation

of any inboard propeller and the surface generated by a line passing through

the center of the propeller hub making an angle of 5 degrees forward or aft

of the plane of rotation of the propeller.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31821, Nov. 19, 1973]

Sec. 23.773   Pilot compartment view.

(a) Each pilot compartment must be--

(1) Arranged with sufficiently extensive, clear and undistorted view to

enable the pilot to safely taxi, takeoff, approach, land, and perform any

maneuvers within the operating limitations of the airplane.

(2) Free from glare and reflections that could interfere with the pilot's

vision. Compliance must be shown in all operations for which certification is

requested; and

(3) Designed so that each pilot is protected from the elements so that

moderate rain conditions do not unduly impair the pilot's view of the flight

path in normal flight and while landing.

(b) Each pilot compartment must have a means to either remove or prevent

the formation of fog or frost on an area of the internal portion of the

windshield and side windows sufficiently large to provide the view specified

in paragraph (a)(1) of this section. Compliance must be shown under all

expected external and internal ambient operating conditions, unless it can be

shown that the windshield and side windows can be easily cleared by the pilot

without interruption of moral pilot duties.

[Amdt. 23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.775  Windshields and windows.

(a) Nonsplintering safety glass must be used in internal glass panes.

(b) The design of windshields, windows, and canopies in pressurized

airplanes must be based on factors peculiar to high altitude operation,

including--

(1) The effects of continuous and cyclic pressurization loadings;

(2) The inherent characteristics of the material used; and

(3) The effects of temperatures and temperature gradients.

(c) On pressurized airplanes that do not comply with the fail-safe

requirements of paragraph (e) of this section, an enclosure canopy including

a representative part of the installation must be subjected to special tests

to account for the combined effects of continuous and cyclic pressurization

loadings and flight loads.

(d) The windshield and side windows forward of the pilot's back when he is

seated in the normal flight position must have a luminous transmittance value

of not less than 70 percent.

(e) If certification for operation above 25,000 feet is requested the

windshields, window panels, and canopies must be strong enough to withstand

the maximum cabin pressure differential loads combined with critical

aerodynamic pressure and temperature effects, after failure of any load-

carrying element of the windshield, window panel, or canopy.

(f) Unless operation in known or forecast icing conditions is prohibited by

operating limitations, a means must be provided to prevent or to clear

accumulations of ice from the windshield so that the pilot has adequate view

for taxi, takeoff, approach, landing, and to perform any maneuvers within the

operating limitations of the airplane.

(g) In the event of any probable single failure, a transparency heating

system must be incapable of raising the temperature of any windshield or

window to a point where there would be--

(1) Structural failure that adversely affects the integrity of the cabin;

or

(2) There would be a danger of fire.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13092, Aug. 13, 1969; Amdt. No. 23-45, 58 FR 42165, Aug. 6, 1993; 58 FR

51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.777  Cockpit controls.

(a) Each cockpit control must be located and (except where its function is

obvious) identified to provide convenient operation and to prevent confusion

and inadvertent operation.

(b) The controls must be located and arranged so that the pilot, when

seated, has full and unrestricted movement of each control without

interference from either his clothing or the cockpit structure.

(c) Powerplant controls must be located--

(1) For multiengine airplanes, on the pedestal or overhead at or near the

center of the cockpit;

(2) For tandem seated single-engine airplanes, on the left side console or

instrument panel;

(3) For other single-engine airplanes at or near the center of the cockpit,

on the pedestal, instrument panel, or overhead; and

(4) For airplanes, with side-by-side pilot seats and with two sets of

powerplant controls, on left and right consoles.

(d) The control location order from left to right must be power (thrust)

lever, propeller (rpm control), and mixture control (condition lever and fuel

cutoff for turbine-powered airplanes). Power (thrust) levers must be at least

one inch higher or longer to make them more prominent than propeller (rpm

control) or mixture controls. Carburetor heat or alternate air control must

be to the left of the throttle or at least eight inches from the mixture

control when located other than on a pedestal. Carburetor heat or alternate

air control, when located on a pedestal must be aft or below the power

(thrust) lever. Supercharger controls must be located below or aft of the

propeller controls. Airplanes with tandem seating or single-place airplanes

may utilize control locations on the left side of the cabin compartment;

however, location order from left to right must be power (thrust) lever,

propeller (rpm control) and mixture control.

(e) Identical powerplant controls for each engine must be located to

prevent confusion as to the engines they control.

(1) Conventional multiengine powerplant controls must be located so that

the left control(s) operates the left engines(s) and the right control(s)

operates the right engine(s).

(2) On twin-engine airplanes with front and rear engine locations (tandem),

the left powerplant controls must operate the front engine and the right

powerplant controls must operate the rear engine.

(f) Wing flap and auxiliary lift device controls must be located--

(1) Centrally, or to the right of the pedestal or powerplant throttle

control centerline; and

(2) Far enough away from the landing gear control to avoid confusion.

(g) The landing gear control must be located to the left of the throttle

centerline or pedestal centerline.

(h) Each fuel feed selector control must comply with Sec. 23.995 and be

located and arranged so that the pilot can see and reach it without moving

any seat or primary flight control when his seat is at any position in which

it can be placed.

(1) For a mechanical fuel selector:

(i) The indication of the selected fuel valve position must be by means of

a pointer and must provide positive identification and feel (detent, etc.) of

the selected position.

(ii) The position indicator pointer must be located at the part of the

handle that is the maximum dimension of the handle measured from the center

of rotation.

(2) For electrical or electronic fuel selector:

(i) Digital controls or electrical switches must be properly labelled.

(ii) Means must be provided to indicate to the flight crew the tank or

function selected. Selector switch position is not acceptable as a means of

indication. The "off" or "closed" position must be indicated in red.

(3) If the fuel valve selector handle or electrical or digital selection is

also a fuel shut-off selector, the off position marking must be colored red.

If a separate emergency shut-off means is provided, it also must be colored

red.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13092, Aug. 13, 1969; Amdt. 23-33, 51 FR 26656, July 24, 1986]

Sec. 23.779  Motion and effect of cockpit controls.

Cockpit controls must be designed so that they operate in accordance with

the following movement and actuation:

(a) Aerodynamic controls:

Motion and effect

(1) Primary controls:

Aileron                              Right (clockwise) for right wing down.

Elevator                             Rearward for nose up.

Rudder                               Right pedal forward for nose right.

(2) Secondary controls:

Flaps (or auxiliary lift devices)    Forward or up for flaps up or

auxiliary device stowed; rearward or

down for flaps down or auxiliary

device deployed.

Trim tabs (or equivalent)            Switch motion or mechanical rotation

of control to produce similar

rotation of the airplane about an

axis parallel to the axis control.

Axis of roll trim control may be

displaced to accommodate comfortable

actuation by the pilot. For single-

engine airplanes, direction of

pilot's hand movement must be in the

same sense as airplane response for

rudder trim if only a portion of a

rotational element is accessible.

(b) Powerplant and auxiliary controls:

Motion and effect

(1) Powerplant controls:

Power (thrust) lever                 Forward to increase forward thrust

and rearward to increase rearward

thrust.

Propellers                           Forward to increase rpm.

Mixture                              Forward or upward for rich.

Carburetor, air heat or alternate    Forward or upward for cold.

air

Supercharger                         Forward or upward for low blower.

Turbosuperchargers                   Forward, upward, or clockwise to

increase pressure.

Rotary controls                      Clockwise from off to full on.

(2) Auxiliary controls:

Fuel tank selector                   Right for right tanks, left for left

tanks.

Landing gear                         Down to extend.

Speed brakes                         Aft to extend.

[Amdt. 23-33, 51 FR 26656, July 24, 1986]

Sec. 23.781  Cockpit control knob shape.

(a) Flap and landing gear control knobs must conform to the general shapes

(but not necessarily the exact sizes or specific proportions) in the

following figure:

[ ...Illustration appears here... ]

Flap Control Knob

[ ...Illustration appears here... ]

Landing Gear Control Knob

(b) Powerplant control knobs must conform to the general shapes (but not

necessarily the exact sizes or specific proportions) in the following figure:

[ ...Illustration appears here... ]

Power (Thrust) Control Knob

[ ...Illustration appears here... ]

RPM Control Knob

[ ...Illustration appears here... ]

Mixture Control Knob

[ ...Illustration appears here... ]

Carb Heat or Alternate Air Control Knob

[ ...Illustration appears here... ]

Supercharger Control Knob

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-33, 51 FR 26657, July 24, 1986]

Sec. 23.783  Doors.

(a) Each closed cabin with passenger accommodations must have at least one

adequate and easily accessible external door.

(b) No passenger door may be located with respect to any propeller disc so

as to endanger persons using that door.

(c) Each external passenger or crew door must comply with the following

requirements:

(1) There must be a means to lock and safeguard the door against

inadvertent opening during flight by persons, by cargo, or as a result of

mechanical failure.

(2) The door must be openable from the inside and the outside when the

internal locking mechanism is in the locked position.

(3) There must be a means of opening which is simple and obvious and is

arranged and marked inside and outside so that the door can be readily

located, unlocked, and opened, even in darkness.

(4) The door must meet the marking requirements of Sec. 23.811 of this

part.

(5) The door must be reasonably free from jamming as a result of fuselage

deformation in an emergency landing.

(6) Auxiliary locking devices that are actuated externally to the airplane

may be used but such devices must be overridden by the normal internal

opening means.

(d) In addition, each external passenger or crew door, for a commuter

category airplane, must comply with the following requirements:

(1) Each door must be openable from both the inside and outside, even

though persons may be crowded against the door on the inside of the airplane.

(2) If inward opening doors are used, there must be a means to prevent

occupants from crowding against the door to the extent that would interfere

with opening the door.

(3) Auxiliary locking devices may be used.

(e) Each external door on a commuter category airplane, each external door

forward of any engine or propeller on a normal, utility, or acrobatic

category airplane, and each door of the pressure vessel on a pressurized

airplane must comply with the following requirements:

(1) There must be a means to lock and safeguard each external door,

including cargo and service type doors, against inadvertent opening in

flight, by persons, by cargo, or as a result of mechanical failure or failure

of a single structural element, either during or after closure.

(2) There must be a provision for direct visual inspection of the locking

mechanism to determine if the external door, for which the initial opening

movement is not inward, is fully closed and locked. The provisions must be

discernible, under operating lighting conditions, by a crewmember using a

flashlight or an equivalent lighting source.

(3) There must be a visual warning means to signal a flight crewmember if

the external door is not fully closed and locked. The means must be designed

so that any failure, or combination of failures, that would result in an

erroneous closed and locked indication is improbable for doors for which the

initial opening movement is not inward.

[Docket No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-36, 53 FR 30813, Aug. 15, 1988]

Sec. 23.785  Seats, berths, litters, safety belts, and shoulder harnesses.

(a) Each seat/restraint system and the supporting structure must be

designed to support occupants weighing at least 215 pounds when subjected to

the maximum load factors corresponding to the specified flight and ground

load conditions, as defined in the approved operating envelope of the

airplane. In addition, these loads must be multiplied by a factor of 1.33 in

determining the strength of all fittings and the attachment of--

(1) Each seat to the structure; and

(2) Each safety belt and shoulder harness to the seat or structure.

(b) Each forward-facing or aft-facing seat/restraint system in normal,

utility, or acrobatic category airplanes must consist of a seat, safety belt,

and shoulder harness that are designed to provide the occupant protection

provisions required in Sec. 23.562 of this part. Other seat orientations must

provide the same level of occupant protection as a forward-facing or aft-

facing seat with a safety belt and shoulder harness, and provide the

protection provisions of Sec. 23.562 of this part.

(c) For commuter category airplanes, each seat and the supporting structure

must be designed for occupants weighing at least 170 pounds when subjected to

the inertia loads resulting from the ultimate static load factors prescribed

in Sec. 23.561(b)(2) of this part, and each occupant must be protected from

serious head injury when subjected to the inertia loads resulting from these

load factors by a safety belt and shoulder harness for the front seats; and a

safety belt, or a safety belt and shoulder harness, for each seat other than

the front seats.

(d) Each restraint system must have a single-point release for occupant

evacuation.

(e) The restraint system for each crewmember must allow the crewmember,

when seated with the safety belt and shoulder harness fastened, to perform

all functions necessary for flight operations.

(f) Each pilot seat must be designed for the reactions resulting from the

application of pilot forces to the primary flight controls as prescribed in

Sec. 23.395 of this part.

(g) There must be a means to secure each safety belt and shoulder harness,

when not in use, to prevent interference with the operation of the airplane

and with rapid occupant egress in an emergency.

(h) Unless otherwise placarded, each seat in a utility or acrobatic

category airplane must be designed to accommodate an occupant wearing a

parachute.

(i) The cabin area surrounding each seat, including the structure, interior

walls, instrument panel, control wheel, pedals, and seats within striking

distance of the occupant's head or torso (with the restraint system fastened)

must be free of potentially injurious objects, sharp edges, protuberances,

and hard surfaces. If energy absorbing designs or devices are used to meet

this requirement, they must protect the occupant from serious injury when the

occupant is subjected to the inertia loads resulting from the ultimate static

load factors prescribed in Sec. 23.561(b)(2) of this part, or they must

comply with the occupant protection provisions of Sec. 23.562 of this part,

as required in paragraphs (b) and (c) of this section.

(j) Each seat track must be fitted with stops to prevent the seat from

sliding off the track.

(k) Each seat/restraint system may use design features, such as crushing or

separation of certain components, to reduce occupant loads when showing

compliance with the requirements of Sec. 23.562 of this part; otherwise, the

system must remain intact.

(l) For the purposes of this section, a front seat is a seat located at a

flight crewmember station or any seat located alongside such a seat.

(m) Each berth, or provisions for a litter, installed parallel to the

longitudinal axis of the airplane, must be designed so that the forward part

has a padded end-board, canvas diaphragm, or equivalent means that can

withstand the load reactions from a 215-pound occupant when subjected to the

inertia loads resulting from the ultimate static load factors of Sec.

23.561(b)(2) of this part. In addition--

(1) Each berth or litter must have an occupant restraint system and may not

have corners or other parts likely to cause serious injury to a person

occupying it during emergency landing conditions; and

(2) Occupant restraint system attachments for the berth or litter must

withstand the inertia loads resulting from the ultimate static load factors

of Sec. 23.561(b)(2) of this part.

(n) Proof of compliance with the static strength requirements of this

section for seats and berths approved as part of the type design and for seat

and berth installations may be shown by--

(1) Structural analysis, if the structure conforms to conventional airplane

types for which existing methods of analysis are known to be reliable;

(2) A combination of structural analysis and static load tests to limit

load; or

(3) Static load tests to ultimate loads.

[Amdt. 23-36, 53 FR 30813, Aug. 15, 1988; Amdt. 23-36, 54 FR 50737, Dec. 11,

1989]

Sec. 23.787   Baggage and cargo compartments.

(a) Each cargo compartment must be designed for its placarded maximum

weight of contents and for the critical load distributions at the appropriate

maximum load factors corresponding to the flight and ground load conditions

of this part.

(b) There must be means to prevent the contents of any cargo compartment

from becoming a hazard by shifting, and to protect any controls, wiring,

lines, equipment or accessories whose damage or failure would affect safe

operations.

(c) There must be a means to protect occupants from injury by the contents

of any baggage or cargo compartment, located aft of the occupants and

separated by structure, when the ultimate forward inertia load factor is 9g

and assuming the maximun allowed baggage or cargo weight for the compartment.

(d) Cargo compartments must be constructed of materials which are at least

flame resistant.

(e) Designs which provide for baggage or cargo to be carried in the same

compartment as passengers must have a means to protect the occupants from

injury when the cargo is subjected to the inertia loads resulting from the

ultimate static load factors of Sec. 23.561(b)(3) of this part, assuming the

maximum allowed baggage or cargo weight for the compartment.

(f) If cargo compartment lamps are installed, each lamp must be installed

so as to prevent contact between lamp bulb and cargo.

(g) Baggage compartments used in commuter category airplanes must also meet

the requirements of paragraphs (a), (b), (d), and (f) of this section.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31822, Nov. 19, 1973; Amdt. 23-17, 41 FR 55464, Dec. 20, 1976; Amdt. 23-34,

52 FR 1830, Jan. 15, 1987; Amdt. 23-34, 52 FR 34745, Sept. 14, 1987; Amdt.

23-36, 53 FR 30814, Aug. 15, 1988; 53 FR 34194, Sept. 2, 1988]

Sec. 23.803   Emergency evacuation.

For commuter category airplanes, an evacuation demonstration must be

conducted utilizing the maximum number of occupants for which certification

is desired. The demonstration must be conducted under simulated night

conditions using only the emergency exits on the most critical side of the

airplane. The participants must be representative of average airline

passengers with no prior practice or rehearsal for the demonstration.

Evacuation must be completed within 90 seconds.

[Amdt. 23-34, 52 FR 1831, Jan. 15, 1987]

Sec. 23.807  Emergency exits.

(a) Number and location. Emergency exits must be located to allow escape

without crowding in any probable crash attitude. The airplane must have at

least the following emergency exits:

(1) For all airplanes with a seating capacity of two or more, excluding

airplanes with canopies, at least one emergency exit on the opposite side of

the cabin from the main door specified in Sec. 23.783 of this part.

(2) [Reserved]

(3) If the pilot compartment is separated from the cabin by a door that is

likely to block the pilot's escape in a minor crash, there must be an exit in

the pilot's compartment. The number of exits required by paragraph (a)(1) of

this section must then be separately determined for the passenger

compartment, using the seating capacity of that compartment.

(b) Type and operation. Emergency exits must be movable windows, panels,

canopies, or external doors, openable from both inside and outside the

airplane, that provide a clear and unobstructed opening large enough to admit

a 19-by-26-inch ellipse. Auxiliary locking devices used to secure the

airplane must be designed to be overridden by the normal internal opening

means. In addition, each emergency exit must--

(1) Be readily accessible, requiring no exceptional agility to be used in

emergencies;

(2) Have a method of opening that is simple and obvious;

(3) Be arranged and marked for easy location and operation, even in

darkness;

(4) Have reasonable provisions against jamming by fuselage deformation; and

(5) In the case of acrobatic category airplanes, allow each occupant to

bail out quickly with parachutes at any speed between VSO and VD.

(c) Tests. The proper functioning of each emergency exit must be shown by

tests.

(d) Doors and exits. In addition, for commuter category airplanes the

following requirements apply:

(1) The passenger entrance door must qualify as a floor level emergency

exit. If an integral stair is installed at such a passenger entry door, the

stair must be designed so that when subjected to the inertia forces specified

in Sec. 23.561, and following the collapse of one or more legs of the landing

gear, it will not interfere to an extent that will reduce the effectiveness

of emergency egress through the passenger entry door. Each additional

required emergency exit, except floor level exits, must be located over the

wing or must be provided with acceptable means to assist the occupants in

descending to the ground. In addition to the passenger entrance door--

(i) For a total passenger seating capacity of 15 or less, an emergency exit

as defined in paragraph (b) of this section is required on each side of the

cabin; and

(ii) For a total passenger seating capacity of 16 through 19, three

emergency exits, as defined in paragraph (b) of this section, are required

with one on the same side as the door and two on the side opposite the door.

(2) A means must be provided to lock each emergency exit and to safeguard

against its opening in flight, either inadvertently by persons or as a result

of mechanical failure. In addition, a means for direct visual inspection of

the locking mechanism must be provided to determine that each emergency exit

for which the initial opening movement is outward is fully locked.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13092, Aug. 13, 1969; Amdt. 23-10, 36 FR 2864, Feb. 11, 1971; Amdt. 23-34, 52

FR 1831, Jan. 15, 1987; Amdt. 23-36, 53 FR 30814, Aug. 15, 1988; 53 FR 34194,

Sept. 2, 1988]

Sec. 23.811   Emergency exit marking.

(a) Each emergency exit and external door in the passenger compartment must

be externally marked and readily identifiable from outside the airplane by--

(1) A conspicuous visual identification scheme; and

(2) A permanent decal or placard on or adjacent to the emergency exit which

shows the means of opening the emergency exit, including any special

instructions, if applicable.

(b) In addition, for commuter category airplanes, these exits and doors

must be internally marked with the word "exit" by a sign which has white

letters 1 inch high on a red background 2 inches high, be self-illuminated or

independently, internally electrically illuminated, and have a minimum

brightness of at least 160 microlamberts. The color may be reversed if the

passenger compartment illumination is essentially the same.

[Amdt. 23-36, 53 FR 30814, Aug. 15, 1988; 53 FR 34194, Sept. 2, 1988]

Sec. 23.813   Emergency exit access.

For commuter category airplanes, access to window-type emergency exits may

not be obstructed by seats or seat backs.

[Amdt. 23-36, 53 FR 30815, Aug. 15, 1988]

Sec. 23.815   Width of aisle.

For commuter category airplanes, the width of the main passenger aisle at

any point between seats must equal or exceed the values in the following

table:

Minimum main

passenger aisle

width

Less

than 25

Number of     inches   25 inches

passenger      from     and more

seats       floor    from floor

10 through 19  9 inches  15 inches.

[Amdt. 23-34, 52 FR 1831, Jan. 15, 1987]

Sec. 23.831  Ventilation.

(a) Each passenger and crew compartment must be suitably ventilated. Carbon

monoxide concentration may not exceed one part in 20,000 parts of air.

(b) For pressurized airplanes, the ventilating air in the flightcrew and

passenger compartments must be free of harmful or hazardous concentrations

of gases and vapors in normal operations and in the event of reasonably

probable failures or malfunctioning of the ventilating, heating,

pressurization, or other systems and equipment. If accumulation of hazardous

quantities of smoke in the cockpit area is reasonably probable, smoke

evacuation must be readily accomplished starting with full pressurization

and without depressurizing beyond safe limits.

[Docket No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-34, 52 FR 1831, Jan. 15, 1987; Amdt. 23-42, 56 FR 354,

Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Pressurization

Sec. 23.841  Pressurized cabins.

(a) If certification for operation over 31,000 feet is requested, the

airplane must be able to maintain a cabin pressure altitude of not more than

15,000 feet in event of any probable failure or malfunction in the

pressurization system.

(b) Pressurized cabins must have at least the following valves, controls,

and indicators, for controlling cabin pressure:

(1) Two pressure relief valves to automatically limit the positive pressure

differential to a predetermined value at the maximum rate of flow delivered

by the pressure source. The combined capacity of the relief valves must be

large enough so that the failure of any one valve would not cause an

appreciable rise in the pressure differential. The pressure differential is

positive when the internal pressure is greater than the external.

(2) Two reverse pressure differential relief valves (or their equivalent)

to automatically prevent a negative pressure differential that would damage

the structure. However, one valve is enough if it is of a design that

reasonably precludes its malfunctioning.

(3) A means by which the pressure differential can be rapidly equalized.

(4) An automatic or manual regulator for controlling the intake or exhaust

airflow, or both, for maintaining the required internal pressures and airflow

rates.

(5) Instruments to indicate to the pilot the pressure differential, the

cabin pressure altitude, and the rate of change of cabin pressure altitude.

(6) Warning indication at the pilot station to indicate when the safe or

preset pressure differential is exceeded and when a cabin pressure altitude

of 10,000 feet is exceeded.

(7) A warning placard for the pilot if the structure is not designed for

pressure differentials up to the maximum relief valve setting in combination

with landing loads.

(8) A means to stop rotation of the compressor or to divert airflow from

the cabin if continued rotation of an engine-driven cabin compressor or

continued flow of any compressor bleed air will create a hazard if a

malfunction occurs.

[Amdt. 23-14, 38 FR 31822, Nov. 19, 1973, as amended by Amdt. 23-17, 41 FR

55464, Dec. 20, 1976]

Sec. 23.843  Pressurization tests.

(a) Strength test. The complete pressurized cabin, including doors,

windows, canopy, and valves, must be tested as a pressure vessel for the

pressure differential specified in Sec. 23.365(d).

(b) Functional tests. The following functional tests must be performed:

(1) Tests of the functioning and capacity of the positive and negative

pressure differential valves, and of the emergency release valve, to simulate

the effects of closed regulator valves.

(2) Tests of the pressurization system to show proper functioning under

each possible condition of pressure, temperature, and moisture, up to the

maximum altitude for which certification is requested.

(3) Flight tests, to show the performance of the pressure supply, pressure

and flow regulators, indicators, and warning signals, in steady and stepped

climbs and descents at rates corresponding to the maximum attainable within

the operating limitations of the airplane, up to the maximum altitude for

which certification is requested.

(4) Tests of each door and emergency exit, to show that they operate

properly after being subjected to the flight tests prescribed in paragraph

(b) (3) of this section.

Fire Protection

Sec. 23.851   Fire extinguishers.

(a) There must be at least one hand fire extinguisher for use in the pilot

compartment that is located within easy access of the pilot while seated.

(b) There must be at least one hand fire extinguisher located conveniently

in the passenger compartment--

(1) Of each airplane accommodating more than 6 passengers; and

(2) Of each commuter category airplane.

(c) For hand fire extinguishers, the following apply:

(1) The type and quantity of each extinguishing agent used must be

appropriate to the kinds of fire likely to occur where that agent is to be

used.

(2) Each extinguisher for use in a personnel compartment must be designed

to minimize the hazard of toxic gas concentrations.

[Amdt. 23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.853  Compartment interiors.

For each compartment to be used by the crew or passengers:

(a) The materials must be at least flame-resistant;

(b) [Reserved]

(c) If smoking is to be prohibited, there must be a placard so stating, and

if smoking is to be allowed--

(1) There must be an adequate number of self-contained, removable ashtrays;

and

(2) Where the crew compartment is separated from the passenger compartment,

there must be at least one illuminated sign (using either letters or symbols)

notifying all passengers when smoking is prohibited. Signs which notify when

smoking is prohibited must--

(i) When illuminated, be legible to each passenger seated in the passenger

cabin under all probable lighting conditions; and

(ii) Be so constructed that the crew can turn the illumination on and off;

and

(d) In addition, for commuter category airplanes the following requirements

apply:

(1) Each disposal receptacle for towels, paper, or waste must be fully

enclosed and constructed of at least fire resistant materials and must

contain fires likely to occur in it under normal use. The ability of the

disposal receptacle to contain those fires under all probable conditions of

wear, misalignment, and ventilation expected in service must be demonstrated

by test. A placard containing the legible words "No Cigarette Disposal" must

be located on or near each disposal receptacle door.

(2) Lavatories must have "No Smoking" or "No Smoking in Lavatory" placards

located conspicuously on each side of the entry door and self-contained,

removable ashtrays located conspicuously on or near the entry side of each

lavatory door, except that one ashtray may serve more than one lavatory door

if it can be seen from the cabin side of each lavatory door served. The

placards must have red letters at least 1/2  inch high on a white background

at least 1 inch high (a "No Smoking" symbol may be included on the placard).

(3) Materials (including finishes or decorative surfaces applied to the

materials) used in each compartment occupied by the crew or passengers must

meet the following test criteria as applicable:

(i) Interior ceiling panels, interior wall panels, partitions, galley

structure, large cabinet walls, structural flooring, and materials used in

the construction of stowage compartments (other than underseat stowage

compartments and compartments for stowing small items such as magazines and

maps) must be self-extinguishing when tested vertically in accordance with

the applicable portions of Appendix F of this part or by other equivalent

methods. The average burn length may not exceed 6 inches and the average

flame time after removal of the flame source may not exceed 15 seconds.

Drippings from the test specimen may not continue to flame for more than an

average of 3 seconds after falling.

(ii) Floor covering, textiles (including draperies and upholstery), seat

cushions, padding, decorative and nondecorative coated fabrics, leather,

trays and galley furnishings, electrical conduit, thermal and acoustical

insulation and insulation covering, air ducting, joint and edge covering,

cargo compartment liners, insulation blankets, cargo covers and

transparencies, molded and thermoformed parts, air ducting joints, and trim

strips (decorative and chafing), that are constructed of materials not

covered in paragraph (d)(3)(iv) of this section must be self extinguishing

when tested vertically in accordance with the applicable portions of Appendix

F of this part or other approved equivalent methods. The average burn length

may not exceed 8 inches and the average flame time after removal of the flame

source may not exceed 15 seconds. Drippings from the test specimen may not

continue to flame for more than an average of 5 seconds after falling.

(iii) Motion picture film must be safety film meeting the Standard

Specifications for Safety Photographic Film PH1.25 (available from the

American National Standards Institute, 1430 Broadway, New York, N.Y. 10018)

or an FAA approved equivalent. If the film travels through ducts, the ducts

must meet the requirements of paragraph (d)(3)(ii) of this section.

(iv) Acrylic windows and signs, parts constructed in whole or in part of

elastomeric materials, edge-lighted instrument assemblies consisting of two

or more instruments in a common housing, seatbelts, shoulder harnesses, and

cargo and baggage tiedown equipment, including containers, bins, pallets,

etc., used in passenger or crew compartments, may not have an average burn

rate greater than 2.5 inches per minute when tested horizontally in

accordance with the applicable portions of Appendix F of this part or by

other approved equivalent methods.

(v) Except for electrical wire cable insulation, and for small parts (such

as knobs, handles, rollers, fasteners, clips, grommets, rub strips, pulleys,

and small electrical parts) that the Administrator finds would not contribute

significantly to the propagation of a fire, materials in items not specified

in paragraphs (d)(3) (i), (ii), (iii), or (iv) of this section may not have a

burn rate greater than 4.0 inches per minute when tested horizontally in

accordance with the applicable portions of Appendix F of this part or by

other approved equivalent methods.

(e) Lines, tanks, or equipment containing fuel, oil, or other flammable

fluids may not be installed in such compartments unless adequately shielded,

isolated, or otherwise protected so that any breakage or failure of such an

item would not create a hazard.

(f) Airplane materials located on the cabin side of the firewall must be

self-extinguishing or be located at such a distance from the firewall, or

otherwise protected, so that ignition will not occur if the firewall is

subjected to a flame temperature of not less than 2,000 degrees F for 15

minutes. For self-extinguishing materials (except electrical wire and cable

insulation and small parts that the Administrator finds would not contribute

significantly to the propagation of a fire), a vertifical self-extinguishing

test must be conducted in accordance with Appendix F of this part or an

equivalent method approved by the Administrator. The average burn length of

the material may not exceed 6 inches and the average flame time after removal

of the flame source may not exceed 15 seconds. Drippings from the material

test specimen may not continue to flame for more than an average of 3 seconds

after falling.

[Amdt. 23-14, 23 FR 31822, Nov. 19, 1973, as amended by Amdt. 23-23, 43 FR

50593, Oct. 30, 1978; Amdt. 23-25, 45 FR 7755, Feb. 4, 1980; Amdt. 23-34, 52

FR 1831, Jan. 15, 1987]

Sec. 23.859  Combustion heater fire protection.

(a) Combustion heater fire regions. The following combustion heater fire

regions must be protected from fire in accordance with the applicable

provisions of Secs.23.1182 through 23.1191 and 23.1203:

(1) The region surrounding the heater, if this region contains any

flammable fluid system components (excluding the heater fuel system) that

could--

(i) Be damaged by heater malfunctioning; or

(ii) Allow flammable fluids or vapors to reach the heater in case of

leakage.

(2) The region surrounding the heater, if the heater fuel system has

fittings that, if they leaked, would allow fuel vapor to enter this region.

(3) The part of the ventilating air passage that surrounds the combustion

chamber.

(b) Ventilating air ducts. Each ventilating air duct passing through any

fire region must be fireproof. In addition--

(1) Unless isolation is provided by fireproof valves or by equally

effective means, the ventilating air duct downstream of each heater must be

fireproof for a distance great enough to ensure that any fire originating in

the heater can be contained in the duct; and

(2) Each part of any ventilating duct passing through any region having a

flammable fluid system must be constructed or isolated from that system so

that the malfunctioning of any component of that system cannot introduce

flammable fluids or vapors into the ventilating airstream.

(c) Combustion air ducts. Each combustion air duct must be fireproof for a

distance great enough to prevent damage from backfiring or reverse flame

propagation. In addition--

(1) No combustion air duct may have a common opening with the ventilating

airstream unless flames from backfires or reverse burning cannot enter the

ventilating airstream under any operating condition, including reverse flow

or malfunctioning of the heater or its associated components; and

(2) No combustion air duct may restrict the prompt relief of any backfire

that, if so restricted, could cause heater failure.

(d) Heater controls: general. Provision must be made to prevent the

hazardous accumulation of water or ice on or in any heater control component,

control system tubing, or safety control.

(e) Heater safety controls. (1) Each combustion heater must have the

following safety controls:

(i) Means independent of the components for the normal continuous control

of air temperature, airflow, and fuel flow must be provided to automatically

shut off the ignition and fuel supply to that heater at a point remote from

that heater when any of the following occurs:

(A) The heater exchanger temperature exceeds safe limits.

(B) The ventilating air temperature exceeds safe limits.

(C) The combustion airflow becomes inadequate for safe operation.

(D) The ventilating airflow becomes inadequate for safe operation.

(ii) Means to warn the crew when any heater whose heat output is essential

for safe operation has been shut off by the automatic means prescribed in

paragraph (e)(1)(i@of this section.

(2) The means for complying with paragraph (e)(1)(i) of this section for

any individual heater must--

(i) Be independent of components serving any other heater whose heat output

is essential for safe operations; and

(ii) Keep the heater off until restarted by the crew.

(f) Air intakes. Each combustion and ventilating air intake must be located

so that no flammable fluids or vapors can enter the heater system under any

operating condition--

(1) During normal operation; or

(2) As a result of the malfunctioning of any other component.

(g) Heater exhaust. Heater exhaust systems must meet the provisions of

Secs. 23.1121 and 23.1123. In addition, there must be provisions in the

design of the heater exhaust system to safely expel the products of

combustion to prevent the occurrence of--

(1) Fuel leakage from the exhaust to surrounding compartments;

(2) Exhaust gas impingement on surrounding equipment or structure;

(3) Ignition of flammable fluids by the exhaust, if the exhaust is in a

compartment containing flammable fluid lines; and

(4) Restrictions in the exhaust system to relieve backfires that, if so

restricted, could cause heater failure.

(h) Heater fuel systems. Each heater fuel system must meet each powerplant

fuel system requirement affecting safe heater operation. Each heater fuel

system component within the ventilating airstream must be protected by

shrouds so that no leakage from those components can enter the ventilating

airstream.

(i) Drains. There must be means to safely drain fuel that might accumulate

within the combustion chamber or the heater exchanger. In addition--

(1) Each part of any drain that operates at high temperatures must be

protected in the same manner as heater exhausts; and

(2) Each drain must be protected from hazardous ice accumulation under any

operating condition.

[Amdt. 23--27, 45 FR 70387, Oct. 23, 1980]

Sec. 23.863   Flammable fluid fire protection.

(a) In each area where flammable fluids or vapors might escape by leakage

of a fluid system, there must be means to minimize the probability of

ignition of the fluids and vapors, and the resultant hazard if ignition does

occur.

(b) Compliance with paragraph (a) of this section must be shown by analysis

or tests, and the following factors must be considered:

(1) Possible sources and paths of fluid leakage, and means of detecting

leakage.

(2) Flammability characteristics of fluids, including effects of any

combustible or absorbing materials.

(3) Possible ignition sources, including electrical faults, overheating of

equipment, and malfunctioning of protective devices.

(4) Means available for controlling or extinguishing a fire, such as

stopping flow of fluids, shutting down equipment, fireproof containment, or

use of extinguishing agents.

(5) Ability of airplane components that are critical to safety of flight to

withstand fire and heat.

(c) If action by the flight crew is required to prevent or counteract a

fluid fire (e.g. equipment shutdown or actuation of a fire extinguisher),

quick acting means must be provided to alert the crew.

(d) Each area where flammable fluids or vapors might escape by leakage of a

fluid system must be identified and defined.

[Amdt. 23-23, 43 FR 50593, Oct. 30, 1978]

Sec. 23.865   Fire protection of flight controls, engine mounts, and other

flight structure.

Flight controls, engine mounts, excluding those portions that are

certificated as part of the engine, and other flight structure located in the

engine compartment must be constructed of fireproof material or shielded so

that they are capable of withstanding the effects of a fire. Engine vibration

isolators must incorporate suitable features to ensure that the engine is

retained if the non-fireproof portions of the isolators deteriorate from the

effects of a fire.

[Amdt. 23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

Lightning Evaluation

Sec. 23.867  Lightning protection of structure.

(a) The airplane must be protected against catastrophic effects from

lightning.

(b) For metallic components, compliance with paragraph (a) of this section

may be shown by--

(1) Bonding the components properly to the airframe; or

(2) Designing the components so that a strike will not endanger the

airplane.

(c) For nonmetallic components, compliance with paragraph (a) of this

section may be shown by--

(1) Designing the components to minimize the effect of a strike; or

(2) Incorporating acceptable means of diverting the resulting electrical

current so as not to endanger the airplane.

[Amdt. 23-7, 34 FR 13092, Aug. 13, 1969]

Miscellaneous

Sec. 23.871  Leveling means.

There must be means for determining when the airplane is in a level

position on the ground.

[Amdt. 23-7, 34 FR 13092, Aug. 13, 1969]

Subpart E--Powerplant

General

Sec. 23.901  Installation.

(a) For the purpose of this part, the airplane powerplant installation

includes each component that--

(1) Is necessary for propulsion; and

(2) Affects the safety of the major propulsive units.

(b) Each powerplant installation must be constructed and arranged to--

(1) Ensure safe operation to the maximum altitude for which approval is

requested.

(2) Be accessible for necessary inspections and maintenance.

(c) Engine cowls and nacelles must be easily removable or openable by the

pilot to provide adequate access to and exposure of the engine compartment

for preflight checks.

(d) Each turbine engine installation must be constructed and arranged to--

(1) Result in vibration characteristics that do not exceed those

established during the type certification of the engine.

(2) Provide continued safe operation without a hazardous loss of power or

thrust while being operated in rain for at least 3 minutes with the rate of

water ingestion being not less than 4 percent by weight, of the engine

induction airflow rate at the maximum installed power or thrust approved for

takeoff and at flight idle. The engine must accelerate and decelerate safely

following stabilized operation under these rain conditions.

(e) The installation must comply with--

(1) The instructions provided under the engine type certificate and the

propeller type certificate.

(2) The applicable provisions of this subpart.

(f) Each auxiliary power unit installation must meet the applicable

portions of this part.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13092, Aug. 13, 1969; Amdt. 23-18, 42 FR 15041, Mar. 17, 1977; Amdt. 23-29,

49 FR 6846, Feb. 23, 1984; Amdt. 23-34, 52 FR 1832, Jan. 15, 1987; Amdt. 23-

34, 52 FR 34745, Sept. 14, 1987; Amdt. 23-43, 58 FR 18970, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.903   Engines.

(a) Engine type certificate.

(1) Each engine must have a type certificate and must meet the applicable

requirements of part 34 of this chapter.

(2) Each turbine engine must either--

(i) Comply with Sec. 33.77 of this chapter in effect on October 31, 1974,

or as later amended; or

(ii) Be shown to have a foreign object ingestion service history in similar

installation locations which has not resulted in any unsafe condition.

(b) Turbine engine installations. For turbine engine installations--

(1) Design precautions must be taken to minimize the hazards to the

airplane in the event of an engine rotor failure or of a fire originating

inside the engine which burns through the engine case.

(2) The powerplant systems associated with engine control devices, systems,

and instrumentation must be designed to give reasonable assurance that those

operating limitations that adversely affect turbine rotor structural

integrity will not be exceeded in service.

(c) The powerplants must be arranged and isolated from each other to allow

operation, in at least one configuration, so that the failure or malfunction

of any engine, or the failure or malfunction (including destruction by fire

in the engine compartment) of any system that can affect an engine (other

than a fuel tank if only one fuel tank is installed), will not:

(1) Prevent the continued safe operation of the remaining engines; or

(2) Require immediate action by any crewmember for continued safe operation

of the remaining engines.

(d) Starting and stopping (piston engine).

(1) The design of the installation must be such that risk of fire or

mechanical damage to the engine or airplane, as a result of starting the

engine in any conditions in which starting is to be permitted, is reduced to

a minimum. Any techniques and associated limitations for engine starting must

be established and included in the Airplane Flight Manual, approved manual

material, or applicable operating placards. Means must be provided for--

(i) Restarting any engine of a multiengine airplane in flight, and

(ii) Stopping any engine in flight, after engine failure, if continued

engine rotation would cause a hazard to the airplane.

(2) In addition, for commuter category airplanes, the following apply:

(i) Each component of the stopping system on the engine side of the

firewall that might be exposed to fire must be at least fire resistant.

(ii) If hydraulic propeller feathering systems are used for this purpose,

the feathering lines must be at least fire resistant under the operating

conditions that may be expected to exist during feathering.

(e) Starting and stopping (turbine engine). Turbine engine installations

must comply with the following:

(1) The design of the installation must be such that risk of fire or

mechanical damage to the engine or the airplane, as a result of starting the

engine in any conditions in which starting is to be permitted, is reduced to

a minimum. Any techniques and associated limitations must be established and

included in the Airplane Flight Manual, approved manual material, or

applicable operating placards.

(2) There must be means for stopping combustion within any engine and for

stopping the rotation of any engine if continued rotation would cause a

hazard to the airplane. Each component of the engine stopping system located

in any fire zone must be fire resistant. If hydraulic propeller feathering

systems are used for stopping the engine, the hydraulic feathering lines or

hoses must be fire resistant.

(3) It must be possible to restart an engine in flight. Any techniques and

associated limitations must be established and included in the Airplane

Flight Manual, approved manual material, or applicable operating placards.

(4) It must be demonstrated in flight that when restarting engines

following a false start, all fuel or vapor is discharged in such a way that

it does not constitute a fire hazard.

(f) Restart capability. An altitude and airspeed envelope must be

established for the airplane for in-flight engine restarting and each

installed engine must have a restart capability within that envelope.

(g) For turbine engine powered airplanes, if the minimum windmilling speed

of the engines, following the in-flight shutdown of all engines, is

insufficient to provide the necessary electrical power for engine ignition, a

power source independent of the engine-driven electrical power generating

system must be provided to permit in-flight engine ignition for restarting.

[Amdt. 23-14, 38 FR 31822, Nov. 19, 1973, as amended by Amdt. 23-17, 41 FR

55464, Dec. 20, 1976; Amdt. 23-26, 45 FR 60171, Sept. 11, 1980; Amdt. 23-29,

49 FR 6847, Feb. 23, 1984; Amdt. 23-34, 52 FR 1832, Jan. 15, 1987; Amdt. 23-

40, 55 FR 32861, Aug. 10, 1990; Amdt. 23-43, 58 FR 18970, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.904  Automatic power reserve system.

If installed, an automatic power reserve (APR) system that automatically

advances the power or thrust on the operating engine(s), when any engine

fails during takeoff, must comply with appendix H of this part.

[Amdt. 23-43, 58 FR 18970, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.905   Propellers.

(a) Each propeller must have a type certificate.

(b) Engine power and propeller shaft rotational speed may not exceed the

limits for which the propeller is certificated.

(c) Each featherable propeller must have a means to unfeather it in flight.

(d) Each component of the propeller blade pitch control system must meet

the requirements of Sec. 35.42 of this chapter.

(e) All areas of the airplane forward of the pusher propeller that are

likely to accumulate and shed ice into the propeller disc during any

operating condition must be suitably protected to prevent ice formation, or

it must be shown that any ice shed into the propeller disc will not create a

hazardous condition.

(f) Each pusher propeller must be marked so that the disc is conspicuous

under normal daylight ground conditions.

(g) If the engine exhaust gases are discharged into the pusher propeller

disc, it must be shown by tests, or analysis supported by tests, that the

propeller is capable of continuous safe operation.

(h) All engine cowling, access doors, and other removable items must be

designed to ensure that they will not separate from the airplane and contact

the pusher propeller.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-26, 45 FR

60171, Sept. 11, 1980; Amdt. 23-29, 49 FR 6847, Feb. 23, 1984; Amdt. 23-43,

58 FR 18970, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.907  Propeller vibration.

(a) Each propeller with metal blades or highly stressed metal components

must be shown to have vibration stresses, in normal operating conditions,

that do not exceed values that have been shown by the propeller manufacturer

to be safe for continuous operation. This must be shown by--

(1) Measurement of stresses through direct testing of the propeller;

(2) Comparison with similar installations for which these measurements have

been made; or

(3) Any other acceptable test method or service experience that proves the

safety of the installation.

(b) Proof of safe vibration characteristics for any type of propeller,

except for conventional, fixed-pitch, wood propellers must be shown where

necessary.

Sec. 23.909  Turbocharger systems.

(a) Each turbocharger must be approved under the engine type certificate or

it must be shown that the turbocharger system, while in its normal engine

installation and operating in the engine environment--

(1) Can withstand, without defect, an endurance test of 150 hours that

meets the applicable requirements of Sec. 33.49 of this subchapter; and

(2) Will have no adverse effect upon the engine.

(b) Control system malfunctions, vibrations, and abnormal speeds and

temperatures expected in service may not damage the turbocharger compressor

or turbine.

(c) Each turbocharger case must be able to contain fragments of a

compressor or turbine that fails at the highest speed that is obtainable with

normal speed control devices inoperative.

(d) Each intercooler installation, where provided, must comply with the

following--

(1) The mounting provisions of the intercooler must be designed to

withstand the loads imposed on the system;

(2) It must be shown that, under the installed vibration environment, the

intercooler will not fail in a manner allowing portions of the intercooler to

be ingested by the engine; and

(3) Airflow through the intercooler must not discharge directly on any

airplane component (e.g., windshield) unless such discharge is shown to cause

no hazard to the airplane under all operating conditions.

(e) Engine power, cooling characteristics, operating limits, and procedures

affected by the turbocharger system installations must be evaluated.

Turbocharger operating procedures and limitations must be included in the

Airplane Flight Manual in accordance with Sec. 23.1581.

[Amdt. 23-7, 34 FR 13092, Aug. 13, 1969, as amended by Amdt. 23-43, 58 FR

18970, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.925  Propeller clearance.

Unless smaller clearances are substantiated, propeller clearances with the

airplane at maximum weight, with the most adverse center of gravity, and with

the propeller in the most adverse pitch position, may not be less than the

following:

(a) Ground clearance. There must be a clearance of at least seven inches

(for each airplane with nose wheel landing gear) or nine inches (for each

airplane with tail wheel landing gear) between each propeller and the ground

with the landing gear statically deflected and in the level, normal takeoff,

or taxing attitude, whichever is most critical. In addition, for each

airplane with conventional landing gear struts using fluid or mechanical

means for absorbing landing shocks, there must be positive clearance between

the propeller and the ground in the level takeoff attitude with the critical

tire completely deflated and the corresponding landing gear strut bottomed.

Positive clearance for airplanes using leaf spring struts is shown with a

deflection corresponding to 1.5g.

(b) Aft-mounted propellers. In addition to the clearances specified in

paragraph (a) of this section, the airplane must be designed such that the

propeller will not contact the runway surface when the airplane is in the

maximum pitch attitude attainable during normal takeoff and landings. If a

tail wheel, bumper, or an energy absorption device is provided to show

compliance with this paragraph, the following apply:

(1) Suitable design loads must be established for the tail wheel, bumper,

or energy absorption device; and

(2) The supporting structure of the tail wheel, bumper, or energy

absorption device must be designed to withstand the loads established in

paragraph (b)(1) of this section and inspection/replacement criteria must be

established for the tail wheel, bumper, or energy absorption device and

provided as part of the information required by Sec. 23.1529.

(c) Water clearance. There must be a clearance of at least 18 inches

between each propeller and the water, unless compliance with Sec. 23.239 can

be shown with a lesser clearance.

(d) Structural clearance. There must be--

(1) At least one inch radial clearance between the blade tips and the

airplane structure, plus any additional radial clearance necessary to prevent

harmful vibration;

(2) At least one-half inch longitudinal clearance between the propeller

blades or cuffs and stationary parts of the airplane; and

(3) Positive clearance between other rotating parts of the propeller or

spinner and stationary parts of the airplane.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18971, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.929  Engine installation ice protection.

Propellers (except wooden propellers) and other components of complete

engine installations must be protected against the accumulation of ice as

necessary to enable satisfactory functioning without appreciable loss of

power when operated in the icing conditions for which certification is

requested.

[Amdt. 23-14, 33 FR 31822, Nov. 19, 1973]

Sec. 23.933  Reversing systems.

(a) For turbojet and turbofan reversing systems. (1) Each system intended

for ground operation only must be designed so that no single failure or

malfunction of the system will result in unwanted reverse thrust under any

expected operating condition. Failure of structural elements need not be

considered if the probability of this type of failure is extremely remote.

(2) Each system intended for in-flight use must be designed so that no

unsafe condition will result during normal operation of the system, or from

any failure, or likely combination of failures, of the reversing system under

any operating condition including ground operation. Failure of structural

elements need not be considered if the probability of this type of failure is

extremely remote.

(3) Each system must have a means to prevent the engine from producing more

than idle forward thrust when the reversing system malfunctions; except that

it may produce any greater forward thrust that is shown to allow directional

control to be maintained, with aerodynamic means alone, under the most

critical reversing condition expected in operation.

(b) For propeller reversing systems. (1) Each system must be designed so

that no single failure, likely combination of failures or malfunction of the

system will result in unwanted reverse thrust under any operating condition.

Failure of structural elements need not be considered if the probability of

this type of failure is extremely remote.

(2) Compliance with paragraph (a)(1) of this section must be shown by

failure analysis, or testing, or both, for propeller systems that allow the

propeller blades to move from the flight low-pitch position to a position

that is substantially less than the normal flight, low-pitch position. The

analysis may include or be supported by the analysis made to show compliance

with Sec. 35.21 for the type certification of the propeller and associated

installation components. Credit will be given for pertinent analysis and

testing completed by the engine and propeller manufacturers.

[Amdt. 23-43, 58 FR 18971, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.934   Turbojet and turbofan engine thrust reverser systems tests.

Thrust reverser systems of turbojet or turbofan engines must meet the

requirements of Sec. 33.97 of this chapter or it must be demonstrated by

tests that engine operation and vibratory levels are not affected.

[Amdt. 23-43, 58 FR 18971, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.937  Turbopropeller-drag limiting systems.

(a) Turbopropeller-powered airplane propeller-drag limiting systems must

be designed so that no single failure or malfunction of any of the systems

during normal or emergency operation results in propeller drag in excess of

that for which the airplane was designed under the structural requirements of

this part. Failure of structural elements of the drag limiting systems need

not be considered if the probability of this kind of failure is extremely

remote.

(b) As used in this section, drag limiting systems include manual or

automatic devices that, when actuated after engine power loss, can move the

propeller blades toward the feather position to reduce windmilling drag to a

safe level.

[Amdt. 23-7, 34 FR 13093, Aug. 13, 1969, as amended by Amdt. 23-43, 58 FR

18971, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.939  Powerplant operating characteristics.

(a) Turbine engine powerplant operating characteristics must be

investigated in flight to determine that no adverse characteristics (such as

stall, surge, or flameout) are present, to a hazardous degree, during normal

and emergency operation within the range of operating limitations of the

airplane and of the engine.

(b) Turbocharged reciprocating engine operating characteristics must be

investigated in flight to assure that no adverse characteristics, as a result

of an inadvertent overboost, surge, flooding, or vapor lock, are present

during normal or emergency operation of the engine(s) throughout the range of

operating limitations of both airplane and engine.

(c) For turbine engines, the air inlet system must not, as a result of

airflow distortion during normal operation, cause vibration harmful to the

engine.

[Amdt. 23-7, 34 FR 13093 Aug. 13, 1969, as amended by Amdt. 23-14, 38 FR

31823, Nov. 19, 1973; Amdt. 23-18, 42 FR 15041, Mar. 17, 1977; Amdt. 23-42,

56 FR 354, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.943  Negative acceleration.

No hazardous malfunction of an engine, an auxiliary power unit approved for

use in flight, or any component or system associated with the powerplant or

auxiliary power unit may occur when the airplane is operated at the negative

accelerations within the flight envelopes prescribed in Sec. 23.333. This

must be shown for the greatest value and duration of the acceleration

expected in service.

[Amdt. 23-18, 42 FR 15041, Mar. 17, 1977, as amended by Amdt. 23-43, 58 FR

18971, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Fuel System

Sec. 23.951  General.

(a) Each fuel system must be constructed and arranged to ensure fuel flow

at a rate and pressure established for proper engine and auxiliary power unit

functioning under each likely operating condition, including any maneuver for

which certification is requested and during which the engine or auxiliary

power unit is permitted to be in operation.

(b) Each fuel system must be arranged so that--

(1) No fuel pump can draw fuel from more than one tank at a time; or

(2) There are means to prevent introducing air into the system.

(c) Each fuel system for a turbine engine must be capable of sustained

operation throughout its flow and pressure range with fuel initially

saturated with water at 80 deg. F and having 0.75cc of free water per gallon

added and cooled to the most critical condition for icing likely to be

encountered in operation.

(d) Each fuel system for a turbine engine powered airplane must meet the

applicable fuel venting requirements of part 34 of this chapter.

[Amdt. 23-15, 39 FR 35459, Oct. 1, 1974, as amended by Amdt. 23-40, 55 FR

32861, Aug. 10, 1990; Amdt. 23-43, 58 FR 18971, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.953  Fuel system independence.

(a) Each fuel system for a multiengine airplane must be arranged so that,

in at least one system configuration, the failure of any one component (other

than a fuel tank) will not result in the loss of power of more than one

engine or require immediate action by the pilot to prevent the loss of power

of more than one engine.

(b) If a single fuel tank (or series of fuel tanks interconnected to

function as a single fuel tank) is used on a multiengine airplane, the

following must be provided:

(1) Independent tank outlets for each engine, each incorporating a shut-off

valve at the tank. This shutoff valve may also serve as the fire wall shutoff

valve required if the line between the valve and the engine compartment does

not contain more than one quart of fuel (or any greater amount shown to be

safe) that can escape into the engine compartment.

(2) At least two vents arranged to minimize the probability of both vents

becoming obstructed simultaneously.

(3) Filler caps designed to minimize the probability of incorrect

installation or inflight loss.

(4) A fuel system in which those parts of the system from each tank outlet

to any engine are independent of each part of the system supplying fuel to

any other engine.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13093 Aug. 13, 1969; Amdt. 23-43, 58 FR 18971, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.954  Fuel system lightning protection.

The fuel system must be designed and arranged to prevent the ignition of

fuel vapor within the system by--

(a) Direct lightning strikes to areas having a high probability of stroke

attachment;

(b) Swept lightning strokes on areas where swept strokes are highly

probable; and

(c) Corona or streamering at fuel vent outlets.

[Amdt. 23-7, 34 FR 13093, Aug. 13, 1969]

Sec. 23.955  Fuel flow.

(a) General. The ability of the fuel system to provide fuel at the rates

specified in this section and at a pressure sufficient for proper engine

operation must be shown in the attitude that is most critical with respect to

fuel feed and quantity of unusable fuel. These conditions may be simulated in

a suitable mockup. In addition--

(1) The quantity of fuel in the tank may not exceed the amount established

as the unusable fuel supply for that tank under Sec. 23.959 plus that

necessary to show compliance with this section; and

(2) If there is a fuel flowmeter, it must be blocked during the flow test

and the fuel must flow through the meter or its bypass; and

(3) If there is a flowmeter without a bypass, it must not have any failure

mode that would restrict fuel flow below the level required in this fuel flow

demonstration; and

(4) The fuel flow must include that flow needed for vapor return flow, jet

pump drive flow, and for all other purposes for which fuel is used.

(b) Gravity systems. The fuel flow rate for gravity systems (main and

reserve supply) must be 150 percent of the takeoff fuel consumption of the

engine.

(c) Pump systems. The fuel flow rate for each pump system (main and reserve

supply) for each reciprocating engine must be 125 percent of the fuel flow

required by the engine at the maximum takeoff power approved under this part.

(1) This flow rate is required for each main pump and each emergency pump,

and must be available when the pump is operating as it would during takeoff;

(2) For each hand-operated pump, this rate must occur at not more than 60

complete cycles (120 single strokes) per minute.

(3) The fuel pressure, with main and emergency pumps operating

simultaneously, must not exceed the fuel inlet pressure limits of the engine

unless it can be shown that no adverse effect occurs.

(d) Auxiliary fuel systems and fuel transfer systems. Paragraphs (b), (c),

and (f) of this section apply to each auxiliary and transfer system, except

that--

(1) The required fuel flow rate must be established upon the basis of

maximum continuous power and engine rotational speed, instead of takeoff

power and fuel consumption; and

(2) If there is a placard providing operating instructions, a lesser flow

rate may be used for transferring fuel from any auxiliary tank into a larger

main tank. This lesser flow rate must be adequate to maintain engine maximum

continuous power but the flow rate must not overfill the main tank at lower

engine powers.

(e) Multiple fuel tanks. For reciprocating engines that are supplied with

fuel from more than one tank, if engine power loss becomes apparent due to

fuel depletion from the tank selected, it must be possible after switching to

any full tank, in level flight, to obtain 75 percent maximum continuous power

on that engine in not more than--

(1) 10 seconds for naturally aspirated single-engine airplanes;

(2) 20 seconds for turbocharged single-engine airplanes, provided that 75

percent maximum continuous naturally aspirated power is regained within 10

seconds; or

(3) 20 seconds for multiengine airplanes.

(f) Turbine engine fuel systems. Each turbine engine fuel system must

provide at least 100 percent of the fuel flow required by the engine under

each intended operation condition and maneuver. The conditions may be

simulated in a suitable mockup. This flow must--

(1) Be shown with the airplane in the most adverse fuel feed condition

(with respect to altitudes, attitudes, and other conditions) that is expected

in operation; and

(2) For multiengine airplanes, notwithstanding the lower flow rate allowed

by paragraph (d) of this section, be automatically uninterrupted with respect

to any engine until all the fuel scheduled for use by that engine has been

consumed. In addition--

(i) For the purposes of this section, "fuel scheduled for use by that

engine" means all fuel in any tank intended for use by a specific engine.

(ii) The fuel system design must clearly indicate the engine for which fuel

in any tank is scheduled.

(iii) Compliance with this paragraph must require no pilot action after

completion of the engine starting phase of operations.

(3) For single-engine airplanes, require no pilot action after completion

of the engine starting phase of operations unless means are provided that

unmistakenly alert the pilot to take any needed action at least five minutes

prior to the needed action; such pilot action must not cause any change in

engine operation; and such pilot action must not distract pilot attention

from essential flight duties during any phase of operations for which the

airplane is approved.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13093, Aug. 13, 1969; Amdt. 23-43, 58 FR 18971, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.957  Flow between interconnected tanks.

(a) It must be impossible, in a gravity feed system with interconnected

tank outlets, for enough fuel to flow between the tanks to cause an overflow

of fuel from any tank vent under the conditions in Sec. 23.959, except that

full tanks must be used.

(b) If fuel can be pumped from one tank to another in flight, the fuel tank

vents and the fuel transfer system must be designed so that no structural

damage to any airplane component can occur because of overfilling of any

tank.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18972, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.959  Unusable fuel supply.

The unusable fuel supply for each tank must be established as not less than

that quantity at which the first evidence of malfunctioning occurs under the

most adverse fuel feed condition occurring under each intended operation and

flight maneuver involving that tank. Fuel system component failures need not

be considered.

[Amdt. 23-7, 34 FR 13093, Aug. 13, 1969, as amended by Amdt. 23-18, 42 FR

15041, Mar. 17, 1977]

Sec. 23.961   Fuel system hot weather operation.

Each fuel system must be free from vapor lock when using fuel at its

critical temperature, with respect to vapor formation, when operating the

airplane in all critical operating and environmental conditions for which

approval is requested. For turbine fuel, the initial temperature must be 110

deg.F, -0 deg., +5 deg.F or the maximum outside air temperature for which

approval is requested, whichever is more critical.

[Amdt. 23-43, 58 FR 18972, Apr. 9, 1993; 58 FR 27060, May 6, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.963  Fuel tanks: general.

(a) Each fuel tank must be able to withstand, without failure, the

vibration, inertia, fluid, and structural loads that it may be subjected to

in operation.

(b) Each flexible fuel tank liner must be of an acceptable kind.

(c) Each integral fuel tank must have adequate facilities for interior

inspection and repair.

(d) The total usable capacity of the fuel tanks must be enough for at least

one-half hour of operation at maximum continuous power.

(e) Each fuel quantity indicator must be adjusted, as specified in Sec.

23.1337(b), to account for the unusable fuel supply determined under Sec.

23.959.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt 23-34, 52 FR 1832, Jan. 15, 1987; Amdt. 23-43, 58 FR 18972,

Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.965  Fuel tank tests.

(a) Each fuel tank must be able to withstand the following pressures

without failure or leakage:

(1) For each conventional metal tank and nonmetallic tank with walls not

supported by the airplane structure, a pressure of 3.5 p.s.i., or that

pressure developed during maximum ultimate acceleration with a full tank,

whichever is greater.

(2) For each integral tank, the pressure developed during the maximum limit

acceleration of the airplane with a full tank, with simultaneous application

of the critical limit structural loads.

(3) For each nonmetallic tank with walls supported by the airplane

structure and constructed in an acceptable manner using acceptable basic tank

material, and with actual or simulated support conditions, a pressure of 2

p.s.i. for the first tank of a specific design. The supporting structure must

be designed for the critical loads occurring in the flight or landing

strength conditions combined with the fuel pressure loads resulting from the

corresponding accelerations.

(b) Each fuel tank with large, unsupported, or unstiffened flat

surfaces,whose failure or deformation could cause fuel leakage, must be able

to withstand the following test without leakage, failure, or excessive

deformation of the tank walls:

(1) Each complete tank assembly and its support must be vibration tested

while mounted to simulate the actual installation.

(2) Except as specified in paragraph (b)(4) of this section, the tank

assembly must be vibrated for 25 hours at a total displacement of not less

than 1/32  of an inch (unless another displacement is substantiated) while

2/3  filled with water or other suitable test fluid.

(3) The test frequency of vibration must be as follows:

(i) If no frequency of vibration resulting from any rpm within the normal

operating range of engine or propeller speeds is critical, the test frequency

of vibration cycles per minute is obtained by multiplying the maximum

continuous propeller speed in rpm by 0.9 for propeller-driven airplanes, and

for non-propeller-driven airplanes, 2,000 cycles per minute.

(ii) If only one frequency of vibration resulting from any rpm within the

normal operating range of engine or propeller speeds is critical, that

frequency of vibration must be the test frequency.

(iii) If more than one frequency of vibration resulting from any rpm within

the normal operating range of engine or propeller speeds is critical, the

most critical of these frequencies must be the test frequency.

(c) Each integral tank using methods of construction and sealing not

previously proven to be adequate by test data or service experience must be

able to withstand the vibration test specified in paragraphs (b) (1) through

(4) of this section.

(d) Each tank with a nonmetallic liner must be subjected to the sloshing

test outlined in paragraph (b)(5) of this section, with the fuel at room

temperature. In addition, a specimen liner of the same basic construction as

that to be used in the airplane must, when installed in a suitable test tank,

withstand the sloshing test with fuel at a temperature of 110 deg. F.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18972, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.967  Fuel tank installation.

(a) Each fuel tank must be supported so that tank loads are not

concentrated. In addition--

(1) There must be pads, if necessary, to prevent chafing between each tank

and its supports;

(2) Padding must be nonabsorbent or treated to prevent the absorption of

fuel;

(3) If a flexible tank liner is used, it must be supported so that it is

not required to withstand fluid loads;

(4) Interior surfaces adjacent to the liner must be smooth and free from

projections that could cause wear, unless--

(i) Provisions are made for protection of the liner at those points; or

(ii) The construction of the liner itself provides such protection; and

(5) A positive pressure must be maintained within the vapor space of each

bladder cell under any condition of operation, except for a particular

condition for which it is shown that a zero or negative pressure will not

cause the bladder cell to collapse; and

(6) Syphoning of fuel (other than minor spillage) or collapse of bladder

fuel cells may not result from improper securing or loss of the fuel filler

cap.

(b) Each tank compartment must be ventilated and drained to prevent the

accumulation of flammable fluids or vapors. Each compartment adjacent to a

tank that is an integral part of the airplane structure must also be

ventilated and drained.

(c) No fuel tank may be on the engine side of the firewall. There must be

at least one-half inch of clearance between the fuel tank and the firewall.

No part of the engine nacelle skin that lies immediately behind a major air

opening from the engine compartment may act as the wall of an integral tank.

(d) Each fuel tank must be isolated from personnel compartments by a fume-

proof and fuel-proof enclosure that is vented and drained to the exterior of

the airplane. The required enclosure must sustain any personnel compartment

pressurization loads without permanent deformation or failure under the

conditions of Secs. 23.365 and 23.843 of this part. A bladder-type fuel cell,

if used, must have a retaining shell at least equivalent to a metal fuel tank

in structural integrity.

(e) Fuel tanks must be designed, located, and installed so as to retain

fuel:

(1) When subjected to the inertia loads resulting from the ultimate static

load factors prescribed in Sec. 23.561(b)(2) of this part; and

(2) Under conditions likely to occur when the airplane lands on a paved

runway at a normal landing speed under each of the following conditions:

(i) The airplane in a normal landing attitude and its landing gear

retracted.

(ii) The most critical landing gear leg collapsed and the other landing

gear legs extended.

In showing compliance with paragraph (e)(2) of this section, the tearing away

of an engine mount must be considered unless all the engines are installed

above the wing or on the tail or fuselage of the airplane.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13903, Aug. 13, 1969; Amdt. 23-14, 38 FR 31823, Nov. 19, 1973; Amdt. 23-18,

42 FR 15041, Mar. 17, 1977; Amdt. 23-26, 45 FR 60171, Sept. 11, 1980; Amdt.

23-36, 53 FR 30815, Aug. 15, 1988; Amdt. 23-43, 58 FR 18972, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.969  Fuel tank expansion space

Each fuel tank must have an expansion space of not less than two percent of

the tank capacity, unless the tank vent discharges clear of the airplane (in

which case no expansion space is required). It must be impossible to fill the

expansion space inadvertently with the airplane in the normal ground

attitude.

Sec. 23.971  Fuel tank sump.

(a) Each fuel tank must have a drainable sump with an effective capacity,

in the normal ground and flight attitudes, of 0.25 percent of the tank

capacity, or 1/16  gallon, whichever is greater.

(b) Each fuel tank must allow drainage of any hazardous quantity of water

from any part of the tank to its sump with the airplane in the normal ground

attitude.

(c) Each reciprocating engine fuel system must have a sediment bowl or

chamber that is accessible for drainage; has a capacity of 1 ounce for every

20 gallons of fuel tank capacity; and each fuel tank outlet is located so

that, in the normal flight attitude, water will drain from all parts of the

tank except the sump to the sediment bowl or chamber.

(d) Each sump, sediment bowl, and sediment chamber drain required by

paragraphs (a), (b), and (c) of this section must comply with the drain

provisions of Sec. 23.999 (b)(1) and (b)(2).

[Amdt. 23-43, 58 FR 18972, Apr. 9, 1993; 58 FR 27060, May 6, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.973  Fuel tank filler connection.

(a) Each fuel tank filler connection must be marked as prescribed in Sec.

23.1557(c).

(b) Spilled fuel must be prevented from entering the fuel tank compartment

or any part of the airplane other than the tank itself.

(c) Each filler cap must provide a fuel-tight seal for the main filler

opening. However, there may be small openings in the fuel tank cap for

venting purposes or for the purpose of allowing passage of a fuel gauge

through the cap provided such openings comply with the requirements of Sec.

23.975(a).

(d) Each fuel filling point, except pressure fueling connection points,

must have a provision for electrically bonding the airplane to ground fueling

equipment.

(e) For airplanes with engines requiring gasoline as the only permissible

fuel, the inside diameter of the fuel filler opening must be no larger than

2.36 inches.

(f) For airplanes with turbine engines, and not equipped with pressure

fueling provisions, the inside diameter of the fuel filler opening must be no

smaller than 2.95 inches.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-18, 42 FR 15041, Mar. 17, 1977; Amdt. 23-43, 58 FR 18972,

Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.975  Fuel tank vents and carburetor vapor vents.

(a) Each fuel tank must be vented from the top part of the expansion space.

In addition--

(1) Each vent outlet must be located and constructed in a manner that

minimizes the possibility of its being obstructed by ice or other foreign

matter;

(2) Each vent must be constructed to prevent siphoning of fuel during

normal operation;

(3) The venting capacity must allow the rapid relief of excessive

differences of pressure between the interior and exterior of the tank;

(4) Airspaces of tanks with interconnected outlets must be interconnected;

(5) There may be no undrainable points in any vent line where moisture can

accumulate with the airplane in either the ground or level flight attitudes.

Any drain valves installed in the vent lines must discharge clear of the

airplane and be accessible for drainage;

(6) No vent may terminate at a point where the discharge of fuel from the

vent outlet will constitute a fire hazard or from which fumes may enter

personnel compartments; and

(7) Vents must be arranged to prevent the loss of fuel, except fuel

discharged because of thermal expansion, when the airplane is parked in any

direction on a ramp having a one-percent slope.

(b) Each carburetor with vapor elimination connections and each fuel

injection engine employing vapor return provisions must have a separate vent

line to lead vapors back to the top of one of the fuel tanks. If there is

more than one tank and it is necessary to use these tanks in a definite

sequence for any reason, the vapor vent line must lead back to the fuel tank

to be used first, unless the relative capacities of the tanks are such that

return to another tank is preferable.

(c) For acrobatic category airplanes, excessive loss of fuel during

acrobatic maneuvers, including short periods of inverted flight, must be

prevented. It must be impossible for fuel to siphon from the vent when normal

flight has been resumed after any acrobatic maneuver for which certification

is requested.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-18, 42 FR 15041, Mar. 17, 1977; Amdt. 23-29, 49 FR 6847,

Feb. 23, 1984; Amdt. 23-43, 58 FR 18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.977  Fuel tank outlet.

(a) There must be a fuel strainer for the fuel tank outlet or for the

booster pump. This strainer must--

(1) For reciprocating engine powered airplanes, have 8 to 16 meshes per

inch; and

(2) For turbine engine powered airplanes, prevent the passage of any object

that could restrict fuel flow or damage any fuel system component.

(b) The clear area of each fuel tank outlet strainer must be at least five

times the area of the outlet line.

(c) The diameter of each strainer must be at least that of the fuel tank

outlet.

(d) Each strainer must be accessible for inspection and cleaning.

[Amdt. 23-17, 41 FR 55465, Dec. 20, 1976, as amended by Amdt. 23-43, 58 FR

18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.979  Pressure fueling systems.

For pressure fueling systems, the following apply:

(a) Each pressure fueling system fuel manifold connection must have means

to prevent the escape of hazardous quantities of fuel from the system if the

fuel entry valve fails.

(b) An automatic shutoff means must be provided to prevent the quantity of

fuel in each tank from exceeding the maximum quantity approved for that tank.

This means must allow checking for proper shutoff operation before each

fueling of the tank.

(c) A means must be provided to prevent damage to the fuel system in the

event of failure of the automatic shutoff means prescribed in paragraph (b)

of this section.

(d) All parts of the fuel system up to the tank which are subjected to

fueling pressures must have a proof pressure of 1.33 times, and an ultimate

pressure of at least 2.0 times, the surge pressure likely to occur during

fueling.

[Amdt. 23-14, 38 FR 31823, Nov. 19, 1973]

Fuel System Components

Sec. 23.991  Fuel pumps.

(a) Main pumps. For main pumps, the following apply:

(1) For reciprocating engine installations having fuel pumps to supply fuel

to the engine, at least one pump for each engine must be directly driven by

the engine and must meet Sec. 23.955. This pump is a main pump.

(2) For turbine engine installations, each fuel pump required for proper

engine operation, or required to meet the fuel system requirements of this

subpart (other than those in paragraph (b) of this section), is a main pump.

In addition--

(i) There must be at least one main pump for each turbine engine;

(ii) The power supply for the main pump for each engine must be independent

of the power supply for each main pump for any other engine; and

(iii) For each main pump, provision must be made to allow the bypass of

each positive displacement fuel pump other than a fuel injection pump

approved as part of the engine.

(b) Emergency pumps. There must be an emergency pump immediately available

to supply fuel to the engine if any main pump (other than a fuel injection

pump approved as part of an engine) fails. The power supply for each

emergency pump must be independent of the power supply for each corresponding

main pump.

(c) Warning means. If both the main pump and emergency pump operate

continuously, there must be a means to indicate to the appropriate flight

crewmembers a malfunction of either pump.

(d) Operation of any fuel pump may not affect engine operation so as to

create a hazard, regardless of the engine power or thrust setting or the

functional status of any other fuel pump.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13093, Aug. 13, 1969; Amdt. 23-26, 45 FR 60171, Sept. 11, 1980; Amdt. 23-43,

58 FR 18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.993  Fuel system lines and fittings.

(a) Each fuel line must be installed and supported to prevent excessive

vibration and to withstand loads due to fuel pressure and accelerated flight

conditions.

(b) Each fuel line connected to components of the airplane between which

relative motion could exist must have provisions for flexibility.

(c) Each flexible connection in fuel lines that may be under pressure and

subjected to axial loading must use flexible hose assemblies.

(d) Each flexible hose must be shown to be suitable for the particular

application.

(e) No flexible hose that might be adversely affected by exposure to high

temperatures may be used where excessive temperatures will exist during

operation or after engine shutdown.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.994   Fuel system components.

Fuel system components in an engine nacelle or in the fuselage must be

protected from damage which could result in spillage of enough fuel to

constitute a fire hazard as a result of a wheels-up landing on a paved

runway.

[Amdt. 23-29, 49 FR 6847, Feb. 23, 1984]

Sec. 23.995  Fuel valves and controls.

(a) There must be a means to allow appropriate flight crew members to

rapidly shut off, in flight, the fuel to each engine individually.

(b) No shutoff valve may be on the engine side of any firewall. In

addition, there must be means to--

(1) Guard against inadvertent operation of each shutoff valve; and

(2) Allow appropriate flight crew members to reopen each valve rapidly

after it has been closed.

(c) Each valve and fuel system control must be supported so that loads

resulting from its operation or from accelerated flight conditions are not

transmitted to the lines connected to the valve.

(d) Each valve and fuel system control must be installed so that gravity

and vibration will not affect the selected position.

(e) Each fuel valve handle and its connections to the valve mechanism must

have design features that minimize the possibility of incorrect installation.

(f) Each check valve must be constructed, or otherwise incorporate

provisions, to preclude incorrect assembly or connection of the valve.

(g) Fuel tank selector valves must--

(1) Require a separate and distinct action to place the selector in the

"OFF" position; and

(2) Have the tank selector positions located in such a manner that it is

impossible for the selector to pass through the "OFF" position when changing

from one tank to another.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31823, Nov. 19, 1973; Amdt. 23-17, 41 FR 55465, Dec. 20, 1976; Amdt. 23-18,

42 FR 15041, Mar. 17, 1977; Amdt. 23-29, 49 FR 6847, Feb. 23, 1984]

Sec. 23.997  Fuel strainer or filter.

There must be a fuel strainer or filter between the fuel tank outlet and

the inlet of either the fuel metering device or an engine driven positive

displacement pump, whichever is nearer the fuel tank outlet. This fuel

strainer or filter must--

(a) Be accessible for draining and cleaning and must incorporate a screen

or element which is easily removable;

(b) Have a sediment trap and drain except that it need not have a drain if

the strainer or filter is easily removable for drain purposes;

(c) Be mounted so that its weight is not supported by the connecting lines

or by the inlet or outlet connections of the strainer or filter itself,

unless adequate strength margins under all loading conditions are provided in

the lines and connections; and

(d) Have the capacity (with respect to operating limitations established

for the engine) to ensure that engine fuel system functioning is not

impaired, with the fuel contaminated to a degree (with respect to particle

size and density) that is greater than that established for the engine during

its type certification.

(e) In addition, for commuter category airplanes, unless means are provided

in the fuel system to prevent the accumulation of ice on the filter, a means

must be provided to automatically maintain the fuel flow if ice clogging of

the filter occurs.

[Amdt. 23-15, 39 FR 35459, Oct. 1, 1974, as amended by Amdt. 23-29, 49 FR

6847, Feb. 23, 1984; Amdt. 23-34, 52 FR 1832, Jan. 15, 1987; Amdt. 23-43, 58

FR 18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.999  Fuel system drains.

(a) There must be at least one drain to allow safe drainage of the entire

fuel system with the airplane in its normal ground attitude.

(b) Each drain required by paragraph (a) of this section and Sec. 23.971

must--

(1) Discharge clear of all parts of the airplane;

(2) Have a drain valve--

(i) That has manual or automatic means for positive locking in the closed

position;

(ii) That is readily accessible;

(iii) That can be easily opened and closed;

(iv) That allows the fuel to be caught for examination;

(v) That can be observed for proper closing; and

(vi) That is either located or protected to prevent fuel spillage in the

event of a landing with landing gear retracted.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55465, Dec. 20, 1976; Amdt. 23-43, 58 FR 18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1001  Fuel jettisoning system.

(a) If the design landing weight is less than that permitted under the

requirements of Sec. 23.473(b), the airplane must have a fuel jettisoning

system installed that is able to jettison enough fuel to bring the maximum

weight down to the design landing weight. The average rate of fuel

jettisoning must be at least 1 percent of the maximum weight per minute,

except that the time required to jettison the fuel need not be less than 10

minutes.

(b) Fuel jettisoning must be demonstrated at maximum weight with flaps and

landing gear up and in--

(1) A power-off glide at 1.4 VS1;

(2) A climb at the one-engine-inoperative best rate-of-climb speed, with

the critical engine inoperative and the remaining engines at maximum

continuous power; and

(3) Level flight at 1.4 VS1, if the results of the tests in the conditions

specified in paragraphs (b)(1) and (2) of this section show that this

condition could be critical.

(c) During the flight tests prescribed in paragraph (b) of this section, it

must be shown that--

(1) The fuel jettisoning system and its operation are free from fire

hazard;

(2) The fuel discharges clear of any part of the airplane;

(3) Fuel or fumes do not enter any parts of the airplane; and

(4) The jettisoning operation does not adversely affect the controllability

of the airplane.

(d) For reciprocating engine powered airplanes, the jettisoning system must

be designed so that it is not possible to jettison the fuel in the tanks used

for takeoff and landing below the level allowing 45 minutes flight at 75

percent maximum continuous power. However, if there is an auxiliary control

independent of the main jettisoning control, the system may be designed to

jettison all the fuel.

(e) For turbine engine powered airplanes, the jettisoning system must be

designed so that it is not possible to jettison fuel in the tanks used for

takeoff and landing below the level allowing climb from sea level to 10,000

feet and thereafter allowing 45 minutes cruise at a speed for maximum range.

(f) The fuel jettisoning valve must be designed to allow flight crewmembers

to close the valve during any part of the jettisoning operation.

(g) Unless it is shown that using any means (including flaps, slots, and

slats) for changing the airflow across or around the wings does not adversely

affect fuel jettisoning, there must be a placard, adjacent to the jettisoning

control, to warn flight crewmembers against jettisoning fuel while the means

that change the airflow are being used.

(h) The fuel jettisoning system must be designed so that any reasonably

probable single malfunction in the system will not result in a hazardous

condition due to unsymmetrical jettisoning of, or inability to jettison,

fuel.

[Amdt. 23-7, 34 FR 13094, Aug. 13, 1969, as amended by Amdt. 23-43, 58 FR

18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Oil System

Sec. 23.1011  General.

(a) For oil systems and components that have been approved under the engine

airworthiness requirements and where those requirements are equal to or more

severe than the corresponding requirements of subpart E of this part, that

approval need not be duplicated. Where the requirements of subpart E of this

part are more severe, substantiation must be shown to the requirements of

subpart E of this part.

(b) Each engine must have an independent oil system that can supply it with

an appropriate quantity of oil at a temperature not above that safe for

continuous operation.

(c) The usable oil tank capacity may not be less than the product of the

endurance of the airplane under critical operating conditions and the maximum

oil consumption of the engine under the same conditions, plus a suitable

margin to ensure adequate circulation and cooling.

(d) For an oil system without an oil transfer system, only the usable oil

tank capacity may be considered. The amount of oil in the engine oil lines,

the oil radiator, and the feathering reserve, may not be considered.

(e) If an oil transfer system is used, and the transfer pump can pump some

of the oil in the transfer lines into the main engine oil tanks, the amount

of oil in these lines that can be pumped by the transfer pump may be included

in the oil capacity.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1013  Oil tanks.

(a) Installation. Each oil tank must be installed to--

(1) Meet the requirements of Sec. 23.967 (a) and (b); and

(2) Withstand any vibration, inertia, and fluid loads expected in

operation.

(b) Expansion space. Oil tank expansion space must be provided so that--

(1) Each oil tank used with a reciprocating engine has an expansion space

of not less than the greater of 10 percent of the tank capacity or 0.5

gallon, and each oil tank used with a turbine engine has an expansion space

of not less than 10 percent of the tank capacity; and

(2) It is impossible to fill the expansion space inadvertently with the

airplane in the normal ground attitude.

(c) Filler connection. Each oil tank filler connection must be marked as

specified in Sec. 23.1557(c). Each recessed oil tank filler connection of an

oil tank used with a turbine engine, that can retain any appreciable quantity

of oil, must have provisions for fitting a drain.

(d) Vent. Oil tanks must be vented as follows:

(1) Each oil tank must be vented to the engine crankcase from the top part

of the expansion space so that the vent connection is not covered by oil

under any normal flight condition.

(2) Oil tank vents must be arranged so that condensed water vapor that

might freeze and obstruct the line cannot accumulate at any point.

(3) For acrobatic category airplanes, there must be means to prevent

hazardous loss of oil during acrobatic maneuvers, including short periods of

inverted flight.

(e) Outlet. No oil tank outlet may be enclosed by any screen or guard that

would reduce the flow of oil below a safe value at any operating temperature.

No oil tank outlet diameter may be less than the diameter of the engine oil

pump inlet. Each oil tank used with a turbine engine must have means to

prevent entrance into the tank itself, or into the tank outlet, of any object

that might obstruct the flow of oil through the system. There must be a

shutoff valve at the outlet of each oil tank used with a turbine engine,

unless the external portion of the oil system (including oil tank supports)

is fireproof.

(f) Flexible liners. Each flexible oil tank liner must be of an acceptable

kind.

(g) Each oil tank filler cap of an oil tank that is used with an engine

must provide an oiltight seal.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-15, 39 FR

35459 Oct. 1, 1974; Amdt. 23-43, 58 FR 18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1015  Oil tank tests.

Each oil tank must be tested under Sec. 23.965, except that--

(a) The applied pressure must be five p.s.i. for the tank construction

instead of the pressures specified in Sec. 23.965(a);

(b) For a tank with a nonmetallic liner the test fluid must be oil rather

than fuel as specified in Sec. 23.965(d), and the slosh test on a specimen

liner must be conducted with the oil at 250 deg. F.; and

(c) For pressurized tanks used with a turbine engine, the test pressure may

not be less than 5 p.s.i. plus the maximum operating pressure of the tank.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-15, 39 FR

35460, Oct. 1, 1974]

Sec. 23.1017  Oil lines and fittings.

(a) Oil lines. Oil lines must meet Sec. 23.993 and must accommodate a flow

of oil at a rate and pressure adequate for proper engine functioning under

any normal operating condition.

(b) Breather lines. Breather lines must be arranged so that--

(1) Condensed water vapor or oil that might freeze and obstruct the line

cannot accumulate at any point;

(2) The breather discharge will not constitute a fire hazard if foaming

occurs, or cause emitted oil to strike the pilot's windshield;

(3) The breather does not discharge into the engine air induction system;

and

(4) For acrobatic category airplanes, there is no excessive loss of oil

from the breather during acrobatic maneuvers, including short periods of

inverted flight.

(5) The breather outlet is protected against blockage by ice or foreign

matter.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13094, Aug. 13, 1969; Amdt. 23-14, 38 FR 31823, Nov. 19, 1973]

Sec. 23.1019  Oil strainer or filter.

(a) Each turbine engine installation must incorporate an oil strainer or

filter through which all of the engine oil flows and which meets the

following requirements:

(1) Each oil strainer or filter that has a bypass, must be constructed and

installed so that oil will flow at the normal rate through the rest of the

system with the strainer or filter completely blocked.

(2) The oil strainer or filter must have the capacity (with respect to

operating limitations established for the engine) to ensure that engine oil

system functioning is not impaired when the oil is contaminated to a degree

(with respect to particle size and density) that is greater than that

established for the engine for its type certification.

(3) The oil strainer or filter, unless it is installed at an oil tank

outlet, must incorporate a means to indicate contamination before it reaches

the capacity established in accordance with paragraph (a)(2) of this section.

(4) The bypass of a strainer or filter must be constructed and installed so

that the release of collected contaminants is minimized by appropriate

location of the bypass to ensure that collected contaminants are not in the

bypass flow path.

(5) An oil strainer or filter that has no bypass, except one that is

installed at an oil tank outlet, must have a means to connect it to the

warning system required in Sec. 23.1305(c)(9).

(b) Each oil strainer or filter in a powerplant installation using

reciprocating engines must be constructed and installed so that oil will flow

at the normal rate through the rest of the system with the strainer or filter

element completely blocked.

[Amdt. 23-15, 39 FR 35460, Oct. 1, 1974, as amended by Amdt. 23-29, 49 FR

6847, Feb. 23, 1984; Amdt. 23-43, 58 FR 18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1021   Oil system drains.

A drain (or drains) must be provided to allow safe drainage of the oil

system. Each drain must--

(a) Be accessible;

(b) Have drain valves, or other closures, employing manual or automatic

shut-off means for positive locking in the closed position; and

(c) Be located or protected to prevent inadvertent operation.

[Amdt. 23-29, 49 FR 6847, Feb. 23, 1984, as amended by Amdt. 23-43, 58 FR

18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1023  Oil radiators.

Each oil radiator and its supporting structures must be able to withstand

the vibration, inertia, and oil pressure loads to which it would be subjected

in operation.

Sec. 23.1027  Propeller feathering system.

(a) If the propeller feathering system uses engine oil and that oil supply

can become depleted due to failure of any part of the oil system, a means

must be incorporated to reserve enough oil to operate the feathering system.

(b) The amount of reserved oil must be enough to accomplish feathering and

must be available only to the feathering pump.

(c) The ability of the system to accomplish feathering with the reserved

oil must be shown.

(d) Provision must be made to prevent sludge or other foreign matter from

affecting the safe operation of the propeller feathering system.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31823, Nov. 19, 1973; Amdt. 23-43, 58 FR 18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Cooling

Sec. 23.1041  General.

The powerplant and auxiliary power unit cooling provisions must maintain

the temperatures of powerplant components and engine fluids, and auxiliary

power unit components and fluids within the limits established for those

components and fluids under the most adverse ground, water, and flight

operations to the maximum altitude for which approval is requested, and after

normal engine and auxiliary power unit shutdown.

[Amdt. 23-43, 58 FR 18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1043  Cooling tests.

(a) General. Compliance with Sec. 23.1041 must be shown under critical

ground, water, and flight operating conditions to the maximum altitude for

which approval is requested. For turbosupercharged engines, each

turbosupercharger must be operated through that part of the climb profile for

which operation with the turbosupercharger is requested and in a manner

consistent with its intended operation. For these tests, the following apply:

(1) If the tests are conducted under conditions deviating from the maximum

ambient atmospheric temperatures specified in paragraph (b) of this section,

the recorded powerplant temperatures must be corrected under paragraphs (c)

and (d) of this section, unless a more rational correction method is

applicable.

(2) No corrected temperature determined under paragraph (a)(1) of this

section may exceed established limits.

(3) The fuel uses during the cooling tests must be of the minimum grade

approved for the engines, and the mixture settings must be those used in

normal operation.

(4) [Reserved]

(5) Water taxing tests must be conducted on each hull seaplane that may

reasonably be expected to be taxied for extended periods.

(b) Maximum ambient atmospheric temperature. A maximum ambient atmospheric

temperature corresponding to sea level conditions of at least 100 degrees F

must be established. The assumed temperature lapse rate is 3.6 degrees F per

thousand feet of altitude above sea level until a temperature of -69.7

degrees F is reached, above which altitude the temperature is considered

constant at -69.7 degrees F. However, for winterization installations, the

applicant may select a maximum ambient atmospheric temperature corresponding

to sea level conditions of less than 100 degrees F.

(c) Correction factor (except cylinder barrels). Unless a more rational

correction applies, temperatures of engine fluids and powerplant components

(except cylinder barrels) for which temperature limits are established, must

be corrected by adding to them the difference between the maximum ambient

atmospheric temperature and the temperature of the ambient air at the time of

the first occurrence of the maximum component or fluid temperature recorded

during the cooling test.

(d) Correction factor for cylinder barrel temperatures. Cylinder barrel

temperatures must be corrected by adding to them 0.7 times the difference

between the maximum ambient atmospheric temperature and the temperature of

the ambient air at the time of the first occurrence of the maximum cylinder

barrel temperature recorded during the cooling test.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13094, Aug. 13, 1969; Amdt. 23-21, 43 FR 2319, Jan. 16, 1978]

Sec. 23.1045  Cooling test procedures for turbine engine powered airplanes.

(a) Compliance with Sec. 23.1041 must be shown for the takeoff, climb, en

route, and landing stages of flight that correspond to the applicable

performance requirements. The cooling tests must be conducted with the

airplane in the configuration, and operating under the conditions, that are

critical relative to cooling during each stage of flight. For the cooling

tests, a temperature is "stabilized" when its rate of change is less than 2

deg. F. per minute.

(b) Temperatures must be stabilized under the conditions from which entry

is made into each stage of flight being investigated, unless the entry

condition normally is not one during which component and engine fluid

temperatures would stabilize (in which case, operation through the full entry

condition must be conducted before entry into the stage of flight being

investigated in order to allow temperatures to reach their natural levels at

the time of entry). The takeoff cooling test must be preceded by a period

during which the powerplant component and engine fluid temperatures are

stabilized with the engines at ground idle.

(c) Cooling tests for each stage of flight must be continued until--

(1) The component and engine fluid temperatures stabilize;

(2) The stage of flight is completed; or

(3) An operating limitation is reached.

[Amdt. 23-7, 34 FR 13094, Aug. 13, 1969]

Sec. 23.1047  Cooling test procedures for reciprocating engine-powered

airplanes.

(a) For each single-engine airplane powered with a reciprocating engine,

engine cooling tests must be conducted as follows:

(1) Engine temperatures must be stabilized in flight with the engines at

not less than 75 percent of maximum continuous power.

(2) After temperatures have stabilized, a climb must be begun at the lowest

pra@icable altitude and continued for 1 minute with the engine at takeoff

power.

(3) At the end of 1 minute, the climb must be continued at maximum

continuous power for at least 5 minutes after the occurrence of the highest

temperature recorded.

(b) The climb required in paragraph (a) of this section must be conducted

at a speed not more than the best rate-of-climb speed with maximum continuous

power unless--

(1) The slope of the flight path at the speed chosen for the cooling test

is equal to or greater than the minimum required angle of climb determined

under Sec. 23.65; and

(2) The airplane has a cylinder head temperature indicator as specified in

Sec. 23.1305(b)(3).

(c) The stabilizing and climb parts of the test must be conducted with cowl

flap settings selected by the applicant.

(d) For each multiengine airplane powered with reciprocating engines, that

meets the minimum one-engine-inoperative climb performance specified in

Sec. 23.67(b)(1), engine cooling tests must be conducted as follows:

(1) The airplane must be in the configuration specified in Sec. 23.67(a),

except that, when above the critical altitude, the operating engines must be

at maximum continuous power or at full throttle.

(2) The stabilizing and climb parts of the tests must be conducted with

cowl flap settings selected by the applicant.

(3) The temperatures of the operating engines must be stabilized in flight,

with the engines at not less than 75 percent of the maximum continuous power.

(4) After engine temperatures have stabilized, a climb must be--

(i) Begun from 1,000 feet below the critical altitude (or, if this is

impracticable, at the lowest altitude that the terrain will allow) or 1,000

feet below the altitude at which the single-engine-inoperative rate of climb

is 0.02 Vso 2  whichever is lower; and

(ii) Continued for at least 5 minutes after the highest temperature has

been recorded.

(5) The climb must be conducted at a speed not more than the highest speed

at which compliance with the climb requirement of Sec. 23.67(b)(1) can be

shown. If the speed used exceeds the speed for best rate of climb with one

engine inoperative, the airplane must have a cylinder head temperature

indicator as specified in Sec. 23.1337(e).

(e) For each multiengine airplane powered with reciprocating engines that

cannot meet the minimum one-engine-inoperative climb performance specified in

Sec. 23.67(b)(1), engine cooling tests must be conducted as prescribed in

paragraph (d) of this section, except that, after stabilizing temperatures

in flight, the climb (or descent, for airplanes with zero or negative

one-engine-inoperative rates of climb) must be--

(1) Begun as close to sea level as is practicable; and

(2) Conducted at the best rate-of-climb speed (or the speed of minimum rate

of descent, for airplanes with zero or negative one-engine-inoperative rates

of climb).

[Amdt. 23-7, 34 FR 13094, Aug. 13, 1969, as amended by Amdt. 23-21, 43 FR

2319, Jan. 16, 1978; Amdt. 23-42, 56 FR 354, Jan. 3, 1991; Amdt. 23-43, 58 FR

18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Liquid Cooling

Sec. 23.1061  Installation.

(a) General. Each liquid-cooled engine must have an independent cooling

system (including coolant tank) installed so that--

(1) Each coolant tank is supported so that tank loads are distributed over

a large part of the tank surface;

(2) There are pads or other isolation means between the tank and its

supports to prevent chafing.

(3) Pads or any other isolation means that is used must be nonabsorbent or

must be treated to prevent absorption of flammable fluids; and

(4) No air or vapor can be trapped in any part of the system, except the

coolant tank expansion space, during filling or during operation.

(b) Coolant tank. The tank capacity must be at least one gallon, plus 10

percent of the cooling system capacity. In addition--

(1) Each coolant tank must be able to withstand the vibration, inertia, and

fluid loads to which it may be subjected in operation;

(2) Each coolant tank must have an expansion space of at least 10 percent

of the total cooling system capacity; and

(3) It must be impossible to fill the expansion space inadvertently with

the airplane in the normal ground attitude.

(c) Filler connection. Each coolant tank filler connection must be marked

as specified in Sec. 23.1557(c). In addition--

(1) Spilled coolant must be prevented from entering the coolant tank

compartment or any part of the airplane other than the tank itself; and

(2) Each recessed coolant filler connection must have a drain that

discharges clear of the entire airplane.

(d) Lines and fittings. Each coolant system line and fitting must meet the

requirements of Sec. 23.993, except that the inside diameter of the engine

coolant inlet and outlet lines may not be less than the diameter of the

corresponding engine inlet and outlet connections.

(e) Radiators. Each coolant radiator must be able to withstand any

vibration, inertia, and coolant pressure load to which it may normally be

subjected. In addition--

(1) Each radiator must be supported to allow expansion due to operating

temperatures and prevent the transmittal of harmful vibration to the

radiator; and

(2) If flammable coolant is used, the air intake duct to the coolant

radiator must be located so that (in case of fire) flames from the nacelle

cannot strike the radiator.

(f) Drains. There must be an accessible drain that--

(1) Drains the entire cooling system (including the coolant tank, radiator,

and the engine) when the airplane is in the normal ground altitude;

(2) Discharges clear of the entire airplane; and

(3) Has means to positively lock it closed.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18973, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1063  Coolant tank tests.

Each coolant tank must be tested under Sec. 23.965, except that--

(a) The test required by Sec. 23.965(a) (1) must be replaced with a similar

test using the sum of the pressure developed during the maximum ultimate

acceleration with a full tank or a pressure of 3.5 pounds per square inch,

whichever is greater, plus the maximum working pressure of the system; and

(b) For a tank with a nonmetallic liner the test fluid must be coolant

rather than fuel as specified in Sec. 23.965(d), and the slosh test on a

specimen liner must be conducted with the coolant at operating temperature.

Induction System

Sec. 23.1091  Air induction system.

(a) The air induction system for each engine and auxiliary power unit and

their accessories must supply the air required by that engine and auxiliary

power unit and their accessories under the operating conditions for which

certification is requested.

(b) Each reciprocating engine installation must have at least two separate

air intake sources and must meet the following:

(1) Primary air intakes may open within the cowling if that part of the

cowling is isolated from the engine accessory section by a fire-resistant

diaphragm or if there are means to prevent the emergence of backfire flames.

(2) Each alternate air intake must be located in a sheltered position and

may not open within the cowling if the emergence of backfire flames will

result in a hazard.

(3) The supplying of air to the engine through the alternate air intake

system may not result in a loss of excessive power in addition to the power

loss due to the rise in air temperature.

(4) Each automatic alternate air door must have an override means

accessible to the flight crew.

(5) Each automatic alternate air door must have a means to indicate to the

flight crew when it is not closed.

(c) For turbine engine powered airplanes--

(1) There must be means to prevent hazardous quantities of fuel leakage or

overflow from drains, vents, or other components of flammable fluid systems

from entering the engine or auxiliary power unit and their accessories intake

system; and

(2) The airplane must be designed to prevent water, slush or other foreign

material on the runway, taxiway, or other airport operating surface from

being directed into the engine or auxiliary power unit air inlet ducts in

hazardous quantities during takeoff, landing, and taxiing.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13095, Aug. 13, 1969; Amdt. 23-43, 58 FR 18973, Apr. 9, 1993; 58 FR 27060,

May 6, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1093  Induction system icing protection.

(a) Reciprocating engines. Each reciprocating engine air induction system

must have means to prevent and eliminate icing. Unless this is done by other

means, it must be shown that, in air free of visible moisture at a

temperature of 30 deg. F.--

(1) Each airplane with sea level engines using conventional venturi

carburetors has a preheater that can provide a heat rise of 90 deg. F. with

the engines at 75 percent of maximum continuous power;

(2) Each airplane with altitude engines using conventional venturi

carburetors has a preheater that can provide a heat rise of 120 deg. F. with

the engines at 75 percent of maximum continuous power;

(3) Each airplane with altitude engines using fuel metering devices

tending to prevent icing has a preheater that, with the engines at 60 percent

of maximum continuous power, can provide a heat rise of--

(i) 100 deg. F.; or

(ii) 40 deg. F., if a fluid deicing system meeting the requirements of

Secs. 23.1095 through 23.1099 is installed;

(4) Each airplane with sea level engine(s) using fuel metering device

tending to prevent icing has a sheltered alternate source of air with a

preheat of not less than 60 deg.F with the engines at 75 percent of maximum

continuous power;

(5) Each airplane with sea level or altitude engine(s) using fuel injection

systems having metering components on which impact ice may accumulate has a

preheater capable of providing a heat rise of 75 deg.F when the engine is

operating at 75 percent of its maximum continuous power; and

(6) Each airplane with sea level or altitude engine(s) using fuel injection

systems not having fuel metering components projecting into the airstream on

which ice may form, and introducing fuel into the air induction system

downstream of any components or other obstruction on which ice produced by

fuel evaporation may form, has a sheltered alternate source of air with a

preheat of not less than 60 deg.F with the engines at 75 percent of its

maximum continuous power.

(b) Turbine engines.

(1) Each turbine engine and its air inlet system must operate throughout

the flight power range of the engine (including idling), without the

accumulation of ice on engine or inlet system components that would adversely

affect engine operation or cause a serious loss of power or thrust--

(i) Under the icing conditions specified in appendix C of part 25 of this

chapter; and

(ii) In snow, both falling and blowing, within the limitations established

for the airplane for such operation.

(2) Each turbine engine must idle for 30 minutes on the ground, with the

air bleed available for engine icing protection at its critical condition,

without adverse effect, in an atmosphere that is at a temperature between 15

deg. and 30 deg.F (between -9 deg. and -1 deg.C) and has a liquid water

content not less than 0.3 grams per cubic meter in the form of drops having a

mean effective diameter not less than 20 microns, followed by momentary

operation at takeoff power or thrust. During the 30 minutes of idle

operation, the engine may be run up periodically to a moderate power or

thrust setting in a manner acceptable to the Administrator.

(c) For airplanes with reciprocating engines having superchargers to

pressurize the air before it enters the fuel metering device, the heat rise

in the air caused by that supercharging at any altitude may be utilized in

determining compliance with paragraph (a) of this section if the heat rise

utilized is that which will be available, automatically, for the applicable

altitudes and operating condition because of supercharging.

[Amdt. 23-7, 34 FR 13095, Aug. 13, 1969, as amended by Amdt. 23-15, 39 FR

35460, Oct. 1, 1974; Amdt. 23-17, 41 FR 55465, Dec. 20, 1976; Amdt. 23-18, 42

FR 15041, Mar. 17, 1977; Amdt. 23-29, 49 FR 6847, Feb. 23, 1984; Amdt. 23-43,

58 FR 18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1095  Carburetor deicing fluid flow rate.

(a) If a carburetor deicing fluid system is used, it must be able to

simultaneously supply each engine with a rate of fluid flow, expressed in

pounds per hour, of not less than 2.5 times the square root of the maximum

continuous power of the engine.

(b) The fluid must be introduced into the air induction system--

(1) Close to, and upstream of, the carburetor; and

(2) So that it is equally distributed over the entire cross section of the

induction system air passages.

Sec. 23.1097  Carburetor deicing fluid system capacity.

(a) The capacity of each carburetor deicing fluid system--

(1) May not be less than the greater of--

(i) That required to provide fluid at the rate specified in Sec. 23.1095

for a time equal to three percent of the maximum endurance of the airplane;

or

(ii) 20 minutes at that flow rate; and

(2) Need not exceed that required for two hours of operation.

(b) If the available preheat exceeds 50 deg. F. but is less than 100 deg.

F., the capacity of the system may be decreased in proportion to the heat

rise available in excess of 50 deg. F.

Sec. 23.1099  Carburetor deicing fluid system detail design.

Each carburetor deicing fluid system must meet the applicable requirements

for the design of a fuel system, except as specified in Secs. 23.1095 and

23.1097.

Sec. 23.1101   Induction air preheater design.

Each exhaust-heated, induction air preheater must be designed and

constructed to--

(a) Ensure ventilation of the preheater when the induction air preheater is

not being used during engine operation;

(b) Allow inspection of the exhaust manifold parts that it surrounds; and

(c) Allow inspection of critical parts of the preheater itself.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1103  Induction system ducts.

(a) Each induction system duct must have a drain to prevent the

accumulation of fuel or moisture in the normal ground and flight attitudes.

No drain may discharge where it will cause a fire hazard.

(b) Each duct connected to components between which relative motion could

exist must have means for flexibility.

(c) Each flexible induction system duct must be capable of withstanding the

effects of temperature extremes, fuel, oil, water, and solvents to which it

is expected to be exposed in service and maintenance without hazardous

deterioration or delamination.

(d) For reciprocating engine installations, each induction system duct must

be--

(1) Strong enough to prevent induction system failures resulting from

normal backfire conditions; and

(2) Fire resistant in any compartment for which a fire extinguishing system

is required.

(e) Each inlet system duct for an auxiliary power unit must be--

(1) Fireproof within the auxiliary power unit compartment;

(2) Fireproof for a sufficient distance upstream of the auxiliary power

unit compartment to prevent hot gas reverse flow from burning through the

duct and entering any other compartment of the airplane in which a hazard

would be created by the entry of the hot gases;

(3) Constructed of materials suitable to the environmental conditions

expected in service, except in those areas requiring fireproof or fire

resistant materials; and

(4) Constructed of materials that will not absorb or trap hazardous

quantities of flammable fluids that could be ignited by a surge or reverse-

flow condition.

(f) Induction system ducts that supply air to a cabin pressurization system

must be suitably constructed of material that will not produce hazardous

quantities of toxic gases or isolated to prevent hazardous quantities of

toxic gases from entering the cabin during a powerplant fire.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13095, Aug. 13, 1969; Amdt. 23-43, 58 FR 18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1105  Induction system screens.

If induction system screens are used--

(a) Each screen must be upstream of the carburetor;

(b) No screen may be in any part of the induction system that is the only

passage through which air can reach the engine, unless--

(1) The available preheat is at least 100 deg. F.; and

(2) The screen can be deiced by heated air;

(c) No screen may be deiced by alcohol alone; and

(d) It must be impossible for fuel to strike any screen.

Sec. 23.1107   Induction system filters.

On reciprocating-engine installations, if an air filter is used to protect

the engine against foreign material particles in the induction air supply--

(a) Each air filter must be capable of withstanding the effects of

temperature extremes, rain, fuel, oil, and solvents to which it is expected

to be exposed in service and maintenance; and

(b) Each air filter shall have a design feature to prevent material

separated from the filter media from interfering with proper fuel metering

operation.

[Amdt. 23-43, 58 FR 18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1109  Turbocharger bleed air system.

The following applies to turbocharged bleed air systems used for cabin

pressurization:

(a) The cabin air system may not be subject to hazardous contamination

following any probable failure of the turbocharger or its lubrication system.

(b) The turbocharger supply air must be taken from a source where it cannot

be contaminated by harmful or hazardous gases or vapors following any

probable failure or malfunction of the engine exhaust, hydraulic, fuel, or

oil system.

[Doc. No. 25811, 56 FR 354, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.1111  Turbine engine bleed air system.

For turbine engine bleed air systems, the following apply:

(a) No hazard may result if duct rupture or failure occurs anywhere between

the engine port and the airplane unit served by the bleed air.

(b) The effect on airplane and engine performance of using maximum bleed

air must be established.

(c) Hazardous contamination of cabin air systems may not result from

failures of the engine lubricating system.

[Amdt. 23-7, 34 FR 13095, Aug. 13, 1969, as amended by Amdt. 23-17, 41 FR

55465, Dec. 20, 1976]

Exhaust System

Sec. 23.1121  General.

For powerplant and auxiliary power unit installations, the following

apply--

(a) Each exhaust system must ensure safe disposal of exhaust gases without

fire hazard or carbon monoxide contamination in any personnel compartment.

(b) Each exhaust system part with a surface hot enough to ignite flammable

fluids or vapors must be located or shielded so that leakage from any system

carrying flammable fluids or vapors will not result in a fire caused by

impingement of the fluids or vapors on any part of the exhaust system

including shields for the exhaust system.

(c) Each exhaust system must be separated by fireproof shields from

adjacent flammable parts of the airplane that are outside of the engine and

auxiliary power unit compartments.

(d) No exhaust gases may discharge dangerously near any fuel or oil system

drain.

(e) No exhaust gases may be discharged where they will cause a glare

seriously affecting pilot vision at night.

(f) Each exhaust system component must be ventilated to prevent points of

excessively high temperature.

(g) If significant traps exists, each turbine engine exhaust system must

have drains discharging clear of the airplane, in any normal ground and

flight attitude, to prevent fuel accumulation after the failure of an

attempted engine start.

(h) Each exhaust heat exchanger must incorporate means to prevent blockage

of the exhaust port after any internal heat exchanger failure.

(i) For the purpose of compliance with Sec. 23.603, the failure of any part

of the exhaust system will be considered to adversely affect safety.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13095, Aug. 13, 1969; Amdt. 23-18, 42 FR 15042, Mar. 17, 1977; Amdt. 23-43,

18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1123  Exhaust system.

(a) Each exhaust system must be fireproof and corrosion-resistant, and

must have means to prevent failure due to expansion by operating

temperatures.

(b) Each exhaust system must be supported to withstand the vibration and

inertia loads to which it may be subjected in operation.

(c) Parts of the system connected to components between which relative

motion could exist must have means for flexibility.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1125  Exhaust heat exchangers.

For reciprocating engine powered airplanes the following apply:

(a) Each exhaust heat exchanger must be constructed and installed to

withstand the vibration, inertia, and other loads that it may be subjected to

in normal operation. In addition--

(1) Each exchanger must be suitable for continued operation at high

temperatures and resistant to corrosion from exhaust gases;

(2) There must be means for inspection of critical parts of each exchanger;

and

(3) Each exchanger must have cooling provisions wherever it is subject to

contact with exhaust gases.

(b) Each heat exchanger used for heating ventilating air must be

constructed so that exhaust gases may not enter the ventilating air.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55465, Dec. 20, 1976]

Powerplant Controls and Accessories

Sec. 23.1141  Powerplant controls: general.

(a) Powerplant controls must be located and arranged under Sec. 23.777 and

marked under Sec. 23.1555(a).

(b) Each flexible control must be of an acceptable kind.

(c) Each control must be able to maintain any necessary position without--

(1) Constant attention by flight crew members; or

(2) Tendency to creep due to control loads or vibration.

(d) Each control must be able to withstand operating loads without failure

or excessive deflection.

(e) For turbine engine powered airplanes, no single failure or malfunction,

or probable combination thereof, in any powerplant control system may cause

the failure of any powerplant function necessary for safety.

(f) The portion of each powerplant control located in the engine

compartment that is required to be operated in the event of fire must be at

least fire resistant.

(g) Powerplant valve controls located in the cockpit must have--

(1) For manual valves, positive stops or in the case of fuel valves

suitable index provisions, in the open and closed position; and

(2) For power-assisted valves, a means to indicate to the flight crew when

the valve--

(i) Is in the fully open or fully closed position; or

(ii) Is moving between the fully open and fully closed position.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13095, Aug. 13, 1969; Amdt. 23-14, 38 FR 31823, Nov. 19, 1973; Amdt. 23-18,

42 FR 15042, Mar. 17, 1977]

Sec. 23.1142   Auxiliary power unit controls.

Means must be provided on the flight deck for the starting, stopping,

monitoring, and emergency shutdown of each installed auxiliary power unit.

[Amdt. 23-43, 58 FR 18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1143  Engine controls.

(a) There must be a separate power or thrust control for each engine and a

separate control for each supercharger that requires a control.

(b) Power, thrust, and supercharger controls must be arranged to allow--

(1) Separate control of each engine and each supercharger; and

(2) Simultaneous control of all engines and all superchargers.

(c) Each power, thrust, or supercharger control must give a positive and

immediate responsive means of controlling its engine or supercharger.

(d) The power, thrust, or supercharger controls for each engine or

supercharger must be independent of those for every other engine or

supercharger.

(e) For each fluid injection (other than fuel) system and its controls not

provided and approved as part of the engine, the applicant must show that the

flow of the injection fluid is adequately controlled.

(f) If a power or thrust control incorporates a fuel shutoff feature, the

control must have a means to prevent the inadvertent movement of the control

into the shutoff position. The means must--

(1) Have a positive lock or stop at the idle position; and

(2) Require a separate and distinct operation to place the control in the

shutoff position.

(g) For reciprocating single-engine airplanes, each power or thrust control

must be designed so that if the control separates at the engine fuel metering

device, the airplane is capable of continued safe flight and landing.

[Amdt. 23-7, 34 FR 13095, Aug. 13, 1969, as amended by Amdt. 23-17, 41 FR

55465, Dec. 20, 1976; Amdt. 23-29, 49 FR 6847, Feb. 23, 1984; Amdt. 23-43, 58

FR 18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1145  Ignition switches.

(a) Ignition switches must control and shut off each ignition circuit on

each engine.

(b) There must be means to quickly shut off all ignition on multiengine

airplanes by the grouping of switches or by a master ignition control.

(c) Each group of ignition switches, except ignition switches for turbine

engines for which continuous ignition is not required, and each master

ignition control must have a means to prevent its inadvertent operation.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-18, 42 FR 15042, Mar. 17, 1977; Amdt. 23-43, 58 FR 18974,

Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1147  Mixture controls.

(a) If there are mixture controls, each engine must have a separate

control, and each mixture control must have guards or must be shaped or

arranged to prevent confusion by feel with other controls.

(1) The controls must be grouped and arranged to allow--

(i) Separate control of each engine; and

(ii) Simultaneous control of all engines.

(2) The controls must require a separate and distinct operation to move the

control toward lean or shut-off position.

(b) For reciprocating single-engine airplanes, each manual engine mixture

control must be designed so that, if the control separates at the engine fuel

metering device, the airplane is capable of continued safe flight and

landing.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13096, Aug. 13, 1969; Amdt. 23-33, 51 FR 26657, July 24, 1986; Amdt. 23-43,

58 FR 18974, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1149  Propeller speed and pitch controls.

(a) If there are propeller speed or pitch controls, they must be grouped

and arranged to allow--

(1) Separate control of each propeller; and

(2) Simultaneous control of all propellers.

(b) The controls must allow ready synchronization of all propellers on

multiengine airplanes.

Sec. 23.1153  Propeller feathering controls.

If there are propeller feathering controls, each propeller must have a

separate control. Each control must have means to prevent inadvertent

operation.

Sec. 23.1155  Turbine engine reverse thrust and propeller pitch settings

below the flight regime.

For turbine engine installations, each control for reverse thrust and for

propeller pitch settings below the flight regime must have means to prevent

its inadvertent operation. The means must have a positive lock or stop at the

flight idle position and must require a separate and distinct operation by

the crew to displace the control from the flight regime (forward thrust

regime for turbojet powered airplanes).

[Amdt. 23-7, 34 FR 13096, Aug. 13, 1969]

Sec. 23.1157  Carburetor air temperature controls.

There must be a separate carburetor air temperature control for each

engine.

Sec. 23.1163   Powerplant accessories.

(a) Each engine mounted accessory must--

(1) Be approved for mounting on the engine involved and use the provisions

on the engines for mounting; or

(2) Have torque limiting means on all accessory drives in order to prevent

the torque limits established for those drives from being exceeded; and

(3) In addition to paragraphs (a)(1) or (a)(2) of this section, be sealed

to prevent contamination of the engine oil system and the accessory system.

(b) Electrical equipment subject to arcing or sparking must be installed to

minimize the probability of contact with any flammable fluids or vapors that

might be present in a free state.

(c) Each generator rated at or more than 6 kilowatts must be designed and

installed to minimize the probability of a fire hazard in the event it

malfunctions.

(d) If the continued rotation of any accessory remotely driven by the

engine is hazardous when malfunctioning occurs, a means to prevent rotation

without interfering with the continued operation of the engine must be

provided.

(e) Each accessory driven by a gearbox that is not approved as part of the

powerplant driving the gearbox must--

(1) Have torque limiting means to prevent the torque limits established for

the affected drive from being exceeded;

(2) Use the provisions on the gearbox for mounting; and

(3) Be sealed to prevent contamination of the gearbox oil system and the

accessory system.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31823, Nov. 19, 1973; Amdt. 23-29, 49 FR 6847, Feb. 23, 1984; Amdt. 23-34, 52

FR 1832, Jan. 15, 1987; Amdt. 23-42, 56 FR 354, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.1165   Engine ignition systems.

(a) Each battery ignition system must be supplemented by a generator that

is automatically available as an alternate source of electrical energy to

allow continued engine operation if any battery becomes depleted.

(b) The capacity of batteries and generators must be large enough to meet

the simultaneous demands of the engine ignition system and the greatest

demands of any electrical system components that draw from the same source.

(c) The design of the engine ignition system must account for--

(1) The condition of an inoperative generator;

(2) The condition of a completely depleted battery with the generator

running at its normal operating speed; and

(3) The condition of a completely depleted battery with the generator

operating at idling speed, if there is only one battery.

(d) There must be means to warn appropriate crewmembers if malfunctioning

of any part of the electrical system is causing the continuous discharge of

any battery used for engine ignition.

(e) Each turbine engine ignition system must be independent of any

electrical circuit that is not used for assisting, controlling, or analyzing

the operation of that system.

(f) In addition, for commuter category airplanes, each turbopropeller

ignition system must be an essential electrical load.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55465 Dec. 20, 1976; Amdt. 23-34, 52 FR 1833, Jan. 15, 1987]

Sec. 23.1181   Designated fire zones; regions included.

Designated fire zones are--

(a) For reciprocating engines--

(1) The power section;

(2) The accessory section;

(3) Any complete powerplant compartment in which there is no isolation

between the power section and the accessory section.

(b) For turbine engines--

(1) The compressor and accessory sections;

(2) The combustor, turbine and tailpipe sections that contain lines or

components carrying flammable fluids or gases.

(c) Any auxiliary power unit compartment; and

(d) Any fuel-burning heater, and other combustion equipment installation

described in Sec. 23.859;

[Amdt. 23-43, 58 FR 18975, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Powerplant Fire Protection

Sec. 23.1182   Nacelle areas behind firewalls.

Components, lines, and fittings, except those subject to the provisions of

Sec. 23.1351(e), located behind the engine-compartment firewall must be

constructed of such materials and located at such distances from the firewall

that they will not suffer damage sufficient to endanger the airplane if a

portion of the engine side of the firewall is subjected to a flame

temperature of not less than 2000 deg. F for 15 minutes.

[Amdt. 23-14, 38 FR 31816, Nov. 19, 1973]

Sec. 23.1183   Lines, fittings, and components.

(a) Except as provided in paragraph (b) of this section, each component,

line, and fitting carrying flammable fluids, gas, or air in any area subject

to engine fire conditions must be at least fire resistant, except that

flammable fluid tanks and supports which are part of and attached to the

engine must be fireproof or be enclosed by a fireproof shield unless damage

by fire to any non-fireproof part will not cause leakage or spillage of

flammable fluid. Components must be shielded or located so as to safeguard

against the ignition of leaking flammable fluid. Flexible hose assemblies

(hose and end fittings) must be approved. An integral oil sump of less than

25-quart capacity on a reciprocating engine need not be fireproof nor be

enclosed by a fireproof shield.

(b) Paragraph (a) of this section does not apply to--

(1) Lines, fittings, and components which are already approved as part of a

type certificated engine; and

(2) Vent and drain lines, and their fittings, whose failure will not result

in, or add to, a fire hazard.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-5, 32 FR

6912, May 5, 1967; Amdt. 23-15, 39 FR 35460, Oct. 1, 1974; Amdt. 23-29, 49 FR

6847, Feb. 23, 1984]

Sec. 23.1189   Shutoff means.

(a) For each multiengine airplane, the following apply:

(1) Each engine installation must have means to shut off or otherwise

prevent hazardous quantities of fuel, oil, deicing fluid, and other flammable

liquids from flowing into, within, or through any engine compartment, except

in lines, fittings, and components forming an integral part of an engine.

(2) The closing of the fuel shutoff valve for any engine may not make any

fuel unavailable to the remaining engines that would be available to those

engines with that valve open.

(3) Operation of any shutoff means may not interfere with the later

emergency operation of other equipment such as propeller feathering devices.

(4) Each shutoff must be outside of the engine compartment unless an equal

degree of safety is provided with the shutoff inside the compartment.

(5) Not more than one quart of flammable fluid may escape into the engine

compartment after engine shutoff. For those installations where the flammable

fluid that escapes after shutdown cannot be limited to one quart, it must be

demonstrated that this greater amount can be safely contained or drained

overboard.

(6) There must be means to guard against inadvertent operation of each

shutoff means, and to make it possible for the crew to reopen the shutoff

means in flight after it has been closed.

(b) Turbine engine installations need not have an engine oil system shutoff

if--

(1) The oil tank is integral with, or mounted on, the engine; and

(2) All oil system components external to the engine are fireproof or

located in areas not subject to engine fire conditions.

(c) Power operated valves must have means to indicate to the flight crew

when the valve has reached the selected position and must be designed so that

the valve will not move from the selected position under vibration conditions

likely to exist at the valve location.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13096, Aug. 13, 1969; Amdt. 23-14, 38 FR 31823, Nov. 19, 1973; Amdt. 23-29,

49 FR 6847, Feb. 23, 1984; Amdt. 23-43, 58 FR 18975, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1191   Firewalls.

(a) Each engine, auxiliary power unit, fuel burning heater, and other

combustion equipment, must be isolated from the rest of the airplane by

firewalls, shrouds, or equivalent means.

(b) Each firewall or shroud must be constructed so that no hazardous

quantity of liquid, gas, or flame can pass from the isolated compartment to

other parts of the airplane.

(c) Each opening in the firewall or shroud must be sealed with close

fitting, fireproof grommets, bushings, or firewall fittings.

(d) [Reserved]

(e) Each firewall and shroud must be fireproof and protected against

corrosion.

(f) Compliance with the criteria for fireproof materials or components must

be shown as follows:

(1) The flame to which the materials or components are subjected must be

2000 +/- 150 deg.F.

(2) Sheet materials approximately 10 inches square must be subjected to the

flame from a suitable burner.

(3) The flame must be large enough to maintain the required test

temperature over an area approximately five inches square.

(g) Firewall materials and fittings must resist flame penetration for at

least 15 minutes.

(h) The following materials may be used in firewalls or shrouds without

being tested as required by this section:

(1) Stainless steel sheet, 0.015 inch thick.

(2) Mild steel sheet (coated with aluminum or otherwise protected against

corrosion) 0.018 inch thick.

(3) Terne plate, 0.018 inch thick.

(4) Monel metal, 0.018 inch thick.

(5) Steel or copper base alloy firewall fittings.

(6) Titanium sheet, 0.016 inch thick.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18975, Apr. 9, 1993; 58 FR 27060, May 6, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1192   Engine accessory compartment diaphragm.

For aircooled radial engines, the engine power section and all portions of

the exhaust sytem must be isolated from the engine accessory compartment by a

diaphragm that meets the firewall requirements of Sec. 23.1191.

[Amdt. 23-14, 38 FR 31823, Nov. 19, 1973]

Sec. 23.1193   Cowling and nacelle.

(a) Each cowling must be constructed and supported so that it can resist

any vibration, inertia, and air loads to which it may be subjected in

operation.

(b) There must be means for rapid and complete drainage of each part of the

cowling in the normal ground and flight attitudes. Drain operation may be

shown by test, analysis, or both, to ensure that under normal aerodynamic

pressure distribution expected in service each drain will operate as

designed. No drain may discharge where it will cause a fire hazard.

(c) Cowling must be at least fire resistant.

(d) Each part behind an opening in the engine compartment cowling must be

at least fire resistant for a distance of at least 24 inches aft of the

opening.

(e) Each part of the cowling subjected to high temperatures due to its

nearness to exhaust sytem ports or exhaust gas impingement, must be fire

proof.

(f) Each nacelle of a multiengine airplane with supercharged engines must

be designed and constructed so that with the landing gear retracted, a fire

in the engine compartment will not burn through a cowling or nacelle and

enter a nacelle area other than the engine compartment.

(g) In addition, for commuter category airplanes, the airplane must be

designed so that no fire originating in any engine compartment can enter,

either through openings or by burn-through, any other region where it would

create additional hazards.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-18, 42 FR 15042, Mar. 17, 1977; Amdt. 23-34, 52 FR 1833,

Jan. 15, 1987; Amdt. 23-43, 58 FR 18975, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1195   Fire extinguishing systems.

(a) For commuter category airplanes, fire extinguishing systems must be

installed and compliance shown with the following:

(1) Except for combustor, turbine, and tailpipe sections of turbine-engine

installations that contain lines or components carrying flammable fluids or

gases for which a fire originating in these sections is shown to be

controllable, a fire extinguisher system must serve each engine compartment;

(2) The fire extinguishing system, the quantity of the extinguishing agent,

the rate of discharge, and the discharge distribution must be adequate to

extinguish fires. An individual "one shot" system may be used.

(3) The fire extinguishing system for a nacelle must be able to

simultaneously protect each compartment of the nacelle for which protection

is provided.

(b) If an auxiliary power unit is installed in any airplane certificated to

this part, that auxiliary power unit compartment must be served by a fire

extinguishing system meeting the requirements of paragraph (a)(2) of this

section.

[Amdt. 23-34, 52 FR 1833, Jan. 15, 1987, as amended by Amdt. 23-43, 58 FR

18975, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1197   Fire extinguishing agents.

For commuter category airplanes, the following applies:

(a) Fire extinguishing agents must--

(1) Be capable of extinguishing flames emanating from any burning of fluids

or other combustible materials in the area protected by the fire

extinguishing system; and

(2) Have thermal stability over the temperature range likely to be

experienced in the compartment in which they are stored.

(b) If any toxic extinguishing agent is used, provisions must be made to

prevent harmful concentrations of fluid or fluid vapors (from leakage during

normal operation of the airplane or as a result of discharging the fire

extinguisher on the ground or in flight) from entering any personnel

compartment, even though a defect may exist in the extinguishing system. This

must be shown by test except for built-in carbon dioxide fuselage compartment

fire extinguishing systems for which--

(1) Five pounds or less of carbon dioxide will be discharged, under

established fire control procedures, into any fuselage compartment; or

(2) Protective breathing equipment is available for each flight crewmember

on flight deck duty.

[Amdt. 23-34, 52 FR 1833, Jan. 15, 1987]

Sec. 23.1199  Extinguishing agent containers.

For commuter category airplanes, the following applies:

(a) Each extinguishing agent container must have a pressure relief to

prevent bursting of the container by excessive internal pressures.

(b) The discharge end of each discharge line from a pressure relief

connection must be located so that discharge of the fire extinguishing agent

would not damage the airplane. The line must also be located or protected to

prevent clogging caused by ice or other foreign matter.

(c) A means must be provided for each fire extinguishing agent container to

indicate that the container has discharged or that the charging pressure is

below the established minimum necessary for proper functioning.

(d) The temperature of each container must be maintained, under intended

operating conditions, to prevent the pressure in the container from--

(1) Falling below that necessary to provide an adequate rate of discharge;

or

(2) Rising high enough to cause premature discharge.

(e) If a pyrotechnic capsule is used to discharge the extinguishing agent,

each container must be installed so that temperature conditions will not

cause hazardous deterioration of the pyrotechnic capsule.

[Amdt. 23-34, 52 FR 1833, Jan. 15, 1987; 52 FR 34745, Sept. 14, 1987]

Sec. 23.1201  Fire extinguishing system materials.

For commuter category airplanes, the following apply:

(a) No material in any fire extinguishing system may react chemically with

any extinguishing agent so as to create a hazard.

(b) Each system component in an engine compartment must be fireproof.

[Amdt. 23-34, 52 FR 1833, Jan. 15, 1987; 52 FR 7262, Mar. 9, 1987]

Sec. 23.1203   Fire detector system.

(a) There must be means that ensure the prompt detection of a fire in--

(1) An engine compartment of--

(i) Multiengine turbine powered airplanes;

(ii) Multiengine reciprocating engine powered airplanes incorporating

turbochargers;

(iii) Airplanes with engine(s) located where they are not readily visible

from the cockpit; and

(iv) All commuter category airplanes.

(2) The auxiliary power unit compartment of any airplane incorporating an

auxiliary power unit.

(b) Each fire detector must be constructed and installed to withstand the

vibration, inertia, and other loads to which it may be subjected in

operation.

(c) No fire detector may be affected by any oil, water, other fluids, or

fumes that might be present.

(d) There must be means to allow the crew to check, in flight, the

functioning of each fire detector electric circuit.

(e) Wiring and other components of each fire detector system in a fire

zone must be at least fire resistant.

[Amdt. 23-18, 42 FR 15042, Mar. 17, 1977, as amended by Amdt. 23-34, 52 FR

1833, Jan. 15, 1987; Amdt. 23-43, 58 FR 18975, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Subpart F--Equipment

General

Sec. 23.1301  Function and installation.

Each item of installed equipment must--

(a) Be of a kind and design appropriate to its intended function.

(b) Be labeled as to its identification, function, or operating

limitations, or any applicable combination of these factors;

(c) Be installed according to limitations specified for that equipment; and

(d) Function properly when installed.

[Amdt. 23-20, 42 FR 36968, July 18, 1977]

Sec. 23.1303  Flight and navigation instruments.

The following are required flight and navigational instruments:

(a) An airspeed indicator.

(b) An altimeter.

(c) A direction indicator (nonstabilized magnetic compass).

(d) For turbine engine powered airplanes, a free air temperature indicator

or an air-temperature indicator which provides indications that are

convertible to free-air.

(e) A speed warning device for--

(1) Turbine engine powered airplanes; and

(2) Other airplanes for which Vmo/Mmo and Vd/Md are established under Secs.

23.335(b)(4) and 23.1505(c) if Vmo/Mmo is greater than 0.8 Vd/Md.

The speed warning device must give effective aural warning (differing

distinctively from aural warnings used for other purposes) to the pilots

whenever the speed exceeds Vmo plus 6 knots or Mmo+0.01. The upper limit of

the production tolerance for the warning device may not exceed the prescribed

warning speed.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55465, Dec. 20, 1976; Amdt. 23-43, 58 FR 18975, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1305  Powerplant instruments.

The following are required powerplant instruments:

(a) For all airplanes.

(1) A fuel quantity indicator for each fuel tank, installed in accordance

with Sec. 23.1337(b).

(2) An oil pressure indicator for each engine.

(3) An oil temperature indicator for each engine.

(4) An oil quantity measuring device for each oil tank which meets the

requirements of Sec. 23.1337(d).

(5) A fire warning means for those airplanes required to comply with Sec.

23.1203.

(b) For reciprocating engine-powered airplanes. In addition to the

powerplant instruments required by paragraph (a) of this section, the

following powerplant instruments are required:

(1) An induction system air temperature indicator for each engine equipped

with a preheater and having induction air temperature limitations that can be

exceeded with preheat.

(2) A tachometer indicator for each engine.

(3) A cylinder head temperature indicator for--

(i) Each air-cooled engine with cowl flaps;

(ii) Each airplane for which compliance with Sec. 23.1041 is shown at a

speed higher than Vy; and

(iii) Each commuter category airplane.

(4) A fuel pressure indicator for each pump fed engine.

(5) A manifold pressure indicator for each altitude engine and for each

engine with a controllable propeller.

(6) For each turbocharger installation:

(i) If limitations are established for either carburetor (or manifold) air

inlet temperature or exhaust gas or turbocharger turbine inlet temperature,

indicators must be furnished for each temperature for which the limitation is

established unless it is shown that the limitation will not be exceeded in

all intended operations.

(ii) If its oil system is separate from the engine oil system, oil pressure

and oil temperature indicators must be provided.

(7) A coolant temperature indicator for each liquid-cooled engine.

(c) For turbine engine-powered airplanes. In addition to the powerplant

instruments required by paragraph (a) of this section, the following

powerplant instruments are required:

(1) A gas temperature indicator for each engine.

(2) A fuel flowmeter indicator for each engine.

(3) A fuel low pressure warning means for each engine.

(4) A fuel low level warning means for any fuel tank that should not be

depleted of fuel in normal operations.

(5) A tachometer indicator (to indicate the speed of the rotors with

established limiting speeds) for each engine.

(6) An oil low pressure warning means for each engine.

(7) An indicating means to indicate the functioning of the powerplant ice

protection system for each engine.

(8) For each engine, an indicating means for the fuel strainer or filter

required by Sec. 23.997 to indicate the occurrence of contamination of the

strainer or filter before it reaches the capacity established in accordance

with Sec. 23.997(d).

(9) For each engine, a warning means for the oil strainer or filter

required by Sec. 23.1019, if it has no bypass, to warn the pilot of the

occurrence of contamination of the strainer or filter screen before it

reaches the capacity established in accordance with Sec. 23.1019(a)(5).

(10) An indicating means to indicate the functioning of any heater used to

prevent ice clogging of fuel system components.

(d) For turbojet/turbofan engine-powered airplanes. In addition to the

powerplant instruments required by paragraphs (a) and (c) of this section,

the following powerplant instruments are required:

(1) For each engine, an indicator to indicate thrust or to indicate a

parameter that can be related to thrust, including a free air temperature

indicator if needed for this purpose.

(2) For each engine, a position indicating means to indicate to the flight

crew when the thrust reverser, if installed, is in the reverse thrust

position.

(e) For turbopropeller-powered airplanes. In addition to the powerplant

instruments required by paragraphs (a) and (c) of this section, the following

powerplant instruments are required:

(1) A torque indicator for each engine.

(2) A position indicating means to indicate to the flight crew when the

propeller blade angle is below the flight low pitch position, for each

propeller, unless it can be shown that such occurrence is highly improbable.

[Amdt. 23-43, 58 FR 18975, Apr. 9, 1993; 58 FR 27060, May 6, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1307  Miscellaneous equipment.

(a) There must be a seat or berth for each occupant.

(b) The following miscellaneous equipment is required as prescribed in this

subpart:

(1) A master switch arrangement.

(2) An adequate source of electrical energy.

(3) Electrical protective devices.

(c) The equipment necessary for an airplane to operate at the maximum

operating altitude and in the kinds of operations and meteorological

conditions for which certification is requested and is approved in accordance

with Sec. 23.1559 must be included in the type design.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-23, 43 FR 50593, Oct. 30, 1978; Amdt. 23-43, 58 FR 18976,

Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1309  Equipment, systems, and installations.

(a) Each item of equipment, each system, and each installation:

(1) When performing its intended function, may not adversely affect the

response, operation, or accuracy of any--

(i) Equipment essential to safe operation; or

(ii) Other equipment unless there is a means to inform the pilot of the

effect.

(2) In a single-engine airplane, must be designed to minimize hazards to

the airplane in the event of a probable malfunction or failure.

(3) In a multiengine airplane, must be designed to prevent hazards to the

airplane in the event of a probable malfunction or failure.

(b) The design of each item of equipment, each system, and each

installation must be examined separately and in relationship to other

airplane systems and installations to determine if the airplane is dependent

upon its function for continued safe flight and landing and, for airplanes

not limited to VFR conditions, if failure of a system would significantly

reduce the capability of the airplane or the ability of the crew to cope with

adverse operating conditions. Each item of equipment, each system, and each

installation identified by this examination as one upon which the airplane is

dependent for proper functioning to ensure continued safe flight and landing,

or whose failure would significantly reduce the capability of the airplane or

the ability of the crew to cope with adverse operating conditions, must be

designed to comply with the following additional requirements:

(1) It must perform its intended function under any foreseeable operating

condition.

(2) When systems and associated components are considered separately and in

relation to other systems--

(i) The occurrence of any failure condition that would prevent the

continued safe flight and landing of the airplane must be extremely

improbable; and

(ii) The occurrence of any other failure condition that would significantly

reduce the capability of the airplane or the ability of the crew to cope with

adverse operating conditions must be improbable.

(3) Warning information must be provided to alert the crew to unsafe system

operating conditions and to enable them to take appropriate corrective

action. Systems, controls, and associated monitoring and warning means must

be designed to minimize crew errors that could create additional hazards.

(4) Compliance with the requirements of paragraph (b)(2) of this section

may be shown by analysis and, where necessary, by appropriate ground, flight,

or simulator tests. The analysis must consider--

(i) Possible modes of failure, including malfunctions and damage from

external sources;

(ii) The probability of multiple failures, and the probability of

undetected faults.;

(iii) The resulting effects on the airplane and occupants, considering the

stage of flight and operating conditions; and

(iv) The crew warning cues, corrective action required, and the crew's

capability of determining faults.

(c) Each item of equipment, each system, and each installation whose

functioning is required by this chapter and that requires a power supply is

an "essential load" on the power supply. The power sources and the system

must be able to supply the following power loads in probable operating

combinations and for probable durations:

(1) Loads connected to the power distribution system with the system

functioning normally.

(2) Essential loads after failure of--

(i) Any one engine on two-engine airplanes; or

(ii) Any two engines on an airplane with three or more engines; or

(iii) Any power converter or energy storage device.

(3) Essential loads for which an alternate source of power is required, as

applicable, by the operating rules of this chapter, after any failure or

malfunction in any one power supply system, distribution system, or other

utilization system.

(d) In determining compliance with paragraph (c)(2) of this section, the

power loads may be assumed to be reduced under a monitoring procedure

consistent with safety in the kinds of operations authorized. Loads not

required in controlled flight need not be considered for the two-engine-

inoperative condition on airplanes with three or more engines.

(e) In showing compliance with this section with regard to the electrical

power system and to equipment design and installation, critical environmental

and atmospheric conditions, including radio frequency energy and the effects

(both direct and indirect) of lightning strikes, must be considered. For

electrical generation, distribution, and utilization equipment required by or

used in complying with this chapter, the ability to provide continuous, safe

service under forseeable environmental conditions may be shown by

environmental tests, design analysis, or reference to previous comparable

service experience on other airplanes.

(f) As used in this section, "system" refers to all pneumatic systems,

fluid systems, electrical systems, mechanical systems, and powerplant systems

included in the airplane design, except for the following:

(1) Powerplant systems provided as part of the certificated engine.

(2) The flight structure (such a wing, empennage, control surfaces and

their systems, the fuselage, engine mounting, and landing gear and their

related primary attachments) whose requirements are specific in subparts C

and D of this part.

[Doc. No. 25812, Amdt. 23-41, 55 FR 43309, Oct. 26, 1990; 55 FR 47028,

Nov. 8, 1990]

*****************************************************************************

55 FR 43306, No. 208, Oct. 26, 1990

SUMMARY: This final rule amends the airworthiness standards for equipment,

systems, and installations and establishes airworthiness standards for the

installation of electronic display instrument systems in normal, utility,

acrobatic, and commuter category airplanes. It also provides alternative

airworthiness standards for the instrument configuration for general, air

taxi and commercial operations. This amendment updates the airworthiness and

operating requirements to reflect advanced technology being incorporated in

current designs while maintaining an acceptable level of safety.

EFFECTIVE DATE: November 26, 1990.

*****************************************************************************

Instruments: Installation

Sec. 23.1311  Electronic display instrument systems.

(a) Electonic display indicator requirements in this section are

independent to each pilot station required by the airworthiness standards or

by the applicable operating rules for each airplane that is to be approved

for operation in IFR conditions.

(b) Electronic display indicators required by Sec. 23.1303(a), (b), and (c)

must be independent of the airplane's electrical power system.

(c) Electronic display indicators, including those with features that make

isolation and independence between powerplant instrument systems impractical

must--

(1) Be easily legible under all lighting conditions encountered in the

cockpit, including direct sunlight, considering the expected electronic

display brightness level at the end of an electronic display indicator's

useful life. Specific limitations on display system useful life must be

addressed in the Instructions for Continued Airworthiness requirements of

Sec. 23.1529;

(2) Not inhibit the primary display of attitude, airspeed, altitude, or

powerplant parameters needed by any pilot to set power within established

limitations, in any normal mode of operation;

(3) Not inhibit the primary display of engine parameters needed by any

pilot to properly set or monitor powerplant limitations during the engine

starting mode of operation;

(4) Have independent secondary attitude and rate-of-turn instruments that

comply with Sec. 23.1321(a) if the primary electronic display instrument

system for a pilot presents this information. Instrument displays that are

located in accordance with Sec. 23.1321(d) are considered the primary

displays. A rate-of-turn instrument is not required if a third attitude

instrument system is installed in accordance with the instrument requirements

prescribed in Sec. 121.305(j) of this chapter.

(5) Incorporate sensory cues for the pilot that are equivalent to those in

the instrument being replaced by the electronic display indicators; and

(6) Incorporate visual displays of instrument markings, required by Secs.

23.1541 through 23.1553, or visual displays that alert the pilot to abnormal

operational values or approaches to established limitation values, for each

parameter required to be displayed by this part.

(d) The electronic display indicators, including their systems and

installations, and considering other airplane systems, must be designed so

that one display of information essential for continued safe flight and

landing will remain available to the crew, without need for immediate action

by any pilot for continued safe operation, after any single failure or

probable combination of failures.

(e) As used in this section, "instrument" includes devices that are

physically contained in one unit, and devices that are composed of two or

more physically separate units or components connected together (such as a

remote indicating gyroscopic direction indicator that includes a magnetic

sensing element, a gyroscopic unit, an amplifier, and an indicator connected

together). As used in this section, "primary" display refers to the display

of a parameter that is located in the instrument panel such that the pilot

looks at it first when wanting to view that parameter.

[Doc. No. 25812, Amdt. 23-41, 55 FR 43310, Oct. 26, 1990; 55 FR 47028,

Nov. 8, 1990]

*****************************************************************************

55 FR 43306, No. 208, Oct. 26, 1990

SUMMARY: This final rule amends the airworthiness standards for equipment,

systems, and installations and establishes airworthiness standards for the

installation of electronic display instrument systems in normal, utility,

acrobatic, and commuter category airplanes. It also provides alternative

airworthiness standards for the instrument configuration for general, air

taxi and commercial operations. This amendment updates the airworthiness and

operating requirements to reflect advanced technology being incorporated in

current designs while maintaining an acceptable level of safety.

EFFECTIVE DATE: November 26, 1990.

*****************************************************************************

Sec. 23.1321   Arrangement and visibility.

(a) Each flight, navigation, and powerplant instrument for use by any

required pilot during takeoff, initial climb, final approach, and landing

must be located so that any pilot seated at the controls can monitor the

airplane's flight path and these instruments with minimum head and eye

movement. The powerplant instruments for these flight conditions are those

needed to set power within powerplant limitations.

(b) For each multiengine airplane, identical powerplant instruments must be

located so as to prevent confusion as to which engine each instrument

relates.

(c) Instrument panel vibration may not damage, or impair the accuracy of,

any instrument.

(d) For each airplane certificated for flight under instrument flight rules

or of more than 6,000 pounds maximum weight, the flight instruments required

by Sec. 23.1303, and, as applicable, by the operating rules of this chapter,

must be grouped on the instrument panel and centered as nearly as practicable

about the vertical plane of each required pilot's forward vision. In

addition:

(1) The instrument that most effectively indicates the attitude must be on

the panel in the top center position;

(2) The instrument that most effectively indicates airspeed must be

adjacent to and directly to the left of the instrument in the top center

position;

(3) The instrument that most effectively indicates altitude must be

adjacent to and directly to the right of the instrument in the top center

position;

(4) The instrument that most effectively indicates direction of flight,

other than the magnetic direction indicator required by Sec. 23.1303(c), must

be adjacent to and directly below the instrument in the top center position;

and

(5) Electronic display indicators may be used for compliance with

paragraphs (d)(1) through (d)(4) of this section when such displays comply

with requirements in Sec. 23.1311.

(e) If a visual indicator is provided to indicate malfunction of an

instrument, it must be effective under all probable cockpit lighting

conditions.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31824, Nov. 19, 1973; Amdt. 23-20, 42 FR 36968, July 18, 1977; Amdt. 23-41,

55 FR 43310, Oct. 26, 1990; 55 FR 46888, Nov. 7, 1990]

*****************************************************************************

55 FR 43306, No. 208, Oct. 26, 1990

SUMMARY: This final rule amends the airworthiness standards for equipment,

systems, and installations and establishes airworthiness standards for the

installation of electronic display instrument systems in normal, utility,

acrobatic, and commuter category airplanes. It also provides alternative

airworthiness standards for the instrument configuration for general, air

taxi and commercial operations. This amendment updates the airworthiness and

operating requirements to reflect advanced technology being incorporated in

current designs while maintaining an acceptable level of safety.

EFFECTIVE DATE: November 26, 1990.

*****************************************************************************

Sec. 23.1322   Warning, caution, and advisory lights.

If warning, caution, or advisory lights are installed in the cockpit, they

must, unless otherwise approved by the Administrator, be--

(a) Red, for warning lights (lights indicating a hazard which may require

immediate corrective action);

(b) Amber, for caution lights (lights indicating the possible need for

future corrective action);

(c) Green, for safe operation lights; and

(d) Any other color, including white, for lights not described in

paragraphs (a) through (c) of this section, provided the color differs

sufficiently from the colors prescribed in paragraphs (a) through (c) of this

section to avoid possible confusion.

(e) Effective under all probable cockpit lighting conditions.

[Amdt. 23-17, 41 FR 55465, Dec. 20, 1976, as amended by Amdt. 23-43, 58 FR

18976, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1323   Airspeed indicating system.

(a) Each airspeed indicating instrucment must be calibrated to indicate

true airspeed (at sea level with a standard atmosphere) with a minimum

practicable instrument calibration error when the corresponding pitot and

static pressures are applied.

(b) Each airspeed system must be calibrated in flight to determine the

system error. The system error, including position error, but excluding the

airspeed indicator instrument calibration error, may not exceed three percent

of the calibrated airspeed or five knots, whichever is greater, throughout

the following speed ranges:

(1) 1.3 VS1 to VMO/MMO or VNE, whichever is appropriate with flaps

retracted.

(2) 1.3 VS1 to VFE with flaps extended.

(c) In addition, for commuter category airplanes, the airspeed indicating

system must be calibrated to determine the system error in flight and during

the accelerate-takeoff ground run. The ground run calibration must be

obtained between 0.8 of the minimum value of V1, and 1.2 times the maximum

value of V1 considering the approved ranges of altitude and weight. The

ground run calibration must be determined assuming an engine failure at the

minimum value of V1.

(d) For commuter category airplanes, the information showing the

relationship between IAS and CAS determined in accordance with paragraph (c)

of this section must be shown in the Airplane Flight Manual.

(e) If certification for instrument flight rules or flight in icing

conditions is requested, each airspeed system must have a heated pitot tube

or an equivalent means of preventing malfunction due to icing.

[Amdt. 23-20, 42 FR 36968, July 18, 1977, as amended by Amdt. 23-34, 52 FR

1834, Jan. 15, 1987; 52 FR 34745, Sept. 14, 1987; Amdt. 23-42, 56 FR 354,

Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.1325   Static pressure system.

(a) Each instrument provided with static pressure case connections must be

so vented that the influence of airplane speed, the opening and closing of

windows, airflow variations, moisture, or other foreign matter will least

affect the accuracy of the instruments except as noted in paragraph (b)(3) of

this section.

(b) If a static pressure system is necessary for the functioning of

instruments, systems, or devices, it must comply with the provisions of

paragraphs (b) (1) through (3) of this section.

(1) The design and installation of a static pressure system must be such

that--

(i) Positive drainage of moisture is provided;

(ii) Chafing of the tubing, and excessive distortion or restriction at

bends in the tubing, is avoided; and

(iii) The materials used are durable, suitable for the purpose intended,

and protected against corrosion.

(2) A proof test must be conducted to demonstrate the integrity of the

static pressure system in the following manner:

(i) Unpressurized airplanes. Evacuate the static pressure system to a

pressure differential of approximately 1 inch of mercury or to a reading on

the altimeter, 1,000 feet above the aircraft elevation at the time of the

test. Without additional pumping for a period of 1 minute, the loss of

indicated altitude must not exceed 100 feet on the altimeter.

(ii) Pressurized airplanes. Evacuate the static pressure system until a

pressure differential equivalent to the maximum cabin pressure differential

for which the airplane is type certificated is achieved. Without additional

pumping for a period of 1 minute, the loss of indicated altitude must not

exceed 2 percent of the equivalent altitude of the maximum cabin differential

pressure or 100 feet, whichever is greater.

(3) If a static pressure system is provided for any instrument, device, or

system required by the operating rules of this chapter, each static pressure

port must be designed or located in such a manner that the correlation

between air pressure in the static pressure system and true ambient

atmospheric static pressure is not altered when the airplane encounters icing

conditions. An antiicing means or an alternate source of static pressure may

be used in showing compliance with this requirement. If the reading of the

altimeter, when on the alternate static pressure system differs from the

reading of the altimeter when on the primary static system by more than 50

feet, a correction card must be provided for the alternate static system.

(c) Except as provided in paragraph (d) of this section, if the static

pressure system incorporates both a primary and an alternate static pressure

source, the means for selecting one or the other source must be designed so

that--

(1) When either source is selected, the other is blocked off; and

(2) Both sources cannot be blocked off simultaneously.

(d) For unpressurized airplanes, paragraph (c)(1) of this section does not

apply if it can be demonstrated that the static pressure system calibration,

when either static pressure source is selected, is not changed by the other

static pressure source being open or blocked.

(e) Each system must be designed and installed so that the error in

indicated pressure altitude, at sea level, with a standard atmosphere,

excluding instrument calibration error, does not result in an error of more

than +30 feet per 100 knots speed for the appropriate configuration in the

speed range between 1.3 Vs0 with flaps extended and 1.8 Vs1 with flaps

retracted. However, the error need not be less than +30 feet.

(f) For commuter category airplanes, the altimeter system calibration,

required by paragraph (e) of this section, must be shown in the Airplane

Flight Manual.

(g) For airplanes prohibited from flight in instrument meteorological

conditions, in accordance with Sec. 23.1559(b) of this part, paragraph (b)(3)

of this section does not apply.

[Amdt. 23-1, 30 FR 8261, June 29, 1965, as amended by Amdt. 23-6, 32 FR 7586,

May 24, 1967; 32 FR 13505, Sept. 27, 1967; 32 FR 13714, Sept. 30, 1967; Amdt.

23-20, 42 FR 36968, July 18, 1977; Amdt. 23-34, 52 FR 1834, Jan. 15, 1987;

Amdt. 23-42, 56 FR 354, Jan. 3, 1991]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Sec. 23.1327   Magnetic direction indicator.

(a) Except as provided in paragraph (b) of this section--

(1) Each magnetic direction indicator must be installed so that its

accuracy is not excessively affected by the airplane's vibration or magnetic

fields; and

(2) The compensated installation may not have a deviation in level flight,

greater than ten degrees on any heading.

(b) A magnetic nonstabilized direction indicator may deviate more than ten

degrees due to the operation of electrically powered systems such as

electrically heated windshields if either a magnetic stabilized direction

indicator, which does not have a deviation in level flight greater than ten

degrees on any heading, or a gyroscopic direction indicator, is installed.

Deviations of a magnetic nonstabilized direction indicator of more than 10

degrees must be placarded in accordance with Sec. 23.1547(e).

[Amdt. 23-20, 42 FR 36969, July 18, 1977]

Sec. 23.1329   Automatic pilot system.

If an automatic pilot system is installed, it must meet the following:

(a) Each system must be designed so that the automatic pilot can--

(1) Be quickly and positively disengaged by the pilots to prevent it from

interfering with their control of the airplane; or

(2) Be sufficiently overpowered by one pilot to let him control the

airplane.

(b) If the provisions of paragraph (a)(1) of this section are applied, the

quick release (emergency) control must be located on the control wheel (both

control wheels if the airplane can be operated from either pilot seat) on the

side opposite the throttles, or on the stick control, such that it can be

operated without moving the hand from its normal position on the control.

(c) Unless there is automatic synchronization, each system must have a

means to readily indicate to the pilot the alignment of the actuating device

in relation to the control system it operates.

(d) Each manually operated control for the system operation must be readily

accessible to the pilot. Each control must operate in the same plane and

sense of motion as specified in Sec. 23.779 for cockpit controls. The

direction of motion must be plainly indicated on or near each control.

(e) Each system must be designed and adjusted so that, within the range of

adjustment available to the pilot, it cannot produce hazardous loads on the

airplane or create hazardous deviations in the flight path, under any flight

condition appropriate to its use, either during normal operation or in the

event of a malfunction, assuming that corrective action begins within a

reasonable period of time.

(f) Each system must be designed so that a single malfunction will not

produce a hardover signal in more than one control axis. If the automatic

pilot integrates signals from auxiliary controls or furnishes signals for

operation of other equipment, positive interlocks and sequencing of

engagement to prevent improper operation are required.

(g) There must be protection against adverse interaction of integrated

components, resulting from a malfunction.

(h) If the automatic pilot system can be coupled to airborne navigation

equipment, means must be provided to indicate to the flight crew the current

mode of operation. Selector switch position is not acceptable as a means of

indication.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-23, 43 FR 50593, Oct. 30, 1978; Amdt. 23-43, 58 FR 18976,

Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1331   Instruments using a power source.

For each instrument that uses a power source, the following apply:

(a) Each instrument must have an integral visual power annunciator or

separate power indicator to indicate when power is not adequate to sustain

proper instrument performance. If a separate indicator is used, it must be

located so that the pilot using the instruments can monitor the indicator

with minimum head and eye movement. The power must be sensed at or near the

point where it enters the instrument. For electric and vacuum/pressure

instruments, the power is considered to be adequate when the voltage or the

vacuum/pressure, respectively, is within approved limits.

(b) The installation and power supply systems must be designed so that--

(1) The failure of one instrument will not interfere with the proper supply

of energy to the remaining instrument; and

(2) The failure of the energy supply from one source will not interfere

with the proper supply of energy from any other source.

(c) There must be at least two independent sources of power (not driven by

the same engine on multiengine airplanes), and a manual or an automatic means

to select each power source.

[Amdt. 23-43, 58 FR 18976, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1335   Flight director systems.

If a flight director system is installed, means must be provided to

indicate to the flight crew its current mode of operation. Selector switch

position is not acceptable as a means of indication.

[Amdt. 23-20, 42 FR 36969, July 18, 1977]

Sec. 23.1337   Powerplant instruments.

(a) Instruments and instrument lines.

(1) Each powerplant and auxiliary power unit instrument line must meet the

requirements of Sec. 23.993.

(2) Each line carrying flammable fluids under pressure must--

(i) Have restricting orifices or other safety devices at the source of

pressure to prevent the escape of excessive fluid if the line fails; and

(ii) Be installed and located so that the escape of fluids would not create

a hazard.

(3) Each powerplant and auxiliary power unit instrument that utilizes

flammable fluids must be installed and located so that the escape of fluid

would not create a hazard.

(b) Fuel quantity indicator. There must be a means to indicate to the

flight crewmembers the quantity of fuel in each tank during flight. An

indicator, calibrated in either gallons or pounds, and clearly marked to

indicate which scale is being used, may be used. In addition--

(1) Each fuel quantity indicator must be calibrated to read "zero" during

level flight when the quantity of fuel remaining in the tank is equal to the

unusable fuel supply determined under Sec. 23.959;

(2) Each exposed sight gauge used as a fuel quantity indicator must be

protected against damage;

(3) Each sight gauge that forms a trap in which water can collect and

freeze must have means to allow drainage on the ground;

(4) Tanks with interconnected outlets and airspaces may be considered as

one tank and need not have separate indicators; and

(5) No fuel quantity indicator is required for an auxiliary tank that is

used only to transfer fuel to other tanks if the relative size of the tank,

the rate of fuel transfer, and operating instructions are adequate to--

(i) Guard against overflow; and

(ii) Give the flight crewmembers prompt warning if transfer is not

proceeding as planned.

(c) Fuel flowmeter system. If a fuel flowmeter system is installed, each

metering component must have a means to by-pass the fuel supply if

malfunctioning of that component severely restricts fuel flow.

(d) Oil quantity indicator. There must be a means to indicate the quantity

of oil in each tank--

(1) On the ground (such as by a stick gauge); and

(2) In flight, to the flight crew members, if there is an oil transfer

system or a reserve oil supply system.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13096, Aug. 13, 1969; Amdt. 23-18, 42 FR 15042, Mar. 17, 1977; Amdt. 23-43,

58 FR 18976, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Electrical Systems and Equipment

Sec. 23.1351   General.

(a) Electrical system capacity. Each electrical system must be adequate for

the intended use. In addition--

(1) Electric power sources, their transmission cables, and their associated

control and protective devices, must be able to furnish the required power at

the proper voltage to each load circuit essential for safe operation; and

(2) Compliance with paragraph (a)(1) of this section must be shown as

follows--

(i) For normal, utility, and acrobatic category airplanes, by an electrical

load analysis or by electrical measurements that account for the electrical

loads applied to the electrical system in probable combinations and for

probable durations; and

(ii) For commuter category airplanes, by an electrical load analysis that

accounts for the electrical loads applied to the electrical system in

probable combinations and for probable durations.

(b) Function. For each electrical system, the following apply:

(1) Each system, when installed, must be--

(i) Free from hazards in itself, in its method of operation, and in its

effects on other parts of the airplane;

(ii) Protected from fuel, oil, water, other detrimental substances, and

mechanical damage; and

(iii) So designed that the risk of electrical shock to crew, passengers,

and ground personnel is reduced to a minimum.

(2) Electric power sources must function properly when connected in

combination or independently, except alternators installed in normal,

utility, and acrobatic category airplanes, may depend on a battery for

initial excitation or for stabilization.

(3) No failure or malfunction of any electric power source may impair the

ability of any remaining source to supply load circuits essential for safe

operation, except the operation of an alternator that depends on a battery

for initial excitation or for stabilization may be stopped by failure of that

battery in normal, utility, and acrobatic category airplanes.

(4) Each electric power source control must allow the independent operation

of each source, except in normal, utility and acrobatic category airplanes,

controls associated with alternators which depend on a battery for initial

excitation or for stabilization need not break the connection between the

alternator and its battery.

(5) In addition, for commuter category airplanes, the following apply:

(i) Each system must be designed so that essential load circuits can be

supplied in the event of reasonably probable faults or open circuits

including faults in heavy current carrying cables;

(ii) A means must be accessible in flight to the flight crewmembers for the

individual and collective disconnection of the electrical power sources from

the system;

(iii) The system must be designed so that voltage and frequency, if

applicable, at the terminals of all essential load equipment can be

maintained within the limits for which the equipment is designed during any

probable operating conditions;

(iv) If two independent sources of electrical power for particular

equipment or systems are required, their electrical energy supply must be

ensured by means such as duplicate electrical equipment, throwover switching,

or multichannel or loop circuits separately routed; and

(v) For the purpose of complying with paragraph (b)(5) of this section, the

distribution system includes the distribution busses, their associated

feeders, and each control and protective device.

(c) Generating System. There must be at least one generator/alternator if

the electrical system supplies power to load circuits essential for safe

operation. In addition--

(1) Each generator/alternator must be able to deliver its continuous rated

power, or such power as is limited by its regulation system.

(2) Generator/alternator voltage control equipment must be able to

dependably regulate the generator/alternator output within rated limits.

(3) Means must be provided to disconnect each generator/alternator from the

battery and other generators/alternators when enough reverse current exists

that might damage the generator/alternator, or will adversely affect the

airplane electrical system.

(4) There must be a means to give immediate warning to the flight crew of a

failure of any generator/alternator.

(5) Each generator/alternator must have an overvoltage control designed and

installed to prevent damage to the electrical system, or to equipment

supplied by the electrical system that could result if that generator/

alternator were to develop an overvoltage condition.

(d) Instruments. A means must exist to indicate to appropriate flight

crewmembers the electric power system quantities essential for safe

operation.

(1) For normal, utility, and acrobatic category airplanes with direct

current systems, an ammeter that can be switched into each generator feeder

may be used and, if only one generator exists, the ammeter may be in the

battery feeder.

(2) For commuter category airplanes, the essential electric power system

quantities include the voltage and current supplied by each generator.

(e) Fire resistance. Electrical equipment must be so designed and installed

that in the event of a fire in the engine compartment, during which the

surface of the firewall adjacent to the fire is heated to 2,000 deg. F for 5

minutes or to a lesser temperature substantiated by the applicant, the

equipment essential to continued safe operation and located behind the

firewall will function satisfactorily and will not create an additional fire

hazard.

(f) External power. If provisions are made for connecting external power to

the airplane, and that external power can be electrically connected to

equipment other than that used for engine starting, means must be provided to

ensure that no external power supply having a reverse polarity, or a reverse

phase sequence, can supply power to the airplane's electrical system.

(g) It must be shown by analysis, tests, or both, that the airplane can be

operated safely in VFR conditions, for a period of not less than five

minutes, with the normal electrical power (electrical power sources excluding

the battery and any other standby electrical sources) inoperative, with

critical type fuel (from the standpoint of flameout and restart capability),

and with the airplane initially at the maximum certificated altitude. Parts

of the electrical system may remain on if--

(1) A single malfunction, including a wire bundle or junction box fire,

cannot result in loss of the part turned off and the part turned on; and

(2) The parts turned on are electrically and mechanically isolated from the

parts turned off.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13096, Aug. 13, 1969; Amdt. 23-14, 38 FR 31824, Nov. 19, 1973; Amdt. 23-17,

41 FR 55465, Dec. 20, 1976; Amdt. 23-20, 42 FR 36969, July 18, 1977; Amdt.

23-34, 52 FR 1834, Jan. 15, 1987; 52 FR 34745, Sept. 14, 1987; Amdt. 23-43,

58 FR 18976, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1353   Storage battery design and installation.

(a) Each storage battery must be designed and installed as prescribed in

this section.

(b) Safe cell temperatures and pressures must be maintained during any

probable charging and discharging condition. No uncontrolled increase in cell

temperature may result when the battery is recharged (after previous complete

discharge)--

(1) At maximum regulated voltage or power;

(2) During a flight of maximum duration; and

(3) Under the most adverse cooling condition likely to occur in service.

(c) Compliance with paragraph (b) of this section must be shown by tests

unless experience with similar batteries and installations has shown that

maintaining safe cell temperatures and pressures presents no problem.

(d) No explosive or toxic gases emitted by any battery in normal operation,

or as the result of any probable malfunction in the charging system or

battery installation, may accumulate in hazardous quantities within the

airplane.

(e) No corrosive fluids or gases that may escape from the battery may

damage surrounding structures or adjacent essential equipment.

(f) Each nickel cadmium battery installation capable of being used to start

an engine or auxiliary power unit must have provisions to prevent any

hazardous effect on structure or essential systems that may be caused by the

maximum amount of heat the battery can generate during a short circuit of the

battery or of its individual cells.

(g) Nickel cadmium battery installations capable of being used to start an

engine or auxiliary power unit must have--

(1) A system to control the charging rate of the battery automatically so

as to prevent battery overheating;

(2) A battery temperature sensing and over-temperature warning system with

a means for disconnecting the battery from its charging source in the event

of an over-temperature condition; or

(3) A battery failure sensing and warning system with a means for

disconnecting the battery from its charging source in the event of battery

failure.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-20, 42 FR 36969, July 18, 1977; Amdt. 23-21, 43 FR 2319,

Jan. 16, 1978]

Sec. 23.1357   Circuit protective devices.

(a) Protective devices, such as fuses or circuit breakers, must be

installed in all electrical circuits other than--

(1) Main circuits of starter motors used during starting only; and

(2) Circuits in which no hazard is presented by their omission.

(b) A protective device for a circuit essential to flight safety may not be

used to protect any other circuit.

(c) Each resettable circuit protective device ("trip free" device in which

the tripping mechanism cannot be overridden by the operating control) must be

designed so that--

(1) A manual operation is required to restore service after tripping; and

(2) If an overload or circuit fault exists, the device will open the

circuit regardless of the position of the operating control.

(d) If the ability to reset a circuit breaker or replace a fuse is

essential to safety in flight, that circuit breaker or fuse must be so

located and identified that it can be readily reset or replaced in flight.

(e) For fuses identified as replaceable in flight--

(1) There must be one spare of each rating or 50 percent spare fuses of

each rating, whichever is greater; and

(2) The spare fuse(s) must be readily accessible to any required pilot.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-20, 42 FR 36969, July 18, 1977; Amdt. 23-43, 58 FR

18976, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1361   Master switch arrangement.

(a) There must be a master switch arrangement to allow ready disconnection

of each electric power source from power distribution systems, except as

provided in paragraph (b) of this section. The point of disconnection must be

adjacent to the sources controlled by the switch arrangement. If separate

switches are incorporated into the master switch arrangement, a means must be

provided for the switch arrangement to be operated by one hand with a single

movement.

(b) Load circuits may be connected so that they remain energized when the

master switch is open, if the circuits are isolated, or physically shielded,

to prevent their igniting flammable fluids or vapors that might be liberated

by the leakage or rupture of any flammable fluid system; and

(1) The circuits are required for continued operation of the engine; or

(2) The circuits are protected by circuit protective devices with a rating

of five amperes or less adjacent to the electric power source.

(3) In addition, two or more circuits installed in accordance with the

requirements of paragraph (b)(2) of this section must not be used to supply a

load of more than five amperes.

(c) The master switch or its controls must be so installed that the switch

is easily discernible and accessible to a crewmember in flight.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-20, 42 FR 36969, July 18, 1977; Amdt. 23-43, 58 FR 18977,

Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1365   Electric cables and equipment.

(a) Each electric connecting cable must be of adequate capacity.

(b) Each cable and associated equipment that would overheat in the event of

circuit overload or fault must be at least flame resistant and may not emit

dangerous quantities of toxic fumes.

(c) Main power cables (including generator cables) in the fuselage must be

designed to allow a reasonable degree of deformation and stretching without

failure and must--

(1) Be separated from flammable fluid lines; or

(2) Be shrouded by means of electrically insulated flexible conduit, or

equivalent, which is in addition to the normal cable insulation.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-14, 38 FR

31824, Nov. 19, 1973; Amdt. 23-43, 58 FR 18977, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1367   Switches.

Each switch must be--

(a) Able to carry its rated current;

(b) Constructed with enough distance or insulating material between current

carrying parts and the housing so that vibration in flight will not cause

shorting;

(c) Accessible to appropriate flight crewmembers; and

(d) Labeled as to operation and the circuit controlled.

Lights

Sec. 23.1381   Instrument lights.

The instrument lights must--

(a) Make each instrument and control easily readable and discernible;

(b) Be installed so that their direct rays, and rays reflected from the

windshield or other surface, are shielded from the pilot's eyes; and

(c) Have enough distance or insulating material between current carrying

parts and the housing so that vibration in flight will not cause shorting.

A cabin dome light is not an instrument light.

Sec. 23.1383   Landing lights.

(a) Each installed landing light must be acceptable.

(b) Each landing light must be installed so that--

(1) No dangerous glare is visible to the pilot;

(2) The pilot is not seriously affected by halation; and

(3) It provides enough light for night landing.

Sec. 23.1385   Position light system installation.

(a) General. Each part of each position light system must meet the

applicable requirements of this section and each system as a whole must meet

the requirements of Secs. 23.1387 through 23.1397.

(b) Left and right position lights. Left and right position lights must

consist of a red and a green light spaced laterally as far apart as

practicable and installed on the airplane such that, with the airplane in the

normal flying position, the red light is on the left side and the green light

is on the right side.

(c) Rear position light. The rear position light must be a white light

mounted as far aft as practicable on the tail or on each wing tip.

(d) Light covers and color filters.  Each light cover or color filter must

be at least flame resistant and may not change color or shape or lose any

appreciable light transmission during normal use.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-17, 41 FR

55465, Dec. 20, 1976; Amdt. 23-43, 58 FR 18977, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1387   Position light system dihedral angles.

(a) Except as provided in paragraph (e) of this section, each position

light must, as installed, show unbroken light within the dihedral angles

described in this section.

(b) Dihedral angle L (left) is formed by two intersecting vertical planes,

the first parallel to the longitudinal axis of the airplane, and the other at

110 degrees to the left of the first, as viewed when looking forward along

the longitudinal axis.

(c) Dihedral angle R (right) is formed by two intersecting vertical planes,

the first parallel to the longitudinal axis of the airplane, and the other at

110 degrees to the right of the first, as viewed when looking forward along

the longitudinal axis.

(d) Dihedral angle A (aft) is formed by two intersecting vertical planes

making angles of 70 degrees to the right and to the left, respectively, to a

vertical plane passing through the longitudinal axis, as viewed when looking

aft along the longitudinal axis.

(e) If the rear position light, when mounted as far aft as practicable in

accordance with Sec. 23.1385(c), cannot show unbroken light within dihedral

angle A (as defined in paragraph (d) of this section), a solid angle or

angles of obstructed visibility totaling not more than 0.04 steradians is

allowable within that dihedral angle, if such solid angle is within a cone

whose apex is at the rear position light and whose elements make an angle of

30 deg. with a vertical line passing through the rear position light.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-12, 36 FR 21278, Nov. 5, 1971; Amdt. 23-43, 58 FR 18977,

Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1389   Position light distribution and intensities.

(a) General. The intensities prescribed in this section must be provided by

new equipment with each light cover and color filter in place. Intensities

must be determined with the light source operating at a steady value equal to

the average luminous output of the source at the normal operating voltage of

the airplane. The light distribution and intensity of each position light

must meet the requirements of paragraph (b) of this section.

(b) Position lights.  The light distribution and intensities of position

lights must be expressed in terms of minimum intensities in the horizontal

plane, minimum intensities in any vertical plane, and maximum intensities in

overlapping beams, within dihedral angles L, R, and A, and must meet the

following requirements:

(1) Intensities in the horizontal plane.  Each intensity in the horizontal

plane (the plane containing the longitudinal axis of the airplane and

perpendicular to the plane of symmetry of the airplane) must equal or exceed

the values in Sec. 23.1391.

(2) Intensities in any vertical plane.  Each intensity in any vertical

plane (the plane perpendicular to the horizontal plane) must equal or exceed

the appropriate value in Sec. 23.1393, where I is the minimum intensity

prescribed in Sec. 23.1391 for the corresponding angles in the horizontal

plane.

(3) Intensities in overlaps between adjacent signals. No intensity in any

overlap between adjacent signals may exceed the values in Sec. 23.1395,

except that higher intensities in overlaps may be used with main beam

intensities substantially greater than the minima specified in Secs. 23.1391

and 23.1393, if the overlap intensities in relation to the main beam

intensities do not adversely affect signal clarity. When the peak intensity

of the left and right position lights is more than 100 candles, the maximum

overlap intensities between them may exceed the values in Sec. 23.1395 if the

overlap intensity in Area A is not more than 10 percent of peak position

light intensity and the overlap intensity in Area B is not more than 2.5

percent of peak position light intensity.

(c) Rear position light installation. A single rear position light may be

installed in a position displaced laterally from the plane of symmetry of an

airplane if--

(1) The axis of the maximum cone of illumination is parallel to the flight

path in level flight; and

(2) There is no obstruction aft of the light and between planes 70 degrees

to the right and left of the axis of maximum illumination.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18977, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1391   Minimum intensities in the horizontal plane of position

lights.

Each position light intensity must equal or exceed the applicable values in

the following table:

Angle from right or left

Dihedral angle (light     of longitudinal axis,

included)          measured from dead ahead    Intensity (candles)

L and R (red and green)   0 deg. to 10 deg.         40

10 deg. to 20 deg.        30

20 deg. to 110 deg.       5

A (rear white)            110 deg. to 180 deg.      20

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18977, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1393   Minimum intensities in any vertical plane of position lights.

Each position light intensity must equal or exceed the applicable values in

the following table:

Angle above or

below the       Intensity,

horizontal plane       l

0 deg.                    1.00

0 deg. to 5 deg.          0.90

5 deg. to 10 deg.         0.80

10 deg. to 15 deg.        0.70

15 deg. to 20 deg.        0.50

20 deg. to 30 deg.        0.30

30 deg. to 40 deg.        0.10

40 deg. to 90 deg.        0.05

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18977, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1395   Maximum intensities in overlapping beams of position lights.

No position light intensity may exceed the applicable values in the

following equal or exceed the applicable values in Sec. 23.1389(b)(3):

Maximum intensity

Area A     Area B

Overlaps             (candles)  (candles)

Green in dihedral angle L              10          1

Red in dihedral angle R                10          1

Green in dihedral angle A               5          1

Red in dihedral angle A                 5          1

Rear white in dihedral angle L          5          1

Rear white in dihedral angle R          5          1

Where--

(a) Area A includes all directions in the adjacent dihedral angle that pass

through the light source and intersect the common boundary plane at more than

10 degrees but less than 20 degrees; and

(b) Area B includes all directions in the adjacent dihedral angle that pass

through the light source and intersect the common boundary plane at more than

20 degrees.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-43, 58 FR

18977, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1397   Color specifications.

Each position light color must have the applicable International Commission

on Illumination chromaticity coordinates as follows:

(a) Aviation red--

"y" is not greater than 0.335; and

"z" is not greater than 0.002.

(b) Aviation green--

"x" is not greater than 0.440-0.320 y;

"x" is not greater than y -0.170; and

"y" is not less than 0.390-0.170 x.

(c) Aviation white--

"x" is not less than 0.300 and not greater than 0.540;

"y" is not less than "x -0.040" or "y0 -0.010," whichever is the smaller;

and

"y" is not greater than "x+0.020" nor "0.636-0.400 x ";

Where "y0" is the "y" coordinate of the Planckian radiator for the value of

"x"  considered.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, amended by Amdt. 23-11, 36 FR

12971, July 10, 1971]

Sec. 23.1399   Riding light.

(a) Each riding (anchor) light required for a seaplane or amphibian, must

be installed so that it can--

(1) Show a white light for at least two miles at night under clear

atmospheric conditions; and

(2) Show the maximum unbroken light practicable when the airplane is moored

or drifting on the water.

(b) Externally hung lights may be used.

Sec. 23.1401   Anticollision light system.

(a) General. If certification for night operation is requested, the

airplane must have an anticollision light system that--

(1) Consists of one or more approved anticollision lights located so that

their light will not impair the flight crewmembers' vision or detract from

the conspicuity of the position lights; and

(2) Meets the requirements of paragraphs (b) through (f) of this section.

(b) Field of coverage. The system must consist of enough lights to

illuminate the vital areas around the airplane, considering the physical

configuration and flight characteristics of the airplane. The field of

coverage must extend in each direction within at least 75 degrees above and

75 degrees below the horizontal plane of the airplane, except that there may

be solid angles of obstructed visibility totaling not more than 0.5

steradians.

(c) Flashing characteristics. The arrangement of the system, that is, the

number of light sources, beam width, speed of rotation, and other

characteristics, must give an effective flash frequency of not less than 40,

nor more than 100, cycles per minute. The effective flash frequency is the

frequency at which the airplane's complete anticollision light system is

observed from a distance, and applies to each sector of light including any

overlaps that exist when the system consists of more than one light source.

In overlaps, flash frequencies may exceed 100, but not 180, cycles per

minute.

(d) Color. Each anticollision light must be either aviation red or aviation

white and must meet the applicable requirements of Sec. 23.1397.

(e) Light intensity. The minimum light intensities in any vertical plane,

measured with the red filter (if used) and expressed in terms of "effective"

intensities, must meet the requirements of paragraph (f) of this section. The

following relation must be assumed:

t2

INTEGRAL       I(t)dt

t1

Ie =

--------------

0.2+(t2-t1)

where:

Ie =effective intensity (candles).

I(t) =instantaneous intensity as a function of time.

t2-t1 =flash time interval (seconds).

Normally, the maximum value of effective intensity is obtained when t2 and t1

are chosen so that the effective intensity is equal to the instantaneous

intensity at t2 and t1.

(f) Minimum effective intensities for anticollision lights. Each

anticollision light effective intensity must equal or exceed the applicable

values in the following table.

Angle above or    Effective

below the       intensity

horizontal plane   (candles)

0 deg. to 5 deg.          400

5 deg. to 10 deg.         240

10 deg. to 20 deg.         80

20 deg. to 30 deg.         40

30 deg. to 75 deg.         20

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-11, 36 FR

12972, July 10, 1971; Amdt. 23-20, 42 FR 36969, July 18, 1977]

Safety Equipment

Sec. 23.1411   General.

(a) Required safety equipment to be used by the flight crew in an

emergency, such as automatic liferaft releases, must be readily accessible.

(b) Stowage provisions for required safety equipment must be furnished and

must--

(1) Be arranged so that the equipment is directly accessible and its

location is obvious; and

(2) Protect the safety equipment from damage caused by being subjected to

the inertia loads resulting from the ultimate static load factors specified

in Sec. 23.561(b)(3) of this part.

[Amdt. 23-17, 41 FR 55465, Dec. 20, 1976, as amended by Amdt. 23-36, 53 FR

30815, Aug. 15, 1988]

Sec. 23.1413   Safety belts and harnesses.

Each safety belt and shoulder harness must be equipped with a metal-to-

metal latching device.

[Amdt. 23-36, 53 FR 30815, Aug. 15, 1988]

Sec. 23.1415   Ditching equipment.

(a) Emergency flotation and signaling equipment required by any operating

rule in this chapter must be installed so that it is readily available to the

crew and passengers.

(b) Each raft and each life preserver must be approved.

(c) Each raft released automatically or by the pilot must be attached to

the airplane by a line to keep it alongside the airplane. This line must be

weak enough to break before submerging the empty raft to which it is

attached.

(d) Each signaling device required by any operating rule in this chapter,

must be accessible, function satisfactorily, and must be free of any hazard

in its operation.

Sec. 23.1416   Pneumatic de-icer boot system.

If certification with ice protection provisions is desired and a pneumatic

de-icer boot system is installed--

(a) The system must meet the requirements specified in Sec. 23.1419.

(b) The system and its components must be designed to perform their

intended function under any normal system operating temperature or pressure,

and

(c) Means to indicate to the flight crew that the pneumatic de-icer boot

system is receiving adequate pressure and is functioning normally must be

provided.

[Amdt. 23-23, 43 FR 50593, Oct. 30, 1978]

Sec. 23.1419   Ice protection.

If certification with ice protection provisions is desired, compliance with

the requirements of this section and other applicable sections of this part

must be shown:

(a) An analysis must be performed to establish, on the basis of the

airplane's operational needs, the adequacy of the ice protection system for

the various components of the airplane. In addition, tests of the ice

protection system must be conducted to demonstrate that the airplane is

capable of operating safely in continuous maximum and intermittent maximum

icing conditions, as described in appendix C of part 25 of this chapter. As

used in this section, "Capable of operating safely," means that airplane

performance, controllability, maneuverability, and stability must not be less

than that required in part 23, subpart B.

(b) Except as provided by paragraph (c) of this section, in addition to the

analysis and physical evaluation prescribed in paragraph (a) of this section,

the effectiveness of the ice protection system and its components must be

shown by flight tests of the airplane or its components in measured natural

atmospheric icing conditions and by one or more of the following tests, as

found necessary to determine the adequacy of the ice protection system--

(1) Laboratory dry air or simulated icing tests, or a combination of both,

of the components or models of the components.

(2) Flight dry air tests of the ice protection system as a whole, or its

individual components.

(3) Flight test of the airplane or its components in measured simulated

icing conditions.

(c) If certification with ice protection has been accomplished on prior

type certificated airplanes whose designs include components that are

thermodynamically and aerodynamically equivalent to those used on a new

airplane design, certification of these equivalent components may be

accomplished by reference to previously accomplished tests, required in Sec.

23.1419 (a) and (b), provided that the applicant accounts for any differences

in installation of these components.

(d) A means must be identified or provided for determining the formation of

ice on the critical parts of the airplane. Adequate lighting must be provided

for the use of this means during night operation. Also, when monitoring of

the external surfaces of the airplane by the flight crew is required for

operation of the ice protection equipment, external lighting must be provided

that is adequate to enable the monitoring to be done at night. Any

illumination that is used must be of a type that will not cause glare or

reflection that would handicap crewmembers in the performance of their

duties. The Airplane Flight Manual or other approved manual material must

describe the means of determining ice formation and must contain information

for the safe operation of the airplane in icing conditions.

[Amdt. 23-43, 58 FR 18977, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Miscellaneous Equipment

Sec. 23.1431   Electronic equipment.

(a) In showing compliance with Sec. 23.1309(b) (1) and (2) with respect to

radio and electronic equipment and their installations, critical

environmental conditions must be considered.

(b) Radio and electronic equipment, controls, and wiring must be installed

so that operation of any unit or system of units will not adversely affect

the simultaneous operation of any other radio or electronic unit, or system

of units, required by this chapter.

[Amdt. 23-43, 58 FR 18977, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1435   Hydraulic systems.

(a) Design. Each hydraulic system must be designed as follows:

(1) Each hydraulic system and its elements must withstand, without

yielding, the structural loads expected in addition to hydraulic loads.

(2) A means to indicate the pressure in each hydraulic system which

supplies two or more primary functions must be provided to the flight crew.

(3) There must be means to ensure that the pressure, including transient

(surge) pressure, in any part of the system will not exceed the safe limit

above design operating pressure and to prevent excessive pressure resulting

from fluid volumetric changes in all lines which are likely to remain closed

long enough for such changes to occur.

(4) The minimum design burst pressure must be 2.5 times the operating

pressure.

(b) Tests. Each system must be substantiated by proof pressure tests. When

proof tested, no part of any system may fail, malfunction, or experience a

permanent set. The proof load of each system must be at least 1.5 times the

maximum operating pressure of that system.

(c) Accumulators. A hydraulic accumulator or pressurized reservoir must not

be installed on the engine side of any firewall unless--

(1) It is an integral part of an engine or propeller, or

(2) It is a nonpressurized reservoir and the total capacity of all such

nonpressurized reservoirs is one quart or less.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13096, Aug. 13, 1969; Amdt. 23-14, 38 FR 31824, Nov. 19, 1973; Amdt. 23-43,

58 FR 18978, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1437   Accessories for multiengine airplanes.

For multiengine airplanes, engine-driven accessories essential to safe

operation must be distributed among two or more engines so that the failure

of any one engine will not impair safe operation through the malfunctioning

of these accessories.

Sec. 23.1438   Pressurization and pneumatic systems.

(a) Pressurization system elements must be burst pressure tested to 2.0

times, and proof pressure tested to 1.5 times, the maximum normal operating

pressure.

(b) Pneumatic system elements must be burst pressure tested to 3.0 times,

and proof pressure tested to 1.5 times, the maximum normal operating

pressure.

(c) An analysis, or a combination of analysis and test, may be substituted

for any test required by paragraph (a) or (b) of this section if the

Administrator finds it equivalent to the required test.

[Amdt. 23-20, 42 FR 36969, July 18, 1977]

Sec. 23.1441   Oxygen equipment and supply.

(a) If certification with supplemental oxygen equipment is requested, or

the airplane is approved for operations at or above altitudes where oxygen is

required to be used by the operating rules, oxygen equipment must be provided

that meets the requirements of this section and Secs. 23.1443 through

23.1449. Portable oxygen equipment may be used to meet the requirements of

this part if the portable equipment is shown to comply with the applicable

requirements, is identified in the airplane type design, and its stowage

provisions are found to be in compliance with the requirements of Sec.

23.561.

(b) The oxygen system must be free from hazards in itself, in its method of

operation, and its effect upon other components.

(c) There must be a means to allow the crew to readily determine, during

the flight, the quantity of oxygen available in each source of supply.

(d) Each required flight crewmember must be provided with--

(1) Demand oxygen equipment if the airplane is to be certificated for

operation above 25,000 feet.

(2) Pressure demand oxygen equipment if the airplane is to be certificated

for operation above 40,000 feet.

(e) There must be a means, readily available to the crew in flight, to turn

on and to shut off the oxygen supply at the high pressure source. This

shutoff requirement does not apply to chemical oxygen generators.

[Amdt. 23-9, 35 FR 6386, Apr. 21, 1970, as amended by Amdt. 23-43, 58 FR

18978, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1443   Minimum mass flow of supplemental oxygen.

(a) If continuous flow oxygen equipment is installed, an applicant must

show compliance with the requirements of either paragraphs (a)(1) and (a)(2)

or paragraph (a)(3) of this section:

(1) For each passenger, the minimum mass flow of supplemental oxygen

required at various cabin pressure altitudes may not be less than the flow

required to maintain, during inspiration and while using the oxygen equipment

(including masks) provided, the following mean tracheal oxygen partial

pressures;

(i) At cabin pressure altitudes above 10,000 feet up to and including

18,500 feet, a mean tracheal oxygen partial pressure of 100 mm. Hg when

breathing 15 liters per minute, Body Temperature, Pressure, Saturated (BTPS)

and with a tidal volume of 700 cc. with a constant time interval between

respirations.

(ii) At cabin pressure altitudes above 18,500 feet up to and including

40,000 feet, a mean tracheal oxygen partial pressure of 83.8 mm. Hg when

breathing 30 liters per minute, BTPS, and with a tidal volume of 1,100 cc.

with a constant time interval between respirations.

(2) For each flight crewmember, the minimum mass flow may not be less than

the flow required to maintain, during inspiration, a mean tracheal oxygen

partial pressure of 149 mm. Hg when breathing 15 liters per minute, BTPS, and

with a maximum tidal volume of 700 cc. with a constant time interval between

respirations.

(3) The minimum mass flow of supplemental oxygen supplied for each user

must be at a rate not less than that shown in the following figure for each

altitude up to and including the maximum operating altitude of the airplane.

[INSERT: Line graph plotting oxygen mass flow in liters per minute

against cabin pressure altitude in thousands of feet]

(b) If demand equipment is installed for use by flight crewmembers, the

minimum mass flow of supplemental oxygen required for each flight crewmember

may not be less than the flow required to maintain, during inspiration, a

mean tracheal oxygen partial pressure of 122 mm. Hg up to and including a

cabin pressure altitude of 35,000 feet, and 95 percent oxygen between cabin

pressure altitudes of 35,000 and 40,000 feet, when breathing 20 liters per

minute BTPS. In addition, there must be means to allow the crew to use

undiluted oxygen at their discretion.

(c) If first-aid oxygen equipment is installed, the minimum mass flow of

oxygen to each user may not be less than 4 liters per minute, STPD. However,

there may be a means to decrease this flow to not less than 2 liters per

minute, STPD, at any cabin altitude. The quantity of oxygen required is based

upon an average flow rate of 3 liters per minute per person for whom first-

aid oxygen is required.

(d) As used in this section:

(1) BTPS means Body Temperature, and Pressure, Saturated (which is, 37

deg.C, and the ambient pressure to which the body is exposed, minus 47 mm.

Hg, which is the tracheal pressure displaced by water vapor pressure when the

breathed air becomes saturated with water vapor at 37 deg.C).

(2) STPD means Standard, Temperature, and Pressure, Dry (which is, 0 deg.C

at 760 mm. Hg with no water vapor).

[Amdt. 23-43, 58 FR 18978, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1445   Oxygen distribution system.

(a) Except for flexible lines from oxygen outlets to the dispensing units,

or where shown to be otherwise suitable to the installation, nonmetallic

tubing must not be used for any oxygen line that is normally pressurized

during flight.

(b) Nonmetallic oxygen distribution lines must not be routed where they may

be subjected to elevated temperatures, electrical arcing, and released

flammable fluids that might result from any probable failure.

[Amdt. 23-43, 58 FR 18978, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1447   Equipment standards for oxygen dispensing units.

If oxygen dispensing units are installed, the following apply:

(a) There must be an individual dispensing unit for each occupant for whom

supplemental oxygen is to be supplied. Each dispensing unit must:

(1) Provide for effective utilization of the oxygen being delivered to the

unit.

(2) Be capable of being readily placed into position on the face of the

user.

(3) Be equipped with a suitable means to retain the unit in position on the

face.

(b) If certification for operation up to and including 18,000 feet (MSL) is

requested, each oxygen dispensing unit must:

(1) Cover the nose and mouth of the user; or

(2) Be a nasal cannula, in which case one oxygen dispensing unit covering

both the nose and mouth of the user must be available. In addition, each

nasal cannula or its connecting tubing must have permanently affixed--

(i) A visible warning against smoking while in use;

(ii) An illustration of the correct method of donning; and

(iii) A visible warning against use with nasal obstructions or head colds

with resultant nasal congestion.

(c) If certification for operation above 18,000 feet (MSL) is requested,

each oxygen dispensing unit must cover the nose and mouth of the user.

(d) For a pressurized airplane designed to operate at flight altitudes

above 25,000 feet (MSL), an oxygen dispensing unit connected to an oxygen

supply terminal must be immediately available to each occupant, wherever

seated.

(e) If certification for operation above 30,000 feet is requested, the

dispensing units must meet the following requirements:

(1) The dispensing units for passengers must be automatically presented to

each occupant before the cabin pressure altitude exceeds 15,000 feet.

(2) The dispensing units for flight crewmembers must be automatically

presented to each flight crewmember before the cabin pressure altitude

exceeds 15,000 feet, or the units must be of the quick-donning type,

connected to an oxygen supply terminal that is immediately available to

flight crewmembers at their station.

(f) If an automatic dispensing unit (hose and mask, or other unit) system

is installed, the crew must be provided with a manual means to make the

dispensing units immediately available in the event of failure of the

automatic system.

[Amdt. 23-9, 35 FR 6387, Apr. 21, 1970, as amended by Amdt. 23-20, 42 FR

36969, July 18, 1977; Amdt. 23-30, 49 FR 7340, Feb. 28, 1984; Amdt. 23-43,

58 FR 18978, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Sec. 23.1449   Means for determining use of oxygen.

There must be a means to allow the crew to determine whether oxygen is

being delivered to the dispensing equipment.

[Amdt. 23-9, 35 FR 6387, Apr. 21, 1970]

Sec. 23.1450   Chemical oxygen generators.

(a) For the purpose of this section, a chemical oxygen generator is defined

as a device which produces oxygen by chemical reaction.

(b) Each chemical oxygen generator must be designed and installed in

accordance with the following requirements:

(1) Surface temperature developed by the generator during operation may not

create a hazard to the airplane or to its occupants.

(2) Means must be provided to relieve any internal pressure that may be

hazardous.

(c) In addition to meeting the requirements in paragraph (b) of this

section, each portable chemical oxygen generator that is capable of sustained

operation by successive replacement of a generator element must be placarded

to show--

(1) The rate of oxygen flow, in liters per minute;

(2) The duration of oxygen flow, in minutes, for the replaceable generator

element; and

(3) A warning that the replaceable generator element may be hot, unless the

element construction is such that the surface temperature cannot exceed 100

deg. F.

[Amdt. 23-20, 42 FR 36969, July 18, 1977]

Sec. 23.1457  Cockpit voice recorders.

(a) Each cockpit voice recorder required by the operating rules of this

chapter must be approved and must be installed so that it will record the

following:

(1) Voice communications transmitted from or received in the airplane by

radio.

(2) Voice communications of flight crewmembers on the flight deck.

(3) Voice communications of flight crewmembers on the flight deck, using

the airplane's interphone system.

(4) Voice or audio signals identifying navigation or approach aids

introduced into a headset or speaker.

(5) Voice communications of flight crewmembers using the passenger

loudspeaker system, if there is such a system and if the fourth channel is

available in accordance with the requirements of paragraph (c)(4)(ii) of this

section.

(b) The recording requirements of paragraph (a)(2) of this section must be

met by installing a cockpit-mounted area microphone, located in the best

position for recording voice communications originating at the first and

second pilot stations and voice communications of other crewmembers on the

flight deck when directed to those stations. The microphone must be so

located and, if necessary, the preamplifiers and filters of the recorder must

be so adjusted or supplemented, so that the intelligibility of the recorded

communications is as high as practicable when recorded under flight cockpit

noise conditions and played back. Repeated aural or visual playback of the

record may be used in evaluating intelligibility.

(c) Each cockpit voice recorder must be installed so that the part of the

communication or audio signals specified in paragraph (a) of this section

obtained from each of the following sources is recorded on a separate

channel:

(1) For the first channel, from each boom, mask, or handheld microphone,

headset, or speaker used at the first pilot station.

(2) For the second channel from each boom, mask, or handheld microphone,

headset, or speaker used at the second pilot station.

(3) For the third channel--from the cockpit-mounted area microphone.

(4) For the fourth channel from:

(i) Each boom, mask, or handheld microphone, headset, or speaker used at

the station for the third and fourth crewmembers.

(ii) If the stations specified in paragraph (c)(4)(i) of this section are

not required or if the signal at such a station is picked up by another

channel, each microphone on the flight deck that is used with the passenger

loudspeaker system, if its signals are not picked up by another channel.

(5) And that as far as is practicable all sounds received by the microphone

listed in paragraphs (c) (1), (2), and (4) of this section must be recorded

without interruption irrespective of the position of the interphone-

transmitter key switch. The design shall ensure that sidetone for the flight

crew is produced only when the interphone, public address system, or radio

transmitters are in use.

(d) Each cockpit voice recorder must be installed so that:

(1) It receives its electric power from the bus that provides the maximum

reliability for operation of the cockpit voice recorder without jeopardizing

service to essential or emergency loads.

(2) There is an automatic means to simultaneously stop the recorder and

prevent each erasure feature from functioning, within 10 minutes after crash

impact; and

(3) There is an aural or visual means for preflight checking of the

recorder for proper operation.

(e) The record container must be located and mounted to minimize the

probability of rupture of the container as a result of crash impact and

consequent heat damage to the record from fire. In meeting this requirement,

the record container must be as far aft as practicable, but may not be where

aft mounted engines may crush the container during impact. However, it need

not be outside of the pressurized compartment.

(f) If the cockpit voice recorder has a bulk erasure device, the

installation must be designed to minimize the probability of inadvertent

operation and actuation of the device during crash impact.

(g) Each recorder container must:

(1) Be either bright orange or bright yellow;

(2) Have reflective tape affixed to its external surface to facilitate its

location under water; and

(3) Have an underwater locating device, when required by the operating

rules of this chapter, on or adjacent to the container which is secured in

such manner that they are not likely to be separated during crash impact.

[Amdt. 23-35, 53 FR 26142, July 11, 1988]

Sec. 23.1459   Flight recorders.

(a) Each flight recorder required by the operating rules of this chapter

must be installed so that:

(1) It is supplied with airspeed, altitude, and directional data obtained

from sources that meet the accuracy requirements of Secs. 23.1323, 23.1325,

and 23.1327, as appropriate;

(2) The vertical acceleration sensor is rigidly attached, and located

longitudinally either within the approved center of gravity limits of the

airplane, or at a distance forward or aft of these limits that does not

exceed 25 percent of the airplane's mean aerodynamic chord;

(3) It receives its electrical power power from the bus that provides the

maximum reliability for operation of the flight recorder without jeopardizing

service to essential or emergency loads;

(4) There is an aural or visual means for preflight checking of the

recorder for proper recording of data in the storage medium.

(5) Except for recorders powered solely by the engine-driven electrical

generator system, there is an automatic means to simultaneously stop a

recorder that has a data erasure feature and prevent each erasure feature

from functioning, within 10 minutes after crash impact; and

(b) Each nonejectable record container must be located and mounted so as to

minimize the probability of container rupture resulting from crash impact and

subsequent damage to the record from fire. In meeting this requirement the

record container must be located as far aft as practicable, but need not be

aft of the pressurized compartment, and may not be where aft-mounted engines

may crush the container upon impact.

(c) A correlation must be established between the flight recorder readings

of airspeed, altitude, and heading and the corresponding readings (taking

into account correction factors) of the first pilot's instruments. The

correlation must cover the airspeed range over which the airplane is to be

operated, the range of altitude to which the airplane is limited, and 360

degrees of heading. Correlation may be established on the ground as

appropriate.

(d) Each recorder container must:

(1) Be either bright orange or bright yellow;

(2) Have reflective tape affixed to its external surface to facilitate its

location under water; and

(3) Have an underwater locating device, when required by the operating

rules of this chapter, on or adjacent to the container which is secured in

such a manner that they are not likely to be separated during crash impact.

(e) Any novel or unique design or operational characteristics of the

aircraft shall be evaluated to determine if any dedicated parameters must be

recorded on flight recorders in addition to or in place of existing

requirements.

[Amdt. 23-35, 53 FR 26143, July 11, 1988]

Sec. 23.1461   Equipment containing high energy rotors.

(a) Equipment containing high energy rotors must meet paragraph (b), (c),

or (d) of this section.

(b) High energy rotors contained in equipment must be able to withstand

damage caused by malfunctions, vibration, abnormal speeds, and abnormal

temperatures. In addition--

(1) Auxiliary rotor cases must be able to contain damage caused by the

failure of high energy rotor blades; and

(2) Equipment control devices, systems, and instrumentation must reasonably

ensure that no operating limitations affecting the integrity of high energy

rotors will be exceeded in service.

(c) It must be shown by test that equipment containing high energy rotors

can contain any failure of a high energy rotor that occurs at the highest

speed obtainable with the normal speed control devices inoperative.

(d) Equipment containing high energy rotors must be located where rotor

failure will neither endanger the occupants nor adversely affect continued

safe flight.

[Amdt. 23-20, 42 FR 36969, July 18, 1977]

Subpart G--Operating Limitations and Information

Sec. 23.1501  General.

(a) Each operating limitation specified in Secs. 23.1505 through 23.1527

and other limitations and information necessary for safe operation must be

established.

(b) The operating limitations and other information necessary for safe

operation must be made available to the crewmembers as prescribed in Secs.

23.1541 through 23.1589.

[Amdt. 23-21, 43 FR 2319, Jan. 16, 1978]

Sec. 23.1505  Airspeed limitations.

(a) The never-exceed speed VNE must be established so that it is--

(1) Not less than 0.9 times the minimum value of VD allowed under Sec.

23.335; and

(2) Not more than the lesser of--

(i) 0.9 VD established under Sec. 23.335; or

(ii) 0.9 times the maximum speed shown under Sec. 23.251.

(b) The maximum structural cruising speed VNO must be established so that

it is--

(1) Not less than the minimum value of VC allowed under Sec. 23.335; and

(2) Not more than the lesser of--

(i) VC established under Sec. 23.335; or

(ii) 0.89 VNE established under paragraph (a) of this section.

(c) Paragraphs (a) and (b) of this section do not apply to turbine

airplanes or to airplanes for which a design diving speed VD/MD is

established under Sec. 23.335(b)(4). For those airplanes, a maximum operating

limit speed (VMO/MMO-airspeed or Mach number, whichever is critical at a

particular altitude) must be established as a speed that may not be

deliberately exceeded in any regime of flight (climb, cruise, or descent)

unless a higher speed is authorized for flight test or pilot training

operations. VMO/MMO must be established so that it is not greater than the

design cruising speed VC/MC and so that it is sufficiently below VD/MD and

the maximum speed shown under Sec. 23.251 to make it highly improbable that

the latter speeds will be inadvertently exceeded in operations. The speed

margin between VMO/MMO and VD/MD or the maximum speed shown under Sec. 23.251

may not be less than the speed margin established between VC/MC  and VD/MD

under Sec. 23.335(b), or the speed margin found necessary in the flight test

conducted under Sec. 23.253.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13096, Aug. 13, 1969]

Sec. 23.1507   Operating maneuvering speed.

The maximum maneuvering speed Vo, must be established as an operating

limitation. Vo is a selected speed that is not greater than Vs[square root]n

established in Sec. 23.335(c).

[Amdt. No. 23-45, 58 FR 42165, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1511   Flap extended speed.

(a) The flap extended speed VFE must be established so that it is--

(1) Not less than the minimum value of VF allowed in Secs. 23.345 and

23.457; and

(2) Not more than the lesser of--

(i) VF established under Sec. 23.345; or

(ii) VF established under Sec. 23.457.

(b) Additional combinations of flap setting, airspeed, and engine power may

be established if the structure has been proven for the corresponding design

conditions.

Sec. 23.1513   Minimum control speed.

The minimum control speed VMC, determined under Sec. 23.149, must be

established as an operating limitation.

Sec. 23.1519   Weight and center of gravity.

The weight and center of gravity limitations determined under Sec. 23.23

must be established as operating limitations.

Sec. 23.1521   Powerplant limitations.

(a) General. The powerplant limitations prescribed in this section must be

established so that they do not exceed the corresponding limits for which the

engines or propellers are type certificated. In addition, other powerplant

limitations used in determining compliance with this part must be

established.

(b) Takeoff operation. The powerplant takeoff operation must be limited

by--

(1) The maximum rotational speed (rpm);

(2) The maximum allowable manifold pressure (for reciprocating engines);

(3) The maximum allowable gas temperature (for turbine engines);

(4) The time limit for the use of the power or thrust corresponding to the

limitations established in paragraphs (b) (1) through (3) of this section;

and

(5) If the time limit in paragraph (b) (4) of this section exceeds two

minutes, the maximum allowable cylinder head (as applicable), liquid coolant,

and oil temperatures.

(c) Continuous operation. The continuous operation must be limited by--

(1) The maximum rotational speed;

(2) The maximum allowable manifold pressure (for reciprocating engines);

(3) The maximum allowable gas temperature (for turbine engines); and

(4) The maximum allowable cylinder head, oil, and liquid coolant

temperatures.

(d) Fuel grade or designation. The minimum fuel grade (for reciprocating

engines), or fuel designation (for turbine engines), must be established so

that it is not less than that required for the operation of the engines

within the limitations in paragraphs (b) and (c) of this section.

(e) Ambient temperature. For turbine engines, ambient temperature

limitations (including limitations for winterization installations if

applicable) must be established as the maximum ambient atmospheric

temperature at which compliance with the cooling provisions of Secs. 23.1041

through 23.1047 is shown.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-21, 43 FR 2319, Jan. 16, 1978; Amdt. 23-45, 58 FR 42165,

Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1522   Auxiliary power unit limitations.

If an auxiliary power unit is installed, the limitations established for

the auxiliary power must be specified in the operating limitations for the

airplane.

[Amdt. No. 23-45, 58 FR 42166, Aug. 6, 1993; 58 FR 51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1523  Minimum flight crew.

The minimum flight crew must be established so that it is sufficient for

safe operation considering--

(a) The workload on individual crewmembers and, in addition for commuter

category airplanes, each crewmember workload determination must consider the

following:

(1) Flight path control,

(2) Collision avoidance,

(3) Navigation,

(4) Communications,

(5) Operation and monitoring of all essential airplane systems,

(6) Command decisions, and

(7) The accessibility and ease of operation of necessary controls by the

appropriate crewmember during all normal and emergency operations when at the

crewmember flight station;

(b) The accessibility and ease of operation of necessary controls by the

appropriate crewmember; and

(c) The kinds of operation authorized under Sec. 23.1525.

[Amdt. 23-21, 43 FR 2319, Jan. 16, 1978, as amended by Amdt. 23-34, 52 FR

1834, Jan. 15, 1987]

Sec. 23.1524   Maximum passenger seating configuration.

The maximum passenger seating configuration must be established.

[Amdt. 23-10, 36 FR 2864, Feb. 11, 1971]

Sec. 23.1525   Kinds of operation.

The kinds of operation authorized (e.g. VFR, IFR, day or night) and the

meteorological conditions (e.g. icing) to which the operation of the airplane

is limited of from which it is prohibited, mut be established appropriate to

the installed equipment.

[Amdt. No. 23-45, 58 FR 42166, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1527   Maximum operating altitude.

(a) The maximum altitude up to which operation is allowed, as limited by

flight, structural, powerplant, functional or equipment characteristics, must

be established.

(b) A maximum operating altitude limitation of not more than 25,000 feet

must be established for pressurized airplanes unless compliance with Sec.

23.775(e) is shown.

[Amdt. 23-45, 45 FR 42166, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1529   Instructions for Continued Airworthiness.

The applicant must prepare Instructions for Continued Airworthiness in

accordance with Appendix G to this part that are acceptable to the

Administrator. The instructions may be incomplete at type certification if a

program exists to ensure their completion prior to delivery of the first

airplane or issuance of a standard certificate of airworthiness, whichever

occurs later.

[Amdt. 23-26, 45 FR 60171, Sept. 11, 1980]

Markings And Placards

Sec. 23.1541  General.

(a) The airplane must contain--

(1) The markings and placards specified in Secs. 23.1545 through 23.1567;

and

(2) Any additional information, instrument markings, and placards required

for the safe operation if it has unusual design, operating, or handling

characteristics.

(b) Each marking and placard prescribed in paragraph (a) of this section--

(1) Must be displayed in a conspicuous place; and

(2) May not be easily erased, disfigured, or obscured.

(c) For airplanes which are to be certificated in more than one category--

(1) The applicant must select one category upon which the placards and

markings are to be based; and

(2) The placards and marking information for all categories in which the

airplane is to be certificated must be furnished in the Airplane Flight

Manual.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-21, 43 FR 2319, Jan. 16, 1978]

Sec. 23.1543   Instrument markings: general.

For each instrument--

(a) When markings are on the cover glass of the instrument, there must be

means to maintain the correct alignment of the glass cover with the face of

the dial; and

(b) Each arc and line must be wide enough and located to be clearly visible

to the pilot.

Sec. 23.1545   Airspeed indicator.

(a) Each airspeed indicator must be marked as specified in paragraph (b) of

this section, with the marks located at the corresponding indicated

airspeeds.

(b) The following markings must be made:

(1) For the never-exceed speed VNE, a radial red line.

(2) For the caution range, a yellow arc extending from the red line

specified in paragraph (b)(1) of this section to the upper limit of the green

arc specified in paragraph (b)(3) of this section.

(3) For the normal operating range, a green arc with the lower limit at VS1

with maximum weight and with landing gear and wing flaps retracted, and the

upper limit at the maximum structural cruising speed VNO established under

Sec. 23.1505(b).

(4) For the flap operating range, a white arc with the lower limit at VS0

at the maximum weight, and the upper limit at the flaps-extended speed VFE

established under Sec. 23.1511.

(5) For the one-engine-inoperative best rate of climb speed, Vy, a blue

sector extending from the Vy speed at sea level to the Vy speed at--

(i) An altitude of 5,000 feet, if the one-engine-inoperative best rate of

climb at that altitude is less than 100 feet per minute, or

(ii) The highest 1,000-foot altitude (at or above 5,000 feet) at which the

one-engine-inoperative best rate of climb is 100 feet per minute or more.

Each side of the sector must be labeled to show the altitude for the

corresponding Vy.

(6) For the minimum control speed (one-engine-inoperative), Vmc', a red

radial line.

(c) If VNE or VNO vary with altitude, there must be means to indicate to

the pilot the appropriate limitations throughout the operating altitude

range.

(d) Paragraphs (b)(1) through (b)(3) and paragraph (c) of this section do

not apply to aircraft for which a maximum operating speed VMO/MMO is

established under Sec. 23.1505(c). For those aircraft there must either be a

maximum allowable airspeed indication showing the variation of VMO/MMO with

altitude or compressibility limitations (as appropriate), or a radial red

line marking for VMO/MMO must be made at lowest value of VMO/MMO established

for any altitude up to the maximum operating altitude for the airplane.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-3, 30 FR

14240, Nov. 13, 1965; Amdt. 23-7, 34 FR 13097, Aug. 13, 1969; Amdt. 23-23, 43

FR 50593, Oct. 30, 1978]

Sec. 23.1547   Magnetic direction indicator.

(a) A placard meeting the requirements of this section must be installed on

or near the magnetic direction indicator.

(b) The placard must show the calibration of the instrument in level flight

with the engines operating.

(c) The placard must state whether the calibration was made with radio

receivers on or off.

(d) Each calibration reading must be in terms of magnetic headings in not

more than 30 degree increments.

(e) If a magnetic nonstabilized direction indicator can have a deviation of

more than 10 degrees caused by the operation of electrical equipment, the

placard must state which electrical loads, or combination of loads, would

cause a deviation of more than 10 degrees when turned on.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-20, 42 FR 36969, July 18, 1977]

Sec. 23.1549   Powerplant and auxiliary power unit instruments.

For each required powerplant and auxiliary power unit instrument, as

appropriate to the type of instruments--

(a) Each maximum and, if applicable, minimum safe operating limit must be

marked with a red radial or a red line;

(b) Each normal operating range must be marked with a green arc or green

line, not extending beyond the maximum and minimum safe limits;

(c) Each takeoff and precautionary range must be marked with a yellow arc

or a yellow line; and

(d) Each engine, auxiliary power unit, or propeller range that is

restricted because of excessive vibration stresses must be marked with red

arcs or red lines.

[Amdt. 23-12, 41 FR 55466, Dec. 20, 1976, as amended by Amdt. 23-28, 47 FR

13315, Mar. 29, 1982; Amdt. No. 23-45, 58 FR 42166, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1551   Oil quantity indicator.

Each oil quantity indicator must be marked in sufficient increments to

indicate readily and accurately the quantity of oil.

Sec. 23.1553   Fuel quantity indicator.

If the unusable fuel supply for any tank exceeds one gallon, or five

percent of the tank capacity, whichever is greater, a red arc must be marked

on its indicator extending from the calibrated zero reading to the lowest

reading obtainable in level flight.

Sec. 23.1555   Control markings.

(a) Each cockpit control, other than primary flight controls and simple

push button type starter switches, must be plainly marked as to its function

and method of operation.

(b) Each secondary control must be suitably marked.

(c) For powerplant fuel controls--

(1) Each fuel tank selector control must be marked to indicate the position

corresponding to each tank and to each existing cross feed position;

(2) If safe operation requires the use of any tanks in a specific sequence,

that sequence must be marked on or near the selector for those tanks;

(3) The conditions under which the full amount of usable fuel in any

restricted usage fuel tank can safely be used must be stated on a placard

adjacent to the selector valve for that tank; and

(4) Each valve control for any engine of a multiengine airplane must be

marked to indicate the position corresponding to each engine controlled.

(d) Usable fuel capacity must be marked as follows:

(1) For fuel systems having no selector controls, the usable fuel capacity

of the system must be indicated at the fuel quantity indicator.

(2) For fuel systems having selector controls, the usable fuel capacity

available at each selector control position must be indicated near the

selector control.

(e) For accessory, auxiliary, and emergency controls--

(1) If retractable landing gear is used, the indicator required by Sec.

23.729 must be marked so that the pilot can, at any time, ascertain that the

wheels are secured in the extreme positions; and

(2) Each emergency control must be red and must be marked as to method of

operation.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-21, 43 FR 2319, Jan. 16, 1978]

Sec. 23.1557   Miscellaneous markings and placards.

(a) Baggage and cargo compartments, and ballast location. Each baggage and

cargo compartment, and each ballast location, must have a placard stating any

limitations on contents, including weight, that are necessary under the

loading requirements.

(b) Seats. If the maximum allowable weight to be carried in a seat is less

than 170 pounds, a placard stating the lesser weight must be permanently

attached to the seat structure.

(c) Fuel, oil, and coolant filler openings. The following apply:

(1) Fuel filter openings must be marked at or near the filler cover with--

(i) For reciprocating engine-powered airplanes--

(A) The word "Avgas"; and

(B) The minimum fuel grade.

(ii) For turbine engine-powered airplanes--

(A) The words "Jet Fuel"; and

(B) The permissible fuel designations, or references to the Airplane Flight

Manual (AFM) for permissible fuel designations.

(iii) For pressure fueling systems, the maximum permissible fueling supply

pressure and the maximum permissible defueling pressure.

(2) Oil filler openings must be marked at or near the filler cover with the

word "Oil" and the permissible oil designations, or references to the

Airplane Flight Manual (AFM) for permissible oil designations.

(3) Coolant filler openings must be marked at or near the filler cover with

the word "Coolant".

(d) Emergency exit placards. Each placard and operating control for each

emergency exit must be red. A placard must be near each emergency exit

control and must clearly indicate the location of that exit and its method of

operation.

(e) The system voltage of each direct current installation must be clearly

marked adjacent to its exernal power connection.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; as amended by Amdt. 23-21, 42 FR

15042, Mar. 17, 1977; Amdt. 23-23, 43 FR 50594, Oct. 30, 1978; Amdt. No.

23-45, 58 FR 42166, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1559   Operating limitations placard.

(a) There must be a placard in clear view of the pilot stating--

(1) For airplanes certificated in one category:

The markings and placards installed in this airplane contain operating

limitations which must be complied with when operating this airplane in the

-------------------- category. (Insert category.) Other operating limitations

which must be complied with when operating this airplane in this category are

contained in the Airplane Flight Manual.

(2) For airplanes certificated in more than one category:

The markings and placards installed in this airplane contain operating

limitations which must be complied with when operating this airplane in the

---------- category. (Insert category.) Other operating limitations which

must be complied with when operating this airplane in this category or in the

---------- category are contained in the Airplane Flight Manual. (Insert

category or categories.)

(b) There must be a placard in clear view of the pilot that specifies the

kind of operations (such as VFR, IFR, day, or night) and the meteorological

conditions (such as icing conditions) to which the operation of the airplane

is limited, or from which it is prohibited, by the equipment installed.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-13, 37 FR 20023, Sept. 23, 1972; 37 FR 21320, Oct. 7,

1972; Amdt. 23-21, 43 FR 2319, Jan. 16, 1978]

Sec. 23.1561   Safety equipment.

(a) Safety equipment must be plainly marked as to method of operation.

(b) Stowage provisions for required safety equipment must be marked for the

benefit of occupants.

Sec. 23.1563   Airspeed placards.

There must be an airspeed placard in clear view of the pilot and as close

as practicable to the airspeed indicator. This placard must list--

(a) The operating maneuvering speed, Vo; and

(b) The maximum landing gear operating speed VLO.

[Amdt. 23-7, 34 FR 13097, Aug. 13, 1969, as amended by Amdt. No. 23-45, 58

FR 42166, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1567   Flight maneuver placard.

(a) For normal category airplanes, there must be a placard in front of and

in clear view of the pilot stating: "No acrobatic maneuvers, including spins,

approved."

(b) For utility category airplanes, there must be--

(1) A placard in clear view of the pilot stating: "Acrobatic maneuvers are

limited to the following ------" (list approved maneuvers and the recommended

entry speed for each); and

(2) For those airplanes that do not meet the spin requirements for

acrobatic category airplanes, an additional placard in clear view of the

pilot stating: "Spins Prohibited."

(c) For acrobatic category airplanes, there must be a placard in clear view

of the pilot listing the approved acrobatic maneuvers and the recommended

entry airspeed for each. If inverted flight maneuvers are not approved, the

placard must bear a notation to this effect.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. 23-13, 37 FR 20023, Sept. 23, 1972; Amdt. 23-21, 43 FR 2319,

Jan. 16, 1978]

Airplane Flight Manual and Approved Manual Material

Sec. 23.1581   General.

(a) Furnishing information. An Airplane Flight Manual must be furnished

with each airplane, and it must contain the following:

(1) Information required by Secs. 23.1583 through 23.1589.

(2) Other information that is necessary for safe operation because of

design, operating, or handling characteristics.

(b) Approved information. (1) Except as provided in paragraph (b)(2) of

this section, each part of the Airplane Flight Manual containing information

prescribed in Secs. 23.1583 through 23.1589 must be approved, segregated,

identified and clearly distinguished from each unapproved part of that

Airplane Flight Manual.

(2) The requirements of paragraph (b)(1) of this section do not apply if

the following is met:

(i) Each part of the Airplane Flight Manual containing information

prescribed in Sec. 23.1583 must be limited to such information, and must be

approved, identified, and clearly distinguished from each other part of the

Airplane Flight Manual.

(ii) The information prescribed in Secs. 23.1585 through 23.1589 must be

determined in accordance with the applicable requirements of this part and

presented in its entirety in a manner acceptable to the Administrator.

(3) Each page of the Airplane Flight Manual containing information

prescribed in this section must be of a type that is not easily erased,

disfigured, or misplaced, and is capable of being inserted in a manual

provided by the applicant, or in a folder, or in any other permanent binder.

(c) [Reserved]

(d) Table of contents. Each Airplane Flight Manual must include a table of

contents if the complexity of the manual indicates a need for it.

(e) Provision must be made for stowing the Airplane Flight Manual in a

suitable fixed container which is readily accessible to the pilot.

(f) Revisions and amendments. Each Airplane Flight Manual (AFM) must

contain a means for recording the incorporation of revisions and amendments.

[Amdt. 23-21, 43 FR 2319, Jan. 16, 1978, as amended by Amdt. 23-34, 52 FR

1834, Jan. 15, 1987; Amdt. No. 23-45, 58 FR 42166, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1583  Operating limitations.

Operating limitations determined during type certification must be stated,

including the following:

(a) Airspeed limitations. The following information must be furnished:

(1) Information necessary for the marking of the airspeed limits on the

indicator as required in Sec. 23.1545, and the significance of each of those

limits and of the color coding used on the indicator.

(2) The speeds VMC, VO, VLE, and VLO, if established, and their

significance.

(3) In addition, for commuter category airplanes--

(i) The maximum operating limit speed, VMO/MMO and a statement that this

speed may not be deliberately exceeded in any regime of flight (climb,

cruise, or descent) unless a higher speed is authorized for flight test or

pilot training;

(ii) If an airspeed limitation is based upon compressibility effects, a

statement to this effect and information as to any symptoms, the probable

behavior of the airplane, and the recommended recovery procedures; and

(iii) The airspeed limits must be shown in terms of VMO/MMO instead of VNO

and VNE.

(b) Powerplant limitations. The following information must be furnished:

(1) Limitations required by Sec. 23.1521.

(2) Explanation of the limitations, when appropriate.

(3) Information necessary for marking the instruments required by Sec.

23.1549 through Sec. 23.1553.

(c) Weight. The airplane flight manual must include--

(1) The maximum weight; and

(2) The maximum landing weight, if the design landing weight selected by

the applicant is less than the maximum weight.

(3) In addition, for commuter category airplanes, the maximum takeoff

weight for each altitude, ambient temperature, and required takeoff runway

length within the range selected by the applicant may not exceed the weight

at which--

(i) The all-engine-operating distance determined under Sec. 23.59 or the

accelerate-stop distance determined under Sec. 23.55, whichever is greater,

is equal to the available runway length;

(ii) The airplane complies with the one-engine-inoperative takeoff distance

requirements of Sec. 23.59; and

(iii) The airplane complies with the one-engine-inoperative takeoff and en

route climb requirements of Secs. 23.57 and 23.67.

(4) In addition, for commuter category airplanes, the maximum landing

weight for each altitude, ambient temperature, and required landing runway

length, within the range selected by the applicant. The maximum landing

weights may not exceed:

(i) The weight at which the landing distance is determined under Sec.

23.75; or

(ii) The weight at which compliance with Sec. 23.77 is shown.

(d) Center of gravity. The established center of gravity limits must be

furnished.

(e) Maneuvers. The following authorized maneuvers, appropriate airspeed

limitations, and unauthorized maneuvers must be furnished as prescribed in

this section.

(1) Normal category airplanes. For normal category airplanes, acrobatic

maneuvers, including spins, are unauthorized. If the airplane has been shown

to be "characteristically incapable of spinning" under Sec. 23.221(d), a

statement to this effect must be entered. Other normal category airplanes

must be placarded against spins.

(2) Utility category airplanes. For utility category airplanes, authorized

maneuvers shown in the type flight tests must be furnished, together with

recommended entry speeds. No other maneuver is authorized. If the airplane

has been shown to be "characteristically incapable of spinning" under Sec.

23.221(d), a statement to this effect must be entered.

(3) Acrobatic category airplanes. For acrobatic category airplanes, the

approved flight maneuvers shown in the type flight tests must be included,

together with recommended entry speeds. A placard listing the use of the

controls required to recover from spinning maneuvers must be in the cockpit.

(4) Commuter category airplanes. For commuter category airplanes, acrobatic

maneuvers, including spins, are unauthorized.

(f) Flight load factor. The positive limit load factors, in g's, must be

furnished.

(g) Flight crew. If a flight crew of more than one is required for safety,

the number and functions of the minimum flight crew must be furnished.

(h) Kinds of operation. A list of the kinds of operation to which the

airplane is limited or from which it is prohibited under Sec. 23.1525, and

also a list of installed equipment that affects any operating limitation and

identification as to the equipment's required operational status for the

kinds of operation for which approval has been given.

(i)--(j) [Reserved]

(k) Maximum operating altitude. The maximum altitude established under Sec.

23.1527 must be furnished.

(l) Maximum passenger seating configuration.  The maximum passenger seating

configuration must be furnished.

(m) Allowable lateral fuel loading. The maximum allowable lateral fuel

loading differential must be furnished if less than the maximum possible.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13097, Aug. 13, 1969; Amdt. 23-10, 36 FR 2864, Feb. 11, 1971; Amdt. 23-21, 43

FR 2320, Jan. 16, 1978; Amdt. 23-23, 43 FR 50594, Oct. 30, 1978; Amdt. 23-34,

52 FR 1834, Jan. 15, 1987; Amdt. No. 23-45, 58 FR 42166, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1585  Operating procedures.

(a) For each airplane, information concerning normal, abnormal, and

emergency procedures and other pertinent information necessary for safe

operation and the achievement of the scheduled performance must be identified

and segregated, including--

(1) The maximum demonstrated values of crosswind velocity for takeoff and

landing and procedures and information pertinent to operations in crosswinds;

(2) The speeds, configurations, and procedures for making a normal takeoff

and the subsequent climb;

(3) Procedure for abandoning a takeoff due to engine failure or other

cause;

(4) The recommended climb speeds, and any variation with altitude;

(5) An explanation of significant or unusual flight or ground handling

characteristics of the airplane;

(6) A recommended speed for flight in rough air. This speed must be chosen

to protect against the occurrence, as a result of gusts, of structural damage

to the airplane and loss of control (for example, stalling); and

(7) For seaplanes and amphibians, water handling procedures and the

demonstrated wave height.

(b) For single-engine airplanes, the procedures, speeds, and configurations

for a glide following an engine failure and subsequent forced landing.

(c) For multiengine airplanes, the information must include--

(1) Procedures and speeds for continuing a takeoff following failure of the

critical engine and the conditions under which takeoff can be safely

continued, or a warning against attempting to continue the takeoff;

(2) Procedures, speeds, and configurations for continuing a climb following

engine failure after takeoff or en route;

(3) Procedures, speeds, and configurations for making an approach and

landing with one engine inoperative;

(4) Procedures, speeds, and configurations for making a go-around with one

engine inoperative and the conditions under which the go-around can safely be

executed, or a warning against attempting the go-around maneuver;

(5) Procedures for restarting engines in flight, including the effects of

altitude, must be set forth in the Airplane Flight Manual (AFM); and

(6) The VSSE determined in Sec. 23.149.

(d) For multiengine airplanes, information identifying each operating

condition in which the fuel system independence prescribed in Sec. 23.953 is

necessary for safety must be furnished, together with instructions for

placing the fuel system in a configuration used to show compliance with that

section.

(e) For each airplane showing compliance with Sec. 23.1353 (g)(2) or

(g)(3), the operating procedures for disconnecting the battery from its

charging source must be furnished.

(f) If the unusable fuel supply in any tank exceeds 5 percent of the tank

capacity, or 1 gallon, whichever is greater, information must be furnished

which indicates that when the fuel quantity indicator reads "zero" in level

flight, any fuel remaining in the fuel tank cannot be used safely in flight.

(g) Information on the total quantity of usable fuel for each fuel tank

must be furnished.

(h) In addition, for commuter category airplanes, the procedures for

restarting turbine engines in flight, including the effects of altitude, must

be set forth in the Airplane Flight Manual.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-3, 30 FR

14240, Nov. 13, 1965; Amdt. 23-5, 32 FR 6912, May 5, 1967; Amdt. 23-7, 34 FR

13097, Aug. 13, 1969; Amdt. 23-21, 43 FR 2320, Jan. 16, 1978; Amdt. 23-23, 43

FR 50594, Oct. 30, 1978; Amdt. 23-34, 52 FR 1835, Jan. 15, 1987; Amdt. No.

23-45, 58 FR 42166, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1587  Performance information.

The following information must be furnished:

(a) For normal, utility, and acrobatic category airplanes:

(1) The takeoff distance determined under Sec. 23.51 and the kind of runway

surface used in the tests.

(2) The climb gradient determined under Secs. 23.65 and 23.77, with the

associated airspeed, power, and the airplane configuration.

(3) The landing distance determined under Sec. 23.75.

(4) The one engine inoperative en route climb/descent gradients determined

under Sec. 23.67 for multiengine airplanes.

(5) The calculated approximate effect on takeoff distance, landing

distance, and climb performance for variations in--

(i) Altitude from sea level to 10,000 feet in a standard atmosphere and

cruise configuration; and

(ii) Temperature, at those altitudes from 60 deg.F below standard to 40

deg.F above standard.

(b) For skiplanes, a statement of the approximate reduction in climb

performance may be used instead of complete new data for the skiplane

configuration if--

(1) The landing gear is fixed in both the landplane and skiplane

configurations;

(2) The climb performance is not critical; and

(3) The climb reduction in the skiplane configuration does not exceed 50

feet per minute.

(c) For each airplane:

(1) Any loss of altitude more than 100 feet, or any pitch more than 30

degrees below level flight attitude, occurring during the recovery part of

maneuvers prescribed in Secs. 23.201(c) and 23.205, if applicable.

(2) The stalling speed, VSO, at maximum weight.

(3) The stalling speed, VS1, at maximum weight and with the landing gear

and wing flaps retracted and the effect upon this stalling speed of angles of

bank up to 60 degrees.

(4) The speed used in showing compliance with the cooling and climb

requirements of Secs. 23.1041 through 23.1047 if this speed is greater than

the best rate of climb with one engine inoperative for multiengine airplanes

and the maximum atmospheric temperature at which compliance with the cooling

requirements has been shown.

(d) Commuter category airplanes. In addition, for commuter category

airplanes, the Airplane Flight Manual must contain at least the following

performance information:

(1) Sufficient information so that the takeoff weight limits specified in

Sec. 23.1583 can be determined for all temperatures and altitudes within the

operational limitations selected by the applicant;

(2) The conditions under which the performance information was obtained

including the airspeed at the 50-foot height used to determine the landing

distance as required by Sec. 23.75;

(3) The performance information (determined by extrapolation and computed

for the range of weights between the maximum landing and maximum takeoff

weights) for--

(i) Climb in the landing configuration as determined by Sec. 23.77; and

(ii) Landing distance as determined by Sec. 23.75;

(4) Procedures information established in accordance with the limitations

and other information for safe operation of the airplane in the form of

recommended procedures;

(5) An explanation of significant or unusual flight and ground handling

characteristics of the airplane; and

(6) Airspeed, as calibrated airspeed, corresponding to those established

while showing compliance to Sec. 23.53, Takeoff speeds. The calibrated

airspeed may be shown in units of indicated airspeed, and identified as

indicated airspeed, provided that all pressure sensing and instrumentation

errors, including the indicator, are accounted for in the flight manual

data.

[Amdt. 23-21, 43 FR 2320, Jan. 16, 1978, as amended by Amdt. 23-28, 47 FR

13315, Mar. 29, 1982; Amdt. 23-34, 52 FR 1835, Jan. 15, 1987; Amdt. 23-39,

55 FR 18575, May 2, 1990; Amdt. No. 23-45, 58 FR 42167, Aug. 6, 1993; 58 FR

51970, Oct. 5, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Sec. 23.1589  Loading information.

The following loading information must be furnished:

(a) The weight and location of each item of equipment that can be easily

removed, relocated, or replaced and that is installed when the airplane was

weighed under the requirement of Sec. 23.25.

(b) Appropriate loading instr@ctions for each possible loading condition

between the maximum and minimum weights determined under Sec. 23.25 that can

result in a center of gravity beyond--

(1) The extremes selected by the applicant;

(2) The extremes within which the structure is proven; or

(3) The extremes within which compliance with each functional requirement

is shown.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42167, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Appendix A to Part 23--Simplified Design Load Criteria for Conventional,

Single-Engine Airplanes of 6,000 Pounds or Less Maximum Weight

A23.1  General.

(a) The design load criteria in this appendix are an approved equivalent of

those in Secs. 23.321 through 23.459 of this subchapter for the certification

of conventional, single-engine airplanes of 6,000 pounds or less maximum

weight.

(b) Unless otherwise stated, the nomenclature and symbols in this Appendix

are the same as the corresponding nomenclature and symbols in Part 23.

A23.3  Special symbols.

n1 =Airplane Positive Maneuvering Limit Load Factor.

n2 =Airplane Negative Maneuvering Limit Load Factor.

n3 =Airplane Positive Gust Limit Load Factor at VC.

n4 =Airplane Negative Gust Limit Load Factor at VC.

nflap =Airplane Positive Limit Load Factor With Flaps Fully Extended at VF.

* V     = Minimum Design Flap Speed =

F min   11.0 <radical> n1 W/S   [kts]

* V     = Mimimum Design Manuevering

A min   Speed = 15.0 <radical> n1 W/S  [kts]

* V     = Minimum Design Cruising Speed

C min  = 17.0 <radical> n1 W/S    [kts]

* V     = Minimum Design Dive Speed =

D min   24.0 <radical> n1 W/S    [kts]

A23.5  Certification in more than one category.

The criteria in this appendix may be used for certification in the normal,

utility, and acrobatic categories, or in any combination of these categories.

If certification in more than one category is desired, the design category

weights must be selected to make the term " n1W " constant for all categories

or greater for one desired category than for others. The wings and control

surfaces (including wing flaps and tabs) need only be investigated for the

maximum value of " n1W ", or for the category corresponding to the maximum

design weight, where " n1W " is constant. If the acrobatic category is

selected, a special unsymmetrical flight load investigation in accordance

with paragraphs A23.9(c)(2) and A23.11(c)(2) of this appendix must be

completed. The wing, wing carrythrough, and the horizontal tail structures

must be checked for this condition. The basic fuselage structure need only be

investigated for the highest load factor design category selected. The local

supporting structure for dead weight items need only be designed for the

highest load factor imposed when the particular items are installed in the

airplane. The engine mount, however, must be designed for a higher side load

factor, if certification in the acrobatic category is desired, than that

required for certification in the normal and utility categories. When

designing for landing loads, the landing gear and the airplane as a whole

need only be investigated for the category corresponding to the maximum

design weight. These simplifications apply to single-engine aircraft of

conventional types for which experience is available, and the Administrator

may require additional investigations for aircraft with unusual design

features.

A23.7  Flight loads.

(a) Each flight load may be considered independent of altitude and, except

for the local supporting structure for dead weight items, only the maximum

design weight conditions must be investigated.

(b) Table 1 and figures 3 and 4 of this Appendix must be used to determine

values of n1, n2, n3, and n4, corresponding to the maximum design weights in

the desired categories.

(c) Figures 1 and 2 of this Appendix must be used to determine values of n3

and n4  corresponding to the minimum flying weights in the desired

categories, and, if these load factors are greater than the load factors at

the design weight, the supporting structure for dead weight items must be

substantiated for the resulting higher load factors.

(d) Each specified wing and tail loading is independent of the center of

gravity range. The applicant, however, must select a c.g. range, and the

basic fuselage structure must be investigated for the most adverse dead

weight loading conditions for the c.g. range selected.

(e) The following loads and loading conditions are the minimums for which

strength must be provided in the structure:

(1) Airplane equilibrium. The aerodynamic wing loads may be considered to

act normal to the relative wind, and to have a magnitude of 1.05 times the

airplane normal loads (as determined from paragraphs A23.9 (b) and (c) of

this appendix) for the positive flight conditions and a magnitude equal to

the airplane normal loads for the negative conditions. Each chordwise and

normal component of this wing load must be considered.

(2) Minimum design airspeeds. The minimum design airspeeds may be chosen by

the applicant except that they may not be less than the minimum speeds found

by using figure 3 of this Appendix. In addition, VCmin need not exceed values

of 0.9 VH actually obtained at sea level for the lowest design weight

category for which certification is desired. In computing these minimum

design airspeeds, n1 may not be less than 3.8.

(3) Flight load factor. The limit flight load factors specified in Table 1

of this Appendix represent the ratio of the aerodynamic force component

(acting normal to the assumed longitudinal axis of the airplane) to the

weight of the airplane. A positive flight load factor is an aerodynamic force

acting upward, with respect to the airplane.

A23.9  Flight conditions.

(a) General. Each design condition in paragraphs (b) and (c) of this

section must be used to assure sufficient strength for each condition of

speed and load factor on or within the boundary of a V-n diagram for the

airplane similar to the diagram in figure 4 of this Appendix. This diagram

must also be used to determine the airplane structural operating limitations

as specified in Secs. 23.1501(c) through 23.1513 and Sec. 23.1519.

(b) Symmetrical flight conditions. The airplane must be designed for

symmetrical flight conditions as follows:

(1) The airplane must be designed for at least the four basic flight

conditions, "A", "D", "E", and "G" as noted on the flight envelope of figure

4 of this Appendix. In addition, the following requirements apply:

(i) The design limit flight load factors corresponding to conditions "D"

and "E" of figure 4 must be at least as great as those specified in Table 1

and figure 4 of this Appendix, and the design speed for these conditions must

be at least equal to the value of VD found from figure 3 of this Appendix.

(ii) For conditions "A" and "G" of figure 4, the load factors must

correspond to those specified in Table 1 of this Appendix, and the design

speeds must be computed using these load factors with the maximum static lift

coefficient CNA determined by the applicant. However, in the absence of more

precise computations, these latter conditions may be based on a value of CNA

=+/-1.35 and the design speed for condition "A" may be less than VAmin.

(iii) Conditions "C" and "F" of figure 4 need only be investigated when

n3W/S or n4W/S are greater than n1W/S or n2W/S  of this Appendix,

respectively.

(2) If flaps or other high lift devices intended for use at the relatively

low airspeed of approach, landing, and takeoff, are installed, the airplane

must be designed for the two flight conditions corresponding to the values of

limit flap-down factors specified in Table 1 of this Appendix with the flaps

fully extended at not less than the design flap speed VFmin from figure 3 of

this Appendix.

(c) Unsymmetrical flight conditions. Each affected structure must be

designed for unsymmetrical loadings as follows:

(1) The aft fuselage-to-wing attachment must be designed for the critical

vertical surface load determined in accordance with paragraph SA23.11(c) (1)

and (2) of this Appendix.

(2) The wing and wing carry-through structures must be designed for 100

percent of condition "A" loading on one side of the plane of symmetry and 70

percent on the opposite side for certification in the normal and utility

categories, or 60 percent on the opposite side for certification in the

acrobatic category.

(3) The wing and wing carry-through structures must be designed for the

loads resulting from a combination of 75 percent of the positive maneuvering

wing loading on both sides of the plane of symmetry and the maximum wing

torsion resulting from aileron displacement. The effect of aileron

displacement on wing torsion at VC or VA using the basic airfoil moment

coefficient modified over the aileron portion of the span, must be computed

as follows:

(i) Cm=Cm +0.01<delta><mu> (up aileron side) wing basic airfoil.

(ii) Cm=Cm -0.01<delta><mu>(down aileron side) wing basic airfoil, where

<delta><mu> is the up aileron deflection and <delta>d is the down aileron

deflection.

(4) <Delta> critical, which is the sum of <delta><mu>+<delta>d must be

computed as follows:

(i) Compute <Delta><alpha> and <Delta>b from the formulas:

VA

D a =  --  x D p    and

VC

VA

Db = 0.5  --  x Dp

VD

Where Dp = the maximum total deflection (sum of both aileron deflections) at

VA with VA, VC, and VD described in subparagraph (2) of Sec. 23.7(e) of this

Appendix.

(ii) Compute K from the formula:

(Cm-0.01db) VD2

K =  ------------------

(Cm-0.01da) VC2

where <delta><alpha> is the down aileron deflection corresponding to

<Delta><alpha>, and <delta>b is the down aileron deflection corresponding to

<Delta>b as computed in step (i).

(iii) If K is less than 1.0, <Delta><alpha> is <Delta> critical and must be

used to determine <delta>u and <delta>d. In this case, VC is the critical

speed which must be used in computing the wing torsion loads over the aileron

span.

(iv) If K is equal to or greater than 1.0, <Delta>b is <Delta> critical and

must be used to determine <delta>u and <delta>d. In this case, Vd is the

critical speed which must be used in computing the wing torsion loads over

the aileron span.

(d) Supplementary conditions; rear lift truss; engine torque; side load on

engine mount. Each of the following supplementary conditions must be

investigated:

(1) In designing the rear lift truss, the special condition specified in

Sec. 23.369 may be investigated instead of condition "G" of figure 4 of this

Appendix. If this is done, and if certification in more than one category is

desired, the value of W/S used in the formula appearing in Sec. 23.369 must

be that for the category corresponding to the maximum gross weight.

(2) Each engine mount and its supporting structures must be designed for

the maximum limit torque corresponding to METO power and propeller speed

acting simultaneously with the limit loads resulting from the maximum

positive maneuvering flight load factor n1. The limit torque must be obtained

by multiplying the mean torque by a factor of 1.33 for engines with five or

more cylinders. For 4, 3, and 2 cylinder engines, the factor must be 2, 3,

and 4, respectively.

(3) Each engine mount and its supporting structure must be designed for the

loads resulting from a lateral limit load factor of not less than 1.47 for

the normal and utility categories, or 2.0 for the acrobatic category.

A23.11  Control surface loads.

(a) General. Each control surface load must be determined using the

criteria of paragraph (b) of this section and must lie within the simplified

loadings of paragraph (c) of this section.

(b) Limit pilot forces. In each control surface loading condition described

in paragraphs (c) through (e) of this section, the airloads on the movable

surfaces and the corresponding deflections need not exceed those which could

be obtained in flight by employing the maximum limit pilot forces specified

in the table in Sec. 23.397(b). If the surface loads are limited by these

maximum limit pilot forces, the tabs must either be considered to be

deflected to their maximum travel in the direction which would assist the

pilot or the deflection must correspond to the maximum degree of "out of

trim" expected at the speed for the condition under consideration. The tab

load, however, need not exceed the value specified in Table 2 of this

Appendix.

(c) Surface loading conditions. Each surface loading condition must be

investigated as follows:

(1) Simplified limit surface loadings and distributions for the horizontal

tail, vertical tail, aileron, wing flaps, and trim tabs are specified in

Table 2 and figures 5 and 6 of this Appendix. If more than one distribution

is given, each distribution must be investigated.

(2) If certification in the acrobatic category is desired, the horizontal

tail must be investigated for an unsymmetrical load of 100 percent w on one

side of the airplane centerline and 50 percent on the other side of the

airplane centerline.

(d) Outboard fins. Outboard fins must meet the requirements of Sec. 23.455.

(e) Special devices. Special devices must meet the requirements of Sec.

23.459.

A23.13  Control system loads.

(a) Primary flight controls and systems. Each primary flight control and

system must be designed as follows:

(1) The flight control system and its supporting structure must be designed

for loads corresponding to 125 percent of the computed hinge moments of the

movable control surface in the conditions prescribed in A23.11 of this

Appendix. In addition--

(i) The system limit loads need not exceed those that could be produced by

the pilot and automatic devices operating the controls; and

(ii) The design must provide a rugged system for service use, including

jamming, ground gusts, taxiing downwind, control inertia, and friction.

(2) Acceptable maximum and minimum limit pilot forces for elevator,

aileron, and rudder controls are shown in the table in Sec. 23.397(b). These

pilots loads must be assumed to act at the appropriate control grips or pads

as they would under flight conditions, and to be reacted at the attachments

of the control system to the control surface horn.

(b) Dual controls. If there are dual controls, the systems must be designed

for pilots operating in opposition, using individual pilot loads equal to 75

percent of those obtained in accordance with paragraph (a) of this section,

except that individual pilot loads may not be less than the minimum limit

pilot forces shown in the table in Sec. 23.397(b).

(c) Ground gust conditions. Ground gust conditions must meet the

requirements of Sec. 23.415.

(d) Secondary controls and systems. Secondary controls and systems must

meet the requirements of Sec. 23.405.

Table 1--Limit Flight Load Factors

[Limit flight load factors]

Flight load   Normal   Utility   Acrobatic

factors    category  category  category

Flaps up:

n1               3.8       4.4        6.0

n2           -0.5 n1

n3             (/1/)

n4             (/2/)

Flaps down:

n flap        0.5 n1

n flap      /3/ Zero

/1/ Find n3 from Fig. 1

/2/ Find n4 from Fig. 2

/3/ Vertical wing load may be assumed

equal to zero and only the flap part of

the wing need be checked for this

condition.

[ ...Illustration appears here... ]

[ ...Illustration appears here... ]

[ ...Illustration appears here... ]

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13097, Aug. 13, 1969; 34 FR 14727, Sept. 24, 1969; Amdt. 23-16, 40 FR 2577,

Jan. 14, 1975; Amdt. 23-28, 47 FR 13315, Mar. 29, 1982]

Appendix B to Part 23--[Reserved]

*****************************************************************************

56 FR 344, No. 2, Jan. 3, 1991

SUMMARY: This final rule upgrades the airworthiness standards for normal,

utility, acrobatic, and commuter category airplanes. This amendment provides

airworthiness standards for advancements in technology being incorporated in

current designs, permits type certification of spin resistant airplanes, and

reduces the regulatory burden in showing compliance with some of the

requirements for the design and type certification of small airplanes. These

new and amended airworthiness standards also result in the need for new

definitions. As a result, new definitions are added.

DATES: February 4, 1991.

*****************************************************************************

Appendix C to Part 23--Basic Landing Conditions

[C23.1 Basic landing conditions]

Tail wheel type

Tail-down

Condition                      Level landing      landing

Reference section                             23.479(a)(1)       23.481(a)(1)

Vertical component at c. g                    nW                 nW

Fore and aft component at c. g                KnW                0

Lateral component in either direction at c.

g                                            0                  0

Shock absorber extension (hydraulic shock

absorber)                                    Note (2)           Note (2)

Shock absorber deflection (rubber or spring

shock absorber), percent                     100                100

Tire deflection                               Static             Static

Main wheel loads (both wheels) (Vr)           (n-L)W             (n-L)W b/d

Main wheel loads (both wheels) (Dr)           KnW                0

Tail (nose) wheel loads (Vf)                  0                  (n-L)W a/d

Tail (nose) wheel loads (Df)                  0                  0

Notes                                         (1), (3), and (4)  (4)

[ ...Table continues... ]

Nose wheel type

Level landing

Level landing    with nose wheel

with inclined     just clear of

Condition          reactions          ground         Tail-down landing

Reference section    23.479(a)(2)(i)  23.479(a)(2)(ii)   23.481(a)(2) and

(b).

Vertical component   nW               nW                 nW.

at c. g

Fore and aft         KnW              KnW                0.

component at c. g

Lateral component    0                0                  0.

in either

direction at c. g

Shock absorber       Note (2)         Note (2)           Note (2).

extension

(hydraulic shock

absorber)

Shock absorber       100              100                100.

deflection (rubber

or spring shock

absorber), percent

Tire deflection      Static           Static             Static.

Main wheel loads     (n-L)W a'/d'     (n-L)W             (n-L)W.

(both wheels) (Vr)

Main wheel loads     KnW a'/d'        KnW                0.

(both wheels) (Dr)

Tail (nose) wheel    (n-L)W b'/d'     0                  0.

loads (Vf)

Tail (nose) wheel    KnW b'/d'        0                  0.

loads (Df)

Notes                (1)              (1), (3), and (4)  (3) and (4).

Note (1). K may be determined as follows: K=0.25 for W=3,000 pounds or less;

K=0.33 for W=6,000 pounds or greater, with linear variation of K between

these weights.

Note (2). For the purpose of design, the maximum load factor is assumed to

occur throughout the shock absorber stroke from 25 percent deflection to 100

percent deflection unless otherwise shown and the load factor must be used

with whatever shock absorber extension is most critical for each element of

the landing gear.

Note (3). Unbalanced moments must be balanced by a rational or conservative

method.

Note (4). L is defined in Sec. 23.735(b).

Note (5). n is the limit inertia load factor, at the c.g. of the airplane,

selected under Sec. 23.473 (d), (f), and (g).

Basic Landing Conditions

[ ...Illustration appears here... ]

Level Landing

[ ...Illustration appears here... ]

Tail Down Landing

[ ...Illustration appears here... ]

Level Landing with Inclined Reactions

[ ...Illustration appears here... ]

Level Landing with Nose Wheel Just Clear of Ground

[ ...Illustration appears here... ]

Tail Down Landing

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964, as amended by Amdt. 23-7, 34 FR

13099, Aug. 13, 1969]

Appendix D to Part 23--Wheel Spin-Up and Spring-Back Loads

D23.1 Wheel spin-up loads.

(a) The following method for determining wheel spin-up loads for landing

conditions is based on NACA T.N. 863. However, the drag component used for

design may not be less than the drag load prescribed in Sec. 23.479(b).

FHmax =1/re <radical> 2Iw(VH--Vc)nFVmax/tS

where--

FHmax=maximum rearward horizontal force acting on the wheel (in pounds);

re=effective rolling radius of wheel under impact based on recommended

operating tire pressure (which may be assumed to be equal to the rolling

radius under a static load of njWe) in feet;

Iw=rotational mass moment of inertia of rolling assembly (in slug feet);

VH=linear velocity of airplane parallel to ground at instant of contact

(assumed to be 1.2 VS0,  in feet per second);

Vc=peripheral speed of tire, if prerotation is used (in feet per second)

(there must be a positive means of pre-rotation before pre-rotation may

be considered);

n=equals effective coefficient of friction (0.80 may be used);

FVmax=maximum vertical force on wheel (pounds)= njWe, where We and nj are

defined in Sec. 23.725;

ts=time interval between ground  contact and attainment of maximum vertical

force on wheel (seconds). (However, if the value of FVmax, from the above

equation exceeds 0.8 FVmax, the latter value must be used for FHmax.)

(b) The equation assumes a linear variation of load factor with time until

the peak load is reached and under this assumption, the equation determines

the drag force at the time that the wheel peripheral velocity at radius re

equals the airplane velocity. Most shock absorbers do not exactly follow a

linear variation of load factor with time. Therefore, rational or

conservative allowances must be made to compensate for these variations. On

most landing gears, the time for wheel spin-up will be less than the time

required to develop maximum vertical load factor for the specified rate of

descent and forward velocity. For exceptionally large wheels, a wheel

peripheral velocity equal to the ground speed may not have been attained at

the time of maximum vertical gear load. However, as stated above, the drag

spin-up load need not exceed 0.8 of the maximum vertical loads.

(c) Dynamic spring-back of the landing gear and adjacent structure at the

instant just after the wheels come up to speed may result in dynamic forward

acting loads of considerable magnitude. This effect must be determined, in

the level landing condition, by assuming that the wheel spin-up loads

calculated by the methods of this appendix are reversed. Dynamic spring-back

is likely to become critical for landing gear units having wheels of large

mass or high landing speeds.

[Doc. No. 4080, 29 FR 17955, Dec. 18, 1964; 30 FR 258, Jan. 9, 1965, as

amended by Amdt. No. 23-45, 58 FR 42167, Aug. 6, 1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************

Appendix E to Part 23--Limited Weight Credit for Airplanes Equipped With

Standby Power

(a) Each applicant for an increase in the maximum certificated takeoff and

landing weights of an airplane equipped with a typecertificated standby power

rocket engine may obtain an increase as specified in paragraph (b) if--

(1) The installation of the rocket engine has been approved and it has been

established by flight test that the rocket engine and its controls can be

operated safely and reliably at the increase in maximum weight; and

(2) The Airplane Flight Manual, or the placard, markings or manuals

required in place thereof, set forth in addition to any other operating

limitations the Administrator may require, the increased weight approved

under this regulation and a prohibition against the operation of the airplane

at the approved increased weight when--

(i) The installed standby power rocket engines have been stored or

installed in excess of the time limit established by the manufacturer of the

rocket engine (usually stenciled on the engine casing); or

(ii) The rocket engine fuel has been expended or discharged.

(b) The currently approved maximum takeoff and landing weights at which an

airplane is certificated without a standby power rocket engine installation

may be increased by an amount which does not exceed any of the following:

(1) An amount equal in pounds to 0.014 IN, where I is the maximum usable

impulse in pounds-seconds available from each standby power rocket engine and

N is the number of rocket engines installed.

(2) An amount equal to 5 percent of the maximum certificated weight

approved in accordance with the applicable airworthiness regulations without

standby power rocket engines installed.

(3) An amount equal to the weight of the rocket engine installation.

(4) An amount that, together with the currently approved maximum weight,

would equal the maximum structural weight established for the airplane

without standby rocket engines installed.

(c) For the purposes of this Appendix, "standby power" is power or thrust,

or both, obtained from rocket engines for a relatively short period and

actuated only in cases of emergency.

(d) For the purposes of limited weight credit for airplanes equipped with

standby power, as set forth in Sec. 23.25(a)(1)(iii) and this Appendix, an

airplane certificated under Part 4a of the Civil Air Regulations is treated

as if it had been certificated under Part 3 of the Civil Air Regulations or

Part 23 of the Federal Aviation Regulations.

[Amdt. 23-2, 30 FR 8468, July 2, 1965]

Appendix F to Part 23--Test Procedure

An Acceptable Test Procedure for Self-Extinguishing Materials for Showing

Compliance with Sec. 23.853.

(a) Conditioning. Specimens must be conditioned to 70 degrees F, plus or

minus 5 degrees, and at 50 percent plus or minus 5 percent relative humidity

until moisture equilibrium is reached or for 24 hours. Only one specimen at a

time may be removed from the conditioning environment immediately before

subjecting it to the flame.

(b) Specimen configuration. Materials must be tested either as a section

cut from a fabricated part as installed in the airplane or as a specimen

simulating a cut section, such as a specimen cut from a flat sheet of the

material or a model of the fabricated part. The specimen may be cut from any

location in a fabricated part; however, fabricated units, such as sandwich

panels, may not be separated for test. The specimen thickness must be no

thicker than the minimum thickness to be qualified for use in the airplane,

except that thick foam parts must be tested in 1/2 -inch thickness. In the

case of fabrics, both the warp and fill direction of the weave must be tested

to determine the most critical flammability conditions. When performing the

test prescribed in paragraphs (d) and (e) of this Appendix, the specimen must

be mounted in a metal frame so that: (1) The two long edges and the upper

edge are held securely; (2) the exposed area of the specimen is at least 2

inches wide and 12 inches long, unless the actual size used in the airplane

is smaller; and (3) the edge to which the burner flame is applied must not

consist of the finished or protected edge of the specimen but must be

representative of the actual cross section of the material or part installed

in the airplane.

(c) Apparatus. Except as provided in paragraph (e) of this Appendix, tests

must be conducted in a draft-free cabinet in accordance with Federal Test

Method Standard 191 Method 5903 (revised Method 5902) which is available from

the General Services Administration, Business Service Center, Region 3,

Seventh and D Streets SW., Washington, D.C. 20407, or with some other

approved equivalent method. Specimens which are too large for the cabinet

must be tested in similar draft-free conditions.

(d) Vertical test. A minimum of three specimens must be tested and the

results averaged. For fabrics, the direction of weave corresponding to the

most critical flammability conditions must be parallel to the longest

dimension. Each specimen must be supported vertically. The specimen must be

exposed to a Bunsen or Tirrill burner with a nominal 3/8 -inch I.D. tube

adjusted to give a flame of 1 1/2  inches in height. The minimum flame

temperature measured by a calibrated thermocouple pryometer in the center of

the flame must be 1550 deg. F. The lower edge of the specimen must be three-

fourths inch above the top edge of the burner. The flame must be applied to

the center line of the lower edge of the specimen. For materials covered by

Secs. 23.853(d)(3)(i) and 23.853(f), the flame must be applied for 60 seconds

and then removed. For materials covered by Sec. 23.853(d)(3)(ii), the flame

must be applied for 12 seconds and then removed. Flame time, burn length, and

flaming time of drippings, if any, must be recorded. The burn length

determined in accordance with paragraph (f) of this Appendix must be measured

to the nearest one-tenth inch.

(e) Horizontal test. A minimum of three specimens must be tested and the

results averaged. Each specimen must be supported horizontally. The exposed

surface when installed in the airplane must be face down for the test. The

specimen must be exposed to a Bunsen burner or Tirrill burner with a nominal

3/8 -inch I.D. tube adjusted to give a flame of 1 1/2  inches in height. The

minimum flame temperature measured by a calibrated thermocouple pyrometer in

the center of the flame must be 1550 deg. F. The specimen must be positioned

so that the edge being tested is three-fourths of an inch above the top of,

and on the center line of, the burner. The flame must be applied for 15

seconds and then removed. A minimum of 10 inches of the specimen must be used

for timing purposes, approximately 1 1/2  inches must burn before the burning

front reaches the timing zone, and the average burn rate must be recorded.

(f) Burn length. Burn length is the distance from the original edge to the

farthest evidence of damage to the test specimen due to flame impingement,

including areas of partial or complete consumption, charring, or

embrittlement, but not including areas sooted, stained, warped, or

discolored, nor areas where material has shrunk or melted away from the heat

source.

[Amdt. 23-23, 43 FR 50594, Oct. 30, 1978, as amended by Amdt. 23-34, 52 FR

1835, Jan. 15, 1987; 52 FR 34745, Sept. 14, 1987]

Appendix G to Part 23--Instructions for Continued Airworthiness

G23.1  General. (a) This appendix specifies requirements for the

preparation of Instructions for Continued Airworthiness as required by Sec.

23.1529.

(b) The Instructions for Continued Airworthiness for each airplane must

include the Instructions for Continued Airworthiness for each engine and

propeller (hereinafter designated 'products'), for each appliance required by

this chapter, and any required information relating to the interface of those

appliances and products with the airplane. If Instructions for Continued

Airworthiness are not supplied by the manufacturer of an appliance or product

installed in the airplane, the Instructions for Continued Airworthiness for

the airplane must include the information essential to the continued

airworthiness of the airplane.

(c) The applicant must submit to the FAA a program to show how changes to

the Instructions for Continued Airworthiness made by the applicant or by the

manufacturers of products and appliances installed in the airplane will be

distributed.

G23.2  Format. (a) The Instructions for Continued Airworthiness must be in

the form of a manual or manuals as appropriate for the quantity of data to be

provided.

(b) The format of the manual or manuals must provide for a practical

arrangement.

G23.3  Content. The contents of the manual or manuals must be prepared in

the English language. The Instructions for Continued Airworthiness must

contain the following manuals or sections, as appropriate, and information:

(a) Airplane maintenance manual or section. (1) Introduction information

that includes an explanation of the airplane's features and data to the

extent necessary for maintenance or preventive maintenance.

(2) A description of the airplane and its systems and installations

including its engines, propellers, and appliances.

(3) Basic control and operation information describing how the airplane

components and systems are controlled and how they operate, including any

special procedures and limitations that apply.

(4) Servicing information that covers details regarding servicing points,

capacities of tanks, reservoirs, types of fluids to be used, pressures

applicable to the various systems, location of access panels for inspection

and servicing, locations of lubrication points, lubricants to be used,

equipment required for servicing, tow instructions and limitations, mooring,

jacking, and leveling information.

(b) Maintenance instructions. (1) Scheduling information for each part of

the airplane and its engines, auxiliary power units, propellers, accessories,

instruments, and equipment that provides the recommended periods at which

they should be cleaned, inspected, adjusted, tested, and lubricated, and the

degree of inspection, the applicable wear tolerances, and work recommended at

these periods. However, the applicant may refer to an accessory, instrument,

or equipment manufacturer as the source of this information if the applicant

shows that the item has an exceptionally high degree of complexity requiring

specialized maintenance techniques, test equipment, or expertise. The

recommended overhaul periods and necessary cross reference to the

Airworthiness Limitations section of the manual must also be included. In

addition, the applicant must include an inspection program that includes the

frequency and extent of the inspections necessary to provide for the

continued airworthiness of the airplane.

(2) Troubleshooting information describing probable malfunctions, how to

recognize those malfunctions, and the remedial action for those malfunctions.

(3) Information describing the order and method of removing and replacing

products and parts with any necessary precautions to be taken.

(4) Other general procedural instructions including procedures for system

testing during ground running, symmetry checks, weighing and determining the

center of gravity, lifting and shoring, and storage limitations.

(c) Diagrams of structural access plates and information needed to gain

access for inspections when access plates are not provided.

(d) Details for the application of special inspection techniques including

radiographic and ultrasonic testing where such processes are specified.

(e) Information needed to apply protective treatments to the structure

after inspection.

(f) All data relative to structural fasteners such as identification,

discard recommendations, and torque values.

(g) A list of special tools needed.

(h) In addition, for commuter category airplanes, the following information

must be furnished:

(1) Electrical loads applicable to the various systems;

(2) Methods of balancing control surfaces;

(3) Identification of primary and secondary structures; and

(4) Special repair methods applicable to the airplane.

G23.4  Airworthiness Limitations section. The Instructions for Continued

Airworthiness must contain a section titled Airworthiness Limitations that is

segregated and@learly distinguishable from the rest of the document. This

section must set forth each mandatory replacement time, structural inspection

interval, and related structural inspection procedure required for type

certification. If the Instructions for Continued Airworthiness consist of

multiple documents, the section required by this paragraph must be included

in the principal manual. This section must contain a legible statement in a

prominent location that reads: "The Airworthiness Limitations section is FAA

approved and specifies maintenance required under Secs. 43.16 and 91.403 of

the Federal Aviation Regulations unless an alternative program has been FAA

approved."

[Amdt. 23-26, 45 FR 60171, Sept. 11, 1980, as amended by Amdt. 23-34, 52 FR

1835, Jan. 15, 1987; 52 FR 34745, Sept. 14, 1987; Amdt. 23-37, 54 FR 34329,

Aug. 18, 1989]

Effective Date Note: At 54 FR 34329, Aug. 18, 1989, Sec. G23.4 in Appendix

G, Part 23 was amended by changing the cross reference "Sec. 91.163" to "Sec.

91.403", effective August 18, 1990.

Appendix H to Part 23--Installation of An Automatic Power Reserve (APR)

System

H23.1, General.

(a) This appendix specifies requirements for installation of an APR engine

power control system that automatically advances power or thrust on the

operating engine(s) in the event any engine fails during takeoff.

(b) With the APR system and associated systems functioning normally, all

applicable requirements (except as provided in this appendix) must be met

without requiring any action by the crew to increase power or thrust.

H23.2, Definitions.

(a) Automatic power reserve system means the entire automatic system used

only during takeoff, including all devices both mechanical and electrical

that sense engine failure, transmit signals, actuate fuel controls or power

levers on operating engines, including power sources, to achieve the

scheduled power increase and furnish cockpit information on system operation.

(b) Selected takeoff power, notwithstanding the definition of "Takeoff

Power" in part 1 of the Federal Aviation Regulations, means the power

obtained from each initial power setting approved for takeoff.

(c) Critical Time Interval, as illustrated in figure H1, means that period

starting at V1 minus one second and ending at the intersection of the engine

and APR failure flight path line with the minimum performance all engine

flight path line. The engine and APR failure flight path line intersects the

one-engine-inoperative flight path line at 400 feet above the takeoff

surface. The engine and APR failure flight path is based on the airplane's

performance and must have a positive gradient of at least 0.5 percent at 400

feet above the takeoff surface.

Figure H1--Critical Time Interval Illustration

[INSERT: Line graph plotting engine and APR failure flight path

against minimum performace all engine flight path]

H23.3, Reliability and performance requirements.

(a) It must be shown that, during the critical time interval, an APR

failure that increases or does not affect power on either engine will not

create a hazard to the airplane, or it must be shown that such failures are

improbable.

(b) It must be shown that, during the critical time interval, there are no

failure modes of the APR system that would result in a failure that will

decrease the power on either engine or it must be shown that such failures

are extremely improbable.

(c) It must be shown that, during the critical time interval, there will be

no failure of the APR system in combination with an engine failure or it must

be shown that such failures are extremely improbable.

(d) All applicable performance requirements must be met with an engine

failure occurring at the most critical point during takeoff with the APR

system functioning normally.

H23.4, Power setting.

The selected takeoff power set on each engine at the beginning of the

takeoff roll may not be less than--

(a) The power necessary to attain, at V1, 90 percent of the maximum takeoff

power approved for the airplane for the existing conditions;

(b) That required to permit normal operation of all safety-related systems

and equipment that are dependent upon engine power or power lever position;

and

(c) That shown to be free of hazardous engine response characteristics when

power is advanced from the selected takeoff power level to the maximum

approved takeoff power.

H23.5, Powerplant controls--general.

(a) In addition to the requirements of Sec. 23.1141, no single failure or

malfunction (or probable combination thereof) of the APR, including

associated systems, may cause the failure of any powerplant function

necessary for safety.

(b) The APR must be designed to--

(1) Provide a means to verify to the flight crew before takeoff that the

APR is in an operating condition to perform its intended function;

(2) Automatically advance power on the operating engines following an

engine failure during takeoff to achieve the maximum attainable takeoff power

without exceeding engine operating limits;

(3) Prevent deactivation of the APR by manual adjustment of the power

levers following an engine failure;

(4) Provide a means for the flight crew to deactivate the automatic

function. This means must be designed to prevent inadvertent deactivation;

and

(5) Allow normal manual decrease or increase in power up to the maximum

takeoff power approved for the airplane under the existing conditions through

the use of power levers, as stated in Sec. 23.1141(c), except as provided

under paragraph (c) of H23.5 of this appendix.

(c) For airplanes equipped with limiters that automatically prevent engine

operating limits from being exceeded, other means may be used to increase the

maximum level of power controlled by the power levers in the event of an APR

failure. The means must be located on or forward of the power levers, must be

easily identified and operated under all operating conditions by a single

action of any pilot with the hand that is normally used to actuate the power

levers, and must meet the requirements of Sec. 23.777 (a), (b), and (c).

H23.6, Powerplant instruments.

In addition to the requirements of Sec. 23.1305:

(a) A means must be provided to indicate when the APR is in the armed or

ready condition.

(b) If the inherent flight characteristics of the airplane do not provide

warning that an engine has failed, a warning system independent of the APR

must be provided to give the pilot a clear warning of any engine failure

during takeoff.

(c) Following an engine failure at V1 or above, there must be means for the

crew to readily and quickly verify that the APR has operated satisfactorily.

[Amdt. 23-43, 58 FR 18979, Apr. 9, 1993]

*****************************************************************************

58 FR 18958, No. 67, Apr. 9, 1993

SUMMARY: This final rule amends the powerplant and equipment airworthiness

standards for normal, utility, acrobatic, and commuter category airplanes.

This amendment is based on certain proposals and recommendations discussed at

the Small Airplane Airworthiness Review Conference held on October 22-26,

1984, in St. Louis, Missouri, and arises from the recognition by both

government and industry, that upgraded standards are needed to maintain an

acceptable level of safety for small airplanes.

EFFECTIVE DATE: May 10, 1993.

*****************************************************************************

Appendix I to Part 23--Seaplane Loads

Appendix I

Figure 1. Pictorial definition of angles, dimensions, and directions

on a seaplane

[INSERT: Diagrams]

Figure 2. Hull station weighing factor

[INSERT: Diagrams]

Figure 3. Transverse pressure distributions

[INSERT: Diagrams]

[Amdt. No. 23-45, 58 FR 42167, Aug. 6, 1993; 58 FR 51970, 51971, Oct. 5,

1993]

*****************************************************************************

58 FR 42136, No. 150, Aug. 6, 1993

SUMMARY: This amendment changes airframe and flight worthiness standards for

normal, utility, acrobatic, and commuter category airplanes. The changes are

based on a number of recommendations discussed at the Small Airplane

Airworthiness Review Conference held on October 22-26, 1984, St. Louis,

Missouri. These updated safety standards will continue to provide an

acceptable level of safety in the design requirements for small airplanes

used in both private and commercial operations. Some of the changes provide

design requirements applicable to advancements in technology being

incorporated in current designs. This amendment will also reduce the

regulatory burden in showing compliance with some requirements while

maintaining an acceptable level of safety.

EFFECTIVE DATE: September 7, 1993.

*****************************************************************************