PART 101--MOORED BALLOONS, KITES, UNMANNED ROCKETS AND UNMANNED FREE BALLOONS

Subpart A--General

Sec.

101.1  Applicability.

101.3  Waivers.

101.5  Operations in prohibited or restricted areas.

101.7  Hazardous operations.

Subpart B--Moored Balloons and Kites

101.11  Applicability.

101.13  Operating limitations.

101.15  Notice requirements.

101.17  Lighting and marking requirements.

101.19  Rapid deflation device.

Subpart C--Unmanned Rockets

101.21  Applicability.

101.22  Special provisions for large model rockets.

101.23  Operating limitations.

101.25  Notice requirements.

Subpart D--Unmanned Free Balloons

101.31  Applicability.

101.33  Operating limitations.

101.35  Equipment and marking requirements.

101.37  Notice requirements.

101.39  Balloon position reports.

Authority: 49 U.S.C. app. 1348, 1354, 1372, 1421, 1442, 1443, 1472, 1510,

and 1522.

Subpart A--General

Sec. 101.1   Applicability.

(a) This part prescribes rules governing the operation in the United

States, of the following:

(1) Except as provided for in Sec. 101.7, any balloon that is moored to the

surface of the earth or an object thereon and that has a diameter of more

than 6 feet or a gas capacity of more than 115 cubic feet.

(2) Except as provided for in Sec. 101.7, any kite that weighs more than 5

pounds and is intended to be flown at the end of a rope or cable.

(3) Any unmanned rocket except:

(i) Aerial firework displays; and,

(ii) Model rockets:

(a) Using not more than four ounces of propellant;

(b) Using a slow-burning propellant;

(c) Made of paper, wood, or breakable plastic, containing no substantial

metal parts and weighing not more than 16 ounces, including the propellant;

and

(d) Operated in a manner that does not create a hazard to persons,

property, or other aircraft.

(4) Except as provided for in Sec. 101.7, any unmanned free balloon that--

(i) Carries a payload package that weighs more than four pounds and has a

weight/size ratio of more than three ounces per square inch on any surface of

the package, determined by dividing the total weight in ounces of the payload

package by the area in square inches of its smallest surface;

(ii) Carries a payload package that weighs more than six pounds;

(iii) Carries a payload, of two or more packages, that weighs more than 12

pounds; or

(iv) Uses a rope or other device for suspension of the payload that

requires an impact force of more than 50 pounds to separate the suspended

payload from the balloon.

(b) For the purposes of this part, a "gyroglider" attached to a vehicle on

the surface of the earth is considered to be a kite.

[Doc. No. 1580, 28 FR 6721, June 29, 1963, as amended by Amdt. 101-1, 29 FR

46, Jan. 3, 1964; Amdt. 101-3, 35 FR 8213, May 26, 1970]

Sec. 101.3   Waivers.

No person may conduct operations that require a deviation from this part

except under a certificate of waiver issued by the Administrator.

[Doc. No. 1580, 28 FR 6721, June 29, 1963]

Sec. 101.5   Operations in prohibited or restricted areas.

No person may operate a moored balloon, kite, unmanned rocket, or unmanned

free balloon in a prohibited or restricted area unless he has permission from

the using or controlling agency, as appropriate.

[Amdt. 101-1, 29 FR 46, Jan. 3, 1964]

Sec. 101.7   Hazardous operations.

(a) No person may operate any moored balloon, kite, unmanned rocket, or

unmanned free balloon in a manner that creates a hazard to other persons, or

their property.

(b) No person operating any moored balloon, kite, unmanned rocket, or

unmanned free balloon may allow an object to be dropped therefrom, if such

action creates a hazard to other persons or their property.

(Sec. 6(c), Department of Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 12800, Amdt. 101-4, 39 FR 22252, June 21, 1974]

Subpart B--Moored Balloons and Kites

Source: Docket No. 1580, 28 FR 6722 June 29, 1963, unless otherwise noted.

Sec. 101.11   Applicability.

This subpart applies to the operation of moored balloons and kites.

However, a person operating a moored balloon or kite within a restricted area

must comply only with Sec. 101.19 and with additional limitations imposed by

the using or controlling agency, as appropriate.

Sec. 101.13   Operating limitations.

(a) Except as provided in paragraph (b) of this section, no person may

operate a moored balloon or kite--

(1) Less than 500 feet from the base of any cloud;

(2) More than 500 feet above the surface of the earth;

(3) From an area where the ground visibility is less than three miles; or

(4) Within five miles of the boundary of any airport.

(b) Paragraph (a) of this section does not apply to the operation of a

balloon or kite below the top of any structure and within 250 feet of it, if

that shielded operation does not obscure any lighting on the structure.

Sec. 101.15   Notice requirements.

No person may operate an unshielded moored balloon or kite more than 150

feet above the surface of the earth unless, at least 24 hours before

beginning the operation, he gives the following information to the FAA ATC

facility that is nearest to the place of intended operation:

(a) The names and addresses of the owners and operators.

(b) The size of the balloon or the size and weight of the kite.

(c) The location of the operation.

(d) The height above the surface of the earth at which the balloon or kite

is to be operated.

(e) The date, time, and duration of the operation.

Sec. 101.17   Lighting and marking requirements.

(a) No person may operate a moored balloon or kite, between sunset and

sunrise unless the balloon or kite, and its mooring lines, are lighted so as

to give a visual warning equal to that required for obstructions to air

navigation in the FAA publication "Obstruction Marking and Lighting".

(b) No person may operate a moored balloon or kite between sunrise and

sunset unless its mooring lines have colored pennants or streamers attached

at not more than 50 foot intervals beginning at 150 feet above the surface of

the earth and visible for at least one mile.

(Sec. 6(c), Department of Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 1580, 28 FR 6722, June 29, 1963, as amended by Amdt. 101-4, 39 FR

22252, June 21, 1974]

Sec. 101.19   Rapid deflation device.

No person may operate a moored balloon unless it has a device that will

automatically and rapidly deflate the balloon if it escapes from its

moorings. If the device does not function properly, the operator shall

immediately notify the nearest ATC facility of the location and time of the

escape and the estimated flight path of the balloon.

Subpart C--Unmanned Rockets

Source: Docket No. 1580, 28 FR 6722, June 29, 1963, unless otherwise noted.

Sec. 101.21   Applicability.

This subpart applies to the operation of unmanned rockets. However, a

person operating an unmanned rocket within a restricted area must comply only

with Sec. 101.23(g) and with additional limitations imposed by the using or

controlling agency, as appropriate.

Sec. 101.22   Special provisions for large model rockets.

Persons operating model rockets that use not more than 125 grams of

propellant; that are made of paper, wood, or breakable plastic; that contain

no substantial metal parts, and that weigh not more than 1,500 grams,

including the propellant, need not comply with Sec. 101.23 (b), (c), (g), and

(h), provided:

(a) That person complies with all provisions of Sec. 101.25; and

(b) The operation is not conducted within 5 miles of an airport runway or

other landing area unless the information required in Sec. 101.25 is also

provided to the manager of that airport.

[Amdt. 101-6, 59 FR 50393, Oct. 3, 1994]

*****************************************************************************

59 FR 50390, No. 190, Oct. 3, 1994

SUMMARY: This action amends the operational guidelines of the Federal

Aviation Regulations (FAR) Part 101 for model rockets that: use not more than

125 grams (4.4 ounces) of propellant; are made of paper, wood, or breakable

plastic; contain no substantial metal parts; and weigh not more than 1,500

grams (53 ounces). This amendment is necessary to provide for the operation

of the technologically advanced, larger category, model rockets and to ensure

that their operation is in concert with the maximum level of safety

protection for aircraft, flight crews, and the flying public. The FAA

believes that this amendment will foster important aeronautical education and

research activities, while retaining appropriate operational safety

precautions.

EFFECTIVE DATE: November 2, 1994.

*****************************************************************************

Sec. 101.23   Operating limitations.

No person may operate an unmanned rocket--

(a) In a manner that creates a collision hazard with other aircraft;

(b) In controlled airspace;

(c) Within five miles of the boundary of any airport;

(d) At any altitude where clouds or obscuring phenomena of more than five-

tenths coverage prevails;

(e) At any altitude where the horizontal visibility is less than five

miles;

(f) Into any cloud;

(g) Within 1,500 feet of any person or property that is not associated with

the operations; or

(h) Between sunset and sunrise.

(Sec. 6(c), Department of Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 1580, 28 FR 6722, June 29, 1963, as amended by Amdt. 101-4, 39 FR

22252, June 21, 1974]

Sec. 101.25   Notice requirements.

No person may operate an unmanned rocket unless that person gives the

following information to the FAA ATC facility nearest to the place of

intended operation no less than 24 hours prior to and no more than 48 hours

prior to beginning the operation:

(a) The names and addresses of the operators; except when there are

multiple participants at a single event, the name and address of the person

so designated as the event launch coordinator, whose duties include

coordination of the required launch data estimates and coordinating the

launch event;

(b) The estimated number of rockets to be operated;

(c) The estimated size and the estimated weight of each rocket; and

(d) The estimated highest altitude or flight level to which each rocket

will be operated.

(e) The location of the operation.

(f) The date, time, and duration of the operation.

(g) Any other pertinent information requested by the ATC facility.

[Doc. No. 1580, 28 FR 6722, June 29, 1963, as amended by Amdt. 101-6, 59 FR

50393, Oct. 3, 1994]

*****************************************************************************

59 FR 50390, No. 190, Oct. 3, 1994

SUMMARY: This action amends the operational guidelines of the Federal

Aviation Regulations (FAR) Part 101 for model rockets that: use not more than

125 grams (4.4 ounces) of propellant; are made of paper, wood, or breakable

plastic; contain no substantial metal parts; and weigh not more than 1,500

grams (53 ounces). This amendment is necessary to provide for the operation

of the technologically advanced, larger category, model rockets and to ensure

that their operation is in concert with the maximum level of safety

protection for aircraft, flight crews, and the flying public. The FAA

believes that this amendment will foster important aeronautical education and

research activities, while retaining appropriate operational safety

precautions.

EFFECTIVE DATE: November 2, 1994.

*****************************************************************************

Subpart D--Unmanned Free Balloons

Source: Docket No. 1457, Amdt. 101-1, 29 FR 47, Jan. 3, 1964, unless

otherwise noted.

Sec. 101.31   Applicability.

This subpart applies to the operation of unmanned free balloons. However, a

person operating an unmanned free balloon within a restricted area must

comply only with Sec. 101.33 (d) and (e) and with any additional limitations

that are imposed by the using or controlling agency, as appropriate.

Sec. 101.33   Operating limitations.

No person may operate an unmanned free balloon--

(a) Unless otherwise authorized by ATC, below 2,000 feet above the surface

within the lateral boundaries of the surface areas of Class B, Class C, Class

D, or Class E airspace designated for an airport;

(b) At any altitude where there are clouds or obscuring phenomena of more

than five-tenths coverage;

(c) At any altitude below 60,000 feet standard pressure altitude where the

horizontal visibility is less than five miles;

(d) During the first 1,000 feet of ascent, over a congested area of a city,

town, or settlement or an open-air assembly of persons not associated with

the operation; or

(e) In such a manner that impact of the balloon, or part thereof including

its payload, with the surface creates a hazard to persons or property not

associated with the operation.

[Dkt. 1457, Amdt. 101-1, 29 FR 47, Jan. 3, 1964, as amended by Amdt. 101-5,

56 FR 65662, Dec. 17, 1991]

EFFECTIVE DATE NOTE: Amdt. 101-5, 56 FR 65662, Dec. 17, 1991, revised

paragraph (a) of Sec. 101.33 effective September 16, 1993. For the

convenience of the user, the superseded text in effect until Sept. 16, 1993,

is set forth as follows:

Sec. 101.33  Operating limitations.

* * * * *

(a) Unless otherwise authorized by ATC, in a control zone below 2,000 feet

above the surface, or in an airport traffic area;

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Sec. 101.35   Equipment and marking requirements.

(a) No person may operate an unmanned free balloon unless--

(1) It is equipped with at least two payload cut-down systems or devices

that operate independently of each other;

(2) At least two methods, systems, devices, or combinations thereof, that

function independently of each other, are employed for terminating the flight

of the balloon envelope; and

(3) The balloon envelope is equipped with a radar reflective device(s) or

material that will present an echo to surface radar operating in the 200 MHz

to 2700 MHz frequency range.

The operator shall activate the appropriate devices required by paragraphs

(a) (1) and (2) of this section when weather conditions are less than those

prescribed for operation under this subpart, or if a malfunction or any other

reason makes the further operation hazardous to other air traffic or to

persons and property on the surface.

(b) No person may operate an unmanned free balloon below 60,000 feet

standard pressure altitude between sunset and sunrise (as corrected to the

altitude of operation) unless the balloon and its attachments and payload,

whether or not they become separated during the operation, are equipped with

lights that are visible for at least 5 miles and have a flash frequency of at

least 40, and not more than 100, cycles per minute.

(c) No person may operate an unmanned free balloon that is equipped with a

trailing antenna that requires an impact force of more than 50 pounds to

break it at any point, unless the antenna has colored pennants or streamers

that are attached at not more than 50 foot intervals and that are visible for

at least one mile.

(d) No person may operate between sunrise and sunset an unmanned free

balloon that is equipped with a suspension device (other than a highly

conspicuously colored open parachute) more than 50 feet along, unless the

suspension device is colored in alternate bands of high conspicuity colors or

has colored pennants or streamers attached which are visible for at least one

mile.

(Sec. 6(c), Department of Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 1457, Amdt. 101-1, 29 FR 47, Jan. 3, 1964, as amended by Amdt. 101-

2, 32 FR 5254, Mar. 29, 1967; Amdt. 101-4, 39 FR 22252, June 21, 1974]

Sec. 101.37   Notice requirements.

(a) Prelaunch notice: Except as provided in paragraph (b) of this section,

no person may operate an unmanned free balloon unless, within 6 to 24 hours

before beginning the operation, he gives the following information to the FAA

ATC facility that is nearest to the place of intended operation:

(1) The balloon identification.

(2) The estimated date and time of launching, amended as necessary to

remain within plus or minus 30 minutes.

(3) The location of the launching site.

(4) The cruising altitude.

(5) The forecast trajectory and estimated time to cruising altitude or

60,000 feet standard pressure altitude, whichever is lower.

(6) The length and diameter of the balloon, length of the suspension

device, weight of the payload, and length of the trailing antenna.

(7) The duration of flight.

(8) The forecast time and location of impact with the surface of the earth.

(b) For solar or cosmic disturbance investigations involving a critical

time element, the information in paragraph (a) of this section shall be given

within 30 minutes to 24 hours before beginning the operation.

(c) Cancellation notice: If the operation is canceled, the person who

intended to conduct the operation shall immediately notify the nearest FAA

ATC facility.

(d) Launch notice: Each person operating an unmanned free balloon shall

notify the nearest FAA or military ATC facility of the launch time

immediately after the balloon is launched.

Sec. 101.39   Balloon position reports.

(a) Each person operating an unmanned free balloon shall:

(1) Unless ATC requires otherwise, monitor the course of the balloon and

record its position at least every two hours; and

(2) Forward any balloon position reports requested by ATC.

(b) One hour before beginning descent, each person operating an unmanned

free balloon shall forward to the nearest FAA ATC facility the following

information regarding the balloon:

(1) The current geographical position.

(2) The altitude.

(3) The forecast time of penetration of 60,000 feet standard pressure

altitude (if applicable).

(4) The forecast trajectory for the balance of the flight.

(5) The forecast time and location of impact with the surface of the earth.

(c) If a balloon position report is not recorded for any two-hour period of

flight, the person operating an unmanned free balloon shall immediately

notify the nearest FAA ATC facility. The notice shall include the last

recorded position and any revision of the forecast trajectory. The nearest

FAA ATC facility shall be notified immediately when tracking of the balloon

is re-established.

(d) Each person operating an unmanned free balloon shall notify the nearest

FAA ATC facility when the operation is ended.

Report is Completed

-------------------------------------------------------------------------------

Please turn off your printer or software capture command

-------------------------------------------------------------------------------

Press <Enter> to continue...............

----------------[You Cited: 14 CFR Part 101 as of Dec. 6, 1994]----------------

PART 101--MOORED BALLOONS, KITES, UNMANNED ROCKETS AND UNMANNED FREE BALLOONS

Subpart A--General

Sec.

101.1  Applicability.

101.3  Waivers.

101.5  Operations in prohibited or restricted areas.

101.7  Hazardous operations.

Subpart B--Moored Balloons and Kites

101.11  Applicability.

101.13  Operating limitations.

101.15  Notice requirements.

101.17  Lighting and marking requirements.

101.19  Rapid deflation device.

Subpart C--Unmanned Rockets

101.21  Applicability.

101.22  Special provisions for large model rockets.

101.23  Operating limitations.

101.25  Notice requirements.

Subpart D--Unmanned Free Balloons

101.31  Applicability.

101.33  Operating limitations.

101.35  Equipment and marking requirements.

101.37  Notice requirements.

101.39  Balloon position reports.

Authority: 49 U.S.C. app. 1348, 1354, 1372, 1421, 1442, 1443, 1472, 1510,

and 1522.

Subpart A--General

Sec. 101.1   Applicability.

(a) This part prescribes rules governing the operation in the United

States, of the following:

(1) Except as provided for in Sec. 101.7, any balloon that is moored to the

surface of the earth or an object thereon and that has a diameter of more

than 6 feet or a gas capacity of more than 115 cubic feet.

(2) Except as provided for in Sec. 101.7, any kite that weighs more than 5

pounds and is intended to be flown at the end of a rope or cable.

(3) Any unmanned rocket except:

(i) Aerial firework displays; and,

(ii) Model rockets:

(a) Using not more than four ounces of propellant;

(b) Using a slow-burning propellant;

(c) Made of paper, wood, or breakable plastic, containing no substantial

metal parts and weighing not more than 16 ounces, including the propellant;

and

(d) Operated in a manner that does not create a hazard to persons,

property, or other aircraft.

(4) Except as provided for in Sec. 101.7, any unmanned free balloon that--

(i) Carries a payload package that weighs more than four pounds and has a

weight/size ratio of more than three ounces per square inch on any surface of

the package, determined by dividing the total weight in ounces of the payload

package by the area in square inches of its smallest surface;

(ii) Carries a payload package that weighs more than six pounds;

(iii) Carries a payload, of two or more packages, that weighs more than 12

pounds; or

(iv) Uses a rope or other device for suspension of the payload that

requires an impact force of more than 50 pounds to separate the suspended

payload from the balloon.

(b) For the purposes of this part, a "gyroglider" attached to a vehicle on

the surface of the earth is considered to be a kite.

[Doc. No. 1580, 28 FR 6721, June 29, 1963, as amended by Amdt. 101-1, 29 FR

46, Jan. 3, 1964; Amdt. 101-3, 35 FR 8213, May 26, 1970]

Sec. 101.3   Waivers.

No person may conduct operations that require a deviation from this part

except under a certificate of waiver issued by the Administrator.

[Doc. No. 1580, 28 FR 6721, June 29, 1963]

Sec. 101.5   Operations in prohibited or restricted areas.

No person may operate a moored balloon, kite, unmanned rocket, or unmanned

free balloon in a prohibited or restricted area unless he has permission from

the using or controlling agency, as appropriate.

[Amdt. 101-1, 29 FR 46, Jan. 3, 1964]

Sec. 101.7   Hazardous operations.

(a) No person may operate any moored balloon, kite, unmanned rocket, or

unmanned free balloon in a manner that creates a hazard to other persons, or

their property.

(b) No person operating any moored balloon, kite, unmanned rocket, or

unmanned free balloon may allow an object to be dropped therefrom, if such

action creates a hazard to other persons or their property.

(Sec. 6(c), Department of Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 12800, Amdt. 101-4, 39 FR 22252, June 21, 1974]

Subpart B--Moored Balloons and Kites

Source: Docket No. 1580, 28 FR 6722 June 29, 1963, unless otherwise noted.

Sec. 101.11   Applicability.

This subpart applies to the operation of moored balloons and kites.

However, a person operating a moored balloon or kite within a restricted area

must comply only with Sec. 101.19 and with additional limitations imposed by

the using or controlling agency, as appropriate.

Sec. 101.13   Operating limitations.

(a) Except as provided in paragraph (b) of this section, no person may

operate a moored balloon or kite--

(1) Less than 500 feet from the base of any cloud;

(2) More than 500 feet above the surface of the earth;

(3) From an area where the ground visibility is less than three miles; or

(4) Within five miles of the boundary of any airport.

(b) Paragraph (a) of this section does not apply to the operation of a

balloon or kite below the top of any structure and within 250 feet of it, if

that shielded operation does not obscure any lighting on the structure.

Sec. 101.15   Notice requirements.

No person may operate an unshielded moored balloon or kite more than 150

feet above the surface of the earth unless, at least 24 hours before

beginning the operation, he gives the following information to the FAA ATC

facility that is nearest to the place of intended operation:

(a) The names and addresses of the owners and operators.

(b) The size of the balloon or the size and weight of the kite.

(c) The location of the operation.

(d) The height above the surface of the earth at which the balloon or kite

is to be operated.

(e) The date, time, and duration of the operation.

Sec. 101.17   Lighting and marking requirements.

(a) No person may operate a moored balloon or kite, between sunset and

sunrise unless the balloon or kite, and its mooring lines, are lighted so as

to give a visual warning equal to that required for obstructions to air

navigation in the FAA publication "Obstruction Marking and Lighting".

(b) No person may operate a moored balloon or kite between sunrise and

sunset unless its mooring lines have colored pennants or streamers attached

at not more than 50 foot intervals beginning at 150 feet above the surface of

the earth and visible for at least one mile.

(Sec. 6(c), Department of Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 1580, 28 FR 6722, June 29, 1963, as amended by Amdt. 101-4, 39 FR

22252, June 21, 1974]

Sec. 101.19   Rapid deflation device.

No person may operate a moored balloon unless it has a device that will

automatically and rapidly deflate the balloon if it escapes from its

moorings. If the device does not function properly, the operator shall

immediately notify the nearest ATC facility of the location and time of the

escape and the estimated flight path of the balloon.

Subpart C--Unmanned Rockets

Source: Docket No. 1580, 28 FR 6722, June 29, 1963, unless otherwise noted.

Sec. 101.21   Applicability.

This subpart applies to the operation of unmanned rockets. However, a

person operating an unmanned rocket within a restricted area must comply only

with Sec. 101.23(g) and with additional limitations imposed by the using or

controlling agency, as appropriate.

Sec. 101.22   Special provisions for large model rockets.

Persons operating model rockets that use not more than 125 grams of

propellant; that are made of paper, wood, or breakable plastic; that contain

no substantial metal parts, and that weigh not more than 1,500 grams,

including the propellant, need not comply with Sec. 101.23 (b), (c), (g), and

(h), provided:

(a) That person complies with all provisions of Sec. 101.25; and

(b) The operation is not conducted within 5 miles of an airport runway or

other landing area unless the information required in Sec. 101.25 is also

provided to the manager of that airport.

[Amdt. 101-6, 59 FR 50393, Oct. 3, 1994]

*****************************************************************************

59 FR 50390, No. 190, Oct. 3, 1994

SUMMARY: This action amends the operational guidelines of the Federal

Aviation Regulations (FAR) Part 101 for model rockets that: use not more than

125 grams (4.4 ounces) of propellant; are made of paper, wood, or breakable

plastic; contain no substantial metal parts; and weigh not more than 1,500

grams (53 ounces). This amendment is necessary to provide for the operation

of the technologically advanced, larger category, model rockets and to ensure

that their operation is in concert with the maximum level of safety

protection for aircraft, flight crews, and the flying public. The FAA

believes that this amendment will foster important aeronautical education and

research activities, while retaining appropriate operational safety

precautions.

EFFECTIVE DATE: November 2, 1994.

*****************************************************************************

Sec. 101.23   Operating limitations.

No person may operate an unmanned rocket--

(a) In a manner that creates a collision hazard with other aircraft;

(b) In controlled airspace;

(c) Within five miles of the boundary of any airport;

(d) At any altitude where clouds or obscuring phenomena of more than five-

tenths coverage prevails;

(e) At any altitude where the horizontal visibility is less than five

miles;

(f) Into any cloud;

(g) Within 1,500 feet of any person or property that is not associated with

the operations; or

(h) Between sunset and sunrise.

(Sec. 6(c), Department of Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 1580, 28 FR 6722, June 29, 1963, as amended by Amdt. 101-4, 39 FR

22252, June 21, 1974]

Sec. 101.25   Notice requirements.

No person may operate an unmanned rocket unless that person gives the

following information to the FAA ATC facility nearest to the place of

intended operation no less than 24 hours prior to and no more than 48 hours

prior to beginning the operation:

(a) The names and addresses of the operators; except when there are

multiple participants at a single event, the name and address of the person

so designated as the event launch coordinator, whose duties include

coordination of the required launch data estimates and coordinating the

launch event;

(b) The estimated number of rockets to be operated;

(c) The estimated size and the estimated weight of each rocket; and

(d) The estimated highest altitude or flight level to which each rocket

will be operated.

(e) The location of the operation.

(f) The date, time, and duration of the operation.

(g) Any other pertinent information requested by the ATC facility.

[Doc. No. 1580, 28 FR 6722, June 29, 1963, as amended by Amdt. 101-6, 59 FR

50393, Oct. 3, 1994]

*****************************************************************************

59 FR 50390, No. 190, Oct. 3, 1994

SUMMARY: This action amends the operational guidelines of the Federal

Aviation Regulations (FAR) Part 101 for model rockets that: use not more than

125 grams (4.4 ounces) of propellant; are made of paper, wood, or breakable

plastic; contain no substantial metal parts; and weigh not more than 1,500

grams (53 ounces). This amendment is necessary to provide for the operation

of the technologically advanced, larger category, model rockets and to ensure

that their operation is in concert with the maximum level of safety

protection for aircraft, flight crews, and the flying public. The FAA

believes that this amendment will foster important aeronautical education and

research activities, while retaining appropriate operational safety

precautions.

EFFECTIVE DATE: November 2, 1994.

*****************************************************************************

Subpart D--Unmanned Free Balloons

Source: Docket No. 1457, Amdt. 101-1, 29 FR 47, Jan. 3, 1964, unless

otherwise noted.

Sec. 101.31   Applicability.

This subpart applies to the operation of unmanned free balloons. However, a

person operating an unmanned free balloon within a restricted area must

comply only with Sec. 101.33 (d) and (e) and with any additional limitations

that are imposed by the using or controlling agency, as appropriate.

Sec. 101.33   Operating limitations.

No person may operate an unmanned free balloon--

(a) Unless otherwise authorized by ATC, below 2,000 feet above the surface

within the lateral boundaries of the surface areas of Class B, Class C, Class

D, or Class E airspace designated for an airport;

(b) At any altitude where there are clouds or obscuring phenomena of more

than five-tenths coverage;

(c) At any altitude below 60,000 feet standard pressure altitude where the

horizontal visibility is less than five miles;

(d) During the first 1,000 feet of ascent, over a congested area of a city,

town, or settlement or an open-air assembly of persons not associated with

the operation; or

(e) In such a manner that impact of the balloon, or part thereof including

its payload, with the surface creates a hazard to persons or property not

associated with the operation.

[Dkt. 1457, Amdt. 101-1, 29 FR 47, Jan. 3, 1964, as amended by Amdt. 101-5,

56 FR 65662, Dec. 17, 1991]

EFFECTIVE DATE NOTE: Amdt. 101-5, 56 FR 65662, Dec. 17, 1991, revised

paragraph (a) of Sec. 101.33 effective September 16, 1993. For the

convenience of the user, the superseded text in effect until Sept. 16, 1993,

is set forth as follows:

Sec. 101.33  Operating limitations.

* * * * *

(a) Unless otherwise authorized by ATC, in a control zone below 2,000 feet

above the surface, or in an airport traffic area;

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Sec. 101.35   Equipment and marking requirements.

(a) No person may operate an unmanned free balloon unless--

(1) It is equipped with at least two payload cut-down systems or devices

that operate independently of each other;

(2) At least two methods, systems, devices, or combinations thereof, that

function independently of each other, are employed for terminating the flight

of the balloon envelope; and

(3) The balloon envelope is equipped with a radar reflective device(s) or

material that will present an echo to surface radar operating in the 200 MHz

to 2700 MHz frequency range.

The operator shall activate the appropriate devices required by paragraphs

(a) (1) and (2) of this section when weather conditions are less than those

prescribed for operation under this subpart, or if a malfunction or any other

reason makes the further operation hazardous to other air traffic or to

persons and property on the surface.

(b) No person may operate an unmanned free balloon below 60,000 feet

standard pressure altitude between sunset and sunrise (as corrected to the

altitude of operation) unless the balloon and its attachments and payload,

whether or not they become separated during the operation, are equipped with

lights that are visible for at least 5 miles and have a flash frequency of at

least 40, and not more than 100, cycles per minute.

(c) No person may operate an unmanned free balloon that is equipped with a

trailing antenna that requires an impact force of more than 50 pounds to

break it at any point, unless the antenna has colored pennants or streamers

that are attached at not more than 50 foot intervals and that are visible for

at least one mile.

(d) No person may operate between sunrise and sunset an unmanned free

balloon that is equipped with a suspension device (other than a highly

conspicuously colored open parachute) more than 50 feet along, unless the

suspension device is colored in alternate bands of high conspicuity colors or

has colored pennants or streamers attached which are visible for at least one

mile.

(Sec. 6(c), Department of Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 1457, Amdt. 101-1, 29 FR 47, Jan. 3, 1964, as amended by Amdt. 101-

2, 32 FR 5254, Mar. 29, 1967; Amdt. 101-4, 39 FR 22252, June 21, 1974]

Sec. 101.37   Notice requirements.

(a) Prelaunch notice: Except as provided in paragraph (b) of this section,

no person may operate an unmanned free balloon unless, within 6 to 24 hours

before beginning the operation, he gives the following information to the FAA

ATC facility that is nearest to the place of intended operation:

(1) The balloon identification.

(2) The estimated date and time of launching, amended as necessary to

remain within plus or minus 30 minutes.

(3) The location of the launching site.

(4) The cruising altitude.

(5) The forecast trajectory and estimated time to cruising altitude or

60,000 feet standard pressure altitude, whichever is lower.

(6) The length and diameter of the balloon, length of the suspension

device, weight of the payload, and length of the trailing antenna.

(7) The duration of flight.

(8) The forecast time and location of impact with the surface of the earth.

(b) For solar or cosmic disturbance investigations involving a critical

time element, the information in paragraph (a) of this section shall be given

within 30 minutes to 24 hours before beginning the operation.

(c) Cancellation notice: If the operation is canceled, the person who

intended to conduct the operation shall immediately notify the nearest FAA

ATC facility.

(d) Launch notice: Each person operating an unmanned free balloon shall

notify the nearest FAA or military ATC facility of the launch time

immediately after the balloon is launched.

Sec. 101.39   Balloon position reports.

(a) Each person operating an unmanned free balloon shall:

(1) Unless ATC requires otherwise, monitor the course of the balloon and

record its position at least every two hours; and

(2) Forward any balloon position reports requested by ATC.

(b) One hour before beginning descent, each person operating an unmanned

free balloon shall forward to the nearest FAA ATC facility the following

information regarding the balloon:

(1) The current geographical position.

(2) The altitude.

(3) The forecast time of penetration of 60,000 feet standard pressure

altitude (if applicable).

(4) The forecast trajectory for the balance of the flight.

(5) The forecast time and location of impact with the surface of the earth.

(c) If a balloon position report is not recorded for any two-hour period of

flight, the person operating an unmanned free balloon shall immediately

notify the nearest FAA ATC facility. The notice shall include the last

recorded position and any revision of the forecast trajectory. The nearest

FAA ATC facility shall be notified immediately when tracking of the balloon

is re-established.

(d) Each person operating an unmanned free balloon shall notify the nearest

FAA ATC facility when the operation is ended.