PART 170--ESTABLISHMENT AND DISCONTINUANCE CRITERIA FOR AIR TRAFFIC

CONTROL SERVICES AND NAVIGATIONAL FACILITIES

Subpart A--General

Sec.

170.1  Scope.

170.3  Definitions.

Subpart B--Airport Traffic Control Tower

170.11  Scope.

170.13  Airport Traffic Control Tower (ATCT) Establishment Criteria.

170.15  ATCT Discontinuance Criteria.

Subpart C--LORAN-C

170.21  Scope.

170.23  LORAN-C establishment criteria.

170.25  LORAN-C discontinuance criteria.

Authority: 49 U.S.C. app. 1343, 1346, 1348, 1354(a), 1355, 1401, 1421, 1422

through 1430, 1472(c), 1502, and 1522; 49 U.S.C. 106(g).

Source: Docket No. 26425, 56 FR 341, Jan. 3, 1991, unless otherwise noted.

Subpart A--General

Sec. 170.1  Scope.

This subpart sets forth establishment and discontinuance criteria for

navigation aids operated and maintained by the United States.

Sec. 170.3  Definitions.

For purposes of this subpart--

Air navigation facility (NAVAID) means any facility used, available for

use, or designated for use in the aid of air navigation. Included are landing

areas; lights; signaling, radio direction-finding, or radio or other

electronic communication; and any other structure or mechanism having a

similar purpose of guiding or controlling flight or the landing or takeoff of

aircraft.

Air traffic clearance means an authorization by air traffic control for an

aircraft to proceed under specified traffic conditions within controlled

airspace for the purpose of preventing collision between known aircraft.

Air traffic control (ATC) means a service that promotes the safe, orderly,

and expeditious flow of air traffic, including airport, approach, departure,

and en route air traffic control.

Air traffic controller means a person authorized to provide air traffic

service, specifically en route and terminal control personnel.

Aircraft operations means the airborne movement of aircraft in controlled

or noncontrolled airport terminal areas, and counts at en route fixes or

other points where counts can be made. There are two types of operations:

local and itinerant.

(1) Local operations mean operations performed by aircraft which:

(i) Operate in the local traffic pattern or within sight of the airport;

(ii) Are known to be departing for, or arriving from flight in local

practice areas located within a 20-mile radius of the airport; or

(iii) Execute simulated instrument approaches or low passes at the airport.

(2) Itinerant operations mean all aircraft operations other than local

operations.

Airport traffic control tower means a terminal facility, which through the

use of air/ground communications, visual signaling, and other devices,

provides ATC services to airborne aircraft operating in the vicinity of an

airport and to aircraft operating on the airport area.

Alternate airport means an airport, specified on a flight plan, to which a

flight may proceed when a landing at the point of first intended landing

becomes inadvisable.

Approach means the flightpath established by the FAA to be used by aircraft

landing on a runway.

Approach control facility means a terminal air traffic control facility

providing approach control service.

Arrival means any aircraft arriving at an airport.

Benefit-cost ratio means the quotient of the discounted life cycle benefits

of an air traffic control service or navigation aid facility (i.e., ATCT)

divided by the discounted life cycle costs.

Ceiling means the vertical distance between the ground or water and the

lowest layer of clouds or obscuring phenomena that is reported as "broken,"

"overcast," or "obstruction."

Control Tower--See Airport Traffic Control Tower.

Criteria means the standards used by the FAA for the determination of

establishment or discontinuance of a service or facility at an airport.

Departure means any aircraft taking off from an airport.

Discontinuance means the withdrawal of a service and/or facility from an

airport.

Establishment means the provision of a service or facility at a candidate

airport.

Instrument approach means a series of predetermined maneuvers for the

orderly transfer of an aircraft under instrument flight conditions from the

beginning of the initial approach to a landing, or to a point from which a

landing may be made visually. It is prescribed and approved for a specific

airport by competent authority.

Instrument flight rules (IFR) means rules governing the procedures for

conducting flight under instrument meteorological conditions (IMC) instrument

flight.

Instrument landing system (ILS) means an instrument landing system whereby

the pilot guides his approach to a runway solely by reference to instruments

in the cockpit. In some instances, the signals received from the ground can

be fed into the automatic pilot for automatically controlled approaches.

Instrument meteorological conditions (IMC) means weather conditions below

the minimums prescribed for flight under Visual Flight Rules (VFR).

Instrument operation means an aircraft operation in accordance with an IFT

flight plan or an operation where IFR separation between aircraft is provided

by a terminal control facility or air route traffic control center (ARTCC).

Life cycle benefits means the value of services provided to aviation users

over the life span of a facility or service.

Life cycle costs means the value of research and development costs,

investment costs, operation costs, maintenance costs, and termination costs

over the life span of a facility or service.

LORAN-C means an electronic navigational system by which hyperbolic lines

of position are determined by measuring differences in time of reception of

synchronized pulse signals from two fixed transmitters.

Maintenance costs means the costs incurred in servicing and maintaining a

facility after establishment.

Mean sea level (MSL) means the base commonly used in measuring altitudes.

Microwave landing system (MLS) means a landing system which enables

equipped aircraft to make curved and closely spaced approaches to properly

instrumented airports.

Noncommercial traffic means all aircraft operations that are conducted free

of compensation.

Nonprecision approach procedure means an FAA standard for approaching an

IFR runway where no electronic glide slope is available.

Nonscheduled commercial service means the carriage by aircraft in air

commerce of persons or property for compensation or hire that are not

operated in regularly scheduled service such as charter flights.

Present value (PV) means the value of a stream of future benefits or costs

that are discounted to the present.

PVB or BPV means the discounted value of life cycle benefits.

PVC or CPV means the discounted value of life cycle benefits.

PVCM or CMPV means the discounted value of operations and maintenance costs

less termination costs over a facility's remaining life cycle.

Runway means a defined rectangular area on a land airport prepared for the

landing and takeoff of aircraft along its length.

Runway visual range means an instrumentally derived value based on standard

calibrations that represent the horizontal distance a pilot will see down the

runway from the approach end.

Scheduled commercial service means the carriage by aircraft in air commerce

under Parts 121, 127, and 135 of persons or property for compensation or hire

based on published flight schedules.

Separation means the spacing of aircraft in flight and while landing and

taking off to achieve their safe and orderly movement.

Takeoff clearance means authorization by an airport traffic control tower

for an aircraft to take off.

Tower cab means an ATC facility located at an airport. Controllers at these

facilities direct ground traffic, takeoffs, and landings.

Traffic advisories means advisories issued to alert pilots to other known

or observed air traffic which may be in such proximity to the position or

intended route of flight of their aircraft to warrant attention.

Traffic pattern means the flow of aircraft operating on and in the vicinity

of an airport during specified wind conditions as established by appropriate

authority.

VFR traffic means aircraft operated solely in accordance with Visual Flight

Rules.

Visual flight rules (VFR) means rules that govern the procedures for

conducting flight under visual conditions. The term "VFR" is also used in the

United States to indicate weather conditions that are equal to or greater

than minimum VFR requirements. In addition, "VFR" is used by pilots and

controllers to indicate the type of flight plan.

Visual meteorological conditions (VMC) means meteorological conditions

expressed in terms of visibility, distance from clouds, and ceiling equal to

or better than specified minima.

Subpart B--Airport Traffic Control Towers

Sec. 170.11   Scope.

This subpart sets forth establishment and discontinuance criteria for

Airport Traffic Control Towers.

Sec. 170.13   Airport Traffic Control Tower (ATCT) Establishment Criteria.

(a) The following criteria along with general facility establishment

standards must be met before an airport can qualify for an ATCT:

(1) The airport, whether publicly or privately owned, must be open to and

available for use by the public as defined in the Airport and Airway

Improvement Act of 1982;

(2) The airport must be recognized by and contained within the National

Plan of Integrated Airport Systems;

(3) The airport owners/authorities must have entered into appropriate

assurances and covenants to guarantee that the airport will continue in

operation for a long enough period to permit the amortization of the ATCT

investment;

(4) The FAA must be furnished appropriate land without cost for

construction of the ATCT; and

(5) The airport must meet the benefit-cost ratio criteria specified herein

utilizing three consecutive FAA annual counts and projections of future

traffic during the expected life of the tower facility. (An FAA annual count

is a fiscal year or a calendar year activity summary. Where actual traffic

counts are unavailable or not recorded, adequately documented FAA estimates

of the scheduled and nonscheduled activity may be used.)

(b) An airport meets the establishment criteria when it satisfies

paragraphs (a)(1) through (a)(5) of this section and its benefit-cost ratio

equals or exceeds one. As defined in Sec. 170.3 of this part, the benefit-

cost ratio is the ratio of the present value of the ATCT life cycle benefits

(BPV) to the present value of ATCT life cycle costs (CPV).

BPV/CPV"1.0

(c) The satisfaction of all the criteria listed in this section does not

guarantee that the airport will receive an ATCT.

Sec. 170.15  ATCT Discontinuance Criteria.

An ATCT will be subject to discontinuance when the continued operation and

maintenance costs less termination costs (CMPV) of the ATCT exceed the

present value of its remaining life-cycle benefits (BPV):

BPV/CMPV<1.0

Subpart C--LORAN-C

Source: Amdt. 170-1, 58 FR 42817, Aug. 11, 1993, unless otherwise noted.

Sec. 170.21   Scope.

This subpart sets forth establishment and discontinuance criteria for

LORAN-C.

Sec. 170.23   LORAN-C establishment criteria.

(a) The criteria in paragraphs (a)(1) through (a)(6) of this section, along

with general facility and navigational aid establishment requirements, must

be met before a runway can be eligible for LORAN-C approach.

(1) A runway must have landing surfaces judged adequate by the FAA to

accommodate aircraft expected to use the approach and meet all FAA-required

airport design criteria for nonprecision runways.

(2) A runway must be found acceptable for instrument flight rules

operations as a result of an airport airspace analysis conducted in

accordance with the current FAA regulations and provisions.

(3) The LORAN-C signal must be of sufficient quality and accuracy to pass

an FAA flight inspection.

(4) It must be possible to remove, mark, or light all approach obstacles in

accordance with FAA marking and lighting provisions.

(5) Appropriate weather information must be available.

(6) Air-to-ground communications must be available at the initial approach

fix minimum altitude and at the missed approach altitude.

(b) A runway meets the establishment criteria for a LORAN-C approach when

it satisfies paragraphs (a)(1) through (a)(6) of this section and the

estimated value of benefits associated with the LORAN-C approach equals or

exceeds the estimated costs (benefit-cost ratio equals or exceeds one). As

defined in Sec. 170.3 of this part, the benefit-cost ratio is the ratio of

the present value of the LORAN-C life-cycle benefits (PVB) to the present

value of LORAN-C life-cycle costs (PVC):

PVB/PVC >/= 1.0

(c) The criteria do not cover all situations that may arise and are not

used as a sole determinant in denying or granting the establishment of non-

precision LORAN-C approach for which there is a demonstrated operational or

air traffic control requirement.

Sec. 170.25   LORAN-C discontinuance criteria.

A LORAN-C nonprecision approach may be subject to discontinuance when the

present value of the continued maintenance costs (PVCM) of the LORAN-C

approach exceed the present value of its remaining life-cycle benefits (PVB):

PVB/PVCM < 1.0