PART 35--AIRWORTHINESS STANDARDS: PROPELLERS

Subpart A--General

Sec.

35.1  Applicability.

35.3  Instruction manual for installing and operating the propeller.

35.4  Instructions for continued airworthiness.

35.5  Propeller operating limitations.

Subpart B--Design and Construction

35.11  Applicability.

35.13  General.

35.15  Design features.

35.17  Materials.

35.19  Durability.

35.21  Reversible propellers.

35.23  Pitch control and indication.

Subpart C--Tests and Inspections

35.31  Applicability.

35.33  General.

35.35  Blade retention test.

35.37  Fatigue limit tests.

35.39  Endurance test.

35.41  Functional test.

35.42  Blade pitch control system component test.

35.43  Special tests.

35.45  Teardown inspection.

35.47  Propeller adjustments and parts replacements.

Appendix A--Instructions for Continued Airworthiness

Authority: Secs. 313, 601, 603, 72 Stat. 752, 775; 49 U.S.C. 1354, 1421,

1423.

Source: Docket No. 2095, 29 FR 7458, June 10, 1964, unless otherwise noted.

Subpart A--General

Sec. 35.1  Applicability.

(a) This part prescribes airworthiness standards for the issue of type

certificates and changes to those certificates, for propellers.

(b) Each person who applies under Part 21 for such a certificate or change

must show compliance with the applicable requirements of this part.

[Amdt. 35-3, 41 FR 55475, Dec. 20, 1976]

Sec. 35.3  Instruction manual for installing and operating the propeller.

Each applicant must prepare and make available an approved manual or

manuals containing instructions for installing and operating the propeller.

[Amdt. 35-5, 45 FR 60181, Sept. 11, 1980]

Sec. 35.4  Instructions for continued airworthiness.

The applicant must prepare Instructions for Continued Airworthiness in

accordance with Appendix A to this part that are acceptable to the

Administrator. The instructions may be incomplete at type certification if a

program exists to ensure their completion prior to delivery of the first

aircraft with the propeller installed, or upon issuance of a standard

certificate of airworthiness for an aircraft with the propeller installed,

whichever occurs later.

[Amdt. 35-5, 45 FR 60181, Sept. 11, 1980]

Sec. 35.5  Propeller operating limitations.

Propeller operating limitations are established by the Administrator, are

included in the propeller type certificate data sheet specified in Sec. 21.41

of this chapter, and include limitations based on the operating conditions

demonstrated during the tests required by this part and any other information

found necessary for the safe operation of the propeller.

[Amdt. 35-5, 45 FR 60182, Sept. 11, 1980]

Subpart B--Design and Construction

Sec. 35.11  Applicability.

This subpart prescribes the design and construction requirements for

propellers.

Sec. 35.13  General.

Each applicant must show that the propeller concerned meets the design and

construction requirements of this subpart.

Sec. 35.15  Design features.

The propeller may not have design features that experience has shown to be

hazardous or unreliable. The suitability of each questionable design detail

or part must be established by tests.

Sec. 35.17  Materials.

The suitability and durability of materials used in the propeller must--

(a) Be established on the basis of experience or tests; and

(b) Conform to approved specifications (such as industry or military

specifications, or Technical Standard Orders) that ensure their having the

strength and other properties assumed in the design data.

(Secs. 313(a), 601, and 603, 72 Stat. 752, 775, 49 U.S.C. 1354(a), 1421, and

1423; sec. 6(c), 49 U.S.C. 1655(c))

[Amdt. 35-4, 42 FR 15047, Mar. 17, 1977]

Sec. 35.19  Durability.

Each part of the propeller must be designed and constructed to minimize the

development of any unsafe condition of the propeller between overhaul

periods.

Sec. 35.21  Reversible propellers.

A reversible propeller must be adaptable for use with a reversing system in

an airplane so that no single failure or malfunction in that system during

normal or emergency operation will result in unwanted travel of the propeller

blades to a position substantially below the normal flight low-pitch stop.

Failure of structural elements need not be considered if the occurrence of

such a failure is expected to be extremely remote. For the purposes of this

section the term "reversing system" means that part of the complete reversing

system that is in the propeller itself and those other parts that are

supplied by the applicant for installation in the aircraft.

Sec. 35.23  Pitch control and indication.

(a) No loss of normal propeller pitch control may cause hazardous

overspeeding of the propeller under intended operating conditions.

(b) Each pitch control system that is within the propeller, or supplied

with the propeller, and that uses engine oil for feathering, must incorporate

means to override or bypass the normally operative hydraulic system

components so as to allow feathering if those components fail or malfunction.

(c) Each propeller approved for installation on a turbopropeller engine

must incorporate a provision for an indicator to indicate when the propeller

blade angle is below the flight low pitch position. The provision must

directly sense the blade position and be arranged to cause an indicator to

indicate that the blade angle is below the flight low pitch position before

the blade moves more than 8 deg. below the flight low pitch stop.

[Amdt. 35-2, 32 FR 3737, Mar. 4, 1967, as amended by Amdt. 35--5, 45 FR

60182, Sept. 11, 1980]

Subpart C--Tests and Inspections

Sec. 35.31  Applicability.

This subpart prescribes the tests and inspections for propellers and their

essential accessories.

Sec. 35.33  General.

(a) Each applicant must show that the propeller concerned and its essential

accessories complete the tests and inspections of this subpart without

evidence of failure or malfunction.

(b) Each applicant must furnish testing facilities, including equipment,

and competent personnel, to conduct the required tests.

Sec. 35.35  Blade retention test.

The hub and blade retention arrangement of propellers with detachable

blades must be subjected to a centrifugal load of twice the maximum

centrifugal force to which the propeller would be subjected during operations

within the limitations established for the propeller. This may be done by

either a whirl test or a static pull test.

(Secs. 313(a), 601, and 603, 72 Stat. 752, 775, 49 U.S.C. 1354(a), 1421, and

1423; sec. 6(c), 49 U.S.C. 1655(c))

[Amdt. 35-4, 42 FR 15047, Mar. 17, 1977]

Sec. 35.37  Fatigue limit tests.

A fatigue evalution must be made and the fatigue limits determined for each

metallic hub and blade, and each primary load carrying metal component of

nonmetallic blades. The fatigue evaluation must include consideration of all

reasonably foreseeable vibration load patterns. The fatigue limits must

account for the permissible service deteriortion (such as nicks, grooves,

galling, bearing wear, and variations in material properties).

[Amdt. 35-5, 45 FR 60182, Sept. 11, 1980]

Sec. 35.39  Endurance test.

(a) Fixed-pitch wood propellers. Fixed-pitch wood propellers must be

subjected to one of the following tests:

(1) A 10-hour endurance block test on an engine with a propeller of the

greatest pitch and diameter for which certification is sought at the rated

rotational speed.

(2) A 50-hour flight test in level flight or in climb. At least five hours

of this flight test must be with the propeller operated at the rated

rotational speed, and the remainder of the 50 hours must be with the

propeller operated at not less than 90 percent of the rated rotational speed.

This test must be conducted on a propeller of the greatest diameter for which

certification is requested.

(3) A 50-hour endurance block test on an engine at the power and propeller

rotational speed for which certification is sought. This test must be

conducted on a propeller of the greatest diameter for which certification is

requested.

(b) Fixed-pitch metal propellers and ground adjustable-pitch propellers.

Each fixed-pitch metal propeller or ground adjustable-pitch propeller must be

subjected to the test prescribed in either paragraph (a)(2) or (a)(3) of this

section.

(c) Variable-pitch propellers. Compliance with this paragraph must be shown

for a propeller of the greatest diameter for which certification is

requested. Each variable-pitch propeller (a propeller the pitch setting of

which can be changed by the flight crew or by automatic means while the

propeller is rotating) must be subjected to one of the following tests:

(1) A 100-hour test on a representative engine with the same or higher

power and rotational speed and the same or more severe vibration

characteristics as the engine with which the propeller is to be used. Each

test must be made at the maximum continuous rotational speed and power rating

of the propeller. If a takeoff rating greater than the maximum continuous

rating is to be established, and additional 10-hour block test must be made

at the maximum power and rotational speed for the takeoff rating.

(2) Operation of the propeller throughout the engine endurance tests

prescribed in Part 33 of this subchapter.

[Doc. No. 2095, 29 FR 7458, June 10, 1964, as amended by Amdt. 35-2, 32 FR

3737, Mar. 4, 1967; Amdt. 35-3, 41 FR 55475, Dec. 20, 1976]

Sec. 35.41  Functional test.

(a) Each variable-pitch propeller must be subjected to the applicable

functional tests of this section. The same propeller used in the endurance

test must be used in the functional tests and must be driven by an engine on

a test stand or on an aircraft.

(b) Manually controllable propellers.  500 complete cycles of control must

be made throughout the pitch and rotational speed ranges.

(c) Automatically controllable propellers.  1,500 complete cycles of

control must be made throughout the pitch and rotational speed ranges.

(d) Feathering propellers. 50 cycles of feathering operation must be made.

(e) Reversible-pitch propellers. Two hundred complete cycles of control

must be made from lowest normal pitch to maximum reverse pitch, and, while in

maximum reverse pitch, during each cycle, the propeller must be run for 30

seconds at the maximum power and rotational speed selected by the applicant

for maximum reverse pitch.

[Doc. No. 2095, 29 FR 7458, June 10, 1964, as amended by Amdt. 35-3, 41 FR

55475, Dec. 20, 1976]

Sec. 35.42  Blade pitch control system component test.

The following durability requirements apply to propeller blade pitch

control system components:

(a) Except as provided in paragraph (b) of this section, each propeller

blade pitch control system component, including governors, pitch change

assemblies, pitch locks, mechanical stops, and feathering system components,

must be subjected in tests to cyclic loadings that simulate the frequency and

amplitude those to which the component would be subjected during 1,000 hours

of propeller operation.

(b) Compliance with paragraph (a) of this section may be shown by a

rational analysis based on the results of tests on similar components.

[Amdt. 35-5, 45 FR 60182, Sept. 11, 1980]

Sec. 35.43  Special tests.

The Administrator may require any additional tests he finds necessary to

substantiate the use of any unconventional features of design, material, or

construction.

Sec. 35.45  Teardown inspection.

(a) After completion of the tests prescribed in this subpart, the propeller

must be completely disassembled and a detailed inspection must be made of the

propeller parts for cracks, wear, distortion, and any other unusual

conditions.

(b) After the inspection the applicant must make any changes to the design

or any additional tests that the Administrator finds necessary to establish

the airworthiness of the propeller.

[Doc. No. 3095, 29 FR 7458, June 10, 1964, as amended by Amdt. 35-3, 41 FR

55475, Dec. 20, 1976]

Sec. 35.47  Propeller adjustments and parts replacements.

The applicant may service and make minor repairs to the propeller during

the tests. If major repairs or replacement of parts are found necessary

during the tests or in the teardown inspection, the parts in question must be

subjected to any additional tests the Administrator finds necessary.

Appendix A--Instructions for Continued Airworthiness

A35.1  GENERAL

(a) This appendix specifies requirements for the preparation of

Instructions for Continued Airworthiness as required by Sec. 35.4.

(b) The Instructions for Continued Airworthiness for each propeller must

include the Instructions for Continued Airworthiness for all propeller parts.

If Instructions for Continued Airworthiness are not supplied by the propeller

part manufacturer for a propeller part, the Instructions for Continued

Airworthiness for the propeller must include the information essential to the

continued airworthiness of the propeller.

(c) The applicant must submit to the FAA a program to show how changes to

the Instructions for Continued Airworthiness made by the applicant or by the

manufacturers of propeller parts will be distributed.

A35.2  FORMAT

(a) The Instructions for Continued Airworthiness must be in the form of a

manual or manuals as appropriate for the quantity of data to be provided.

(b) The format of the manual or manuals must provide for a practical

arrangement.

A35.3  CONTENT

The contents of the manual must be prepared in the English language. The

Instructions for Continued Airworthiness must contain the following sections

and information:

(a) Propeller Maintenance Section. (1) Introduction information that

includes an explanation of the propeller's features and data to the extent

necessary for maintenance or preventive maintenance.

(2) A detailed description of the propeller and its systems and

installations.

(3) Basic control and operation information describing how the propeller

components and systems are controlled and how they operate, including any

special procedures that apply.

(4) Instructions for uncrating, acceptance checking, lifting, and

installing the propeller.

(5) Instructions for propeller operational checks.

(6) Scheduling information for each part of the propeller that provides the

recommended periods at which it should be cleaned, adjusted, and tested, the

applicable wear tolerances, and the degree of work recommended at these

periods. However, the applicant may refer to an accessory, instrument, or

equipment manufacturer as the source of this information if it shows that the

item has an exceptionally high degree of complexity requiring specialized

maintenance techniques, test equipment, or expertise. The recommended

overhaul periods and necessary cross-references to the Airworthiness

Limitations section of the manual must also be included. In addition, the

applicant must include an inspection program that includes the frequency and

extent of the inspections necessary to provide for the continued

airworthiness of the propeller.

(7) Troubleshooting information describing probable malfunctions, how to

recognize those malfunctions, and the remedial action for those malfunctions.

(8) Information describing the order and method of removing and replacing

propeller parts with any necessary precautions to be taken.

(9) A list of the special tools needed for maintenance other than for

overhauls.

(b) Propeller Overhaul Section. (1) Disassembly information including the

order and method of disassembly for overhaul.

(2) Cleaning and inspection instructions that cover the materials and

apparatus to be used and methods and precautions to be taken during overhaul.

Methods of overhaul inspection must also be included.

(3) Details of all fits and clearances relevant to overhaul.

(4) Details of repair methods for worn or otherwise substandard parts and

components along with information necessary to determine when replacement is

necessary.

(5) The order and method of assembly at overhaul.

(6) Instructions for testing after overhaul.

(7) Instructions for storage preparation including any storage limits.

(8) A list of tools needed for overhaul.

A35.4  AIRWORTHINESS LIMITATIONS SECTION

The Instructions for Continued Airworthiness must contain a section titled

Airworthiness Limitations that is segregated and clearly distinguishable from

the rest of the document. This section must set forth each mandatory

replacement time, inspection interval, and related procedure required for

type certification. This section must contain a legible statement in a

prominent location that reads: "The Airworthiness Limitations section is FAA

approved and specifies maintenance required under Secs. 43.16 and 91.403 of

the Federal Aviation Regulations unless an alternative program has been FAA

approved."

[Amdt. 35-5, 45 FR 60182, Sept. 11, 1980, as amended by Amdt. 35-6, 54 FR

34330, Aug. 18, 1989]

Effective Date Note: At 54 FR 34330, Aug. 18, 1989, Sec. A35.4 in Appendix

A, Part 35 was amended by changing the cross reference "Sec. 91.163" to "Sec.

91.403", effective August 18, 1990.