PART 830--NOTIFICATION AND REPORTING OF AIRCRAFT ACCIDENTS OR INCIDENTS AND

OVERDUE AIRCRAFT, AND PRESERVATION OF AIRCRAFT WRECKAGE, MAIL, CARGO, AND

RECORDS

Subpart A--General

Sec.

830.1  Applicability.

830.2  Definitions.

Subpart B--Initial Notification of Aircraft Accidents, Incidents, and Overdue

Aircraft

830.5  Immediate notification.

830.6  Information to be given in notification.

Subpart C--Preservation of Aircraft Wreckage, Mail, Cargo, and Records

830.10  Preservation of aircraft wreckage, mail, cargo, and records.

Subpart D--Reporting of Aircraft Accidents, Incidents, and Overdue Aircraft

830.15  Reports and statement to be filed.

Subpart E--Reporting of Public Aircraft Accidents and Incidents

830.20  Reports to be filed.

Authority: 49 U.S.C. 1441 and 1901 et seq.

Source: 53 FR 36982, Sept. 23, 1988, unless otherwise noted.

Subpart A--General

Sec. 830.1  Applicability.

This part contains rules pertaining to:

(a) Notification and reporting aircraft accidents and incidents and certain

other occurrences in the operation of aircraft when they involve civil

aircraft of the United States wherever they occur, or foreign civil aircraft

when such events occur in the United States, its territories or possessions.

(b) Reporting aircraft accidents and listed incidents in the operation of

aircraft when they involve certain public aircraft.

(c) Preservation of aircraft wreckage, mail, cargo, and records involving

all civil aircraft in the United States, its territories or possessions.

Sec. 830.2  Definitions.

As used in this part the following words or phrases are defined as follows:

"Aircraft accident" means an occurrence associated with the operation of an

aircraft which takes place between the time any person boards the aircraft

with the intention of flight and all such persons have disembarked, and in

which any person suffers death or serious injury, or in which the aircraft

receives substantial damage.

"Civil aircraft" means any aircraft other than a public aircraft.

"Fatal injury" means any injury which results in death within 30 days of

the accident.

"Incident" means an occurrence other than an accident, associated with the

operation of an aircraft, which affects or could affect the safety of

operations.

"Operator" means any person who causes or authorizes the operation of an

aircraft, such as the owner, lessee, or bailee of an aircraft.

"Public aircraft" means an aircraft used exclusively in the service of any

government or of any political subdivision thereof, including the government

of any State, Territory, or possession of the United States, or the District

of Columbia, but not including any government-owned aircraft engaged in

carrying persons or property for commercial purposes. For purposes of this

section "used exclusively in the service of" means, for other than the

Federal Government, an aircraft which is owned and operated by a governmental

entity for other than commercial purposes or which is exclusively leased by

such governmental entity for not less than 90 continuous days.

"Serious injury" means any injury which: (1) Requires hospitalization for

more than 48 hours, commencing within 7 days from the date of the injury was

received; (2) results in a fracture of any bone (except simple fractures of

fingers, toes, or nose); (3) causes severe hemorrhages, nerve, muscle, or

tendon damage; (4) involves any internal organ; or (5) involves second- or

third-degree burns, or any burns affecting more than 5 percent of the body

surface.

"Substantial damage" means damage or failure which adversely affects the

structural strength, performance, or flight characteristics of the aircraft,

and which would normally require major repair or replacement of the affected

component. Engine failure or damage limited to an engine if only one engine

fails or is damaged, bent fairings or cowling, dented skin, small punctured

holes in the skin or fabric, ground damage to rotor or propeller blades, and

damage to landing gear, wheels, tires, flaps, engine accessories, brakes, or

wingtips are not considered "substantial damage" for the purpose of this

part.

Subpart B--Initial Notification of Aircraft Accidents, Incidents, and Overdue

Aircraft

Sec. 830.5  Immediate notification.

The operator of an aircraft shall immediately, and by the most expeditious

means available, notify the nearest National Transportation Safety Board

(Board), field office /1/ when:

NOTE /1/ The National Transportation Safety Board field offices are listed

under U.S. Government in the telephone directories in the following cities:

Anchorage, Alaska; Atlanta, Ga.; Chicago, Ill.; Denver, Colo.; Fort Worth,

Tex.; Kansas City, Mo.; Los Angeles, Calif.; Miami, Fla.; New York, N.Y.;

Seattle, Wash.

(a) An aircraft accident or any of the following listed incidents occur:

(1) Flight control system malfunction or failure;

(2) Inability of any required flight crewmember to perform normal flight

duties as a result of injury or illness;

(3) Failure of structural components of a turbine engine excluding

compressor and turbine blades and vanes;

(4) In-flight fire; or

(5) Aircraft collide in flight.

(6) Damage to property, other than the aircraft, estimated to exceed

$25,000 for repair (including materials and labor) or fair market value in

the event of total loss, whichever is less.

(7) For large multiengine aircraft (more than 12,500 pounds maximum

certificated takeoff weight):

(i) In-flight failure of electrical systems which requires the sustained

use of an emergency bus powered by a back-up source such as a battery,

auxiliary power unit, or air-driven generator to retain flight control or

essential instruments;

(ii) In-flight failure of hydraulic systems that results in sustained

reliance on the sole remaining hydraulic or mechanical system for movement of

flight control surfaces;

(iii) Sustained loss of the power or thrust produced by two or more

engines; and

(iv) An evacuation of an aircraft in which an emergency egress system is

utilized.

(b) An aircraft is overdue and is believed to have been involved in an

accident.

Sec. 830.6  Information to be given in notification.

The notification required in Sec. 830.5 shall contain the following

information, if available:

(a) Type, nationality, and registration marks of the aircraft;

(b) Name of owner, and operator of the aircraft;

(c) Name of the pilot-in-command;

(d) Date and time of the accident;

(e) Last point of departure and point of intended landing of the aircraft;

(f) Position of the aircraft with reference to some easily defined

geographical point;

(g) Number of persons aboard, number killed, and number seriously injured;

(h) Nature of the accident, the weather and the extent of damage to the

aircraft, so far as is known; and

(i) A description of any explosives, radioactive materials, or other

dangerous articles carried.

Subpart C--Preservation of Aircraft Wreckage, Mail, Cargo, and Records

Sec. 830.10  Preservation of aircraft wreckage, mail, cargo, and records.

(a) The operator of an aircraft involved in an accident or incident for

which notification must be given is responsible for preserving to the extent

possible any aircraft wreckage, cargo, and mail aboard the aircraft, and all

records, including all recording mediums of flight, maintenance, and voice

recorders, pertaining to the operation and maintenance of the aircraft and to

the airmen until the Board takes custody thereof or a release is granted

pursuant to Sec. 831.12(b) of this chapter.

(b) Prior to the time the Board or its authorized representative takes

custody of aircraft wreckage, mail, or cargo, such wreckage, mail, or cargo

may not be disturbed or moved except to the extent necessary:

(1) To remove persons injured or trapped;

(2) To protect the wreckage from further damage; or

(3) To protect the public from injury.

(c) Where it is necessary to move aircraft wreckage, mail or cargo,

sketches, descriptive notes, and photographs shall be made, if possible, of

the original positions and condition of the wreckage and any significant

impact marks.

(d) The operator of an aircraft involved in an accident or incident shall

retain all records, reports, internal documents, and memoranda dealing with

the accident or incident, until authorized by the Board to the contrary.

Subpart D--Reporting of Aircraft Accidents, Incidents, and Overdue Aircraft

Sec. 830.15  Reports and statements to be filed.

(a) Reports. The operator of an aircraft shall file a report on Board Form

6120.1 (OMB No. 3147-005) or Board Form 7120.2 (OMB No. 3147-0001) /1/ within

10 days after an accident, or after 7 days if an overdue aircraft is still

missing. A report on an incident for which notification is required by Sec.

830.5(a) shall be filed only as requested by an authorized representative of

the Board.

NOTE /1/ Forms are obtainable from the Board field offices (see footnote 1

of Sec. 830.5), the National Transportation Safety Board, Washington, DC

20594, and the Federal Aviation Administration, Flight Standards District

Office.

(b) Crewmember statement. Each crewmember, if physically able at the time

the report is submitted, shall attach a statement setting forth the facts,

conditions, and circumstances relating to the accident or incident as they

appear to him. If the crewmember is incapacitated, he shall submit the

statement as soon as he is physically able.

(c) Where to file the reports. The operator of an aircraft shall file any

report with the field office of the Board nearest the accident or incident.

Subpart E--Reporting of Public Aircraft Accidents and Incidents

Sec. 830.20  Reports to be filed.

The operator of a public aircraft other than an aircraft of the Armed

Forces or Intelligence Agencies shall file a report on NTSB Form 6120.1 (OMB

No. 3147-001) /1/ within 10 days after an accident or incident listed in Sec.

830.5(a). The operator shall file the report with the field office of the

Board nearest the accident or incident./2/

NOTE /1/ To obtain this form, see footnote 1 of Sec. 830.15.

NOTE /2/ The locations of the Board's field offices are set forth in footnote

1 of Sec. 830.5