PART 31--AIRWORTHINESS STANDARDS: MANNED FREE BALLOONS

Subpart A--General

Sec.

31.1  Applicability.

Subpart B--Flight Requirements

31.12  Proof of compliance.

31.14  Weight limits.

31.16  Empty weight.

31.17  Performance: Climb.

31.19  Performance: Uncontrolled descent.

31.20  Controllability.

Subpart C--Strength Requirements

31.21  Loads.

31.23  Flight load factor.

31.25  Factor of safety.

31.27  Strength.

Subpart D--Design Construction

31.31  General.

31.33  Materials.

31.35  Fabrication methods.

31.37  Fastenings.

31.39  Protection.

31.41  Inspection provisions.

31.43  Fitting factor.

31.45  Fuel cells.

31.46  Pressurized fuel systems.

31.47  Heaters.

31.49  Control systems.

31.51  Ballast.

31.53  Drag rope.

31.55  Deflation means.

31.57  Rip cords.

31.59  Trapeze, basket, or other means provided for occupants.

31.61  Static discharge.

31.63  Safety belts.

31.65  Position lights.

Subpart E--Equipment

31.71  Function and installation.

Subpart F--Operating Limitations and Information

31.81  General.

31.82  Instructions for Continued Airworthiness.

31.83  Conspicuity.

31.85  Required basic equipment.

Appendix A to Part 31--Instructions for Continued Airworthiness

Authority: Secs. 313, 601, 603, 72 Stat. 752, 775; 49 U.S.C. 1354, 1421,

1423.

Source: Docket No. 1437, 29 FR 8258, July 1, 1964, as amended by Amdt. 31-

1, 29 FR 14563, Oct. 24, 1964, unless otherwise noted.

Subpart A--General

Sec. 31.1  Applicability.

(a) This part prescribes airworthiness standards for the issue of type

certificates and changes to those certificates, for manned free balloons.

(b) Each person who applies under Part 21 for such a certificate or change

must show compliance with the applicable requirements of this part.

(c) For purposes of this part--

(1) A captive gas balloon is a balloon that derives its lift from a captive

lighter-than-air gas;

(2) A hot air balloon is a balloon that derives its lift from heated air;

(3) The envelope is the enclosure in which the lifting means is contained;

(4) The basket is the container, suspended beneath the envelope, for the

balloon occupants;

(5) The trapeze is a harness or is a seat consisting of a horizontal bar or

platform suspended beneath the envelope for the balloon occupants; and

(6) The design maximum weight is the maximum total weight of the balloon,

less the lifting gas or air.

[Doc. No. 1437, 29 FR 8258, July 1, 1964, as amended by Amdt. 31-3, 41 FR

55474, Dec. 20, 1976]

Subpart B--Flight Requirements

Sec. 31.12  Proof of compliance.

(a) Each requirement of this subpart must be met at each weight within the

range of loading conditions for which certification is requested. This must

be shown by--

(1) Tests upon a balloon of the type for which certification is requested

or by calculations based on, and equal in accuracy to, the results of

testing; and

(2) Systematic investigation of each weight if compliance cannot be

reasonably inferred from the weights investigated.

(b) Except as provided in Sec. 31.17(b), allowable weight tolerances during

flight testing are +5 percent and -10 percent.

[Amdt. 31-4, 45 FR 60179, Sept. 11, 1980]

Sec. 31.14  Weight limits.

(a) The range of weights over which the balloon may be safely operated must

be established.

(b) Maximum weight. The maximum weight is the highest weight at which

compliance with each applicable requirement of this part is shown. The

maximum weight must be established so that it is not more than--

(i) The highest weight selected by the applicant;

(ii) The design maximum weight which is the highest weight at which

compliance with each applicable structural loading condition of this part is

shown; or

(iii) The highest weight at which compliance with each applicable flight

requirement of this part is shown.

(c) The information established under paragraphs (a) and (b) of this

section must be made available to the pilot in accordance with Sec. 31.81.

[Amdt. 31-3, 41 FR 55474, Dec. 20, 1976]

Sec. 31.16   Empty weight.

The empty weight must be determined by weighing the balloon with installed

equipment but without lifting gas or heater fuel.

[Amdt. 31-4, 45 FR 60179, Sept. 11, 1980]

Sec. 31.17   Performance: Climb.

(a) Each balloon must be capable of climbing at least 300 feet in the first

minute after takeoff with a steady rate of climb. Compliance with the

requirements of this section must be shown at each altitude and ambient

temperature for which approval is sought.

(b) Compliance with the requirements of paragraph (a) of this section must

be shown at the maximum weight with a weight tolerance of +5 percent.

[Amdt. 31-4, 45 FR 60179, Sept. 11, 1980]

Sec. 31.19  Performance: Uncontrolled descent.

(a) The following must be determined for the most critical uncontrolled

descent that can result from any single failure of the heater assembly, fuel

cell system, gas value system, or maneuvering vent system, or from any single

tear in the balloon envelope between tear stoppers:

(1) The maximum vertical velocity attained.

(2) The altitude loss from the point of failure to the point at which

maximum vertical velocity is attained.

(3) The altitude required to achieve level flight after corrective action

is inititated, with the balloon descending at the maximum vertical velocity

determined in paragraph (a)(1) of this section.

(b) Procedures must be established for landing at the maximum vertical

velocity determined in paragraph (a)(1) of this section and for arresting

that descent rate in accordance with paragraph (a)(3) of this section.

[Amdt. 31-4, 45 FR 60179, Sept. 11, 1980]

Sec. 31.20  Controllability.

The applicant must show that the balloon is safely controllable and

maneuverable during takeoff, ascent, descent, and landing without requiring

exceptional piloting skill.

[Amdt. 31-3, 41 FR 55474, Dec. 20, 1976]

Subpart C--Strength Requirements

Sec. 31.21  Loads.

Strength requirements are specified in terms of limit loads, that are the

maximum load to be expected in service, and ultimate loads, that are limit

loads multiplied by prescribed factors of safety. Unless otherwise specified,

all prescribed loads are limit loads.

Sec. 31.23  Flight load factor.

In determining limit load, the limit flight load factor must be at least

1.4.

Sec. 31.25  Factor of safety.

(a) Except as specified in paragraphs (b) and (c) of this section, the

factor of safety is 1.5.

(b) A factor of safety of at least five must be used in envelope design. A

reduced factor of safety of at least two may be used if it is shown that the

selected factor will preclude failure due to creep or instantaneous rupture

from lack of rip stoppers. The selected factor must be applied to the more

critical of the maximum operating pressure or envelope stress.

(c) A factor of safety of at least five must be used in the design of all

fibrous or non-metallic parts of the rigging and related attachments of the

envelope to basket, trapeze, or other means provided for carrying occupants.

The primary attachments of the envelope to the basket, trapeze, or other

means provided for carrying occupants must be designed so that failure is

extremely remote or so that any single failure will not jeopardize safety of

flight.

(d) In applying factors of safety, the effect of temperature, and other

operating characteristics, or both, that may affect strength of the balloon

must be accounted for.

(e) For design purposes, an occupant weight of at least 170 pounds must be

assumed.

[Doc. No. 1437, 29 FR 8258, July 1, 1964, as amended by Amdt. 31-2, 30 FR

3377, Mar. 13, 1965]

Sec. 31.27  Strength.

(a) The structure must be able to support limit loads without detrimental

effect.

(b) The structure must be substantiated by test to be able to withstand the

ultimate loads for at least three seconds without failure. For the envelope,

a test of a representative part is acceptable, if the part tested is large

enough to include critical seams, joints, and load attachment points and

members.

(c) An ultimate free-fall drop test must be made of the basket, trapeze, or

other place provided for occupants. The test must be made at design maximum

weight on a horizontal surface, with the basket, trapeze, or other means

provided for carrying occupants, striking the surface at angles of 0, 15, and

30 degrees. The weight may be distributed to simulate actual conditions.

There must be no distortion or failure that is likely to cause serious injury

to the occupants. A drop test height of 36 inches, or a drop test height that

produces, upon impact, a velocity equal to the maximum vertical velocity

determined in accordance with Sec. 31.19, whichever is higher, must be used.

[Doc. No. 1437, 29 FR 8258, July 1, 1964, as amended by Amdt. 31-4, 45 FR

60179, Sept. 11, 1980]

Subpart D--Design Construction

Sec. 31.31  General.

The suitability of each design detail or part that bears on safety must be

established by tests or analysis.

Sec. 31.33  Materials.

(a) The suitability and durability of all materials must be established on

the basis of experience or tests. Materials must conform to approved

specifications that will ensure that they have the strength and other

properties assumed in the design data.

(b) Material strength properties must be based on enough tests of material

conforming to specifications so as to establish design values on a

statistical basis.

Sec. 31.35  Fabrication methods.

The methods of fabrication used must produce a consistently sound

structure. If a fabrication process requires close control to reach this

objective, the process must be performed in accordance with an approved

process specification.

Sec. 31.37  Fastenings.

Only approved bolts, pins, screws, and rivets may be used in the structure.

Approved locking devices or methods must be used for all these bolts, pins,

and screws, unless the installation is shown to be free from vibration. Self-

locking nuts may not be used on bolts that are subject to rotation in

service.

Sec. 31.39  Protection.

Each part of the balloon must be suitably protected against deterioration

or loss of strength in service due to weathering, corrosion, or other causes.

Sec. 31.41  Inspection provisions.

There must be a means to allow close examination of each part that require

repeated inspection and adjustment.

Sec. 31.43  Fitting factor.

(a) A fitting factor of at least 1.15 must be used in the analysis of each

fitting the strength of which is not proven by limit and ultimate load tests

in which the actual stress conditions are simulated in the fitting and

surrounding structure. This factor applies to all parts of the fitting, the

means of attachment, and the bearing on the members joined.

(b) Each part with an integral fitting must be treated as a fitting up to

the point where the section properties become typical of the member.

(c) The fitting factor need not be used if the joint design is made in

accordance with approved practices and is based on comprehensive test data.

Sec. 31.45  Fuel cells.

If fuel cells are used, the fuel cells, their attachments, and related

supporting structure must be shown by tests to be capable of withstanding,

without detrimental distortion or failure, any inertia loads to which the

installation may be subjected, including the drop tests prescribed in Sec.

31.27(c). In the tests, the fuel cells must be loaded to the weight and

pressure equivalent to the full fuel quantity condition.

[Amdt. 31-3, 41 FR 55474, Dec. 20, 1976]

Sec. 31.46  Pressurized fuel systems.

For pressurized fuel systems, each element and its connecting fittings and

lines must be tested to an ultimate pressure of at least twice the maximum

pressure to which the system will be subjected in normal operation. No part

of the system may fail or malfunction during the test. The test configuration

must be representative of the normal fuel system installation and balloon

configuration.

[Amdt. 31-3, 41 FR 55474, Dec. 20, 1976]

Sec. 31.47  Heaters.

(a) If a heater is used to provide the lifting means, the system must be

designed and installed so as not to create a fire hazard.

(b) There must be shielding to protect parts adjacent to the burner flame,

and the occupants, from heat effects.

(c) There must be controls, instruments, or other equipment essential to

the safe control and operation of the heater. They must be shown to be able

to perform their intended functions during normal and emergency operation.

(d) The heater system (including the burner unit, controls, fuel lines,

fuel cells, regulators, control valves, and other related elements) must be

substantiated by an endurance test of at least 50 hours. In making the test,

each element of the system must be installed and tested so as to simulate the

actual balloon installation. The test program must be conducted so that each

10-hour part of the test includes 7 hours at maximum heat output of the

heater and 3 hours divided into at least 10 equal increments between minimum

and maximum heat output ranges.

(e) The test must also include at least three flameouts and restarts.

(f) Each element of the system must be serviceable at the end of the test.

[Doc. No. 1437, 29 FR 8258, July 1, 1964, as amended by Amdt. 31-2, 30 FR

3377, Mar. 13, 1965]

Sec. 31.49  Control systems.

(a) Each control must operate easily, smoothly, and positively enough to

allow proper performance of its functions. Controls must be arranged and

identified to provide for convenience of operation and to prevent the

possibility of confusion and subsequent inadvertent operation.

(b) Each control system and operating device must be designed and installed

in a manner that will prevent jamming, chafing, or interference from

passengers, cargo, or loose objects. Precaution must be taken to prevent

foreign objects from jamming the controls. The elements of the control system

must have design features or must be distinctly and permanently marked to

minimize the possibility of incorrect assembly that could result in

malfunctioning of the control system.

(c) Each balloon using a captive gas as the lifting means must have an

automatic valve or appendix that is able to release gas automatically at the

rate of at least three percent of the total volume per minute when the

balloon is at its maximum operating pressure.

(d) Each hot air balloon must have a means to allow the controlled release

of hot air during flight.

(e) Each hot air balloon must have a means to indicate the maximum envelope

skin temperatures occurring during operation. The indicator must be readily

visible to the pilot and marked to indicate the limiting safe temperature of

the envelope material. If the markings are on the cover glass of the

instrument, there must be provisions to maintain the correct alignment of the

glass cover with the face of the dial.

[Doc. No. 1437, 29 FR 8258, July 1, 1964, as amended by Amdt. 31-2, 30 FR

3377, Mar. 13, 1965]

Sec. 31.51  Ballast.

Each captive gas balloon must have a means for the safe storage and

controlled release of ballast. The ballast must consist of material that, if

released during flight, is not hazardous to persons on the ground.

Sec. 31.53  Drag rope.

If a drag rope is used, the end that is released overboard must be

stiffened to preclude the probability of the rope becoming entangled with

trees, wires, or other objects on the ground.

Sec. 31.55  Deflation means.

There must be a means to allow emergency deflation of the envelope so as to

allow a safe emergency landing. If a system other than a manual system is

used, the reliability of the system used must be substantiated.

[Amdt. 31-2, 30 FR 3377, Mar. 13, 1965]

Sec. 31.57  Rip cords.

(a) If a rip cord is used for emergency deflation, it must be designed and

installed to preclude entanglement.

(b) The force required to operate the rip cord may not be less than 25, or

more than 75, pounds.

(c) The end of the rip cord to be operated by the pilot must be colored

red.

(d) The rip cord must be long enough to allow an increase of at least 10

percent in the vertical dimension of the envelope.

Sec. 31.59  Trapeze, basket, or other means provided for occupants.

(a) The trapeze, basket, or other means provided for carrying occupants may

not rotate independently of the envelope.

(b) Each projecting object on the trapeze, basket, or other means provided

for carrying occupants, that could cause injury to the occupants, must be

padded.

Sec. 31.61  Static discharge.

Unless shown not to be necessary for safety, there must be appropriate

bonding means in the design of each balloon using flammable gas as a lifting

means to ensure that the effects of static discharges will not create a

hazard.

[Amdt. 31-2, 30 FR 3377, Mar. 13, 1965]

Sec. 31.63  Safety belts.

(a) There must be a safety belt, harness, or other restraining means for

each occupant, unless the Administrator finds it unnecessary. If installed,

the belt, harness, or other restraining means and its supporting structure

must meet the strength requirements of Subpart C of this part.

(b) This section does not apply to balloons that incorporate a basket or

gondola.

[Amdt. 31-2, 30 FR 3377, Mar. 13, 1965, as amended by Amdt. 31-3, 41 FR

55474, Dec. 20, 1976]

Sec. 31.65  Position lights.

(a) If position lights are installed, there must be one steady aviation

white position light and one flashing aviation red (or flashing aviation

white) position light with an effective flash frequency of at least 40, but

not more than 100, cycles per minute.

(b) Each light must provide 360 deg. horizontal coverage at the intensities

prescribed in this paragraph. The following light intensities must be

determined with the light source operating at a steady state and with all

light covers and color filters in place and at the manufacturer's rated

mimimum voltage. For the flashing aviation red light, the measured values

must be adjusted to correspond to a red filter temperature of at least 130

deg. F:

(1) The intensities in the horizontal plane passing through the light unit

must equal or exceed the following values:

Minimum

intensity

Position light      (candles)

Steady white                  20

Flashing red or white         40

(2) The intensities in vertical planes must equal or exceed the following

values. An intensity of one unit corresponds to the applicable horizontal

plane intensity specified in paragraph (b)(1) of this section.

Angles

above and

below the

horizontal

in any

vertical    Minimum

plane     intensity

(degrees)    (units)

0                1.00

0 to 5           0.90

5 to 10          0.80

10 to 15         0.70

15 to 20         0.50

20 to 30         0.30

30 to 40         0.10

40 to 60         0.05

(c) The steady white light must be located not more than 20 feet below the

basket, trapeze, or other means for carrying occupants. The flashing red or

white light must be located not less than 7, nor more than 10, feet below the

steady white light.

(d) There must be a means to retract and store the lights.

(e) Each position light color must have the applicable International

Commission on Illumination chromaticity coordinates as follows:

(1) Aviation red--

"y" is not greater than 0.335; and "z" is not greater than 0.002.

(2) Aviation white--

"x" is not less than 0.300 and not greater than 0.540;

"y" is not less than "x"-0.040" or "yo-0.010", whichever is the smaller; and

"y" is not greater than "x+0.020" nor "0.636-0.0400 x";

Where "yo" is the "y" coordinate of the Planckian radiator for the value of

"x" considered.

[Doc. No. 1437, 29 FR 8258, July 1, 1964, as amended by Amdt. 31-1, 29 FR

14563, Oct. 24, 1964; Amdt. 31-4, 45 FR 60179, Sept. 11, 1980]

Subpart E--Equipment

Sec. 31.71  Function and installation.

(a) Each item of installed equipment must--

(1) Be of a kind and design appropriate to its intended function;

(2) Be permanently and legibly marked or, if the item is too small to mark,

tagged as to its identification, function, or operating limitations, or any

applicable combination of those factors;

(3) Be installed according to limitations specified for that equipment; and

(4) Function properly when installed.

(b) No item of installed equipment, when performing its function, may

affect the function of any other equipment so as to create an unsafe

condition.

(c) The equipment, systems, and installations must be designed to prevent

hazards to the balloon in the event of a probable malfunction or failure.

[Amdt. 31-4, 45 FR 60180, Sept. 11, 1980]

Subpart F--Operating Limitations and Information

Sec. 31.81  General.

(a) The following information must be established:

(1) Each operating limitation, including the maximum weight determined

under Sec. 31.14.

(2) The normal and emergency procedures.

(3) Other information necessary for safe operation, including--

(i) The empty weight determined under Sec. 31.16;

(ii) The rate of climb determined under Sec. 31.17, and the procedures and

conditions used to determine performance;

(iii) The maximum vertical velocity, the altitude drop required to attain

that velocity, and altitude drop required to recover from a descent at that

velocity, determined under Sec. 31.19, and the procedures and conditions used

to determine performance; and

(iv) Pertinent information peculiar to the balloon's operating

characteristics.

(b) The information established in compliance with paragraph (a) of this

section must be furnished by means of--

(1) A Balloon Flight Manual; or

(2) A placard on the balloon that is clearly visible to the pilot.

[Amdt. 31-4, 45 FR 60180, Sept. 11, 1980]

Sec. 31.82  Instructions for Continued Airworthiness.

The applicant must prepare Instructions for Continued Airworthiness in

accordance with Appendix A to this part that are acceptable to the

Administrator. The instructions may be incomplete at type certification if a

program exists to ensure their completion prior to delivery of the first

balloon or issuance of a standard certificate of airworthiness, whichever

occurs later.

[Amdt. 31-4, 45 FR 60180, Sept. 11, 1980]

Sec. 31.83  Conspicuity.

The exterior surface of the envelope must be of a contrasting color or

colors so that it will be conspicuous during operation. However, multicolored

banners or streamers are acceptable if it can be shown that they are large

enough, and there are enough of them of contrasting color, to make the

balloon conspicuous during flight.

Sec. 31.85  Required basic equipment.

In addition to any equipment required by this subchapter for a specific

kind of operation, the following equipment is required:

(a) For all balloons:

(1) [Reserved]

(2) An altimeter.

(3) A rate of climb indicator.

(b) For hot air balloons:

(1) A fuel quantity gauge. If fuel cells are used, means must be

incorporated to indicate to the crew the quantity of fuel in each cell during

flight. The means must be calibrated in appropriate units or in percent of

fuel cell capacity.

(2) An envelope temperature indicator.

(c) For captive gas balloons, a compass.

[Amdt. 31-2, 30 FR 3377, Mar. 13, 1965, as amended by Amdt. 31-3, 41 FR

55474, Dec. 20, 1976; Amdt. 31-4, 45 FR 60180, Sept. 11, 1980]

Appendix A--Instructions for Continued Airworthiness

A31.1  GENERAL

(a) This appendix specifies requirements for the preparation of

Instructions for Continued Airworthiness as required by Sec. 31.82.

(b) The Instructions for Continued Airworthiness for each balloon must

include the Instructions for Continued Airworthiness for all balloon parts

required by this chapter and any required information relating to the

interface of those parts with the balloon. If Instructions for Continued

Airworthiness are not supplied by the part manufacturer for a balloon part,

the Instructions for Continued Airworthiness for the balloon must include the

information essential to the continued airworthiness of the balloon.

(c) The applicant must submit to the FAA a program to show how changes to

the Instructions for Continued Airworthiness made by the applicant or by the

manufacturers of balloon parts will be distributed.

A31.2  FORMAT

(a) The Instructions for Continued Airworthiness must be in the form of a

manual or manuals as appropriate for the quantity of data to be provided.

(b) The format of the manual or manuals must provide for a practical

arrangement.

A31.3  CONTENT

The contents of the manual or manuals must be prepared in the English

language. The Instructions for Continued Airworthiness must contain the

following information:

(a) Introduction information that includes an explanation of the balloon's

features and data to the extent necessary for maintenance or preventive

maintenance.

(b) A description of the balloon and its systems and installations.

(c) Basic control and operation information for the balloon and its

components and systems.

(d) Servicing information that covers details regarding servicing of

balloon components, including burner nozzles, fuel tanks, and valves during

operations.

(e) Maintenance information for each part of the balloon and its envelope,

controls, rigging, basket structure, fuel systems, instruments, and heater

assembly that provides the recommended periods at which they should be

cleaned, adjusted, tested, and lubricated, the applicable wear tolerances,

and the degree of work recommended at these periods. However, the applicant

may refer to an accessory, instrument, or equipment manufacturer as the

source of this information if the applicant shows that the item has an

exceptionally high degree of complexity requiring specialized maintenance

techniques, test equipment, or expertise. The recommended overhaul periods

and necessary cross references to the Airworthiness Limitations section of

the manual must also be included. In addition, the applicant must include an

inspection program that includes the frequency and extent of the inspections

necessary to provide for the continued airworthiness of the balloon.

(f) Troubleshooting information describing probable malfunctions, how to

recognize those malfunctions, and the remedial action for those malfunctions.

(g) Details of what, and how, to inspect after a hard landing.

(h) Instructions for storage preparation including any storage limits.

(i) Instructions for repair on the balloon envelope and its basket or

trapeze.

A31.4  AIRWORTHINESS LIMITATIONS SECTION

The Instructions for Continued Airworthiness must contain a section titled

Airworthiness Limitations that is segregated and clearly distinguishable from

the rest of the document. This section must set forth each mandatory

replacement time, structural inspection interval, and related structural

inspection procedure, including envelope structural integrity, required for

type certification. If the Instructions for Continued Airworthiness consist

of multiple documents, the section required by this paragraph must be

included in the principal manual. This section must contain a legible

statement in a prominent location that reads: "The Airworthiness Limitations

section is FAA approved and specifies maintenance required under Secs. 43.16

and 91.403 of the Federal Aviation Regulations."

[Amdt. 31-4, 45 FR 60180, Sept. 11, 1980, as amended by Amdt. 31-5, 54 FR

34330, Aug. 18, 1989]

Effective Date Note: At 54 FR 34330, Aug. 18, 1989, Sec. A31.4 in Appendix

A, Part 31 was amended by changing the cross reference "Sec. 91.163" to "Sec.

91.403", effective August 18, 1990.