PART 34--FUEL VENTING AND EXHAUST EMISSION REQUIREMENTS FOR TURBINE ENGINE

POWERED AIRPLANES

Subpart A--General Provisions

Sec.

34.1 Definitions.

34.2 Abbreviations.

34.3 General requirements.

34.4 [Reserved].

34.5 Special test procedures.

34.6 Aircraft safety.

34.7 Exemptions.

Subpart B--Engine Fuel Venting Emissions (New and In-Use Aircraft Gas Turbine

Engines)

34.10  Applicability.

34.11  Standard for fuel venting emissions.

Subpart C--Exhaust Emissions (New Aircraft Gas Turbine Engines)

34.20  Applicability.

34.21  Standards for exhaust emissions.

Subpart D--Exhaust Emissions (In-Use Aircraft Gas Turbine Engines)

34.30  Applicability.

34.31  Standards for exhaust emissions.

Subpart E--[Reserved]

Subpart F--[Reserved]

Subpart G--Test Procedures for Engine Exhaust Gaseous Emissions (Aircraft and

Aircraft Gas Turbine Engines)

34.60  Introduction.

34.61  Turbine fuel specifications.

34.62  Test procedure (propulsion engines).

34.63  [Reserved]

34.64  Sampling and analytical procedures for measuring gaseous exhaust

emissions.

34.65 to Sec. 34.70  [Reserved]

34.71  Compliance with gaseous emission standards.

Subpart H--Test Procedures for Engine Smoke Emissions (Aircraft Gas Turbine

Engines)

34.80  Introduction.

34.81  Fuel specifications.

34.82  Sampling and analytical procedures for measuring smoke exhaust

emissions.

34.83 to Sec. 34.88  [Reserved]

34.89  Compliance with smoke emission standards.

Authority: 42 U.S.C. 1857f-10; 49 U.S.C. 106(g); 49 U.S.C. App. 1348(c),

1354(a), 1421, 1423.

Source: Docket No. 25613, Amdt. 91-218, 55 FR 32861, Aug. 10, 1990, unless

otherwise noted.

Subpart A--General Provisions

Sec. 34.1  Definitions.

As used in this part, all terms not defined herein shall have the meaning

given them in the Clean Air Act, as amended (42 U.S.C. 7401 et. seq.):

Act means the Clean Air Act, as amended (42 U.S.C. 7401 et. seq.).

Administrator means the Administrator of the Federal Aviation

Administration or any person to whom he has delegated his authority in the

matter concerned.

Administrator of the EPA means the Administrator of the Environmental

Protection Agency and any other officer or employee of the Environmental

Protection Agency to whom the authority involved may be delegated.

Aircraft as used in this part means any airplane as defined in 14 CFR part

1 for which a U.S. standard airworthiness certificate or equivalent foreign

airworthiness certificate is issued.

Aircraft engine means a propulsion engine which is installed in, or which

is manufactured for installation in, an aircraft.

Aircraft gas turbine engine means a turboprop, turbofan, or turbojet

aircraft engine.

Class TP means all aircraft turboprop engines.

Class TF means all turbofan or turbojet aircraft engines except engines of

Class T3, T8, and TSS.

Class T3 means all aircraft gas turbine engines of the JT3D model family.

Class T8 means all aircraft gas turbine engines of the JT8D model family.

Class TSS means all aircraft gas turbine engines employed for propulsion of

aircraft designed to operate at supersonic flight speeds.

Commercial aircraft engine means any aircraft engine used or intended for

use by an "air carrier" (including those engaged in "intrastate air

transportation") or a "commercial operator" (including those engaged in

"intrastate air transportation") as these terms are defined in the Federal

Aviation Act and the Federal Aviation Regulations.

Commercial aircraft gas turbine engine means a turboprop, turbofan, or

turbojet commercial aircraft engine.

Date of manufacture of an engine is the date the inspection acceptance

records reflect that the engine is complete and meets the FAA approved type

design.

Emission measurement system means all of the equipment necessary to

transport the emission sample and measure the level of emissions. This

includes the sample system and the instrumentation system.

Engine model means all commercial aircraft turbine engines which are of the

same general series, displacement, and design characteristics and are

approved under the same type certificate.

Exhaust emissions means substances emitted into the atmosphere from the

exhaust discharge nozzle of an aircraft or aircraft engine.

Fuel venting emissions means raw fuel, exclusive of hydrocarbons in the

exhaust emissions, discharged from aircraft gas turbine engines during all

normal ground and flight operations.

In-use aircraft gas turbine engine means an aircraft gas turbine engine

which is in service.

New aircraft turbine engine means an aircraft gas turbine engine which has

never been in service.

Power setting means the power or thrust output of an engine in terms of

kilonewtons thrust for turbojet and turbofan engines or shaft power in terms

of kilowatts for turboprop engines.

Rated output (r0) means the maximum power/thrust available for takeoff at

standard day conditions as approved for the engine by the Federal Aviation

Administration, including reheat contribution where applicable, but excluding

any contribution due to water injection and excluding any emergency power/

thrust rating.

Rated pressure ratio (rPR) means the ratio between the combustor inlet

pressure and the engine inlet pressure achieved by an engine operation at

rated output.

Reference day conditions means the reference ambient conditions to which

the gaseous emissions (HC and smoke) are to be corrected. The reference day

conditions are as follows: Temperature=15 deg.C, specific humidity=0.00629 kg

H2O/kg of dry air, and pressure=101325 Pa.

Sample system means the system which provides for the transportation of the

gaseous emission sample from the sample probe to the inlet of the

instrumentation system.

Shaft power means only the measured shaft power output of a turboprop

engine.

Smoke means the matter in exhaust emissions which obscures the transmission

of light.

Smoke number (SN) means the dimensionless term quantifying smoke emissions.

Standard day conditions means standard ambient conditions as described in

the United States Standard Atmosphere 1976, (i.e., temperature=15 deg. C,

specific humidity=0.00 kg H20/kg dry air, and pressure=101325 Pa.)

Taxi/idle (in) means those aircraft operations involving taxi and idle

between the time of landing roll-out and final shutdown of all propulsion

engines.

Taxi/idle (out) means those aircraft operations involving taxi and idle

between the time of initial starting of the propulsion engine(s) used for the

taxi and the turn onto the duty runway.

[Dkt. No. 25613, 55 FR 32861, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

Sec. 34.2  Abbreviations.

The abbreviations used in this part have the following meanings in both

upper and lower case:

EPA  United States Environmental Protection Agency

FAA  Federal Aviation Administration, United States Department of

Transportation

HC Hydrocarbon(s)

HP Horsepower

hr Hour(s)

H20  water

kg  Kilogram(s)

kJ  Kilojoule(s)

LTO  Landing and takeoff

min  Minute(s)

Pa  Pascal(s)

rO  Rated output

rPR  Rated pressure ratio

sec  Second(s)

SP  Shaft power

SN  Smoke number

T  Temperature, degrees Kelvin

TIM  Time in mode

W  Watt(s)

deg.C  Degrees Celsius

%  Percent

Sec. 34.3  General requirements.

(a) This part provides for the approval or acceptance by the Administrator

or the Administrator of the EPA of testing and sampling methods, analytical

techniques, and related equipment not identical to those specified in this

part. Before either approves or accepts any such alternate, equivalent, or

otherwise nonidentical procedures or equipment, the Administrator or the

Administrator of the EPA shall consult with the other in determining whether

or not the action requires rulemaking under sections 231 and 232 of the Clean

Air Act, as amended, consistent with the responsibilities of the

Administrator of the EPA and the Secretary of Transportation under sections

231 and 232 of the Clean Air Act.

(b) Under section 232 of the Act, the Secretary of Transportation issues

regulations to ensure compliance with 40 CFR part 87. This authority has been

delegated to the Administrator of the FAA (49 CFR 1.47).

(c) U.S. airplanes. This Federal Aviation Regulation (FAR) applies to civil

airplanes that are powered by aircraft gas turbine engines of the classes

specified herein and that have U.S. standard airworthiness certificates.

(d) Foreign airplanes. Pursuant to the definition of "aircraft" in 40 CFR

87.1(c), this FAR applies to civil airplanes that are powered by aircraft gas

turbine engines of the classes specified herein and that have foreign

airworthiness certificates that are equivalent to U.S. standard airworthiness

certificates. This FAR applies only to those foreign civil airplanes that, if

registered in the United States, would be required by applicable Federal

Aviation Regulations to have a U.S. standard airworthiness certificate in

order to conduct the operations intended for the airplane. Pursuant to 40 CFR

87.3(c), this FAR does not apply where it would be inconsistent with an

obligation assumed by the United States to a foreign country in a treaty,

convention, or agreement.

(e) Reference in this regulation to 40 CFR part 87 refers to title 40 of

the Code of Federal Regulations, chapter I--Environmental Protection Agency,

part 87, Control of Air Pollution from Aircraft and Aircraft Engines (40 CFR

part 87).

(f) This part contains regulations to ensure compliance with certain

standards contained in 40 CFR part 87. If EPA takes any action, including the

issuance of an exemption or issuance of a revised or alternate procedure,

test method, or other regulation, the effect of which is to relax or delay

the effective date of any provision of 40 CFR part 87 that is made applicable

to an aircraft under this FAR, the Administrator of FAA will grant a general

administrative waiver of its more stringent requirements until this FAR is

amended to reflect the more relaxed requirements prescribed by EPA.

(g) Unless otherwise stated, all terminology and abbreviations in this FAR

that are defined in 40 CFR part 87 have the meaning specified in that part,

and all terms in 40 CFR part 87 that are not defined in that part but that

are used in this FAR have the meaning given them in the Clean Air Act, as

amended by Public Law 91-604.

(h) All interpretations of 40 CFR part 87 that are rendered by the EPA also

apply to this FAR.

(i) If the EPA, under 40 CFR 87.3(a), approves or accepts any testing and

sampling procedures or methods, analytical techniques, or related equipment

not identical to those specified in that part, this FAR requires an applicant

to show that such alternate, equivalent, or otherwise nonidentical procedures

have been complied with, and that such alternate equipment was used to show

compliance, unless the applicant elects to comply with those procedures,

methods, techniques, and equipment specified in 40 CFR part 87.

(j) If the EPA, under 40 CFR 87.5, prescribes special test procedures for

any aircraft or aircraft engine that is not susceptible to satisfactory

testing by the procedures in 40 CFR part 87, the applicant must show the

Administrator that those special test procedures have been complied with.

(k) Wherever 40 CFR part 87 requires agreement, acceptance, or approval by

the Administrator of the EPA, this FAR requires a showing that such agreement

or approval has been obtained.

(l) Pursuant to 42 U.S.C. 7573, no state or political subdivision thereof

may adopt or attempt to enforce any standard respecting emissions of any air

pollutant from any aircraft or engine thereof unless that standard is

identical to a standard made applicable to the aircraft by the terms of this

FAR.

(m) If EPA, by regulation or exemption, relaxes a provision of 40 CFR part

87 that is implemented in this FAR, no state or political subdivision thereof

may adopt or attempt to enforce the terms of this FAR that are superseded by

the relaxed requirement.

(n) If any provision of this FAR is rendered inapplicable to a foreign

aircraft as provided in 40 CFR 87.3(c) (international agreements), and Sec.

34.3(d) of this FAR, that provision may not be adopted or enforced against

that foreign aircraft by a state or political subdivision thereof.

(o) For exhaust emissions requirements of this FAR that apply beginning

February 1, 1974, January 1, 1976, January 1, 1978, January 1, 1984, and

August 9, 1985, continued compliance with those requirements is shown for

engines for which the type design has been shown to meet those requirements,

if the engine is maintained in accordance with applicable maintenance

requirements for 14 CFR chapter I. All methods of demonstrating compliance

and all model designations previously found acceptable to the Administrator

shall be deemed to continue to be an acceptable demonstration of compliance

with the specific standards for which they were approved.

(p) Each applicant must allow the Administrator to make, or witness, any

test necessary to determine compliance with the applicable provisions of this

FAR.

[Dkt. No. 25613, 55 FR 32861, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

Sec. 34.4  [Reserved]

Sec. 34.5  Special test procedures.

The Administrator or the Administrator of the EPA may, upon written

application by a manufacturer or operator of aircraft or aircraft engines,

approve test procedures for any aircraft or aircraft engine that is not

susceptible to satisfactory testing by the procedures set forth herein. Prior

to taking action on any such application, the Administrator or the

Administrator of the EPA shall consult with the other.

Sec. 34.6  Aircraft safety.

(a) The provisions of this part will be revised if at any time the

Administrator determines that an emission standard cannot be met within the

specified time without creating a safety hazard.

(b) Consistent with 40 CFR 87.6, if the FAA Administrator determines that

any emission control regulation in this part cannot be safely applied to an

aircraft, that provision may not be adopted or enforced against that aircraft

by any state or political subdivision thereof.

Sec. 34.7  Exemptions.

Notwithstanding part 11 of the Federal Aviation Regulations (14 CFR part

11), all petitions for rulemaking involving either the substance of an

emission standard or test procedure prescribed by the EPA that is

incorporated in this FAR, or the compliance date for such standard or

procedure, must be submitted to the EPA. Information copies of such petitions

are invited by the FAA. Petitions for rulemaking or exemption involving

provisions of this FAR that do not affect the substance or the compliance

date of an emission standard or test procedure that is prescribed by the EPA,

and petitions for exemptions under the provisions for which the EPA has

specifically granted exemption authority to the Secretary of Transportation

are subject to part 11 of the Federal Aviation Regulations (14 CFR part 11).

Petitions for rulemaking or exemptions involving these FARs must be submitted

to the FAA.

(a) Exemptions based on flights for short durations at infrequent

intervals. The emission standards of this part do not apply to engines which

power aircraft operated in the United States for short durations at

infrequent intervals. Such operations are limited to:

(1) Flights of an aircraft for the purpose of export to a foreign country,

including any flights essential to demonstrate the integrity of an aircraft

prior to a flight to a point outside the United States.

(2) Flights to a base where repairs, alterations or maintenance are to be

performed, or to a point of storage, or for the purpose of returning an

aircraft to service.

(3) Official visits by representatives of foreign governments.

(4) Other flights the Administrator determines, after consultation with the

Administrator of the EPA, to be for short durations at infrequent intervals.

A request for such a determination shall be made before the flight takes

place.

(b) Exemptions for very low production engine models. The emissions

standards of this part do not apply to engines of very low production after

the date of applicability. For the purpose of this part, "very low

production" is limited to a maximum total production for United States civil

aviation applications of no more than 200 units covered by the same type

certificate after January 1, 1984. Engines manufactured under this provision

must be reported to the FAA by serial number on or before the date of

manufacture and exemptions granted under this provision are not transferable

to any other engine.

(c) Exemptions for new engines in other categories. The emissions standards

of this part do not apply to engines for which the Administrator determines,

with the concurrence of the Administrator of the EPA, that application of any

standard under Sec. 34.21 is not justified, based upon consideration of--

(1) Adverse economic impact on the manufacturer;

(2) Adverse economic impact on the aircraft and airline industries at

large;

(3) Equity in administering the standards among all economically competing

parties;

(4) Public health and welfare effects; and

(5) Other factors which the Administrator, after consultation with the

Administrator of the EPA, may deem relevant to the case in question.

(d) Time-limited exemptions for in-use engines. The emissions standards of

this part do not apply to aircraft or aircraft engines for time periods which

the Administrator determines, with the concurrence of the Administrator of

the EPA, that any applicable standard under Sec. 34.11(a), or Sec. 34.31(a),

should not be applied based upon consideration of--

(1) Documentation demonstrating that all good faith efforts to achieve

compliance with such standard have been made;

(2) Documentation demonstrating that the inability to comply with such

standard is due to circumstances beyond the control of the owner or operator

of the aircraft; and

(3) A plan in which the owner or operator of the aircraft shows that he

will achieve compliance in the shortest time which is feasible.

(e) Applications for exemption from this part shall be submitted in

duplicate to the Administrator in accordance with the procedures established

by the Administrator in part 11.

(f) The Administrator shall publish in the Federal Register the name of the

organization to whom exemptions are granted and the period of such

exemptions.

(g) No state or political subdivision thereof may attempt to enforce a

standard respecting emissions from an aircraft or engine if such aircraft or

engine has been exempted from such standard under this part.

Subpart B--Engine Fuel Venting Emissions (New and In-Use Aircraft Gas Turbine

Engines)

Sec. 34.10  Applicability.

(a) The provisions of this subpart are applicable to all new aircraft gas

turbine engines of classes T3, T8, TSS, and TF equal to or greater than 36

kilonewtons (8090 pounds) rated output, manufactured on or after January 1,

1974, and to all in-use aircraft gas turbine engines of classes T3, T8, TSS,

and TF equal to or greater than 36 kilonewtons (8090 pounds) rated output

manufactured after February 1, 1974.

(b) The provisions of this subpart are also applicable to all new aircraft

gas turbine engines of class TF less than 36 kilonewtons (8090 pounds) rated

output and class TP manufactured on or after January 1, 1975, and to all in-

use aircraft gas turbine engines of class TF less than 36 kilonewtons (8090

pounds) rated output and class TP manufactured after January 1, 1975.

Sec. 34.11  Standard for fuel venting emissions.

(a) No fuel venting emissions shall be discharged into the atmosphere from

any new or in-use aircraft gas turbine engine subject to the subpart. This

paragraph is directed at the elimination of intentional discharge to the

atmosphere of fuel drained from fuel nozzle manifolds after engines are shut

down and does not apply to normal fuel seepage from shaft seals, joints, and

fittings.

(b) Conformity with the standard set forth in paragraph (a) of this section

shall be determined by inspection of the method designed to eliminate these

emissions.

(c) As applied to an airframe or an engine, any manufacturer or operator

may show compliance with the fuel venting and emissions requirements of this

section that were effective beginning February 1, 1974 or January 1, 1975, by

any means that prevents the intentional discharge of fuel from fuel nozzle

manifolds after the engines are shut down. Acceptable means of compliance

include one of the following:

(1) Incorporation of an FAA-approved system that recirculates the fuel back

into the fuel system.

(2) Capping or securing the pressurization and drain valve.

(3) Manually draining the fuel from a holding tank into a container.

Subpart C--Exhaust Emissions (New Aircraft Gas Turbine Engines)

Sec. 34.20  Applicability.

The provisions of this subpart are applicable to all aircraft gas turbine

engines of the classes specified beginning on the dates specified in Sec.

34.21.

Sec. 34.21  Standards for exhaust emissions.

(a) Exhaust emissions of smoke from each new aircraft gas turbine engine of

class T8 manufactured on or after February 1, 1974, shall not exceed a smoke

number (SN) of 30.

(b) Exhaust emissions of smoke from each new aircraft gas turbine engine of

class TF and of rated output of 129 kilonewtons (29,000 pounds) thrust or

greater, manufactured on or after January 1, 1976, shall not exceed

SN=83.6 (rO)**-0.274 (rO is in kilonewtons).

(c) Exhaust emission of smoke from each new aircraft gas turbine engine of

class T3 manufactured on or after January 1, 1978, shall not exceed a smoke

number (SN) of 25.

(d) Gaseous exhaust emissions from each new commercial aircraft gas turbine

engine that is manufactured on or after January 1, 1984, shall not exceed:

(1) Classes, TF, T3, T8 engines with rated output equal to or greater than

26.7 kilonewtons (6000 pounds)

Hydrocarbons: 19.6 grams/kilonewton rO.

(2) Class TSS

Hydrocarbons: 140(0.92)**rPR grams/kilonewton rO.

(e) Smoke exhaust emissions from each gas turbine engine of the classes

specified below shall not exceed:

(1) Class TF of rated output less than 26.7 kilonewtons (6000 pounds)

manufactured on or after August 9, 1985

SN=83.6(rO)**-0.274 (rO is in kilonewtons) not to exceed a maximum of SN=50.

(2) Classes T3, T8, TSS, and TF of rated output equal to or greater than

26.7 kilonewtons (6000 pounds) manufactured on or after January 1, 1984

SN=83.6(rO)**-0.274 (rO is in kilonewtons) not to exceed a maximum of SN=50.

(3) Class TP of rated output equal to or greater than 1,000 kilowatts (1340

HP) manufactured on or after January 1, 1984

SN=187(rO)**-0.168 (rO is in kilowatts).

(f) The standards set forth in paragraphs (a), (b), (c), (d), and (e) of

this section refer to a composite gaseous emission sample representing the

operating cycles set forth in the applicable sections of subpart G of this

part, and exhaust smoke emissions emitted during operations of the engine as

specified in the applicable sections of subpart H of this part, measured and

calculated in accordance with the procedures set forth in those subparts.

[Dkt. No. 25613, 55 FR 32861, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

Subpart D--Exhaust Emissions (In-use Aircraft Gas Turbine Engines)

Sec. 34.30  Applicability.

The provisions of this subpart are applicable to all in-use aircraft gas

turbine engines certificated for operation within the United States of the

classes specified, beginning on the dates specified in Sec. 34.31.

Sec. 34.31  Standards for exhaust emissions.

(a) Exhaust emissions of smoke from each in-use aircraft gas turbine engine

of Class T8, beginning February 1, 1974, shall not exceed a smoke number (SN)

of 30.

(b) Exhaust emissions of smoke from each in-use aircraft gas turbine engine

of Class TF and of rated output of 129 kilonewtons (29,000 pounds) thrust or

greater, beginning January l, 1976, shall not exceed

SN=83.6(rO)**-0.274 (rO is in kilonewtons).

(c) The standards set forth in paragraphs (a) and (b) of this section refer

to exhaust smoke emissions emitted during operations of the engine as

specified in the applicable section of subpart H of this part, and measured

and calculated in accordance with the procedure set forth in this subpart.

Subpart E--[Reserved]

Subpart F--[Reserved]

Subpart G--Test Procedures for Engine Exhaust Gaseous Emissions (Aircraft

and Aircraft Gas Turbine Engines)

Sec. 34.60  Introduction.

(a) Except as provided under Sec. 34.5, the procedures described in this

subpart shall constitute the test program used to determine the conformity of

new aircraft gas turbine engines with the applicable standards set forth in

this part.

(b) The test consists of operating the engine at prescribed power settings

on an engine dynamometer (for engines producing primarily shaft power) or

thrust measuring test stand (for engines producing primarily thrust). The

exhaust gases generated during engine operation must be sampled continuously

for specific component analysis through the analytical train.

(c) The exhaust emission test is designed to measure hydrocarbons, carbon

monoxide and carbon dioxide concentrations, and to determine mass emissions

through calculations during a simulated aircraft landing-takeoff (LTO) cycle.

The LTO cycle is based on time in mode data during high activity periods at

major airports. The test for non-TSS class propulsion engines consists of at

least the following four modes of engine operations: taxi/idle, takeoff,

climbout, and approach. The TSS class propulsion engine test requires an

additional mode for descent. The mass emission for the modes are combined to

yield the reported values.

(d) When an engine is tested for exhaust emissions on an engine dynamometer

or test stand, the complete engine (with all accessories which might

reasonably be expected to influence emissions to the atmosphere installed and

functioning), shall be used if not otherwise prohibited by Sec. 34.62(a)(2).

Use of service air bleed and shaft power extraction to power auxiliary,

gearbox-mounted components required to drive aircraft systems is not

permitted.

(e) Other gaseous emissions measurement systems may be used if shown to

yield equivalent results and if approved in advance by the Administrator or

the Administrator of the EPA.

Sec. 34.61  Turbine fuel specifications.

For exhaust emission testing, fuel meeting the specifications listed below

shall be used. Additives used for the purpose of smoke suppression (such as

organometallic compounds) shall not be present.

Specification for Fuel to be Used in Aircraft Turbine

Engine Emission Testing

Allowable range

Property                              of values

Specific Gravity at 15 deg.C                 0.78-0.82.

Distillation Temperature, deg.C

10% Boiling Point                            160-201.

Final Boiling Point                          240-285.

Net Heat of Combustion, kj/Kg                42,860-43,500.

Aromatics, Volume %                          15-20.

Naphthalenes, Volume %                       1.0-3.0.

Smoke Point, mm                              20-28.

Hydrogen, Mass %                             13.4-14.0.

Sulphur, Mass %                              less than 0.3%.

Kinematic Viscosity at--20 deg.C, mm**2/sec  4.0-6.5.

[Dkt. No. 25613, 55 FR 32861, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

Sec. 34.62  Test procedure (propulsion engines).

(a)(1) The engine shall be tested in each of the following engine operating

modes which simulate aircraft operation to determine its mass emission rates.

The actual power setting, when corrected to standard day conditions, should

correspond to the following percentages of rated output. Analytical

correction for variations from reference day conditions and minor variations

in actual power setting should be specified and/or approved by the

Administrator:

Class

TF,

T3,

Mode     TP   T8   TSS

Taxi/idle  (*)  (*)  (*)

Takeoff    100  100  100

Climbout    90   85   65

Descent     NA   NA   15

Approach    30   30   34

*See paragraph (a) of this section.

(2) The taxi/idle operating modes shall be carried out at a power setting

of 7 percent rated thrust unless the Administrator determines that the unique

characteristics of an engine model undergoing certification testing at 7

percent would result in substantially different HC emissions than if the

engine model were tested at the manufacturers', recommended idle power

setting. In such cases the Administrator shall specify an alternative test

condition.

(3) The times in mode (TIM) shall be as specified below:

Class

TF, T3,

Mode        TP        or T8       TSS

Taxi/idle  26.0 Min.  26.0 Min.  26.0 Min.

Takeoff          0.5        0.7        1.2

Climbout         2.5        2.2        2.0

Descent          N/A        N/A        1.2

Approach         4.5        4.0        2.3

(b) Emissions testing shall be conducted on warmed-up engines which have

achieved a steady operating temperature.

[Dkt. No. 25613, 55 FR 32861, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

Sec. 34.63  [Reserved]

Sec. 34.64  Sampling and analytical procedures for measuring gaseous exhaust

emissions.

The system and procedures for sampling and measurement of gaseous emissions

shall be done in accordance with appendices 3 and 5 to ICAO Annex 16, Volume

II--Aircraft Engine Emissions, First Edition, June 1981. This incorporation

by reference was approved by the Director of the Federal Register in

accordance with 5 U.S.C. 552(a) and 1 CFR Part 51. This document can be

obtained from the International Civil Aviation, P.O. Box 400, Succursale:

Place de L'Aviation Internationale, 1000 Sherbrooke Street West, Montreal,

Quebec, Canada H3H 2R2. Copies may be inspected at the FAA Office of the

Chief Counsel, Rules Docket, room 916, Federal Aviation Administration

Headquarters Building, 800 Independence Avenue, SW., Washington, DC, or at

the FAA New England Regional Office, 12 New England Executive Park,

Burlington, Massachusetts, or at the Office of the Federal Register, 800

North Capitol Street, NW., suite 700, Washington, DC.

[Dkt. No. 25613, 55 FR 32861, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

Editorial note: At 57 FR 31947, July 20, 1992, the Office of the Federal

Register's address for inspection of materials incorporated by reference was

changed effective July 20, 1992.

Secs. 34.65-34.70  [Reserved]

Sec. 34.71  Compliance with gaseous emission standards.

Compliance with each gaseous emission standard by an aircraft engine shall

be determined by comparing the pollutant level in grams/kilonewton/thrust/

cycle or grams/kilowatt/cycle as calculated pursuant to Sec. 34.64 with the

applicable emission standard under this part.

Subpart H--Test Procedures for Engine Smoke Emissions (Aircraft Gas

Turbine Engines)

Sec. 34.80  Introduction.

Except as provided under Sec. 34.5, the procedures described in this

subpart shall constitute the test program to be used to determine the

conformity of new and in-use gas turbine engines with the applicable

standards set forth in this part. The test is essentially the same as that

described in Secs. 34.60-34.62, except that the test is designed to determine

the smoke emission level at various operating points representative of engine

usage in aircraft. Other smoke measurement systems may be used if shown to

yield equivalent results and if approved in advance by the Administrator or

the Administrator of the EPA.

Sec. 34.81  Fuel specifications.

Fuel having specifications as provided in Sec. 34.61 shall be used in smoke

emission testing.

Sec. 34.82  Sampling and analytical procedures for measuring -smoke exhaust

emissions.

The system and procedures for sampling and measurement of smoke emissions

shall be done in accordance with appendix 2 to ICAO Annex 16, Volume II--

Aircraft Engine Emissions, First Edition, June 1981. This incorporation by

reference was approved by the Director of the Federal Register in accordance

with 5 U.S.C. 552(a) and 1 CFR part 51. This document can be obtained from

the International Civil Aviation, P.O. Box 400, Succursale: Place de L'

Aviation Internationale, 1000 Sherbrooke Street West, Montreal, Quebec,

Canada H3H 2R2. Copies may be inspected at the FAA Office of the Chief

Counsel, Rules Docket, room 916, Federal Aviation Administration Headquarters

Building, 800 Independence Avenue, SW., Washington, DC, or at the FAA New

England Regional Office, 12 New England Executive Park, Burlington,

Massachusetts, or at the Office of the Federal Register, 800 North Capitol

Street, NW., suite 700, Washington, DC.

[Dkt. No. 25613, 55 FR 32861, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

Editorial note: At 57 FR 31947, July 20, 1992, the Office of the Federal

Register's address for inspection of materials incorporated by reference was

changed effective July 20, 1992.

Sec. 34.83--Sec. 34.88  [Reserved]

Sec. 34.89  Compliance with smoke emission standards.

Compliance with each smoke emission standard shall be determined by

comparing the plot of the smoke number as a function of power setting with

the applicable emission standard under this part. The smoke number at every

power setting must be such that there is a high degree of confidence that the

standard will not be exceeded by any engine of the model being tested. An

acceptable alternative to testing every engine is described in appendix 6 to

ICAO Annex 16, Volume II--Aircraft Engine Emissions, First Edition, June

1981. Other methods of demonstrating compliance may be approved by the

Administrator with the concurrence of the Administrator of the EPA.