PART 36--NOISE STANDARDS: AIRCRAFT TYPE AND AIRWORTHINESS CERTIFICATION

Special Federal Aviation Regulation No. 41 [Note]

Subpart A--General

Sec.

36.1  Applicability and definitions.

36.2  Special retroactive requirements.

36.3  Compatibility with airworthiness requirements.

36.5  Limitation of part.

36.6  Incorporations by reference.

36.7  Acoustical change: Transport category large airplanes and turbojet

powered airplanes.

36.9  Acoustical change: Propeller-driven small airplanes and propeller-

driven commuter category airplanes.

36.11  Acoustical change: Helicopters.

Subpart B--Noise Measurement and Evaluation for Transport Category Large

Airplanes and Turbojet Powered Airplanes

36.101  Noise measurement.

36.103  Noise evaluation.

Subpart C--Noise Limits for Subsonic Transport Category Large Airplanes and

Subsonic Turbojet Powered Airplanes

36.201  Noise limits.

Subpart D--Noise Limits for Supersonic Transport Category Airplanes

36.301  Noise limits: Concorde.

Subpart E--[Reserved]

Subpart F--Propeller Driven Small Airplanes and Propeller-Driven, Commuter

Category Airplanes

36.501  Noise limits.

Subpart G--[Reserved]

Subpart H--Helicopters

36.801  Noise Measurement.

36.803  Noise evaluation and calculation.

36.805  Noise limits.

Subparts I--N  [Reserved]

Subpart O--Operating Limitations and Information

36.1501  Procedures, noise levels and other information.

36.1581  Manuals, markings, and placards.

36.1583  Noncomplying agricultural and fire fighting airplanes.

Appendix A--Aircraft Noise Measurement under Sec. 36.101

Appendix B--Aircraft Noise Evaluation under Sec. 36.103

Appendix C--Noise Levels for Transport Category and Turbojet Powered

Airplanes under Sec. 36.201

Appendix D--E  [Reserved]

Appendix F--Flyover Noise Requirements for Propeller-Driven Small Airplane

and Propeller-Driven, Commuter Category Airplane Certification Tests

Prior to December 22, 1988

Appendix G--Takeoff Noise Requirements for Propeller-Driven Small Airplane

and Propeller-Driven, Commuter Category Airplane Certification Tests on

or After December 22, 1988

Appendix H--Noise Requirements for Helicopters Under Subpart H

Appendix I--[Reserved]

Appendix J--Alternative Noise Certification Procedure for Helicopters Under

Subpart H Having A Maximum Certificated Takeoff Weight Of Not More Than

6,000 Pounds

Authority: 49 U.S.C. App. 1344, 1348, 1354(a), 1355, 1421, 1423, 1424,

1425, 1428, 1429, 1430, 1431(b), 1651(b)(2), 2101, 2121 through 2125; 42

U.S.C. 4321 et. seq.; E.O. 11514, 35 FR 4247, 3 CFR, 1966-1970 Comp., p. 902;

49 U.S.C. 106(g).

Source: Docket No. 9337, 34 FR 18364, Nov. 18, 1969, unless otherwise

noted.

Special Federal Aviation Regulation No. 41

Editorial Note: For the text of SFAR No. 41 see Part 21 of this chapter.

Subpart A--General

Sec. 36.1  Applicability and definitions.

(a) This part prescribes noise standards for the issue of the following

certificates:

(1) Type certificates, and changes to those certificates, and standard

airworthiness certificates, for subsonic transport category large airplanes,

and for subsonic turbojet powered airplanes regardless of category.

(2) Type certificates and changes to those certificates, standard

airworthiness certificates, and restricted category airworthiness

certificates, for propeller-driven, small airplanes, and for propeller-

driven, commuter category airplanes except those airplanes that are designed

for "agricultural aircraft operations" (as defined in Sec. 137.3 of this

chapter, as effective on January 1, 1966) or for dispersing fire fighting

materials to which Sec. 36.1583 of this part does not apply.

(3) A type certificate and changes to that certificate, and standard

airworthiness certificates, for Concorde airplanes.

(4) Type certificates, and changes to those certificates, for helicopters

except those helicopters that are designated exclusively for "argicultural

aircraft operations" (as defined in Sec. 137.3 of this chapter, as effective

on January 1, 1966), for dispensing fire fighting materials, or for carrying

external loads (as defined in Sec. 133.1(b) of this chapter, as effective on

December 20, 1976).

(b) Each person who applies under Part 21 of this chapter for a type of

airworthiness certificate specified in this part must show compliance with

the applicable requirements of this part, in addition to the applicable

airworthiness requirements of this chapter.

(c) Each person who applies under Part 21 of this chapter for approval of

an acoustical change described in Sec. 21.93(b) of this chapter must show

that the aircraft complies with the applicable provisions of Secs. 36.7,

36.9, or 36.11 of this part in addition to the applicable airworthiness

requirements of this chapter.

(d) Each person who applies for the original issue of a standard

airworthiness certificate for a transport category large airplane or for a

turbojet powered airplane under Sec. 21.183 must, regardless of date of

application, show compliance with the following provisions of this part

(including Appendix C):

(1) The provisions of this part in effect on December 1, 1969, for subsonic

airplanes that have not had any flight time before--

(i) December 1, 1973, for airplanes with maximum weights greater than

75,000 pounds, except for airplanes that are powered by Pratt & Whitney Turbo

Wasp JT3D series engines;

(ii) December 31, 1974, for airplanes with maximum weights greater than

75,000 pounds and that are powered by Pratt & Whitney Turbo Wasp JT3D series

engines; and

(iii) December 31, 1974, for airplanes with maximum weights of 75,000

pounds and less.

(2) The provisions of this part in effect on October 13, 1977, including

the stage 2 noise limits, for Concorde airplanes that have not had flight

time before January 1, 1980.

(3) December 31, 1974, for airplanes with maximum weights of 75,000 lbs.

and less.

(e) Each person who applies for the original issue of a standard

airworthiness certificate under Sec. 21.183, or for the original issue of a

restricted category airworthiness certificate under Sec. 21.185, for

propeller-driven, commuter category airplanes for a propeller driven small

airplane that has not had any flight time before January 1, 1980, must show

compliance with the applicable provisions of this part.

(f) For the purpose of showing compliance with this part for transport

category large airplanes and turbojet powered airplanes regardless of

category, the following terms have the following meanings:

(1) A "Stage 1 noise level" means a takeoff, sideline or approach noise

level greater than the Stage 2 noise limits prescribed in section C36.5(a)(2)

of Appendix C of this part.

(2) A "Stage 1 airplane" means an airplane that has not been shown under

this part to comply with the takeoff, sideline, and approach noise levels

required for Stage 2 or Stage 3 airplanes.

(3) A "Stage 2 noise level" means a noise level at or below the Stage 2

noise limits prescribed in section C36.5(a)(2) of Appendix C of this part but

higher than the Stage 3 noise limits prescribed in section C36.5(a)(3) of

Appendix C of this part.

(4) A "Stage 2 airplane" means an airplane that has been shown under this

part to comply with Stage 2 noise levels prescribed in section C36.5 of

Appendix C of this part (including use of the applicable tradeoff provisions)

and that does not comply with the requirements for a Stage 3 airplane.

(5) A "Stage 3 noise level" means a noise level at or below the Stage 3

noise limits prescribed in section C36.5(a)(3) of Appendix C of this part.

(6) A "Stage 3 airplane" means an airplane that has been shown under this

part to comply with Stage 3 noise levels prescribed in section C36.5 of

Appendix C of this part (including use of the applicable tradeoff

provisions).

(7) A "subsonic airplane" means an airplane for which the maximum operating

limit speed, Mmo, does not exceed a Mach number of 1.

(8) A "supersonic airplane" means an airplane for which the maximum

operating limit speed, Mmo, exceeds a Mach number of 1.

(g) For the purpose of showing compliance with this part for transport

category large airplanes and turbojet airplanes regardless of category, each

airplane may not be identified as complying with more than one stage or

configuration simultaneously.

(h) For the purpose of showing compliance with this part, for helicopters

in the primary, normal, transport, and restricted categories, the following

terms have the specified meanings:

(1) Stage 1 noise level means a takeoff, flyover, or approach noise level

greater than the Stage 2 noise limits prescribed in section H36.305 of

Appendix H of this part, or a flyover noise level greater than the Stage 2

noise limits prescribed in section J36.305 of appendix J of this part.

(2) Stage 1 helicopter means a helicopter that has not been shown under

this part to comply with the takeoff, flyover, and approach noise levels

required for Stage 2 helicopters as prescribed in section H36.305 of Appendix

H of this part, or a helicopter that has not been shown under this part to

comply with the flyover noise level required for Stage 2 helicopters as

prescribed in section J36.305 of Appendix J of this part.

(3) Stage 2 noise level means a takeoff, flyover, or approach noise level

at or below the Stage 2 noise limits prescribed in section H36.305 of

Appendix H of this part, or a flyover noise level at or below the Stage 2

limit prescribed in section J36.305 of Appendix J of this part.

(4) Stage 2 helicopter means a helicopter that has been shown under this

part to comply with Stage 2 noise limits (including applicable tradeoffs)

prescribed in section H36.305 of Appendix H of this part, or a helicopter

that has been shown under this part to comply with the Stage 2 noise limit

prescribed in section J36.305 of Appendix J of this part.

[Doc. No. 13243, Amdt. 36-4, 40 FR 1034, Jan. 6, 1975 as amended by Amdt. 36-

7, 42 FR 12370, Mar. 3, 1977; Amdt. 36-10, 43 FR 28419, June 29, 1978; Amdt.

36-11, 45 FR 67066, Oct. 9, 1980; Amdt. 36-13, 52 FR 1836, Jan. 15, 1987;

Amdt. 36-14, 53 FR 3540, Feb. 5, 1988; 53 FR 7728, Mar. 10, 1988; Amdt. 36-

15, 53 FR 16366, May 6, 1988; Amdt. 36-19, 57 FR 41369, Sept. 9, 1992; Amdt.

36-20, 57 FR 42854, Sept. 16, 1992]

Sec. 36.2  Special retroactive requirements.

(a) Notwithstanding Sec. 21.17 of this chapter, each person who applies for

a type certificate:

(1) For an airplane covered by this part, irrespective of the date of

application for the type certificate, or

(2) For a helicopter covered by this part, on or after March 6, 1986,

must show compliance with the applicable provisions of this part.

(b) Notwithstanding Sec. 21.101(a) of this chapter, each person who applies

for an acoustical change to a type design specified in Sec. 21.93(b) of this

chapter must show compliance with the applicable provisions of this part.

[Doc. No. 9337, 34 FR 18364, Nov. 18, 1969, as amended by Amdt. 36-14, 53 FR

3540, Feb. 5, 1988]

Sec. 36.3  Compatibility with airworthiness requirements.

It must be shown that the aircraft meets the airworthiness regulations

constituting the type certification basis of the aircraft under all

conditions in which compliance with this part is shown, and that all

procedures used in complying with this part, and all procedures and

information for the flight crew developed under this part, are consistent

with the airworthiness regulations constituting the type certification basis

of the aircraft.

[Doc. No. 9337, 34 FR 18364, Nov. 18, 1969, as amended by Amdt. 36-14, 53 FR

3540, Feb. 5, 1988]

Sec. 36.5  Limitation of part.

Pursuant to 49 U.S.C. 1431(b)(4), the noise levels in this part have been

determined to be as low as is economically reasonable, technologically

practicable, and appropriate to the type of aircraft to which they apply. No

determination is made, under this part, that these noise levels are or should

be acceptable or unacceptable for operation at, into, or out of, any airport.

Sec. 36.6  Incorporations by reference.

(a) General. This part prescribes certain standards and procedures which

are not set forth in full text in the rule. Those standards and procedures

are contained in published material which is reasonably available to the

class of persons affected and has been approved for incorporation by

reference by the Director of the Federal Register under 5 U.S.C. 552 (a) and

1 CFR Part 51.

(b) Incorporated matter. (1) Each publication, or part of a publication,

which is referenced but not set forth in full-text in this part and which is

identified in paragraph (c) of this section is hereby incorporated by

reference and made a part of Part 36 of this chapter with the approval of the

Director of the Federal Register.

(2) Incorporated matter which is subject to subsequent change is

incorporated by reference according to the specific reference and to the

identification statement. Adoption of any subsequent change in incorporated

matter is made under Part 11 of this chapter and 1 CFR Part 51.

(c) Identification statement. The complete title or description which

identifies each published matter incorporated by reference in this part is as

follows:

(1) International Electrotechnical Commission (IEC) Publications. (i) IEC

Publication No. 179, entitled "Precision Sound Level Meters," dated 1973.

(ii) IEC Publication No. 225, entitled "Octave, Half-Octave, Third Octave

Band Filters Intended for the Analysis of Sounds and Vibrations," dated 1966.

(iii) IEC Publication No. 651, entitled "Sound Level Meters," first

edition, dated 1979.

(iv) IEC Publication No. 561, entitled "Electro-acoustical Measuring

Equipment for Aircraft Noise Certification," first edition, dated 1976.

(v) IEC Publication No. 804, entitled "Integrating-averaging Sound Level

Meters," first edition, dated 1985.

(2) Society of Automotive Engineers (SAE) Publications. (i) SAE ARP 866A,

entitled "Standard Values at Atmospheric Absorption as a Function of

Temperature and Humidity for Use in Evaluating Aircraft Flyover Noise," dated

March 15, 1975.

(d) Availability for purchase. Published material incorporated by reference

in this part may be purchased at the price established by the publisher or

distributor at the following mailing addresses:

(1) IEC publications. (i) The Bureau Central de la Commission

Electrotechnique, Internationale, 1, rue de Varembe, Geneva, Switzerland.

(ii) American National Standard Institute, 1430 Broadway, New York City,

New York 10018.

(2) SAE publications. Society of Automotive Engineers, Inc., 400

Commonwealth Drive, Warrentown, Pennsylvania 15096.

(e) Availability for inspection. A copy of each publication incorporated by

reference in this part is available for public inspection at the following

locations:

(1) FAA Office of the Chief Counsel, Rules Docket, Room 916, Federal

Aviation Administration Headquarters Building, 800 Independence Avenue, SW.,

Washington, D.C.

(2) Department of Transportation, Branch Library, Room 930, Federal

Aviation Administration Headquarters Building, 800 Independence Avenue, SW.,

Washington, D.C.

(3) The respective Region Headquarters of the Federal Aviation

Administration as follows:

(i) New England Region Headquarters, 12 New England Executive Park,

Burlington, Massachusetts 01803.

(ii) Eastern Region Headquarters, Federal Building, John F. Kennedy (JFK)

International Airport, Jamaica, New York 11430.

(iii) Southern Region Headquarters, 3400 Norman Berry Drive, East Point,

Georgia 30344.

(iv) Great Lakes Region Headquarters, O'Hare Lake Office Center, 2300 East

Devon Avenue, Des Plaines, Illinois 60018.

(v) Central Region Headquarters, Federal Building, 601 East 12th Street,

Kanasa City Missouri 64106.

(vi) Southwest Region Headquarters, 4400 Blue Mound Road, Fort Worth, Texas

76193-0000.

(vii) Northwest Mountain Region Headquarters, 17900 Pacific Highway South,

Seattle, Washington 98168.

(viii) Western-Pacific Region Headquarters, 15000 Aviation Boulevard,

Hawthorne, California 92007.

(ix) Alaskan Region Headquarters, 701 C Street, Anchorage, Alaska 99513.

(x) European Office Headquarters, 15, Rue de la Loi (3rd Floor), B-1040

Brussels, Belgium.

[Amdt. 36-9, 43 FR 8739, Mar. 3, 1978, as amended by Amdt. 36-16, 53 FR

47400, Nov. 22, 1988; Amdt. 36-20, 57 FR 42854, Sept. 16, 1992]

Sec. 36.7  Acoustical change: Transport category large airplanes and turbojet

powered airplanes.

(a) Applicability. This section applies to all transport category large

airplanes and turbojet powered airplanes for which an acoustical change

approval is applied for under Sec. 21.93(b) of this chapter.

(b) General requirements. Except as otherwise specifically provided, for

each airplane covered by this section, the acoustical change approval

requirements are as follows:

(1) In showing compliance, noise levels must be measured and evaluated in

accordance with the applicable procedures and conditions prescribed in

Appendices A and B of this part.

(2) Compliance with the noise limits prescribed in section C36.5 of

Appendix C must be shown in accordance with the applicable provisions of

sections C36.7 and C36.9 of Appendix C of this part.

(c) Stage 1 airplanes. For each Stage 1 airplane prior to the change in

type design, in addition to the provisions of paragraph (b) of this section,

the following apply:

(1) If an airplane is a Stage 1 airplane prior to the change in type

design, it may not, after the change in type design, exceed the noise levels

created prior to the change in type design. The tradeoff provisions of

section C36.5(b) of Appendix C of this part may not be used to increase the

Stage 1 noise levels, unless the aircraft qualifies as a Stage 2 airplane.

(2) In addition, for an airplane for which application is made after

September 17, 1971--

(i) There may be no reduction in power or thrust below the highest

airworthiness approved power or thrust, during the tests conducted before and

after the change in type design; and

(ii) During the takeoff and sideline noise tests conducted before the

change in type design, the quietest airworthiness approved configuration

available for the highest approved takeoff weight must be used.

(d) Stage 2 airplanes. If an airplane is a Stage 2 airplane prior to the

change in type design, the following apply, in addition to the provisions of

paragraph (b) of this section:

(1) Airplanes with high bypass ratio turbojet engines. For an airplane that

has turbojet engines with a bypass ratio of 2 or more before a change in type

design--

(i) The airplane, after the change in type design, may not exceed either

(A) each Stage 3 noise limit by more than 3 EPNdB, or (B) each Stage 2 noise

limit, whichever is lower:

(ii) The tradeoff provisions of section C36.5(b) of Appendix C of this part

may be used in determining compliance under this paragraph with respect to

the Stage 2 noise limit or to the Stage 3 plus 3 EPNdB noise limits, as

applicable; and

(iii) During the takeoff and sideline noise test conducted before the

change in type design, the quietest airworthiness approved configuration

available for the highest approved takeoff weight must be used.

(2) Airplanes that do not have high bypass ratio turbojet engines. For an

airplane that does not have turbojet engines with a bypass ratio of 2 or more

before a change in type design--

(i) The airplane may not be a Stage 1 airplane after the change in type

design; and

(ii) During the takeoff and sideline noise tests conducted before the

change in type design, the quietest airworthiness approved configuration

available for the highest approved takeoff weight must be used.

(e) Stage 3 airplanes. If an airplane is a Stage 3 airplane prior to the

change in type design, the following apply, in addition to the provisions of

paragraph (b) of this section:

(1) If compliance with Stage 3 noise levels is not required before the

change in type design, the airplane must--

(i) Be a Stage 2 airplane after the change in type design and compliance

must be shown under the provisions of paragraph (d)(1) or (d)(2) of this

section, as appropriate; or

(ii) Remain a Stage 3 airplane after the change in type design. Compliance

must be shown under the provisions of paragraph (e)(2) of this section.

(2) If compliance with Stage 3 noise levels is required before the change

in type design, the airplane must be a Stage 3 airplane after the change in

type design.

(3) Applications on or after [August 14, 1989.] The airplane must remain a

Stage 3 airplane after the change in type design.

[Amdt. 36-7, 42 FR 12371, Mar. 3, 1977; Amdt. 36-8, 43 FR 8730, Mar. 2, 1978;

Amdt. 36-10, 43 FR 28420, June 29, 1978; Amdt. 36-12, 46 FR 33464, June 29,

1981; Amdt. 36-15, 53 FR 16366, May 6, 1988; 53 FR 18950, May 25, 1988; Amdt.

36-17, 54 FR 21040, May 15, 1989]

Sec. 36.9  Acoustical change: Propeller-driven small airplanes and propeller-

driven commuter category airplanes.

For propeller-driven small airplanes in the primary, normal, utility,

acrobatic, transport, and restricted categories and for propeller-driven,

commuter category airplanes for which an acoustical change approval is

applied for under Sec. 21.93(b) of this chapter after January 1, 1975, the

following apply:

(a) If the airplane was type certificated under this part prior to a change

in type design, it may not subsequently exceed the noise limits specified in

Sec. 36.501 of this part.

(b) If the airplane was not type certificated under this part prior to a

change in type design, it may not exceed the higher of the two following

values:

(1) The noise limit specified in Sec. 36.501 of this part, or

(2) The noise level created prior to the change in type design, measured

and corrected as prescribed in Sec. 36.501 of this part.

[Amdt. 36-16, 53 FR 47400, Nov. 22, 1988; 53 FR 50157, Dec. 13, 1988; Amdt.

36-19, 57 FR 41369, Sept. 9, 1992]

Sec. 36.11   Acoustical change: Helicopters.

This section applies to all helicopters in the primary, normal, transport,

and restricted categories for which an acoustical change approval is applied

for under Sec. 21.93(b) of this chapter on or after March 6, 1986. Compliance

with the requirements of this section must be demonstrated under appendix H

of this part, or, for helicopters having a maximum certificated takeoff

weight of not more than 6,000 pounds, compliance with this section may be

demonstrated under Appendix J of this part.

(a) General requirements. Except as otherwise provided, for helicopters

covered by this section, the acoustical change approval requirements are as

follows:

(1) In showing compliance with the requirements of appendix H of this part,

noise levels must be measured, evaluated, and calculated in accordance with

the applicable procedures and conditions prescribed in parts B and C of

appendix H of this part. For helicopters having a maximum certificated

takeoff weight of not more than 6,000 pounds that alternatively demonstrate

compliance under appendix J of this part, the flyover noise level prescribed

in appendix J of this part must be measured, evaluated, and calculated in

accordance with the applicable procedures and conditions prescribed in parts

B and C of appendix J of this part.

(2) Compliance with the noise limits prescribed in section H36.305 of

appendix H of this part must be shown in accordance with the applicable

provisions of part D of appendix H of this part. For those helicopters that

demonstrate compliance with the requirements of appendix J of this part,

compliance with the noise levels prescribed in section J36.305 of appendix J

of this part must be shown in accordance with the applicable provisions of

part D of appendix J of this part.

(b) Stage 1 helicopters. Except as provided in Sec. 36.805(c), for each

Stage 1 helicopter prior to a change in type design, the helicopter noise

levels may not, after a change in type design, exceed the noise levels

specified in section H36.305(a)(1) of appendix H of this part where the

demonstration of compliance is under appendix H of this part. The tradeoff

provisions under section H36.305(b) of appendix H of this part may not be

used to increase any Stage 1 noise level beyond these limits. If an applicant

chooses to demonstrate compliance under appendix J of this part, for each

Stage 1 helicopter prior to a change in type design, the helicopter noise

levels may not, after a change in type design, exceed the Stage 2 noise

levels specified in section J36.305(a) of Appendix J of this part.

(c) Stage 2 helicopters. For each helicopter that is Stage 2 prior to a

change in type design, the helicopter must be a Stage 2 helicopter after a

change in type design.

[Amdt. 36-20, 57 FR 42854, Sept. 16, 1992]

Subpart B--Noise Measurement and Evaluation for Transport Category Large

Airplanes and Turbojet Powered Airplanes

Sec. 36.101  Noise measurement.

For transport category large airplanes and turbojet powered airplanes the

noise generated by the airplane must be measured under Appendix A of this

part or under an approved equivalent procedure.

[Doc. No. 9337, 34 FR 18364, Nov. 18, 1969, as amended by Amdt. 36-10, 43 FR

28420, June 29, 1968]

Sec. 36.103  Noise evaluation.

For transport category large airplanes and turbojet powered airplanes noise

measurement information obtained under Sec. 36.101 must be evaluated under

Appendix B of this part or under an approved equivalent procedure.

[Doc. No. 9337, 34 FR 18364, Nov. 18, 1969, as amended by Amdt. 36-10, 43 FR

28420, June 29, 1978]

Subpart C--Noise Limits for Subsonic Transport Category Large Airplanes and

Subsonic Turbojet Powered Airplanes

Sec. 36.201  Noise limits.

(a) For subsonic transport category large airplanes and subsonic turbojet

powered airplanes compliance with this section must be shown with noise

levels measured and evaluated as prescribed in Subpart B of this part, and

demonstrated at the measuring points, and in accordance with the flight test

conditions under sections C36.7 and C36.9 (or an approved equivalent

procedure), prescribed under Appendix C of this part.

(b) Type certification applications for subsonic transport category large

airplanes and all subsonic turbojet powered airplanes must show that the

noise levels of the airplane are no greater than the Stage 3 noise limits

prescribed in section C36.5(a)(3) of Appendix C of this part.

[Doc. No. 9337, 34 FR 18364, Nov. 18, 1969, as amended by Amdt. 36-7, 42 FR

12371, Mar. 3, 1977; Amdt. 36-8, 43 FR 8730, Mar. 2, 1978; Amdt. 36-10, 43 FR

28420, June 29, 1978; Amdt. 36-12, 46 FR 33464, June 29, 1981; Amdt. 36-15,

53 FR 16366, May 6, 1988]

Subpart D--Noise Limits for Supersonic Transport Category Airplanes

Sec. 36.301  Noise limits: Concorde.

(a) General. For the Concorde airplane, compliance with this subpart must

be shown with noise levels measured and evaluated as prescribed in Subpart B

of this part, and demonstrated at the measuring points prescribed in Appendix

C of this part.

(b) Noise limits. It must be shown, in accordance with the provisions of

this part in effect on October 13, 1977, that the noise levels of the

airplane are reduced to the lowest levels that are economically reasonable,

technologically practicable, and appropriate for the Concorde type design.

[Amdt. 36-10, 43 FR 28420, June 29, 1978]

Subpart E--[Reserved]

Subpart F--Propeller Driven Small Airplanes And Propeller-Driven, Commuter

Category Airplanes

Sec. 36.501  Noise limits.

(a) Compliance with this subpart must be shown for--

(1) Propeller driven small airplanes for which application for the issuance

of a new, amended, or supplemental type certificate in the normal, utility,

acrobatic, transport, or restricted category is made on or after October 10,

1973; and propeller-driven, commuter category airplanes for which application

for the issuance of a type certificate in the commuter category is made on or

after January 15, 1987.

(2) Propeller driven small airplanes and propeller-driven, commuter

category airplanes for which application is made for the original issuance of

a standard airworthiness certificate or restricted category airworthiness

certificate, and that have not had any flight time before January 1, 1980

(regardless of date of application).

(3) Airplanes in the primary category:

(i) Except as provided in paragraph (a)(3)(ii) of this section, for an

airplane for which application for a type certificate in the primary category

is made, and that was not previously certificated under appendix F of this

part, compliance with appendix G of this part must be shown.

(ii) For an airplane in the normal, utility or acrobatic category that (A)

has a type certificate issued under this chapter, (B) has a standard

airworthiness certificate issued under this chapter, (C) has not undergone an

acoustical change from its type design, (D) has not previously been

certificated under appendix F or G of this part, and (E) for which

application for conversion to the primary category is made, no further

showing of compliance with this part is required.

(b) For aircraft covered by this subpart for which certification tests are

completed before December 22, 1988, compliance must be shown with noise

levels as measured and prescribed in Parts B and C of Appendix F, or under

approved equivalent procedures. It must be shown that the noise level of the

airplane is no greater than the applicable limit set in Part D of Appendix F.

(c) For aircraft covered by this subpart for which certification tests are

not completed before December 22, 1988, compliance must be shown with noise

levels as measured and prescribed in Parts B and C of Appendix G, or under

approved equivalent procedures. It must be shown that the noise level of the

airplane is no greater than the applicable limits set in Part D of Appendix

G.

[Doc. No. 13243, 40 FR 1034, Jan. 6, 1975, as amended by Amdt. 36-13, 52 FR

1836, Jan. 15, 1987; Amdt. 36-16, 53 FR 47400, Nov. 22, 1988; Amdt. 36-19,

57 FR 41369, Sept. 9, 1992]

Subpart G--[Reserved]

Subpart H--Helicopters

Source: Amdt. 36-14, 53 FR 3540, Feb. 5, 1988; 53 FR 7728, Mar. 10, 1988,

unless otherwise noted.

Sec. 36.801   Noise measurement.

For primary, normal, transport, or restricted category helicopters for

which certification is sought under appendix H of this part, the noise

generated by the helicopter must be measured at the noise measuring points

and under the test conditions prescribed in part B of appendix H of this

part, or under an FAA-approved equivalent procedure. For those primary,

normal, transport, and restricted category helicopters having a maximum

certificated takeoff weight of not more than 6,000 pounds for which

compliance with appendix J of this part is demonstrated, the noise generated

by the helicopter must be measured at the noise measuring point and under the

test conditions prescribed in part B of appendix J of this part, or an FAA-

approved equivalent procedure.

[Amdt. 36-20, 57 FR 42854, Sept. 16, 1992]

Sec. 36.803   Noise evaluation and calculation.

The noise measurement data required under Sec. 36.801 and obtained under

appendix H of this part must be corrected to the reference conditions

contained in part A of appendix H of this part, and evaluated under the

procedures of part C of appendix H of this part, or an FAA-approved

equivalent procedure. The noise measurement data required under Sec. 36.801

and obtained under appendix J of this part must be corrected to the reference

conditions contained in part A of appendix J of this part, and evaluated

under the procedures of part C of appendix J of this part, or an FAA-approved

equivalent procedure.

[Amdt. 36-20, 57 FR 42854, Sept. 16, 1992]

Sec. 36.805   Noise limits.

(a) Compliance with the noise levels prescribed under part D of appendix H

of this part, or under part D of appendix J of this part, must be shown for

helicopters for which application for issuance of a type certificate in the

primary, normal, transport, or restricted category is made on or after March

6, 1986.

(b) For helicopters covered by this section, except as provided in

paragraph (c) or (d)(2) of this section, it must be shown either:

(1) For those helicopters demonstrating compliance under Appendix H of this

part, the noise levels of the helicopter are no greater than the applicable

limits prescribed under section H36.305 of Appendix H of this part, or

(2) For helicopters demonstrating compliance under Appendix J of this part,

the noise level of the helicopter is no greater than the limit prescribed

under section J36.305 of appendix J of this part.

(c) For helicopters for which application for issuance of an original type

certificate in the primary, normal, transport, or restricted category is made

on or after March 6, 1986, and which the FAA finds to be the first civil

version of a helicopter that was designed and constructed for, and accepted

for operational use by, an Armed Force of the United States or the U.S. Coast

Guard on or before March 6, 1986, it must be shown that the noise levels of

the helicopter are no greater than the noise limits for a change in type

design as specified in section H36.305(a)(1)(ii) of Appendix H of this part

for compliance demonstrated under appendix H of this part, or as specified in

section J36.305 of appendix J of this part for compliance demonstrated under

appendix J of this part. Subsequent civil versions of any such helicopter

must meet the Stage 2 requirements.

(d) Helicopters in the primary category:

(1) Except as provided in paragraph (d)(2) of this section, for a

helicopter for which application for a type certificate in the primary

category is made, and that was not previously certificated under Appendix H

of this part, compliance with Appendix H of this part must be shown.

(2) For a helicopter that:

(i) Has a normal or transport type certificate issued under this chapter,

(ii) Has a standard airworthiness certificate issued under this chapter,

(iii) Has not undergone an acoustical change from its type design,

(iv) Has not previously been certificated under Appendix H of this part,

and

(v) For which application for conversion to the primary category is made,

no further showing of compliance with this part is required.

[Amdt. 36-20, 57 FR 42855, Sept. 16, 1992]

Subparts I--N  [Reserved]

Subpart O--Operating Limitations and Information

Sec. 36.1501  Procedures, noise levels and other information.

(a) All procedures, weights, configurations, and other information or data

employed for obtaining the certified noise levels prescribed by this part,

including equivalent procedures used for flight, testing, and analysis, must

be developed and approved. Noise levels achieved during type certification

must be included in the approved airplane (rotorcraft) flight manual.

(b) Where supplemental test data are approved for modification or extension

of an existing flight data base, such as acoustic data from engine static

tests used in the certification of acoustical changes, the test procedures,

physical configuration, and other information and procedures that are

employed for obtaining the supplemental data must be developed and approved.

[Amdt. 36-15, 53 FR 16366, May 6, 1988]

Sec. 36.1581  Manuals, markings, and placards.

(a) If an Airplane Flight Manual or Rotorcraft Flight Manual is approved,

the approved portion of the Airplane Flight Manual or Rotorcraft Flight

Manual must contain the following information, in addition to that specified

under Sec. 36.1583 of this part. If an Airplane Flight Manual or Rotorcraft

Flight Manual is not approved, the procedures and information must be

furnished in any combination of approved manual material, markings, and

placards.

(1) For transport category large airplanes and turbojet powered airplanes,

the noise level information must be one value for each takeoff, sideline, and

approach as defined and required by Appendix C of this part, along with the

maximum takeoff weight, maximum landing weight, and configuration.

(2) For propeller driven small airplanes the noise level information must

be one value for flyover as defined and required by Appendix F of this part,

along with the maximum takeoff weight and configuration.

(b) If supplemental operational noise level information is included in the

approved portion of the Airplane Flight Manual, it must be segregated,

identified as information in addition to the certificated noise levels, and

clearly distinguished from the information required under Sec. 36.1581(a).

(c) The following statement must be furnished near the listed noise levels:

No determination has been made by the Federal Aviation Administration that

the noise levels of this aircraft are or should be acceptable or unacceptable

for operation at, into, or out of, any airport.

(d) For transport category large airplanes and turbojet powered airplanes,

for which the weight used in meeting the takeoff or landing noise

requirements of this part is less than the maximum weight established under

the applicable airworthiness requirements, those lesser weights must be

furnished, as operating limitations in the operating limitations section of

the Airplane Flight Manual. Further, the maximum takeoff weight must not

exceed the takeoff weight that is most critical from a takeoff noise

standpoint.

(e) For propeller driven small airplanes and for propeller-driven, commuter

category airplanes for which the weight used in meeting the flyover noise

requirements of this part is less than the maximum weight by an amount

exceeding the amount of fuel needed to conduct the test, that lesser weight

must be furnished, as an operating limitation, in the operating limitations

section of an approved Airplane Flight Manual, in approved manual material,

or on an approved placard.

(f) For primary, normal, transport, and restricted category helicopters, if

the weight used in meeting the takeoff, flyover, or approach noise

requirements of appendix H of this part, or the weight used in meeting the

flyover noise requirement of appendix J of this part, is less than the

certificated maximum takeoff weight established under either Sec. 27.25(a) or

Sec. 29.25(a) of this chapter, that lesser weight must be furnished as an

operating limitation in the operating limitations section of the Rotorcraft

Flight Manual, in FAA-approved manual material, or on an FAA-approved

placard.

(g) Except as provided in paragraphs (d), (e), and (f) of this section, no

operating limitations are furnished under this part.

[Doc. 13243, 40 FR 1035, Jan. 6, 1975 as amended by Amdt. 36-10, 43 FR 28420,

June 29, 1978; Amdt. 36-11, 45 FR 67066, Oct. 9, 1980; Amdt. 36-13, 52 FR

1836, Jan. 15, 1987. Redesignated and amended by Amdt. 36-14, 53 FR 3540,

Feb. 5, 1988; 53 FR 7728, Mar. 10, 1988; Amdt. 36-15, 53 FR 16366, May 6,

1988; 53 FR 18950, May 25, 1988; Amdt. 36-20, 57 FR 42855, Sept. 16, 1992]

Sec. 36.1583  Noncomplying agricultural and fire fighting airplanes.

(a) This section applies to propeller-driven, small airplanes that--

(1) Are designed for "agricultural aircraft operations" (as defined in Sec.

137.3 of this chapter, effective on January 1, 1966) or for dispensing fire

fighting materials; and

(2) Have not been shown to comply with the noise levels prescribed under

Appendix F of this part--

(i) For which application is made for the original issue of a standard

airworthiness certificate and that do not have any flight time before January

1, 1980; or

(ii) For which application is made for an acoustical change approval, for

airplanes which have a standard airworthiness certificate after the change in

the type design, and that do not have any flight time in the changed

configuration before January 1, 1980.

(b) For airplanes covered by this section an operating limitation reading

as follows must be furnished in the manner prescribed in Sec. 36.1581:

Noise abatement: This airplane has not been shown to comply with the noise

limits in FAR Part 36 and must be operated in accordance with the noise

operating limitation prescribed under FAR Sec. 91.815.

[Amdt. 36-11, 45 FR 67066, Oct. 9, 1980. Redesignated by Amdt. 36-14, 53 FR

3540, Feb. 5, 1988; Amdt. 36-18, 54 FR 34330, Aug. 18, 1989]

Effective Date Note: At 54 FR 34330, Aug. 18, 1989, Sec. 36.1583(b) was

amended by changing the cross reference "Sec. 91.56" to "Sec. 91.815",

effective August 18, 1990.

Appendix A--Aircraft Noise Measurement Under Sec. 36.101

Sec.

A36.1  Noise certification test and measurement conditions.

A36.3  Measurement of aircraft noise received on the ground.

A36.5  Reporting and correcting measured data.

A36.7  Symbols and units.

A36.9  Atmospheric attenuation of sound.

A36.11  Detailed correction procedures.

Section A36.1  Noise certification test and measurement conditions.

(a) General. This section prescribes the conditions under which aircraft

noise certification tests must be conducted and the measurement procedures

that must be used to measure aircraft noise during each test conducted on or

after April 3, 1978.

(b) Test site requirements. (1) Tests to show compliance with established

aircraft noise certification levels must consist of a series of takeoffs and

approaches (or stabilized flight path segments thereof) during which

measurements must be taken at noise measuring stations located at the

measuring points prescribed in section C36.3 of Appendix C of this part. Each

recorded segment must include measurements throughout the entire time period

in which the recorded signal is within 10 dB of PNLTM.

(2) During each test takeoff, simultaneous measurements should be made at

the sideline noise measuring stations on each side of the runway and also at

the takeoff noise measuring station. However, if test site conditions make it

impractical to simultaneously measure takeoff and sideline noise, and if each

of the other sideline measurement requirements is met, independent

measurements may be made of the sideline noise under simulated flight path

techniques. If the reference flight path includes a power cutback before the

maximum possible sideline noise level is developed, the reduced sideline

noise level which is the maximum value developed by the simulated flight path

technique must be the certificated sideline noise value.

(3) If the height of the ground at a noise measuring station differs from

that of the nearest point on the runway by more than 20 feet, corrections

must be made as prescribed in section A36.5(d) of this appendix.

(4) The location of each noise measuring station must be surrounded by

relatively flat terrain having no excessive sound absorption characteristics,

such as might be caused by thick, matted, or tall grass, shrubs, or wooded

areas.

(5) An airport tower, or other facility, used to obtain required

measurements of meteorological conditions at the test site must be approved

in accordance with section A36.9(b)(1) of this appendix.

(6) During the period when the flyover noise/time record indicates the

noise measurement is within 10 dB of PNLTM, no obstruction that significantly

influences the sound field from the aircraft may exist--

(i) For a takeoff, approach, or sideline measuring station, within a

conical space above the measuring position (the point on the ground

vertically below the microphone), the cone being defined by an axis normal to

the ground and by a half-angle 80 degrees from this axis; and

(ii) For a sideline noise measuring station, above the line of sight

between the microphone and the aircraft.

(7) A minimum of two noise measuring stations, symmetrically positioned

about the test flight track, must be used to define the maximum sideline

noise with respect to location and level as required by section C36.3 of

Appendix C of this part. For turbojet powered aircraft, when approved by the

FAA, the maximum sideline noise at takeoff thrust may be assumed to occur at

the point (or its approved equivalent) along the extended centerline of the

runway where the aircraft reaches 1000 feet (305 meters) altitude above

ground level. A height of 1440 feet (439 meters) may be assumed for Stage 1

or Stage 2 four engine airplanes. The altitude of the aircraft as it passes

the microphone stations must be within +500 to -0 feet (+150 to -0 meters) of

the target altitude. For aircraft powered by other than turbojet engines, the

altitude for maximum sideline noise must be determined experimentally.

(c) Weather restrictions. The tests must be conducted under the following

atmospheric conditions:

(1) No rain or other precipitation.

(2) Ambient air temperature between 36 degrees F and 95 degrees F (2.2

degrees C and 35 degrees C), inclusively, over that portion of the sound

propagation path between the aircraft and a point 10 meters above the ground

at the noise measuring station.

(3) Relative humidity and ambient temperature over that portion of the

sound propagation path between the aircraft and a point 10 meters above the

ground at the noise measuring station is such that the sound attenuation in

the one-third octave band centered a 8 kHz is not greater than 12 dB/100

meters and the relative humidity is between 20 and 95 percent, inclusively.

However, if the dew point and dry bulb temperature used for obtaining

relative humidity are measured with a device which is accurate to within +/-

0.5 deg.C, the sound attenuation rate shall not exceed 14 dB/100 meters in

the one-third octave band centered at 8kHz.

(4) Average wind velocity 10 meters above ground is not to exceed 12 knots

and the crosswind velocity for the airplane is not to exceed 7 knots. The

average wind velocity shall be determined using a thirty-second averaging

period spanning the 10 dB down time interval. Maximum wind velocity 10 meters

above ground is not to exceed 15 knots and the crosswind velocity is not to

exceed 10 knots during the 10 dB down time interval.

(5) No anomalous wind conditions (including turbulence) which will

significantly affect the noise level of the aircraft when the noise is

recorded at each noise measuring station.

(d) Aircraft testing procedures.--(1) The aircraft testing procedures and

noise measurements must be conducted and processed in an approved manner

which yields the noise evaluation measure designated as Effective Perceived

Noise Level (EPNL) in units of EPNdB, as prescribed in Appendix B of this

part.

(2) The aircraft height and lateral position relative to the extended

centerline of the runway must be determined by an FAA approved method which

is independent of normal flight instrumentation, such as radar tracking,

theodolite triangulation, laser trajectography, or photographic scaling

techniques.

(3) The aircraft position along the flight path must be related to the

noise recorded at the noise measuring stations by means of synchronizing

signals at an approved sampling rate. The position of the aircraft must be

recorded relative to the runway during the entire time period in which the

recorded signal is within 10 dB of PNLTM. Measuring and sampling equipment

must be approved by the FAA.

(4) Each takeoff test must meet the conditions of section C36.7 of Appendix

C of this part.

(5) If a takeoff test series is conducted at weights other than the maximum

takeoff weight for which noise certification is requested, the following

additional requirements apply:

(i) At least one takeoff test must be conducted at a weight at, or above,

the maximum certification weight.

(ii) Each test weight must be within +5 percent or -10 percent of the

maximum certification weight.

(6) Each approach test must be conducted with the aircraft stabilized and

following a 3.0 degree +/-0.5 degree approach angle and must meet the

requirements of section C36.9 of Appendix C of this part.

(7) If an approach test series is conducted at weights other than the

maximum landing weight for which certification is requested, the following

additional requirements apply:

(i) At least one approach test must be conducted at a weight at, or above,

the maximum landing weight.

(ii) Each test weight must exceed 90 percent of the maximum landing weight.

(8) Aircraft performance data sufficient to make the correction required

under section A36.5 of this appendix must be recorded at an approved sampling

rate using FAA approved equipment.

Section A36.3  Measurement of aircraft noise received on the ground.

(a) General. (1) The measurements prescribed in this section provide the

data for determining the one-third octave band noise produced by aircraft

during testing at specific noise measuring stations, as a function of time.

(2) Sound pressure level data for aircraft noise certification purposes

must be obtained with approved acoustical equipment and measurement

practices.

(3) Paragraphs (b), (c), and (d) of this section prescribe the required

equipment specifications. Paragraphs (e) and (f) prescribe the calibration

and measurement procedures required for each certification test series.

(b) Measurement system. The acoustical measurement system must consist of

approved equipment equivalent to the following:

(1) A microphone system with frequency response and directivity which are

compatible with the measurement and analysis system accuracy prescribed in

paragraph (c) of this section.

(2) Tripods or similar microphone mountings that minimize interference with

the sound energy being measured.

(3) Recording and reproducing equipment whose characteristics, frequency

response, and dynamic range are compatible with the response and accuracy

requirements of paragraph (c) of this section.

(4) Calibrators using sine wave, or pink noise, of known levels. When pink

noise (defined in paragraph (e)(1) of this section) is used, the signal must

be described in terms of its root-mean-square (rms) value.

(5) Analysis equipment with the response and accuracy which meets or

exceeds the requirements of paragraph (d) of this section.

(6) Attenuators used for range changing in sensing, recording, reproducing,

or analyzing aircraft sound must be capable of being operated in equal-

interval decibel steps with no error between any two settings which exceeds

0.2 dB.

(c) Sensing, recording, and reproducing equipment. (1) The sound produced

by the aircraft must be recorded in such a way that the complete information,

including time history, is retained. A magnetic tape recorder is acceptable.

(2) The microphone must be a pressure sensitive capacitive type, or its

approved equivalent, such as free field type with incidence corrector.

(i) After an adequate "warm-up" period, at least as long as that specified

by the equipment manufacturer, the system output for constant acoustical

input shall change by not more than 0.3 dB within any one hour nor by more

than 0.4 dB within 5 hours.

(ii) The variation of microphone and preamplifier system sensitivity within

an angle of +/-30 degrees of grazing (60-120 degrees from the normal to the

diaphragm) must not exceed the following values:

Change in

sensitivity

Frequency (HZ)      (dB)

45 to 1,120      1.0

1,120 to 2,240   1.5

2,240 to 4,500   2.5

4,500 to 7,100   4.0

7,100 to 11,200  5.0

With the wind screen in place, the variation in sensitivity in the plane of

the diaphragm of the microphone system shall not exceed 1.0 dB over the

frequency range 45 to 11,200 Hz.

(iii) The free-field frequency response of the microphone system at the

reference incidence direction shall lie within an envelope having the

following values:

Change in

Tolerance

Frequency (HZ)       (dB)

45 to 4,500      +/-1.0

4,500 to 5,600   +/-1.5

5,600 to 7,100   +1.5 to -2.0

7,100 to 9,000   +1.5 to -3.0

9,000 to 11,200  +2.0 to -4.0

Note: The requirements of this paragraph may be determined by a pressure

response calibration (which may be obtained from an electrostatic calibrator

in combination with manufacturer provided corrections) or an anechoic free-

field facility.

(iv) Specifications concerning sensitivity to environmental factors such as

temperature, relative humidity, and vibration must in conformity with the

recommendations of International Electrotechnical Commission (IEC)

Publication No. 179, entitled "Precision Sound Level Meters" (as incorporated

by reference under Sec. 36.6 of this part).

(v) If the wind speed exceeds 6 knots, a windscreen must be employed with

the microphone during each measurement of aircraft noise. Correction for any

insertion loss produced by the windscreen as a function of frequency, must be

applied to the measured data and any correction applied must be reported.

(3) If a magnetic tape recorder is used to store data for subsequent

analysis, the record/replay system (including tape) must conform to the

following:

(i) The electric background noise produced by the system in each one-third

octave must be at least 35 dB below the standand recording level, which is

defined as that level which is either 10 dB below the 3 pecent harmonic

distortion level for direct recording or +/-40 percent deviation for

frequency modulation (FM) recording.

(ii) At the standard recording level, the corrected frequency response in

each selected one-third octave band between 44 Hz and 180 Hz must be flat

within +/-0.75 dB, and in each band between 180 Hz and 11,200 Hz must be flat

within +/-0.25 dB.

(iii) If the overall system satisfies the requirements of paragraph

(c)(2)(ii) of this section, and if the limitations of the dynamic range of

the equipment are insufficient to obtain adequate spectral information, high

frequency pre-emphasis may be added to the recording channel with the

converse de-emphasis on playback. If pre-emphasis is added, the

instantaneously recorded sound pressure level between 800 Hz and 11,200 Hz of

the maximum measured noise signal must not vary more than 20 dB between the

levels of the maximum and minimum one-third octave bands.

(d) Analysis equipment. (1) A frequency analysis of the acoustic signal

must be performed using one-third octave filters which conform to the

recommendations of International Electrotechnical Commission (IEC)

Publication No. 225, entitled "Octave, Half-Octave, and Third-Octave Band

Filters Intended for Analysis of Sounds and Vibrations" (as incorporated by

reference under Sec. 36.6 of this part).

(2) A set of 24 consecutive one-third octave filters must be used. The

first filter of the set must be centered at a geometric mean frequency of 50

Hz and the last filter at 10,000 Hz.

(i) The output of each filter must contain less than 0.5 dB ripple.

(ii) The correction for effective bandwidth relative to the response at the

center frequency response for each one-third octave band filter must be

determined by measuring the filter response to sinusoidal signals at a

minimum of 20 frequencies equally spaced between the two adjacent preferred

one-third octave frequencies or by using an approved equivalent procedure.

(3) The analyzer indicating device may be either analog or digital, or a

combination of both. The preferred sequence of signal processing is:

(i) Squaring the one-third octave filter outputs;

(ii) Averaging or intergrating; and

(iii) Coverting linear formulation to logarithmic.

(4) Each detector must operate over a minimum dynamic range of 60 dB and

perform as a true-mean-square device for sinusoidal tone bursts having crest

factors of at least 3 over the following dynamic range:

(i) Up to 30 dB below full-scale reading must be accurate within +0.5 dB;

(ii) Between 30 dB and 40 dB below full-scale reading must be accurate

within +1.0 dB; and

(iii) In excess of 40 dB below full-scale reading must be accurate within

+2.5 dB.

(5) The averaging properties of the integrator must be tested as follows:

(i) White noise must be passed through the 200 Hz one-third octave band

filter and the output fed in turn to each detector/integrator. The standard

deviation of the measured levels must then be determined from a large number

of samples of the filtered white noise taken at intervals of not less than 5

seconds. The value of the standard deviation must be within the interval

0.48+/-0.06 dB for a probability limit of 95 percent. (An approved equivalent

method may be substituted for this test on those analyzers where the test

signal cannot readily be fed directly to each detector/integrator.)

(ii) For each detector/integrator, the response to a sudden onset or

interruption of a constant amplitude sinusoidal signal at the respective one-

third octave band center, frequency must be measured at sampling times 0.5,

1.0, 1.5, and 2.0 seconds after the onset or interruption. The rising

responses must be the following amounts before the steady-state level:

0.5 seconds......................................................4.0+/-1.0 dB

1.0 seconds....................................................1.75+/-0.75 dB

1.5 seconds......................................................1.0+/-0.5 dB

2.0 seconds......................................................0.6+/-0.5 dB

(iii) The falling response must be such that the sum of the decibel

readings (below the initial steady-state level) and the corresponding rising

response reading are 6.5+/-1.0 dB, at each sampling time.

(iv) Analyzers using true integration cannot meet the requirements of

paragraphs (d)(5) (i), (ii), and (iii) of this section directly, because

their overall average time is greater than the sampling interval. For these

analyzers, compliance must be demonstrated in terms of the equivalent output

of the data processor. Further, in cases where readout and resetting require

a dead-time during acquisition, the percentage loss of the total data must

not exceed one percent.

(6) The sampling interval between successive readouts shall not exceed 500

milliseconds and its precise value must be known to within +/-one (1)

percent. The instant in time by which a readout is characterized, shall be

the midpoint of the average period. (The averaging period is defined as twice

the effective time constant of the analyzer.)

(7) The amplitude resolution of the analyzer must be at least 0.25 dB.

(8) After all systematic errors have been eliminated, each output level

from the analyzer must be accurate within +/-1.0 dB of the level of the input

signal. The total systematic errors for each of the output levels must not

exceed +/-3.0 dB. For contiguous filter systems, the systematic correction

between adjacent one-third octave channels must not exceed 4.0 dB.

(9) The dynamic range capability of the analyzer for display of a single

aircraft noise event (in terms of the difference between full-scale output

level and the maximum noise level of the analyzer equipment) must be at least

60 dB.

(e) Calibrations. (1) Within the five days before the beginning of each

test series, the complete electronic system (as installed in the field,

including cables) must be electronically calibrated for frequency and

amplitude by the use of a pink noise signal of known amplitudes covering the

range of signal levels furnished by the microphone. For purposes of this

section, a "pink noise" means a noise whose noise-power/unit-frequency is

inversely proportional to frequency at frequencies within the range of 44 Hz

to 11,200 Hz. The signal used must be described in terms of its average root-

mean-square (rms) values for a nonoverload signal level. This system

calibration must be repeated within five days of the end of each test series,

or as required by the FAA.

(2) Immediately before and after each day's testing, a recorded acoustic

calibration of the system must be made in the field with an acoustic

calibrator to check the system sensitivity and provide an acoustic reference

level for the analysis of the sound level data. The performance of equipment

in the system will be considered satisfactory if, during each day's testing,

the variation does not exceed 0.5 dB.

(3) A normal incidence pressure calibration of the combined microphone/

preamplifier must be performed with pure tones at each preferred one-third

octave frequency from 50 Hz to 10,000 Hz. This calibration must be completed

within the 90 days before the beginning of each test series.

(4) Each reel of magnetic tape must:

(i) Be pistonphone calibrated; and

(ii) At its beginning and end, carry a calibration signal consisting of at

least a 15 second burst of pink noise, as defined in paragraph (e)(1) of this

section.

(5) Data obtained from tape recorded signals are not considered reliable if

the difference between the pink noise signal levels, before and after the

tests in each one-third octave band, exceeds 0.75 dB.

(6) The one-third octave filters must have been demonstrated to be in

conformity with the recommendations of IEC Publication 225 (as incorporated

by reference under Sec. 36.6 of this part) during the six calendar months

preceding the beginning of each test series. However, the correction for

effective bandwidth relative to the center frequency response may be

determined for each filter--

(i) By measuring the filter response to sinusoidal signals at a minimum of

twenty frequencies equally spaced between the two adjacent preferred one-

third octave frequencies; or

(ii) By using an approved alternative technique.

(7) A performance calibration analysis of each piece of calibration

equipment, including piston phones, reference microphones, and voltage insert

devices, must have been made during the six calendar months preceding the

beginning of each day's test series. Each calibration must be traceable to

the National Bureau of Standards.

(f) Noise measurement procedures. (1) Each microphone must be oriented so

that the diaphragm is substantially in the plane defined by the flight path

of the aircraft and the measuring station. The microphone located at each

noise measuring station must be placed so that its sensing element is

approximately 4 feet above ground.

(2) Immediately before and immediately after each series of test runs and

each day's testing, a recorded acoustic calibration of the system prescribed

in section A36.3(e)(2) of this appendix must be made in the field to check

the acoustic reference level for the analysis of the sound level data.

Ambient noise must be recorded for at least 10 seconds and be representative

of the acoustical background, including systemic noise, that exists during

the flyover test run. During that recorded period, each component of the

system must be set at the gain-levels used for aircraft noise measurement.

(3) The mean background noise spectrum must contain the sound pressure

levels, which, in each preferred third octave band in the range of 50 Hz to

10,000 Hz, are the averages of the energy of the sound pressure levels in

every preferred third octave. When analyzed in PNL, the resulting mean

background noise level must be at least 20 PNdB below the maximum PNL of the

aircraft.

(4) Corrections for recorded levels of background noise are allowed, within

the limits prescribed in Sec. A36.5(d)(3) of this appendix.

Section A36.5 Reporting and correcting measured data.

(a) General. Data representing physical measurements, or corrections to

measured data, including corrections to measurements for equipment response

deviations, must be recorded in permanent form and appended to the record.

Each correction must be reported and is subject to FAA approval. An estimate

must be made of each individual error inherent in each of the operations

employed in obtaining the final data.

(b) Data reporting. (1) Measured and corrected sound pressure levels must

be presented in one-third octave band levels obtained with equipment

conforming to the standards prescribed in section A36.3 of this appendix.

(2) The type of equipment used for measurement and analysis of all

acoustics, aircraft performance, and meteorological data must be reported.

(3) The atmospheric environmental data required to demonstrate compliance

with section A36.1(c) of this appendix, measured throughout the test period

under section A36.9(b)(3) of this appendix, must be reported.

(4) Conditions of local topography, ground cover, or events which may

interfere with sound recording must be reported.

(5) The following aircraft information must be reported:

(i) Type, model, and serial numbers (if any) of aircraft engines.

(ii) Gross dimensions of aircraft and location of engines.

(iii) Aircraft gross weight for each test run.

(iv) Aircraft configuration, including flap and landing gear positions.

(v) Airspeed in knots.

(vi) Engine performance parameters relevant to noise generation, such as

net thrust, engine pressure ratio, exhaust temperatures, and fan or

compressor rotational speeds.

(vii) Aircraft flight path (above ground level in feet) determined by an

FAA approved method which is independent of normal flight instrumentation,

such as radar tracking, theodolite triangulation, laser trajectography, or

photographic scaling techniques.

(6) Aircraft speed and position, and engine performance parameters must be

recorded at an approved sampling rate sufficient to correct to the noise

certification reference conditions prescribed in paragraph (c) of this

section. Lateral position relative to the extended centerline of the runway,

configuration, and gross weight must be reported.

(c) Noise certification reference conditions. (1) Meteorological

conditions. Aircraft position and performance data and the noise measurements

must be corrected to the following homogeneous noise certification reference

atmospheric conditions:

(i) Sea level pressure of 2116 psf (76 cm mercury).

(ii) Ambient temperature of 77 degrees F (25 degrees C).

(iii) Relative humidity of 70 percent.

(iv) Zero wind.

(2) Aircraft conditions. The reference condition for takeoff is the maximum

weight, except as provided in Sec. 36.1581(b) of this part. The reference

conditions for approach tests consist of--

(i) Maximum landing weight, except as provided in Sec. 36.1581(d) of this

part;

(ii) Approach angle of 3 degrees; and

(iii) Aircraft height of 394 feet above the ground at the noise measuring

station.

(d) Data corrections. (1) Aircraft position and performance data and the

noise measurement must be corrected to the noise certification reference

conditions as prescribed in paragraph (c) of this section. The measured

atmospheric conditions must be those obtained in accordance with section

A36.1(c) of this appendix and paragraph (b)(3) of this section. Atmospheric

attenuation sound corrections must be made under section A36.9 of this

appendix.

(2) The measured flight path must be corrected by an amount equal to the

difference between the applicants predicted flight path for the certification

reference conditions and the measured flight path at the test conditions.

Necessary corrections relating to aircraft flight path or performance may be

derived from approved data other than certification test data. The source

noise must be corrected from approved data for the difference between

measured and reference engine conditions, together with appropriate

allowances for sound attenuation with distance. The Effective Perceived Noise

Level (EPNL) correction must be less than 2.0 EPNdB for any combination of

the following:

(i) The aircraft's not passing vertically above the measuring station.

(ii) Any difference between 394 feet and the actual minimum distance of the

aircraft's ILS antenna from the approach measuring station.

(iii) Any difference between the actual approach angle and the noise

certification reference approach flight path.

(iv) Any correction of the measured noise levels which accounts for any

difference between the test engine thrust or power and the reference engine

thrust or power.

Detailed correction requirements are prescribed in section A36.11 of this

appendix.

(3) Aircraft sound pressure levels within the 10 dB-down points (described

in section B36.9 of Appendix B) must exceed the mean background sound

pressure levels determined under section A36.3(f)(3) by at least 3 dB in each

one-third octave band (or be corrected under an FAA approved method) to be

included in the computation of the overall noise level of the aircraft. An

EPNL may not be computed or reported from data from which more than four one-

third octave bands in any spectrum within the 10 dB-down points have been

excluded under this paragraph.

(4) Where more than seven one-third octaves are within 3 dB of the ambient

noise levels, a time/frequency interpolation of the noise data shall be

performed using an approved procedure.

(5) If equivalent test procedures, different from the reference procedures,

are used, the test procedures and all methods for adjusting the results to

the reference procedures must be approved by the FAA. The amounts of

adjustments must not exceed 16 EPNdB on takeoff and 8 EPNdB on approach, and

if the adjustments are more than 8 EPNdB and 4 EPNdB respectively, the

resulting numbers must not be within 2 EPNdB of the appropriate Appendix C

noise levels including tradeoffs.

(e) Validity of results. (1) The test results must produce three mean EPNL

values within the 90 percent confidence limits, each value consisting of the

arithmetic mean of the corrected noise measurements for all valid test runs

at the takeoff, approach, and sideline measuring stations, respectively. If

more than one noise measurement system is used at any single measuring

station, the resulting data for each test run (after correction) must be

averaged as a single measurement. If more than one test site or noise

measuring station location is used, each valid test run must be included in

the computation of the mean EPNL values and their confidence limits.

(2) The minimum sample size acceptable for each of the three certification

measurements (takeoff, approaches, and sideline) is six. The number of

samples must be large enough to establish statistically for each of the three

mean noise certification levels a 90 percent confidence limit which does not

exceed +/-1.5 EPNdB. No test result may be omitted from the averaging

process, unless otherwise specified by the FAA.

(3) The mean EPNL values and their 90 percent confidence limits obtained by

the procedure described in this paragraph must be those by which the noise

emission of the aircraft is assessed against the noise certification

criteria, and must be reported.

(4) If equivalent procedures are to be used to certificate several airplane

configurations of the same type from noise tests of a single airplane, the

test procedures and analysis methods must be approved by the FAA. The request

for approval must identify the noise measurement test procedures and data

base, the airplane configurations, procedures and analysis methods, the

method for establishing the 90 percent confidence limit for each noise

certification level, and the proposed equivalent procedures.

Section A36.7  Symbols and units.

(a) General. The symbols used in Appendixes A and B of this part have the

following meanings.

Symbol                     Unit                    Meaning

ant                                                 Antilogarithm to the

Base 10.

C(k)                      dB                        Tone Correction. The

factor to be added to

PLN(k) to account for

the presence of

spectral irregularities

such as tones at the k-

th increment of time.

d                         Sec                       Duration Time. The

length of the

significant noise time

history being the time

interval between the

limits of t(1) and t(2)

to the nearest second.

D                         dB                        Duration Correction. The

factor to be added to

PNLM to account for the

duration of the noise.

EPNL                      EPNdB                     Effective Perceived

Noise Level. The value

of PNL adjusted for

both the presence or

discrete frequencies

and the time history.

(The unit EPNdB is used

instead of the unit

dB.)

f(i) or fi                Hz                        Frequency. The

geometrical mean

frequency for the i-th

one-third octave band.

F(i,k)                    dB                        Delta-dB. The difference

between the original

and background sound

pressure levels in the

i-th one-third octave

band at the k-th

interval of time.

h                         dB                        dB-Down. The level to be

subtracted from PNLTM

that defines the

duration of the noise.

H                         %                         Relative Humidity. The

ambient atmospheric

relative humidity.

(i) or i                                            Frequency Band Index.

The numerical indicator

that denotes any one of

the 24 one-third octave

bands with geometrical

mean frequencies from

50 to 10,000 Hz.

(k)                                                 Time Increment Index.

The numerical indicator

that denotes the number

of equal time

increments that have

elapsed from a

reference zero.

log                                                 Logarithm to the Base

10.

log n (a)                                           Noy discontinuity

Coordinate. The log n

value of the

intersection point of

the straight lines

representing the

variation of SPL with

log n.

M(b), M(c)                                          Noy Inverse Slope. The

reciprocals of the

slopes of the straight

lines representing the

variation of SPL with

log n.

n                         noy                       Perceived Noisiness. The

perceived noisiness at

any instant of time

that occurs in a

specified frequency

range.

n(i, k)                   noy                       Perceived Noisiness. The

perceived noisiness at

the k-th instant of

time that occurs in the

i-th one-third octave

band.

n(k)                      noy                       Maximum Perceived

Noisiness. The maximum

value of all of the 24

values of n(i) that

occurs at the k-th

instant of time.

N(k)                      noy                       Total Perceived

Noisiness. The total

perceived noisiness at

the k-th instant of

time calculated from

the 24-instantaneous

values of n(i, k).

p(b), p(c)                                          Noy Slope. The slopes of

the straight lines

representing the

variation of SPL with

log n.

PNL                       PNdB                      Perceived Noise Level.

The perceived noise

level at any instant of

time (the unit PNdB is

used instead of the

unit dB).

PNL(k)                    PNdB                      Perceived Noise Level.

The perceived noise

level calculated from

the 24 values of SPL

(i, k) at the k-th

increment of time. (The

unit PNdB is used

instead of the unit

dB.)

PNLM                      PNdB                      Maximum Perceived Noise

Level. The maximum

value of PNL(k) that

occurs during the

aircraft flyover. (The

unit PNdB is used

instead of the unit

dB.)

PNLT                      PNdB                      Tone Corrected Perceived

Noise Level. The value

of PNL adjusted for the

presence of spectral

irregularities

(discrete frequencies)

at any instant of time.

(The unit PNdB is used

instead of the unit

dB.)

PNLT(k)                   PNdB                      Tone Corrected Perceived

Noise Level. The value

of PNL(k) adjusted for

the presence of

discrete frequencies

that occurs at the k-th

increment of time. (The

unit PNdB is used

instead of the unit

dB.)

PNLTM                     PNdB                      Maximum tone Corrected

Perceived Noise Level.

The maximum value of

PNLT(k) that occurs

during the aircraft

flyover. (The unit PNdB

is used instead of the

unit dB.)

s(i, k)                   dB                        Slope of Sound Pressure

Level. The change in

level between adjacent

one-third octave band

sound pressure levels

at the i-th band for

the k-th instant of

time.

<Delta>s(i, k)            dB                        Change in Slope of Sound

Pressure Level.

s'(i, k)                  dB                        Adjusted Slope of Sound

Pressure Level. The

change in level between

adjacent adjusted one-

third octave band sound

pressure levels at the

i-th band for the k-th

instant of time.

s(i, k)                   dB                        Average Slope of Sound

Pressure Level.

SPL                       dB re 0.0002 microbar     Sound Pressure Level.

The sound pressure

level at any instant of

time that occurs in a

specified frequency

range.

SPL(a)                    dB re 0.002 microbar      Noy Discontinuity

Coordinate. The SPL

value of the

intersection point of

the straight lines

representing the

variation of SPL with

log n.

SPL(b)                    dB re 0.002 microbar      Noy Intercept. The

SPL(c)                                              intercepts on the SPL-

axis of the straight

lines representing the

variation of SPL with

log n.

SPL(l, k)                 dB re 0.002 microbar      Sound Pressure Level.

The sound pressure

level at the k-th

instant of time that

occurs in the i-th one-

third octave band.

SPL'(l, k)                dB re 0.002 microbar      Adjusted Sound Pressure

Level. The first

approximation to

background level in the

i-th one-third octave

band for the k-th

instant of time.

SPL''(l, k)               dB re 0.002 microbar      Background Sound

Pressure Level. The

final approximation to

background level in the

i-th one-third octave

band for the k-th

instant of time.

SPLi                      dB re 0.002 microbar      Maximum Sound Pressure

Level. The sound

pressure level that

occurs in the i-th one-

third octave band of

the spectrum for PNL-

TM.

SPLic                     dB re 0.002 microbar      Corrected Maximum Sound

Pressure Level. The

sound pressure level

that occurs in the i-th

one-third octave band

of the spectrum for

PNLTM corrected for

atmospheric sound

absorption.

t                         Sec                       Elapsed Time. The length

of time measured from a

reference zero.

t(1), t(2)                Sec                       Time Limit. The

beginning and end of

the significant noise

time history defined by

h.

<Delta>t                  Sec                       Time Increment. The

equal increments of

time for which PNL(k)

and PNLT (k) are

calculated.

T                         Sec                       Normalizing Time

Constant. The length of

time used as a

reference in the

integration method for

computing duration

corrections.

T                         deg.F                     Temperature. The ambient

atmospheric temperature

<alpha>i                  dB/ft                     Test Atmospheric

<alpha>i'                 dB/1000 ft                Absorption. The

atmospheric attenuation

of sound that occurs in

the i-th one-third

octave band for the

measured atmospheric

temperature and

relative humidity.

<alpha>io                 dB/ft                     Reference Atmospheric

<alpha>io'                dB/1000 ft                Absorption. The

atmospheric attenuation

of sound that occurs in

the i-th one-third

octave band for the

reference atmospheric

temperature and

relative humidity.

<beta>                    Degrees                   First Constant Climb

Angle.

<psi>                     Degrees                   Second Constant Climb

Angle.

<delta>                   Degrees                   Thrust Cutback Angles.

<epsilon>                 Degrees                   The angles defining the

points on the takeoff

flight path at which

thrust reduction is

started and ended

respectively.

<eta>                     Degrees                   Approach Angle.

<theta>                   Degrees                   Takeoff Noise Angle. The

angle between the

flight path and noise

path for takeoff

operation. It is

identical for both

measured and corrected

flight paths.

<mu>                      Degrees                   Approach Noise Angle.

The angle between the

flight path and the

noise path for approach

operation. It is

identical for both

measured and corrected

flight paths.

<Delta>l                  EPNdB                     PNLT Correction. The

correction to be added

to the EPNL calculated

from measured data to

account for noise level

changes due to

differences in

atmospheric absorption

and noise path length

between reference and

test conditions.

<Delta>2                  EPNdB                     Noise Path Duration

Correction. The

correction to be added

to the EPNL calculated

from measured data to

account for noise level

changes due to the

noise duration because

of differences in

flyover altitude

between reference and

test condition.

<Delta>3                  EPNdB                     Weight Correction. The

correction to be added

to the EPNL calculated

from measured data to

account for noise level

changes due to

differences between

maximum and test

aircraft weights.

<Delta>4                  EPNdB                     Approach Angle

Correction. The

correction to be added

to the EPNL calculated

from measured data to

account for noise level

changes due to

differences between 3

deg. and the test

approach angle.

<Delta>AB                 Feet                      (/1/ )

<Delta><beta>             Degrees                   (/1/ )

<Delta><gamma>            Degrees                   (/1/ )

<Delta><delta>            Degrees                   (/1/ )

<Delta><epsilon>          Degrees                   (/1/ )

<Delta>e                  Degrees                   (/1/ )

/1/ Takeoff Profile Changes. The changes in the basic parameters defining

the takeoff profile due to differences between reference and test

conditions.

Flight Profile Identification Positions

Position                              Description

A         Start of takeoff roll.

B         Liftoff.

C         Start of first constant climb.

D         Start of thrust reduction.

E         Start of second constant climb.

Ec        Start of second constant climb on corrected flight path.

F         End of noise certification takeoff flight path.

Fc        End of second constant climb on corrected flight path.

G         Start of noise certification approach flight path.

Gr        Start of noise certification approach on reference flight path.

H         Position on approach path directly above noise measuring station.

I         Start of level off.

Ir        Start of level off on reference approach flight path.

J         Touchdown.

K         Takeoff noise measuring station.

L         Sideline noise measuring station (not on flight track).

M         End of noise type certification takeoff flight track.

N         Approach noise measuring station.

O         Threshold of approach end of runway.

P         Start of noise type certification approach flight track.

Q         Position on measured takeoff flight path corresponding to PNLTM at

station K.

Qc        Position on corrected takeoff flight path corresponding to PNLTM at

station K.

R         Position on measured takeoff flight path nearest to station K.

Rc        Position on corrected takeoff flight path nearest to station K.

S         Position on measured approach flight path corresponding to PNLTM at

station N.

Sr        Position on reference approach flight path corresponding to PNLTM

at station N.

T         Position on measured approach flight path nearest to station N.

Tr        Position on reference approach flight path nearest to station N.

X         Position on measured takeoff flight path corresponding to PNLTM at

station L.

Xc        Position on corrected takeoff flight path corresponding to PNLTM at

station L.

Flight Profile Distances

Distance  Unit                             Meaning

AB        feet  Length of Takeoff Roll. The distance along the runway between

the start of takeoff roll and lift off.

AK        feet  Takeoff Measurement Distance. The distance from the start of

roll to the takeoff noise measurement station along the

extended centerline of the runway.

AM        feet  Takeoff Flight Track Distance. The distance from the start of

roll to the takeoff flight track position along the extended

centerline of the runway for which the position of the

aircraft need no longer be recorded.

KQ        feet  Measured Takeoff Noise Path. The distance from station K to

the measured aircraft position Q.

KQc       feet  Corrected Takeoff Noise Path. The distance from station K to

the corrected aircraft position Qc.

KR        feet  Measured Takeoff Minimum Distance. The distance from station

K to point R on the measured flight path.

KRc       feet  Corrected Takeoff Minimum Distance. The distance from station

K to point Rc on the corrected flight path.

LX        feet  Measured Sideline Noise Path. The distance from station L to

the measured aircraft position X.

LXc       feet  Corrected Sideline Noise Path. The distance from station L to

the corrected aircraft position Xc.

NH        feet  Aircraft Approach Height. The vertical distance between the

aircraft and the approach measuring station.

NS        feet  Measured Approach Noise Path. The distance from station N to

the measured aircraft position S.

NSr       feet  Reference Approach Noise Path. The distance from station N to

the reference aircraft position Sr.

NT        feet  Measured Approach Minimum Distance. The distance from station

N to point T on the measured flight path.

NTr       feet  Reference Approach Minimum Distance. The distance from

station N to point Tr on the corrected flight path; it

equals 393 feet.

ON        feet  Approach Measurement Distance. The distance from the runway

threshold to the approach measurement station along the

extended centerline of the runway.

OP        feet  Approach Flight Track Distance. The distance from the runway

threshold to the approach flight track position along the

extended centerline of the runway for which the position of

the aircraft need no longer be recorded.

Section A36.9  Atmospheric attenuation of sound.

(a) General. The measured values of the one-third octave band spectra must

conform, or be corrected, to the reference-day conditions listed in section

A36.5(c) of this appendix. Each correction must account for any differences

in the atmospheric attenuation of sound between the test-day conditions and

the reference-day conditions along the sound propagation path between the

aircraft and the microphone. Unless the meteorological conditions conform to

those prescribed in section A36.1(c) of this appendix, the test data are not

acceptable.

(b) Meteorological measurements. (1) The wind velocity, temperature and

relative humidity measurements required under this part must be measured in

the vicinity of the noise measuring stations. The location of the

meteorological measurements must be approved by the FAA as representative of

those atmospheric conditions existing near the surface over the geographical

area in which aircraft noise measurements are made. In some cases, a fixed

meteorological station (such as those found at airports or other facilities)

may meet this requirement.

(2) The temperature and relative humidity must be measured from a point 10

meters above the surface at the measuring stations to the altitude of the

aircraft, using previously approved equipment and methods.

(3) Meteorological measurements must be obtained within 25 minutes of each

noise test measurement. Meteorological data must be interpolated to actual

times of each noise measurement.

(c) Attenuation rates. The atmospheric attenuation rates of sound with

distance for each one-third octave band from 50Hz to 10,000 Hz must be

determined in accordance with the formulations and tabulations of SAE ARP

866A, entitled "Standard Values of Atmospheric Absorption as a Function of

Temperatures and Humidity for Use in Evaluating Aircraft Flyover Noise" (as

incorporated by reference under Sec. 36.6 of this part).

(d) Correction for atmospheric attenuation. (1) EPNL values calculated for

measured data must be corrected by the methods prescribed in section

A36.11(d) of this appendix whenever--

(i) The ambient atmospheric conditions of temperature and relative humidity

do not conform to the reference conditions (77 degrees F. and 70 percent,

respectively), or

(ii) The measured takeoff and approach flight paths do not conform to the

reference flight paths.

(2) If the atmospheric absorption coefficients do not vary over the PNLTM

sound propagation path by more than +/- 1.6 dB/1000 ft (+/- 0.5 dB/100

meters) in the 3150 Hz one-third octave band from the value of the absorption

coefficient derived from the meteorological measurement obtained at 10 meters

above the surface, the mean of the values of the atmospheric absorption

coefficients at 10 meters above the surface and at the altitude of the

aircraft at PNLTM may be used to determine the atmospheric attenuation rates

for each one-third octave band. The resulting atmospheric attenuation rate

may be used to compute the PNLTM correction under section A36.11(d) of this

appendix.

(3) If the conditions do not conform to those prescribed in paragraph

(d)(2) of this section, the corrections for atmospheric attenuation must be

determined by the following layered-atmosphere procedure:

(i) The sound propagation path must be divided into increments no greater

than 100 feet in altitude, and the average temperature and relative humidity

that exists within each increment at the time of the test must be calculated

from the meteorological data required under paragraph (b) of this section.

(ii) Atmospheric attenuation rates must be determined under paragraph (c)

of this section for each one-third octave band in each altitude increment.

(iii) The mean attenuation rate over the complete sound propagation path

from the aircraft to the microphone must be computed for each one-third

octave band from 50 Hz to 10,000 Hz. These rates must be used in computing

the corrections required in section A36.11(d) of this appendix.

Section A36.11  Detailed correction procedures.

(a) General. If the test conditions do not conform to those prescribed as

noise certification reference conditions under section A36.5 of this

appendix, the following correction procedure and requirements apply:

(1) If a positive value results from any difference between reference and

test conditions, and appropriate positive correction must be made to the EPNL

calculated from the measured data. Conditions which can result in a positive

value include:

(i) Atmospheric absorption of sound under test conditions which is greater

than the reference;

(ii) Test flight path at an altitude which is higher than the reference; or

(iii) Test weight which is less than maximum certification weight.

(2) If a negative value results from any difference between reference and

test conditions, no correction may be made to the EPNL calculated from the

measured data, unless the difference results from:

(i) An atmospheric absorption of sound under test conditions which is less

than the reference; or

(ii) A test flight path at an altitude which is lower than the reference.

(3) The following correction procedures may produce one or more possible

correction values which must be added algebraically to the EPNL calculated as

if the tests were conducted completely under the noise certification

reference conditions:

(i) The flight profiles must be determined for both takeoff and approach,

and for both reference and test conditions. The procedures require noise and

flight path recording with a synchronized time signal from which the test

profile can be delineated, including the aircraft position for which PNLTM is

observed at the noise measuring station. For takeoff, the flight profile

corrected to reference conditions may be derived from FAA approved

manufacturer's data; however, for approach, the reference profile is

prescribed under paragraph (c)(2) of this section.

(ii) The sound propagation paths to the microphone from the aircraft

position corresponding to PNLTM are determined for both the test and

reference profiles. The SPL values in the spectrum of PNLTM must then be

corrected for the effects of--

(A) Change in atmospheric sound absorption;

(B) Atmospheric sound absorption on the change in sound propagation path

length; and

(C) Inverse square law on the change in sound propagation path length. The

corrected values of SPL are then converted to PNLT from which must be

subtracted PNLTM. The resulting difference represents the correction which

must be added algebraically to the EPNL calculated from the measured data.

(iii) The minimum distances from both the test and reference profiles to

the noise measuring station must be calculated and used to determine a noise

duration correction due to any change in the altitude of aircraft flyover.

The duration correction must be added algebraically to the EPNL calculated

from the measured data.

(iv) From approved data in the form of curves or tables giving the

variation of EPNL with engine thrust or test speed, corrections are

determined and must be added to the EPNL (which is calculated from the

measured data) to account for noise level changes due to differences between

test conditions and reference conditions.

(v) From approved data corrections are determined and must be added

algebraically to the EPNL (which is calculated from measured data) to account

for noise level changes due to differences between 3 degrees and the test

approach angle.

(b) Takeoff profiles. (1) Figure A1 illustrates a typical takeoff profile.

(i) The aircraft begins the takeoff roll at point A, lifts off at point B,

and initiates the first constant climb at point C at an angle <beta>. The

noise abatement thrust cutback is started at point D and completed at point E

where the second constant climb is defined by the angle <gamma> (usually

expressed in terms of the gradient in percent). The end of the noise

certification takeoff flight path is represented by aircraft position F whose

vertical projection on the flight track (extended centerline of the runway)

is point M. The position of the aircraft must be recorded for the entire

interval during which the measured aircraft noise level is within 10 dB of

PNLTM. Position K is the takeoff noise measuring station whose distance AK is

specified as 21,325 feet (6,500 meters). However, if it is necessary to

reduce AK to less than 21,325 feet, the procedures prescribed in paragraph

(f) of this section must be followed. Position L is the sideline noise

measuring station located on a line parallel to, and the prescribed distance

from, the runway centerline where the noise level during takeoff is greatest.

(ii) The takeoff profile is defined by five parameters--(A) AB, the length

of takeoff roll; (B) <beta> the first constant climb angle; (C) <gamma>, the

second constant climb angle; and (D) >>, and e, the thrust cutback angles.

These five parameters are functions of the aircraft performance and weight,

and the atmospheric conditions of temperature, pressure, and wind velocity

and direction.

(2) If the test conditions do not conform to those prescribed as reference

conditions under section A36.5 of this appendix, the corresponding test and

reference profile parameters will be different, as shown in Figure A2. The

profile parameter changes, identifies as <Delta>AB, <Delta><beta>,

<Delta><gamma>, <Delta>>>, and <Delta><epsilon> may be derived from the

manufacturer's data (if approved by the FAA) and may be used to define the

fight profile corrected to the reference conditions. The relationships

between the measured and corrected takeoff flight profiles may then be used

to determine the corrections, which, if positive, must be applied to the EPNL

calculated from the measured data.

Figure A1 -- Measured Takeoff Profile

Figure A2 -- Comparison of Measured and Corrected

Takeoff Profiles

[ ...Illustrations appear here... ]

Note: Under reference atmospheric conditions and with maximum takeoff

weight, the gradient of the second constant climb angle (<gamma>) may not be

less than 4 percent. However, the actual gradient will depend upon the test

atmospheric conditions, assuming maximum takeoff weight and the parameters

characterizing engine performance are constant (rpm, epr, or any other

parameter used by the pilot).

(3) Figure A3 illustrates portions of the measured and corrected takeoff

flight paths including the significant geometrical relationships influencing

sound propagation. EF represents the measured second constant flight path

with climb angle <gamma>, and EcFc represents the corrected second constant

flight path at reduced climb angle <gamma>-<Delta><gamma>. Position Q

represents the aircraft location on the measured takeoff flight path for

which PNLTM is observed at the noise measuring station K, and Qc is the

corresponding position on the corrected flight path. The measured and

corrected sound propagation paths are KQ and KQc, respectively, which form

the same angle <theta> with their flight paths. Position R represents the

point on the measured takeoff flight path nearest the noise measuring station

K, and Rc is the corresponding position on the corrected flight path. The

minimum distance to the measured and corrected flight paths are indicated by

the lines KR and KRc, respectively, which are normal to their flight paths.

Figure A3 -- Takeoff Profile Characteristics Influencing

Sound Propagation

Figure A4 -- Measured Approach Profile

[ ...Illustrations appear here... ]

(c) Approach profiles. (1) Figure A4 illustrates a typical approach

profile.

(i) The beginning of the noise certification approach profile is

represented by aircraft position G whose vertical projection on the flight

track (extended centerline of the runway) is point P. The position of the

aircraft should be recorded for a distance OP from the runway threshold O to

ensure recording of the entire interval during which the measured aircraft

noise is within 10 dB of PNLTM.

(ii) The aircraft approaches at an angle passes vertically over the noise

measuring station N at a height of NH, begins the level off at position I,

and touches down at position J. The distance ON is prescribed as 6,562 feet

(2,000 meters).

(iii) The approach profile is defined by the approach angle    and the

height NH which are functions of the aircraft operating conditions controlled

by the pilot. If the measured approach profile parameters do not conform to

the corresponding reference approach parameters (3 degrees and 394 feet,

respectively, as shown in Figure A5), corrections, if positive, must be

applied to the EPNL calculated from the measured data.

Figure A5 -- Comparison of Measured and Corrected

Approach Profiles

Figure A6 -- Approach Profile Characteristics Influencing

Sound Propagation

[ ...Illustrations appear here... ]

(2) Figure A6 illustrates portions of the measured and reference approach

flight paths, including the significant geometrical relationships influencing

sound propagation. GI represents the measured approach path with approach

angle --, and GrIr represents the reference approach flight path at lower

altitude and approach angle of 3 degrees. Position S represents the aircraft

location on the measured approach flight path for which PNLTM is observed at

the noise measuring station N, and Sr is the corresponding position on the

reference approach flight path. The measured and corrected sound propagation

paths are NS and NSr, respectively, which form the same angle l with their

flight paths. Position T represents the point on the measured approach flight

path nearest the noise measuring station N, and Tr is the corresponding point

on the reference approach flight path. The minimum distances to the measured

and reference flight paths are indicated by the lines NT and NTr,

respectively, which are normal to their flight paths. NOTE: The reference

approach flight path is defined by --=3 degrees and NH=394 feet. Consequently

NTr can also be defined; NTr=393 feet to the nearest foot and is, therefore,

considered to be one of the reference parameters.

(d) PNLT corrections. If the ambient atmospheric conditions of temperature

and relative humidity are not those prescribed as reference conditions under

Sec. A36.5(c) of this appendix (77 degrees F and 70 percent, respectively),

corrections to the EPNL values must be calculated from the measured data

under paragraph (a) of this section as follows:

(1) Takeoff flight path. For the takeoff flight path shown in Figure A3,

the spectrum of PNLTM observed at station K for the aircraft at position Q is

decomposed into its individual SPLi values.

(i) Step 1. A set of corrected values are then computed as follows:

SPLic=SPLi                          +]]io (KQ-KQc)

+(]]i-]]io) KQ    +20 log (KQ/KQc)

where SPLi and SPLic are the measured and corrected sound pressure levels,

respectively, in the i-th one-third octave band. The first correction term

accounts for the effects of change in atmospheric sound absorption where

<alpha>i and ]]io are the sound absorption coefficients for the test

(determined under section A36.9(d)) and reference atmospheric conditions,

respectively, for the i-th one-third octave band and KQ is the measured

takeoff sound propagation path. The second correction term accounts for the

effects of atmospheric sound absorption on the change in the sound

propagation path length where KQc is the corrected takeoff sound propagation

path. The third correction term accounts for the effects of the inverse

square law on the change in the sound propagation path length.

(ii) Step 2. The corrected values of SPLic are then converted to PNLT and a

correction term calculated as follows:

<Delta>1=PNLT-PNLTM

which represents the correction to be added algebraically to the EPNL

calculated from the measured data.

(2) Approach flight path.

(i) The procedure prescribed in paragraph (d)(1) of this section for

takeoff flight paths is also used for the approach flight path, except that

the value for SPLic relate to the approach sound propagation paths shown in

Figure A6 as follows:

SPLic=SPLi                +(<alpha>i-<alpha>io) NS

+<alpha>io (NS-NSr)

+20 log (NS/NSr)

where NS and NSr are the measured and reference approach sound propagation

paths, respectively.

(ii) The remainder of the procedure is the same as that prescribed in

paragraph (d)(1)(ii) of this section, regarding takeoff flight path.

(3) Sideline flight path. The procedure prescribed in paragraph (d)(1) of

this section for takeoff flight paths is also used for the sideline flight

path, except that the values of SPLic relate only to the measured sideline

sound propagation path as follows:

SPLic=SPLi                +(<alpha>i-<alpha>io) LX

+<alpha>io (LX-LXc)

+20 log (LX/LXc)

where LX is the measured sideline sound propagation path from station L

(Figure A1) to position X of the aircraft for which PNLTM is observed at

station L and LXc is the corrected sideline sound propagation path.

(e) Duration corrections. If the measured takeoff and approach flight paths

do not conform to those prescribed as the corrected and reference flight

paths, under section A36.11 (b) and (c) respectively, it will be necessary to

apply duration corrections to the EPNL values calculated from the measured

data. Such corrections must be calculated as follows:

(1) Takeoff flight path. For the takeoff flight path shown in Figure A3,

the correction term is calculated using the formula--

D2 = -7.5 log (KR/KRc)

which represents the correction which must be added algebraically to the EPNL

calculated from the measured data. The lengths KR and KRc are the measured

and corrected takeoff minimum distances from the noise measuring station K to

the measured and the corrected flight paths respectively. A negative sign

indicates that, for the particular case of a duration correction, the EPNL

calculated from the measured data must be reduced if the measured flight path

is at a greater altitude than the corrected flight path.

(2) Approach flight path. For the approach flight path shown in Figure A6,

the correction term is calculated using the formula--

D 2 = -7.5 log (NT/393)

where NT is the measured approach minimum distance from the noise measuring

station N to the measured flight path and 393 feet is the minimum distance

from station N to the reference flight path.

(3) Sideline flight path. For the sideline flight path, the correction term

is calculated using the formula--

D 2 = -7.5 log (LX/LXc)

where LX and LXc are the measured and corrected sideline noise measuring

distances, respectively, from the noise measuring station L to the aircraft

position X or Xc, respectively on the takeoff flight path.

(f) Nonstandard location correction. When takeoff and approach noise

measurements are conducted at points other than those prescribed in section

C36.1 of Appendix C, the EPNL value computed from these measurements must be

corrected to the value that would have occurred at the prescribed measuring

points under one of the following procedures:

(1) Simplified procedure. Unless the amount of adjustment exceeds 8 dB on

takeoff or 4 dB on approach, or the correction results in a final EPNL value

which is within 1.0 dB of the noise levels prescribed in Appendix C of this

part, the correction procedures prescribed in paragraphs (d) and (e) of this

section may be used. Since this procedure accounts for extrapolation of PNLTM

from the close-in measurement station to the prescribed measuring point, the

remaining corrections for differences between test and reference conditions,

including thrust and airspeed, must be made afterward.

(2) Integrated procedure. If the correction factor exceeds 8 dB on takeoff

or 4 dB on approach, or the correction results in a final EPNL value which is

within 1.0 dB of the noise levels prescribed in Appendix C of this part, the

following correction procedure must be used:

(i) Each 1/2  second spectrum measured during a flyover at a noise

measuring station which is closer to the flight path than the prescribed

reference distance must be adjusted under a procedure similar to that

prescribed under paragraph (d)(1) of this section, regarding PNLT

corrections. However, the distances which must be used are those values of KQ

and KQc for the sound propagation path (and hence value of <theta>) for PNLTM

which represents the actual, measured sound propagation path (and path

angle), and the corresponding sound propagation path (and path angle) as if

the measurements had been made at the 21,325 foot measuring point under

reference acoustic-day conditions.

(ii) After the measured one half (1/2) second spectra have been corrected

to the measuring points prescribed in section C36.1 of Appendix C, the

remaining noise evaluation must be conducted under the procedures prescribed

in Appendix B of this part, including the appropriate reference thrust and

air speed corrections.

[Amdt. 36-9, 43 FR 8739, Mar. 2, 1978, as amended at 44 FR 3031, Jan. 15,

1979; Amdt. 36-15, 53 FR 16367, May 6, 1988; 53 FR 18835, May 25, 1988; 53 FR

51087, Dec. 19, 1988]

Appendix B--Aircraft Noise Evaluation Under Sec. 36.103

Sec.

B36.1  General.

B36.3  Perceived noise level.

B36.5  Correction for spectral irregularities.

B36.7  Maximum tone corrected perceived noise level.

B36.9  Duration correction.

B36.11  Effective perceived noise level.

B36.13  Mathematical formulation of noy tables.

Section B36.1 General. The procedures in this appendix must be used to

determine the noise evaluation quantity designated as effective perceived

noise level, EPNL, under Secs. 36.103 and 36.803. These procedures, which use

the physical properties of noise measured as prescribed by Appendix A of this

part, consist of the following:

(a) The 24 one-third octave bands of sound pressure level are converted to

perceived noisiness by means of a noy table. The noy values are combined and

then converted to instantaneous perceived noise levels, PNL(k).

(b) A tone correction factor, C(k), is calculated for each spectrum to

account for the subjective response to the presence of the maximum tone.

(c) The tone correction factor is added to the perceived noise level to

obtain tone corrected perceived noise levels, PNLT(k), at each one-half

second increment of time. The instantaneous values of tone corrected

perceived noise level are noted with respect to time and the maximum value,

PNLTM, is determined.

PNLT(k)=PNL(k)+C(k)

(d) A duration correction factor, D, is computed by integration under the

curve of tone corrected perceived noise level versus time.

(e) Effective perceived noise level, EPNL, is determined by the algebraic

sum of the maximum tone corrected perceived noise level and the duration

correction factor.

EPNL=PNLTM+D

Section B36.3 Perceived noise level. Instantaneous perceived noise levels,

PNL(k), must be calculated from instantaneous one-third octave band sound

pressure levels, SPL(i,k), as follows:

(a) Step 1. Convert each one-third octave band SPL(i,k), from 50 to 10,000

Hz, to perceived noisiness, n(i,k), by reference to Table B1, or to the

mathematical formulation of the noy table given in Sec. B36.13 of this

appendix.

(b) Step 2. Combine the perceived noisiness values, n(i,k), found in step 1

by the following formula:

N(k)=n(k)+0.15 [[SUM  2  4 i-1

n(i,k)]-n (k)]=0.85

n(K)+0.15SUM 24 i-1n(i,k)

where n(k) is the largest of the 24 values of n(i,k) and N(k) is the total

perceived noisiness.

(c) Step 3. Convert the total perceived noisiness, N(k), into perceived

noise level, PNL(k), by the following formula:

PNL(k)=40.0+33.22 log N(k)

which is plotted in Figure B1. PNL(k) may also be obtained by choosing N(k)

in the 1,000 Hz column of Table B1 and then reading the corresponding value

of SPL(i,k) which, at 1,000 Hz, equals PNL(k).

Figure B1 -- Perceived Noise Level As A Function of Noys

[ ...Illustration appears here... ]

Table B1 Perceived Noisiness (NOYs) as a Function of Sound

Pressure Level

1/3  Octave

Band Center

Frequencies

in Hz (c/s)

**

SPL      50        63    80    100   125   160   200   250   315   400   500

4

5

6

7

8

9

10

11

12

12

14

15

16                                                                0.10  0.10

17                                                                0.11  0.11

18                                                          0.10  0.13  0.13

19                                                          0.11  0.14  0.14

20                                                          0.13  0.16  0.16

21                                                    0.10  0.14  0.18  0.18

22                                                    0.11  0.16  0.21  0.21

23                                                    0.13  0.18  0.24  0.24

24                                              0.10  0.14  0.21  0.27  0.27

25                                              0.11  0.16  0.24  0.30  0.30

26                                              0.13  0.18  0.27  0.33  0.33

27                                        0.10  0.14  0.21  0.30  0.35  0.35

28                                        0.11  0.16  0.24  0.33  0.38  0.38

29                                        0.13  0.18  0.27  0.35  0.41  0.41

30                                  0.10  0.14  0.21  0.30  0.38  0.45  0.45

31                                  0.11  0.16  0.24  0.33  0.41  0.49  0.49

32                                  0.13  0.18  0.27  0.36  0.45  0.53  0.53

33                                  0.14  0.21  0.30  0.39  0.49  0.57  0.57

34                            0.10  0.16  0.24  0.33  0.42  0.53  0.62  0.62

35                            0.11  0.18  0.27  0.36  0.46  0.57  0.67  0.67

36                            0.13  0.21  0.30  0.40  0.50  0.62  0.73  0.73

37                            0.15  0.24  0.33  0.43  0.55  0.67  0.79  0.79

38                            0.17  0.27  0.37  0.48  0.60  0.73  0.85  0.85

39                      0.10  0.20  0.30  0.41  0.52  0.65  0.79  0.92  0.92

40                      0.12  0.23  0.33  0.45  0.57  0.71  0.85  1.00  1.00

41                      0.14  0.26  0.37  0.50  0.63  0.77  0.92  1.07  1.07

42                      0.16  0.30  0.41  0.55  0.69  0.84  1.00  1.15  1.15

43                      0.19  0.33  0.45  0.61  0.76  0.92  1.07  1.23  1.23

44               0.10   0.22  0.37  0.50  0.67  0.83  1.00  1.15  1.32  1.32

45               0.12   0.26  0.42  0.55  0.74  0.91  1.08  1.24  1.41  1.41

46               0.14   0.30  0.46  0.61  0.82  1.00  1.16  1.33  1.52  1.52

47               0.16   0.34  0.52  0.67  0.90  1.08  1.25  1.42  1.62  1.62

48               0.19   0.38  0.58  0.74  1.00  1.17  1.34  1.53  1.74  1.74

49         0.10  0.22   0.43  0.65  0.82  1.08  1.26  1.45  1.64  1.87  1.87

50         0.12  0.26   0.49  0.72  0.90  1.17  1.36  1.56  1.76  2.00  2.00

51         0.14  0.30   0.55  0.80  1.00  1.26  1.47  1.68  1.89  2.14  2.14

52         0.17  0.34   0.62  0.90  1.08  1.36  1.58  1.80  2.03  2.30  2.30

53         0.21  0.39   0.70  1.00  1.18  1.47  1.71  1.94  2.17  2.46  2.46

54         0.25  0.45   0.79  1.09  1.28  1.50  1.85  2.09  2.33  2.64  2.64

55         0.30  0.51   0.89  1.18  1.39  1.71  2.00  2.25  2.50  2.83  2.83

56         0.34  0.59   1.00  1.29  1.50  1.85  2.15  2.42  2.69  3.03  3.03

57         0.39  0.67   1.09  1.40  1.63  2.00  2.33  2.61  2.88  3.25  3.25

58         0.45  0.77   1.18  1.53  1.77  2.15  2.51  2.81  3.10  3.48  3.48

59         0.51  0.87   1.29  1.66  1.92  2.33  2.71  3.03  3.32  3.73  3.73

60         0.59  1.00   1.40  1.81  2.08  2.51  2.93  3.26  3.57  4.00  4.00

61         0.67  1.10   1.53  1.97  2.26  2.71  3.16  3.51  3.83  4.29  4.29

62         0.77  1.21   1.66  2.15  2.45  2.93  3.41  3.79  4.11  4.59  4.59

63         0.87  1.32   1.81  2.34  2.65  3.16  3.69  4.06  4.41  4.92  4.92

64         1.00  1.45   1.97  2.54  2.88  3.41  3.98  4.39  4.73  5.28  5.28

65         1.11  1.60   2.15  2.77  3.12  3.69  4.30  4.71  5.08  5.66  5.66

66         1.22  1.75   2.34  3.01  3.39  3.99  4.64  5.07  5.45  6.06  6.06

67         1.35  1.92   2.54  3.28  3.68  4.30  5.01  5.46  5.85  6.50  6.50

68         1.49  2.11   2.77  3.57  3.99  4.64  5.41  5.88  6.27  6.96  6.96

69         1.65  2.32   3.01  3.80  4.33  5.01  5.84  6.33  6.73  7.46  7.46

70         1.82  2.59   3.28  4.23  4.69  5.41  6.31  6.81  7.23  8.00  8.00

71         2.02  2.79   3.57  4.60  5.09  5.84  6.81  7.33  7.75  8.57  8.57

72         2.23  3.07   3.88  5.01  5.52  6.31  7.36  7.90  8.32  9.19  9.19

73         2.46  3.37   4.23  5.45  5.99  6.81  7.94  8.50  8.93  9.85  9.85

74         2.72  3.70   4.60  5.94  6.50  7.36  8.57  9.15  9.59  10.6  10.6

75         3.01  4.06   5.01  6.46  7.05  7.94  9.19  9.85  10.3  11.3  11.3

76         3.32  4.46   5.45  7.03  7.65  8.57  9.85  10.6  11.0  12.1  12.1

77         3.67  4.89   5.94  7.66  8.29  9.19  10.6  11.3  11.8  13.0  13.0

78         4.06  5.37   6.46  8.33  9.00  9.85  11.3  12.1  12.7  13.9  13.9

79         4.49  5.90   7.03  9.07  9.76  10.6  12.1  13.0  13.6  14.9  14.9

80         4.96  6.48   7.66  9.85  10.6  11.3  13.0  13.9  14.6  16.0  16.0

81         5.48  7.11   8.33  10.6  11.3  12.1  13.9  14.9  15.7  17.1  17.1

82         6.06  7.81   9.07  11.3  12.1  13.0  14.9  16.0  16.9  18.4  18.4

83         6.70  8.57   9.87  12.1  13.0  13.9  16.0  17.1  18.1  19.7  19.7

84         7.41  9.41   10.7  13.0  13.9  14.9  17.1  18.4  19.4  21.1  21.1

85         8.19  10.3   11.7  13.9  14.9  16.0  18.4  19.7  20.8  22.6  22.6

86         9.05  11.3   12.7  14.9  16.0  17.1  19.7  21.1  22.4  24.3  24.3

87         10.0  12.1   13.9  16.0  17.1  18.4  21.1  22.6  24.0  26.0  26.0

88         11.1  13.0   14.9  17.1  18.4  19.7  22.6  24.3  25.8  27.9  27.9

89         12.2  13.9  16 .0  18.4  19.7  21.1  24.3  26.0  27.7  29.9  29.9

90         13.5  14.9   17.1  19.7  21.1  22.6  26.0  27.9  29.7  32.0  32.0

91         14.9  16.0   18.4  21.1  22.6  24.3  27.9  29.9  31.8  34.3  34.3

92         16.0  17.1   19.7  22.6  24.3  26.0  29.9  32.0  34.2  36.8  36.8

93         17.1  18.4   21.1  24.3  26.0  27.9  32.0  34.3  36.7  39.4  39.4

94         18.4  19.7   22.6  26.0  27.9  29.9  34.3  36.8  39.4  42.2  42.2

95         19.7  21.1   24.3  27.9  29.9  32.0  36.8  39.4  42.2  45.3  45.3

96         21.1  22.6   26.0  29.9  32.0  34.3  39.4  42.2  45.3  48.5  48.5

97         22.6  24.3   27.9  32.0  34.3  36.8  42.2  45.3  48.5  52.0  52.0

98         24.3  26.0   29.9  34.3  36.8  39.4  45.3  48.5  52.0  55.7  55.7

99         26.0  27.9   32.0  36.8  39.4  42.2  48.5  52.0  55.7  59.7  59.7

100         27.9  29.9   34.3  39.4  42.2  45.3  52.0  55.7  59.7  64.0  64.0

101         29.9  32.0   36.8  42.2  45.3  48.5  55.7  59.7  64.0  68.6  68.6

102         32.0  34.3   39.4  45.3  48.5  52.0  59.7  64.0  68.6  73.5  73.5

103         34.3  36.8   42.2  48.5  52.0  55.7  64.0  68.6  73.5  78.8  78.8

104         36.8  39.4   45.3  52.0  55.7  59.7  68.6  73.5  78.8  84.4  84.4

105         39.4  42.2   48.5  55.7  59.7  64.0  73.5  78.8  84.4  90.5  90.5

106         42.2  45.3   52.0  59.7  64.0  68.6  78.8  84.4  90.5  97.0  97.0

107         45.3  48.5   55.7  64.0  68.6  73.5  84.4  90.5  97.0   104   104

108         48.5  52.0   59.7  68.6  73.5  78.8  90.5  97.0   104   111   111

109         52.0  55.7   64.0  73.5  78.8  84.4  97.0   104   111   119   119

110         55.7  59.7   68.6  78.8  84.4  90.5   104   111   119   128   128

111         59.7  64.0   73.5  84.4  90.5  97.0   111   119   128   137   137

112         64.0  68.6   78.8  90.4  97.0   104   119   128   137   147   147

113         64.6  73.5   84.4  97.0   104   111   128   137   147   158   158

114         73.5  78.8   50.5   104   111   119   137   147   158   169   169

115         78.8  84.4   97.0   111   119   128   147   158   169   181   181

116         84.4  90.5    104   119   128   137   158   169   181   194   194

117         90.5  97.6    111   128   137   147   169   181   194   208   208

118         97.0   104    119   137   147   158   181   194   208   223   223

119          104   111    128   147   158   169   194   208   223   239   239

120          111   119    137   158   169   181   208   223   239   256   256

121          119   128    147   169   181   194   223   239   256   274   274

122          128   137    158   181   194   208   239   256   274   294   294

123          137   147    169   194   208   223   256   274   294   315   315

124          147   158    181   208   223   239   274   294   315   338   338

125          158   169    194   223   239   256   294   315   338   362   362

126          169   181    208   239   256   274   315   338   362   388   388

127          181   194    223   256   274   294   338   362   388   416   416

128          194   208    239   274   294   315   362   388   416   446   446

129          208   223    256   294   315   338   388   416   446   478   478

130          223   239    274   315   338   362   416   446   478   512   512

131          239   256    294   338   362   388   446   478   512   549   549

132          256   274    315   362   388   416   478   512   549   588   588

133          274   294    338   388   416   446   512   549   588   630   630

134          294   315    362   416   446   478   549   588   630   676   676

135          315   338    388   446   478   512   588   630   676   724   724

136          338   362    416   478   512   549   630   676   724   776   776

137          362   388    446   512   549   588   676   724   776   832   832

138          398   416    478   549   588   630   724   776   832   891   891

139          416   446    512   588   630   676   776   832   891   955   955

140          446   478    549   630   676   724   832   891   955  1024  1024

141          478   512    588   676   724   776   891   955  1024  1098  1098

142          512   549    630   724   776   832   955  1024  1098  1176  1176

143          549   588    676   776   832   891  1024  1098  1176  1261  1261

144          588   630    724   832   891   955  1098  1176  1261  1351  1351

145          630   676    776   891   955  1024  1176  1261  1351  1448  1448

146          676   724    832   955  1024  1098  1261  1351  1448  1552  1552

147          724   776    891  1024  1098  1176  1351  1448  1552  1664  1664

148          776   832    955  1098  1176  1261  1448  1552  1664  1783  1783

149          832   891   1024  1176  1261  1351  1552  1664  1783  1911  1911

150          891   955   1098  1261  1351  1448  1664  1783  1911  2048  2048

[ ...Table continues... ]

SPL  630   800   1000  1250  1600  2000  2500

4

5                                      0.10

6                                      0.11

7                                      0.13

8                                      0.14

9                                0.10  0.16

10                                0.11  0.17

11                                0.13  0.19

12                          0.10  0.14  0.22

12                          0.11  0.16  0.24

14                          0.13  0.18  0.27

15                    0.10  0.14  0.21  0.30

16  0.10  0.10  0.10  0.11  0.16  0.24  0.33

17  0.11  0.11  0.11  0.13  0.18  0.27  0.35

18  0.13  0.13  0.13  0.15  0.21  0.30  0.38

19  0.14  0.14  0.14  0.17  0.24  0.33  0.41

20  0.16  0.16  0.16  0.20  0.27  0.36  0.45

21  0.18  0.18  0.18  0.23  0.30  0.39  0.49

22  0.21  0.21  0.21  0.26  0.33  0.42  0.53

23  0.24  0.24  0.24  0.30  0.36  0.46  0.57

24  0.27  0.27  0.27  0.33  0.40  0.50  0.62

25  0.30  0.30  0.30  0.35  0.43  0.55  0.67

26  0.33  0.33  0.33  0.38  0.48  0.60  0.73

27  0.35  0.35  0.35  0.41  0.52  0.65  0.79

28  0.38  0.38  0.38  0.45  0.57  0.71  0.85

29  0.41  0.41  0.41  0.49  0.63  0.77  0.92

30  0.45  0.45  0.45  0.53  0.69  0.84  1.00

31  0.49  0.49  0.49  0.57  0.76  0.92  1.07

32  0.53  0.53  0.53  0.62  0.83  1.00  1.15

33  0.57  0.57  0.57  0.67  0.91  1.07  1.23

34  0.62  0.62  0.62  0.73  1.00  1.15  1.32

35  0.67  0.67  0.67  0.79  1.07  1.23  1.41

36  0.73  0.73  0.73  0.85  1.15  1.32  1.51

37  0.79  0.79  0.79  0.92  1.23  1.41  1.62

38  0.85  0.85  0.85  1.00  1.32  1.51  1.74

39  0.92  0.92  0.92  1.07  1.41  1.62  1.86

40  1.00  1.00  1.00  1.15  1.51  1.74  1.99

41  1.07  1.07  1.07  1.23  1.62  1.86  2.14

42  1.15  1.15  1.15  1.32  1.74  1.99  2.29

43  1.23  1.23  1.23  1.41  1.86  2.14  2.45

44  1.32  1.32  1.32  1.52  1.99  2.29  2.63

45  1.41  1.41  1.41  1.62  2.14  2.45  2.81

46  1.52  1.52  1.52  1.74  2.29  2.63  3.02

47  1.62  1.62  1.62  1.87  2.45  2.81  3.23

48  1.74  1.74  1.74  2.00  2.63  3.02  3.46

49  1.87  1.87  1.87  2.14  2.81  3.23  3.71

50  2.00  2.00  2.00  2.30  3.02  3.46  3.97

51  2.14  2.14  2.14  2.46  3.23  3.71  4.26

52  2.30  2.30  2.30  2.64  3.46  3.97  4.56

53  2.46  2.46  2.46  2.83  3.71  4.26  4.69

54  2.64  2.64  2.64  3.03  3.97  4.56  5.24

55  2.83  2.83  2.83  3.25  4.26  4.89  5.61

56  3.03  3.03  3.03  3.48  4.56  5.24  6.01

57  3.25  3.25  3.25  3.73  4.89  5.61  6.44

58  3.48  3.48  3.48  4.00  5.24  6.01  6.90

59  3.73  3.73  3.73  4.29  5.61  6.44  7.39

60  4.00  4.00  4.00  4.59  6.01  6.90  7.92

61  4.29  4.29  4.29  4.92  6.44  7.39  8.49

62  4.59  4.59  4.59  5.28  6.90  7.92  9.09

63  4.92  4.92  4.92  5.66  7.39  8.49  9.74

64  5.28  5.28  5.28  6.06  7.52  9.09  10.4

65  5.66  5.66  5.66  6.50  8.49  9.74  11.2

66  6.06  6.06  6.06  6.96  9.09  10.4  12.0

67  6.50  6.50  6.50  7.46  9.74  11.2  12.8

68  6.96  6.96  6.96  8.00  10.4  12.0  13.8

69  7.46  7.46  7.46  8.57  11.2  12.8  14.7

70  8.00  8.00  8.00  9.19  12.0  13.8  15.8

71  8.57  8.57  8.57  9.85  12.8  14.7  16.9

72  9.19  9.19  9.19  10.6  13.8  15.8  18.1

73  9.85  9.85  9.85  11.3  14.7  16.9  19.4

74  10.6  10.6  10.6  12.1  15.8  18.1  20.8

75  11.3  11.3  11.3  13.0  16.9  19.4  22.3

76  12.1  12.1  12.1  13.9  18.1  20.8  23.9

77  13.0  13.0  13.0  14.9  19.4  22.3  25.6

78  13.9  13.9  13.9  16.0  20.8  23.9  27.4

79  14.9  14.9  14.9  17.1  22.3  25.6  29L4

80  16.0  16.0  16.0  18.4  23.9  27.4  31.5

81  17.1  17.1  17.1  19.7  25.6  29.4  33.7

82  18.4  18.4  18.4  21.1  27.4  31.5  36.1

83  19.7  19.7  19.7  22.6  29.4  33.7  38.7

84  21.1  21.1  21.1  24.3  31.5  36.1  41.5

85  22.6  22.6  22.6  26.0  33.7  38.7  44.4

86  24.3  24.3  24.3  27.9  36.1  41.5  47.6

87  26.0  26.0  26.0  29.9  38.7  44.4  51.0

88  27.9  27.9  27.9  32.0  41.5  47.6  54.7

89  29.9  29.9  29.9  34.3  44.4  51.0  58.6

90  32.0  32.0  32.0  36.8  47.6  54.7  62.7

91  34.3  34.3  34.3  39.4  51.0  58.6  67.2

92  36.8  36.8  36.8  42.2  54.7  62.7  72.0

93  39.4  39.4  39.4  45.3  58.6  67.2  77.2

94  42.2  42.2  42.2  48.5  62.7  72.0  82.7

95  45.3  45.3  45.3  52.0  67.2  77.2  88.6

96  48.5  48.5  48.5  55.7  72.0  82.7  94.9

97  52.0  52.0  52.0  59.7  77.2  88.6   102

98  55.7  55.7  55.7  64.0  82.7  94.9   109

99  59.7  59.7  59.7  68.6  88.6   102   117

100  64.0  64.0  64.0  73.5  94.9   109   125

101  68.6  68.6  68.6  78.8   102   117   134

102  73.5  73.5  73.5  84.4   109   125   144

103  78.8  78.8  78.8  90.5   117   134   154

104  84.4  84.4  84.4  97.0   125   144   165

105  90.5  90.5  90.5   104   134   154   177

106  97.0  97.0  97.0   111   144   165   189

107   104   104   104   119   154   177   203

108   111   111   111   128   165   189   217

109   119   119   119   137   177   203   233

110   128   128   128   137   189   217   249

111   137   137   137   158   203   233   267

112   147   147   147   169   217   249   286

113   158   158   158   181   233   267   307

114   169   169   169   194   249   286   329

115   181   181   181   208   267   307   352

116   194   194   194   223   286   329   377

117   208   208   208   239   307   352   404

118   223   223   223   256   329   377   433

119   239   239   239   274   352   404   464

120   256   256   256   294   377   433   497

121   274   274   274   315   404   464   533

122   294   294   294   338   433   497   571

123   315   315   315   362   464   533   611

124   338   338   338   388   497   571   655

125   362   362   362   416   533   611   702

126   388   388   388   446   571   655   752

127   416   416   416   479   611   702   806

128   446   446   446   512   655   752   863

129   478   478   478   549   702   806   925

130   512   512   512   588   752   863   991

131   549   549   549   630   806   925  1062

132   588   588   588   676   863   991  1137

133   630   630   630   724   925  1062  1219

134   676   676   676   776   991  1137  1306

135   724   724   724   832  1062  1219  1399

136   776   776   776   891  1137  1306  1499

137   832   832   832   955  1219  1399  1606

138   891   891   891  1024  1306  1499  1721

139   955   955   955  1098  1399  1606  1844

140  1024  1024  1024  1176  1499  1721  1975

141  1098  1098  1098  1261  1606  1844

142  1176  1176  1176  1351  1721  1975

143  1261  1261  1261  1448  1844

144  1351  1351  1351  1552  1975

145  1448  1448  1448  1664

146  1552  1552  1552  1783

147  1664  1664  1664  1911

148  1783  1783  1783  2048

149  1911  1911  1911

150  2048  2048  2048

[ ...Table continues... ]

SPL  3150  4000  5000  6300  8000  10000

4  0.10        0.10

5  0.11  0.10

6  0.12  0.11

7  0.14  0.13  0.10

8  0.16  0.14  0.11

9  0.17  0.16  0.14

10  0.19  0.18  0.16  0.10

11  0.22  0.21  0.18  0.12

12  0.24  0.24  0.21  0.14

12  0.27  0.27  0.24  0.16

14  0.30  0.30  0.27  0.19

15  0.33  0.33  0.30  0.22

16  0.35  0.35  0.33  0.26

17  0.38  0.38  0.35  0.30  0.10

18  0.41  0.41  0.38  0.33  0.12

19  0.45  0.45  0.41  0.36  0.14

20  0.49  0.49  0.45  0.39  0.17

21  0.53  0.53  0.46  0.42  0.21   0.10

22  0.57  0.57  0.53  0.46  0.25   0.11

23  0.62  0.62  0.57  0.50  0.30   0.13

24  0.67  0.67  0.62  0.55  0.33   0.15

25  0.73  0.73  0.67  0.60  0.36   0.17

26  0.79  0.79  0.73  0.65  0.39   0.20

27  0.85  0.85  0.79  0.71  0.42   0.23

28  0.92  0.92  0.85  0.77  0.46   0.26

29  1.00  1.00  0.92  0.84  0.50   0.30

30  1.07  1.07  1.00  0.92  0.55   0.33

31  1.15  1.15  1.07  1.00  0.60   0.37

32  1.23  1.23  1.15  1.07  0.65   0.41

33  1.32  1.32  1.23  1.15  0.71   0.45

34  1.41  1.41  1.32  1.23  0.77   0.50

35  1.51  1.51  1.41  1.32  0.84   0.55

36  1.62  1.62  1.51  1.41  0.92   0.61

37  1.74  1.74  1.62  1.51  1.00   0.67

38  1.86  1.86  1.74  1.62  1.10   0.74

39  1.99  1.99  1.86  1.74  1.21   0.82

40  2.14  2.14  1.99  1.86  1.34   0.90

41  2.29  2.29  2.14  1.99  1.48   1.00

42  2.45  2.45  2.29  2.14  1.63   1.10

43  2.63  2.63  2.45  2.29  1.79   1.21

44  2.81  2.81  2.63  2.45  1.99   1.34

45  3.02  3.02  2.81  2.63  2.14   1.48

46  3.23  3.23  3.02  2.81  2.29   1.63

47  3.46  3.46  3.23  3.02  2.45   1.79

48  3.71  3.71  3.46  3.23  2.63   1.98

49  3.97  3.97  3.71  3.46  2.81   2.18

50  4.26  4.26  3.97  3.71  3.02   2.40

51  4.56  4.56  4.26  3.97  3.23   2.63

52  4.89  4.89  4.56  4.26  3.46   2.81

53  5.24  5.24  4.89  4.56  3.71   3.02

54  5.61  5.61  5.24  4.89  3.97   3.23

55  6.01  6.01  5.61  5.24  4.26   3.46

56  6.44  6.44  6.01  5.61  4.56   3.71

57  6.90  6.90  6.44  6.01  4.89   3.97

58  7.39  7.39  6.90  6.44  5.24   4.26

59  7.92  7.92  7.39  6.90  5.61   4.56

60  8.49  8.49  7.92  7.39  6.01   4.89

61  9.09  9.09  8.49  7.92  6.44   5.24

62  9.74  9.74  9.09  8.49  6.90   5.61

63  10.4  10.4  9.74  9.09  7.39   6.01

64  11.2  11.2  10.4  9.74  7.92   6.44

65  12.0  12.0  11.2  10.4  8.49   6.90

66  12.8  12.8  12.0  11.2  9.09   7.39

67  13.8  13.8  12.8  12.0  9.74   7.92

68  14.7  14.7  13.8  12.8  10.4   8.49

69  15.8  15.8  14.7  13.8  11.2   9.09

70  16.9  16.9  15.8  14.7  12.0   9.74

71  18.1  18.1  16.9  15.8  12.8   10.4

72  19.4  19.4  18.1  16.9  13.8   11.2

73  20.8  20.8  19.4  18.1  14.7   12.0

74  22.3  22.3  20.8  19.4  15.8   12.8

75  23.9  23.9  22.3  20.8  16.9   13.8

76  25.6  25.6  23.9  22.3  18.1   14.7

77  27.4  27.4  25.6  23.9  19.4   15.8

78  29.4  29.4  27.4  25.6  20.8   16.9

79  31.5  31.5  29.4  27.4  22.3   18.1

80  33.7  33.7  31.5  29.4  23.9   19.4

81  36.1  36.1  33.7  31.5  25.6   20.8

82  38.7  38.7  36.1  33.7  27.4   22.3

83  41.5  41.5  38.7  36.1  29.4   23.9

84  44.4  44.4  41.5  38.7  31.5   25.6

85  47.6  47.6  44.4  41.5  33.7   27.4

86  51.0  51.0  47.6  44.4  36.1   29.4

87  54.7  54.7  51.0  47.6  38.7   31.5

88  58.6  58.6  54.7  51.0  41.5   33.7

89  62.7  62.7  58.6  54.7  44.4   36.1

90  67.2  67.2  62.7  58.6  47.6   38.7

91  72.6  72.0  67.2  62.7  51.0   41.5

92  77.2  77.2  72.0  67.2  54.7   44.4

93  82.7  82.7  77.2  72.0  58.6   47.6

94  88.6  88.6  82.7  77.2  62.7   51.0

95  94.9  94.9  88.6  82.7  67.2   54.7

96   102   102  94.9  88.6  72.0   58.6

97   109   109   102  94.9  77.2   62.7

98   117   117   105   102  82.7   67.2

99   125   125   117   109  88.6   72.0

100   134   134   125   117  94.9   77.2

101   144   144   134   125   102   82.7

102   154   154   144   134   109   88.6

103   165   165   154   144   117   94.9

104   177   177   165   154   125    102

105   189   189   177   165   134    109

106   203   203   189   177   144    117

107   217   217   203   109   154    125

108   233   233   217   203   165    134

109   249   249   233   217   177    144

110   267   267   249   233   189    154

111   286   286   267   249   203    165

112   307   307   286   267   217    177

113   329   329   307   286   233    189

114   352   352   329   307   249    203

115   377   377   352   329   267    217

116   404   404   377   352   286    233

117   433   433   404   377   307    249

118   464   464   433   404   329    267

119   497   497   464   433   352    286

120   533   533   497   464   377    307

121   571   571   533   497   404    329

122   611   611   571   533   433    352

123   655   655   611   571   464    377

124   702   702   655   611   497    404

125   752   752   702   655   533    433

126   806   806   752   702   571    464

127   863   863   606   732   611    497

128   925   925   863   806   655    533

129   991   991   925   863   702    571

130  1062  1062   991   925   752    611

131  1137  1137  1062   991   606    655

132  1219  1219  1137  1062   863    702

133  1306  1306  1219  1137   925    752

134  1399  1399  1306  1219   991    606

135  1499  1499  1399  1306  1062    863

136  1606  1606  1499  1399  1197    925

137  1721  1721  1606  1499  1219    991

138  1844  1844  1721  1606  1306   1062

139  1975  1975  1844  1721  1399   1137

140              1975  1844  1499   1219

141                    1975  1606   1306

142                          1721   1399

143                          1844   1499

144                          1975   1606

145                                 1721

146                                 1844

147                                 1975

148

149

150

[** This header applies to all remaining

columns in this segment and extends to

the next segment.]

Section B36.5 Correction for spectral irregularities. Noise having

pronounced irregularities in the spectrum (for example, discrete frequency

components or tones), must be adjusted by the correction factor C(k)

calculated as follows:

(a) Step 1. Starting with the corrected sound pressure level in the 80 Hz

one-third octave band (band number 3), calculate the changes in sound

pressure level (or "slopes") in the remainder of the one-third octave bands

as follows:

s(3,k)=no value

s(4,k)=SPL(4,k)-SPL(3,k)

.

.

.

s(i,k)=SPL(i,k)-SPL[(i-l),k]

.

.

.

s(24,k)=SPL(24,k)-SPL(23,k)

(b) Step 2. Encircle the value of the slope, s(i,k), where the absolute

value of the change in slope is greater than 5; that is, where

+/- <Delta>s(i,k) +/- = +/- s(i,k)-s[(i-1),k] +/- >5

(c) Step 3. (1) If the encircled value of the slope s(i,k) is positive and

algebraically greater than the slope s[(i-1),k], encircle SPL(I,K).

(2) If the encircled value of the slope s(1,k) is zero or negative and the

slope s[i-1),k] is positive, encircle (SPL[(i-1),k])

(3) For all other cases, no sound pressure level value is to be encircled.

(d) Step 4. Omit all SPL(i,k) encircled in Step 3 and compute new sound

pressure levels SPL'(i,k) as follows:

(1) For nonencircled sound pressure levels, let the new sound pressure

levels equal the original sound pressure levels,

SPL'(i,k)=SPL(i,k)

(2) For encircled sound pressure levels in bands 1-23, let the new sound

pressure level equal the arithmetic average of the preceding and following

sound pressure levels.

SPL'(i,k)=(1/2)[SPL[(i-1),k]+SPL[(i+1),k]]

(3) If the sound pressure level in the highest frequency band (i=24) is

encircled, let the new sound pressure level in that band equal

SPL'(24,k)=SPL(23,k+s(23,k).

(e) Step 5. Recompute new slopes s' (i,k), including one for an imaginary

25-th band, as follows:

s'(3,k)=s'(4,k)

s'(4,k)=SPL'(4,k)-SPL'(3,k)

.

.

.

s'(i,k)=SPL'(i,k)-SPL'[(i-1),k]

s'(24,k)=SPL'(24,k)-SPL'(23,k)

s'(25,k)=s'(24,k)

(f) Step 6. For i from 3 to 23, compute the arithmetic average of the three

adjacent slopes as follows:

s(i,k)=(1/3)[s'(i,k)+s'[(i+1),k] +s'[(i+2),k]]

(g) Step 7. Compute final adjusted one-third octave-band sound pressure

levels, SPL'' (i,k), by beginning with band number 3 and proceeding to band

number 24 as follows:

SPL''(3,k)=SPL(3,k)

SPL''(4,k)=SPL''(3.k)+s(3.k)

.

.

.

SPL''(i,k)=SPL''[(i-1),k]+s[(i-1),k]

.

.

.

SPL''(24,k)=SPL''(23,k)+s(23,k)

(h) Step 8. Calculate the differences, F(i,k), between the original and the

adjusted sound pressure levels as follows:

F(i,k)=SPL(i,k)-SPL''(i,k)

and note only value greater than one and a half.

(i) Step 9. For each of the 24 one-third octave bands, determine tone

correction factors from the sound pressure level differences F(i,k) and Table

B2.

Table B2 -- Tone Correction Factors

[ ...Illustration appears here... ]

Level         Tone

difference   correction

Frequency f, Hz      F, dB        C, dB

50</=f<500        1 1/2*</=F<3  F/3-1/2

3</=F<20      F/6

20</=F        3 1/2

500</=f</=5,000   1 1/2*</=F<3  2 F/3-1

3</=F<20      F/3

20</=F        6 2/3

5,000<f</=10,000  1 1/2*</=F<3  F/3-1/2

3</=F<20      F/6

20</=F        3 1/3

* See Step 8.

(j) Step 10. Designate the largest of the tone correction factors,

determined in Step 9, as C(k). An example of the tone correction procedure is

given in Table B3.

(k) Tone corrected perceived noise levels PNLT(k) are determined by adding

the C(k) values to corresponding PNL(k) values, that is,

PNLT(k)=PNL(k)+C(k)

(l) For any i-th one-third octave band, at any k-th increment of time, for

which the tone correction factor is suspected to result from something other

than (or in addition to) an actual tone (or any special irregularity other

than aircraft noise), an additional analysis may be made using a filter with

a bandwidth narrower than one-third of an octave. If the narrow band analysis

corroborates that suspicion, then a revised value for the background sound

pressure level, SPL''(i,k) may be determined from the analysis and used to

compute a revised tone correction factor, F(i,k), for that particular one-

third octave band.

(m) Tones resulting from ground-plane reflections in the 800 Hz and lower

one-third octave bands may be excluded from the calculation of corrections

for spectral irregularities. To qualify for this exclusion, the pseudotones

must be clearly identified as not being related to the engine noise. This

identification may be made either by comparing measured data with data from a

flush mounted microphone, or by observing the Doppler shift characteristics

of the tone during the flyover-noise/time history. Since pseudotones are

related to ground reflections, a microphone mounted flush to the ground will

yield a spectral shape which can be distinguished from that produced by the

4-foot high microphone at those frequencies which can be related to ground

reflection's geometrical relationships. Identification through Doppler

shifting (the symmetric variation of frequency with time) can be made because

the Doppler frequency variation yields a frequency increase for an

approaching signal and a frequency decrease for a receding signal.

Pseudotones at frequencies above 800 Hz generally should not yield

significant tone corrections. However, for consistency, each tone correction

value must be included in the computation for spectral irregularities. While

the tone corrections below 800 Hz may be ignored for the spectral

irregularity correction, the SPL values must be included in the noy

calculation prescribed in section B36.13 of this appendix.

(n) After the value of PNLTM for each flyover-noise/time history, is

identified, the frequency for the largest tone correction factor (C(k)) must

be identified for the two preceding and the two succeeding, 500-milli-second

time intervals, to identify possible tone suppression at PNLTM as a result of

band sharing of the tone. If the value of C(k) for PNLTM is less than the

average value of C(k) for those five consecutive time intervals, that average

value of C(k) must be used to compute a new value for PNLTM.

Section B36.7 Maximum tone corrected perceived noise level. (a) The maximum

tone corrected perceived noise level, PNLTM, is the maximum calculated value

of the tone corrected perceived noise level, PNLT(k), calculated in

accordance with the procedure of section B36.5 of this appendix. Figure B2 is

an example of a flyover noise time history where the maximum value is clearly

indicated. Half-second time intervals, <Delta>t, are small enough to obtain a

satisfactory noise time history.

Table B3 -- Example of Tone Correction Calculation for a

Turbofan Engine

[ ...Illustration appears here... ]

(b) If there are no pronounced irregularities in the spectrum, then the

procedure of Sec. B36.5 of this Appendix would be redundant since PNLT(k)

would be identically equal to PNL(k). For this case, PNLTM would be the

maximum value of PNL(k) and would equal PNLM.

Section B36.9 Duration correction. The duration correction factor D is

determined by the integration technique defined by the expression:

t(2)

D = 10 log [(1/T)   INTEGRAL  ant [PNLT/10] dt ] - PNLTM

t(1)

Figure B2 -- Example of Perceived Noise Level Corrected

For Tones As A Function of Aircraft Flyover Time

[ ...Illustration appears here... ]

Where T is a normalizing time constant, PNLTM is the maximum value of PNLT,

and t(1) and t(2) are the limits of the significant noise time history.

(a) Since PNLT is calculated from measured values of SPL, there will, in

general, be no obvious equation for PNLT as a function of time. Consequently,

the equation can be rewritten with a summation sign instead of an integral

sign as follows:

d/ Dt

D = 10 log [(1/T)    SUM  Dt ant [PNLT(k)/10]] - PNLTM

k=0

where <Delta>t is the length of the equal increments of time for which

PNLT(k) is calculated and d is the time interval to the nearest 1.0 second

during which PNLT(k) is within a specified value, h, of PNLTM.

(b) Half-second time intervals for <Delta>t are small enough to obtain a

satisfactory history of the perceived noise level. A shorter time interval

may be selected by the applicant provided approved limits and constants are

used.

(c) The following values for T, <Delta>t,and h, must be used in calculating

D:

T=10 sec,

<Delta>t=0.5 sec. (or the approved sampling time interval), and

h=10 dB.

Using the above values, the equation for D becomes

2d

D = 10 log [   SUM  ant [PNLT(k)/10]] - PNLTM - 13

k=0

Where the integer d is the duration time defined by the points that are 10 dB

less than PNLTM.

(d) If the 10 dB-down points fall between calculated PNLT(k) values (the

usual case), the applicable limits for the duration time must be chosen from

the PNLT(k) values closest to PNLTM-10. For those cases with more than one

peak value of PNLT(k), the applicable limits must be chosen to yield the

largest possible value for the duration time.

(e) If the value of PNLT(k) at the 10 dB-down points is 90 PNdB or less,

the value of d may be taken as the time interval between the initial and the

final times for which PNLT(k) equals 90 PNdB.

(f) The aircraft testing procedures must include the 10 dB-down points in

the flyover noise/time record.

Section B36.11 Effective perceived noise level. (a) The total subjective

effect of an aircraft flyover is designated "effective perceived noise

level," EPNL, and is equal to the algebraic sum of the maximum value of the

tone corrected perceived noise level, PNLTM, and the duration correction, D.

That is,

EPNL=PNLTM+D

where PNLTM and D are calculated under sections B36.7 and B36.9 of this

appendix.

(b) The above equation can be rewritten by substituting the equation for D

from Sec. B36.9 of this appendix, that is,

2d

EPNL = 10 log [   SUM  ant [PNLT(k)/10]] - 13

k=0

(c) If, during a test flight, one or more peak values of PNLT are observed

which are within 2 dB of PNLTM, the value of EPNL shall be calculated for

each, as well as for PNLTM. If any EPNL value exceeds the value at the moment

of PNLTM, the maximum value of such exceedance must be added as a further

adjustment to the EPNL calculated from the measured data.

Section B36.13  Mathematical formulation of noy tables.

(a) The relationship between sound pressure level and perceived noisiness

given in Table B1 is illustrated in Figure B3. The variation of log (n) with

SPL for a given one-third octave band can be expressed by straight lines as

shown in Figure B3.

(1) The slopes of the straight lines M(b), M(c), and M(d) and M(e);

(2) The intercepts of the lines on the SPL axis, SPL (b) and SPL (c); and

(3) The coordinates of the discontinuities, SPL (a) and log n(a); SPL (d)

and log n = -1.0; and SPL (e) and log n = log (0.3).

(b) The important aspects of the mathematical formulation are:

(1) SPL " SPL (a)

n = antilog [M(c)*(SPL-SPL(c))]

(2) SPL (b) > SPL < SPL (a)

n = antilog [M(b)*(SPL-SPL(b))]

(3) SPL (e) > SPL < SPL (b)

n = antilog [M(e)*(SPL-SPL(b))]

(4) SPL (d) > SPL < SPL (e)

n = 0.1 antilog [M(d)*(SPL-SPL(d))]

(c) Table B4 lists the values of the important constants necessary to

calculate sound pressure level as a function of perceived noisiness.

Figure B3 -- Perceived Noisiness As A Function of Sound Pressure Level

Table B4 -- Constants for Mathematically Formulated NOY Values

[ ...Illustrations appear here... ]

[Doc. No. 9337, 34 FR 18364, Nov. 18, 1969, as amended by Amdt. 36-5, 41 FR

35058, Aug. 19, 1976; Amdt. 36-9, 43 FR 8748, Mar. 2, 1978; Amdt. 36-14, 53

FR 3541, Feb. 5, 1988; Amdt. 36-15, 53 FR 16368, May 6, 1988]

Appendix C--Noise Levels for Transport Category and Turbojet Powered

Airplanes Under Sec. 36.201

Sec.

C36.1  Noise measurement and evaluation.

C36.3  Noise measuring points.

C36.5  Noise levels.

C36.7  Takeoff reference and test limitations.

C36.9  Approach reference and test limitations.

Section C36.1 Noise measurement and evaluation. Compliance with this

appendix must be shown with noise levels measured and evaluated as

prescribed, respectively, by Appendix A and Appendix B of this part, or under

approved equivalent procedures.

Section C36.3 Noise measuring points. Compliance with the noise level

standards of section C36.5 must be shown--

(a) For takeoff, at a point 21, 325 feet (6,500 meters) from the start of

the takeoff roll on the extended centerline of the runway;

(b) For approach, at a point 6,562 feet (2,000 meters) from the threshold

on the extended centerline of the runway; and

(c) For the sideline, at the point, on a line parallel to and 1,476 feet

(450 meters) from the extended centerline of the runway, where the noise

level after liftoff is greatest, except that, for an airplane powered by more

than three turbojet engines, this distance must be 0.35 nautical miles for

the purpose of showing compliance with Stage 1 or Stage 2 noise limits (as

applicable).

Sec. C36.5  Noise levels.

(a) Limits. Except as provided in paragraphs (b) and (c) of this section,

it must be shown by flight test that the noise levels of the airplane, at the

measuring points described in section C36.3, do not exceed the following

(with appropriate interpolation between weights);

(1) Stage 1 noise limits for acoustical changes for airplanes regardless of

the number of engines are those noise levels prescribed under Sec. 36.7(c) of

this part.

(2) Stage 2 noise limits for airplanes regardless of the number of engines

are as follows:

(i) For takeoff. 108 EPNdB for maximum weights of 600,000 pounds or more,

reduced by 5 EPNdB per halving of the 600,000 pounds maximum weight down to

93 EPNdB for maximum weights of 75,000 pounds and less.

(ii) For sideline and approach.--108 EPNdB for maximum weights of 600,000

pounds or more, reduced by 2 EPNdB per halving of the 600,000 pounds maximum

weight down to 102 EPNdB for maximum weights of 75,000 pounds and less.

(3) Stage 3 noise limits are as follows:

(i) For takeoff.

(A) For airplanes with more than 3 engines. 106 EPNdB for maximum weights

of 850,000 pounds or more, reduced by 4 EPNdB per halving of the 850,000

pounds maximum weight down to 89 EPNdB for maximum weights of 44,673 pounds

or less;

(B) For airplanes with 3 engines--104 EPNdB for maximum weights of 850,000

pounds or more, reduced by 4 EPNdB per halving of the 850,000 pounds maximum

weight down to 89 EPNdB for maximum weights of 63,177 pounds and less; and

(C) For airplanes with fewer than 3 engines--101 EPNdB for maximum weights

of 850,000 pounds or more, reduced by 4 EPNdB per halving of the 850,000

pounds maximum weight down to 89 EPNdB for maximum weights of 106,250 pounds

and less.

(ii) For sideline, regardless of the number of engines. 103 EPNdB for

maximum weights of 882,000 pounds or more, reduced by 2.56 EPNdB per halving

of the 882,000 pounds maximum weight down to 94 EPNdB for maximum weights of

77,200 pounds or less.

(iii) For approach, regardless of the number of engines--105 EPNdB for

maximum weights of 617,300 pounds or more, reduced by 2.33 EPNdB per halving

of the 617,300 pounds weight down to 98 EPNdB for maximum weights of 77,200

pounds or less.

(b) Tradeoffs. Except to the extent limited under Secs. 36.7(c)(1) and

36.7(d)(3)(i)(B) of this part, the noise level limits prescribed in paragraph

(a) of this section may be exceeded at one or two of the measuring points

specified in section C36.3 of this appendix, if--

(1) The sum of the exceedances is not greater than 3 EPNdB;

(2) No exceedance is greater than 2 EPNdB; and

(3) The exceedances are completely offset by reductions at other required

measuring points.

Sec. C36.7  Takeoff Reference and Test Limitations.

(a) This section applies to all takeoff noise tests conducted under this

appendix in showing compliance with this part.

(b) Takeoff power or thrust must be used from the start of takeoff roll to

at least the following altitude above the runway:

(1) For Stage 1 airplanes and for Stage 2 airplanes that do not have

turbojet engines with a bypass ratio of 2 or more, the following apply:

(i) For airplanes with more than three turbojet engines--700 feet (214

meters).

(ii) For all other airplanes--1,000 feet (305 meters).

(2) For Stage 2 airplanes that have turbojet engines with a bypass ratio of

2 or more and for Stage 3 airplane, the following apply:

(i) For airplanes with more than three turbojet engines--689 feet (210

meters).

(ii) For airplanes with three turbojet engines--853 feet (260 meters).

(iii) For airplanes with fewer than three turbojet engines--984 feet (300

meters).

(iv) For airplanes not powered by turbojet engines--1,000 feet (305

meters).

(c) Upon reaching the altitude specified in paragraph (b) of this section,

the power or thrust may not be reduced below that needed to maintain level

flight with one engine inoperative, or to maintain a four percent climb

gradient, whichever power or thrust is greater.

(d) A constant takeoff configuration, selected by the applicant, must be

maintained throughout the takeoff noise test, except that the landing gear

may be retracted.

(e) For applications made for subsonic airplanes after September 17, 1971,

and for Concorde airplanes, the following apply:

(1) For subsonic airplanes the test day speeds and the acoustic day

reference speed must be the minimum approved value of V2+10 knots, or the

all-engines-operating speed at 35 feet (for turbine engine powered airplanes)

or 50 feet (for reciprocating engine powered airplanes), whichever speed is

greater as determined under the regulations constituting the type

certification basis of the airplane. These tests must be conducted at the

test day speeds+/-3 knots. Noise values measured at the test day speeds must

be corrected to the acoustic day reference speed.

(2) For Concorde airplanes, the test day speeds and the acoustic day

reference speed must be the minimum approved value of V2 +35 knots, or the

all-engines-operating speed at 35 feet, whichever speed is greater as

determined under the regulations constituting the type certification basis of

the airplane, except that the reference speed may not exceed 250 knot. These

tests must be conducted at the test day speeds +3 knots. Noise values

measured at the test day speeds must be corrected to the acoustic day

reference speed.

(3) If a negative runway gradient exists in the direction of takeoff,

performance and acoustic data must be corrected to the zero slope condition.

Sec. C36.9  Approach reference and test limitations.

(a) This section applies to all approaches conducted in showing compliance

with this part.

(b) The airplane's configuration must be that used in showing compliance

with the landing requirements in the airworthiness regulations constituting

the type certification basis of the airplane. If more than one configuration

is used in showing compliance with the landing requirements in the

airworthiness regulations constituting the type certification basis of the

airplane, the configuration that is most critical from a noise standpoint

must be used.

(c) The approaches must be conducted with a steady glide angle of 3

deg.+0.5 deg. and must be continued to a normal touchdown with no airframe

configuration change.

(d) All engines must be operating at approximately the same power or

thrust.

(e) For applications made for subsonic airplanes after September 17, 1971,

and for Concorde airplanes, the following apply:

(1) For subsonic airplanes a steady approach speed, that is either 1.30 Vs

+10 knots or the speed used in establishing the approved landing distance

under the airworthiness regulations constituting the type certification basis

of the airplane, whichever speed is greatest, must be established and

maintained over the approach measuring point.

(2) For Concorde airplanes a steady approach speed, that is either the

landing reference speed + 10 knots or the speed used in establishing the

approved landing distance under the airworthiness regulations constituting

the type certification basis of the airplane, whichever speed is greater,

must be established and maintained over the approach measuring point.

(3) A tolerance of +3 knots may be used throughout the approach noise

testing.

[Doc. No. 9337, 34 FR 18364, Nov. 18, 1969, as amended by Amdt. 36-1, 34 FR

18815, Nov. 25, 1969; 34 FR 19025, Nov. 29, 1969; Amdt. 36-5, 41 FR 35058,

Aug. 19, 1976, Amdt. 36-7, 42 FR 12371, Mar. 3, 1977; Amdt. 36-8, 43 FR 8730,

Mar. 2, 1978; Amdt. 36-10, 43 FR 28420, June 29, 1978; 43 FR 44475, Sept. 28,

1978; 43 FR 47489, Oct. 16, 1978; Amdt. 36-12, 46 FR 33465, June 29, 1981;

Amdt. 36-15, 53 FR 16372, May 6, 1988]

Appendices D-E--[Reserved]

Appendix F--Flyover Noise Requirements for Propeller-Driven Small Airplane

and Propeller-Driven, Commuter Category Airplane Certification Tests

Prior to December 22, 1988

PART A--GENERAL

Sec.

F36.1  Scope.

PART B--NOISE MEASUREMENT

F36.101  General test conditions.

F36.103  Acoustical measurement system.

F36.105  Sensing, recording, and reproducing equipment.

F36.107  Noise measurement procedures.

F36.109  Data recording, reporting, and approval.

F36.111  Flight procedures.

PART C--DATA CORRECTION

F36.201  Correction of data.

F36.203  Validity of results.

PART D--NOISE LIMITS

F36.301  Aircraft noise limits.

PART A--GENERAL

Section F36.1 Scope. This appendix prescribes noise level limits and

procedures for measuring and correcting noise data for the propeller driven

small airplanes specified in Secs. 36.1 and 36.501(b).

PART B--NOISE MEASUREMENT

Sec. F36.101   General test conditions.

(a) The test area must be relatively flat terrain having no excessive sound

absorption characteristics such as those caused by thick, matted, or tall

grass, by shrubs, or by wooded areas. No obstructions which significantly

influence the sound field from the airplane may exist within a conical space

above the measurement position, the cone being defined by an axis normal to

the ground and by a half-angle 75 degrees from this axis.

(b) The tests must be carried out under the following conditions:

(1) There may be no precipitation.

(2) Relative humidity may not be higher than 90 percent or lower than 30

percent.

(3) Ambient temperature may not be above 86 degrees F. or below 41 degrees

F. at 33' above ground. If the measurement site is within 1 n.m. of an

airport thermometer the airport reported temperature may be used.

(4) Reported wind may not be above 10 knots at 33' above ground. If wind

velocities of more than 4 knots are reported, the flight direction must be

aligned to within +15 degrees of wind direction and flights with tail wind

and head wind must be made in equal numbers. If the measurement site is

within 1 n.m. of an airport anemometer, the airport reported wind may be

used.

(5) There may be no temperature inversion or anomalous wind conditions that

would significantly alter the noise level of the airplane when the noise is

recorded at the required measuring point.

(6) The flight test procedures, measuring equipment, and noise measurement

procedures must be approved by the FAA.

(7) Sound pressure level data for noise evaluation purposes must be

obtained with acoustical equipment that complies with section F36.103 of this

appendix.

Sec. F36.103   Acoustical measurement system. The acoustical measurement

system must consist of approved equipment equivalent to the following:

(a) A microphone system with frequency response compatible with measurement

and analysis system accuracy as prescribed in section F36.105 of this

appendix.

(b) Tripods or similar microphone mountings that minimize interference with

the sound being measured.

(c) Recording and reproducing equipment characteristics, frequency

response, and dynamic range compatible with the response and accuracy

requirements of section F36.105 of this appendix.

(d) Acoustic calibrators using sine wave or broadband noise of known sound

pressure level. If broadband noise is used, the signal must be described in

terms of its average and maximum root-mean-square (rms) value for nonoverload

signal level.

Sec. F36.105   Sensing, recording, and reproducing equipment.

(a) The noise produced by the airplane must be recorded. A magnetic tape

recorder is acceptable.

(b) The characteristics of the system must comply with the recommendations

in International Electrotechnical Commission (IEC) Publication No. 179,

entitled "Precision Sound Level Meters" as incorporated by reference in Part

36 under Sec. 36.6 of this part.

(c) The response of the complete system to a sensibly plane progressive

sinusoidal wave of constant amplitude must lie within the tolerance limits

specified in IEC Publication No. 179, dated 1973, over the frequency range 45

to 11,200 Hz.

(d) If limitations of the dynamic range of the equipment make it necessary,

high frequency pre-emphasis must be added to the recording channel with the

converse de-emphasis on playback. The pre-emphasis must be applied such that

the instantaneous recorded sound pressure level of the noise signal between

800 and 11,200 Hz does not vary more than 20 dB between the maximum and

minimum one-third octave bands.

(e) If requested by the Administrator, the recorded noise signal must be

read through an "A" filter with dynamic characteristics designated "slow," as

defined in IEC Publication No. 179, dated 1973. The output signal from the

filter must be fed to a rectifying circuit with square law rectification,

integrated with time constants for charge and discharge of about 1 second or

800 milliseconds.

(f) The equipment must be acoustically calibrated using facilities for

acoustic freefield calibration and if analysis of the tape recording is

requested by the Administrator, the analysis equipment shall be

electronically calibrated by a method approved by the FAA.

(g) A windscreen must be employed with microphone during all measurements

of aircraft noise when the wind speed is in excess of 6 knots.

Sec. F36.107   Noise measurement procedures.

(a) The microphones must be oriented in a known direction so that the

maximum sound received arrives as nearly as possible in the direction for

which the microphones are calibrated. The microphone sensing elements must be

approximately 4' above ground.

(b) Immediately prior to and after each test; a recorded acoustic

calibration of the system must be made in the field with an acoustic

calibrator for the two purposes of checking system sensitivity and providing

an acoustic reference level for the analysis of the sound level data.

(c) The ambient noise, including both acoustical background and electrical

noise of the measurement systems, must be recorded and determined in the test

area with the system gain set at levels that will be used for aircraft noise

measurements. If aircraft sound pressure levels do not exceed the background

sound pressure levels by at least 10 dB(A), approved corrections for the

contribution of background sound pressure level to the observed sound

pressure level must be applied.

Sec. F36.109   Data recording, reporting, and approval.

(a) Data representing physical measurements or corrections to measured data

must be recorded in permanent form and appended to the record except that

corrections to measurements for normal equipment response deviations need not

be reported. All other corrections must be approved. Estimates must be made

of the individual errors inherent in each of the operations employed in

obtaining the final data.

(b) Measured and corrected sound pressure levels obtained with equipment

conforming to the specifications described in section F36.105 of this

appendix must be reported.

(c) The type of equipment used for measurement and analysis of all

acoustic, airplane performance, and meteorological data must be reported.

(d) The following atmospheric data, measured immediately before, after, or

during each test at the observation points prescribed in section F36.101 of

this appendix must be reported:

(1) Air temperature and relative humidity.

(2) Maximum, minimum, and average wind velocities.

(e) Comments on local topography, ground cover, and events that might

interfere with sound recordings must be reported.

(f) The following airplane information must be reported:

(1) Type, model and serial numbers (if any) of airplanes, engines, and

propellers.

(2) Any modifications or nonstandard equipment likely to affect the noise

characteristics of the airplane.

(3) Maximum certificated takeoff weights.

(4) Airspeed in knots for each overflight of the measuring point.

(5) Engine performance in terms of revolutions per minute and other

relevant parameters for each overflight.

(6) Aircraft height in feet determined by a calibrated altimeter in the

aircraft, approved photographic techniques, or approved tracking facilities.

(g) Aircraft speed and position and engine performance parameters must be

recorded at an approved sampling rate sufficient to ensure compliance with

the test procedures and conditions of this appendix.

Sec. F36.111   Flight procedures.

(a) Tests to demonstrate compliance with the noise level requirements of

this appendix must include at least six level flights over the measuring

station at a height of 1,000' +30' and +10 degrees from the zenith when

passing overhead.

(b) Each test over flight must be conducted:

(1) At not less than the highest power in the normal operating range

provided in an Airplane Flight Manual, or in any combination of approved

manual material, approved placard, or approved instrument markings; and

(2) At stabilized speed with propellers synchronized and with the airplane

in cruise configuration, except that if the speed at the power setting

prescribed in this paragraph would exceed the maximum speed authorized in

level flight, accelerated flight is acceptable.

PART C--DATA CORRECTION

Sec. F36.201   Correction of data.

(a) Noise data obtained when the temperature is outside the range of 68

degrees F. +/-9 degrees F., or the relative humidity is below 40 percent,

must be corrected to 77 degrees F. and 70 percent relative humidity by a

method approved by the FAA.

(b) The performance correction prescribed in paragraph (c) of this section

must be used. It must be determined by the method described in this appendix,

and must be added algebraically to the measured value. It is limited to

5dB(A).

(c) The performance correction must be computed by using the following

formula:

R/C

DdB = 60-20 log 10 E(11,430-D50  ------  + 50F

Vy

Where:

D50=Takeoff distance to 50 feet at maximum certificated takeoff weight.

R/C=Certificated best rate of climb (fpm).

Vy =Speed for best rate of climb in the same units as rate of climb.

(d) When takeoff distance to 50' is not listed as approved performance

information, the figures of 2000 for single-engine airplanes and 1600' for

multi-engine airplanes must be used.

Sec. F36.203   Validity of results.

(a) The test results must produce an average dB(A) and its 90 percent

confidence limits, the noise level being the arithmetic average of the

corrected acoustical measurements for all valid test runs over the measuring

point.

(b) The samples must be large enough to establish statistically a 90 pecent

confidence limit not to exceed +/-1.5 dB(A). No test result may be omitted

from the averaging process, unless omission is approved by the FAA.

PART D--NOISE LIMITS

Sec. F36.301   Aircraft noise limits.

(a) Compliance with this section must be shown with noise data measured and

corrected as prescribed in Parts B and C of this appendix.

(b) For airplanes for which application for a type certificate is made on

or after October 10, 1973, the noise level must not exceed 68 dB(A) up to and

including aircraft weights of 1,320 pounds (600 kg.). For weights greater

than 1,320 pounds up to and including 3,630 pounds (1.650 kg.) the limit

increases at the rate of 1 dB/165 pounds (1 dB/75 kg.) to 82 dB(A) at 3,630

pounds, after which it is constant at 82 dB(A). However, airplanes produced

under type certificates covered by this paragraph must also meet paragraph

(d) of this section for the original issuance of standard airworthiness

certificates or restricted category airworthiness certificates if those

airplanes have not had flight time before the date specified in that

paragraph.

(c) For airplanes for which application for a type certificate is made on

or after January 1, 1975, the noise levels may not exceed the noise limit

curve prescribed in paragraph (b) of this section, except that 80 dB(A) may

not be exceeded.

(d) For airplanes for which application is made for a standard

airworthiness certificate or for a restricted category airworthiness

certificate, and that have not had any flight time before January 1, 1980,

the requirements of paragraph (c) of this section apply, regardless of date

of application, to the original issuance of the certificate for that

airplane.

[Doc. No. 13243, 40 FR 1035, Jan. 6, 1975; 40 FR 6347, Feb. 11, 1975, as

amended by Amdt. 36-6, 41 FR 56064, Dec. 23, 1976; Amdt. 36-6, 42 FR 4113,

Jan. 24, 1977; Amdt. 36-9, 43 FR 8754, Mar. 2, 1978; Amdt. 36-13, 52 FR 1836,

Jan. 15, 1987; Amdt. 36-16, 53 FR 47400, Nov. 22, 1988]

Appendix G--Takeoff Noise Requirements for Propeller-Driven Small Airplane

and Propeller-Driven, Commuter Category Airplane Certification Tests on

or After December 22, 1988

PART A--GENERAL

Sec.

G36.1  Scope.

PART B--NOISE MEASUREMENT

G36.101  General Test Conditions.

G36.103  Acoustical measurement system.

G36.105  Sensing, recording, and reproducing equipment.

G36.107  Noise measurement procedures.

G36.109  Data recording, reporting, and approval.

G36.111  Flight procedures.

PART C--DATA CORRECTIONS

G36.201  Corrections to Test Results.

G36.203  Validity of results.

PART D--NOISE LIMITS

G36.301  Aircraft Noise Limits.

PART A--GENERAL

Section G36.1 Scope. This appendix prescribes limiting noise levels and

procedures for measuring noise and adjusting these data to standard

conditions, for propeller driven small airplanes and propeller-driven,

commuter category airplanes specified in Secs. 36.1 and 36.501(c).

PART B--NOISE MEASUREMENT

Sec. G36.101 General Test Conditions.

(a) The test area must be relatively flat terrain having no excessive sound

absorption characteristics such as those caused by thick, matted, or tall

grass, by shrubs, or by wooded areas. No obstructions which significantly

influence the sound field from the airplane may exist within a conical space

above the measurement position, the cone being defined by an axis normal to

the ground and by a half-angle 75 degrees from the normal ground axis.

(b) The tests must be carried out under the following conditions:

(1) No precipitation;

(2) Ambient air temperature between 36 and 95 degrees F (2.2 and 35 degrees

C);

(3) Relative humidity between 20 percent and 95 percent, inclusively;

(4) Wind speed may not exceed 10 knots (19 km/h) and cross wind may not

exceed 5 knots (9 km/h), using a 30-second average;

(5) No temperature inversion or anomalous wind condition that would

significantly alter the noise level of the airplane when the nose is recorded

at the required measuring point, and

(6) The meteorological measurements must be made between 4 ft. (1.2 m) and

33 ft. (10 m) above ground level. If the measurement site is within 1 n.m. of

an airport meteorological station, measurements from that station may be

used.

(c) The flight test procedures, measuring equipment, and noise measurement

procedures must be approved by the FAA.

(d) Sound pressure level data for noise evaluation purposes must be

obtained with acoustical equipment that complies with section G36.103 of this

appendix.

Sec. G36.103 Acoustical Measurement System.

The acoustical measurement system must consist of approved equipment with

the following characteristics: (a) A microphone system with frequency

response compatible with measurement and analysis system accuracy as

prescribed in section G36.105 of this appendix.

(b) Tripods or similar microphone mountings that minimize interference with

the sound being measured.

(c) Recording and reproducing equipment characteristics, frequency

response, and dynamic range compatible with the response and accuracy

requirements of section G36.105 of this appendix.

(d) Acoustic calibrators using sine wave or broadband noise of known sound

pressure level. If broadband noise is used, the signal must be described in

terms of its average and maximum root-mean-square (rms) value for non-

overload signal level.

Sec. G36.105 Sensing, Recording, and Reproducing Equipment.

(a) The noise produced by the airplane must be recorded. A magnetic tape

recorder, graphic level recorder, or sound level meter is acceptable when

approved by the regional certificating authority.

(b) The characteristics of the complete system must comply with the

requirements in International Electrotechnical Commission (IEC) Publications

No. 651, entitled "Sound Level Meters" and No. 561, entitled "Electro-

acoustical Measuring Equipment for Aircraft Noise Certification" as

incorporated by reference under Sec. 36.6 of this part. Sound level meters

must comply with the requirements for Type 1 sound level meters as specified

in IEC Publication No. 651.

(c) The response of the complete system to a sensibly plane progressive

sinusoidal wave of constant amplitude must be within the tolerance limits

specified in IEC Publication No. 651, over the frequency range 45 to 11,200

Hz.

(d) If equipment dynamic range limitations make it necessary, high

frequency pre-emphasis must be added to the recording channel with the

converse de-emphasis on playback. The pre-emphasis must be applied such that

the instantaneous recorded sound pressure level of the noise signal between

800 and 11,200 Hz does not vary more than 20 dB between the maximum and

minimum one-third octave bands.

(e) The output noise signal must be read through an "A" filter with dynamic

characteristics designated "slow" as defined in IEC Publication No. 651. A

graphic level recorder, sound level meter, or digital equivalent may be used.

(f) The equipment must be acoustically calibrated using facilities for

acoustic free-field calibration and if analysis of the tape recording is

requested by the Administrator, the analysis equipment shall be

electronically calibrated by a method approved by the FAA. Calibrations shall

be performed, as appropriate, in accordance with paragraph A36.3(e) of

Appendix A of this part.

(g) A windscreen must be employed with the microphone during all

measurements of aircraft noise when the wind speed is in excess of 5 knots (9

km/hr).

Sec. G36.107  Noise Measurement Procedures.

(a) The microphones must be oriented in a known direction so that the

maximum sound received arrives as nearly as possible in the direction for

which the microphones are calibrated. The microphone sensing elements must be

4 ft. (1.2m) above ground level.

(b) Immediately prior to and after each test, a recorded acoustic

calibration of the system must be made in the field with an acoustic

calibrator for the purposes of checking system sensitivity and providing an

acoustic reference level for the analysis of the sound level data. If a tape

recorder or graphic level recorder is used, the frequency response of the

electrical system must be determined at a level within 10 dB of the full-

scale reading used during the test, utilizing pink or pseudorandom noise.

(c) The ambient noise, including both acoustic background and electrical

systems noise, must be recorded and determined in the test area with the

system gain set at levels which will be used for aircraft noise measurements.

If aircraft sound pressure levels do not exceed the background sound pressure

levels by at least 10 dB(A), a takeoff measurement point nearer to the start

of the takeoff roll must be used and the results must be adjusted to the

reference measurement point by an approved method.

Sec. G36.109  Data Recording, Reporting, and Approval.

(a) Data representing physical measurements and adjustments to measured

data must be recorded in permanent form and appended to the record, except

that corrections to measurements for normal equipment response deviations

need not be reported. All other adjustments must be approved. Estimates must

be made of the individual errors inherent in each of the operations employed

in obtaining the final data.

(b) Measured and corrected sound pressure levels obtained with equipment

conforming to the specifications in section G36.105 of this appendix must be

reported.

(c) The type of equipment used for measurement and analysis of all

acoustical, airplane performance, and meteorological data must be reported.

(d) The following atmospheric data, measured immediately before, after, or

during each test at the observation points prescribed in section G36.101 of

this appendix must be reported:

(1) Ambient temperature and relative humidity.

(2) Maximum and average wind speeds and directions for each run.

(e) Comments on local topography, ground cover, and events that might

interfere with sound recordings must be reported.

(f) The aircraft position relative to the takeoff reference flight path

must be determined by an approved method independent of normal flight

instrumentation, such as radar tracking, theodolite triangulation, or

photographic scaling techniques.

(g) The following airplane information must be reported:

(1) Type, model, and serial numbers (if any) of airplanes, engines, and

propellers;

(2) Any modifications or nonstandard equipment likely to affect the noise

characteristics of the airplane;

(3) Maximum certificated takeoff weight;

(4) For each test flight, airspeed and ambient temperature at the flyover

altitude over the measuring site determined by properly calibrated

instruments;

(5) For each test flight, engine performance parameters, such as manifold

pressure or power, propeller speed (rpm) and other relevant parameters. Each

parameter must be determined by properly calibrated instruments. For

instance, propeller RPM must be validated by an independent device accurate

to within +/-1 percent, when the airplane is equipped with a mechanical

tachometer.

(6) Airspeed, position, and performance data necessary to make the

corrections required in section G36.201 of this appendix must be recorded by

an approved method when the airplane is directly over the measuring site.

Sec. G36.111  Flight Procedures.

(a) The noise measurement point is on the extended centerline of the runway

at a distance of 8200 ft (2500 m) from the start of takeoff roll. The

aircraft must pass over the measurement point within +/-10 degrees from the

vertical and within 20% of the reference altitude. The flight test program

shall be initiated at the maximum approved takeoff weight and the weight

shall be adjusted back to this maximum weight after each hour of flight time.

Each flight test must be conducted at the speed for the best rate of climb

(Vy) +/-5 knots (+/-9 km/hour) indicated airspeed. All test, measurement, and

data correction procedures must be approved by the FAA.

(b) The takeoff reference flight path must be calculated for the following

atmospheric conditions:

(1) Sea level atmospheric pressure of 1013.25 mb (013.25 hPa);

(2) Ambient air temperature of 59 deg.F (15 deg.C);

(3) Relative humidity of 70 percent; and

(4) Zero wind.

(c) The takeoff reference flight path must be calculated assuming the

following two segments:

(1) First segment.

(i) Takeoff power must be used from the brake release point to the point at

which the height of 50 ft (15m) above the runway is reached.

(ii) A constant takeoff configuration selected by the applicant must be

maintained through this segment.

(iii) The maximum weight of the airplane at brake-release must be the

maximum for which noise certification is requested.

(iv) The length of this first segment must correspond to the airworthiness

approved value for a takeoff on a level paved runway (or the corresponding

value for seaplanes).

(2) Second segment.

(i) The beginning of the second segment corresponds to the end of the first

segment.

(ii) The airplane must be in the climb configuration with landing gear up,

if retractable, and flap setting corresponding to normal climb position

throughout this second segment.

(iii) The airplane speed must be the speed for the best rate of climb (Vy).

(iv) Maximum continuous installed power and rpm for variable pitch

propeller(s) shall be used. For fixed pitch propeller(s), the maximum power

and rpm that can be delivered by the engine(s) must be maintained throughout

the second segment.

PART C--DATA CORRECTIONS

Sec. G36.201 Corrections to Test Results.

(a) These corrections account for the effects of:

(1) Differences in atmospheric absorption of sound between meteorological

test conditions and reference conditions.

(2) Differences in the noise path length between the actual airplane flight

path and the reference flight path.

(3) The change in the helical tip Mach number between test and reference

conditions.

(4) The change in the engine power between test and reference conditions.

(b) Atmospheric absorption correction is required for noise data obtained

when the test conditions are outside those specified in Figure G1. Noise data

outside the applicable range must be corrected to 77 F and 70 percent

relative humidity by a FAA approved method.

(c) Helical tip Mach number and power corrections must be made if:

(1) The propeller is a variable pitch type, or

(2) The propeller is a fixed pitch type and the test power is not within 5

percent of the reference power.

(d) When the test conditions are outside those specified, corrections must

be applied by an approved procedure or by the following simplified procedure:

(1) Measured sound levels must be corrected from test day meteorological

conditions to reference conditions by adding an increment equal to

Delta (M) = (]]-0.7) HT/1000

where HT is the height in feet of the test aircraft when directly over the

noise measurement point and ]] is the rate of absorption for the test day

conditions at 500 Hz as specified in SAE ARP 866A, entitled "Standard Values

of Atmospheric Absorption as a function of Temperature and Humidity for use

in Evaluating Aircraft Flyover Noise" as incorporated by reference under Sec.

36.6 of this part.

(2) Measured sound levels in decibels must be corrected for height by

algebraically adding an increment equal to Delta (1). When test day

conditions are within those specified in figure G1:

Delta (1) = 22 log (HT/HR)

where HT is the height of the test aircraft when directly over the noise

measurement point and HR is the reference height.

When test day conditions are outside those specified in figure G1:

Delta (1) = 20 log (HT/HR)

(3) Measured sound levels in decibels must be corrected for helical tip

Mach number by algebraically adding an increment equal to:

Delta (2) = k log (MR/MT)

where MT and MR are the test and reference helical tip Mach numbers,

respectively. The constant "k" is equal to the slope of the line obtained for

measured values of the sound level in dB(A) versus helical tip Mach number.

The value of k may be determined from approved data. A nominal value of k =

150 may be used when MT is smaller than

Figure G1 -- Measurement Window For No Absorption Correction

[ ...Illustration appears here... ]

MR. No correction may be made using the nominal value of k when MT is larger

than MR. The reference helical tip Mach number MR is the Mach number

corresponding to the reference conditions (RPM, airspeed, temperature) above

the measurement point.

(4) Measured sound levels in decibels must be corrected for engine power by

algebraically adding an increment equal to:

Delta (3) = 17 log (PR/PT)

where PT and PR are the test and reference engine powers respectively.

Sec. G36.203 Validity of Results.

(a) The measuring point must be overflown at least six times. The test

results must produce an average noise level (LAmax) value within a 90 percent

confidence limit. The average noise level is the arithmetic average of the

corrected acoustical measurements for all valid test runs over the measuring

point.

(b) The samples must be large enough to establish statistically a 90

percent confidence limit not exceeding +/-1.5 dB(A). No test results may be

omitted from the averaging process unless omission is approved by the FAA.

PART D--NOISE LIMITS

Sec. G36.301 Aircraft noise limits.

(a) Compliance with this section must be shown with noise data measured and

corrected as prescribed in Parts B and C of this appendix.

(b) The noise level must not exceed 73 dB(A) up to and including aircraft

weights of 1,320 pounds (600 kg). For weights greater than 1,320 pounds the

limit increases at the rate of 1 dB/165

Figure G2 -- Noise Level Vs. Airplane Weight

[ ...Illustration appears here... ]

pounds (1 dB/75 kg) up to 85 dB(A) at 3,300 pounds (1,500 kg), after which it

is constant at 85 dB(A) up to and including 19,000 pounds (8,640). Figure G2

shows noise level limits vs airplane weight.

(Secs. 313(a), 603, and 611(b), Federal Aviation Act of 1958 as amended (49

U.S.C. 1354(a), 1423, and 1431(b)); sec. 6(c), Department of Transportation

Act (49 U.S.C. 1655 (c)); Title I, National Environmental Policy Act of 1969

(42 U.S.C. 4321 et seq.); E. O. 11514, March 5, 1970 and 14 CFR 11.45).

[Amdt. 36-16, 53 FR 47400, Nov. 22, 1988; 53 FR 50157, Dec. 13, 1988]

Appendix H--Noise Requirements For Helicopters Under Subpart H

Part A--Reference Conditions

Sec.

H36.1  General.

H36.3  Reference Test Conditions.

H36.5  Symbols and Units.

Part B--Noise Measurement Under Sec. 36.801

H36.101  Noise certification test and measurement conditions.

H36.103  Takeoff test conditions.

H36.105  Flyover test conditions.

H36.107  Approach test conditions.

H36.109  Measurement of helicopter noise received on the ground.

H36.111  Reporting and correcting measured data.

H36.113  Atmospheric attenuation of sound.

Part C--Noise Evaluation and Calculation Under Sec. 36.803

H36.201  Noise evaluation in EPNdB.

H36.203  Calculation of noise levels.

H36.205  Detailed data correction procedures.

Part D--Noise Limits Under Sec. 36.805

H36.301  Noise measurement, evaluation, and calculation.

H36.303  [Reserved]

H36.305  Noise levels.

Part A--Reference Conditions

Section H36.1 General. This appendix prescribes noise requirements for

helicopters specified under Sec. 36.1, including:

(a) The conditions under which helicopter noise certification tests under

Part H must be conducted and the measurement procedures that must be used

under Sec. 36.801 to measure helicopter noise during each test;

(b) The procedures which must be used under Sec. 36.803 to correct the

measured data to the reference conditions and to calculate the noise

evaluation quantity designated as Effective Perceived Noise Level (EPNL); and

(c) The noise limits for which compliance must be shown under Sec. 36.805.

Section H36.3 Reference Test Conditions.

(a) Meteorological conditions. Aircraft position, performance data and

noise measurements must be corrected to the following noise certification

reference atmospheric conditions which shall be assumed to exist from the

surface to the aircraft altitude:

(1) Sea level pressure of 2116 psf (76 cm mercury).

(2) Ambient temperature of 77 degrees F (25 degrees C).

(3) Relative humidity of 70 percent.

(4) Zero wind.

(b) Reference test site. The reference test site is flat and without line-

of-sight obstructions across the flight path that encompasses the 10 dB down

points.

(c) Takeoff reference profile. (1) Figure H1 illustrates a typical takeoff

profile, including reference conditions.

(2) The reference flight path is defined as a straight line segment

inclined from the starting point (1640 feet prior to the center microphone

location at 65 feet above ground level) at an angleb defined by the

certificated best rate of climb and Vy for minimum engine performance. The

constant climb angle B is derived from the manufacturer's data (FAA-approved

by the FAA) to define the flight profile for the reference conditions. The

constant climb angle b is drawn through Cr and continues, crossing over

station A, to the position corresponding to the end of the type certification

takeoff path represented by position Ir.

(d) Level flyover reference profile. The beginning of the level flyover

reference profile is represented by helicopter position D (Figure H2). The

helicopter approaches position D in level flight 492 feet above ground level

as measured at station A. Airspeed is stabilized at either 0.9 VH or 0.45 VH

+ 65 knots (0.45 VH + 120 km/hr), whichever speed is less. Rotor speed is

stabilized at the maximum continuous RPM throughout the 10 dB down time

period. The helicopter crosses station A in level flight and proceeds to

position J.

(e) For noise certification purposes, VH is defined as the airspeed in

level flight obtained using the minimum specification engine torque

corresponding to maximum continuous power available for sea level, 25 deg. C

ambient conditions at the relevant maximum certificated weight. The value of

VH thus defined must be listed in the Rotorcraft Flight Manual.

(f) Approach reference profile. (1) Figure H3 illustrates approach profile,

including reference conditions.

(i) The beginning of the approach profile is represented by helicopter

position E. The position of the helicopter is recorded for a sufficient

distance (EK) to ensure recording of the entire interval during which the

measured helicopter noise level is within 10 dB of Maximum Tone Corrected

Perceived Noise Level (PNLTM), as required. EK represents a stable flight

condition in terms of torque, rpm, indicated airspeed, and rate of descent

resulting in a 6 deg. +/- 0.5 deg. approach angle.

(ii) The approach profile is defined by the approach angle b passing

directly over the station A at a height of AH, to position K, which

terminates the approach noise certification profile.

(2) The helicopter approaches position H along a constant 6 deg. approach

slope throughout the 10 dB down time period. The helicopter crosses position

E and proceeds along the approach slope crossing over station A until it

reaches position K.

Section H36.5 Symbols and units. The following symbols and units as used in

this appendix for helicopter noise certification have the following meanings.

Flight Profile Identification--Positions

Position                              Description

A         Location of the noise measuring point at the flight-track noise

measuring station vertically below the reference (takeoff,

flyover, or approach) flight path.

C         Start of noise certification takeoff flight path.

Cr        Start of noise certification reference takeoff flight path.

D         Start of noise certification flyover flight path.

Dr        Start of noise certification reference flyover path.

E         Start of noise certification approach flight path.

Er        Start of noise certification reference approach flight path.

F         Position on takeoff flight path directly above noise measuring

station A.

G         Position on flyover flight path directly above noise measuring

station A.

H         Position on approach flight path directly above noise measuring

station A.

I         End of noise type certification takeoff flight path.

Ir        End of noise type certification reference takeoff flight path.

J         End of noise type certification flyover flight path.

Jr        End of noise type certification reference flyover flight path.

K         End of noise certification approach type flight path.

Kr        End of noise type certification reference approach flight path.

L         Position on measured takeoff flight path corresponding to PNLTM at

station A.

Lr        Position on reference takeoff flight path corresponding to PNLTM of

station A.

M         Position on measured flyover flight path corresponding to PNLTM of

station A.

Mr        Position on reference flyover flight path corresponding to PNLTM of

station A.

N         Position on measured approach flight path corresponding to PNLTM at

station A.

Nr        Position on reference approach flight path corresponding to PNLTM

at station A.

S         Position on measured approach path nearest to station A.

Sr        Position on reference approach path nearest to station A.

T         Position on measured takeoff path nearest to station A.

Tr        Position on reference takeoff path nearest to station A.

Flight Profile Distances

Distance  Unit                             Meaning

AF        Feet  Takeoff Height. The vertical distance between helicopter and

station A.

AG        Feet  Flyover Height. The vertical distance between the helicopter

and station A.

AH        Feet  Approach Height. The vertical distance between the helicopter

and station A.

AL        Feet  Measured Takeoff Noise Path. The distance from station A to

the measured helicopter position L.

ALr       Feet  Reference Takeoff Noise Path. The distance from station A to

the reference helicopter position Lr.

AM        Feet  Measured Flyover Noise Path. The distance from station A to

the measured helicopter position M.

AMr       Feet  Reference Flyover Noise Path. The distance from station A to

helicopter position Mr on the reference flyover flight path.

AN        Feet  Measured Approach Noise Path. The distance from station A to

the measured helicopter noise position N.

ANr       Feet  Reference Approach Noise Path. The distance from station A to

the reference helicopter position Nr.

AS        Feet  Measured Approach Minimum Distance. The distance from station

A to the position S on the measured approach flight path.

ASr       Feet  Reference Approach Minimum Distance. The distance from

station A to the position Sr on the reference approach

flight path.

AT        Feet  Measured Takeoff Minimum Distance. The distance from station

A to the position T on the measured takeoff flight path.

ATr       Feet  Reference Takeoff Minimum Distance. The distance from station

A to the position Tr on the reference takeoff flight path.

CI        Feet  Takeoff Flight Path Distance. The distance from position C at

which the helicopter establishes a constant climb angle on

the takeoff flight path passing over station A and

continuing to position I at which the position of the

helicopter need no longer be recorded.

DJ        Feet  Flyover Flight Path Distance. The distance from position D at

which the helicopter is established on the flyover flight

path passing over station A and continuing to position J at

which the position of the helicopter need no longer be

recorded.

EK        Feet  Approach Flight Path Distance. The distance from position E

at which the helicopter establishes a constant angle on the

approach flight path passing over station A and continuing

to position K at which the position of the helicopter need

no longer be recorded.

Part B--Noise Measurement Under Sec. 36.801

Section H36.101 Noise certification test and measurement conditions.

(a) General. This section prescribes the conditions under which aircraft

noise certification tests must be conducted and the measurement procedures

that must be used to measure helicopter noise during each test.

(b) Test site requirements. (1) Tests to show compliance with established

helicopter noise certification levels must consist of a series of takeoffs,

level flyovers, and approaches during which measurement must be taken at

noise measuring stations located at the measuring points prescribed in this

section.

(2) Each takeoff test, flyover test, and approach test includes

simultaneous measurements at the flight-track noise measuring station

vertically below the reference flight path and at two sideline noise

measuring stations, one on each side of the reference flight track 492 feet

(150m) from, and on a line perpendicular to, the flight track of the noise

measuring station.

(3) The difference between the elevation of either sideline noise measuring

station may not differ from the flight-track noise measuring station by more

than 20 feet.

(4) Each noise measuring station must be surrounded by terrain having no

excessive sound absorption characteristics, such as might be caused by thick,

matted, or tall grass, shrubs, or wooded areas.

(5) During the period when the takeoff, flyover, or approach noise/time

record indicates the noise measurement is within 10 dB of PNLTM, no

obstruction that significantly influences the sound field from the aircraft

may exist--

(i) For any flight-track or sideline noise measuring station, within a

conical space above the measuring position (the point on the ground

vertically below the microphone), the cone being defined by an axis normal to

the ground and by half-angle 80 deg. from this axis; and

(ii) For any sideline noise measuring station, above the line of sight

between the microphone and the helicopter.

(6) If a takeoff or flyover test series is conducted at weights other than

the maximum takeoff weight for which noise certification is requested, the

following additional requirements apply:

(i) At least one takeoff test must be conducted at a weight at, or above,

the maximum certification weight.

(ii) Each test weight must be within +5 percent or -10 percent of the

maximum certification weight.

(iii) FAA-approved data must be used to determine the variation of EPNL

with weight for takeoff test conditions.

(7) Each approach test must be conducted with the aircraft stabilized and

following a 6.0 degree +/-0.5 degree approach angle and must meet the

requirements of section H36.107 of this part.

(8) If an approach test series is conducted at weights other than the

maximum landing weight for which certification is requested, the following

additional requirements apply:

(i) At least one approach test must be conducted at a weight at, or above,

the maximum landing weight.

(ii) Each test weight must exceed 90 percent of the maximum landing weight.

(iii) FAA-approved data must be used to determine the variation of EPNL

with weight for approach test conditions.

(9) Aircraft performance data sufficient to make the corrections required

under section H36.205 of this appendix must be recorded at an FAA-approved

sampling rate using FAA approved equipment.

(c) Weather restrictions. The tests must be conducted under the following

atmospheric conditions:

(1) No rain or other precipitation.

(2) Ambient air temperature between 36 deg. F and 95 deg. F (2.2 C deg. and

35 deg. C), inclusively, over that portion of the sound propagation path

between the aircraft and a point 10 meters above the ground at the noise

measuring station. The temperature and relative humidity measured at aircraft

altitude and at 10 meters above ground shall be averaged and used to adjust

for propagation path absorption.

(3) Relative humidity and ambient temperature over the portion of the sound

propagation path between the aircraft and a point 10 meters above the ground

at the noise measuring station is such that the sound attenuation in the one-

third octave band centered at 8 kHz is not greater than 12 dB/100 meters and

the relative humidity is between 20 percent and 95 percent, inclusively.

(4) Wind velocity as measured at 10 meters above ground does not exceed 10

knots (19 km/h) and the crosswind component does not exceed 5 knots (9 km/h).

The wind shall be determined using a continuous thirty-second averaging

period spanning the 10dB down time interval.

(5) No anomalous wind conditions (including turbulence) which will

significantly affect the noise level of the aircraft when the noise is

recorded at each noise measuring station.

(6) The wind velocity, temperature, and relative humidity measurements

required under the appendix must be measured in the vicinity of noise

measuring stations 10 meters above the ground. The location of the

meteorological measurements must be approved by the FAA as representative of

those atmospheric conditions existing near the surface over the geographical

area which aircraft noise measurements are made. In some cases, a fixed

meteorological station (such as those found at airports or other facilities)

may meet this requirement.

(7) Temperature and relative humidity measurements must be obtained within

25 minutes of each noise test measurement. Meteorological data must be

interpolated to actual times of each noise measurement.

(d) Aircraft testing procedures. (1) The aircraft testing procedures and

noise measurements must be conducted and processed in a manner which yields

the noise evaluation measure designated as Effective Perceived Noise Level

(EPNL) in units of EPNdB, as prescribed in Appendix B of this part.

(2) The aircraft height and lateral position relative to the centerline of

the reference flight-track (which passes through the noise measuring point)

must be determined by an FAA approved method which is independent of normal

flight instrumentation, such as radar tracking, theodolite triangulation,

laser trajectography, or photographic scaling techniques.

(3) The aircraft position along the flight path must be related to the

noise recorded at the noise measuring stations by means of synchronizing

signals at an approved sampling rate. The position of the aircraft must be

recorded relative to the runway during the entire time period in which the

recorded signal is within 10 dB of PNLTM. Measuring and sampling equipment

must be approved by the FAA.

Section H36.103 Takeoff test conditions.

(a) This section, in addition to the applicable requirements of sections

H36.101 and H36.205(b) of this appendix, applies to all takeoff noise tests

conducted under this appendix to show compliance with Part 36.

(b) A test series must consist of at least six flights over the flight-

track noise measuring station (with simultaneous measurements at all three

noise measuring stations) as follows:

(1) An airspeed of either Vy +/-5 knots or the lowest approved speed +/-5

knots for the climb after takeoff, whichever speed is greater, must be

established during the horizontal portion of each test flight and maintained

during the remainder of the test flight.

(2) The horizontal portion of each test flight must be conducted at an

altitude of 65 feet (20 meters) above the ground level at the flight-track

noise measuring station.

(3) Upon reaching a point 1,640 feet (500 meters) from the noise measuring

station, the helicopter shall be stabilized at:

(i) The torque used to establish the takeoff distance for an ambient

temperature at sea level of 25 deg. C for helicopters for which the

determination of takeoff performance is required by airworthiness

regulations; or

(ii) The torque corresponding to minimum installed power available for an

ambient temperature at sea level of 25 deg. C for all other helicopters.

(4) The helicopter shall be maintained throughout the takeoff reference

procedure at:

(i) The speed used +/-5 knots to establish takeoff distance for an ambient

temperature at sea level of 25 deg. C for helicopters for which the

determination of takeoff performance is required by airworthiness

regulations; or

(ii) The best rate of climb speed Vy +/-5 knots, or the lowest approved

speed for climb after takeoff, whichever is greater, for an ambient

temperature at sea level of 25 deg. C for all other helicopters.

(5) The rotor speed must be stabilized at the normal operating RPM (+/-1%)

during the entire period of the test flight when the measured helicopter

noise level is within 10 dB of PNLTM.

(6) The helicopter must pass over the flight-track noise measuring station

within +/-10 deg. from the zenith.

Section H36.105 Flyover test conditions.

(a) This section, in addition to the applicable requirements of sections

H36.101 and H36.205(c) of this appendix, applies to all flyover noise tests

conducted under this appendix to show compliance with Part 36.

(b) A test series must consist of at least six flights (three in each

direction) over the flight-track noise measuring station (with simultaneous

measurements at all three noise measuring stations)--

(1) In level flight;

(2) At a height of 492 feet +/-30 feet (150+/-9 meters) above the ground

level at the flight-track noise measuring station; and

(3) Within +/-5 deg. from the zenith.

(c) Each flyover noise test must be conducted--

(1) At a speed of 0.9 VH or 0.45 VH+120 km/hr (0.45 VH+65 kt), whichever is

less, maintained throughout the measured portion of the flyover;

(2) At rotor speed stabilized at the normal operating rotor RPM (+/-1

percent); and

(3) With the power stabilized during the period when the measured

helicopter noise level is within 10 dB of PNLTM.

(d) The airspeed shall not vary from the reference airspeed by more than

+/-5 knots (9 km/hr).

Section H36.107 Approach test conditions.

(a) This section, in addition to the requirements of sections H36.101 and

H36.205(d) of this appendix, applies to all approach tests conducted under

this appendix to show compliance with Part 36.

(b) A test series must consist of at least six flights over the flight-

track noise measuring station (with simultaneous measurements at the three

noise measuring stations)--

(1) On an approach slope of 6 deg.+/-0.5 deg.;

(2) At a height of 394+/-30 feet (120+/-9 meters) above the ground level at

the flight-track noise measuring station;

(3) Within +/-10 deg. of the zenith;

(4) At stabilized airspeed equal to the certificated best rate of climb Vy,

or the lowest approved speed for approach, whichever is greater, with power

stabilized during the approach and over the flight path reference point, and

continued to a normal touchdown; and

(5) At rotor speed stabilized at the maximum normal operating rotor RPM

(+/-1 percent).

(c) The airspeed shall not vary from the reference airspeed by more than

+/-5 knots (+/-9 km/hr).

Section H36.109 Measurement of helicopter noise received on the ground.

(a) General. (1) The measurements prescribed in this section provide the

data needed to determine the one-third octave band noise produced by an

aircraft during testing, at specific noise measuring stations, as a function

of time.

(2) Sound pressure level data for aircraft noise certification purposes

must be obtained with FAA-approved acoustical equipment and measurement

practices.

(3) Paragraphs (b), (c), and (d) of this section prescribe the required

equipment specifications. Paragraphs (e) and (f) prescribe the calibration

and measurement procedures required for each certification test series.

(b) Measurement system. The acoustical measurement system must consist of

FAA-approved equipment equivalent to the following:

(1) A microphone system with frequency response and directivity which are

compatible with the measurement and analysis system accuracy prescribed in

paragraph (c) of this section.

(2) Tripods or similar microphone mountings that minimize interference with

the sound energy being measured.

(3) Recording and reproducing equipment, the characteristics, frequency

response, and dynamic range of which are compatible with the response and

accuracy requirements of paragraph (c) of this section.

(4) Calibrators using sine wave, or pink noise, of known levels. When pink

noise (defined in paragraph (e)(1) of this section) is used, the signal must

be described in terms of its root-mean-square (rms) value.

(5) Analysis equipment with the response and accuracy which meets or

exceeds the requirements of paragraph (d) of this section.

(6) Attenuators used for range changing in sensing, recording, reproducing,

or analyzing aircraft sound must be capable of being operated in equal-

interval decibel steps with no error between any two settings which exceeds

0.2 dB.

(c) Sensing, recording, and reproducing equipment. (1) The sound produced

by the aircraft must be recorded in such a way that the complete information,

including time history, is retained. A magnetic tape recorder is acceptable.

(2) The microphone must be a pressure-sensitive capacitive type, or its

FAA-approved equivalent, such as a free-field type with incidence corrector.

(i) The variation of microphone and preamplifier system sensitivity within

an angle of +/-30 degrees of grazing (60-120 degrees from the normal to the

diaphragm) must not exceed the following values:

Change in

sensitivity

Frequency (Hz)      (dB)

45 to 1,120                1

1,120 to 2,240           1.5

2,240 to 4,500           2.5

4,500 to 7,100             4

7,100 to 11,200            5

With the windscreen in place, the sensitivity variation in the plane of the

microphone diaphragm shall not exceed 1.0 dB over the frequency range 45 to

11,200 Hz.

(ii) The overall free-field frequency response at 90 degrees (grazing

incidence) of the combined microphone (including incidence corrector, if

applicable) preamplifier, and windscreen must be determined by using either

(A) an electrostatic calibrator in combination with manufacturer-provided

corrections, or (B) an anechoic free-field facility. The calibration unit

must include pure tones at each preferred one-third octave frequency from 50

Hz to 10,000 Hz. The frequency response (after corrections based on that

determination) must be flat and within the following tolerances:

44-3,549 Hz........................................................+/-0.25 dB

3,550-7,099 Hz......................................................+/-0.5 dB

7,100-11,200 Hz.....................................................+/-1.0 dB

(iii) Specifications concerning sensitivity to environmental factors such

as temperature, relative humidity, and vibration must be in conformity with

the recommendations of International Electrotechnical Commission (IEC)

Publication No. 179, entitled "Precision Sound Level Meters", as incorporated

by reference under Sec. 36.6 of this part.

(iv) If the wind speed exceeds 6 knots, a windscreen must be employed with

the microphone during each measurement of aircraft noise. Correction for any

insertion loss produced by the windscreen, as a function of frequency, must

be applied to the measured data and any correction applied must be reported.

(3) If a magnetic tape recorder is used to store data for subsequent

analysis, the record/replay system (including tape) must conform to the

following:

(i) The electric background noise produced by the system in each one-third

octave must be at least 35 dB below the standard recording level, which is

defined as the level that is either 10 dB below the 3 percent harmonic

distortion level for direct recording or +/-40 percent deviation for

frequency modulation (FM) recording.

(ii) At the standard recording level, the corrected frequency response in

each selected one-third octave band between 44 Hz and 180 Hz must be flat and

within +/-0.75 dB, and in each band between 180 Hz and 11,200 Hz must be flat

and within +/-0.25 dB.

(iii) If the overall system satisfies the requirements of paragraph

(c)(2)(ii) of this section, and if the limitations of the dynamic range of

the equipment are insufficient to obtain adequate spectral information, high

frequency pre-emphasis may be added to the recording channel with the

converse de-emphasis on playback. If pre-emphasis is added, the

instantaneously recorded sound-pressure level between 800 Hz and 11,200 Hz of

the maximum measured noise signal must not vary more than 20 dB between the

levels of the maximum and minimum one-third octave bands.

(d) Analysis equipment. (1) A frequency analysis of the acoustic signal

must be performed using one-third octave filters which conform to the

recommendations of International Electrotechnical Commission (IEC)

Publication No. 225, entitled "Octave, Half-Octave, and Third-Octave Band

Filters Intended for Analysis of Sound and Vibrations," as incorporated by

reference under Sec. 36.6 of this part.

(2) A set of 24 consecutive one-third octave filters must be used. The

first filter of the set must be centered at a geometric mean frequency of 50

Hz and the last filter at 10,000 Hz. The output of each filter must contain

less than 0.5 dB ripple.

(3) The analyzer indicating device may be either analog or digital, or a

combination of both. The preferred sequence of signal processing is:

(i) Squaring the one-third octave filter outputs;

(ii) Averaging or integrating; and

(iii) Converting linear formulation to logarithmic.

(4) Each detector must operate over a minimum dynamic range of 60 dB and

perform as a root-mean-square device for sinusoidal tone bursts having crest

factors of at least 3 over the following dynamic range:

(i) Up to 30 dB below full-scale reading must be accurate within +/-0.5 dB;

(ii) Between 30 dB and 40 dB below full-scale reading must be accurate

within +/-1.0 dB; and

(iii) In excess of 40 dB below full-scale reading must be accurate within

+/-2.5 dB.

(5) The averaging properties of the integrator must be tested as follows:

(i) White noise must be passed through the 200 Hz one-third octave band

filter and the output fed in turn to each detector/integrator. The standard

deviation of the measured levels must then be determined from a statistically

significant number of samples of the filtered white noise taken at intervals

of not less than 5 seconds. The value of the standard deviation must be

within the interval 0.48+/-0.06 dB for a probability limit of 95 percent. An

approved equivalent method may be substituted for this test on those

analyzers where the test signal cannot readily be fed directly to each

detector/integrator.

(ii) For each detector/integrator, the response to a sudden onset or

interruption of a constant amplitude sinusoidal signal at the respective one-

third octave band center frequency must be measured at sampling times 0.5,

1.0, 1.5, and 2.0 seconds after the onset or interruption. The rising

responses must be in the following amounts before the steady-state level:

0.5 seconds, 4.0+/-1.0 dB

1.0 seconds, 1.75+/-0.5 dB

1.5 seconds, 1.0+/-0.5 dB

2.0 seconds, 0.6+/-0.25 dB

(iii) The falling response must be such that the sum of the decibel

readings below the initial steady-state level, and the corresponding rising

response reading is   6.5+/- 1.0 dB, at both 0.5 and 1.0 seconds and, on

subsequent records, the sum of the onset plus decay must be greater than 7.5

decibels.

Note 1: For analyzers with linear detection, an approximation of this

response would be given by:

SPL (i, k)-10 log  [0.17 (100.1 (Li, k-3))

+10.21 (00.1 (Li, k-2))

+0.24 (100.1 (Li, k-1))

+0.33 (100.1 (Li, k))]

When this approximation is used, the calibration signal should be

established without this weighting.

Note 2: Some analyzers have been shown to have signal sampling rates that

are insufficiently accurate to detect signals with crest factor ratios

greater than three which is common to helicopter noise. Preferably, such

analyzers should not be used for helicopter certification. Use of analysis

systems with high signal sampling rates (greater than 40KHz) or those with

analog detectors prior to digitization at the output of each one-third octave

filter is encouraged.

(iv) Analyzers using true integration cannot meet the requirements of (i),

(ii), and (iii) directly, because their overall average time is greater than

the sampling interval. For these analyzers, compliance must be demonstrated

in terms of the equivalent output of the data processor. Further, in cases

where readout and resetting require a dead-time during acquisition, the

percentage loss of the total data must not exceed one percent.

(6) The sampling interval between successive readouts shall not exceed 500

milliseconds and its precise value must be known to within +/-1 one percent.

The instant in time by which a readout is characterized shall be the midpoint

of the average period where the averaging period is defined as twice the

effective time constant of the analyzer.

(7) The amplitude resolution of the analyzer must be at least 0.25 dB.

(8) After all systematic errors have been eliminated, each output level

from the analyzer must be accurate within +/-1.0 dB of the level of the input

signal. The total systematic errors for each of the output levels must not

exceed +/-3.0 dB. For contiguous filter systems, the systematic corrections

between adjacent one-third octave channels must not exceed 4.0 dB.

(9) The dynamic range capability of the analyzer to display a single

aircraft noise event, in terms of the difference between full-scale output

level and the maximum noise level of the analyzer equipment, must be at least

60 dB.

(e) Calibrations. (1) Within five days prior to beginning each test series,

the complete electronic system, as installed in field including cables, must

be electronically calibrated for frequency and amplitude by the use of a pink

noise signal of known amplitudes covering the range of signal levels

furnished by the microphone. For purposes of this section, "pink noise" means

a noise whose noise-power/unit-frequency is inversely proportional to

frequency at frequencies within the range of 44 Hz to 11,200 Hz. The signal

used must be described in terms of its average root-mean-square (rms) values

for a nonoverload signal level. This system calibration must be repeated

within five days of the end of each test series, or as required by the FAA.

(2) Immediately before and after each day's testing, a recorded acoustic

calibration of the system must be made in the field with an acoustic

calibrator to check the system sensitivity and provide an acoustic reference

level for the sound level data analysis. The performance of equipment in the

system will be considered satisfactory if, during each day's testing, the

variation in the calibration value does not exceed 0.5 dB.

(3) A normal incidence pressure calibration of the combined microphone/

preamplifier must be performed with pure tones at each preferred one-third

octave frequency from 50 Hz to 10,000 Hz. This calibration must be completed

within 90 days prior to the beginning of each test series.

(4) Each reel of magnetic tape must:

(i) Be pistonphone calibrated; and

(ii) At its beginning and end, carry a calibration signal consisting of at

least a 15 second burst of pink noise, as defined in paragraph (e)(1) of this

section.

(5) Data obtained from tape recorded signals are not considered reliable if

the difference between the pink noise signal levels, before and after the

tests in each one-third octave band, exceeds 0.75 dB.

(6) The one-third octave filters must have been demonstrated to be in

conformity with the recommendations of IEC Publication 225 as incorporated by

reference under Sec. 36.6 of this part, during the six calendar months

preceding the beginning of each test series. However, the correction for

effective bandwidth relative to the center frequency response may be

determined for each filter by:

(i) Measuring the filter response to sinusoidal signals at a minimum of

twenty frequencies equally spaced between the two adjacent preferred one-

third octave frequencies; or

(ii) Using an FAA approved equivalent technique.

(7) A performance calibration analysis of each piece of calibration

equipment, including pistonphones, reference microphones, and voltage insert

devices, must have been made during the six calendar months preceding the

beginning of each day's test series. Each calibration must be traceable to

the National Bureau of Standards.

(f) Noise measurement procedures. (1) Each microphone must be oriented so

that the diaphragm is substantially in the plane defined by the flight path

of the aircraft and the measuring station. The microphone located at each

noise measuring station must be placed so that its sensing element is

approximately 4 feet above ground.

(2) Immediately before and immediately after each series of test runs and

each day's testing, acoustic calibrations of the system prescribed in this

section of this appendix must be recorded in the field to check the acoustic

reference level for the analysis of the sound level data. Ambient noise must

be recorded for at least 10 seconds and be representative of the acoustical

background, including system noise, that exists during the flyover test run.

During that recorded period, each component of the system must be set at the

gain-levels used for aircraft noise measurement.

(3) The mean background noise spectrum must contain the sound pressure

levels, which, in each preferred third octave band in the range of 50 Hz to

10,000 Hz, are the averages of the energy of the sound pressure levels in

every preferred third octave. When analyzed in PNL, the resulting mean

background noise level must be at least 20 PNdB below the maximum PNL of the

helicopter.

(4) Corrections for recorded levels of background noise are allowed, within

the limits prescribed in section H36.111(c)(3) of this appendix.

Section H36.111  Reporting and correcting measured data

(a) General. Data representing physical measurements, and corrections to

measured data, including corrections to measurements for equipment response

deviations, must be recorded in permanent form and appended to the record.

Each correction must be reported and is subject to FAA approval. An estimate

must be made of each individual error inherent in each of the operations

employed in obtaining the final data.

(b) Data reporting. (1) Measured and corrected sound pressure levels must

be presented in one-third octave band levels obtained with equipment

conforming to the standards prescribed in section H36.109 of this appendix.

(2) The type of equipment used for measurement and analysis of all

acoustic, aircraft performance, and meteorological data must be reported.

(3) The atmospheric environmental data required to demonstrate compliance

with this appendix, measured throughout the test period, must be reported.

(4) Conditions of local topography, ground cover, or events which may

interfere with sound recording must be reported.

(5) The following aircraft information must be reported:

(i) Type, model, and serial numbers, if any, of aircraft engines and

rotors.

(ii) Gross dimensions of aircraft and location of engines.

(iii) Aircraft gross weight for each test run.

(iv) Aircraft configuration, including landing gear positions.

(v) Airspeed in knots.

(vi) Helicopter engine performance as determined from aircraft instruments

and manufacturer's data.

(vii) Aircraft flight path, above ground level in feet, determined by an

FAA approved method which is independent of normal flight instrumentation,

such as radar tracking, theodolite triangulation, laser trajectography, or

photographic scaling techniques.

(6) Aircraft speed, and position, and engine performance parameters must be

recorded at an approved sampling rate sufficient to correct to the noise

certification reference test conditions prescribed in section H36.3 of this

appendix. Lateral position relative to the reference flight-track must be

reported.

(c) Data corrections. (1) Aircraft position, performance data and noise

measurement must be corrected to the noise certification reference conditions

as prescribed in sections H36.3 and H36.205 of this appendix.

(2) The measured flight path must be corrected by an amount equal to the

difference between the applicant's predicted flight path for the

certification reference conditions and the measured flight path at the test

conditions. Necessary corrections relating to aircraft flight path or

performance may be derived from FAA-approved data for the difference between

measured and reference engine conditions, together with appropriate

allowances for sound attenuation with distance. The Effective Perceived Noise

Level (EPNL) correction must be less than 2.0 EPNdB for any combination of

the following:

(i) The aircraft's not passing vertically above the measuring station.

(ii) Any difference between the reference flight-track and the actual

minimum distance of the aircraft's ILS antenna from the approach measuring

station.

(iii) Any difference between the actual approach angle and the noise

certification reference approach flight path.

(iv) Any correction of the measured level flyover noise levels which

accounts for any difference between the test engine thrust or power and the

reference engine thrust or power.

Detailed correction requirements are prescribed in section H36.205 of this

appendix.

(3) Aircraft sound pressure levels within the 10 dB-down points must exceed

the mean background sound pressure levels determined under section

A36.3(f)(3) by at least 5 dB in each one-third octave band or be corrected

under an FAA approved method to be included in the computation of the overall

noise level of the aircraft. An EPNL may not be computed or reported from

data from which more than four one-third octave bands in any spectrum within

the 10 dB-down points have been excluded under this paragraph.

(d) Validity of results. (1) The test results must produce three average

EPNL values within the 90 percent confidence limits, each value consisting of

the arithmetic average of the corrected noise measurements for all valid test

runs at the takeoff, level flyovers, and approach conditions. The 90 percent

confidence limit applies separately to takeoff, flyover, and approach.

(2) The minimum sample size acceptable for each takeoff, approach, and

flyover certification measurements is six. The number of samples must be

large enough to establish statistically for each of the three average noise

certification levels a 90 percent confidence limit which does not exceed +/-

1.5 EPNdB. No test result may be omitted from the averaging process, unless

otherwise specified by the FAA.

(3) To comply with this appendix, a minimum of six takeoffs, six

approaches, and six level flyovers is required. To be counted toward this

requirement, each flight event must be validly recorded at all three noise

measuring stations.

(4) The approved values of VH and Vy used in calculating test and reference

conditions and flight profiles must be reported along with measured and

corrected sound pressure levels.

Section H36.113  Atmospheric attenuation of sound.

(a) The values of the one-third octave band spectra measured during

helicopter noise certification tests under this appendix must conform, or be

corrected, to the reference conditions prescribed in section H36.3(a). Each

correction must account for any differences in the atmospheric attenuation of

sound between the test-day conditions and the reference-day conditions along

the sound propagation path between the aircraft and the microphone. Unless

the meteorological conditions are within the test window prescribed in this

appendix, the test data are not acceptable.

(b) Attenuation rates. The atmospheric attenuation rates of sound with

distance for each one-third octave band from 50 Hz to 10,000 Hz must be

determined in accordance with the formulations and tabulations of SAE ARP

866A, entitled "Standard Values of Atmospheric Absorption as a Function of

Temperatures and Humidity for Use in Evaluating Aircraft Flyover Noise", as

incorporated by reference under Sec. 36.6 of this part.

(c) Correction for atmospheric attenuation. (1) EPNL values calculated for

measured data must be corrected whenever--

(i) The ambient atmospheric conditions of temperature and relative humidity

do not conform to the reference conditions, 77  deg.F and 70%, respectively,

or

(ii) The measured flight paths do not conform to the reference flight

paths.

(iii) The temperature and relative humidity measured at aircraft altitude

and at 10 meters above the ground shall be averaged and used to adjust for

propagation path absorption.

(2) The mean attenuation rate over the complete sound propagation path from

the aircraft to the microphone must be computed for each one-third octave

band from 50 Hz to 10,000 Hz. These rates must be used in computing the

corrections required in section H36.111(d) of this appendix.

Part C--Noise Evaluation and Calculation Under Sec. 36.803

Section H36.201  Noise Evaluation in EPNdB.

(a) Effective Perceived Noise Level (EPNL), in units of effective perceived

noise decibels (EPNdB), shall be used for evaluating noise level values under

Sec. 36.803 of this part. Except as provided in paragraph (b) of this

section, the procedures in Appendix B of Part 36 must be used for computing

EPNL. Appendix B includes requirements governing determination of noise

values, including calculations of:

(1) Instantaneous perceived noise levels;

(2) Corrections for spectral irregularities;

(3) Tone corrections;

(4) Duration corrections;

(5) Effective perceived noise levels; and

(6) Mathematical formulation of noy tables.

(b) Notwithstanding the provisions of section B36.5(a), for helicopter

noise certification, corrections for spectral irregularities shall start with

the corrected sound pressure level in the 50 Hz one-third octave band.

Section H36.203  Calculation of noise levels.

(a) To demonstrate compliance with the noise level limits of section

H36.305, the noise values measured simultaneously at the three noise

measuring points must be arithmetically averaged to obtain a single EPNdB

value for each flight.

(b) The calculated noise level for each noise test series, i.e., takeoff,

flyover, or approach must be the numerical average of at least six separate

flight EPNdB values. The 90 percent confidence limit for all valid test runs

under section H36.111(d) of this appendix applies separately to the EPNdB

values for each noise test series.

Section H36.205  Detailed data correction procedures

(a) General. If the test conditions do not conform to those prescribed as

noise certification reference conditions under section H36.305 of this

appendix, the following correction procedure shall apply:

(1) If a positive value results from any difference between reference and

test conditions, an appropriate positive correction must be made to the EPNL

calculated from the measured data. Conditions which can result in a positive

value include:

(i) Atmospheric absorption of sound under test conditions which is greater

than the reference;

(ii) Test flight path at an altitude which is higher than the reference; or

(iii) Test weight which is less than maximum certification weight.

(2) If a negative value results from any difference between reference and

test conditions, no correction may be made to the EPNL calculated from the

measured data, unless the difference results from:

(i) An atmospheric absorption of sound under test conditions which is less

than the reference; or

(ii) A test flight path at an altitude which is lower than the reference.

(3) The following correction procedures may produce one or more possible

correction values which must be added algebraically to the calculated EPNL to

bring it to reference conditions:

(i) The flight profiles must be determined for both reference and test

conditions. The procedures require noise and flight path recording with a

synchronized time signal from which the test profile can be delineated,

including the aircraft position for which PNLTM is observed at the noise

measuring station. For takeoff, the flight profile corrected to reference

conditions may be derived from FAA approved manufacturer's data.

(ii) The sound propagation paths to the microphone from the aircraft

position corresponding to PNLTM are determined for both the test and

reference profiles. The SPL values in the spectrum of PNLTM must then be

corrected for the effects of--

(A) Change in atmospheric sound absorption;

(B) Atmospheric sound absorption on the linear difference between the two

sound path lengths; and

(C) Inverse square law on the difference in sound propagation path length.

The corrected values of SPL are then converted to PNLTM from which PNLTM must

be subtracted. The resulting difference represents the correction which must

be added algebraically to the EPNL calculated from the measured data.

(iii) The minimum distances from both the test and reference profiles to

the noise measuring station must be calculated and used to determine a noise

duration correction due to any change in the altitude of aircraft flyover.

The duration correction must be added algebraically to the EPNL calculated

from the measured data.

(iv) From FAA approved data in the form of curves or tables giving the

variation of EPNL with rotor rpm and test speed, corrections are determined

and must be added to the EPNL, which is calculated from the measured data to

account for noise level changes due to differences between test conditions

and reference conditions.

(v) From FAA approved data in the form of curves or tables giving the

variation of EPNL with approach angle, corrections are determined and must be

added algebraically to the EPNL, which is calculated from measured data, to

account for noise level changes due to differences between the 6 degree and

the test approach angle.

(b) Takeoff profiles. (1) Figure H1 illustrates a typical takeoff profile,

including reference conditions.

(i) The reference takeoff flight path is described in section H36.3(c).

(ii) The test parameters are functions of the helicopter's performance and

weight and the atmospheric conditions of temperature, pressure, wind velocity

and direction.

(2) For the actual takeoff, the helicopter approaches position C in level

flight at 65 feet (20 meters) above ground level at the flight track noise

measuring station and at either Vy+/-5 knots (+/-9 km/hr) or the maximum

speed of the curve tangential at the ordinate of the height-speed envelope

plus 3.0 knots (+/-5 knots), whichever speed is greater. Rotor speed is

stabilized at the normal operating RPM (+/-1 percent), specified in the

flight manual. The helicopter is stabilized in level flight at the speed for

best rate of climb using minimum engine specifications (power or torque and

rpm) along a path starting from a point located 1640 feet (500 meters)

forward of the flight-track noise measuring station and 65 feet (20 meters)

above the ground. Starting at point B, the helicopter climbs through point C

to the end of the noise certification takeoff flight path represented by

position I. The position of point C may vary within limits allowed by the

FAA. The position of the helicopter shall be recorded for a distance (CI)

sufficient to ensure recording of the entire interval during which the

measured helicopter noise level is within 10 dB of PNLTM, as required by this

rule. Station A is the flight-track noise measuring station. The

relationships between the measured and corrected takeoff flight profiles can

be used to determine the corrections which must be applied to the EPNL

calculated from the measured data.

(3) Figure H1 also illustrates the significant geometrical relationships

influencing sound propagation. Position L represents the helicopter location

on the measured takeoff flight path from which PNLTM is observed at station

A, and Lr is the A and Nr corresponding position on the reference sound

propagation path. AL and ALr both form the angle F with their respective

flight paths. Position T represents the point on the measured takeoff flight

path nearest station A, and Tr is the corresponding position on the reference

flight path. The minimum distance to the measured and reference flight paths

are indicated by the lines AT and ATr, respectively, which are normal to

their flight paths.

(c) Level flyover profiles. (1) The noise type certification level flyover

profile is shown in Figure H2. Airspeed must be stabilized within +/-5 knots

of the reference airspeed given in section H36.3(d). For each run, the

difference between airspeed and ground speed shall not exceed 10 knots

between the 10 dB down points. Rotor speed must be stabilized at the maximum

continuous RPM within one percent, throughout the 10 dB down time period. If

the test requirements are otherwise met, flight direction may be reversed for

each subsequent flyover, to obtain three test runs in each direction.

Figure H1. Comparison of Measured and Corrected Takeoff Profiles

[ ...Illustrations appear here... ]

Figure H2. Comparison of Measured and Corrected Flyover Profiles

[ ...Illustrations appear here... ]

(2) Figure H2 illustrates comparative flyover profiles when test conditions

do not conform to prescribed reference conditions. The position of the

helicopter shall be recorded for a distance (DJ) sufficient to ensure

recording of the entire interval during which the measured helicopter noise

level is within 10 dB of PNLTM, as required. The flyover profile is defined

by the height AG which is a function of the operating conditions controlled

by the pilot. Position M represents the helicopter location on the measured

flyover flight path for which PNLTM is observed at station A, and Mr is the

corresponding position on the reference flight path.

(d) Approach profiles. (1) Figure H3 illustrates a typical approach

profile, including reference conditions.

(2) The helicopter approaches position H along a 6 deg. (+/-0.5 deg.)

average approach slope throughout the 10 dB down period. The approach

procedure shall be acceptable to the FAA and shall be included in the Flight

Manual.

(3) Figure H3 illustrates portions of the measured and reference approach

flight paths including the significant geometrical relationships influencing

sound propagation. EK represents the measured approach path with approach

angle h, and Er and Kr represent the reference approach angle of 6 deg..

Position N represents the helicopter location on the measured approach flight

path for which PNLTM is observed at station A, and Nr is the corresponding

position on the reference approach flight path. The measured and corrected

noise propagation paths are AN and ANr, respectively, both of which form the

same angle with their flight paths. Position S represents the point on the

measured approach flight path nearest station A, and Sr is the corresponding

point on the reference approach flight path. The minimum distance to the

measured and reference flight paths are indicated by the lines AS and ASr,

respectively, which are normal to their flight paths.

Figure H3. Comparison of Measured and Corrected Approach Profiles

[ ...Illustration appears here... ]

(e) Correction of noise at source during level flyover. (1) For level

overflight, if any combination of the following three factors, 1) airspeed

deviation from reference, 2) rotor speed deviation from reference, and 3)

temperature deviation from reference, results in an advancing blade tip Mach

number which deviates from the reference Mach value, then source noise

adjustments shall be determined. This adjustment shall be determined from the

manufacturer supplied data approved by the FAA.

(2) Off-reference tip Mach number adjustments shall be based upon a

sensitivity curve of PNLTM versus advancing blade tip Mach number, deduced

from overflights carried out at different airspeeds around the reference

airspeed. If the test aircraft is unable to attain the reference value, then

an extrapolation of the sensitivity curve is permitted if data cover at least

a range of 0.3 Mach units. The advancing blade tip Mach number shall be

computed using true airspeed, onboard outside air temperature, and rotor

speed. A separate PNLTM versus advancing blade tip Mach number function shall

be derived for each of the three certification microphone locations, i.e.,

centerline, sideline left, and sideline right. Sideline left and right are

defined relative to the direction of the flight on each run. PNLTM

adjustments are to be applied to each microphone datum using the appropriate

PNLTM function.

(f) PNLT corrections. If the ambient atmospheric conditions of temperature

and relative humidity are not those prescribed as reference conditions under

this appendix (77 degrees F and 70 percent, respectively), corrections to the

EPNL values must be calculated from the measured data under paragraph (a) of

this section as follows:

(1) Takeoff flight path. For the takeoff flight path shown in Figure H1,

the spectrum of PNLTM observed at station A for the aircraft at position Lr

is decomposed into its individual SPLi values.

(i) Step 1. A set of corrected values are then computed as follows:

SPLic = SPLi + (a i-a io)AL

+ (a io)AL-ALr)

+ 20 log(AL/ALr)

Where SPLi and SPLic are the measured and corrected sound pressure levels,

respectively, in the i-th one-third octave band. The first correction term

accounts for the effects of change in atmospheric sound absorption where ai

and aio are the sound absorption coefficients for the test and reference

atmospheric conditions, respectively, for the -ith one-third octave band and

LrA is the measured takeoff sound propagation path. The second correction

term accounts for the effects of atmospheric sound absorption on the change

in the sound propagation path length where LrA is the corrected takeoff sound

propagation path. The third correction term accounts for the effects of the

inverse square law on the change in the sound propagation path length.

(ii) Step 2. The corrected values of the SPLic are then converted to PNLT

and a correction term calculated as follows:

D1=PNLT-PNLTM

Which represents the correction to be added algebraically to the EPNL

calculated from the measured data.

(2) Approach flight path. (i) The procedure described in paragraph (f)(1)

of this section for takeoff flight paths is also used for the approach flight

path, except that the value for SPLic relate to the approach sound

propagation paths shown in Figure H3 as follows:

SPLic = SPLi+(a-aio) AM+

a (AM-AMr)+ 20 log (AM/AMr)

Where the lines NS and NrSr are the measured and referenced approach sound

propagation paths, respectively.

(ii) The remainder of the procedure is the same as that prescribed in

paragraph (d)(1)(ii) of this section, regarding takeoff flight path.

(3) Sideline microphones. The procedure prescribed in paragraph (f)(1) of

this section for takeoff flight paths is also used for the propagation to the

sideline microphones, except that the values of SPLic relate only in the

measured sideline sound propagation path as follows:

SPLic - SPLi + (aio-a+io)KX

+ aio (KX-KXr) + 20 log (KX/KXr)

K is the sideline measuring station where

X=L and Xr=Ln for takeoff

X=M and Xr=Mn for approach

X=N and Xr=Nr for flyover

(4) Level flyover flight path. The procedure prescribed in paragraph (f)(1)

of this section for takeoff flight paths is also used for the level flyover

flight path, except that the values of SPLic relate only to the flyover sound

propagation paths as follows:

SPLic=SPLi+(a-aio) AN + aio (AN-ANr)+20 log (AN/ANr)

(g) Duration corrections. (1) If the measured takeoff and approach flight

paths do not conform to those prescribed as the corrected and reference

flight paths, respectively, under section A36.5(d)(2) it will be necessary to

apply duration corrections to the EPNL values calculated from the measured

data. Such corrections must be calculated as follows:

(i) Takeoff flight path. For the takeoff flight path shown in Figure H1,

the correction term is calculated using the formula--

D 2 = -10 log (AT/ATr) + 10 log (V/Vr)

which represents the correction which must be added algebraically to the EPNL

calculated from the measured data. The lengths AT and ATr are the measured

and corrected takeoff minimum distances from the noise measuring station A to

the measured and the corrected flight paths, respectively. A negative sign

indicates that, for the particular case of a duration correction, the EPNL

calculated from the measured data must be reduced if the measured flight path

is at greater altitude than the corrected flight path.

(ii) Approach flight path. For the approach flight path shown in Figure H3,

the correction term is calculated using the formula--

D 2 = -10 log (AS/ASr) + 10 log (V/Vr)

where AS is the measured approach minimum distance from the noise measuring

station A to the measured flight path and 394 feet is the minimum distance

from station A to the reference flight path.

(iii) Sideline microphones. For the sideline flight path, the correction

term is calculated using the formula--

D 2 = -10 log (KX/KXr)+10 log (V/Vr)

K is the sideline measuring station

where X=T and Xr=Tr for takeoff

where X=S and Xr=Sr for approach

where X=G and Xr=Gr for flyover

(iv) Level flyover flight paths. For the level flyover flight path, the

correction term is calculated using the formula--

D 2 = -10 log (AG/AGr)+10 log (V/Vr)

where AG is the measured flyover altitude over the noise measuring station A.

(2) The adjustment procedure described in this section shall apply to the

sideline microphones in the take-off, overflight, and approach cases.

Although the noise emission is strongly dependent on the directivity pattern,

variable from one helicopter type to another, the propagation angle u shall

be the same for test and reference flight paths. The elevation angle c shall

not be constrained but must be determined and reported. The certification

authority shall specify the acceptable limitations on c. Corrections to data

obtained when these limits are exceeded shall be applied using FAA approved

procedures.

Part D--Noise Limits Under Sec. 36.805

Section H36.301  Noise measurement, evaluation, and calculation

Compliance with this part of this appendix must be shown with noise levels

measured, evaluated, and calculated as prescribed under Parts B and C of this

appendix.

Section H36.303  [Reserved]

Section H36.305  Noise levels

(a) Limits. For compliance with this appendix, it must be shown by flight

test that the calculated noise levels of the helicopter, at the measuring

points described in section H36.305 (a) of this appendix, do not exceed the

following, with appropriate interpolation between weights:

(1) Stage 1 noise limits for acoustical changes for helicopters are as

follows:

(i) For takeoff, flyover, and approach calculated noise levels, the noise

levels of each Stage 1 helicopter that exceed the Stage 2 noise limits plus 2

EPNdB may not, after a change in type design, exceed the noise levels created

prior to the change in type design.

(ii) For takeoff, flyover, and approach calculated noise levels, the noise

levels of each Stage 1 helicopter that do not exceed the Stage 2 noise limits

plus 2 EPNdB may not, after the change in type design, exceed the Stage 2

noise limits plus 2 EPNdB.

(2) Stage 2 noise limits are as follows:

(i) For takeoff calculated noise levels--109 EPNdB for maximum takeoff

weights of 176,370 pounds or more, reduced by 3.01 EPNdB per halving of the

weight down to 89 EPNdB for maximum weights of 1,764 pounds or less.

(ii) For flyover calculated noise levels--108 EPNdB for maximum weights of

176,370 pounds or more, reduced by 3.01 EPNdB per halving of the weight down

to 88 EPNdB for maximum weights of 1,764 pounds or less.

(iii) For approach calculated noise levels--110 EPNdB for maximum weights

of 176,370 pounds or more, reduced by 3.01 EPNdB per halving of the weight

down 90 EPNdB for maximum weight of 1,764 pounds or less.

(b) Tradeoffs. Except to the extent limited under Sec. 36.11(b) of this

part, the noise limits prescribed in paragraph (a) of this section may be

exceeded by one or two of the takeoff, flyover, or approach calculated noise

levels determined under section H36.203 of this appendix if

(1) The sum of the exceedances is not greater than 4 EPNdB;

(2) No exceedance is greater than 3 EPNdB; and

(3) The exceedances are completely offset by reduction in the other

required calculated noise levels.

[Amdt. 36-14, 53 FR 3541, Feb. 5, 1988; 53 FR 4099, Feb. 11, 1988; 53 FR

7728, Mar. 10, 1988]

Appendix I--[Reserved. Amdt. 36-20, 57 FR 42855, Sept. 16, 1992]

Appendix J--Alternative Noise Certification Procedure For Helicopters Under

Subpart H Having A Maximum Certificated Takeoff Weight Of Not More Than 6,000

Pounds

Part A--Reference Conditions

J36.1  General.

J36.3  Reference Test Conditions.

J36.5  [Reserved]

Part B--Noise Measurement Procedure Under Sec. 36.801

J36.101  Noise certification test and measurement conditions.

J36.103  [Reserved]

J36.105  Flyover test conditions.

J36.107  [Reserved]

J36.109  Measurement of helicopter noise received on the ground.

J36.111  Reporting requirements.

J36.113  [Reserved]

Part C--Noise Evaluation and Calculation Under Sec. 36.803

J36.201  Noise evaluation in SEL.

J36.203  Calculation of noise levels.

J36.205  Detailed data correction procedures.

Part D--Noise Limits Procedure Under Sec. 36.805

J36.301  Noise measurement, evaluation, and calculation.

J36.303  [Reserved]

J36.305  Noise limits.

Part A--Reference Conditions

Section J36.1 General

This appendix prescribes the alternative noise certification requirements

identified under Sec. 36.1 of this part and subpart H of this part for

helicopters in the primary, normal, transport, and restricted categories

having maximum certificated takeoff weight of not more than 6,000 pounds

including:

(a) The conditions under which an alternative noise certification test

under subpart H of this part must be conducted and the alternative

measurement procedure that must be used under Sec. 36.801 of this part to

measure the helicopter noise during the test;

(b) The alternative procedures which must be used under Sec. 36.803 of this

part to correct the measured data to the reference conditions and to

calculate the noise evaluation quantity designated as Sound Exposure Level

(SEL); and

(c) The noise limits for which compliance must be shown under Sec. 36.805

of this part.

Section J36.3  Reference Test Conditions

(a) Meteorological conditions. The following are the noise certification

reference atmospheric conditions which shall be assumed to exist from the

surface to the helicopter altitude:

(1) Sea level pressure of 2116 pounds per square foot (76 centimeters

mercury);

(2) Ambient temperature of 77 degrees Fahrenheit (25 degrees Celsius);

(3) Relative humidity of 70 percent; and

(4) Zero wind.

(b) Reference test site. The reference test site is flat and without line-

of-sight obstructions across the flight path that encompasses the 10 dB down

points of the A-weighted time history.

(c) Level flyover reference profile. The reference flyover profile is a

level flight 492 feet (150 meters) above ground level as measured at the

noise measuring station. The reference flyover profile has a linear flight

track and passes directly over the noise monitoring station. Airspeed is

stabilized at 0.9VH; 0.9VNE; 0.45VH + 65 kts (0.45VH + 120 km/h); or 0.45VNE

+ 65 kts (0.45VNE + 120 km/h), whichever of the four speeds is least. Rotor

speed is stabilized at the power on maximum normal operating RPM throughout

the 10 dB down time period.

(1) For noise certification purposes, VH is defined as the airspeed in

level flight obtained using the minimum specification engine power

corresponding to maximum continuous power available for sea level, 77 degree

Fahrenheit (25 degrees Celsius) ambient conditions at the relevant maximum

certificated weight. The value of VH thus defined must be listed in the

Rotorcraft Flight Manual.

(2) VNE is the never-exceed airspeed.

(d) The weight of the helicopter shall be the maximum takeoff weight at

which noise certification is requested.

Section J36.5  [Reserved]

Part B--Noise Measurement Procedure Under Sec. 36.801

Section J36.101  Noise certification test and measurement conditions

(a) General. This section prescribes the conditions under which helicopter

noise certification tests must be conducted and the measurement procedures

that must be used to measure helicopter noise during each test.

(b) Test site requirements. (1) The noise measuring station must be

surrounded by terrain having no excessive sound absorption characteristics,

such as might be caused by thick, matted, or tall grass, shrubs, or wooded

areas.

(2) During the period when the flyover noise measurement is within 10 dB of

the maximum A-weighted sound level, no obstruction that significantly

influences the sound field from the helicopter may exist within a conical

space above the noise measuring position (the point on the ground vertically

below the microphone), the cone is defined by an axis normal to the ground

and by half-angle 80 degrees from this axis.

(c) Weather restrictions. The test must be conducted under the following

atmospheric conditions:

(1) No rain or other precipitation;

(2) Ambient air temperature between 36 degrees and 95 degrees Fahrenheit (2

degrees and 35 degrees Celsius), inclusively, and relative humidity between

20 percent and 95 percent inclusively, except that testing may not take place

where combinations of temperature and relative humidity result in a rate of

atmospheric attenuation greater than 10 dB per 100 meters (30.5 dB per 1000

ft) in the one-third octave band centered at 8 kiloHertz.

(3) Wind velocity that does not exceed 10 knots (19 km/h) and a crosswind

component that does not exceed 5 knots (9 km/h). The wind shall be determined

using a continuous averaging process of no greater than 30 seconds;

(4) Measurements of ambient temperature, relative humidity, wind speed, and

wind direction must be made between 4 feet (1.2 meters) and 33 feet (10

meters) at the noise monitoring station. Unless otherwise approved by the

FAA, ambient temperature and relative humidity must be measured at the noise

measuring station at the same height above the ground.

(5) No anomalous wind conditions (including turbulence) or other anomalous

meteorological conditions that will significantly affect the noise level of

the helicopter when the noise is recorded at the noise measuring station; and

(6) The location of the meteorological instruments must be approved by the

FAA as representative of those atmospheric conditions existing near the

surface over the geographical area where the helicopter noise measurements

are made. In some cases, a fixed meteorological station (such as those found

at airports or other facilities) may meet this requirement.

(d) Helicopter testing procedures. (1) The helicopter testing procedures

and noise measurements must be conducted and processed in a manner which

yields the noise evaluation measure designated Sound Exposure Level (SEL) as

defined in section J36.109(b) of this appendix.

(2) The helicopter height relative to the noise measurement point

sufficient to make corrections required under section J36.205 of this

appendix must be determined by an FAA-approved method that is independent of

normal flight instrumentation, such as radar tracking, theodolite

triangulation, laser trajectography, or photographic scaling techniques.

(3) If an applicant demonstrates that the design characteristics of the

helicopter would prevent flight from being conducted in accordance with the

reference test conditions prescribed under section J36.3 of this appendix,

then with FAA approval, the reference test conditions used under this

appendix may vary from the standard reference test conditions, but only to

the extent demanded by those design characteristics which make compliance

with the reference test conditions impossible.

Section J36.103  [Reserved]

Section J36.105  Flyover test conditions

(a) This section prescribes the flight test conditions and allowable random

deviations for flyover noise tests conducted under this appendix.

(b) A test series must consist of at least six flights with equal numbers

of flights in opposite directions over the noise measuring station:

(1) In level flight and in cruise configuration;

(2) At a height of 492 feet +/-50 feet (150 +/-15 meters) above the ground

level at the noise measuring station; and

(3) Within +/-10 degrees from the zenith.

(c) Each flyover noise test must be conducted:

(1) At the reference airspeed specified in section J36.3(c) of this

appendix, with such airspeed adjusted as necessary to produce the same

advancing blade tip Mach number as associated with the reference conditions;

(i) Advancing blade tip Mach number (MAT) is defined as the ratio of the

arithmetic sum of blade tip rotational speed (VR) and the helicopter true air

speed (VT) over the speed of sound (c) at 77 degrees Fahrenheit (1135.6 ft/

sec or 346.13 m/sec) such that MAT = (VR + VT)/c; and

(ii) The airspeed shall not vary from the adjusted reference airspeed by

more than +/-3 knots (+/-5 km/hr) or an equivalent FAA-approved variation

from the reference advancing blade tip Mach number. The adjusted reference

airspeed shall be maintained throughout the measured portion of the flyover.

(2) At rotor speed stabilized at the power on maximum normal operating

rotor RPM (+/-1 percent); and

(3) With the power stabilized during the period when the measured

helicopter noise level is within 10 dB of the maximum A-weighted sound level

(LAMAX).

(d) The helicopter test weight for each flyover test must be within plus 5

percent or minus 10 percent of the maximum takeoff weight for which

certification under this part is requested.

(e) The requirements of paragraph (b)(2) of this section notwithstanding,

flyovers at an FAA-approved lower height may be used and the results adjusted

to the reference measurement point by an FAA-approved method if the ambient

noise in the test area, measured in accordance with the requirements

prescribed in section J36.109 of this appendix, is found to be within 15

dB(A) of the maximum A-weighted helicopter noise level (LAMAX) measured at

the noise measurement station in accordance with section J36.109 of this

appendix.

Section J36.107  [Reserved]

Section J36.109  Measurement of helicopter noise received on the ground

(a) General. (1) The helicopter noise measured under this appendix for

noise certification purposes must be obtained with FAA-approved acoustical

equipment and measurement practices.

(2) Paragraph (b) of this section identifies and prescribes the

specifications for the noise evaluation measurements required under this

appendix. Paragraphs (c) and (d) of this section prescribe the required

acoustical equipment specifications. Paragraphs (e) and (f) of this section

prescribe the calibration and measurement procedures required under this

appendix.

(b) Noise unit definition. (1) The value of sound exposure level (SEL, or

as denoted by symbol, LAE), is defined as the level, in decibels, of the time

integral of squared 'A'-weighted sound pressure (PA) over a given time period

or event, with reference to the square of the standard reference sound

pressure (PO) of 20 micropascals and a reference duration of one second.

(2) This unit is defined by the expression:

[ ...Illustration appears here... ]

Where TO is the reference integration time of one second and (t2-t1) is the

integration time interval.

(3) The integral equation of paragraph (b)(2) of this section can also be

expressed as:

[ ...Illustration appears here... ]

Where LA(t) is the time varying A-weighted sound level.

(4) The integration time (t2-t1) in practice shall not be less than the

time interval during which LA(t) first rises to within 10 dB(A) of its

maximum value (LAMAX) and last falls below 10 dB(A) of its maximum value.

(5) The SEL may be approximated by the following expression:

LAE = LAMAX + <delta> A

where <delta> A is the duration allowance given by:

<delta> A = 10 log10 (T)

where T = (t2-t1)/2 and LAMAX is defined as the maximum level, in decibels,

of the A-weighted sound pressure (slow response) with reference to the square

of the standard reference sound pressure (P0).

(c) Measurement system. The acoustical measurement system must consist of

FAA-approved equipment equivalent to the following:

(1) A microphone system with frequency response that is compatible with the

measurement and analysis system accuracy prescribed in paragraph (d) of this

section;

(2) Tripods or similar microphone mountings that minimize interference with

the sound energy being measured;

(3) Recording and reproducing equipment with characteristics, frequency

response, and dynamic range that are compatible with the response and

accuracy requirements of paragraph (d) of this section; and

(4) Acoustic calibrators using sine wave noise and, if a tape recording

system is used, pink noise, of known levels. When pink noise (defined in

section H36.109(e)(1) of Appendix H of this part) is used, the signal must be

described in terms of its root-mean-square (rms) value.

(d) Sensing, recording, and reproducing equipment. (1) The noise levels

measured from helicopter flyovers under this appendix may be determined

directly by an integrating sound level meter, or the A-weighted sound level

time history may be written onto a graphic level recorder set at "slow"

response from which the SEL value may be determined. With the approval of the

FAA, the noise signal may be tape recorded for subsequent analysis.

(i) The SEL values from each flyover test may be directly determined from

an integrating sound level meter complying with the Standards of the

International Electrotechnical Commission (IEC) Publication No. 804,

"Integrating-averaging Sound Level Meters," as incorporated by reference

under Sec. 36.6 of this part, for a Type 1 instrument set at "slow" response.

(ii) The acoustic signal from the helicopter, along with the calibration

signals specified under paragraph (e) of this section and the background

noise signal required under paragraph (f) of this section may be recorded on

a magnetic tape recorder for subsequent analysis by an integrating sound

level meter identified in paragraph (d)(1)(i) of this section. The record/

playback system (including the audio tape) of the tape recorder must conform

to the requirements prescribed in section H36.109(c)(3) of Appendix H of this

part. The tape recorder shall comply with specifications of IEC Publication

No. 561, "Electro-acoustical Measuring Equipment for Aircraft Noise

Certification," as incorporated by reference under Sec. 36.6 of this part.

(iii) The characteristics of the complete system shall comply with the

recommendations given in IEC Publication No. 651, "Sound Level Meters," as

incorporated by reference under Sec. 36.6 of this part, with regard to the

specifications concerning microphone, amplifier, and indicating instrument

characteristics.

(iv) The response of the complete system to a sensibly plane progressive

wave of constant amplitude shall lie within the tolerance limits specified in

Table IV and Table V for Type 1 instruments in IEC Publication No. 651,

"Sound Level Meters," as incorporated by reference under Sec. 36.6 of this

part, for weighting curve "A" over the frequency range of 45 Hz to 11500 Hz.

(v) A windscreen must be used with the microphone during each measurement

of the helicopter flyover noise. Correction for any insertion loss produced

by the windscreen, as a function of the frequency of the acoustic calibration

required under paragraph (e) of this section, must be applied to the measured

data and any correction applied must be reported.

(e) Calibrations. (1) If the helicopter acoustic signal is tape recorded

for subsequent analysis, the measuring system and components of the recording

system must be calibrated as prescribed under section H36.109(e) of Appendix

H of this part.

(2) If the helicopter acoustic signal is directly measured by an

integrating sound level meter:

(i) The overall sensitivity of the measuring system shall be checked before

and after the series of flyover tests and at intervals (not exceeding one-

hour duration) during the flyover tests using an acoustic calibrator using

sine wave noise generating a known sound pressure level at a known frequency.

(ii) The performance of equipment in the system will be considered

satisfactory if, during each day's testing, the variation in the calibration

value does not exceed 0.5 dB. The SEL data collected during the flyover tests

shall be adjusted to account for any variation in the calibration value.

(iii) A performance calibration analysis of each piece of calibration

equipment, including acoustic calibrators, reference microphones, and voltage

insertion devices, must have been made during the six calendar months

proceeding the beginning of the helicopter flyover series. Each calibration

shall be traceable to the National Institute of Standards and Technology.

(f) Noise measurement procedures. (1) The microphone shall be of the

pressure-sensitive capacitive type designed for nearly uniform grazing

incidence response. The microphone shall be mounted with the center of the

sensing element 4 feet (1.2 meters) above the local ground surface and shall

be oriented for grazing incidence such that the sensing element, the

diaphragm, is substantially in the plane defined by the nominal flight path

of the helicopter and the noise measurement station.

(2) If a tape recorder is used, the frequency response of the electrical

system must be determined at a level within 10 dB of the full-scale reading

used during the test, utilizing pink or pseudorandom noise.

(3) The ambient noise, including both acoustical background and electrical

noise of the measurement systems shall be determined in the test area and the

system gain set at levels which will be used for helicopter noise

measurements. If helicopter sound levels do not exceed the background sound

levels by at least 15 dB(A), flyovers at an FAA-approved lower height may be

used and the results adjusted to the reference measurement point by an FAA-

approved method.

(4) If an integrating sound level meter is used to measure the helicopter

noise, the instrument operator shall monitor the continuous A-weighted (slow

response) noise levels throughout each flyover to ensure that the SEL

integration process includes, at minimum, all of the noise signal between the

maximum A-weighted sound level (LAMAX) and the 10 dB down points in the

flyover time history. The instrument operator shall note the actual db(A)

levels at the start and stop of the SEL integration interval and document

these levels along with the value of LAMAX and the integration interval (in

seconds) for inclusion in the noise data submitted as part of the reporting

requirements under section J36.111(b) of this appendix.

Section J36.111  Reporting Requirements

(a) General. Data representing physical measurements, and corrections to

measured data, including corrections to measurements for equipment response

deviations, must be recorded in permanent form and appended to the record.

Each correction is subject to FAA approval.

(b) Data reporting. After the completion of the test the following data

must be included in the test report furnished to the FAA:

(1) Measured and corrected sound levels obtained with equipment conforming

to the standards prescribed in section J36.109 of this appendix;

(2) The type of equipment used for measurement and analysis of all

acoustic, aircraft performance and flight path, and meteorological data;

(3) The atmospheric environmental data required to demonstrate compliance

with this appendix, measured throughout the test period;

(4) Conditions of local topography, ground cover, or events which may

interfere with the sound recording;

(5) The following helicopter information:

(i) Type, model, and serial numbers, if any, of helicopter, engine(s) and

rotor(s);

(ii) Gross dimensions of helicopter, location of engines, rotors, type of

antitorque system, number of blades for each rotor, and reference operating

conditions for each engine and rotor;

(iii) Any modifications of non-standard equipment likely to affect the

noise characteristics of the helicopter;

(iv) Maximum takeoff weight for which certification under this appendix is

requested;

(v) Aircraft configuration, including landing gear positions;

(vi) VH or VNE (whichever is less) and the adjusted reference airspeed;

(vii) Aircraft gross weight for each test run;

(viii) Indicated and true airspeed for each test run;

(ix) Ground speed, if measured, for each run;

(x) Helicopter engine performance as determined from aircraft instruments

and manufacturer's data; and

(xi) Aircraft flight path above ground level, referenced to the elevation

of the noise measurement station, in feet, determined by an FAA-approved

method which is independent of normal flight instrumentation, such as radar

tracking, theodolite triangulation, laser trajectography, or photoscaling

techniques; and

(6) Helicopter position and performance data required to make the

adjustments prescribed under section J36.205 of this appendix and to

demonstrate compliance with the performance and position restrictions

prescribed under section J36.105 of this appendix must be recorded at an FAA-

approved sampling rate.

Section J36.113  [Reserved]

Part C--Noise Evaluation and Calculations Under Sec. 36.803

Section J36.201  Noise Evaluation in SEL

The noise evaluation measure shall be the sound exposure level (SEL) in

units of dB(A) as prescribed under section J36.109(b) of this appendix. The

SEL value for each flyover may be directly determined by use of an

integrating sound level meter. Specifications for the integrating sound level

meter and requirements governing the use of such instrumentation are

prescribed under section J36.109 of this appendix.

Section J36.203  Calculation of Noise Levels

(a) To demonstrate compliance with the noise level limits specified under

section J36.305 of this appendix, the SEL noise levels from each valid

flyover, corrected as necessary to reference conditions under section J36.205

of this appendix, must be arithmetically averaged to obtain a single SEL

dB(A) mean value for the flyover series. No individual flyover run may be

omitted from the averaging process, unless otherwise specified or approved by

the FAA.

(b) The minimum sample size acceptable for the helicopter flyover

certification measurements is six. The number of samples must be large enough

to establish statistically a 90 percent confidence limit that does not exceed

+/-1.5 dB(A).

(c) All data used and calculations performed under this section, including

the calculated 90 percent confidence limits, must be documented and provided

under the reporting requirements of section J36.111 of this appendix.

Section J36.205  Detailed Data Correction Procedures

(a) When certification test conditions measured under part B of this

appendix differ from the reference test conditions prescribed under section

J36.3 of this appendix, appropriate adjustments shall be made to the measured

noise data in accordance with the methods set out in paragraphs (b) and (c)

of this section. At minimum, appropriate adjustments shall be made for off-

reference altitude and for the difference between reference airspeed and

adjusted reference airspeed.

(b) The adjustment for off-reference altitude may be approximated from:

<delta>J1=12.5 log10(HT/492) dB;

where <delta>J1 is the quantity in decibels that must be algebraically added

to the measured SEL noise level to correct for an off-reference flight path,

HT is the height, in feet, of the test helicopter when directly over the

noise measurement point, and the constant (12.5) accounts for the effects on

spherical spreading and duration from the off-reference altitude.

(c) The adjustment for the difference between reference airspeed and

adjusted reference airspeed is calculated from:

<delta>J3=10 log10(VRA/VR) dB;

Where <delta>J3 is the quantity in decibels that must be algebraically added

to the measured SEL noise level to correct for the influence of the

adjustment of the reference airspeed on the duration of the measured flyover

event as perceived at the noise measurement station, VR is the reference

airspeed as prescribed under section J36.3.(c) of this appendix, and VRA is

the adjusted reference airspeed as prescribed under section J36.105(c) of

this appendix.

(d) No correction for source noise during the flyover other than the

variation of source noise accounted for by the adjustment of the reference

airspeed prescribed for under section J36.105(c) of this appendix need be

applied.

(e) No correction for the difference between the reference ground speed and

the actual ground speed need be applied.

(f) No correction for off-reference atmospheric attenuation need be

applied.

(g) The SEL adjustments must be less than 2.0 dB(A) for differences between

test and reference flight procedures prescribed under section J36.105 of this

appendix unless a larger adjustment value is approved by the FAA.

(h) All data used and calculations performed under this section must be

documented and provided under the reporting requirements specified under

section J36.111 of this appendix.

Part D--Noise Limits Procedure Under Sec. 36.805

Section J36.301  Noise Measurement, Evaluation, and Calculation

Compliance with this part of this appendix must be shown with noise levels

measured, evaluated, and calculated as prescribed under parts B and C of this

appendix.

Section J36.303  [Reserved]

Section J36.305  Noise Limits

For compliance with this appendix, the calculated noise levels of the

helicopter, at the measuring point described in section J36.101 of this

appendix, must be shown to not exceed the following (with appropriate

interpolation between weights):

(a) For primary, normal, transport, and restricted category helicopters

having a maximum certificated takeoff weight of not more than 6,000 pounds

and noise tested under this appendix, the Stage 2 noise limit is 82 decibels

SEL for helicopters with maximum certificated takeoff weight at which the

noise certification is requested, of up to 1,764 pounds and increasing at a

rate of 3.01 decibels per doubling of weight thereafter. The limit may be

calculated by the equation:

LAE(limit)=82+3.01[log10(MTOW/1764)/log10(2)] dB;

where MTOW is the maximum takeoff weight, in pounds, for which certification

under this appendix is requested.

(b) The procedures required in this amendment shall be done in accordance

with the International Electrotechnical Commission IEC Publication No. 804,

entitled "Integrating-averaging Sound Level Meters," First Edition, dated

1985. This incorporation by reference was approved by the Director of the

Federal Register in accordance with 5 U.S.C. 552(a) and 1 CFR part 51. Copies

may be obtained from the Bureau Central de la Commission Electrotechnique

Internationale, 1, rue de Varembe, Geneva, Switzerland or the American

National Standard Institute, 1430 Broadway, New York City, New York 10018,

and can be inspected at the Office of the Federal Register, 800 North Capitol

Street NW., suite 700, Washington, DC.

[Amdt. 36-20, 57 FR 42855, Sept. 16, 1992; 57 FR 46243, Oct. 7, 1992]