PART 93--SPECIAL AIR TRAFFIC RULES AND AIRPORT TRAFFIC PATTERNS

Special Federal Aviation Regulation No. 43

Special Federal Aviation Regulation No. 48

Special Federal Aviation Regulation No. 60

Subpart A--General

Sec.

93.1  Applicability.

Subparts B-C--[Reserved]

Subpart D--Anchorage, Alaska, Terminal Area

93.51  Applicability.

93.53  Description of area.

93.55  Subdivision of area.

93.57  General rules: All segments.

93.59  General rules: International segment.

93.61  General rules: Lake Hood segment.

93.63  General rules: Merrill segment.

93.65  General rules: Elmendorf segment.

93.67  General rules: Bryant segment.

93.69  Special Requirements, Lake Campbell and Sixmile Lake Airports.

Subpart E--VOR Federal Airway No. 16

93.71  Applicability.

93.73  Crossing VOR Federal Airway No. 16; VFR Jet training operations.

93.75  Crossing and operating along VOR Federal Airway No. 16.

Subpart F--Valparaiso, Florida, Terminal Area

93.81  Applicability and description of area.

93.83  Aircraft operations.

Subpart G--[Reserved]

Subpart H--[Reserved]

Subpart I--[Reserved]

Subpart J--Lorain County Regional Airport Traffic Rule

93.117  Applicability.

93.119  Aircraft operations.

Subpart K--High Density Traffic Airports

93.121  Applicability.

93.123  High density traffic airports.

93.125  Arrival or departure reservation.

93.129  Additional operations.

93.130  Suspension of allocations.

93.133  Exceptions.

Subpart L--[Reserved]

Subpart M--Ketchikan International Airport Traffic Rule

93.151  Applicability.

93.153  Communications.

93.155  Aircraft operations.

Subpart N--[Reserved]

Subpart O--[Reserved]

Subpart P--[Reserved]

Subpart Q--[Reserved]

Subpart R--[Reserved]

Subpart S--Allocation of Commuter and Air Carrier IFR Operations at High

Density Traffic Airports

93.211  Applicability.

93.213  Definitions and General Provisions.

93.215  Initial Allocation of Slots.

93.217  Allocation of Slots for International Operations and Applicable

Limitations.

93.219  Allocation of Slots for Essential Air Service Operations and

Applicable Limitations.

93.221  Transfer of Slots.

93.223  Slot Withdrawal.

93.225  Lottery of Available Slots.

93.226  Allocation of slots in low-demand periods.

93.227  Slot Use and Loss.

Subpart T--Washington National Airport Traffic Rules

93.251  Applicability.

93.253  Nonstop operations.

Appendix A--Anchorage Airport Traffic Area: Traffic Patterns

Authority: 49 U.S.C. app. 1302, 1303, 1348, 1354(a), 1421(a), 1424, 2451

et seq.; 49 U.S.C. 106(g).

EFFECTIVE DATE NOTE: Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992, removed and reserved subparts I, N, O, Q, and R of Part 93

effective September 16, 1993. For the convenience of the user, the headings

of the reserved items in effect until September 16, 1993 are set forth as

follows:

PART 93--SPECIAL AIR TRAFFIC RULES AND AIRPORT TRAFFIC PATTERNS

*     *     *     *     *

Subpart I--Locations at Which Special VFR Weather Minimums Do Not Apply

93.111  Applicability.

93.113  Control zones within which special VFR weather minimums are not

authorized.

*     *     *     *     *

Subpart N--Sabre U.S. Army Heliport (Tenn.) Airport Traffic Area

93.161  Applicability.

93.163  Description of area.

Subpart O--Jacksonville, Fla., Navy Airport Traffic Area

93.171  Applicability.

93.173  Description of area.

93.175  Aircraft operations.

*     *     *     *     *

Subpart Q--Abbotsford, BC, and Sault Ste. Marie, ON: Special Airport Traffic

Areas and Air Traffic Rules

93.195  Applicability; scope.

93.197  Special airport traffic areas.

93.199  Communications.

Subpart R--MCAS El Toro, CA, Special Air Traffic Rules

93.200  Applicability.

93.202  MCAS El Toro, CA, special air traffic rules area.

93.204  Communications.

93.206  Ultralight operations.

93.208  Parachute jumping.

*     *     *     *     *

Special Federal Aviation Regulation No. 43

Contrary provisions of Part 93 of the Federal Aviation Regulations

notwithstanding, IFR reservations available to air carriers except air taxis

at Washington National Airport will be issued in accordance with Appendix A

for the period December 1, 1980 to April 26, 1981.

APPENDIX A

(a) The allocation agreed upon by the Airline Scheduling Committee for

Washington National Airport for November, 1980, shown in Table 1 below, shall

be in effect from December 1, 1980 until April 26, 1981, with adjustments

made in accordance with paragraphs (b) through (f) of this appendix.

(b) Slots shall be set aside for redistribution in accordance with

paragraphs (d) and (e) of this appendix in the following numbers by hour: at

0700(2 slots), 0800(2), 0900(2), 1000(2), 1100(1), 1200(1), 1300(1), 1400(1),

1500(1), 1600(2), 1700(2), 1800(2), 1900(2), 2000(2), 2100(1), and 2200(4),

as shown in Table 2.

(c) The Department shall ask the scheduling committee to reconvene on

Friday, October 31.

(d) For the purpose of determining the distribution of the set-aside slot

reductions among the carriers, the following procedure shall be used:

(1) The total number of slots available to each carrier for December is

established as shown in the right hand column of Table 1.

(2) The five carriers with fewer total slots in December than in the

November schedule, as shown in Table 1 (U.S. Air, Braniff, Eastern, Pan

American, and Piedmont), shall be ranked in accordance with the number of

slots they hold in the December totals.

(3) In rank order, beginning with the carrier with the fewest slots, each

of the five carriers shall be called to select the slots in its November

schedule to be deleted. The slots selected by each carrier must fall within

the hours 0700 to 2100, must correspond to set-aside slots not selected by

another carrier, and must equal in total number the difference between the

slots shown for each carrier in the November and December totals.

(4) Thereafter the twelve carriers with the most slots in the November

totals shall be ranked in order of the number of slots they hold. The rank of

carriers with an equal number of slots shall be established by the toss of a

coin.

(5) The ranked carrier with the fewest slots shall be called upon to select

one of its slots corresponding to one of the remaining set-aside slots in the

0700 to 2100 hours to be shifted to the 2200 hour, or deleted from its

schedule at the carrier's option. The other ranked carriers shall do the

same, following in rank order.

(6) Any carrier not participating when called or unable to respond within

15 minutes shall be put at the bottom of the rank list.

(7) If any ranked carrier is unwilling or unable to participate, the set-

aside slots to be lost or shifted by those carriers shall be determined by

lot.

(e) For the purpose of distributing the set-aside slots to the gaining

carriers, the following procedure shall be used:

(1) Air North shall select two slots.

(2) Texas International shall select four slots.

(3) Air North shall select two slots.

(4) Texas International shall select four slots.

(5) Empire and Mid-South, in order determined by the toss of a coin, shall

select two slots each.

(6) Midway shall select two slots.

(7) Texas International shall receive the remaining ten slots.

Under this section, each carrier may select only one slot in any hour,

except the 2200 hour, unless no other slot is available. Each carrier with a

total of more than four slots for December shall select one slot from the

2200 hour if one is available.

(f) Subject to CAB authorization, the carriers may then exchange slots in

any hour.

(g) The Department shall then issue and publish a final schedule in the

Federal Register. Thereafter, existing procedures (or any new procedure

approved by the CAB) of the scheduling committee for the relinquishment and

reassignment of unused slots may be employed.

(h) The FAA will accept at any time a schedule developed by the committee

in accordance with its rules for part or all of the scheduling period, in

lieu of the schedule published under paragraph (f) of this appendix.

Table 1--November Schedule and Total Slots for

December

Carriers    07  08  09  10  11  12  13  14  15  16

American       5   3   4   4   3   6   2   4   3   2

Altair         1       2   1   2   1               2

USAir          7   7   6   3   5   3   6   7   5   4

Braniff        1   1       1   1   2   2   2   3   1

Delta          3   3   1   2   2   2   3   2   1   3

Eastern        9   8   7  12   8   7   8   9   9   7

Aeromech           1   1               1   1

Midway         1               1   1       1   1

New Air            2                   1   1

Air North

Northwest      3   2   3   2   2   4   3   1   4   3

Ozark                          1   1               1

Pan American   2   2   3   5   1   1   4   2   1   2

Piedmont       2   4   6   4   4   5   4   5   3   4

Pilgrim            2                               1

Air Florida    1   1   1       1   1   1   1   1   1

Republic                       1   1

Texas Int'l

Trans World    1   2   3   2   1   3   3   2   5   3

United         4   1   2   2   4   2   1   2   3   3

Empire             1   1               1       1

Mid-South                  2   2                   2

Western                        1                   1

Total         40  40  40  40  40  40  40  40  40  40

[ ...Table continues... ]

Carriers    17  18  19  20  21  22  23  Total      December

American       5   4   5   3   3   2         58  AA        58

Altair             1   1   1   2   2         16  AK        16

USAir          6   5   5   6   5   4         84  AL        82

Braniff        2       2   2   2             22  BN        20

Delta          3   2   2   1   2             32  DL        32

Eastern        5   8   8   7  12   4        128  EA       124

Aeromech               2                      6  KC         6

Midway         1   1           1              8  ML        10

New Air        1   1                          6  NC         6

Air North                                        NO         4

Northwest      2   2   2   5   1   1         40  NW        40

Ozark          1                              4  OZ         4

Pan American   3   5   2   1   2   2         38  PA        36

Piedmont       4   4   4   4   3   4         64  PI        62

Pilgrim        1                              4  PM         4

Air Florida        1   1   1   1   1         14  QH        14

Republic                   2                  4  RC         4

Texas Int'l                                      TI        18

Trans World    3   2   2   3   2   1         38  TW        38

United         2   4   2   4   3   1         40  UA        40

Empire                 2                      6  UR         8

Mid-South                                     6  VL         8

Western        1               1              4  WA         4

Total         40  40  40  40  40  22        622           638

Table 2--Set-Aside Slots

07  08  09  10  11  12  13  14  15  16  17  18  19  20  21  22  Total

2   2   2   2   1   1   1   1   1   2   2   2   2   2   1   4     28

[45 FR 72640, Nov. 3, 1980]

SFAR No. 48--Special Slot Withdrawal and Reallocation Procedures

Section 1. Purpose.

The purpose of this Special Federal Aviation Regulation (SFAR) is to

accomplish a special one-time withdrawal of slots that were allocated under

Subpart S of Part 93 to incumbent carriers and a reallocation by lottery of

those slots and slots otherwise available to new entrant carriers and limited

incumbent carriers desiring additional slots at O'Hare, LaGuardia, and

Washington National Airports.

Sec. 2. Definitions and Relationship to 14 CFR Part 93, Subparts S and K.

Terms used in this SFAR have the same meanings as used in Subparts K and S

of Part 93. The provisions of those subparts shall be considered when

applying this SFAR. However, this SFAR supersedes inconsistent provisions in

both subparts.

Sec. 3. Lottery Process--General.

(a) Utilizing the air carrier slot pools established by the FAA under Sec.

93.223(a)(3), the FAA shall hold lotteries to determine those slots to be

withdrawn from incumbent carriers at O'Hare, LaGuardia, and Washington

National Airports. The drawings shall be conducted within 30 days after

issuance of this SFAR.

(b) Separate drawings shall be conducted for air carrier slots for each of

the high density airports identified in paragraph (a).

(c) The slots selected in each drawing shall be determined in accordance

with sections 4(c) and (e).

(d) The FAA may issue special procedures to be in effect for the lotteries

(e) None of the slots withdrawn shall be slots used for international

operations, as specified in Sec. 93.217(a)(1), or necessary for essential air

service.

(f) None of the slots withdrawn shall be from carriers holding eight or

fewer non-international air carrier slots at the particular airport.

(g) Each U.S. air carrier operating at the airport with fewer than eight

total slots (including all international or domestic, commuter or air carrier

slots) shall be included in the lotteries to obtain slots under this SFAR if

the carrier notifies the Office of the Chief Counsel, Docket Section, AGC-

204, 800 Independence Avenue, SW., Washington, D.C. 20591, that it desires to

participate in the reallocation lottery. The notification must be in writing

and must be submitted in duplicate by 5:00 p.m. Eastern Standard Time on

March 24, 1986. The notification must include a certified statement signed by

an officer of the operator indicating that the operator operates or has

contracted for sufficient aircraft having a maximum seating capacity of 56 or

more to use the slots to be obtained and that the operator has bona fide

plans to use all requested slots within the timeframe described in section 6.

(h) Any U.S. new entrant air carrier wishing to initiate scheduled service

at the airport, shall be included in the lottery if it: (1) Has appropriate

economic authority under Title IV of the Federal Aviation Act of 1958, as

amended; (2) has FAA operating authority under Part 121; and (3) notifies the

Office of the Chief Counsel, Docket Section, AGC-204, 800 Independence

Avenue, SW., Washington, D.C. 20591, that it desires to participate in the

lottery. The notification must be in writing and must be submitted in

duplicate by 5:00 p.m. Eastern Standard Time on March 24, 1986. The

notification must include a certified statement signed by an officer of the

operator indicating that the operator operates or has contracted for

sufficient aircraft having a maximum certificated passenger seating capacity

of 56 or more to use the slots to be obtained and that the operator has bona

fide plans to use all requested slots within the timeframe described in

section 6.

Sec. 4. Withdrawal Process.

(a) The name of each air carrier with more than eight non-international air

carrier slots (5 days a week or more) at the airport shall be placed in

individual capsules for inclusion in a drawing. The name of each such carrier

shall be placed in a number of capsules equal to 10 percent of the total

number of air carrier slots held by that carrier (5 days a week or more) at

that airport in accordance with Part 93. Subpart S, rounded to the next

lowest number, but not less than one. The date used to determine slot

holdings under this paragraph shall be March 20, 1986.

(b) After each capsule is placed in a drum, a random drawing shall be held

to determine the order for the selection of slots to be withdrawn.

(c) Before any carrier is required to designate slots for withrawal,

unallocated slots and slots available in accordance with 14 CFR 93.224 and

93.227 shall be designated for reallocation under section 5 of this SFAR.

Those slots shall be used to satisfy the provisions of paragraph (e) of this

section.

(d) Each time a carrier's name is called, it shall designate two slots (or

one if that is all that is required) to be withdrawn and reallocated from the

most current base available to the FAA. A carrier shall have 5 minutes to

designate its slots to be withdrawn or the FAA representative will. Any slot

designated must be in the carrier's base at least 5 days a week.

(e) Each slot designated must be within hours in which slots are needed

consistent with the hourly minimum slot column (Column C) for that airport,

as specified below:

(A)       (B)       (C)

Total              Minimum

number              slots

of    Hours of   in each

Airport         Category    slots    the day    hour

LaGuardia            Air Carrier      36  0700-2159        2

Washington National  ......do         28  0700-2159        2

O'Hare               ......do         84  0645-2114        4

If the required minimum number of slots has been designated in each of the

hours of the day (Column B), a carrier required to designate additional slots

may do so in any of those hours. If a carrier does not have a slot in a

required hour, it may name a slot in any hour listed in Column B. The total

number of slots available under paragraph (c) during the hours specified in

Column B plus the slots designated by carriers for withrawal pursuant to

paragraph (d) shall not exceed the total number of slots specified in Column

A for each airport.

(f) No carrier shall be required to designate more than 10 percent of its

total of non-international air carrier slots (5 days a week or more) at the

particular airport.

(g) Slots designated for withdrawal under this section shall retain the

same withdrawal priority numbers as established under Subpart S of Part 93

when reallocated pursuant to section 5 of this SFAR. However, slots

reallocated under section 5 shall not be withdrawn under Subpart S, for EAS

or international operations, from the carrier selecting the slot for a period

of 2 years after the effective date of this SFAR.

Sec. 5. Reallocation Lotteries.

(a) Within 48 hours following the withdrawal lottery, a lottery to allocate

the slots withdrawn pursuant to section 4 and those otherwise available shall

be held on an airport-by-airport basis.

(b) Random drawings shall be held to determine the order of slot selection

by the carriers eligible under section 3.

(c) An operator may select up to two slots available at the airport during

each selection sequence, except that new entrant carriers may select up to

four slots, if available, in the first sequence.

(d) At the lottery, each operator must make its selection within 5 minutes

after being called or it shall lose its turn. If capacity still remains after

each operator has had an opportunity to select slots, the allocation sequence

will be repeated in the same order.

(e) No carrier may select more than eight slots under this SFAR or select

any slots that would result in it having a total of more than eight slots at

any one airport, less any slots lost under Sec. 93.227(a).

(f) If all eligible carriers decide not to select additional slots or have

selected their maximum number of slots, the lottery at that airport will end

and the remaining slots will be returned (subject to section 7) to the

carriers designating them, if any, in accordance with section 4.

Sec. 6. Slot Use.

(a) The initiation of operations with slots reallocated pursuant to this

SFAR shall be accomplished no earlier than 90 days and no later than 180 days

after the lottery is held pursuant to section 5.

(b) Each carrier obtaining a slot under section 5 shall give the carrier

losing the slot under section 4, if any, and the FAA, at least 90 days

written notice prior to initiation of service. The notice to the FAA must be

mailed to the address set forth in section 3(g).

(c) Slots obtained under section 5 shall only be used by the carrier

obtaining them for scheduled non-international passenger service.

(d) Slots allocated under section 5 which were withdrawn from an incumbent

carrier under section 4 will be temporarily returned to the incumbent carrier

and all other slots allocated under section 5 will be returned to the FAA if:

(1) The carrier obtaining the slot under section 5 fails to give the notice

required by section 5 or does not utilize the slot within 14 days after the

date which it states it will begin to utilize the slot; or

(2) The carrier obtaining the slot loses it under 14 CFR 93.227(a) within

180 days after it initiates service.

(e) The slot "use-or-lose" provisions of Sec. 93.227(a) of the Federal

Aviation Regulations shall apply to those slots allocated under this SFAR as

of the date the carrier obtaining the slot actually initiates service. The

provisions of Sec. 93.227(b) do not apply to the slots allocated under this

SFAR.

(f) A slot obtained under section 5 may only be traded on a one-for-one

basis or for an increased number of slots at the same airport for 90 days

after the carrier obtaining it under section 5 initiates use of that slot.

The restrictions of this SFAR apply to any slots obtained in such a trade.

(g) For 90 days after initiation of service with any slot obtained under

this section, it must be utilized with aircraft having a certificated maximum

passenger seating capacity of 56 or more. After 90 days, the provisions of 14

CFR Subpart S apply as to the kind of aircraft eligible to be utilized with

the slot.

Sec. 7. Secondary Lotteries.

(a) By December 15, 1986, a second set of lotteries to allocate slots under

this SFAR shall be held to allocate only:

(1) Those slots designated under section 4(e) for reallocation that were

not reallocated under section 5; and

(2) Those slots that were reallocated under section 5 that were returned,

under section 6, to the FAA or the carrier from which they were withdrawn.

(b) The second set of lotteries shall be subject to the rules set forth in

sections 1 through 6 of this SFAR except: (1) The slots to be allocated are

only those specified in paragraph (a) of this section; and (2) carriers which

obtained slots under section 5 and failed to use them in accordance with

section 6 or sold them shall not be eligible to participate in this lottery.

Sales under Subpart S of slots not obtained under this SFAR shall not be

deemed to affect a carrier's eligibility to participate in this lottery.

(c) A notice shall be published before the second set of lotteries

announcing the time and place of the lotteries and any special procedures

that will apply.

Sec. 8. Effective and Termination Dates.

This SFAR is effective March 17, 1986 and terminates on January 1, 1988.

(49 U.S.C. 1302, 1303, 1348, 1354(a), 1421(a); 49 U.S.C. 106 (Revised Pub. L.

97-449, January 12, 1983))

[Doc. No. 24105, 51 FR 8639, Mar. 12, 1986]

Special Federal Aviation Regulation No. 60--Air Traffic Control System

Emergency Operation

1. Each person shall, before conducting any operation under the Federal

Aviation Regulations (14 CFR chapter I), be familiar with all available

information concerning that operation, including Notices to Airmen issued

under Sec. 91.139 and, when activated, the provisions of the National Air

Traffic Reduced Complement Operations Plan available for inspection at

operating air traffic facilities and Regional air traffic division offices,

and the General Aviation Reservation Program. No operator may change the

designated airport of intended operation for any flight contained in the

October 1, 1990, OAG.

2. Notwithstanding any provision of the Federal Aviation Regulations to the

contrary, no person may operate an aircraft in the Air Traffic Control

System:

a. Contrary to any restriction, prohibition, procedure or other action

taken by the Director of the Office of Air Traffic Systems Management

(Director) pursuant to paragraph 3 of this regulation and announced in a

Notice to Airmen pursuant to Sec. 91.139 of the Federal Aviation Regulations.

b. When the National Air Traffic Reduced Complement Operations Plan is

activated pursuant to paragraph 4 of this regulation, except in accordance

with the pertinent provisions of the National Air Traffic Reduced Complement

Operations Plan.

3. Prior to or in connection with the implementation of the RCOP, and as

conditions warrant, the Director is authorized to:

a. Restrict, prohibit, or permit VFR and/or IFR operations at any airport,

terminal control area, airport radar service area, or other terminal and en

route airspace.

b. Give priority at any airport to flights that are of military necessity,

or are medical emergency flights, Presidential flights, and flights

transporting critical Government employees.

c. Implement, at any airport, traffic management procedures, that may

include reduction of flight operations. Reduction of flight operations will

be accomplished, to the extent practical, on a pro rata basis among and

between air carrier, commercial operator, and general aviation operations.

Flights cancelled under this SFAR at a high density traffic airport will be

considered to have been operated for purposes of part 93 of the Federal

Aviation Regulations.

4. The Director may activate the National Air Traffic Reduced Complement

Operations Plan at any time he finds that it is necessary for the safety and

efficiency of the National Airspace System. Upon activation of the RCOP and

notwithstanding any provision of the FAR to the contrary, the Director is

authorized to suspend or modify any airspace designation.

5. Notice of restrictions, prohibitions, procedures and other actions taken

by the Director under this regulation with respect to the operation of the

Air Traffic Control system will be announced in Notices to Airmen issued

pursuant to Sec. 91.139 of the Federal Aviation Regulations.

6. The Director may delegate his authority under this regulation to the

extent he considers necessary for the safe and efficient operation of the

National Air Traffic Control System.

[55 FR 40760, Oct. 4, 1990]

*****************************************************************************

55 FR 40758, No. 193, Oct. 4, 1990

SUMMARY: The Administrator of the FAA has determined that, if the

sequestration provisions of the Gramm-Rudman-Hollings Act take effect on

October 1, 1990, an emergency will exist requiring that immediate measures be

taken in order to maintain air safety. All FAA employees, including air

traffic controllers, may expect to be furloughed for a specified number of

days within each pay period of work. If the furlough is implemented, such

action will result in a reduction in the number of air traffic controllers

available on the job and will significantly affect the FAA's ability to

operate the Air Traffic Control (ATC) system and provide full ATC services.

This Special Federal Aviation Regulation authorizes special provisions for

the operation of the ATC system during the period that the emergency

conditions exist in order to provide for the safe and orderly movement of air

Ttraffic.

EFFECTIVE DATE: September 28, 1990.

*****************************************************************************

Subpart A--General

Sec. 93.1   Applicability.

(a) This part prescribes special airport traffic patterns and airport

traffic areas. It also prescribes special air traffic rules for operating

aircraft in those traffic patterns and traffic areas and in the vicinity of

airports described in this part.

(b) Unless otherwise authorized by ATC, each person operating an aircraft

shall do so in accordance with the special air traffic rules in this part in

addition to other applicable rules in Part 91 of this chapter.

[Doc. No. 1580, 28 FR 6715, June 29, 1963, as amended by Amdt. 93-10, 33 FR

4096, Mar. 2, 1968; Amdt. 93-15, 34 FR 2603, Feb. 26, 1969; Amdt. 93-33, 41

FR 14879, Apr. 8, 1976; Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992]

EFFECTIVE DATE NOTE: Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992, revised paragraph (b) of Sec. 93.1 effective September 16,

1993. For the convenience of the user, the superseded text in effect until

Sept. 16, 1993, is set forth as follows:

Sec. 93.1   Applicability.

* * * * *

(b) Unless otherwise authorized by ATC (with the exception of Sec. 93.113),

each person operating an aircraft shall do so in accordance with the special

air traffic rules in this part in addition to other applicable rules in Part

91 of this chapter.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Subparts B-C-- [Reserved]

Subpart D--Anchorage, Alaska, Terminal Area

Source: Docket No. 1580, 28 FR 6715, June 29, 1963, unless otherwise noted.

Sec. 93.51   Applicability.

This subpart prescribes the Anchorage, Alaska, Airport traffic area and

special traffic patterns for that airport and other airports in the vicinity

of Anchorage. It prescribes special air traffic rules for that area and those

patterns. In addition, it prescribes rules governing the operation of

aircraft in the vicinity of the airports described herein.

Sec. 93.53   Description of area.

The Anchorage Airport traffic area is designated as that airspace extending

upward from the surface to, but not including, 3,000 feet MSL. It is bounded

by a line beginning at Point MacKenzie extending westerly along the bank of

Knik Arm to a point intersecting an arc of five-statute-mile radius circle

centered on the geographical center of Anchorage International Airport;

thence counterclockwise along that arc to its intersection with the new

Seward Highway; thence northerly along the new Seward Highway to its

intersection with Tudor Road; thence easterly along Tudor Road to its

intersection with Muldoon Road; thence northerly along Muldoon Road to a

point one-half statute mile south of Glenn Highway; thence northeasterly

along a line one-half statute mile east of and parallel to Glenn Highway to

its intersection with a line one-half statute mile east of and parallel to

Bryant Airport runway 16/34; thence northeastward along this line to its

intersection with a point which is lat. 61 deg.17'15" N., long. 149

deg.37'10" W.; thence west to lat. 61 deg.17'15" N., long. 149 deg.42'25" W.;

thence northwest to lat. 61 deg.19'12" N., long. 149 deg.46'36" W.; thence

via an arc of a five-statute-mile radius circle centered on the geographical

center of Elmendorf Air Force Base; thence counterclockwise along this arc to

its intersection with the west bank of Knik Arm; thence southerly along the

west bank of Knik Arm to the point of beginning. However, it does not include

the following:

(a) That airspace at and below 600 feet MSL, north of a line beginning at

the intersection of Farrell Road and the northeast boundary of the airport

traffic area extending westerly along Farrell Road to the east end of Sixmile

Lake; thence along a line bearing on the middle of Lake Lorraine to the

boundary of the airport traffic area.

(b) That airspace at and below 600 feet MSL, south of a line beginning at

the intersection of the new Seward Highway and Dimond Boulevard extending

westerly along Dimond Boulevard to Sand Lake Road thence due west to the

boundary of the airport traffic area.

(c) [Reserved]

(d) That airspace described as the "Bryant segment" in Sec. 93.55(e), when

the Bryant control tower is not in operation.

[Doc. No. 12425, Amdt. 93-29, 39 FR 32551, Sept. 9, 1974, as amended by Amdt.

93-51, 51 FR 18312, May 19, 1986]

Sec. 93.55   Subdivision of area.

The Anchorage Airport traffic area is subdivided as follows:

(a) International segment. That area lying within a line beginning at the

International Airport control tower extending northwesterly on a direct line

toward the substation to the airport traffic area boundary; thence

counterclockwise along the airport traffic area boundary to its intersection

with International Airport Road; thence westerly along International Airport

Road to the point of beginning.

(b) Merrill segment. That area lying within a line beginning at Point

MacKenzie extending directly to the mouth of Fish Creek; thence along Fish

Creek to Northern Lights Boulevard; thence direct to the intersection of

Tudor Road and the new Seward Highway; thence east and north along the

airport traffic area boundary to a point directly east of the intersection of

Glenn Highway and Boniface Parkway; thence due west on a direct line through

that intersection to Ship Creek; thence along Ship Creek to its mouth; thence

on a direct line toward the center of Lake Lorraine to the airport traffic

area boundary; thence counterclockwise along the airport traffic area

boundary to the point of beginning.

(c) Lake Hood segment. That area lying between the International segment

and the Merrill segment.

(d) Elmendorf segment. That area lying within a line beginning at the

intersection of the airport traffic area boundary with Loop Road, extending

southerly along Loop Road to Davis Highway; thence due south to the north

boundary of the Merrill segment; thence westward along the north boundary of

the Merrill segment to the airport traffic area boundary; thence clockwise

along the airport traffic area boundary to the point of beginning.

(e) Bryant segment. That area lying east of the Elmendorf segment.

[Doc. No. 1580, 28 FR 6715, June 29, 1963, as amended by Amdt. 93-29, 39 FR

32552, Sept. 9, 1974; Amdt. 93-51, 51 FR 18312, May 19, 1986]

Sec. 93.57   General rules: All segments.

(a) Each person piloting an aircraft to, from or on an airport within the

airport traffic area shall operate it according to the rules set forth in

this section and Sec. 93.59, Sec. 93.61, Sec. 93.63, Sec. 93.65, or Sec.

93.67, as applicable, unless otherwise authorized or required by ATC.

(b) Each person piloting an airplane shall conform to the flow of traffic

shown on the appropriate diagram in Appendix A.

(c) Each person piloting a helicopter shall operate it in a manner avoiding

the flow of airplanes.

(d) Except as provided in Sec. 93.65 (d) and (e), each person piloting an

aircraft in the airport traffic area shall operate it only within the

designated segment containing the airport of landing or takeoff.

(e) Except as provided in Sec. 93.63(d), each person piloting an aircraft

shall maintain two-way radio communications with the control tower serving

the segment containing the airport of landing or takeoff.

[Doc. No. 1580, 28 FR 6715, June 29, 1963, as amended by Amdt. 93-8, 30 FR

8568, July 7, 1965; Amdt. 93-29, 39 FR 32552, Sept. 9, 1974]

Sec. 93.59   General rules: International segment.

(a) No person may pilot an aircraft at an altitude between 1,200 feet MSL

and 2,000 feet MSL in that part of the segment lying north of the midchannel

of Knik Arm.

(b) Each person piloting an airplane at a speed of more than 105 knots

within the segment (except that part described in paragraph (a) of this

section) shall operate it at an altitude of at least 1,600 feet MSL until

maneuvering for a safe landing requires further descent.

(c) Each person piloting an airplane at a speed of 105 knots or less within

the segment (except that part described in paragraph (a) of this section)

shall operate it at an altitude of at least 900 feet MSL until maneuvering

for a safe landing requires further descent.

Sec. 93.61   General rules: Lake Hood segment.

(a) No person may pilot an aircraft at an altitude between 1,200 feet MSL

and 2,000 feet MSL in that part of the segment lying north of the midchannel

of Knik Arm.

(b) Each person piloting an airplane within the segment (except that part

described in paragraph (a) of this section) shall operate it at an altitude

of at least 600 feet MSL until maneuvering for a safe landing requires

further descent.

[Doc. No. 1580, 28 FR 6715, June 29, 1963, as amended by Amdt. 93-51, 51 FR

18312, May 19, 1986

Sec. 93.63   General rules: Merrill segment.

(a) No person may pilot an aircraft at an altitude between 600 feet MSL and

2,000 feet MSL in that part of the segment lying north of the midchannel of

Knik Arm.

(b) Each person piloting an airplane at a speed of more than 105 knots

within the segment (except that part described in paragraph (a) of this

section) shall operate it at an altitude of at least 1,200 feet MSL until

maneuvering for a safe landing requires further descent.

(c) Each person piloting an airplane at a speed of 105 knots or less within

the segment (except that part described in paragraph (a) of this section)

shall operate it at an altitude of at least 900 feet MSL until maneuvering

for a safe landing requires further descent.

(d) Whenever the Merrill control tower is not operating, each person

piloting an aircraft within the segment shall maintain two-way radio

communication with the Anchorage Flight Service Station.

[Doc. No. 1580, Amdt. 1-1, 28 FR 6715, June 29, 1963, as amended by Amdt. 93-

6, 29 FR 15949, Dec. 1, 1964; Amdt. 93-29, 39 FR 32552, Sept. 9, 1974]

Sec. 93.65   General rules: Elmendorf segment.

(a) Each person piloting a turbine-powered airplane within the segment

shall operate it at an altitude of at least 1,700 feet MSL until maneuvering

for a safe landing requires further descent.

(b) Each person piloting an airplane (other than turbine powered) at a

speed of more than 105 knots within the segment shall operate it at an

altitude of at least 1,200 feet MSL until maneuvering for a safe landing

requires further descent.

(c) Each person piloting an airplane (other than turbine powered) at a

speed of 105 knots or less within the segment shall operate it at an altitude

of at least 700 feet MSL until maneuvering for a safe landing requires

further descent.

(d) A person landing or taking off an aircraft from Elmendorf Airport may

operate it at an altitude between 1,500 feet MSL and 1,700 feet MSL within

those parts of the International and Lake Hood segments lying north of the

midchannel of Knik Arm.

(e) A person landing or taking off an aircraft from Elmendorf Airport may

operate it at an altitude between 900 feet MSL and 1,700 feet MSL within that

part of the Merrill segment lying north of the midchannel of Knik Arm.

Sec. 93.67   General rules: Bryant segment.

(a) Each person piloting an airplane within the segment shall operate it at

an altitude of at least 1,000 feet MSL until maneuvering for a safe landing

requires further descent.

(b) Whenever Bryant control tower is not operating, each person piloting an

aircraft to or from the Bryant Airport shall conform to the flow of traffic

shown on the appropriate diagram in Appendix A of this part, and, while in

the traffic pattern, shall operate at an altitude of at least 1,000 feet

m.s.l. until maneuvering for a safe landing requires further descent.

[Doc. No. 1580, 28 FR 6715, June 29, 1963, as amended by Amdt. 93-8, 30 FR

8568, July 7, 1965]

Sec. 93.69   Special Requirements, Lake Campbell and Sixmile Lake Airports.

Each person piloting an aircraft to or from Lake Campbell or Sixmile Lake

Airport shall conform to the flow of traffic for the Lake operations that are

shown on the appropriate diagram in Appendix A.

[Doc. No. 1580, 28 FR 6715, June 29, 1963, as amended by Amdt. 93-8, 30 FR

8568, July 7, 1965]

Subpart E--VOR Federal Airway No. 16

Source: Amdt. 93-20, 35 FR 10856, July 3, 1970, unless otherwise noted.

Effective Date Note: At 54 FR 50992, December 11, 1989, Subpart E,

consisting of Secs. 93.71 through 93.75, was removed, effective January 11,

1990.

Sec. 93.71   Applicability.

This subpart applies to aircraft operated under Visual Flight Rules within

VOR Federal Airway 16 between longitudes 112 deg.23'00'' W. and 112

deg.41'30'' W., excluding that portion of the airspace within a 4-statute-

mile radius of Phoenix-Litchfield Airport (latitude 33 deg.25'25'' N.,

longitude 112 deg.22'30'' W.), Monday through Saturday, from 0600 to 0100 the

following day, local time.

Sec. 93.73   Crossing VOR Federal Airway No. 16; VFR Jet Training Operations.

Each pilot in command of a Luke Air Force Base jet aircraft operating

outside of Luke Air Force Base airport traffic pattern under Visual Flight

Rules and engaged in a training operation that requires crossing of VOR

Federal Airway No. 16 within the airspace specified in Sec. 93.71, shall

cross within that specified airspace at altitudes from 2,500 feet m.s.l. to

5,000 m.s.., inclusive.

Sec. 93.75   Crossing and operating along VOR Federal Airway No. 16.

Each person piloting an aircraft (other than an aircraft to which Sec.

93.73 applies, and aircraft departing Luke Air Force Base) crossing or

operating along VOR Federal Airway No. 16 in the area specified in Sec. 93.71

shall operate--

(a) At 2,000 feet m.s.l., or lower, or

(b) At 5,500 feet m.s.l., or higher.

Subpart F--Valparaiso, Florida, Terminal Area

*****************************************************************************

59 FR 46152, No. 171, Sept. 6, 1994

Amdt. 93-70

SUMMARY: This action retains the Valparaiso, Florida Terminal Area and

Special Air Traffic Rules in part 93 of the Federal Aviation Regulations

(FAR); revises the Class D airspace areas for Eglin Air Force Base (AFB), the

Eglin Air Force (AF) Auxiliary No. 3 Duke Field, and Hurlburt Field; revises

the Crestview Class E airspace area; and deletes the Eglin Class D North-

South corridor. Additionally, this action modifies the established North-

South and East-West corridors associated with the Valparaiso, Florida

Terminal Area and Eglin AFB in part 93 of the FAR. This action is necessary

to simplify operating procedures, airspace assignment and airspace use within

the Valparaiso, Florida Terminal Area.

EFFECTIVE DATE: This amendment is effective on December 8, 1994.

*****************************************************************************

Sec. 93.81   Applicability and description of area.

(a) This subpart prescribes the Valparaiso, Florida Terminal Area, and the

special air traffic rules for operating aircraft within that Area.

(b) The Valparaiso, Florida Terminal Area is designated as follows:

(1) North-South Corridor. The North-South Corridor includes the airspace

extending upward from the surface up to, but not including, 18,000 feet MSL,

bounded by a line beginning at:

Latitude 30 deg.42'51" N., Longitude 86 deg.38'02" W.; to

Latitude 30 deg.43'18" N., Longitude 86 deg.27'37" W.; to

Latitude 30 deg.37'01" N., Longitude 86 deg.27'37" W.; to

Latitude 30 deg.37'01" N., Longitude 86 deg.25'30" W.; to

Latitude 30 deg.33'01" N., Longitude 86 deg.25'30" W.; to

Latitude 30 deg.33'01" N., Longitude 86 deg.25'00" W.; to

Latitude 30 deg.25'01" N., Longitude 86 deg.25'00" W.; to

Latitude 30 deg.25'01" N., Longitude 86 deg.38'12" W.; to

Latitude 30 deg.29'02" N., Longitude 86 deg.38'02" W.; to point of beginning.

(2) East-West Corridor--The East-West Corridor is divided into three

sections to accommodate the different altitudes as portions of the corridor

underlie restricted areas R-2915C, R-2919B, and R-2914B.

(i) The west section would include that airspace extending upward from the

surface to but not including 8,500 feet MSL, bounded by a line beginning at:

Latitude 30 deg.22'47" N., Longitude 86 deg.51'30" W.: then along the

shoreline to Latitude 30 deg.23'46" N., Longitude 86 deg.38'15" W.; to

Latitude 30 deg.20'51" N., Longitude 86 deg.38'50" W.; then 3 NM from and

parallel to the shoreline to Latitude 30 deg.19'31" N., Longitude 86

deg.51'30" W.; to the beginning.

(ii) The center section would include that airspace extending upward from

the surface to but not including 18,000 feet MSL, bounded by a line beginning

at:

Latitude 30 deg.25'01" N., Longitude 86 deg.38'12" W.; to

Latitude 30 deg.25'01" N., Longitude 86 deg.25'00" W.; to

Latitude 30 deg.25'01" N., Longitude 86 deg.22'26" W.; to

Latitude 30 deg.19'46" N., Longitude 86 deg.23'45" W.; then 3 NM from and

parallel to the shoreline to Latitude 30 deg.20'51" N.,

Longitude 86 deg.38'50" W.; to Latitude 30 deg.23'46" N.,

Longitude 86 deg.38'15" W.; to the beginning.

(iii) The east section would include that airspace extending upward from

the surface to but not including 8,500 feet MSL, bounded by a line beginning

at:

Latitude 30 deg.25'01" N., Longitude 86 deg.22'26" W.; to

Latitude 30 deg.22'01" N., Longitude 86 deg.08'00" W.; to

Latitude 30 deg.19'16" N., Longitude 85 deg.56'00" W.; to

Latitude 30 deg.11'01" N., Longitude 85 deg.56'00" W.; then 3 NM from and

parallel to the shoreline to Latitude 30 deg.19'46" N., Longitude 86

deg.23'45" W.; to the beginning.

[Doc. No. 26968, Amdt. 93-70, 59 FR 46154, Sept. 6, 1994]

*****************************************************************************

59 FR 46152, No. 171, Sept. 6, 1994

SUMMARY: This action retains the Valparaiso, Florida Terminal Area and

Special Air Traffic Rules in part 93 of the Federal Aviation Regulations

(FAR); revises the Class D airspace areas for Eglin Air Force Base (AFB), the

Eglin Air Force (AF) Auxiliary No. 3 Duke Field, and Hurlburt Field; revises

the Crestview Class E airspace area; and deletes the Eglin Class D North-

South corridor. Additionally, this action modifies the established North-

South and East-West corridors associated with the Valparaiso, Florida

Terminal Area and Eglin AFB in part 93 of the FAR. This action is necessary

to simplify operating procedures, airspace assignment and airspace use within

the Valparaiso, Florida Terminal Area.

EFFECTIVE DATE: This amendment is effective on December 8, 1994.

*****************************************************************************

Sec. 93.83  Aircraft Operations.

(a) North-South Corridor. Unless otherwise authorized by ATC (including the

Eglin Radar Control Facility), no person may operate an aircraft in flight

within the North-South Corridor designated in Sec. 93.81(b)(1) unless--

(1) Before operating within the corridor, that person obtains a clearance

from the Eglin Radar Control Facility or an appropriate FAA ATC facility; and

(2) That person maintains two-way radio communication with the Eglin Radar

Control Facility or an appropriate FAA ATC facility while within the

corridor.

(b) East-West Corridor. Unless otherwise authorized by ATC (including the

Eglin Radar Control Facility), no person may operate an aircraft in flight

within the East-West Corridor designated in Sec. 93.81(b)(2) unless--

(1) Before operating within the corridor, that person establishes two-way

radio communications with Eglin Radar Control Facility or an appropriate FAA

ATC facility and receives an ATC advisory concerning operations being

conducted therein; and

(2) That person maintains two-way radio communications with the Eglin Radar

Control Facility or an appropriate FAA ATC facility while within the

corridor.

[Doc. No. 26968, Amdt. 93-70, 59 FR 46155, Sept. 6, 1994]

*****************************************************************************

59 FR 46152, No. 171, Sept. 6, 1994

SUMMARY: This action retains the Valparaiso, Florida Terminal Area and

Special Air Traffic Rules in part 93 of the Federal Aviation Regulations

(FAR); revises the Class D airspace areas for Eglin Air Force Base (AFB), the

Eglin Air Force (AF) Auxiliary No. 3 Duke Field, and Hurlburt Field; revises

the Crestview Class E airspace area; and deletes the Eglin Class D North-

South corridor. Additionally, this action modifies the established North-

South and East-West corridors associated with the Valparaiso, Florida

Terminal Area and Eglin AFB in part 93 of the FAR. This action is necessary

to simplify operating procedures, airspace assignment and airspace use within

the Valparaiso, Florida Terminal Area.

EFFECTIVE DATE: This amendment is effective on December 8, 1994.

*****************************************************************************

Subpart G--[Reserved]

Subpart H--[Reserved]

Subpart I--[Reserved. Amdt. 93-64, 56 FR 65662, Dec. 17, 1991;

57 FR 11575, Apr. 6, 1992]

EFFECTIVE DATE NOTE: Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992, removed and reserved subpart I (Secs. 93.111 to 93.113) of

part 93 effective September 16, 1993. The heading of subpart I in effect

until September 16, 1993 reads as follows:

Subpart I--Locations at Which Special VFR Weather Minimums Do Not Apply

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Subpart J--Lorain County Regional Airport Traffic Rule

Sec. 93.117   Applicability.

This subpart prescribes a special air traffic rule for the Lorain County

Regional Airport, Lorain County, Ohio.

[Doc. No. 8669, 33 FR 11749, Aug. 20, 1968]

Sec. 93.119   Aircraft operations.

Each person piloting an airplane landing at the Lorain County Regional

Airport shall enter the traffic pattern north of the airport and shall

execute a right traffic pattern for a landing to the southwest or a left

traffic pattern for a landing to the northeast. Each person taking off from

the airport shall execute a departure turn to the north as soon as

practicable after takeoff.

[Doc. No. 8669, 33 FR 11749, Aug. 20, 1968]

Subpart K--High Density Traffic Airports

Sec. 93.121   Applicability.

This subpart designates high density traffic airports and prescribes air

traffic rules for operating aircraft, other than helicopters, to or from

those airports.

[Amdt. 93-21, 35 FR 16592, Oct. 24, 1970, as amended by Amdt. 93-27, 38 FR

29464, Oct. 25, 1973]

Sec. 93.123   High density traffic airports.

(a) Each of the following airports is designated as a high density traffic

airport and, except as provided in Sec. 93.129 and paragraph (b) of this

section, or unless otherwise authorized by ATC, is limited to the hourly

number of allocated IFR operations (takeoffs and landings) that may be

reserved for the specified classes of users for that airport:

IFR Operations per Hour

Airport

O'Hare  Washington

Class of    LaGuardia           /2/     National

user         /4/     Newark   /3/       /1/

Air carriers         48      40     120          37

Commuters            14      10      25          11

Other                 6      10      10          12

John F. Kennedy

Air

carriers  Commuters  Other

1500        69         15      2

1600        74         12      2

1700        80         13      0

1800        75         10      2

1900        63         12      2

/1/ Washington National Airport

operations are subject to modifications

per Section 93.124.

/2/ The hour period in effect at O'Hare

begins at 6:45 a.m. and continues in

30-minute increments until 9:15 p.m.

/3/ Operations at O'Hare International

Airport shall not--

(a) Except as provided in paragraph (c)

of the note, exceed 62 for air carriers

and 13 for commuters and 5 for "other"

during any 30-minute period beginning

at 6:45 a.m. and continuing every 30

minutes thereafter.

(b) Except as provided in paragraph (c)

of the note, exceed more than 120 for

air carriers, 25 for commuters, and 10

for "other" in any two consecutive 30-

minute periods.

(c) For the hours beginning at 6:45

a.m., 7:45 a.m., 11:45 a.m., 7:45 p.m.

and 8:45 p.m., the hourly limitations

shall be 105 for air carriers, 40 for

commuters and 10 for "other," and the

30-minute limitations shall be 55 for

air carriers, 20 for commuters and 5

for "other." For the hour beginning at

3:45 p.m., the hourly limitations shall

be 115 for air carriers, 30 for

commuters and 10 for "others", and the

30-minute limitations shall be 60 for

air carriers, 15 for commuters and 5

for "other."

/4/ Operations at LaGuardia Airport

shall not--

(a) Exceed 26 for air carriers, 7 for

commuters and 3 for "other" during any

30-minute period.

(b) Exceed 48 for air carriers, 14 for

commuters, and 6 for "other" in any two

consecutive 30-minute periods.

(b) The following exceptions apply to the allocations of reservations

prescribed in paragraph (a) Of this section.

(1) The allocations of resevations among the several classes of users do

not apply from 12 midnight to 6 a.m. local time, but the totala hourly

limitation remains applicable.

(2) [Reserved]

(3) The allocation of 37 IFR reservations per hour for air carriers except

commuters at Washington National Airport does not include charter flights, or

other nonscheduled flights of scheduled or supplemental air carriers. These

flights may be conducted without regard to the limitation of 37 IFR

reservations per hour.

(4) The allocation of IFR reservations for air carriers except commuters at

LaGuardia, Newark, O'Hare, and Washington National Airports does not include

extra sections of scheduled flights. The allocation of IFR reservations for

scheduled commuters at Washington National Airport does not include extra

sections of scheduled flights. These flights may be conducted without regard

to the limitation upon the hourly IFR reservations at those airports.

(5) Any reservation allocated to, but not taken by, air carrier operations

(except commuters) is available for a scheduled commuter operation.

(6) Any reservation allocated to, but not taken by, air carrier operations

(except commuters) or scheduled commuter operations is available for other

operations.

(c) For purposes of this subpart--

(1) The number of operations allocated to "air carriers except commuters,"

as used in paragraph (a) of this section refers to the number of operations

conducted by air carriers with turboprop and reciprocating engine aircraft

having a certificated maximum passenger seating capacity of 75 or more or

with turbojet powered aircraft having a certificated maximum passenger

seating capacity of 56 or more, or, if used for cargo service in air

transportation, with any aircraft having a maximum payload capacity of 18,000

pounds or more.

(2) The number of operations allocated to "scheduled commuters," as used in

paragraph (a) of this section, refers to the number of operations conducted

by air carriers with turboprop and reciprocating engine aircraft having a

certificated maximum passenger seating capacity of less than 75 or by

turbojet aircraft having a certificated maximum passenger seating capacity of

less than 56, or, if used for cargo service in air transportation, with any

aircraft having a maximum payload capacity of less than 18,000 pounds.

(3) Notwithstanding the provisions of paragraph (c)(2) of this Section, a

limited number of operations allocated for "scheduled commuters" under

paragraph (a) of this section may be conducted with aircraft described in

Sec. 93.221(e) of this part pursuant to the requirements of Sec. 93.221(e).

Note: The effective date of paragraph (c), to the extent it defines

turbojet aircraft with a maximum certificated seating capacity of less than

56 seats as air carrier aircraft, is suspended indefinitely.

[Doc. No. 9113, 34 FR 2603, Feb. 26, 1969, as amended by Amdt. 93-37, 45 FR

62408, Sept. 18, 1980; Amdt. 93-44, 46 FR 58048, Nov. 27, 1981; Amdt. 93-46,

49 FR 8244, Mar. 6, 1984; Amdt. 93-57, 54 FR 34906, Aug. 22, 1989; 54 FR

37303, Sept. 8, 1989; Amdt. 93-59, 54 FR 39843, Sept. 28, 1989; 56 FR 41207,

Aug. 19, 1991]

*****************************************************************************

56 FR 41200, No. 160, Aug. 19, 1991

SUMMARY: This action amends the regulations pertaining to the allocation

and definition of commuter operator slots (i.e., allocated instrument flight

rules (IFR) takeoff and landing reservations) at O'Hare International

Airport. Under the rule as adopted, the FAA will permit a limited number of

commuter slots at O'Hare International Airport to be used by aircraft having

a maximum seating capacity of up to 110 passenger seats. This amendment is in

response to a petition for rulemaking submitted by American Airlines and

subsequent comments received on the petition and notice of proposed

rulemaking. The FAA will limit the number of commuter slots available for

operation of such aircraft to 25 percent of each operator's commuter slots at

O'Hare International Airport, and limit the number of such operations in any

half hour. This amendment will remain in effect for a 2-year period to allow

the FAA to evaluate the effect of the change on the operation of the airport

and air traffic facilities, and may be extended. This action will relieve

airlines at O'Hare of certain existing restrictions and permit (but not

necessarily result in) additional jet service to some smaller communities

while still preserving the class of commuter slots as distinct from air

carrier slots.

EFFECTIVE DATE: Rule effective September 18, 1991.

*****************************************************************************

Sec. 93.125   Arrival or departure reservation.

Except between 12 Midnight and 6 a.m. local time, no person may operate an

aircraft to or from an airport designated as a high density traffic airport

unless he has received, for that operation, an arrival or departure

reservation from ATC.

[Doc. No. 9974, Amdt. 93-25, 37 FR 22794, Oct. 25, 1972]

Sec. 93.129   Additional operations.

(a) IFR. The operator of an aircraft may take off or land the aircraft

under IFR at a designated high density traffic airport without regard to the

maximum number of operations allocated for that airport if the operation is

not a scheduled operation to or from a high density airport and he obtains a

departure or arrival reservation, as appropriate, from ATC. The reservation

is granted by ATC whenever the aircraft may be accommodated without

significant additional delay to the operations allocated for the airport for

which the reservations is requested.

(b) VFR. The operator of an aircraft may take off and land the aircraft

under VFR at a designated high density traffic airport without regard to the

maximum number of operations allocated for that airport if the operation is

not a scheduled operation to or from a high density airport and he obtains a

departure or arrival reservation, as appropriate, from ATC. The reservation

is granted by ATC whenever the aircraft may be accommodated without

significant additional delay to the operations allocated for the airport for

which the reservation is requested and the ceiling reported at the airport is

at least 1,000 feet and the ground visibility reported at the airport is at

least 3 miles.

(c) For the purpose of this section a "scheduled operation to or from the

high density airport" is any operation regularly conducted by an air carrier

or commuter between a high density airport and another point regularly served

by that operator unless the service is conducted pursuant to irregular

charter or hiring of aircraft or is a nonpassenger flight.

(d) An aircraft operator must obtain an IFR reservation in accordance with

procedures established by the Administrator. For IFR flights to or from a

high density airport, reservations for takeoff and arrival shall be obtained

prior to takeoff.

[Doc. No. 9113, 34 FR 2603, Feb. 26, 1969, as amended by Amdt. 93-25, 37 FR

22794, Oct. 25, 1972; Amdt. 93-44, 46 FR 58049, Nov. 27, 1981; Amdt. 93-46,

49 FR 8244, Mar. 6, 1984]

Sec. 93.130   Suspension of allocations.

The Administrator may suspend the effectiveness of any allocation

prescribed in Sec. 93.123 and the reservation requirements prescribed in Sec.

93.125 if he finds such action to be consistent with the efficient use of the

airspace. Such suspension may be terminated whenever the Administrator

determines that such action is necessary for the efficient use of the

airspace.

[Amdt. 93-21, 35 FR 16592, Oct. 24, 1970, as amended by Amdt. 93-21, 35 FR

16636, Oct. 27, 1970; Amdt. 93-27, 38 FR 29464, Oct. 25, 1973]

Sec. 93.133  Exceptions.

Except as provided in Sec. 93.130, the provisions of Secs. 93.123 and

93.125 do not apply to--

(a) The Newark Airport, Newark, New Jersey;

(b) The Kennedy International Airport, New York, New York, except during

the hours from 3:00 p.m. through 7:59 p.m., local time; and

(c) O'Hare International Airport from 9:15 p.m. to 6:44 a.m., local time.

[Doc. No. 24471, Amdt. 93-46, 49 FR 8244, Mar. 6, 1984]

Subpart L--[Reserved]

Subpart M--Ketchikan International Airport Traffic Rule

Source: Docket No. 14687, Amdt. 93-33, 41 FR 14879, Apr. 8, 1976, unless

otherwise noted.

Sec. 93.151  Applicability.

This subpart prescribes special air traffic rules and communications

requirements for persons operating aircraft, under VFR--

(a) To, from, or in the vicinity of the Ketchikan International Airport or

Ketchikan Harbor.

(b) Within the airspace below 3,000 feet MSL within the lateral boundary of

the surface area of the Ketchikan Class E airspace regardless of whether that

airspace is in effect.

[Docket No. 26653, Amdt. No. 93-63, 56 FR 48094, Sept. 23, 1991, as amended

at 58 FR 32839, June 14, 1993; Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR

11575, Apr. 6, 1992; 58 FR 32839, June 14, 1993]

EFFECTIVE DATE NOTE: Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992, as amended at 58 FR 32839, June 14, 1993, revised the

introductory text of Sec. 93.151 effective September 16, 1993. For the

convenience of the user, the superseded text in effect until Sept. 16, 1993,

is set forth as follows:

Sec. 93.151   Applicability.

This subpart prescribes special air traffic rules and communication

requirements for persons operating aircraft under VFR--

* * * * *

*****************************************************************************

58 FR 32838, No. 112, June 14, 1993

SUMMARY: This action corrects the Airspace Reclassification final rule

that was promulgated on December 17, 1991, by deleting Class B from the

airspace areas where the indicated airspeed of aircraft is restricted to 200

knots; by continuing the special communications requirements currently in

effect at Ketchikan International Airport, Alaska; and by deleting Class B

and Class C airspace areas from the exception for agricultural aircraft

operating to and from dispensing areas. The correction is necessary to ensure

the operating rules continue as intended on September 16, 1993, the

implementation date of Airspace Reclassification.

EFFECTIVE DATE: This amendment is effective September 16, 1993.

*****************************************************************************

Sec. 93.153  Communications.

(a) When the Ketchikan Flight Service Station is in operation, no person

may operate an aircraft within the airspace specified in Sec. 93.151, or taxi

onto the runway at Ketchikan International Airport, unless that person has

established two-way radio communications with the Ketchikan Flight Service

Station for the purpose of receiving traffic advisories and continues to

monitor the advisory frequency at all times while operating within the

specified airspace.

(b) When the Ketchikan Flight Service Station is not in operation, no

person may operate an aircraft within the airspace specified in Sec. 93.151,

or taxi onto the runway at Ketchikan International Airport, unless that

person continuously monitors and communicates, as appropriate, on the

designated common traffic advisory frequency as follows:

(1) For inbound flights. Announces position and intentions when no less

than 10 miles from Ketchikan International Airport, and monitors the

designated frequency until clear of the movement area on the airport or

Ketchikan Harbor.

(2) For departing flights. Announces position and intentions prior to

taxiing onto the active runway on the airport or onto the movement area of

Ketchikan Harbor and monitors the designated frequency until outside the

airspace described in Sec. 93.151 and announces position and intentions upon

departing that airspace.

(c) Notwithstanding the provisions of paragraphs (a) and (b) of this

section, if two-way radio communications failure occurs in flight, a person

may operate an aircraft within the airspace specified in Sec. 93.151, and

land, if weather conditions are at or above basic VFR weather minimums.

[Docket No. 26653, Amdt. No. 93-63, 56 FR 48094, Sept. 23, 1991]

*****************************************************************************

56 FR 48092, No. 184, Sept. 23, 1991

SUMMARY: This action amends the special air traffic rule at Ketchikan,

Alaska, by establishing rule applicability in all portions of the Ketchikan

Control Zone. The rule formerly excluded certain portions of the airspace

below 600 feet mean sea level (MSL). This action also clarifies the original

intent of the rule by specifying that pilots must comply with certain traffic

advisory and self-announcement procedures while operating in the control

zone. The FAA believes that the level of safety provided for aircraft

operations in the Ketchikan area will be enhanced by this amendment.

EFFECTIVE DATE: October 23, 1991.

*****************************************************************************

Sec. 93.155   Aircraft operations.

(a) When an advisory is received from the Ketchikan Flight Service Station

stating that an aircraft is on final approach to the Ketchikan International

Airport, no person may taxi onto the runway of that airport until the

approaching aircraft has landed and has cleared the runway.

(b) Unless otherwise authorized by ATC, each person operating a large

airplane or a turbine engine powered airplane shall--

(1) When approaching to land at the Ketchikan International Airport,

maintain an altitude of at least 900 feet MSL until within three miles of the

airport; and

(2) After takeoff from the Ketchikan International Airport, maintain runway

heading until reaching an altitude of 900 feet MSL.

Subpart N--[Reserved. Amdt. 93-64, 56 FR 65662, Dec. 17, 1991;

57 FR 11575, Apr. 6, 1992]

EFFECTIVE DATE NOTE: Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992, removed and reserved subpart N (Secs. 93.161 to 93.163) of

part 93 effective September 16, 1993. The heading of subpart N in effect

until September 16, 1993 reads as follows:

Subpart N--Sabre U.S. Army Heliport (Tenn.) Airport Traffic Area

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Subpart O--[Reserved. Amdt. 93-64, 56 FR 65662, Dec. 17, 1991;

57 FR 11575, Apr. 6, 1992]

EFFECTIVE DATE NOTE: Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992, removed and reserved subpart O (Secs. 93.171 to 93.175) of

part 93 effective September 16, 1993. The heading of subpart O in effect

until September 16, 1993 reads as follows:

Subpart O--Jacksonville, Florida, Navy Airport Traffic Area

Source: Docket No. 15384, Amdt. No. 93-95, 42 FR 47824, Sept. 22, 1977,

unless otherwise noted.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Subpart P--[Reserved. Amdt. 93-67, 58 FR 12137, Mar. 2, 1993]

*****************************************************************************

58 FR 12128, No. 39, Mar. 2, 1993

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) by

designating additional control areas as offshore airspace areas or en route

domestic airspace areas, as appropriate; revising certain additional control

areas; including restricted and prohibited areas in the Continental Control

Area; eliminating domestic area low routes; eliminating control areas

associated with jet routes outside the Continental Control Area; eliminating

domestic high altitude reporting points; eliminating certain domestic low

altitude reporting points; eliminating the special air traffic rules for

Flushing, New York; and replacing the Valparaiso, Florida terminal area and

special air traffic rules with the Elgin, Florida Class D airspace areas.

These amendments respond to recommendations from the National Airspace Review

(NAR) and meet a goal of the Airspace Reclassification final rule--to

simplify airspace assignment and use.

EFFECTIVE DATES: The amendments to Secs. 71.1 and 71.9 which are currently

in effect, become effective April 1, 1993, through September 15, 1993; the

removal of Sec. 71.6 becomes effective April 1, 1993; the removal of subpart

P of part 93 becomes effective June 20, 1993; the amendments to Secs. 71.1,

71.33 and 71.71 which are effective September 16, 1993, and the removal of

Sec. 71.77, become effective September 16, 1993; and the removal of subpart F

of part 93 becomes effective December 9, 1993.

*****************************************************************************

Subpart Q--[Reserved. Amdt. 93-64, 56 FR 65662, Dec. 17, 1991;

57 FR 11575, Apr. 6, 1992]

EFFECTIVE DATE NOTE: Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992, removed and reserved subpart Q (Secs. 93.195 to 93.199) of

part 93 effective September 16, 1993. The heading of subpart Q in effect

until September 16, 1993 reads as follows:

Subpart Q--Abbotsford, BC, and Sault Ste. Marie, ON: Special Airport Traffic

Areas and Air Traffic Rules

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Subpart R--[Reserved. Amdt. 93-64, 56 FR 65662, Dec. 17, 1991;

57 FR 11575, Apr. 6, 1992]

EFFECTIVE DATE NOTE: Amdt. 93-64, 56 FR 65662, Dec. 17, 1991; 57 FR 11575,

Apr. 6, 1992, removed and reserved subpart R (Secs. 93.200 to 93.208) of

part 93 effective September 16, 1993. The heading of subpart R in effect

until September 16, 1993 reads as follows:

Subpart R--MCAS El Toro, CA, Special Air Traffic Rules

Source: Docket No. 24117, 50 FR 50267, Dec. 9, 1985, unless otherwise

noted.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Subpart S--Allocation of Commuter and Air Carrier IFR Operations at High

Density Traffic Airports

Source: Docket No. 24105, Amdt. 93-49, 50 FR 52195, Dec. 20, 1985, unless

otherwise noted.

Sec. 93.211  Applicability.

(a) This subpart prescribes rules applicable to the allocation and

withdrawal of IFR operational authority (takeoffs and landings) to individual

air carriers and commuter operators at the High Density Traffic Airports

identified in Subpart K of this part except for Newark Airport.

(b) This subpart also prescribes rules concerning the transfer of allocated

IFR operational authority and the use of that authority once allocated.

Sec. 93.213   Definitions and general provisions.

(a) For purposes of this subpart--

(1) New entrant carrier means a commuter operator or air carrier which does

not hold a slot at a particular airport and has never sold or given up a slot

at that airport after December 16, 1985.

(2) Slot means the operational authority to conduct one IFR landing or

takeoff operation each day during a specific hour or 30 minute period at one

of the High Density Traffic Airports, as specified in Subpart K of this part.

(3) Summer season means the period of time from the first Sunday in April

until the last Sunday in October.

(4) Winter season means the period of time from the fourth Sunday in

October until the first Sunday in April.

(5) Limited incumbent carrier means an air carrier or commuter operator

that holds or operates fewer than 12 air carrier or commuter slots, in any

combination, at a particular airport, not including international slots,

Essential Air Service Program slots, or slots between the hours of 2200 and

0659 at Washington National Airport or LaGuardia Airport. However, for the

purposes of this paragraph (a)(5), the carrier is considered to hold the

number of slots at that airport that the carrier has, since December 16,

1985:

(i) Returned to the FAA;

(ii) Had recalled by the FAA under Sec. 93.227(a); or

(iii) Transferred to another party other than by trade for one or more

slots at the same airport.

(b) The definitions specified in Subpart K of this part also apply to this

subpart.

(c) For purposes of this subpart, if an air carrier, commuter operator, or

other person has more than a 50-percent ownership or control of one or more

other air carriers, commuter operators, or other persons, they shall be

considered to be a single air carrier, commuter operator, or person. In

addition, if a single company has more than a 50-percent ownership or control

of two or more air carriers and/or commuter operators or any combination

thereof, those air carriers and/or commuter operators shall be considered to

be a single operator. A single operator may be considered to be both an air

carrier and commuter operator for purposes of this subpart.

[Doc. No. 24105, Amdt. 93-49, 50 FR 52195, Dec. 20, 1985, as amended by Amdt.

93-52, 51 FR 21717, June 13, 1986; Amdt. 93-57, 54 FR 34906, Aug. 22, 1989;

54 FR 37303, Sept. 8, 1989; Amdt. 93-65, 57 FR 37314, Aug. 18, 1992]

*****************************************************************************

57 FR 37308, No. 160, Aug. 18, 1992

SUMMARY: This action amends the Federal Aviation Regulations governing the

allocation and transfer of air carrier and commuter operator slots (i.e.,

instrument flight rules (IFR) takeoff and landing reservations) at Kennedy

International Airport, LaGuardia Airport, O'Hare International Airport, and

Washington National Airport. This amendment adds a definition of "limited

incumbent carrier" to refer to air carriers and commuter operators holding

fewer than 12 slots at a high density airport; provides for allocation of

slots by lottery to certain new entrant and limited incumbent carriers;

restricts the transfer of newly acquired lottery slots and provides for the

recall of those slots upon the sale or merger of the limited incumbent slot

holder; amends the minimum slot use requirements; amends the slot use

reporting requirements; and removes an obsolete penalty provision. This

amendment is designed to promote the availability of slots to new entrant and

limited incumbent carriers at the high density airports, and thereby enhance

competition.

EFFECTIVE DATE: November 1, 1992.

*****************************************************************************

Sec. 93.215   Initial allocation of slots.

(a) Each air carrier and commuter operator holding a permanent slot on

December 16, 1985, as evidenced by the records of the air carrier and

commuter operator scheduling committees, shall be allocated those slots

subject to withdrawal under the provisions of this subpart. The Chief Counsel

of the FAA shall be the final decisionmaker for initial allocation

determinations.

(b) Any permanent slot whose use on December 16, 1985 is divided among

different operators, by day of the week, or otherwise, as evidenced by

records of the scheduling committees, shall be allocated in conformity with

those records. The Chief Counsel of the FAA shall be the final decisionmaker

for these determinations.

(c) A carrier may permanently designate a slot it holds at Kennedy

International Airport as a seasonal slot, to be held by the carrier only

during the corresponding season in future years, if it notifies the FAA (at

the address specified in Sec. 93.225(e)), in writing, the preceding winter

seasons or by October 15 of the preceding year for summer seasons.

(d) Within 30 days after December 16, 1985, each U.S. air carrier and

commuter operator must notify the office specified in Sec. 93.221(a)(1), in

writing, of those slots used for operations described in Sec. 93.217(a)(1) on

December 16, 1985.

(e) Any slot not held by an operator on December 16, 1985 shall be

allocated in accordance with the provisions of Secs.93.217, 93.219 or 93.225

of this subpart.

[Doc. No. 24105, Amdt. 93-49, 50 FR 52195, Dec. 20, 1985, as amended by Amdt.

93-52, 51 FR 21717, June 13, 1986]

Sec. 93.217  Allocation of slots for international operations and applicable

limitations.

(a) Any air carrier or commuter operator having the authority to conduct

international operations shall be provided slots for those operations subject

to the following conditions and the other provisions of this section:

(1) The slot may be used only for a flight segment in which either the

takeoff or landing is at a foreign point or, for foreign operators, the

flight segment is a continuation of a flight that begins or ends at a foreign

point. Slots may be obtained and used under this section only for operations

at Kennedy and O'Hare airports unless otherwise required by bilateral

agreement and only for scheduled service unless the requesting carrier

qualifies for the slot on the basis of historic seasonal operations, under

Sec. 93.217(a)(5).

(2) Slots used for an operation described in paragraph (a)(1) of this

section may not be bought, sold, leased, or otherwise transferred, except

that such a slot may be traded to another slot-holder on a one-for-one basis

for a slot at the same airport in a different hour or half-hour period if the

trade is for the purpose of conducting such an operation in a different hour

or half-hour period.

(3) Slots used for operations described in paragraph (a)(1) of this section

must be returned to the FAA if the slot will not be used for such operations

for more than a 2-week period.

(4) Each air carrier or commuter operator having a slot that is used for

operations described in paragraph (a)(1) of this section but is not used

every day of the week shall notify the office specified in Sec. 93.221(a)(1)

in writing of those days on which the slots will not be used.

(5) Except as provided in paragraph (a)(10) of this section, at Kennedy

and O'Hare Airports, a slot shall be allocated, upon request, for seasonal

international operations, including charter operations, if the Chief Counsel

of the FAA determines that the slot had been permanently allocated to and

used by the requesting carrier in the same hour and for the same time period

during the corresponding season of the preceding year. Requests for such

slots must be submitted to the office specified in Sec. 93.221(a)(1) by May

15 for operations to be conducted during the following winter season and by

October 15 for the following summer season. For operations during the 1986

summer season, requests under this paragraph must have been submitted to the

FAA on or before February 1, 1986. Each carrier requesting a slot under this

paragraph must submit its entire international schedule at the relevant

airport for the particular season, noting which requests are in addition to

or changes from the previous year.

(6) Except as provided in paragraph (a)(10) of this section, additional

slots shall be allocated at O'Hare Airport for international scheduled air

carrier and commuter operations (beyond those slots allocated under Sec.

93.215 and Sec. 93.217(a)(5)) if a request is submitted to the office

specified in Sec. 93.221(a)(1) by May 15 for operations to commence during

the following winter season and by October 15 for operations to commence

during the following summer season. These slots will be allocated at the

time requested unless a slot is available within one hour of the requested

time, in which case the unallocated slot will be used to satisfy the request.

(7) If required by bilateral agreement, additional slots shall be allocated

at LaGuardia Airport for international scheduled passenger operations within

the hour requested.

(8) To the extent vacant slots are available, additional slots during the

high-density hours shall be allocated at Kennedy Airport for new

international scheduled air carrier and commuter operations (beyond those

operations for which slots have been allocated under Secs. 93.215 and

93.217(a)(5)), if a request is submitted to the office specified in Sec.

93.221(a)(1) by May 15 for operations to commence during the following winter

season and by October 15 for operations to commence during the following

summer season. In addition, slots may be withdrawn from domestic operations

for operations at Kennedy Airport under this paragraph if required by

international obligations.

(9) In determining the hour in which a slot request under Secs.

93.217(a)(6) and 93.217(a)(8) will be granted, the following will be taken

into consideration, among other things:

(i) The availability of vacant slot times;

(ii) International obligations;

(iii) Airport terminal capacity, including facilities and personnel of the

U.S. Customs Service and the U.S. Immigration and Naturalization Service;

(iv) The extent and regularity of intended use of a slot; and

(v) Schedule constraints of carriers requesting slots.

(10) At O'Hare Airport, a slot will not be allocated under this section to

a carrier holding or operating 100 or more permanent slots on the previous

May 15 for a winter season or October 15 for a summer season unless

(i) Allocation of the slot does not result in a total allocation to that

carrier under this section that exceeds the number of slots allocated to and

scheduled by that carrier under this section on February 23, 1990, and does

not exceed by more than 2 the number of slots allocated to and scheduled by

that carrier during any half hour of that day, or

(ii) Notwithstanding the number of slots allocated under paragraph

(a)(10)(i) of this section, a slot is available for allocation without

withdrawal of a permanent slot from any carrier.

(b) If a slot allocated under Sec. 93.215 was scheduled for an operation

described in paragraph (a)(1) of this section on December 16, 1985, its use

shall be subject to the requirements of paragraphs (a)(1) through (a)(4) of

this section. The requirements also apply to slots used for international

operations at LaGuardia Airport.

(c) If a slot is offered to a carrier in other than the hour requested, the

carrier shall have 14 days after the date of the offer to accept the newly

offered slot. Acceptance must be in writing and sent to the office specified

in Sec. 93.221(a)(1) and must repeat the certified statements required by

paragraph (e) of this section.

(d) The Office of the Secretary of Transportation reserves the right not to

apply the provisions of this section, concerning the allocation of slots, to

any foreign air carrier or commuter operator of a country that provides slots

to U.S. air carriers and commuter operators on a basis more restrictive than

provided by this subpart. Decisions not to apply the provisions of this

section will be made by the Office of the Secretary of Transportation.

(e) Each request for slots under this section shall state the airport, days

of the week and time of the day of the desired slots and the period of time

the slots are to be used. Each request shall identify whether the slot is

requested under paragraph (a)(5), (6), or (8) and identify any changes from

the previous year if requested under both paragraphs. The request must be

accompanied by a certified statement signed by an officer of the operator

indicating that the operator has or has contracted for aircraft capable of

being utilized in using the slots requested and that the operator has bona

fide plans to use the requested slots for operations described in paragraph

(a).

[Doc. No. 24105, Amdt. 93-52, 51 FR 21717, June 13, 1986, as amended by

Amdt. 93-61, 55 FR 53243, Dec. 27, 1990; 56 FR 1059, Jan. 10, 1991]

*****************************************************************************

55 FR 53238, No. 249, Dec. 27, 1990

SUMMARY: This action amends the procedures for allocation of air carrier

and commuter operator takeoff and landing slots at O'Hare International

Airport, to limit the availability of seasonal international slots at O'Hare

Airport for carriers with 100 or more permanent slots. The action responds

to a petition by United Airlines to limit the requirement for U.S. carriers

to furnish domestic slots for international operations by other carriers.

Under the rule adopted, slots generally will be not withdrawn from domestic

operators to accommodate international operations by carriers with 100 or

more slots at that airport, if the resulting allocation would exceed the

schedule operated by each such carrier for the winter 1989-90 season. As a

result, each large slot holder at the airport will be required to accommodate

new international operations primarily from its own slot base, rather than

the domestic slots of other carriers.

EFFECTIVE DATE: January 28, 1991. The rule applies to the allocation of

slots for flights that will be operated on or after October 27, 1991.

*****************************************************************************

Sec. 93.219   Allocation of slots for essential air service operations and

applicable limitations.

Whenever the Office of the Secretary of Transportation determines that

slots are needed for operations to or from a High Density Traffic Airport

under the Department of Transportation's Essential Air Service (EAS) Program,

those slots shall be provided to the designated air carrier or commuter

operator subject to the following limitations:

(a) Slots obtained under this section may not be bought, sold, leased or

otherwise transferred, except that such slots may be traded for other slots

on a one-for-one basis at the same airport.

(b) Any slot obtained under this section must be returned to the FAA if it

will not be used for EAS purposes for more than a 2-week period. A slot

returned under this paragraph may be reallocated to the operator which

returned it upon request to the FAA office specified in Sec. 93.221(a)(1) if

that slot has not been reallocated to an operator to provide substitute

essential air service.

(c) Slots shall be allocated for EAS purposes in a time period within 90

minutes of the time period requested.

(d) The Department will not honor requests for slots for EAS purposes to a

point if the requesting carrier has previously traded away or sold slots it

had used or obtained for use in providing essential air service to that

point.

(e) Slots obtained under Civil Aeronautics Board Order No. 84-11-40 shall

be considered to have been obtained under this section.

Sec. 93.221   Transfer of slots.

(a) Except as otherwise provided in this subpart, effective April 1, 1986,

slots may be bought, sold or leased for any consideration and any time period

and they may be traded in any combination for slots at the same airport or

any other high density traffic airport. Transfers, including leases, shall

comply with the following conditions:

(1) Requests for confirmation must be submitted in writing to Slot

Administration Office, AGC-230, Office of the Chief Counsel, Federal

Aviation Administration, 800 Independence Avenue, SW., Washington, DC 20591,

in a format to be prescribed by the Administrator. Requests will provide the

names of the transferor and recipient; business address and telephone number

of the persons representing the transferor and recipient; whether the slot

is to be used for an arrival or departure; the date the slot was acquired by

the transferor; the section of this subpart under which the slot was

allocated to the transferor; whether the slot has been used by the

transferor for international or essential air service operations; and

whether the slot will be used by the recipient for international or

essential air service operations. After withdrawal priorities have been

established under Sec. 93.223 of this part, the requests must include the

slot designations of the transferred slots as described in Sec. 93.223(b)(5).

(2) The slot transferred must come from the transferor's then-current FAA-

approved base.

(3) Written evidence of each transferor's consent to the transfer must be

provided to the FAA.

(4) The recipient of a transferred slot may not use the slot until written

confirmation has been received from the FAA.

(5)(i) Until a slot obtained by a new entrant or limited incumbent carrier

in a lottery held under Sec. 93.225 after June 1, 1991, has been used by the

carrier that obtained it for a continuous 24-month period after the lottery

in accordance with Sec. 93.227(a), that slot may be transferred only by trade

for one or more slots at the same airport or to other new entrant or limited

incumbent carriers under Sec. 93.221(a)(5)(iii). This transfer restriction

shall apply to the same extent to any slot or slots acquired by trading the

slot obtained in a lottery. To remove the transfer restriction, documentation

of 24 months' continuous use must be submitted to the FAA Office of the Chief

Counsel.

(ii) Failure to use a slot acquired by trading a slot obtained in a lottery

for a continuous 24-month period after the lottery, shall void all trades

involving the lottery slot, which shall be returned to the FAA. All use of

the lottery slot shall be counted toward fulfilling the minimum use

requirements under Sec. 93.227(a) applicable to the slot or slots for which

the lottery slot was traded, including subsequent trades.

(iii) Slots obtained by new entrant or limited incumbent carriers in a

lottery may be sold, leased, or otherwise transferred to another entrant or

limited incumbent carrier after a minimum of 60 days of use by the obtaining

carrier. The transfer restrictions of Sec. 93.221(a)(5)(i) shall continue to

apply to the slot until documentation of 24 months' continuous use has been

submitted and the transfer restriction removed.

(6) The Office of the Secretary of Transportation must determine that the

transfer will not be injurious to the essential air service program.

(b) A record of each slot transfer shall be kept on file by the office

specified in paragraph (a)(1) of this section and will be made available to

the public upon request.

(c) Any person may buy or sell slots and any air carrier or commuter may

use them. Notwithstanding Sec. 93.123, air carrier slots may be used with

aircraft of the kind described in Sec. 93.123 (c)(1) or (c)(2) but commuter

slots may only be used with aircraft of the kind described in Sec.

93.0123(c)(2).

(d) Air carriers and commuter operators considered to be a single operator

under the provisions of Sec. 93.213(c) of this Subpart but operating under

separate names shall report transfers of slots between them.

(e) Notwithstanding Sec. 93.123(c)(2) of this part, a commuter slot at

O'Hare International Airport may be used with an aircraft described in Sec.

93.123(c)(1) of this part on the following conditions:

(1) Air carrier aircraft that may be operated under this paragraph are

limited to aircraft:

(i) Having an actual seating configuration of 110 or fewer passengers; and

(ii) Having a maximum certificated takeoff weight of less than 126,000

pounds.

(2) No more than 50 percent of the total number of commuter slots held by a

slot holder at O'Hare International Airport may be used with aircraft

described in paragraph (e)(1) of this section.

(3) An air carrier or commuter operator planning to operate an aircraft

described in paragraph (e)(1) of this section in a commuter slot shall notify

ATC at least 75 days in advance of the planned start date of such operation.

The notice shall include the slot number, proposed time of operation,

aircraft type, aircraft series, actual aircraft seating configuration, and

planned start date. ATC will approve or disapprove the proposed operation no

later than 45 days prior to the planned start date. If an operator does not

initiate operation of a commuter slot under this section within 30 days of

the planned start date first submitted to the FAA, the ATC approval for that

operation will expire. That operator may file a new or revised notice for the

same half-hour slot time.

(4) An operation may not be conducted under paragraph (e)(1) of this

section unless a gate is available for that operation without planned waiting

time.

(5) For the purposes of this paragraph (e), notice to ATC shall be

submitted in writing to: Director, Air Traffic System Management, ATM-1,

Federal Aviation Administration, 800 Independence Avenue SW., Washington, DC

20591.

[Doc. No. 24105, Amdt. 93-49, 50 FR 52195, Dec. 20, 1985, as amended by Amdt.

93-52, 51 FR 21717, June 13, 1986; Amdt. 93-58, 54 FR 39293, Sept. 25, 1989;

Doc. No. 26339, Amdt. 93-62, 56 FR 41208, Aug. 19, 1991; Amdt. 93-65, 57 FR

37314, Aug. 18, 1992; Amdt. 93-68, 58 FR 39616, July 23, 1993]

*****************************************************************************

58 FR 39610, No. 140, July 23, 1993

SUMMARY: This action amends the regulations pertaining to the definition and

allocation of commuter operator slots (i.e., allocated instrument flight

rules takeoff and landing reservations) at O'Hare International Airport

(O'Hare). It permits an air carrier to use larger aircraft in up to 50

percent of its commuter slot holdings at O'Hare, expands the category of

aircraft that may be used in those slots, removes the restriction on the

maximum number of larger aircraft operations in commuter slots per half hour

or consecutive half hour periods, and makes permanent the use of the larger

aircraft in commuter slots at O'Hare by eliminating the provision that limits

the use of larger aircraft to a temporary 2-year trial period. This rule does

not increase the number of operations at O'Hare nor direct the enhancement or

reduction of air transport services to smaller communities.

EFFECTIVE DATE: August 25, 1993.

*****************************************************************************

Sec. 93.223  Slot withdrawal.

(a) Slots do not represent a property right but represent an operating

privilege subject to absolute FAA control. Slots may be withdrawn at any time

to fulfill the Department's operational needs, such as providing slots for

international or essential air service operations or eliminating slots.

Before withdrawing any slots under this section to provide them for

international operations, essential air services or other operational needs,

those slots returned under Sec. 93.224 of this part and those recalled by the

agency under Sec. 93.227 will be allocated.

(b) Separate slot pools shall be established for air carriers and commuter

operators at each airport. The FAA shall assign, by random lottery,

withdrawal priority numbers for the recall priority of slots at each airport.

Each additional permanent slot, if any, will be assigned the next higher

number for air carrier or commuter slots, as appropriate, at each airport.

Each slot shall be assigned a designation consisting of the applicable

withdrawal priority number; the airport code; a code indicating whether the

slot is an air carrier or commuter operator slot; and the time period of the

slot. The designation shall also indicate, as appropriate, if the slot is

daily or for certain days of the week only; is limited to arrivals or

departures; is allocated for international operations or for EAS purposes;

and, at Kennedy International Airport, is a summer or winter slot.

(c) Whenever slots must be withdrawn, they will be withdrawn in accordance

with the priority list established under paragraph (b) of this section,

except:

(1) Slots obtained in a lottery held pursuant to Sec. 93.225 of this part

shall be subject to withdrawal pursuant to paragraph (i) of that section, and

(2) Slots necessary for international and essential air service operations

shall be exempt from withdrawal for use for other international or essential

air service operations.

(3) Except as provided in Sec. 93.227(a), the FAA shall not withdraw slots

held at an airport by an air carrier or commuter operator holding and

operating 12 or fewer slots at that airport (excluding slots used for

operations described in Sec. 93.212(a)(1)), if withdrawal would reduce the

number of slots held below the number of slots operated.

(d) The following withdrawal priority rule shall be used to permit

application of the one-for-one trade provisions for international and

essential air service slots and the slot withdrawal provisions where the

slots are needed for other than international or essential air service

operations. If an operator has more than one slot in a specific time period

in which it also has a slot being used for international or essential air

service operations, the international and essential air service slots will be

considered to be those with the lowest withdrawal priority.

(e) The operator(s) using each slot to be withdrawn shall be notified by

the FAA of the withdrawal and shall cease operations using that slot on the

date indicated in the notice. Generally, the FAA will provide at least 30

days after notification for the operator to cease operations unless

exigencies require a shorter time period.

(f) For 24 months following a lottery held after June 1, 1991, a slot

acquired in that lottery shall be withdrawn by the FAA upon the sale, merger,

or acquisition of more than 50 percent ownership or control of the carrier

using that slot or one acquired by trade of that slot, if the resulting total

of slots held or operated at the airport by the surviving entity would exceed

12 slots.

[Doc. No. 24105, Amdt. 93-49, 50 FR 52195, Dec. 20, 1985, as amended by Amdt.

93-52, 51 FR 21718, June 13, 1986; Amdt. 93-57, 54 FR 34906, Aug. 22, 1989;

Amdt. 93-65, 57 FR 37314, Aug. 18, 1992]

*****************************************************************************

57 FR 37308, No. 160, Aug. 18, 1992

SUMMARY: This action amends the Federal Aviation Regulations governing the

allocation and transfer of air carrier and commuter operator slots (i.e.,

instrument flight rules (IFR) takeoff and landing reservations) at Kennedy

International Airport, LaGuardia Airport, O'Hare International Airport, and

Washington National Airport. This amendment adds a definition of "limited

incumbent carrier" to refer to air carriers and commuter operators holding

fewer than 12 slots at a high density airport; provides for allocation of

slots by lottery to certain new entrant and limited incumbent carriers;

restricts the transfer of newly acquired lottery slots and provides for the

recall of those slots upon the sale or merger of the limited incumbent slot

holder; amends the minimum slot use requirements; amends the slot use

reporting requirements; and removes an obsolete penalty provision. This

amendment is designed to promote the availability of slots to new entrant and

limited incumbent carriers at the high density airports, and thereby enhance

competition.

EFFECTIVE DATE: November 1, 1992.

*****************************************************************************

Sec. 93.224  Return of slots.

(a) Whenever a slot is required to be returned under this subpart, the

holder must notify the office specified in Sec. 93.221(a)(1) in writing of

the date after which the slot will not be used.

(b) Slots may be voluntarily returned for use by other operators by

notifying the office specified in Sec. 93.221(a)(1) in writing.

Sec. 93.225  Lottery of available slots.

(a) Whenever the FAA determines that sufficient slots have become available

for distribution for purposes other than international or essential air

service operations, but generally not more than twice a year, they shall be

allocated in accordance with the provisions of this section.

(b) A random lottery shall be held to determine the order of slot

selection.

(c) Slot allocation lotteries shall be held on an airport-by-airport basis

with separate lotteries for air carrier and commuter operator slots. The

slots to be allocated in each lottery will be each unallocated slot not

necessary for international or Essential Air Service Program operations,

including any slot created by an increase in the operating limits set forth

in Sec. 93.123(a).

(d) The FAA shall publish a notice in the Federal Register announcing any

lottery dates. The notice may include special procedures to be in effect for

the lotteries.

(e) Participation in a lottery is open to each U.S. air carrier or commuter

operator operating at the airport and providing scheduled passenger service

at the airport. Any U.S. carrier that is not operating scheduled service at

the airport and has not failed to operate slots obtained in the previous

lottery, or slots traded for those obtained by lottery, but wishes to

initiate scheduled passenger service at the airport, shall be included in the

lottery if that operator notifies, in writing, the Slot Administration

Office, AGC-230, Office of the Chief Counsel, Federal Aviation

Administration, 800 Independence Avenue SW., Washington, DC 20591. The

notification must be received 15 days prior to the lottery date and state

whether there is any common ownership or control of, by, or with any other

air carrier or commuter operator as defined in Sec. 93.213(c). New entrant

and limited incumbent carriers will be permitted to complete their selections

before participation by other incumbent carriers is initiated.

(f) At the lottery, each operator must make its selection within 5 minutes

after being called or it shall lose its turn. If capacity still remains after

each operator has had an opportunity to select slots, the allocation sequence

will be repeated in the same order. An operator may select any two slots

available at the airport during each sequence, except that new entrant

carriers may select four slots, if available, in the first sequence.

(g) To select slots during a slot lottery session, a carrier must have

appropriate economic authority for scheduled passenger service under Title IV

of the Federal Aviation Act of 1958, as amended (49 U.S.C. App. 1371 et

seq.), and must hold FAA operating authority under part 121 or part 135 of

this chapter as appropriate for the slots the operator seeks to select.

(h) During the first selection sequence, 25 percent of the slots available

but no less than two slots shall be reserved for selection by new entrant

carriers. If new entrant carriers do not select all of the slots set aside

for new entrant carriers, limited incumbent carriers may select the remaining

slots. If every participating new entrant carrier and limited incumbent

carrier has ceased selection of available slots or has obtained 12 slots at

that airport, other incumbent carriers may participate in selecting the

remaining slots; however, slots selected by non-limited incumbent carriers

will be allocated only until the date of the next lottery.

(i) Slots obtained under this section shall retain their withdrawal

priority as established under Sec. 93.223. If the slot is newly created, a

withdrawal priority shall be assigned. That priority number shall be higher

than any other slot assigned a withdrawal number previously.

[Doc. No. 24105, Amdt. 93-49, 50 FR 52195, Dec. 20, 1985, as amended by Amdt.

93-52, 51 FR 21718, June 13, 1986; Amdt. 93-58, 54 FR 39293, Sept. 25, 1989;

Amdt. 93-65, 57 FR 37314, Aug. 18, 1992; 57 FR 47993, Oct. 21, 1992]

*****************************************************************************

57 FR 37308, No. 160, Aug. 18, 1992

SUMMARY: This action amends the Federal Aviation Regulations governing the

allocation and transfer of air carrier and commuter operator slots (i.e.,

instrument flight rules (IFR) takeoff and landing reservations) at Kennedy

International Airport, LaGuardia Airport, O'Hare International Airport, and

Washington National Airport. This amendment adds a definition of "limited

incumbent carrier" to refer to air carriers and commuter operators holding

fewer than 12 slots at a high density airport; provides for allocation of

slots by lottery to certain new entrant and limited incumbent carriers;

restricts the transfer of newly acquired lottery slots and provides for the

recall of those slots upon the sale or merger of the limited incumbent slot

holder; amends the minimum slot use requirements; amends the slot use

reporting requirements; and removes an obsolete penalty provision. This

amendment is designed to promote the availability of slots to new entrant and

limited incumbent carriers at the high density airports, and thereby enhance

competition.

EFFECTIVE DATE: November 1, 1992.

*****************************************************************************

Sec. 93.226   Allocation of slots in low-demand periods.

(a) If there are available slots in the following time periods and there

are no pending requests for international or EAS operations at these times,

FAA will allocate slots upon request on a first-come, first-served basis, as

set forth in this section:

(1) Any period for which a slot is available less than 5 days per week.

(2) Any time period for which a slot is available for less than a full

season.

(3) For LaGuardia and Washington National Airports:

(i) 6:00 a.m.-6:59 a.m.

(ii) 10:00 p.m.-midnight.

(b) Slots will be allocated only to operators with the economic and

operating authority and aircraft required to use the slots.

(c) Requests for allocations under this section shall be submitted in

writing to the address listed in Sec. 93.221(a)(1) and shall identify the

request as made under this section.

(d) The FAA may deny requests made under this section after a determination

that all remaining slots in a particular category should be distributed by

lottery.

(e) Slots may be allocated on a seasonal or temporary basis under this

provision.

[Doc. No. 24105, Amdt. 93-52, 51 FR 21718, June 13, 1986]

Sec. 93.227  Slot use and loss.

(a) Except as provided in paragraphs (b), (c), (d), (g), and (l) of this

section, any slot not utilized 80 percent of the time over a 2-month period

shall be recalled by the FAA.

(b) Paragraph (a) of this section does not apply to slots obtained under

Sec. 93.225 of this part during:

(i) The first 90 days after they are allocated to a new entrant carrier; or

(ii) The first 60 days after they are allocated to a limited incumbent or

other incumbent carrier.

(c) Paragraph (a) of this section does not apply to slots of an operator

forced by a strike to cease operations using those slots.

(d) In the case of a carrier that files for protection under the Federal

bankruptcy laws and has not received a Notice of Withdrawal from the FAA for

the subject slot or slots, paragraph (a) of this section does not apply:

(1) During a period after the initial petition in bankruptcy, to any slot

held or operated by that carrier, for:

(i) 60 days after the carrier files the initial petition in bankruptcy; and

(ii) 30 days after the carrier, in anticipation of transferring slots,

submits information to a Federal government agency in connection with a

statutory antitrust, economic impact, or similar review of the transfer,

provided that the information is submitted more than 30 days after filing the

initial petition in bankruptcy, and provided further that any slot to be

transferred has not become subject to withdrawal under any other provision of

this Sec. 93.227; and

(2) During a period after a carrier ceases operations at an airport, to any

slot held or operated by that carrier at that airport, for:

(i) 30 days after the carrier ceases operations at that airport, provided

that the slot has not become subject to withdrawal under any other provision

of this Sec. 93.227; and

(ii) 30 days after the parties to a proposed transfer of any such slot

comply with requests for additional information by a Federal government

agency in connection with an antitrust, economic impact, or similar

investigation of the transfer, provided that--

(A) The original notice of the transfer is filed with the Federal agency

within 30 days after the carrier ceases operation at the airport;

(B) The request for additional information is made within 10 days of the

filing of the notice by the carrier;

(C) The carrier submits the additional information to the Federal agency

within 15 days of the request by such agency; and

(D) Any slot to be transferred has not become subject to withdrawal under

any other provision of this Sec. 93.227.

(e) Persons having slots withdrawn pursuant to paragraph (a) of this

section must cease all use of those slots upon receipt of notice from the

FAA.

(f) Persons holding slots but not using them pursuant to the provisions of

paragraphs (b), (c) and (d) may lease those slots for use by others. A slot

obtained in a lottery may not be leased after the expiration of the

applicable time period specified in paragraph (b) of this section unless it

has been operated for a 2-month period at least 65 percent of the time by the

operator which obtained it in the lottery.

(g) This section does not apply to slots used for the operations described

in Sec. 93.217(a)(1) except that a U.S. air carrier or commuter operator

required to file a report under paragraph (i) of this section shall include

all slots operated at the airport, including slots described in Sec.

93.217(a)(1).

(h) Within 30 days after an operator files for protection under the Federal

bankruptcy laws, the FAA shall recall any slots of that operator, if--(1) the

slots were formerly used for essential air service and (2) the Office of the

Secretary of Transportation determines those slots are required to provide

substitute essential air service to or from the same points.

(i) Every air carrier and commuter operator or other person holding a slot

at a high density airport shall, within 14 days after the last day of the 2-

month period beginning January 1, 1986, and every 2 months thereafter,

forward, in writing, to the address identified in Sec. 93.221(a)(1), a list

of all slots held by the air carrier, commuter operator or other person along

with a listing of which air carrier or commuter operator actually operated

the slot for each day of the 2-month period. The report shall identify the

flight number for which the slot was used and the equipment used, and shall

identify the flight as an arrival or departure. The report shall identify any

common ownership or control of, by, or with any other carrier as defined in

Sec. 93.213(c) of this Subpart. The report shall be signed by a senior

official of the air carrier or commuter operator. If the slot is held by an

"other person," the report must be signed by an official representative.

(j) The Chief Counsel of the FAA may waive the requirements of paragraph

(a) of this section in the event of a highly unusual and unpredictable

condition which is beyond the control of the slot-holder and which exists for

a period of 9 or more days. Examples of conditions which could justify waiver

under this paragraph are weather conditions which result in the restricted

operation of an airport for an extended period of time or the grounding of an

aircraft type.

(k) The Chief Counsel of the FAA may, upon request, grant a waiver from the

requirements of paragraph (a) of this section for a slot used for the

domestic segment of an intercontinental all-cargo flight. To qualify for a

waiver, a carrier must operate the slot a substantial percentage of the time

and must return the slot to the FAA in advance for the time periods it will

not be used.

(l) The FAA will treat as used any slot held by a carrier at a High Density

Traffic Airport on Thanksgiving Day, the Friday following Thanksgiving Day,

and the period from December 24 through the first Saturday in January.

[Doc. No. 24105, Amdt. 93-49, 50 FR 52195, Dec. 20, 1985, as amended by Amdt.

93-52, 51 FR 21718, June 13, 1986; Amdt. 93-65, 57 FR 37315, Aug. 18, 1992;

Amdt. 93-71, 59 FR 58771, Nov. 15, 1994]

*****************************************************************************

59 FR 58770, No. 219, Nov. 15, 1994

SUMMARY: This action amends the Federal Aviation Regulations pertaining to

the slot use and loss provisions for air carrier and commuter operator slots

(i.e., instrument flight rules (IFR) takeoff and landing reservations) at

John F. Kennedy International Airport (JFK), LaGuardia Airport, O'Hare

International Airport (O'Hare), and Washington National Airport. This action

codifies the agency's historical practice of treating as used any slot held

but not actually operated on Thanksgiving Day, the Friday following

Thanksgiving Day, and the period from December 24 through the first Saturday

in January. The amendment will permit carriers and commuters to choose which

flights to operate at any of the High Density Rule airports during certain

days of the winter holiday season without jeopardizing the status of the

slots under the "use or lose" requirement.

EFFECTIVE DATE: November 15, 1994.

*****************************************************************************

Sec. 93.229   [Removed. Amdt. 93-65, 57 FR 37315, Aug. 18, 1992]

*****************************************************************************

57 FR 37308, No. 160, Aug. 18, 1992

SUMMARY: This action amends the Federal Aviation Regulations governing the

allocation and transfer of air carrier and commuter operator slots (i.e.,

instrument flight rules (IFR) takeoff and landing reservations) at Kennedy

International Airport, LaGuardia Airport, O'Hare International Airport, and

Washington National Airport. This amendment adds a definition of "limited

incumbent carrier" to refer to air carriers and commuter operators holding

fewer than 12 slots at a high density airport; provides for allocation of

slots by lottery to certain new entrant and limited incumbent carriers;

restricts the transfer of newly acquired lottery slots and provides for the

recall of those slots upon the sale or merger of the limited incumbent slot

holder; amends the minimum slot use requirements; amends the slot use

reporting requirements; and removes an obsolete penalty provision. This

amendment is designed to promote the availability of slots to new entrant and

limited incumbent carriers at the high density airports, and thereby enhance

competition.

EFFECTIVE DATE: November 1, 1992.

*****************************************************************************

Subpart T--Washington National Airport Traffic Rules

Source: Docket No. 25143, Amdt. 93-54, 51 FR 43587, Dec. 3, 1986, unless

otherwise noted.

Sec. 93.251   Applicability.

This subpart prescribes rules applicable to the operation of aircraft to or

from Washington National Airport.

Sec. 93.253   Nonstop operations.

No person may operate an aircraft nonstop in air transportation between

Washington National Airport and another airport that is more than 1,250 miles

away from Washington National Airport.

Appendix A

I. Anchorage Airport Traffic Area; Traffic Patterns

[ ...Illustration appears here... ]

[Doc. No. 1580, Amdt. 1-1, 28 FR 6717, June 29, 1963, as amended by Amdt. 93-

29, 39 FR 32552, Sept. 9, 1974]

Appendix B to Part 93--[Removed. Amdt. 93-68, 58 FR 39616, July 23, 1993]

*****************************************************************************

58 FR 39610, No. 140, July 23, 1993

SUMMARY: This action amends the regulations pertaining to the definition and

allocation of commuter operator slots (i.e., allocated instrument flight

rules takeoff and landing reservations) at O'Hare International Airport

(O'Hare). It permits an air carrier to use larger aircraft in up to 50

percent of its commuter slot holdings at O'Hare, expands the category of

aircraft that may be used in those slots, removes the restriction on the

maximum number of larger aircraft operations in commuter slots per half hour

or consecutive half hour periods, and makes permanent the use of the larger

aircraft in commuter slots at O'Hare by eliminating the provision that limits

the use of larger aircraft to a temporary 2-year trial period. This rule does

not increase the number of operations at O'Hare nor direct the enhancement or

reduction of air transport services to smaller communities.

EFFECTIVE DATE: August 25, 1993.

*****************************************************************************